| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MO, 9–18 Jan. 2019 | Document: JVET-M0839 v2 |
| --- | --- |

| Title: | CE2-related: On number of fast merge candidates for Affine Merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert, F. Le Léannec, F. Galpin, T. Poirier | Email: | antoine.robert@technicolor.com fabrice.leleannec@technicolor.com franck.galpin@technicolor.com tangi.poirier@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes to increase the number of fast merge candidates considered in Affine Merge mode to take into account the added ATMVP candidate. The proposed process has been implemented in the JVET VTM3.0. The reported simulation results show average luma BD-rate gains of:

- -0.04% in RA (up to -0.06% in class A1) and -0.06% in LDB, when increasing to 2 candidates in case ATMVP is present,

- -0.05% in RA (up to -0.09% in class A1) and -0.05% in LDB, when increasing to 2 candidates,

- -0.07% in RA (up to -0.13% in class A1) and -0.07% in LDB, when increasing to 3 candidates,

- -0.07% in RA (up to -0.14% in class A1) and -0.08% in LDB, when increasing to 4 candidates.

In RA, the encoding and decoding times stay identical to the VTM-3.0 ones in all tested configurations. In LDB, a slight increase of 1% encoding time is observed when 3 candidates are considered, and 2% for 4 candidates.

# Introduction

In the VTM-3.0 and previous versions, both Merge and Affine Merge modes use a fast merge encoder method that computes the SAD of the different possible candidates before performing the full RDO computation. In Merge mode, up to 4 candidates can be selected after this fast merge method, but in Affine Merge mode, it is limited to 1 candidate.

At the last meeting in Macao, it has been adopted to move the ATMVP candidate from Merge to Affine Merge mode.

This contribution proposes to increase the number of fast merge candidates considered in Affine Merge mode to consider the additional ATMVP candidate.

# Description

The ATMVP candidate has been moved from Merge to Affine Merge mode, but the number of fast merge candidates considered in Affine Merge mode has not been increased consequently. So, when the fast merge method selects the ATMVP candidate, the Affine Merge mode cannot use an affine model.

Fast merge method performs the motion compensation of each candidate in the list and computes the SAD between the prediction and the current CU to sort the candidates in ascending order of SAD. Then, the fast merge method selects the smallest one and the following ones that have a SAD smaller than one quarter more than the first candidate’s SAD within the limit of the allowed number of fast merge candidates. Finally, the end of the RDO process (without motion compensation already done) is only performed for the selected fast merge candidates.

This contribution then proposes to increase the number of fast merge candidates:

- By one more candidate, when there is an ATMVP candidate,

- To 2 candidates,

- To 3 candidates (as in Merge as the list size minus 2),

- To 4 candidates as in Merge.

# Simulation results

The simulations are conducted using the VTM configuration as described in JVET-L1010 [1]. VTM-3.0 reference software is used with VTM configuration files.

## Test a): ATMVP increase

Table 1 and Table 2 show the simulation results of the proposed number of fast merge candidate increase in case of presence of an ATMVP candidate in random access and low delay B configurations. In both configurations, the BD-rate performances are increased up to -0.06% in classes A with the same complexity in encoding/decoding time.

Table 1: Simulation results of the proposed STAD candidates increase in case of ATMVP in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.02% | 0.00% | 99% | 98% |
| Class A2 | -0.06% | -0.05% | 0.04% | 100% | 99% |
| Class B | -0.03% | 0.02% | -0.08% | 100% | 98% |
| Class C | -0.04% | -0.01% | 0.00% | 100% | 98% |
| Overall | -0.04% | -0.01% | -0.02% | 100% | 98% |
| Class D | 0.01% | -0.07% | -0.06% | 100% | 98% |
| Class F (optional) | -0.01% | -0.04% | -0.03% | 100% | 98% |

Table 2: Simulation results of the proposed STAD candidates increase in case of ATMVP in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.01% | -0.17% | 0.09% | 102% | 99% |
| Class C | -0.01% | 0.04% | -0.06% | 101% | 101% |
| Class E | -0.20% | -0.26% | -0.44% | 101% | 99% |
| Overall | -0.06% | -0.12% | -0.09% | 101% | 100% |
| Class D | 0.00% | -0.11% | -0.20% | 101% | 99% |
| Class F (optional) | -0.01% | -0.36% | -0.31% | 101% | 100% |

## Test b): 2 fast merge candidates

Table 3 and Table 4 show the simulation results of increasing to 2 the number of fast merge candidates in Affine Merge mode in random access and low delay B configurations. In both configurations, the BD-rate performances are increased up to -0.09% in class A1 with the same complexity in encoding/decoding time.

Table 3: Simulation results for 2 fast Merge candidates in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.17% | -0.08% | 99% | 98% |
| Class A2 | -0.07% | -0.03% | 0.01% | 100% | 99% |
| Class B | -0.05% | 0.04% | -0.03% | 100% | 97% |
| Class C | -0.02% | -0.05% | -0.16% | 100% | 99% |
| Overall | -0.05% | -0.04% | -0.07% | 100% | 98% |
| Class D | -0.04% | -0.04% | -0.06% | 101% | 97% |
| Class F (optional) | -0.01% | -0.02% | -0.04% | 100% | 99% |

Table 4: Simulation results for 2 fast Merge candidates in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.04% | -0.32% | -0.02% | 101% | 100% |
| Class C | 0.00% | 0.08% | 0.37% | 101% | 100% |
| Class E | -0.15% | 0.34% | -0.46% | 100% | 98% |
| Overall | -0.05% | -0.02% | 0.00% | 101% | 100% |
| Class D | 0.00% | -0.17% | -0.14% | 101% | 99% |
| Class F (optional) | 0.05% | -0.03% | 0.32% | 101% | 100% |

## Test c): 3 fast merge candidates

Table 5 and Table 6 show the simulation results of increasing to 3 the number of fast merge candidates in Affine Merge mode in random access and low delay B configurations. In both configurations, the BD-rate performances are increased up to -0.13% in class A1 with the same complexity in encoding/decoding time in RA and 1% increase in encoding time in LDB.

Table 5: Simulation results for 3 fast Merge candidates in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.02% | -0.11% | 100% | 98% |
| Class A2 | -0.09% | -0.09% | -0.12% | 101% | 98% |
| Class B | -0.05% | 0.05% | 0.01% | 100% | 97% |
| Class C | -0.01% | 0.05% | -0.12% | 101% | 99% |
| Overall | -0.07% | 0.01% | -0.07% | 100% | 98% |
| Class D | -0.05% | -0.05% | -0.06% | 101% | 98% |
| Class F (optional) | -0.03% | -0.04% | -0.02% | 101% | 99% |

Table 6: Simulation results for 3 fast Merge candidates in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.04% | -0.39% | -0.13% | 102% | 97% |
| Class C | -0.01% | 0.27% | 0.18% | 102% | 101% |
| Class E | -0.18% | 0.30% | 0.00% | 101% | 100% |
| Overall | -0.07% | 0.00% | 0.00% | 102% | 99% |
| Class D | -0.04% | -0.07% | 0.07% | 101% | 99% |
| Class F (optional) | 0.05% | 0.03% | -0.25% | 101% | 100% |

## Test d): 4 fast merge candidates

Table 7 and Table 8 show the simulation results of increasing to 4 the number of fast merge candidates in Affine Merge mode in random access and low delay B configurations. In both configurations, the BD-rate performances are increased up to -0.14% in class A1 with the same complexity in encoding/decoding time in RA and 2% increase in encoding time in LDB.

Table 7: Simulation results for 4 fast Merge candidates in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.12% | -0.07% | 100% | 99% |
| Class A2 | -0.09% | -0.11% | -0.06% | 101% | 99% |
| Class B | -0.07% | -0.01% | -0.09% | 100% | 97% |
| Class C | -0.01% | -0.04% | -0.04% | 101% | 99% |
| Overall | -0.07% | -0.06% | -0.07% | 100% | 98% |
| Class D | -0.04% | 0.00% | 0.01% | 102% | 99% |
| Class F (optional) | -0.03% | -0.02% | -0.07% | 101% | 99% |

Table 8: Simulation results for 4 fast Merge candidates in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.05% | -0.19% | 0.10% | 103% | 99% |
| Class C | -0.07% | -0.07% | 0.04% | 103% | 100% |
| Class E | -0.14% | -0.31% | -0.15% | 102% | 99% |
| Overall | -0.08% | -0.18% | 0.02% | 103% | 99% |
| Class D | -0.10% | 0.02% | 0.22% | 102% | 99% |
| Class F (optional) | -0.02% | 0.46% | -0.38% | 102% | 100% |

# Conclusion

This contribution proposes to increase the number of fast merge candidates considered in Affine Merge mode to take into account the added ATMVP candidate. Four different strategies are explored by increasing this number of fast merge candidates to: 2 when ATMVP candidate is present, to 2, 3, or 4 The simulation results show average luma BD-rate gains of:

- -0.04% in RA (up to -0.06% in class A1) and -0.06% in LDB, when increasing to 2 candidates in case ATMVP is present,

- -0.05% in RA (up to -0.09% in class A1) and -0.05% in LDB, when increasing to 2 candidates,

- -0.07% in RA (up to -0.13% in class A1) and -0.07% in LDB, when increasing to 3 candidates,

- -0.07% in RA (up to -0.14% in class A1) and -0.08% in LDB, when increasing to 4 candidates.

In RA, the encoding and decoding times stay identical to the VTM-3.0 ones in all tested configurations. In LDB, a slight increase of 1% encoding time is observed when 3 candidates are considered, and 2% for 4 candidates.
It is suggested that the increase of number of fast merge candidates considered in Affine Merge mode to 2 when ATMVP candidate is present (test a) or always 2 (test b) as proposed in this contribution should be adopted into the reference test model.

# References

[1] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1010, 12th Meeting, Macao, CN, October 2018.

[2] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 2 (CE2): Sub-block based motion prediction”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1022, 12th Meeting, Macao, CN, October 2018.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).