| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0851-v3 |
| --- | --- |

| Title: | CE10-related: Using inter merge list derivation for triangle mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang, Wei-Jung Chien, Vadim Seregin, Yung-Hsuan Chao, Han Huang, Marta Karczewicz (Qualcomm) Xianglin Wang, Yi-Wen Chen (Kwai) Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei (MediaTek) Anish Tamse, Min Woo Park, Seungsoo Jeong, Minsoo Park, Kiho Choi (Samsung) X. Zheng (DJI), X.W. Meng (Peking University) | Tel: Email: | 1-858-658-4535 hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Incorporated, Kwai Inc., MediaTek Inc., Samsung Electronics Co., Ltd, Peking University, SZ DJI Technology Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents test results for using regular inter merge list construction process to derive initial merge candidates for triangle mode. Test results reportedly show 0.07% and 0.08% BD rate loss in RA and LB configurations, respectively.

# Introduction

In VVC triangular PU mode, the merge candidate derivation consists of two steps:

- A procedure similar to regular merge candidate list derivation is performed to generate a candidate set (S0) containing up to 7 motion vectors, where 5 spatial and 2 TMVP candidates are derived.

- A final candidate set of 5 uni-directional motion vectors (S1) is further generated based on S0. The consideration order of motion vectors is as follows:

  - Uni-directional MV(s).

  - List 0 part of bi-directional MV(s).

  - List 1 part of bi-directional MV(s).

  - The average motion vector of List 0 and List 1 part of bi-directional MV(s).

  - Zero MVs.

# Proposed method

In the contributions submitted to 13th JVET meeting: JVET-M0184, JVET-M0194, JVET-M0233, JVET-M0350, JVET-M0399, it was proposed to simplify triangle mode merge candidate list derivation and the common part of those contributions is to reuse regular merge list construction process.

In this contribution, it proposed to use inter merge candidate derivation process to derive candidate list S0.

# Test results

The proposed method was implemented on top of VTM-3.0 software and tested under common test conditions described in JVET-L1010.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.12% | 0.10% | 99% | 99% |
| Class A2 | 0.03% | 0.13% | 0.15% | 100% | 99% |
| Class B | 0.04% | 0.11% | -0.02% | 101% | 97% |
| Class C | 0.15% | 0.15% | 0.19% | 99% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.13% | 0.09% | 100% | 97% |
| Class D | 0.11% | 0.12% | 0.16% | 100% | 97% |
| Class F (optional) | 0.04% | 0.03% | 0.04% | 101% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.15% | 0.00% | 99% | 94% |
| Class C | 0.16% | 0.40% | 0.23% | 101% | 91% |
| Class E | 0.04% | -0.12% | 0.25% | 101% | 93% |
| Overall | 0.08% | 0.04% | 0.14% | 100% | 93% |
| Class D | 0.13% | -0.08% | 0.45% | 100% | 95% |
| Class F (optional) | -0.08% | 0.25% | 0.03% | 100% | 92% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).