| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0853-v2 |
| --- | --- |

| Title: | On Tile Grouping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Hendry Ye-Kui Wang Miska M. Hannuksela Yong He Lulin Chen Woong Il Choi B. D. Choi Rickard Sjöberg Robert Skupin | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc., Huawei, Nokia, Interdigital, MediaTek, Samsung, Tencent, Ericsson, HHI |  |  |

_____________________________

# Abstract

Proposed text is provided as a requested by BoG on Tiles and WPP. The revision reflects modifications done during BoG presentation of the document.

# Introduction

BoG on Tiles and WPP[2] recommended following:

2. Tile grouping

a. To have rectangular tile grouping?

    - Yes? (M0121, M0130, M0160, M0209, M0416, M0261 option 2 without MCTS)

Agreed that rectangular tile grouping is a desirable feature. Proponents and interested parties were suggested to work offline to harmonize the syntax and semantics, together with the offline activity below for supporting of both rectangular tile groups and tile raster scan ordered tile groups.

…

b. …

ii) (Rectangular tile grouping only with tile group information in PPS only) or (tile grouping with tiles within a tile group in tile raster scan order with tile grouping in tile group header only), the switching flag in the PPS. This option was preferred by the BoG. Interested parties were suggested to create a text, including harmonization of the syntax and semantics for rectangular tile grouping. To be coordinated by Sachin and Hendry. Revisit when the text is available.

…

- How to signal tile group address in the tile group header?

For rectangular tile grouping, have one flag in the PPS to determine whether there is a mapping in the PPS between tile group indices and tile group IDs, and signal the tile group ID in the tile group header. If the PPS flag is equal to 0, tile group ID in the tile group header is tile group index.

The above was agreed, pending on review of a harmonized text. Proponents and interested parties were suggested to work offline to harmonize the syntax and semantics. To be coordinated by Sachin.

# Proposed text

The proposed text is shown below. The changes with respect to WD [1] are highlighted.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| } |  |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| if(!single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Semantics for new / modified elements are shown below.

single_tile_per_tile_group equal to 1 specifies that each tile group that refers to this PPS includes one tile. single_tile_per_tile_group equal to 0 specifies that a tile group that refers to this PPS may include more than one tile.

rect_tile_group_flag equal to 0 specifies that tiles within each tile group are in raster scan order and the tile group information is not signalled in PPS. rect_tile_group_flag equal to 1 specifies that tiles within each tile group cover a rectangular region of the picture and the tile group information is signalled in the PPS. When single_tile_per_tile_group_flag is equal to 1 rect_tile_group_flag is inferred to be equal to 1.

num_tile_groups_in_pic_minus1 plus 1 specifies the number of tile groups in each picture referring to the PPS. The value of num_tile_groups_in_pic_minus1 shall be in the range of 0 to (NumTilesInPic − 1), inclusive. When not present and single_tile_per_tile_group_fla is equal to 1, the value of  num_tile_groups_in_pic_minus1 is inferred to be equal to (NumTilesInPic – 1).

top_left_tile_idx[ i ] specifies the tile index of the tile located at the top-left corner of the i-th tile group. The value of top_left_tile_idx[ i ] shall not be equal to the value of top_left_tile_idx[ j ] for any i not equal to j. When not present, top_left_tile_idx[ i ] is inferred to be equal to i. The length of the top_left_tile_idx[ i ] syntax element is Ceil( Log2(  NumTilesInPic ) bits.

bottom_right_tile_idx[ i ] specifies the tile index of the tile located at the bottom-right corner of the i-th tile group. When single_tile_per_tile_group_flag is equal to 1 bottom_right_tile_idx[ i ] is inferred to be equal to top_left_tile_idx[ i ]. The length of the bottom_right_tile_idx[ i ] syntax element is Ceil( Log2( NumTilesInPic) ) bits.

It is a requirement of bitstream conformance that any particular tile shall only be included in one tile group.

The variable NumTilesInTileGroup[ i ], which specifies the number of tiles in the tile group, and related variables, are derived as follows:

deltaTileIdx[ i ]  = bottom_right_tile_idx[ i ] – top_right_tile_idx[ i ]
NumTileRowsInTileGroupMinus1[ i ]  = ( deltaTileIdx[ i ] / ( num_tile_columns_minus1 + 1 ) )  
NumTileColumnsInTileGroupMinus1[ i ]   = ( deltaTileIdx[ i ] % ( num_tile_columns_minus1 + 1 ) )  
NumTilesInTileGroup[ i ]  = ( NumTileRowsInTileGroupMinus1[ i ] + 1 ) * 
	( NumTileColumnsInTileGroupMinus1[ i ] + 1 )

signalled_tile_group_id_flag equal to 1 specifies that the tile group ID for each tile group is signalled. signalled_tile_group_index_flag equal to 0 specifies that tile group IDs are not signalled. When rect_tile_group_flag is equal to 0, the value of signalled_tile_group_index_flag is inferred to be equal to 0.

signalled_tile_group_id_length_minus1 plus 1 specifies the number of bits used to represent the syntax element tile_group_id[ i ] when present, and the syntax element tile_group_address in tile group headers. The value of signalled_tile_group_index_length_minus1 shall be in the range of 0 to 15, inclusive. When not present, the value of signalled_tile_group_index_length_minus1 is inferred to be equal to Ceil( Log2( num_tile_groups_in_pic_minus1 + 1 ) ) − 1.

tile_group_id[ i ] specifies the tile group ID of the i-th tile group. The length of the tile_group_id[ i ] syntax element is tile_set_id_length_minus1 + 1 bits.  When not present, tile_group_id[ i ] is inferred to be equal to i, for each i in the range of 0 to num_tile_groups_in_pic_minus1, inclusive.

### 6.5.1 CTB raster and tile scanning process

The list ColWidth[ i ] for i ranging from 0 to num_tile_columns_minus1, inclusive, specifying the width of the i-th tile column in units of CTBs, is derived as follows:

if( uniform_tile_spacing_flag )
	for( i = 0; i  <=  num_tile_columns_minus1; i++ )
		ColWidth[ i ] = ( ( i + 1 ) * PicWidthInCtbsY ) / ( num_tile_columns_minus1 + 1 ) − 
						( i * PicWidthInCtbsY ) / ( num_tile_columns_minus1 + 1 )
else {
	ColWidth[ num_tile_columns_minus1 ] = PicWidthInCtbsY	(6-1)
	for( i = 0; i < num_tile_columns_minus1; i++ ) {
		ColWidth[ i ] = tile_column_width_minus1[ i ] + 1
		ColWidth[ num_tile_columns_minus1 ]  −=  ColWidth[ i ]
	}
}

The list RowHeight[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of CTBs, is derived as follows:

if( uniform_tile_spacing_flag )
	for( j = 0; j  <=  num_tile_rows_minus1; j++ )
		RowHeight[ j ] = ( ( j + 1 ) * PicHeightInCtbsY ) / ( num_tile_rows_minus1 + 1 ) − 
						( j * PicHeightInCtbsY ) / ( num_tile_rows_minus1 + 1 )
else {
	RowHeight[ num_tile_rows_minus1 ] = PicHeightInCtbsY	(6-2)
	for( j = 0; j < num_tile_rows_minus1; j++ ) {
		RowHeight[ j ] = tile_row_height_minus1[ j ] + 1
		RowHeight[ num_tile_rows_minus1 ]  −=  RowHeight[ j ]
	}
}

The list ColBd[ i ] for i ranging from 0 to num_tile_columns_minus1 + 1, inclusive, specifying the location of the i-th tile column boundary in units of CTBs, is derived as follows:

for( ColBd[ 0 ] = 0, i = 0; i  <=  num_tile_columns_minus1; i++ )
		ColBd[ i + 1 ] = ColBd[ i ] + ColWidth[ i ]		(6-3)

The list RowBd[ j ] for j ranging from 0 to num_tile_rows_minus1 + 1, inclusive, specifying the location of the j-th tile row boundary in units of CTBs, is derived as follows:

for( RowBd[ 0 ] = 0, j = 0; j  <=  num_tile_rows_minus1; j++ )
	RowBd[ j + 1 ] = RowBd[ j ] + RowHeight[ j ]		(6-4)

The list CtbAddrRsToTs[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in CTB raster scan of a picture to a CTB address in tile scan, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ ) {
	tbX = ctbAddrRs % PicWidthInCtbsY
	tbY = ctbAddrRs / PicWidthInCtbsY
	for( i = 0; i  <=  num_tile_columns_minus1; i++ )
		if( tbX  >=  ColBd[ i ] )
			tileX = i
	for( j = 0; j  <=  num_tile_rows_minus1; j++ )		(6-5)
		if( tbY  >=  RowBd[ j ] )
			tileY = j
	CtbAddrRsToTs[ ctbAddrRs ] = 0
	for( i = 0; i < tileX; i++ )
		CtbAddrRsToTs[ ctbAddrRs ]  +=  RowHeight[ tileY ] * ColWidth[ i ]
	for( j = 0; j < tileY; j++ )
		CtbAddrRsToTs[ ctbAddrRs ]  +=  PicWidthInCtbsY * RowHeight[ j ]
	CtbAddrRsToTs[ ctbAddrRs ]  +=  ( tbY − RowBd[ tileY ] ) * ColWidth[ tileX ] + tbX − ColBd[ tileX ]
}

The list CtbAddrTsToRs[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a CTB address in CTB raster scan of a picture, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ )	(6-6)
	CtbAddrTsToRs[ CtbAddrRsToTs[ ctbAddrRs ] ] = ctbAddrRs

The list TileId[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile ID, is derived as follows:

for( j = 0, tileIdx = 0; j  <=  num_tile_rows_minus1; j++ )
	for( i = 0; i  <=  num_tile_columns_minus1; i++, tileIdx++ )
		for( y = RowBd[ j ]; y < RowBd[ j + 1 ]; y++ )		(6-7)
			for( x = ColBd[ i ]; x < ColBd[ i + 1 ]; x++ ) {
				TileId[ CtbAddrRsToTs[ y * PicWidthInCtbsY+ x ] ] = tileIdx
		}

The list NumCtusInTile[ tileIdx ] for tileIdx ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a tile index to the number of CTUs in the tile, is derived as follows:

for( j = 0, tileIdx = 0; j  <=  num_tile_rows_minus1; j++ )
	for( i = 0; i  <=  num_tile_columns_minus1; i++, tileIdx++ ) 	(6-8)
		NumCtusInTile[ tileIdx ] = ColWidth[ i ] * RowHeight[ j ]

The set TileIdToIdx[ tileId ] for a set of NumTilesInPic tileId values specifying the conversion from a tile ID to a tile index and the list FirstCtbAddrTs[ tileIdx ] for tileIdx ranging from 0 to NumTilesInPic − 1, inclusive, specifying the conversion from a tile ID to the CTB address in tile scan of the first CTB in the tile are derived as follows:

for( ctbAddrTs = 0, tileIdx = 0, tileStartFlag = 1; ctbAddrTs < PicSizeInCtbsY; ctbAddrTs++ ) {
	if( tileStartFlag ) {
		TileIdToIdx[ TileId[ ctbAddrTs ] ] = tileIdx
		FirstCtbAddrTs[ tileIdx ] = ctbAddrTs		(6-9)
		tileStartFlag = 0
	}
	tileEndFlag = ctbAddrTs  = =  PicSizeInCtbsY − 1  | |  TileId[ ctbAddrTs + 1 ]  !=  TileId[ ctbAddrTs ]
	if( tileEndFlag ) {
		tileIdx++
		tileStartFlag = 1
	}
}

The values of ColumnWidthInLumaSamples[ i ], specifying the width of the i-th tile column in units of luma samples, are set equal to ColWidth[ i ]  <<  CtbLog2SizeY for i ranging from 0 to num_tile_columns_minus1, inclusive.

The values of RowHeightInLumaSamples[ j ], specifying the height of the j-th tile row in units of luma samples, are set equal to RowHeight[ j ]  <<  CtbLog2SizeY for j ranging from 0 to num_tile_rows_minus1, inclusive.

Tile Group Header and Tile group data signaling

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag ) |  |
| num_tiles_in_tile_group_minus1 | ue(v) |
| tile_group_type | ue(v) |
| if ( tile_group_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| NumTilesInCurrTileGroup = rect_tile_group_flag ? NumTilesInTileGroup[ tile_group_address] : ( num_tiles_in_tile_group_minus1 + 1) |  |
| if( NumTilesInCurrTileGroup > 1) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumTilesInCurrTileGroup − 1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

…

tile_group_address specifies the tile address of the first tile in the tile group.

If rect_tile_group_flag is equal to 0, the following applies:

- The tile address is the tile ID as specified by Equation 6-7.

- The length of tile_group_address is Ceil( Log2 ( NumTilesInPic ) ) bits.

- The value of tile_group_address shall be in the range of 0 to NumTilesInPic − 1, inclusive.

Otherwise (rect_tile_group_flag is equal to 1), the following applies:

- The tile address is the tile group ID of the tile group.

- The length of tile_group_address is signalled_tile_group_index_length_minus1 + 1 bits.

- If signalled_tile_group_id_flag is equal to 0, the value of tile_group_address shall be in the range of 0 to num_tile_groups_in_pic_minus1, inclusive. Otherwise, the value of tile_group_address shall be in the range of 0 to 2( signalled_tile_group_index_length_minus1 + 1 ) − 1, inclusive.

The value of tile_group_address shall not be equal to the value of tile_group_address of any other coded tile group NAL unit of the same coded picture.

When not present, the value of tile_group_address is inferred to be equal to 0.

The variable NumTilesInTileGroup, which specifies the number of tiles in the tile group, and TgTileIdx[ i ], which specifies the tile index of the i-th tile in the tile group, are derived as follows:

if( rect_tile_group_flag ) {
	tileGroupIdx = 0
	while( tile_group_address  !=  rect_tile_group_id[ tileGroupIdx ] )
		tileGroupIdx++
	
	
	NumTilesInCurrTileGroup = NumTilesInTileGroup[ tileGroupIdx ]
	tileIdx = top_left_tile_idx[ tileGroupIdx ]
	
	
	for( j = 0, tIdx = 0; j < (NumTileRowsInTileGroupMinus1[ tileGroupIdx ] + 1); j++, tileIdx  +=  num_tile_columns_minus1 + 1 ) {
		for( i = 0, currTileIdx = tileIdx; i < (NumTileColumnsInTileGroupMinus1[ tileGroupIdx ] + 1); i++, currTileIdx++, tIdx++ ) {
			TgTileIdx[ tIdx ] = currTileIdx
} } else {
	NumTilesInTileGroup = num_tiles_in_tile_group_minus1 + 1
	TgTileIdx[ 0 ] = tile_group_address
	for( i = 1; i < NumTilesInTileGroup; i++ )
		TgTileIdx[ i ] = TgTileIdx[ i − 1 ] + 1
}

...

offset_len_minus1 plus 1 specifies the length, in bits, of the entry_point_offset_minus1[ i ] syntax elements. The value of offset_len_minus1 shall be in the range of 0 to 31, inclusive.

entry_point_offset_minus1[ i ] plus 1 specifies the i-th entry point offset in bytes, and is represented by offset_len_minus1 plus 1 bits. The slice data that follow the slice header consists of NumTilesInCurrTileGroup subsets, with subset index values ranging from 0 to NumTilesInCurrTileGroup − 1, inclusive. The first byte of the slice data is considered byte 0. When present, emulation prevention bytes that appear in the slice data portion of the coded slice NAL unit are counted as part of the slice data for purposes of subset identification. Subset 0 consists of bytes 0 to entry_point_offset_minus1[ 0 ], inclusive, of the coded slice segment data, subset k, with k in the range of 1 to NumTilesInCurrTileGroup − 2, inclusive, consists of bytes firstByte[ k ] to lastByte[ k ], inclusive, of the coded slice data with firstByte[ k ] and lastByte[ k ] defined as:

(7-37)

lastByte[ k ] = firstByte[ k ] + entry_point_offset_minus1[ k ]	(7-38)

The last subset (with subset index equal to NumTilesInCurrTileGroup − 1 consists of the remaining bytes of the coded slice data.

Tile group data syntax

| tile_group_data( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < NumTilesInCurrTileGroup; i++ ) { |  |
| ctbAddrInTs = FirstCtbAddrTs[ tileIdx ] |  |
| for( j = 0; j < NumCtusInTile[ TgTileIdx[ i ] ]; j++, ctbAddrInTs++ ) { |  |
| CtbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| } |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < NumTilesInCurrTileGroup − 1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |

# Conclusion

This document proposes text for rectangular tile group mapping. It is proposed to adopt the proposed text in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., Huawei, Nokia, Interdigital, MediaTek, Samsung, Tencent, Ericsson, HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-L1001-v7, Versatile Video Coding (Draft 3), Macao, December 2018

[2] JVET-M0782-v1, Report of BoG on tiles and WPP, Marrakech, January 2019