| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0857-v2 |
| --- | --- |

| Title: | BoG report on intra prediction and mode coding (CE3-related) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera | Email: | geertv@qti.qualcomm.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG reviewed related input contributions to Core Experiment 3 on intra prediction and mode coding, and formulated recommendations for consideration by the track (A).

The CE3-related documents were categorized as follows:

- Cross-component prediction (14)

- Luma intra mode coding (8)

- Chroma intra mode coding (6)

- Interpolation of intra reference samples (4)

- PDPC-related (7)

- Various (7)

The BoG met on 11 Jan. 2019 from 9:00am–1:00pm, from 3pm–7pm, and on 12 Jan. 2019, from 9am–1:00pm, 3pm–6pm.

The BoG makes the following recommendations:

- Adoptions:

  - JVET-M0064: simplification of the current VTM3 division operation used in CCLM modelling; VTM3 includes 2×512 tables (16 bit elements), which is reduced to one table with 16 entries (4 bit elements), and in addition the bit depth of the model slope parameter (alpha) is reduced from 15 bits to 5 bits.

  - JVET-M0095 (editorial): third proposed aspect harmonizes the filtering decision for the reference samples for all directional intra modes (VVC draft Table 8-4 value for nTbS=2 entry modified from 20 to 16).

  - JVET-M0238: linear interpolation that PDPC uses on the secondary boundary for adjacent angular modes is simplified to nearest neighbour.

- CE tests:

  - Reducing the number of reference samples used to compute the cross-component linear model parameters: JVET-M0108, JVET-M0211, JVET-M0212, JVET-M0219, JVET-M0229, JVET-M0274

  - Multi-model LM (MMLM): JVET-M0384 (determine the block size restriction that is required to improve the worst case of MMLM and evaluate different formulas to determine the knee-point for deriving the two models)

  - Harmonization and simplification of MPM list construction: JVET-M0210, JVET-M0239, JVET-M0295 (method 1 + JVET-M0783 = JVET-M0815), JVET-M0494 (methods 2, 4, 5), JVET-M0528

    - Question: Does the scope of this test include MPM list construction of ISP, CIIP?

    - Remark: It is proposed to have one software for testing the unification proposals and separate the unification tests from intra search RD effects. The planar prioritization aspects, context changes, can be tested on top of the unification tests in additional tests.

  - Intra reference sample deblocking: JVET-M0138 (comparing the proposed method with strong intra smoothing method from HEVC in terms of complexity, artifact reduction)

- Open issues:

  - BoG recommends to present and discuss following documents related to small block size restrictions in AHG16 category: JVET-M0065, JVET-M0169

  - JVET-M0099: BoG recommends revisit after other proposals are presented in the luma mode coding category

  - BoG recommends revisiting the block size restriction for PDPC preferably when additional hardware/software experts are present (Track A or AHG16): JVET-M0045, JVET-M0122, JVET-M0238

  - JVET-M0358: BoG recommends testing as additional result (AI and RA conditions; 1 tile/CTU configuration) the ‘AND condition’ version of the reference sample availability conditions and report back during the current meeting.

# CE3-related proposals

## Cross-component prediction

JVET-M0044 Non-CE3: Alternative LM Chroma Implementation [S. Keating, K. Sharman (Sony)]

This document describes an alternative LM Chroma architecture where the alpha value (gradient) derivation is moved from the decoder to the encoder to reduce complexity and inter-dependencies in the encoder. The alpha value is then signalled in the bit-stream, with the beta value (intercept) derived by the decoder.  The implementation tested gives a BD-rate loss of 0.29%(Y), 0.33%(U), 0.83%(V) over VTM3.0 in AI.

It was commented that there are benefits, but the loss may not be acceptable and recommend to further study and reduce loss (outside CE). Linear regression was used in the encoder for estimating the alpha parameter. The proponent asserted that sending beta would not have further benefit.

JVET-M0064 Non-CE3: CCLM table reduction and bit range control [Y. Yasugi, F. Bossen, E. Sasaki (Sharp)]

This contribution proposes a method to reduce the CCLM bit range and table size.  It is asserted that the VTM3.0 CCLM has two problems: (i) that the current design requires the use of two, large tables and (ii) that the precision of the entries in these tables necessitates high bit-depth multiplication.  Specifically, it is observed that the VTM-3.0 CCLM employs two tables, each with 512 elements and where each element is 17 bits.  This table is designed to avoid a division operation during the derivation process.  However, when the internal bit depth of the decoded sample values is 10 bits, it is asserted that this design results in a 28 bit multiplication and 37 bit accumulation for the gradient a and intercept b, respectively, in the CCLM algorithm.  It is argued that it would be desirable to reduce the calculation bit-depth to fit within 16 bits, which would have a benefit of both reducing the complexity of the calculation as well as reducing the memory required for storing any needed tables.

In JVET-M0064 it is proposed to use a single table with 16 elements and with 4 bits per element.  Furthermore, experimental results show that the proposed change preserved almost same compression efficiency as the current VTM-3.0.

The following table summarizes the aspects of proposals related to the same topic:

|  | Table size | Table entry bitdepth | Alpha bitdepth | AI average BD-rate (Y/U/V) | RA average BD-rate (Y/U/V) |
| --- | --- | --- | --- | --- | --- |
| VTM3 | 2x512 | 16 bit | 15 bits |  |  |
| JVET-M0064 | 16 | 4 | 5 | 0.01%/0.02%/-0.05% (A1: 0.02%/0.03%/-0.22%) | 0.00%/0.10%/-0.05% (A1: 0.02%/0.21%/-0.20%) |
| JVET-M0108 | 8 | 16 | 15 | 0.02%/-0.04%/-0.02% (A1: 0.05%/-0.11%/0.11%) | 0.01%/0.00%/0.09% (A1: 0.05%/-0.05%/0.24%) |
| JVET-M0274 | 64 | 16 | 15 | 0.00%/-0.02%/-0.03% (A1: 0.01%/-0.21%/-0.05%) | / |
| JVET-M0356 | 48 | 8 | 6 | 0.01%/-0.08%/-0.04% (A1: 0.05%/-0.25%/0.1%) | 0.01%/-0.1%/-0.09% (A1: 0.07%/-0.28%/0.01%) |
| JVET-M0383 | 128 | 8 | 6 | 0.02%/-0.01%/-0.04% (A1:0.07%/-0.11%/-0.03%) | 0.01%/0.05%/0.10% (A1: 0.05%/-0.03%/0.28%) |
| JVET-M0493 | 64 | 16 | 15 | 0.01%/-0.08%/-0.06% (A1:0.03%/-0.20%/-0.01%) | 0.00%/0.00%/0.03% (A1:0.06%/-0.20%/0.03%) |

The BoG agrees that a simplification of the current VTM3 division operation in CCLM modelling is important and recommends adoption of JVET-M0064 since it significantly simplifies this aspect.

JVET-M0548 Crosscheck of JVET-M0064 (Non-CE3: CCLM table reduction and bit range control) [C.-M. Tsai (MediaTek)] [late] [miss]

JVET-M0093 Non-CE3: Improved robustness for calculation of cross-component linear model parameters [C. Helmrich (HHI)]

In JVET-L0191, a simplified parameter calculation for the cross-component linear model (CCLM) predictor in the Versatile Video Coding (VVC) standard was presented and, at the 12th JVET meeting in 2018, adopted into the draft specification.  According to this simplification, the CCLM parameters α and β can be derived by means of a straight-line fitting between a maximum and a minimum luma-chroma pair of sample values, as opposed to the previously used more complex linear regression from multiple luma and chroma samples. As a result, the algorithmic complexity of the CCLM tool is reduced without losing much coding efficiency.

This contribution presents evidence that the simplified line fitting between two extreme sample values is quite susceptible to statistical outliers and suggests two straightforward modifications to the CCLM method:

- fitting of the straight line between the average of the largest two luma and the average of the smallest two luma samples (instead of only the largest and only the smallest value), with accurate averaging,

- fitting of the straight line between the average of the largest two luma and the average of the smallest two luma samples, with inaccurate averaging (no offset of 1 before division) saving some additions.

It is claimed that both modifications have negligible impact on the VTM encoder and decoder complexity (encoder runtime 100%, decoder runtime 100–102%) and reportedly provide 0.3–0.4% coding efficiency gain in each of the chromatic channels (Cb and Cr BD-rate gains for both All-Intra and Random Access).  It is requested to study one of the proposed modifications – preferably, variant 2 – in a CE test after the upcoming JVET meeting, possibly as an alternative to the current CE 3 test 2.5 of JVET-L0342.

The BoG recommends no action, given that it is a model change and there are minimal gains reported.

JVET-M0108 CE3-related: Reducing the number of reference samples and table size in LM Chroma process [E. François, T. Poirier, F. Le Léannec (Technicolor)]

In this contribution two modifications of the LM chroma process are proposed. The first modification consists in reducing the number of reference samples used to compute the cross-component linear model parameters to up to 6. The second modification consists in reducing the size of the table required to perform the division for computing the linear model parameters from 2×512 to 64. The first modification is reported to result in PSNR-Y, U, V BD-rate variations of 0.0%, 0.1%, 0.1% in AI configuration, 0.0%, 0.1%, 0.1% in RA configuration, while reducing the number of checks to derive the cross-component linear model parameters. The second modification is reported to in PSNR-Y, U, V BD-rate variations of 0.0%, 0.0%, 0.0% in AI configuration, 0.0%, 0.0%, 0.1% in RA configuration, while reducing the memory size from 2048 bytes to 128 bytes. The combination of both modifications results in PSNR-Y, U, V BD-rate variations of 0.0%, 0.1%, 0.1% in AI configuration, 0.0%, 0.0%, 0.0% in RA configuration.

See discussion under JVET-M0064 for division table aspect.

The proponent clarified that there may be some slight loss in class A when the number of samples for model estimation is reduced. It was remarked that in proposal JVET-M0211 up to 8 samples are chosen depending on the block size.

The following proposals are similarly addressing the aspect of reducing the number of samples used for CCLM model derivation:

JVET-M0108, JVET-M0211, JVET-M0212, JVET-M0219, JVET-M0274

The BoG recommends defining a CE experiment comparing the various aspects of these proposals and potentially define combinations that are promising for VVC simplification.

JVET-M0571 Crosscheck of JVET-M0108 (CE3-related: Reducing the number of reference samples and table size in LM Chroma process) [L. Zhang (Bytedance)] [late] [miss]

JVET-M0643 Crosscheck of JVET-M0108 (CE3-related: Reducing the number of reference samples and table size in LM Chroma process) [P. Hanhart (InterDigital)] [late] [miss]

JVET-M0146 Non-CE3: MDLM template downsampling [P. Hanhart, Y. He (InterDigital)]

Multi-direction linear model (MDLM) prediction was adopted in VTM-3.0 to further improve the coding efficiency of cross-component linear model prediction. MDLM is an alternative to cross-component linear model (CCLM), where either the left template or the above template is used to calculate the linear model parameters. MDLM uses extended templates, which includes non-adjacent reconstructed samples. However, if the chroma CU is not square, the number of samples used in linear model parameters estimation may be increased in MDLM compared to CCLM. This may impact the throughput on hardware implementations. In this contribution, it is proposed to downsample the MDLM templates to use the same number of samples to calculate the linear model parameters in MDLM as in CCLM when the chroma CU is not square. In this way, the number of operations to calculate the linear model parameters can be identical for CCLM and MDLM, independently of the CU aspect ratio. Overall results for all intra configuration show a slight gain of -0.04% and -0.06% for the U and V components, respectively. For RA and LB configurations, it has similar results as anchors.

Goal is to harmonize the number of samples for CCLM model (minmax) computation between CCLM, CCLM-A, CCLM-T. It was commented that is not as much a simplification as a harmonization and that there are proposals that further reduce model computation. BoG recommends no action.

JVET-M0733 Crosscheck of JVET-M0146 (Non-CE3: MDLM template downsampling) [C. Chevance (Technicolor)] [late]

JVET-M0211 CE3-related: Fixed Reference Samples Design for CCLM [J.-Y. Huo, X.-W. Li, J.-L. Wang, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

The current VVC includes a simplified CCLM method, named as the MaxMin method. This contribution further reduces the number of the reference samples for the MaxMin method. Given the block size, reference samples at fixed positions are used. The proposed method is used for CCLM mode. The results can get -0.05%, -0.35%, and -0.40% BD-rates on Y, Cb, and Cr components in All Intra configuration and -0.01%, -0.32%, and -0.30% BD-rates on Y, Cb, and Cr components in Random Access configuration compared to VTM3.0.

The proponent clarified that there is an offset needed for shifting the sampling grid, because then the samples are more related to the block. It was commented that for software implementations, there is no significant simplification expected. There may be a benefit for hardware implementations, though.

Also see comments under JVET-M0108.

JVET-M0716 Crosscheck of JVET-M0211 (CE3-related:Fixed Reference Samples Design for CCLM) [X. Ma (Huawei)] [late] [miss]

JVET-M0212 CE3-related: Improved reference samples range for MDLM [S. Wan (NPU), Q.-H.Ran, X.-W. Li, Y.-Z. Ma, J.-Y. Huo, F.-Z. Yang (Xidian Univ.), Y.-F. Yu, Y. Liu (OPPO)]

Multi-directional LM (MDLM) has been adopted into VVC at the previous JVET meeting. This contribution designs a method to reduce the reference samples range for MDLM parameter derivation. Specifically, we limit the reference samples range to the last three quarter of the top line or the left column. This method can get -0.02%, -0.11%, and -0.18% BD-rates on Y, Cb, and Cr components in All Intra configuration.

This proposal reduces samples for CCLM-A, CCLM-T by skipping first quarter samples. There are some gains reported, but they are rather minor. The proponent asserts that it is better not to use those samples and refers to the coding gain.

Also see comments under JVET-M0108.

JVET-M0717 Crosscheck of JVET-M0212 (CE3-related:Improved reference samples range for MDLM) [X. Ma (Huawei)] [late] [miss]

JVET-M0219 CE3-related: Reduced number of reference samples for CCLM parameter calculation [J. Choi, J. Heo, S. Yoo, L. Li, J. Choi, J. Lim, S. Kim (LGE)]

The CCLM (cross-component linear model) method in the current VTM3 requires complex process to derive the linear model parameter α and β based on the straight-line equation with reference pixel comparison step. Considering that the derivation process of the linear model parameters is also required in decoder side, it is necessary to reduce the required complexity. In this proposal, the number of reference sample pairs, which are used in the CCLM parameter determination is reduced to half based on pre-defined upper limit so that the worst pixel comparison complexity is reduced by half (i.e., from 4 samples to 2 samples).

Experimental results with 8 upper limit reference sample pairs show -0.01%, -0.13%, and -0.17% BD-rates on Y, Cb, and Cr components, respectively in All Intra configuration, and 0.00%, -0.08%, and -0.07% in Random Access configuration.

Maximum number of reference samples is limited to 8. CCLM, CCLM-A, CCLM-T sampling is changed in this proposal. The position change (sampling offset) is not applied. The worst case of 4x4 luma complexity, which is number of comparisons is halved.

Also see comments under JVET-M0108.

JVET-M0586 Crosscheck of JVET-M0219 (CE3-related: Reduced number of reference samples for CCLM parameter calculation) [K. Zhang (Bytedance)] [late] [miss]

JVET-M0229 CE3-related: Simplification of LM Mode [Y.-W. Chen, X. Wang (Kwai Inc.)]

In this contribution, a modification to the cross-component linear model (CCLM) deriving method is presented. To determine the α and β for the CCLM model, currently in VVC the minimum and maximum luma samples are searched among the down-sampled top and left neighbouring reconstructed luma samples. α and β are then determined by the linear model formed from the minimum/maximum down-sampled luma samples and the corresponding chroma samples. In this proposal, it is proposed to search the minimum and maximum luma samples among the top and left neighbouring reconstructed luma samples without the down-sampling process. After the minimum and maximum luma samples are identified, their corresponding down-sampled luma samples are then generated to derive the α and β. With the proposed modification, the number of the operations involved during the derivation of α and β are reduced by around 97% for a 32x32 chroma block. Simulation results reportedly show BD-rates on Y, Cb and Cr components are 0.03%, 0.14%, and 0.19% for AI configuration over VTM-3.0, with 100% encoding time and 98% decoding time.

Simplification of the number of filtering operations as opposed to reduction of the number of samples. This way the number of operations is significantly reduced. Applied to CCLM, CCLM-A, CCLM-T. There was a question about whether different chroma sampling types were tested. This is not the case. The BoG recommends to test the method in CE, particularly, whether this method can be combined with other downsampling methods that reduce the number of comparisons.

JVET-M0632 Crosscheck of JVET-M0229 (CE3-related: Simplification of LM Mode) [K. Abe (Panasonic)] [late] [miss]

JVET-M0274 CE3-related: Modified linear model derivation for CCLM modes [M. Wang, K. Zhang, L. Zhang, H. Liu, J. Xu, S. Wang (Bytedance), J. Li, S. Wang, W. Gao (Peking University)]

In VTM-3.0, the linear model for CCLM modes are derived by a max-min method. 128 neighbouring luma samples are down-sampled and 127 comparisons are performed on them to find the minimum and maximum values for a 64×64 chroma block. Besides, a lookup table with 2K bytes are stored on ROM to mimic the division operation. In this contribution, it is proposed that four neighbouring luma samples are down-sampled and three comparisons are performed on them for a 64×64 chroma block. And the lookup table size is reduced to be 128 bytes. Simulation results reportedly show 0.03% and 0.01% BD-rate loss under All Intra (AI) and Random Access (RA) configurations, respectively.

Similarly as in JVET-M0064 the table size for the division operator in the CCLM model is reduced. In this proposal there are 8 table values, however the bitdepth is not modified, which is further reduced in JVET-M0064.

This proposal also has an aspect that reduces the division table.

Also see comments under JVET-M0064 and JVET-M0108.

JVET-M0641 Crosscheck of JVET-M0274 (CE3-related: Modified linear model derivation for CCLM modes) [E. François (Technicolor)] [late] [miss]

JVET-M0356 CE3-related: simplified calculation for CCLM parameters derivation [A. Filippov, X. Ma, V. Rufitskiy, H. Yang, J. Chen (Huawei)]

This document proposes a further simplification of Cross-Component Linear Model (CCLM) originally proposed in [1]. This contribution consists of two parts: reduction of bit-depth of parameter α and size reduction of LUT that is used for integer division implementation. Low-bit LUT is removed completely, high-bit LUT precision is lowered up to 8 bits as well as several methods to reduce the number of its entries are considered. This proposal reportedly shows that a non-linear sampling of LUT (Test 1) provides the overall results of 0.01% (Y), -0.08% (U), -0.04% (V) for AI configuration and 0.01% (Y), -0.10% (U), -0.09% (V) for RA configuration. Regular LUT sampling for 32 entries is reported to provide the following results  (Test 3):  0.08% (Y), 0.34% (U), 0.41% (V) for AI configuration and 0.06% (Y), 0.46% (U), 0.57% (V) for RA configuration. Regular LUT sampling for 64 entries is reported to provide the following results (Test 2):  0.03% (Y), 0.09% (U), 0.11% (V) for AI configuration and 0.03% (Y), 0.16% (U), 0.24% (V) for RA configuration.

Proponent prefers table with 48 entries (test 1), however there is slight loss of 0.05% in class A1 for AI, 0.07% in class A1 for RA.

See discussion under JVET-M0064 for division table aspect.

JVET-M0679 Crosscheck of JVET-M0356 (CE3-related: On CCLM simplification) [F. Racapé (Technicolor)] [late] [miss]

JVET-M0723 Crosscheck of JVET-M0356 (CE3-related: simplified calculation for CCLM parameters derivation) [G. Laroche, P. Onno (Canon)] [late] [miss]

JVET-M0383 Non-CE3: Table size reduction and bit width limitation for CCLM implementation [P. Onno, C. Gisquet, G. Laroche, J. Taquet (Canon)]

This contribution proposes to modify the derivation of the linear model parameters for the CCLM mode.

In one aspect of the proposal, the two tables of 512 integers coded on 16 bits used for deriving the  and  parameters of the CCLM mode in the VTM 3.0 are reduced to two tables of 128 (or 256) entries where each integer value is coded on 8 bits. It is asserted that the results provided by the modifications are respectively 0.01%/0.00% (or 0.01/-0.01%) for the AI/RA configurations while reducing the storage of the table by 8 (or by 4).

In a second aspect of the proposal, the  parameter is coded on a maximum number of 6 bits in order to limit the bit width of the multiplication in the CCLM prediction loop. It is asserted that the results provided by this modification above the first aspect are respectively 0.02%/0.01% for the AI/RA configurations.

Table size 128, entries coded on 8 bits + representation of alpha on 6 bits is preferred by proponent. There is 0.07% luma BD-rate loss for class A1 (AI), 0.05% luma BD-rate loss for class A1 (RA).

See discussion under JVET-M0064 for division table aspect.

JVET-M0741 Crosscheck of JVET-M0383 (Non-CE3: Table size reduction and bit width limitation for CCLM implementation) [A. Filippov, V. Rufitskiy (Huawei)] [late] [miss]

JVET-M0384 Non-CE3: LM in the middle [C. Gisquet, G. Laroche, P. Onno, J. Taquet (Canon)]

This proposal presents a method to perform the multiple-model linear mode using straight lines. This goes through the definition of a knee point between the two lines, derived from the luma and chroma sample averages and extremum points. The method only operates on larger blocks to not affect the linear model worst case.

There was a question raised about how the knee-point chroma value is determined for clarification. It was also discussed how the block size restriction is determined to decide whether to apply CCLM or MMLM. The BoG recommends to define a CE test to determine the block size restriction that is required to improve the worst case of MMLM and evaluate different formulas to determine the knee-point for deriving the two models.

JVET-M0689 Crosscheck of JVET-M0384 (Non-CE3: LM in the middle) [J. Lainema (Nokia)] [late] [miss]

JVET-M0493 CE3-related: Simplified look-up table for CCLM mode [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

In VTM3.0, two look-up tables (LUT) are employed in CCLM mode to avoid division operations, and size of each LUT is 512. In this contribution, one LUT is removed and the size of the remaining LUT is reduced by 87.5% (from 512 to 64). It is reportedly shown that, the proposed method achieved 0.01% and 0.00% luma BD-rates with around 100% encoding and decoding time for All Intra (AI) and Random Access (RA) configurations, respectively.

Table size is reduced, but not bit depth. Proponent prefers table size 64. There is a slight loss of 0.03% for class A1 (AI), and slight loss of 0.06% for class A1 (RA).

See discussion under JVET-M0064 for division table aspect.

JVET-M0624 Crosscheck of JVET-M0493 (CE3-related: Simplified look-up table for CCLM mode) [Y.-W. Chen (Kwai Inc.)] [late] [miss]

JVET-M0799 Bit-width reduction of multiplier in CCLM derivation and prediction [K. Kawamura, S. Naito (KDDI)] [late]

No need to present, because it is covered in the division simplification discussion of proposals JVET-M0064, etc.

## Chroma intra mode coding

JVET-M0048 Non-CE3: Modified Chroma Derived Mode [F. Zeng, J. Lin, D. Jiang, C. Fang (Dahua)]

This contribution proposes a method to get the derived Mode (DM) by counting the intra prediction modes of the corresponding luma blocks. The simulation results show 0.03% coding gain in terms of average BD-rate of Cb in all intra configuration with the same encoder complexity.

Was not presented (presenter unavailable).

JVET-M0100 CE3-related: DM-dependent chroma intra prediction modes [G. Rath, F. Urban, F. Racapé (Technicolor)]

This contribution presents new default prediction modes in chroma intra prediction. The default modes are adaptive to the value of the direct mode (DM). If the DM is directional, two adjacent modes of the DM at offset ± 2 are chosen, instead of the vertical and horizontal modes. Otherwise, if the dual-tree mode is supported, it is proposed to check the intra prediction mode of the luma block at the top left for a directional mode. If the mode is directional, then it is included in the set of four prediction modes along with its two adjacent modes. Otherwise the original default modes are chosen. Simulation results reportedly show an average chroma BD-rate variation of -0.74%, resp. -0.49%, in All Intra (AI), resp. Random Access (RA) configuration, compared with VTM-3.0 in same configurations.

This proposal is similar to CE3.3 tests that were discussed and there is no need for presentation. No action.

JVET-M0439 Crosscheck of JVET-M0100 (CE3-related: DM-dependent chroma intra prediction modes) [H. Liu (Bytedance)] [late] [miss]

JVET-M0213 CE3-related: Chroma intra candidates modification based on directional DM [Y.-Z. Ma, J.-L. Wang, X.-W. Li, J.-Y. Huo, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

In current VVC, chroma intra prediction modes are assigned as DM, CCLM, DC, Planar, Vertical and Horizontal. This contribution modifies the chroma intra mode candidates according to the DM information. Specifically, if DM is a directional mode, Vertical and Horizontal candidates are replaced by two modes which are obtained by adding -4 or +4 to the DM value.

The simulation results show -0.03%, -0.31%, and -0.39% BD-rates on Y, Cb, and Cr components, respectively, compared to VTM3.0 in All Intra configuration. The proposed method has negligible impact on the VTM encoder and decoder complexity.

During the CE3.3 review in track A on Jan. 11th, it was decided to not allow changes to the chroma mode coding if the ‘primary’ modes (planar, DC, horizontal, vertical) are not available to the encoder to choose from. The present proposal similarly as in the related CE3.3 tests disallows this under certain conditions and, therefore, the BoG recommends to not take action.

JVET-M0718 Crosscheck of JVET-M0213 (CE3-related: Chroma intra candidates modification based on directional DM) [X. Ma (Huawei)] [late] [miss]

JVET-M0214 CE3-related: Uniform Chroma intra candidates modification based on DM [Y.-Z. Ma, X.-W. Li, J.-L. Wang, J.-Y. Huo, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

This contribution presents a uniform chroma intra candidates modification based on DM. Specifically, Vertical and Horizontal are replaced by two derived modes of DM, no matter DM is directional mode or not. The derived modes are obtained by adding -5 or +5 to DM mode.

The simulation results show -0.04%, -0.17%, and -0.26% BD-rates on Y, Cb, and Cr components, respectively, compared to VTM3.0 in All Intra configuration. The proposed method has negligible impact on the VTM encoder and decoder complexity. It has -0.02%, 0.03%, 0.03% BD-rates on Y, Cb, and Cr components, respectively, compared to the method based on directional DM proposed in JVET-M0213.

During the CE3.3 review in track A on Jan. 11th, it was decided to not allow changes to the chroma mode coding if the ‘primary’ modes (planar, DC, horizontal, vertical) are not available to the encoder to choose from. The present proposal similarly as in the related CE3.3 tests disallows this under certain conditions and, therefore, the BoG recommends to not take action.

JVET-M0258 CE3-related: Chroma intra candidates modification based on non-directional DM [J.-Y. Huo, J.-L. Wang, X.-W. Li, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

This contribution modifies chroma intra mode candidates when DM is non-directional. Specifically, if DM is Planar or DC, directional modes (mode 6,40,61) are used as the chroma intra mode.

During the CE3.3 review in track A on Jan. 11th, it was decided to not allow changes to the chroma mode coding if the ‘primary’ modes (planar, DC, horizontal, vertical) are not available to the encoder to choose from. The present proposal similarly as in the related CE3.3 tests disallows this under certain conditions and, therefore, the BoG recommends to not take action.

JVET-M0324 CE3-related: Modified Chroma Intra Mode Coding [J. Park, B. Jeon (SKKU)]

This contribution proposes a modified coding method for chroma intra prediction modes. While the current version of VVC differentiates the four chroma intra modes of planar, DC, horizontal, vertical prediction modes using 2 bit fixed length codes, this contribution proposes to differentiate them using the unary max coding by modifying the binarization and association. With respect to VTM-3.0 as the anchor, the proposed method shows that an average BDBR of 0.01% (Y), -0.07% (Cb), and -0.08% (Cr) for AI; and 0.06% (Y), -0.20% (Cb), and -0.22%(Cr) for RA, respectively, with no run-time change.

It was commented that ‘proposal 1’ is interesting. In this aspect the DM index in the specification text table for chroma mode coding is redefined to have the same value when CCLM is enabled or disabled. However, there is no result for this modification. The BoG recommends to further study ‘proposal 1’ (outside CE).

## Interpolation of intra reference samples

JVET-M0095 Non-CE3: Intra simplifications [G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)]

Firstly, it is proposed to reduce the number of interpolation filter phases from 32 to 16 for interpolation of the neighbouring reference samples used in intra prediction. It is asserted that this change halves the number of coefficients to be stored for the 4-tap filters. The reported BD-rate increase is 0.02% and 0.00% for the AI and RA configurations, respectively, while runtimes are unaffected. Secondly, two VVC draft text harmonizations are proposed with reported BD-rate changes of 0.00% and −0.01% for the AI and RA configurations, respectively, while runtimes are unaffected.

1st aspect: reduces number of phases from 32 to 16. No action.

2nd aspect: change “ref sample smoothing + copy” vs “no smooth + filter” for modes 2, 32 and 66 in specification. Some concern expressed about needing multiple reference sample buffers, in particular for combination with PDPC.

It simplifies the text. Agreed.

No action.

3rd aspect: editorial change (bit exact). Changes one value in text, removes a couple lines and shortens a few more.

Recommendation (Ed): accept 3rd aspect

JVET-M0676 Crosscheck of JVET-M0095 (Non-CE3: Intra simplifications) [P. Merkle (HHI)] [late]

JVET-M0158 Non-CE3: LUT-free interpolation filters for intra prediction [A. Filippov, V. Rufitskiy, J. Chen (Huawei)]

This document proposes LUT-free interpolation filters for intra prediction. Replacing the Gaussian interpolation filter by an analytic representation reportedly shows the following overall results (Test 1): -0.00% (Y), -0.02% (U), -0.01% (V) and -0.01% (Y), 0.03% (U), 0.09% (V) for AI and RA configurations, respectively. Replacing both the Gaussian and DCT-IF interpolation filters by analytic representations reportedly shows the following overall results (Test 2): -0.01% (Y), -0.02% (U), -0.05% (V) and -0.01% (Y), 0.03% (U), 0.02% (V) for AI and RA configurations, respectively. Additional constraint on the number of phases for modified DCT-IF interpolation filtering reportedly results in the following coding performance (Test 3): 0.01% (Y), -0.03% (U), 0.01% (V) and -0.01% (Y), 0.05% (U), 0.01% (V) for AI and RA configurations, respectively. Results for modified Gaussian interpolation filter, unmodified DCT-IF restricted to 16 phases and 16-phases chroma linear interpolation filter are reported as follows (Test 4): 0.02% (Y), 0.03% (U), -0.01% (V) and 0.00% (Y), 0.00% (U), 0.01% (V) for AI and RA configurations, respectively. Results for modified Gaussian interpolation filter, modified DCT-IF restricted to 16 phases and 16-phases chroma linear interpolation filter are reported as follows (Test 5): 0.00% (Y), -0.02% (U), 0.03% (V) and -0.01% (Y), 0.00% (U), 0.03% (V) for AI and RA configurations, respectively. The running time is unchanged at both encoder and decoder sides for AI and RA configurations as compared with VTM-3.0.

It was commented that the current tables are not large and the BoG recommends no action at this time.

JVET-M0680 Crosscheck of JVET-M0158 (Non-CE3: LUT-free interpolation filters for intra prediction) [F. Racapé (Technicolor)] [late] [miss]

JVET-M0392 Non-CE3: Extended Mode-Dependent Intra Smoothing [A. Filippov, V. Rufitskiy, J. Chen (Huawei)]

This document proposes to introduce reference sample filtering for angular modes that currently use 4-tap interpolation filter to obtain predicted samples. It is proposed to introduce block size dependent sampling into reference sampling filter, and to replace Gaussian filter with a 2-tap filter that is currently used in chroma interpolation filtering. The following overall results are reported for Test 1 (the one with 4-tap Gaussian filter): -0.09% (Y), -0.07% (U), -0.03% for AI and -0.07% (Y), -0.10% (U), -0.04% for RA; and for Test 2 (the one with 2-tap linear filter): -0.04% (Y), -0.02% (U), -0.02% for AI and -0.04% (Y), -0.02% (U), 0.06% for RA. Noticeable coding performance is observed for Class A1 in both tests. The running time is negligibly changed at both encoder and decoder sides for AI and RA configurations as compared with VTM-3.0.

It was commented that in Macau meeting the current two 4-tap interpolation filters were adopted and that intra smoothing filter was integrated with the interpolation filter by deciding between them. In this proposal we are reverting back. The proponent argues that we are having larger gain for class A1.

The BoG recommends more study at this time (outside CE).

JVET-M0653 Non-CE3: Harmonization of integer-slope directional modes without interpolation filtering process [A. Filippov, V. Rufitskiy, J. Chen (Huawei)] [late] [miss]

This document proposes harmonization of integer-slope directional modes that do not require interpolation filtering process. The experimental results reportedly shows the following overall results: 0.00% (Y), 0.02% (U), -0.01% (V) and 0.00% (Y), -0.03% (U), 0.06% (V) for AI and RA configurations, respectively. The running time is negligibly changed at both encoder and decoder sides for AI and RA configurations as compared with VTM-3.0.

The proponent suggested that there is no need to present the proposal.

## Luma intra mode coding

JVET-M0099 Non-CE3: Partial sorting for non-MPM modes [A. K. Ramasubramonian, G. Van der Auwera, T. Hsieh, V. Seregin, M. Karczewicz (Qualcomm)]

Two modifications to the non-MPM mode derivation and signalling are proposed. Firstly, it is proposed to change the signalling of non-MPM modes from truncated binary to 6-bit fixed length. The second modification involves replacing the 6 MPM sorting algorithm currently used with a partial sorting algorithm. It is asserted that the proposed partial sorting method simplifies the non-MPM mode derivation by reducing the number of operations (conditions, comparisons, swaps, and increments/additions) as follows: from (11, 11, 6, 4) to (1, 1, 0, 1) in the worst case. The BD-rate for the proposed method is (0.04%, 0.01%, 0.02%) for (Y, Cb, Cr) with runtimes of 98% EncT and 97% DecT for the AI configuration and (0.00%, 0.09%, 0.02%) for (Y, Cb, Cr) with runtimes of 99% EncT and 98% DecT for the RA configuration. It is asserted that the loss observed is only attributable to the change of truncated binary to fixed length coding.

It was commented that there are sorting algorithms with lower complexity than described in this proposal and asserted that sorting can be done without comparisons. It was also asserted that no swapping is required.

One non-proponent commented that the proposed algorithm is a useful simplification.

The BoG recommends revisit after other proposals are presented in this category.

JVET-M0658 Crosscheck of JVET-M0099 (Non-CE3: Partial sorting for non-MPM modes) [K. Choi (Samsung)] [late] [miss]

JVET-M0139 Non-CE3: History-based intra most probable modes derivation [Z. Zhang, P. Wennersten, R. Yu, J. Ström, R. Sjöberg (Ericsson)]

This contribution presents a History-based Intra Most Probable Modes (MPM) derivation method for intra coding. A table is applied to store the intra prediction mode for decoded blocks. The table is applied for deriving the 6 MPMs. The table size is 6. When one block is decoded as intra predicted block, the table is updated following FIFO rule. Only angular intra prediction mode is stored in the table. Only unique mode is stored in the table. The table is initialized with 6 pre-defined angular intra prediction modes. The table is updated through entire slice. The luma BDR impact for AI/RA/LDB is -0.10%/-0.08%/-0.01% compared to VTM-3.0. Encoding time impact for AI/RA/LDB is 102%/101%/100% and decoding time impact for AI/RA/LDB is 100%/100%/100%, compared to VTM-3.0.

It was requested to compare the complexity of the proposed method with the anchor method. The proponent asserted that the number of comparisons is increased from 6 to 32. In addition, a buffer is required with size 6 (history) + 67 (to control that unique mode is allowed in list). The question was raised whether a default table was used to start each CTU to eliminate the CTU dependency. The proponent asserts that there is somewhat less gain in this case.

It was commented that it is hard to justify the complexity increase for the reported gain. The BoG recommends further study to improve performance/complexity trade-off (outside CE).

JVET-M0639 Crosscheck of JVET-M0139 (Non-CE3: History-based intra most probable modes derivation) [B. Wang (Huawei)] [late] [miss]

JVET-M0443 Crosscheck of JVET-M0139 (Non-CE3: History-based intra most probable modes derivation) [J. Li, L. Zhang (Bytedance)] [late] [miss]

JVET-M0191 CE3-related: Construction of non-MPM mode list in intra prediction [S. Cha, G. Lee, G. Kim, J. Han (Sejong University)]

In this contribution, it is proposed to construct the non-MPM mode list for each coding block, where the order of modes in the non-MPM list is derived based on the angular modes which are the neighboring blocks of the current coding block. The number of modes in the non-MPM list depends on the angular modes of the neighboring blocks.

If MPM flag is false, the truncated binary code is used to signal the index of mode in the non-MPM list, where the shorter and longer codes are used to signal the modes in the top and bottom parts of the non-MPM list, respectively, when the number of modes in non-MPM list is less than 2n. In this contribution, the modes which are similar to those of the neighbouring blocks are placed in the top part of the non-MPM list.

When the proposed method1 is applied, simulation results reportedly show that the average -0.14% and -0.xx% BD-rate change is achieved on top of VTM3.0 for all intra (AI) and random access (RA) configuration under the common test condition. The increments of encoding and decoding complexity are 5% and 2% for AI configuration, x.x% and x.x% for RA configuration.

When the proposed method2 is applied, the average -0.10% and -0.xx% BD-rate change is achieved on top of VTM3.0 for all intra (AI) and random access (RA) configuration. The increments of encoding and decoding complexity are 4% and 2% for AI configuration, x.x% and x.x% for RA configuration.

It was commented that class F results are missing, but the proponent has them available. The BoG recommends no action, given that the number of non-MPM candidates is reduced and restricts the encoder’s options (similar to CE3 test 3-3.2).

JVET-M0242 Crosscheck of JVET-M0191 (CE3-related: Construction of non-MPM mode list in intra prediction) [J. Lee, H. Lee, S.-C. Lim, J. Kang (ETRI)] [late]

JVET-M0210 Non-CE3: Intra prediction information coding [J. Yao, J. Zhu, W. Cai, K. Kazui (Fujitsu)]

This contribution proposes a new Intra prediction information coding method in which a syntax element,  intra_luma_non_ang_flag, is signaled at first for intra block to determine whether IntraMode is angular or non angular. For non_angular mode, intra_luma_planar_flag is signaled to indicate whether IntraMode is planar or DC. For angular mode, syntax elements intra_luma_ref_idx, intra_luma_mpm_flag, intra_luma_mpm_idx and intra_luma_mpm_remainder are signaled as in VTM-3.0. The mpm list is only applied for angular mode thus simplify MPM list derivation process. The experimental results reportedly show AI 0.3%, RA 0.12% bitrate savings with AI 109%/100%, RA 106%/100% encoding/decoding time over VTM-3.0.

The BoG recommends a CE test together with the other unification proposals.

JVET-M0758 Crosscheck of JVET-M0210 (Non-CE3: Intra prediction information coding) [C.-C. Kuo, C.-H. Yau, C.-C. Lin (ITRI)] [late] [miss]

JVET-M0239 Non-CE3: Modification of MPM derivation [J. Lee, H. Lee, S.-C. Lim, J. Kang, H. Y. Kim (ETRI)]

This contribution presents a modification method of most probable mode (MPM) derivation when luma reference line index is not equal to zero. It reduces number of condition checks to derive 6 MPM candidates. It is reported that the average luma BD-rate for AI and RA case is 0.05% and 0.02% compared to VTM 3.0, respectively.

It was commented that small changes to one of the branches of the MPM list construction for MRLidx>=0 is what we want to test in CE.

It was also commented that the current MPM list construction in VTM3 for MRLidx>0 has 6MPM and that this was not proposed at the last meeting (3MPM was proposed, because VTM2 had 3MPM and DC, PL were removed to be consistent). It is requested to ask the opinion of the Track A chair whether this change is acceptable. It was commented that we need to focus on further unification, not only intra, but all locations where MPM list are used, including inter.

BoG recommends defining a CE test for this proposed implication in case of MRLidx>0 and potentially study combinations with other tests on this subject, because unification and simplification are two aspects of the CE test.

JVET-M0609 Crosscheck of JVET-M0239 (Non-CE3: Modification of MPM derivation) [B. Wang (Huawei)] [late] [miss]

JVET-M0295 CE3-related: Harmonization of MPM list construction [B. Wang, S. Esenlik, A.M. Kotra, H. Gao, J. Chen (Huawei)]

This contribution proposes three methods to harmonize the two divergent MPM list construction in VTM3.0 when reference line index is not equal to zero. The first method uses four modes excluding Planar and DC. The second method uses two more extra modes: horizontal and vertical. The third method use a 6-MPM list by removing Planar and DC modes and add two more modes based on the intra modes of the left and above blocks.

Experimental results show that the three harmonization methods bring BD-rate difference of:

- 0.05% for All Intra and 0.03% for Random Access with the first method

- 0.05% for All Intra and 0.02% for Random Access with the second method

- 0.03% for All Intra and 0.00% for Random Access with the third method

For all proposed methods, no encoding and decoding time change is observed.

It was commented that changing the planar position in ‘method2’ has impact on intra search RD and that the gain can come from this. It was commented that there is a need for unification of the MPM lists (MRLidx) and support was given for studying in CE test.

It was remarked that JVET-M0494 (planar prioritization) is included in JVET-M0783.

The BoG recommends to test ‘method 1’ + JVET-M0783 in CE, which is equivalent to proposal JVET-M0815.

JVET-M0494 CE3-related: Modifications on MPM list generation [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

In this contribution, modifications on MPM list generation are proposed. Firstly, for non-zero reference lines, three branches for MPM derivation are removed from working draft when the left and above neighboring modes are different angular modes. Secondly, for zero reference line, if one neighboring mode is Planar or DC mode, and the other neighboring mode is angular mode, it is proposed to firstly insert Planar mode into the MPM list. Thirdly, on top of the second test, it is proposed that, Planar and DC modes are always put as the first two MPM candidates, and zero line and non-zero lines share the same MPM list generation process which is used for zero-line in VTM-3.0. Planar and DC modes in zero reference line MPM list are excluded for the non-zero reference lines MPM list, and all other angular modes in zero reference line MPM list are used for MPM list of non-zero reference lines with the same order. It is reportedly that, the proposed method achieved -0.05% and -0.01% luma BD-rates with around 100% encoding and decoding time for All Intra (AI) and Random Access (RA) configurations, respectively.

The proponent asserts that the focus is on unification among all the proposed aspects. The BoG recommends to study ‘method 4’ in the CE test on MPM list unification (MRLidx). Regarding the prioritization change of planar mode in the MPM list, there is disagreement whether the coding gain comes from MPM list change or intra search RD. It is proposed to have one software for testing the unification proposals and separate the unification tests from intra search RD effects (there has to be discussion on what option to use here, for example, full search?), while testing the planar prioritization in second phase (‘method 2’ in this proposal, also see JVET-M0783). In addition, ‘method 5’ can be studied in CE, however, there is disagreement on whether the context change is to be studied. It is suggested to define an additional test in case of context changes.

JVET-M0745 Crosscheck of JVET-M0494 (CE3-related: Modifications on MPM list generation) [J. Luo (InterDigital)] [late] [miss]

JVET-M0528 Non-CE3: A unified luma intra mode list construction process [F. Bossen, K. Misra (Sharp Labs of America)]

In the current VVC WD there exists three different luma intra mode list construction process: one for zero reference line index, a second for non-zero reference line index and a third for multi-hypothesis intra. This contribution proposes the use of a single list construction process. The contribution achieves this by first signalling if a luma intra mode is angular, and for angular mode constructing lists comprising only of angular modes (which is the same as currently being used for non-zero reference line index). For non-angular luma intra mode (i.e. PLANAR, DC) an additional flag is signalled that indicates the selected mode. It is asserted that the proposed modification simplifies the luma intra mode list construction process and on average has no significant coding efficiency impact. The proposed change is implemented in VTM-3.0 and evaluated under common test conditions. Note, that the unified list construction is used only for entropy coding and the encoder mode search is the same as in VTM-3.0. The proposed modification reportedly provides an average luma BD rate performance of -0.07% (all-intra), -0.03% (random-access), 0.05% (low delay B). It is also reported that for class A1 sequences in all-intra configuration the proposed scheme provides an average BD rate of -0.29%.

This proposal is related to JVET-M0210.

The question was raised about what elements are context coded. The proponent asserted there are approx. 7 contexts. There is some context dependency introduced, for example, 4 neighbouring blocks for intra_luma_nonangular_flag.

The aspect in the proposal on multi-hypothesis may need coordination with the CE on multi-hypothesis (intra-inter).

The BoG recommends including the proposed method in the CE test on unification of MPM lists.

Suggested to discuss the scope of the CE test on unification with the Track A chair, since adoption of ISP also includes MPM aspect that may require unification.

JVET-M0626 Crosscheck of JVET-M0528 (Non-CE3: A unified luma intra mode list construction process) [J. Yao (Fujitsu)] [late] [miss]

JVET-M0783 CE3-related: Modification of MPM list order [J. Heo, J. Choi, J. Choi, S. Yoo, L. Li, J. Lim, S. Kim (LGE)] [late]

It was commented that there is some loss (0.30% AI, 0.25% RA) on class F and questioned whether we would like to have this.

Also see comments under JVET-M0494, where ‘method 2’ is also included in the current proposal.

JVET-M0815 CE3-related: Harmonization on MPM list [L Li, J Heo, J Choi, S Yoo, J Choi, J Lim, S.Kim (LGE)] [late] [miss]

= ‘method 1’ (extraction of 4MPM from 6MPM for MRLidx>0) from JVET-M0295 + JVET-M0783

It was commented that JVET-M0815 is missing results. Please also see comments under JVET-M0295.

## PDPC-related

JVET-M0045 Non-CE3: PDPC Restriction [S. Keating, K. Sharman (Sony)]

Switch off PDPC for 4x4 blocks or smaller (0.05% AI (class E: 0.09%); 0.03% RA; slight gain on class F -0.17% AI). The proponent commented that reported runtimes are not very accurate.

BoG recommends revisiting the block size restriction for PDPC preferably when additional hardware/software expertise is present (Track A or AHG16).

JVET-M0561 Crosscheck of JVET-M0045 (Non-CE3: PDPC Restriction) [J. Lee, H. Lee, S.-C. Lim, J. Kang (ETRI)] [late]

JVET-M0047 Non-CE3: Intra Angular Prediction and Modified PDPC Based on Two Reference Lines [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

Was not presented (presenter unavailable).

JVET-M0122 Non-CE3: On block size restrictions for PDPC [A. Filippov, V. Rufitskiy, J. Chen (Huawei)]

This document proposes block size restrictions for PDPC. In Tests 1-3, PDPC is disabled for small and large blocks that reportedly shows the following overall results (Test 3): 0.01% (Y), -0.03% (U), 0.00% (V) and 0.02% (Y), 0.07% (U), 0.04% (V) for AI and RA configurations, respectively. In Tests 4, a mode-dependent mechanism for calculating PDPC scales is introduced to compensate for coding efficiency loss caused by disabling PDPC that reportedly demonstrates the following overall results: -0.05% (Y), -0.05% (U), -0.08% (V) and -0.03% (Y), -0.01% (U), 0.11% (V) for AI and RA configurations, respectively. A combination of Test 3 and Test 4 reportedly shows the following overall results: -0.05% (Y), -0.06% (U), -0.12% (V) and -0.02% (Y), -0.03% (U), -0.06% (V) for AI and RA configurations, respectively. For all the proposed tests, the running time is changed negligibly at both encoder and decoder sides for AI and RA configurations as compared with VTM-3.0.

BoG recommends revisiting the block size restriction for PDPC preferably when additional hardware/software expertise is present (Track A or AHG16).

BoG recommends further studying the proposed mode-dependent PDPC scales (MDS) outside of CE.

JVET-M0149 Non-CE3: On simplification of PDPC basic equation [A. Filippov, V. Rufitskiy, J. Chen (Huawei)]

This document proposes a simplification of PDPC basic equation. In Test 1, a top-left reference sample is excluded from PDPC basic equation that reportedly shows the following overall results: 0.01% (Y), -0.01% (U), 0.02% (V) and 0.01% (Y), 0.05% (U), 0.06% (V) for AI and RA configurations, respectively. In Test 2, a clipping operation is removed from PDPC basic equation due to minor modification of PDPC for horizontal and vertical modes that reportedly shows the following overall results: 0.01% (Y), -0.02% (U), -0.01% (V) and 0.00% (Y), 0.09% (U), 0.07% (V) for AI and RA configurations, respectively. The running time is changed negligibly at both encoder and decoder sides for AI and RA configurations as compared with VTM-3.0.

The proponent reports that there is no significant loss (0.01% RA) when top-left sample is removed from DC-mode PDPC. In addition, the clipping operation can be removed for DC-mode PDPC. A potential mismatch has been determined in the present version of the VVC spec text, where the top-left weight for DC, horizontal, vertical mode does not correspond with the software. BoG recommends that this mismatch is fixed. BoG also recommends no action on this proposal.

JVET-M0677 Crosscheck of JVET-M0149 (Non-CE3: simplification of PDPC basic equation) [F. Racapé (Technicolor)] [late] [miss]

JVET-M0238 Non-CE3: Modification of PDPC [J. Lee, H. Lee, S.-C. Lim, J. Kang, H. Y. Kim (ETRI)]

This contribution presents modification of position dependent intra prediction combination (PDPC). It includes two modifications which are large block size restriction and removal of interpolation. It is reported that the average luma BD-rate of the block size restriction and the removal of interpolation for AI case is -0.01% and 0.01% compared to VTM 3.0, respectively.

The proposal asserts that PDPC does not give much benefit for large blocks (64xN, Nx64).

BoG recommends revisiting the block size restriction for PDPC preferably when additional hardware/software expertise is present (Track A or AHG16).

In a second aspect the linear interpolation that PDPC uses on the secondary boundary for adjacent angular modes is proposed to be simplified to nearest neighbour. There is BD-rate difference of 0.01% for RA (class A1: 0.03%). BoG recommends adopting the nearest neighbour simplification.

JVET-M0678 Crosscheck of JVET-M0238 (Non-CE3: Modification of PDPC) [S. Keating (Sony)] [late]

JVET-M0358 CE3-related: disabling PDPC based on availability of reference samples [V. Drugeon (Panasonic)]

This contribution proposes to disable PDPC for certain intra blocks based on the availability of reference samples around the block. The results show tiny gains in both complexity and coding efficiency. Although the benefit is very small, the author believes that this is a logical modification of the PDPC process that is more beneficial for low resolution sequences and test conditions that differ from the CTC.

It is commented that the subjective impact is on the tile boundary, not so much on the picture boundary. It is remarked that it is not really a complexity reduction. It is commented that for subjective verification, the same intra prediction mode is chosen. The BoG recommends testing as additional result (AI and RA conditions; 1 tile/CTU configuration) the ‘AND condition’ version of the reference sample availability conditions and report back during the current meeting.

JVET-M0629 Crosscheck of JVET-M0358 (CE3-related: disabling PDPC based on availability of reference samples) [C. Rosewarne (Canon)] [late] [miss]

JVET-M0365 Non-CE3: modified PDPC for horizontal and vertical modes [A. Filippov, V. Rufitskiy, J. Chen (Huawei)]

This document proposes a modification of PDPC mechanism for screen content coding. Simulation results for test 1 are reported to be as follows for CTC Overall case: 0.03% (Y), -0.01% (U), -0.02% (V) and 0.02% (Y), 0.05% (U), 0.07% (V) for AI and RA configurations, respectively. For TGM class under CTC conditions results a reported as follows: -2.85% (Y), -2.82% (U), -2.87% (V) and -1.20% (Y), -1.15% (U), -1.12% (V) for AI and RA configurations, respectively. Simulation results for test 2 are reported to be as follows for CTC Overall case: 0.03% (Y), -0.02% (U), -0.02% (V) and 0.01% (Y), 0.13% (U), 0.02% (V) for AI and RA configurations, respectively. For TGM class under CTC conditions results a reported as follows: -3.15% (Y), -2.98% (U), -3.01% (V) and -1.33% (Y), -1.28% (U), -1.26% (V) for AI and RA configurations, respectively. The running time is changed negligibly at both encoder and decoder sides for AI and RA configurations as compared with VTM-3.0.

It is commented that if BD-rate reduction is targeted for class F, the CPR tool is recommended to test against and report the gain. BoG recommends no action.

JVET-M0478 Non-CE3: PDPC extension [G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)]

It is proposed to extend PDPC to non-square block diagonal directions that were adopted into VTM3 for intra prediction. The reported BD-rate gain is 0.05% and 0.04% with runtimes (enc/dec) 100%/101% and 99%/99% for the AI and RA configurations, respectively.

BoG recommends no action.

JVET-M0682 Crosscheck of JVET-M0478 (Non-CE3: PDPC extension) [F. Racapé (Technicolor)] [late] [miss]

JVET-M0814 Non-CE3: block size restriction on PDPC [L Li, J Heo, J Choi, S Yoo, J Choi, J Lim, S.Kim (LGE)] [late] [miss]

It is suggested to disable PDPC for small blocks (not apply w*h<=16) and large blocks (up to 32x32 for luma; 16x16 for chroma).

BoG recommends revisiting the block size restriction for PDPC preferably when additional hardware/software expertise is present (Track A or AHG16).

JVET-M0832 Non-CE3: On block size restrictions for PDPC with disabled linear filtering for PDPC in the case of skew non-diagonal modes [A. Filippov, V. Rufitskiy, J. Chen (Huawei), J. Lee, J. Kang (ETRI)] [late]

JVET-M0832 is the combination of aspects from JVET-M0122 and JVET-M0238 regarding block size restriction. Test 3, test 5, test 7 JVET-M0122 and JVET-M0238.

## Various

JVET-M0065 Non-CE3: Intra chroma partitioning and prediction restriction [T. Zhou, T. Ikai (Sharp)]

This contribution proposes to restrict intra chroma block partitioning in dual tree and intra chroma prediction modes in small blocks such as 2x2, 2x4, 4x2. Specifically, in this proposal, QT and MTT partitioning for small chroma blocks are disallowed in dual tree as a partition restriction. Also intra chroma prediction mode in small blocks is restricted where DC mode is always used without mode (intra_chroma_pred_mode) signalling. Two options with different threshold size are proposed.

Option1: 2x2 restriction

Option2: 2x2/2x4/4x2 restriction.

Option1 (2x2 restriction): the overall BD-rate loss is 0.00%/0.04%/0.03% in IO, 0.01%/0.17%/0.17% in RA, 0.03%/0.27%/ -0.11% in LB.

Option2 (2x2/4x2/2x4 restriction): the overall BD-rate loss is 0.03%/0.26%/0.38% in IO, 0.09%/0.63%/0.84% in RA, 0.00%/0.24%/ -0.43% in LB.

It was commented that only using DC mode in case of restriction is not a simplification, in addition PDPC is enabled. Also, AHG16 is looking into block size restrictions and there are proposals that disable certain block sizes. It is suggested to further study the single tree aspects of this proposal and also present together with AHG16 documents.

JVET-M0442 Crosscheck of JVET-M0065 (Non-CE3: Intra chroma partitioning and prediction restriction) [K. Zhang (Bytedance)] [late]

JVET-M0138 Non-CE3: Intra reference sample deblocking [Z. Zhang, K. Andersson, R. Sjöberg (Ericsson)]

Angular intra prediction modes can create annoying artifacts along the prediction direction for large blocks due to use of reference samples which are populated from a boundary between two large blocks. This contribution presents an Intra Reference Sample Deblocking (IRSDB) method for intra prediction. IRSDB is applied to the luma plane of large blocks that have both a block width and a block height larger than or equal to 32 luma samples. IRSDB is only applied to unfiltered reference samples for blocks that are predicted by an angular intra prediction mode. IRSDB is designed to symmetrically deblock 7 unfiltered reference samples on respective side of a block boundary to eliminate the discontinuity between reference samples which are populated from two neighboring large blocks. IRSDB is asserted to significantly reduce artifacts from angular intra prediction. The luma BDR impact for AI/RA/LDB is -0.02%/-0.02%/0.01% compared to VTM-3.0. Encoding time impact for AI/RA/LDB is 100%/100%/100% and decoding time impact for AI/RA/LDB is 100%/99%/99%, compared to VTM-3.0.

It was commented that it would be helpful to show some sequences with artifacts for expert viewing. 7 samples are modified on both sides of the edge, which is located on 32x32 grid. The BoG recommends to define a CE test comparing the proposed method with strong intra smoothing method from HEVC.

JVET-M0550 Crosscheck of JVET-M0138 (Non-CE3: Intra reference sample deblocking) [C.-M. Tsai (MediaTek)] [late] [miss]

JVET-M0169 CE3-related: Shared reference samples for multiple chroma intra CBs [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution presents a method to enable parallel generation of multiple chroma intra coding blocks (CBs) within a parallel region of size larger than or equal to 16 chroma samples. In the proposed method, parallel regions are first determined, and all of the chroma blocks within the same parallel region use the reference samples that belong to neighbouring blocks of the parallel region and are at the top and left boundaries of the parallel region for chroma intra prediction. It is reported that the proposed method results in 0.04%/0.23%/0.31% and 0.07%/0.43%/0.54% Y/Cb/Cr BD-rates with 100% and 96% of encoding time and 102% and 100% of decoding time for VTM3.0-AI and VTM3.0-RA, respectively. It is asserted that the worst-case processing throughput for chroma intra prediction can be increased to 4X of that in VTM3.0.

The question was raised whether there is any issue with class F sequences. There is some loss for class F but not major. It was clarified that the reference samples are used from a distance to predict the units not neighbouring the border of the region.

Also, AHG16 is looking into block size restrictions and there are proposals that disable certain block sizes. It is suggested to also present together with AHG16 documents.

JVET-M0715 Crosscheck of JVET-M0169 (CE3-related: Shared reference samples for multiple chroma intra CBs) [X. Ma (Huawei)] [late] [miss]

JVET-M0391 CE3-related: Improvements on the Decoder-side Intra Mode Derivation [M. Abdoli, E. Mora, T. Guionnet, M. Raulet (ATEME)]

This contribution proposes two modifications to JVET-M0094 – the Decoder-side Intra Mode Derivation (DIMD) algorithms. The first proposal changes the RDO check of the DIMD mode for the second EMT pass, while, the second proposal changes the virtual intra mode of a DIMD block presented to its corresponding Chroma block for mode coding. In VTM3.0, the original DIMD algorithm brings -0.11% BD-R gain in AI configuration. In this document, we introduce three tests based on different combinations of the above two proposals: Test 1) applying only the first proposal brings: -0.15% BD-R gain, Test 2) applying only the second proposal: -0.12% BD-R gain and Test 3) applying both proposals: -0.15% BD-R gain.

BoG recommends further studying proposed aspects (outside CE).

JVET-M0651 Crosscheck of JVET-M0391 (CE3-related: Improvements on the Decoder-side Intra Mode Derivation) [F. Henry, G. Clare (Orange)] [late] [miss]

JVET-M0426 CE3-related: Improvement on the Intra Sub-Partitions Coding Mode [S. De-Luxán-Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

This contribution presents an improvement of the Intra Sub-Partitions (ISP) coding mode proposed in the CE3 Tests 1.1.1 and 1.1.2 (JVET-M0102). Particularly, an improvement for Test CE3-1.1.2 is proposed. The changes in the design create a more symmetric and harmonic structure of the sub-partitions. Results show a gain of 0,06% for the AI configuration and a gain of 0,01% in the case of RA (with respect to Test CE3-1.1.2). There is no impact on the run-times in both configurations and both encoder and decoder. On the other hand, class F shows higher gains: 0.41% for AI and 0.30% for RA.

Since CE3 test 3-1.1.1 was adopted, this additional result was not presented.

JVET-M0458 Non-CE3: Combined-Hypothesis Intra-Prediction [G. Kulupana, A. Seixas Dias, S. Blasi (BBC)]

This contribution presents a new intra-prediction technique for VTM 3.0. Two intra-prediction hypotheses are computed for a block, one obtained by means of Planar prediction and the other obtained using one of the Angular intra-predictions. The two hypotheses are averaged on a sample-by-sample basis to obtain a combined-hypothesis intra-prediction. Additional syntax elements are signalled to indicate whether a block is predicted using the new technique. In particular, when the intra-prediction mode for a given block is Angular, a flag is signalled in the bitstream to indicate that the block is predicted using combined-hypothesis intra-prediction. Average BD-rates of -0.16% (Y), -0.09% (U) and -0.09% (V) with particularly significant gains in Class A1 (-0.47% (Y), -0.28% (U) and -0.25% ) are reported for AI with 105% average encoding time and 100% decoding time with respect to VTM 3.0.

The question was raised whether PDPC is used. It was confirmed that PDPC is applied to the combined prediction and angular mode part of PDPC is used. In hardware there is increased complexity, but the proponent asserts that larger blocks are selected which reduce complexity, however, the worst-case is not changed. It was asked how the method works with MRL. This would be studied further. It was commented that there is parsing dependency of the mode flag on the intra mode. BoG recommends further study of these aspects (outside CE).

JVET-M0613 Crosscheck of JVET-M0458 (Non-CE3: Combined-Hypothesis Intra-Prediction) [S. De-Luxán-Hernández (HHI)] [late] [miss]

JVET-M0524 CE3/6-related: Unification of RQT-like transform partitioning for intra and inter blocks [H. Egilmez, V. Seregin, A. Said, M. Karczewicz (Qualcomm)]

This contribution presents two unified extensions of the transform unit (TU) partitioning method, tested in CE6.4.2 for inter blocks. The first unification is a trivial extension of the method in CE6.4.2, which is applied to both intra and inter predicted blocks. The second unified design is a more flexible variation of the same method allowing smaller TU partitions.

Over VTM-3.0, the first extension provides

- 0.19% AI, 0.45% RA and 0.65% LDB average luma coding gains;

- 0.19% AI, 0.74% RA and 1.00% LDB average luma coding gains when Inter MTS is also enabled.

The second extension achieves

- 0.29% AI, 0.56% RA and 0.71% LDB average luma coding gains;

0.29% AI, 0.83% RA and 1.11% LDB average luma coding gains when Inter MTS is also enabled.

Presented in CE6-related.