| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0877-v2 |
| --- | --- |

| Title: | BoG report on CE6 related contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xin Zhao | Tel: Email: | xinzzhao@tencent.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This BoG is mandated to review the CE6 related contributions of the 13th JVET meeting. Totally 25 contributions have been reviewed, which are categorized as follows,

- Complexity reduction (14 proposals)

- Transform skip (6 proposals)

- Transform selection (2 proposals)

- Others (5 proposals)

The BoG met on Sunday January 13rd from 15:35 to 21:24 and Monday 14th from 20:30 to 22:10.

The following BOG recommendations were made:

- Adoptions:

  - Context reduction (6 contexts reduced to 1 context) of MTS flag signaling (JVET-M0347) (Crosscheck?)

  - Bug-fix for mismatch between spec text and SW related to cbf signaling (JVET-M0361, bug-fix on spec text)

- CE tests:

  - Modified transform selection

    - Transform selection for Chroma components (JVET-M0480, JVET-M0501)

    - Transform selection to multi-hypothesis inter-intra mode (JVET-M0482)

    - Unification between MTS and Transform Skip (JVET-M0501)

- Open issues:

  - Complexity reduction of 32-length DST-7/DCT-8

    - With zero-out approach (JVET-M0297, JVET-M0046) (Crosscheck?)

      - BOG recommends revisiting in track A to discuss whether a CE study or Adoption can be considered for related contributions (JVET-M0046, JVET-M0297)

      - JVET-M0046:

        - Low QP configuration: peak loss is 0.85% (AI, ParkRunning3 Class A2)

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |  | Low Delay Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT |  | DecT | Y | U | V | EncT |  | DecT | Y |  | U | V | EncT |  | DecT |
| CTC |  | 0.12% | 0.05% | 0.02% | 98% |  | 102% | 0.04% | 0.05% | 0.08% | 99% |  | 99% | 0.02% |  | 0.08% | 0.00% | 100% |  | 101% |
| Inter MTS |  | 0.12% | 0.05% | 0.02% | 98% |  | 100% | 0.05% | 0.01% | 0.10% | 94% |  | 89% | 0.14% |  | 0.10% | -0.05% | 90% |  | 90% |
| Low QP |  | 0.07% | 0.11% | 0.11% | 102% |  | 115% | 0.04% | 0.14% | 0.14% | 103% |  | 107% | 0.01% |  | 0.06% | 0.06% | 90% |  | 93% |
| Low QP, Inter MTS |  | 0.07% | 0.11% | 0.11% | 102% |  | 115% | 0.10% | 0.26% | 0.24% | 93% |  | 97% | 0.06% |  | 0.24% | 0.21% | 95% |  | 96% |

      - JVET-M0297:

        - Test 1 is same with JVET-M0046.

        - Test 2 (including residual coding changes):

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  | Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U | V | EncT | DecT | Y |  | U | V | EncT | DecT | Y |  | U | V | EncT | DecT |
| CTC | 0.10% |  | 0.01% | 0.00% | 97% | 94% | 0.00% |  | 0.09% | 0.11% | 99% | 99% | 0.05% |  | 0.34% | 0.11% | 99% | 98% |
| Inter MTS |  |  |  |  |  |  | 0.02% |  | 0.01% | 0.04% | 96% | 99% | 0.12% |  | 0.15% | -0.30% | 96% | 99% |
| Low QP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Low QP, Inter MTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

      - Spec text to be confirmed by Editor (BB).

    - Using TAF scheme

      - BOG recommends revisiting in track A to collect more opinions from hardware expert and decide whether a CE study should be continued for JVET-M0538.

  - MTS simplifications with modified transform candidates and signaling

    - BOG recommends revisiting in track A to discuss whether a CE study is to be set up for related contributions (JVET-M0304, JVET-M0366, JVET-M0340, JVET-M0202, JVET-M0464).

    - BOG recommends extending transform skip up to 16x16 (or 32x32?) only for class F (and TGM test sequences).

  - MTS syntax modification

    - BOG recommends revisiting in track A to discuss further actions regarding the syntax changes of MTS signaling (JVET-M0464, JVET-M0201).

  - Software tool for computing transform throughput (JVET-M0540)

    - BOG recommends revisiting in track A to collect more opinions whether the proposed software tool can be used for evaluating the CE6 study.

# BoG Report

The following CE6 related proposals are categorized and reviewed in this BoG, see notes below each proposal.

## Complexity reduction

JVET-M0046 CE6-related: A study of primary transforms [M. Zhou, Y. Hu (Broadcom)]

A 32x32 DST7/DCT8 based transform unit (TU) is about 60% more computationally intensive than a 32x32 DCT2 based TU. To bring the implementation cost of 32-point DST7/DCT8 to a reasonable level, it is proposed to keep non-zero transform coefficients only in the top-left 16x16 region of a 32x32/32x16 or 16x32 DST7/DCT8 based TU. Experimental results reveal that in CTC (i.e. the MTS is enabled in intra-coded CUs only) the zero-out leads to an average coding loss of 0.12% in AI, 0.04% in RA, 0.02% in LD_B and -0.02% in LD_P, respectively. When the MTS is enabled in both the intra- and inter-coded CUs, the average coding efficiency loss by using the zero-out is 0.12% in AI, 0.05% in RA, 0.14% in LD_B and 0.12% in LD_P, respectively.

The following analysis on the operation counts/per sample of primary transform in VVC3 is provided. For 64x64/64x32/32x64/32x32 DCT2 based TUs and for 32x32/32x16/16x32 DST7/DCT8 based TUs, the numbers of multiplications per transform coefficient are summarized in Table 1.

Table 1. Number of multiplications per transform coefficient of different transforms sizes

The following five tests were carried out:

- Test 1: VTM3.0 plus enabling the MTS in both the intra- and inter-coded CUs (the 2nd anchor).

- Test 2: disabling 32-point DST7/DCT8 and enabling the MTS in intra-coded CUs only.

- Test 3: disabling 32-point DST7/DCT8 and enabling the MTS in both intra- and inter-coded CUs.

- Test 4: keeping the non-zero transform coefficients only in top-left 16x16 region of a 32x32/32x16/16x32 DST7/DCT8 based TU, and enabling the MTS in intra-coded CUs only.

- Test 5: keeping the non-zero transform coefficients only in top-left 16x16 region of a 32x32/32x16/16x32 DST7/DCT8 based TU, and enabling the MTS in both intra- and inter-coded CUs.

It was mentioned that the run-time is not provided. It was mentioned by the proponent of JVET-M0297 that the same test was performed, and AI: 97% (enc), 94% (dec), RA: 99% (enc), 99% (dec) run-time is achieved. It was that mentioned that there is another transform speedup proposal JVET-M0464 which also achieved run-time reduction.

It was commented that in the current VTM SW, for 64x64 DCT zero-out, i.e., residual coding part is not accommodated to the zero-out scheme. In the current VVC draft, the residual coding is aligned with the VTM3.0 SW.

It was commented by the proponent that, if the proposed solution is not used, we would need to double the number of multiplies to support the primary transform design of VVC.

JVET-M0465 Cross-check of JVET-M0046: CE6-related: A study of primary transforms [S. Bandyopadhyay (InterDigital)]

JVET-M0297 CE6-related: 32 point MTS based on skipping high frequency coefficients [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

In JVET-L0132, reduced 32-point MTS (RMTS32) based on skipping high frequency coefficients was proposed and tested in the previous CE. Considering that 32-point MTS requires 64 multiplications per pixel and it becomes worst computational complexity, RMTS32 is proposed to reduce computational complexity as well as memory usage with reasonable performance degradation. Specifically two sub-tests are shown as follows;

- Test 1: RMTS32 always produces 16 coefficients for every 32-length row or column

- Test 2: Test 1 + residual coding part is adjusted to accommodate the region of non-zero coefficients

For Test 1, luma BD-rate changes are 0.12% (AI), 0.03% (RA), and 0.02 % (LD). The encoding/decoding time changes are 97%/94% (AI), 99%/99% (RA), and 100%/100% (LD).

For Test 2, luma BD-rate changes are 0.10% (AI), 0.00% (RA), and 0.05 % (LD). The encoding/decoding time changes are 97%/94% (AI), 99%/99% (RA), and 99%/98% (LD).

Test 1 is identical to JVET-M0046. In Test 2, normative changes related to the residual coding is included. It was clarified that the proposed method in Test 2 changes the residual coding for both 64-length DCT2 and 32-length MTS. Test 2 is supported by two non-proponents. It is mentioned by proponent that 1K bytes can be saved by the proposed method.

Spec text of both JVET-M0046 and this proposed method are available and to be reviewed by Editor, to be confirmed offline.

It was questioned that whether Low QP results are provided, and it was mentioned that for AI configuration, 0.1% loss is observed, no results available for RA or LDB.

JVET-M0046 (low QP): AI: 0.07%, RA: 0.04% (inter MTS enabled: 0.01%), LDB: 0.01% (inter MTS enabled: 0.06%), however, low QP results are not crosschecked and it was mentioned there is a bug regarding the 64x64 DCT2 zero-out in current VTM-3.0. The peak loss for Low QP is 0.85% (AI, ParkRunning3, Class A2, for CTC the loss is 0.24%). With the Test 2 of JVET-M0297 which resolved the bug, the peak loss becomes 0.85% (AI, ParkRunning3).

It was mentioned that there are several CE7 related proposals JVET-M0257, JVET-M0250, JVET-M0251, which solve the same issue (residual coding of 64x64 DCT2 does not accommodate the zero-out).

To be revisited in track A (JVET-M0297, JVET-M0046).

JVET-M0801 Cross-check report of JVET-M0297 on 32 point MTS based on skipping high frequency coefficients (CE6-related) [K. Naser (Technicolor)] [late]

JVET-M0202 CE6-related: Simplification related to MTS with reduced modes [K. Choi, M. Park, M. W. Park, W. Choi (Samsung)]

This contribution presents a simplification related to MTS with reduced mode. Currently, MTS in VTM3.0 supports 4 modes with a combination of DST7 and DCT8 along with each direction. In order to reduce encoding complexity of using MTS, four testing results based on reduced modes of MTS with non-normative and normative changes are presented in this contribution. Testing results on the proposed method showed about 11% encoding time reduction with no coding loss in AI scenario, compared to VTM3.0.

- Test1: Non-normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}, {H: DCT8, V:DCT8}

- Test2:Non-normative change with 2modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}, {H: DCT8, V:DCT8}

- Test3:Normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}, {H:DCT8, V:DST7}

- Test4:Normative change with 3modes

  - {H:DST7, V:DST7}, {H:DST7, V:DCT8}

Table 1. Syntax design for Test3

|  | Horizontal Kernel | Vertical Kernel | Code |
| --- | --- | --- | --- |
| Mode 0 | DST7 | DST7 | 0 |
| Mode 1 | DST7 | DCT8 | 10 |
| Mode 3 | DCT8 | DST7 | 11 |

Table 2. Syntax design for Test4

|  | Horizontal Kernel | Vertical Kernel | Code |
| --- | --- | --- | --- |
| Mode 0 | DST7 | DST7 | 0 |
| Mode 1 | DST7 | DCT8 | 1 |

It was mentioned that at the Macao meeting, the similar topic has been discussed, encoder side changes are preferable than the normative changes for achieving the encoding time saving. It was mentioned that the encoder-side speedup on MTS is also proposed in JVET-M0464.

JVET-M0672 Crosscheck of JVET-M0202 (CE6-related: Simplification related to MTS with reduced modes) [X. Cao (Hikvision)] [late] [miss]

JVET-M0304 CE6-related: 2-mode MTS with shape adaptive transform selection [J. Lainema (Nokia)]

This document proposes to reduce MTS (Multiple Transform Selection) modes from 5 to 2 and apply the shape adaptivity aspect of CE6.3.1 on top of the two MTS modes. In addition, it is proposed to allow ternary and binary CU splitting for 64x64 CUs in intra slices, as already done in inter slices. The proposed configuration is reported to have a BD-rate impact of -0.39 %, -0.03 % and 0.07 % for AI, RA and LDB configurations, respectively. The encoder runtimes are reported as 94 %, 96 % and 97 % in those categories. It is further reported that the runtime of the longest AI encoding is going down to 71% in this configuration.

The 2-mode MTS proposed in this contribution is identical to the earlier proposals by e.g. LGE (JVET-L0292) and Ericsson (JVET-L0489) and uses only DCT2-DCT2 and DST7-DST7 pairs. mts_idx syntax element is removed as the tu_mts_flag is enough to select between the two modes.

The shape adaptivity aspect is identical to Test 3.1b of CE6 (JVET-M0303), which applies DST7 to the shorter side of the transform block when shape adaptivity is allowed to be performed. In this case it is proposed to be allowed if spatial intra prediction is used and the shorter side of the block is between 4 and 16 samples for luma and 4 and 8 samples for chroma.

The 2-mode MTS alone is reported to have a BD-rate impact of 0.27 %, 0.11 % and 0.10 % for AI, RA and LDB configurations, respectively. The encoder runtimes are reported as 64 %, 95 % and 98 % in those categories.

The 2-mode MTS together with the shape adaptive transform selection aspect is reported to have a BD-rate impact of 0.20 %, 0.10 % and 0.08 % for AI, RA and LDB configurations, respectively. The encoder runtimes are reported as 65 %, 95 % and 98 % in those categories.

The 2-mode MTS together with the shape adaptive transform selection aspect and allowing ternary CU splitting for 64x64 CUs in intra slices is reported to have a BD-rate impact of -0.08 %, 0.03 % and 0.03 % for AI, RA and LDB configurations, respectively. The encoder runtimes are reported as 77 %, 95 % and 97 % in those categories.

When MTS is turned off, the proposed shape adaptive transform selection approach (identical to the CE6 Test 3.1b) provides -1.61 %, -0.71 % and -0.14 % bitrate impact for AI, RA and LDB configurations, respectively.

Proposal 1: Adopt the 2-mode MTS as it appears to provide a good trade-off between complexity and coding efficiency.

Proposal 2: Increase the Intra slice maximum TT and BB split thresholds to 64. Alternatively, only the TT split size could be increased to 64.

Proposal 3: Adopt the shape adaptive transform selection tool as it appears to have a positive impact on coding efficiency when 2-mode MTS is enabled; and especially desirable effects when MTS is disabled.

It was mentioned that at the Macao meeting, the similar topic has been discussed, encoder side changes are more preferable than the normative changes for achieving the encoding time saving.

It was commented by several proponents that a CE study may be needed to study the modified MTS signaling. It was mentioned by a non-proponent that similar changes have been tested in previous CE in Macao meeting, and the discussion in the last meeting is not encouraging normative changes on MTS signaling.

Revisit in track A to collect more opinions whether a CE should be set up for related proposals, including:

JVET-M0304, JVET-M0366, JVET-M0340, JVET-M0202, JVET-M0464.

JVET-M0340 CE6-related: Simplification on MTS for intra residual coding [X. Cao, F. Chen, L.Wang (Hikvision)]

This contribution presents two simplified MTS methods for intra residual coding in VTM3.0. Three transform pairs are considered at most when the MTS_CU_FLAG is equal to 1. The first simplification was based on block shape, it is reported that the BD-rate are 0.07%, 0.02%, 0.06% for AI, RA, and LB respectively with 21%, 3% and 1% encoding time reduced. The second simplification was based on both block shape and intra mode, it is reported that the BD-Rate are 0.00%, -0.02%, 0.02% for AI, RA, and LB respectively with 21%, 3% and 1% encoding time reduced.

The additional gain of adding intra mode dependency is 0.07% gain for AI and 0.04% for RA, which does not clearly justify the normative changes. It was questioned whether DCT-8 is used, and it is confirmed by the proponent that DCT-8 is used.

JVET-M0654 Crosscheck of JVET-M0340 (CE6-related: Simplification on MTS for intra residual coding) [K. Choi (Samsung)] [late] [miss]

JVET-M0366 CE6 related: Transform Simplification [C. Hollman, D. Saffar, P. Wennersten, J. Ström (Ericsson)]

The Multiple Transform Selection (MTS) tool adopted into the VVC Test Model VTM evaluates DCT-2 in horizontal and vertical direction, and four combinations of DCT-8 and DST-7 transforms. Among all the combinations, DCT-8 in horizontal and vertical direction is used only about 2% of the time. This contribution proposes to exclude this combination from the MTS tool. As this removal comes with the expense of compression deficiency, the following changes are proposed. First, DCT-2 is employed instead of DCT-8 for block edges of size 32. Second, the coding of mts_tu_idx is removed and replaced by two new flags: mts_dst_flag, which signals the usage of DST-7 in both directions, and mts_tu_flag, which signals which of the other combinations of DCT-X (DCT-2 or DCT-8) and DST-7 is used.

Each of the new proposed flags mts_dst_flag and mts_tu_flag use one context each for intra and one context each for inter predicted blocks. For the mts_tu_flag, the value states the selection of a more probable or less probable combination on the decoder side. For intra predicted blocks, the selection is made based on the intra mode and for the inter predicted blocks, it is made based on the block size.

In this proposal, the coding of the mts_cu_flag has been revised. For intra predicted blocks, the context is selected based on the block size and the intra direction mode. On the other hand, for inter predicted blocks, the context is designated based on the block height and width.

It is reported that, compared to the VTM-3.0, this proposal achieves -0.03% and -0.01% luma BD-rates for AI and RA, respectively, while the encoding time decreases by 12% for AI and by 2% for RA. The decoder runtime decreases by 1% for AI and remains the same for RA. The contribution also states that the proposal leads to a simplification, since a 32-sample DCT-8 transform is no longer needed.

It was commented that the worst case of MTS (32-sample DST-7 in both directions) is unchanged. It was commented that there may be a parsing dependency for the context selection. It was commented that the proposed method enables transform combinations of (DST-7, DCT-2) and (DCT-2, DST-7), which are covered in the adopted shape-adaptive transform scheme.

JVET-M0085 CE6-related: Fast algorithm for DST-4/DCT-4 as alternative transforms for MTS [T. Toma, K. Abe (Panasonic), M. Ikeda, T. Tsukuba (Sony)]

A new set of MTS transforms, i.e. replacing DST-7 and DCT-8 by DST-4 and DCT-4, was proposed in JVET-L0262 [5] in Macao meeting. Since fast algorithm for DCT-2 is widely known, it was discussed in the Macao meeting if fast algorithm is also available for non-DCT-2 transform basis functions that are used in MTS. This contribution proposes fast algorithm for DST-4 and DCT-4. By utilizing symmetric property of matrix coefficient, the number of multiplication operations is reduced to 75 % as compared to matrix multiplication.

Also, following additional evaluations related to JVET-L0262 are conducted and these results are provided. It is generally observed that BD-rate loss of DST-4 / DCT-4 approach compared to DST-7 / DCT-8 gets smaller if the maximum MTS size is restricted.

- Combination with CE6-3.2b and CE6-3.2c: Restrict maximum MTS size

- Combination with CE6-4.1d: Sub-block Transform (SBT) for inter blocks

- Combination with JVET-M0046 [6]: Keep non-zero coefficients only in the top-left 16x16 region

In v2, simulation results of the additional results in section 4 are updated and summary of the additional results is added.

No need to present due to the current design of MTS.

JVET-M0800 Cross-check report of JVET-M0085 on Fast algorithm for DST-4/DCT-4 as alternative transforms for MTS (CE6-related) [K. Naser (Technicolor)] [late]

JVET-M0379 CE6-related: Further Simplification on top of CE6-2.2 [K. Naser, E. François, F. Le Léannec (Technicolor)]

In tests CE6-2.2, simplified implementation of DCT8/DST7 is proposed. The simplification is based on DFT decomposition of DCT8/DST7 transform for the sizes of 32x32 and 16x16 where matrix multiplications are used with reduced number of operations and overall memory.

This contribution studies further simplifications of CE6-2.2 by considering the following:

- Odd basis functions of DCT2 of large sizes are replaced by DCT8 with DFT based implementation

- DCT8/DST7 for small block sizes are replaced by DCT4/DST4 that can be inherited from DCT2

The overall impact is a further reduction of the overall memory and a reduced number of operations for computing DCT2 for large block sizes.

Further study in case CE6-2.2 is to be continued.

JVET-M0638 Crosscheck of JVET-M0379 (CE6-related: Further Simplification on top of tests CE6-2.2) [T. Toma, K. Abe (Panasonic)] [late] [miss]

JVET-M0398 CE6-related Further simplification of CE6-1.5 [P. Philippe (bcom Orange)]

This contribution studies the impact of retaining only the 16 lower coefficients for MTS when operated in size 32. The results indicate that additional simplification is revealed with respect to the CE6-1.5 and the coding performance impact is measured.

Further study if CE6-1.5 is to be continued.

JVET-M0396 CE6-related: MTS kernel derivation for efficient memory usage [S. Shrestha, A. Kumar, B. Lee (Chosun Univ), Y. Lee, J. Park (Humax)] [late] [miss]

Initial upload rejected as a placeholder.

In Multiple Transform Selection (MTS) of VTM3.0, DCT-2, DST-7 and DCT-8 have been used as transform kernels. In this contribution, we propose an MTS kernel derivation scheme to save memory consumption for kernel storage. In the proposed method, a complete matrix kernel of DST-7 can be derived with only a few numbers of rows of DST-7. The DCT-8 can also be derived from DST-7 with simple sign changing and flipping computations. For derivation of DCT-2, a sparse unified matrix from DST-3 is defined and it can be obtained using a sparse unified matrix and unit-element matrices consisting of -1, 0 and 1. The proposed method makes use of 1680 bytes for 8-bit precision for all kernels with different block size for MTS, which is significantly less than 4096 bytes in current VTM3.0.

The proposed method is supposed to be a lossless method, but significant coding loss is noticed, which may be caused by an incorrect implementation. No action.

JVET-M0773 Crosscheck of JVET-M0396 (“CE6-related: MTS kernel derivation for efficient memory usage”) [J. Jung, D. Kim, G. Ko, J. Son, J. Kwak (WILUS)] [late] [miss]

JVET-M0397 CE6-related: DST-3 based transform kernels derivation [S. Shrestha, A. Kumar, B. Lee (Chosun Univ.), Y. Lee, J. Park (Humax)]

In Multiple Transform Selection (MTS) in VTM3.0, three transform kernels types (DCT-2, DST-7 and DCT-8) are used. This contribution proposes an efficient and low complexity transform kernels derivation method from a 64×64 DST-3 kernel. In this proposed method, a 64x64 DST-3 integer kernel can be partitioned into 32×32 DST-4, 16×16 DST-4, 8×8 DST-4, 4×4 DST-4, 2×2 DST-4 and 2×2 DST-3 with multiplication of unit-element matrices consisting of 0, 1 and -1. Any block size of DCT-2, DST-4 and DCT-4 kernel matrices can be derived from the proposed method. The proposed method overcomes the drawback of memory consumption by storing only 1368 elements (1368 bytes for 8-bit precision) instead of total 4096 elements (4096 bytes for 8-bit precision). In this contribution, DST-4 and DCT-4 have been used instead of DST-7 and DCT-8, respectively.

Instead of DST-7 and DCT-8, it is proposed to use DST-4 and DCT-4 for all block sizes. Implementation for the transform core generation is identical to the method proposed in JVET-M0396. No action based on the CE6 discussion.

JVET-M0852 Crosscheck of JVET-M0397 (CE6-related: DST-3 based transform kernels derivation) [J. Kim, K. Ko, J. Jung] [late]

JVET-M0538 CE6: Efficient Implementations of MTS with Transform Adjustments (tests 1.4a-d) [A. Said, H.E. Egilmez, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm)] [late]

This had been included in the CE report but was a late contribution that was not in the CE plan.

One 8x8 eight-bit adjustment matrix is used for implementing the proposed method. It was questioned why there is a coding gain if the proposed method is targeting for simplification, no clear answer is given.

When TAF is only applied to 32-length MTS transform, there is a 0.14% gain in Class A1 and average is 0.00%, the peak coding gain is -0.20% and peak coding loss is 0.16%.

Only 32-point MTS is being proposed to be further studied in CE. It is claimed in the proposal that the worst-case multiplications per sample is 36, which is inconsistent with the numbers provided in JVET-M0046. It is confirmed that the worst-case multiplication per sample is 4 more than 32-length DCT2 in both dimensions because of the additional adjustment stage, number needs to be crosschecked. Method modifies the 8 lowest frequency coefficients using TAF and reuse the remaining DCT-2 coefficients. It is mentioned by the proponents that the worst-case in current VTM-3.0 primary transform is 64 multiplies per sample, which is same conclusion with JVET-M0046.

It was mentioned by a non-proponent that during the CE6 review, several hardware experts requested more offline studies on this proposal and provide feedback. Revisit in track A to collect the feedback if available.

JVET-M0539 CE6-related: Efficient computation of MTS transform combinations [A. Said, H.E. Egilmez, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm)] [late]

This contribution describes a encoder-only and lossless modification to the computation of MTS transforms using the adjustment method of JVET-M0538, to enable reducing its computational complexity. Instead of computing separately all possible combinations of the DST-7 and DCT-8 along horizontal and vertical directions, it computes the DCT-2 coefficients, and exploits the properties of the DCT-2 and the adjustment matrices to compute all transform combinations simultaneously, and with significant reduction in complexity. Experimental transform throughput measurements show how the proposed methods reduces complexity.

This is an informational proposal. Higher throughput (transform calculation per second) can be achieved by reusing the first forward DST-7 (or DCT-8) for computation of the second 1-D MTS transformation. It is also mentioned that, reusing the DCT-2 coefficients can also increase the throughput of the proposed method with lossless changes.

JVET-M0540 CE6-related: Software tool for computing transform throughput [A. Said, H.E. Egilmez, Y.H. Chao, V. Seregin, M. Karczewicz (Qualcomm)] [late]

Transform computational complexity is an important part of a codec’s implementation costs. It is becoming difficult to evaluate a transform-related proposal’s performance from only full encoder and decoder times, since their measurements are becoming less reliable in large clusters and parallel execution. Furthermore, current codecs include many high complexity tools that can mask very important differences. Standard software profilers do not help because they cannot differentiate the unique characteristics and measurement problems defined by separable transforms. This proposal shows that, due to the VTM code organization, it is easy to implement a profiler specifically tailored to measure transform throughputs (i.e., transform coefficients per second), provides code for that purpose, and discusses how to enable consistent results and cross-checking.

It is recommended to use one profiling method to measure the run-time similar to the proposed one for the purpose of CE6 study. It is also suggested to disable all compiler optimization when using the proposed profiling method if it is agreed to use the SW run-time measurement as part of CE6 test report.

It was also questioned about the impact of the overhead of activating the profiling.

It was mentioned by the proponent that CE5 studies are conducted using a profiling tool for measuring the throughput of different proposed CABAC engine.

To be revisited in track A for collecting more opinions, preferable from the SW coordinators.

## Transform skip

JVET-M0072 Non-CE6: On transform skip for larger block [T. Tsukuba, M. Ikeda, T. Suzuki (Sony)]

This contribution proposes to allow transform skip for larger transform size, and proposes to add signaling to indicate the maximum size of transform skip. Simulation results show that bd-rate changes are -0.03%, -0.05% and -0.07% up to 8x8 transform skip size for AI, RA and LB, respectively; -0.03%, -0.09% and -0.15% up to 16x16 transform skip size for AI, RA, and LB, respectively; -0.03%, -0.11% and -0.21% up to 32x32 transform skip size for AI, RA, and LB, respectively; -0.03%, -0.10% and -0.19% up to 64x64 transform skip size for AI, RA, and LB, respectively. Especially, in Class F, up to 16x16 transform skip size provides bd-rate reduction by -2.15%, -2.36% and -2.53 % with 124%, 113% and 111% Encoding time for AI, RA and LB, respectively. The proponents of this contribution believe that the maximum size of transform skip should be 16x16. As conclusion, this contribution proposes to add the syntax and semantics to signal the maximum size of transform skip in the WD, and proposes to study further the appropriate maximum transform skip size by CE or AHG.

In v2, results are updated.

In range extension, transform skip is supported up to 32x32. A SPS syntax which is same as HEVC, is added to control the largest transform skip block size. It was questioned whether 64x64 transform skip is implemented, it was answered by proponent that 64x64 transform skip was supported. It was questioned whether statistics (frequency of the usage of transform skip) have been collected regarding transform skip. Minor gain is observed for CTC. It was also suggested to use different transform skip configuration for different class of test sequences. It was proposed to use different transform skip configuration used for VTM-4.0.

JVET-M0748 Crosscheck of JVET-M0072, in aspect of 8x8 transform skip extension (Non-CE6: On transform skip for lager block) [S. Yoo, J.Lim (LGE)] [late]

JVET-M0637 Crosscheck of JVET-M0072 (Non-CE6: On transform skip for lager block) [T. Toma, K. Abe (Panasonic)] [late] [miss]

JVET-M0269 Non-CE6: Extension of transform skip block size to 8x8 [S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim (LGE)]

In this contribution, the extension of transform skip mode up to 8x8 transform size has been investigated. It is reportedly shown that Y BD-rate changes are -0.03% (AI), -0.05% (RA) and -0.07% (LDB) under the common test conditions. The BD-rate changes for Class F and SCC sequences (which are studied in CE8) are -1.88% and -6.81% in AI, -1.83% and -6.77% in RA, and -1.69% and -6.78% in LDB, respectively.

The same configuration has been tested and reported in JVET-M0072, which includes size 8, 16, 32 and 64 for transform skip. Transform skip size larger than 4 is suggested by the proponent.

JVET-M0709 Crosscheck of JVET-M0269 (Non-CE6: Extension of transform skip block size to 8x8) [T. Tsukuba (Sony)] [late] [miss]

JVET-M0275 Non-CE6: On transform skip conditions [S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim (LGE)]

In this document, it is proposed to use the number of pixels for transform skip enable condition instead of width and height of the current block so that transform skip mode can be more generally applied. Specifically, it shows benefits when transform skip enabled size is extended. The experimental result is reportedly shown that Y BD-rate changes in current CTC condition (i.e., equivalent to applying transform skip up to 16 pixels) are 0.00%, 0.00%, X.XX% in AI, RA, LDB, respectively. However, when the transform skip is allowed up to 64 pixels, it is reportedly shown that Y BD-rate changes are -0.01%, -0.02%, X.XX% in AI, RA, LDB, respectively. Note that if the transform skip is extended to larger block sizes (i.e., 8x8), more block types can be enabled using the proposed scheme such as 4x16 and 16x4 in luma component.

Different criterion of transform skip enabling condition is proposed, i.e., block area size instead of block height or width. It was reported that with -0.88% (AI), -0.91% (RA) with 2% to 3% enc-time increase comparing to the current VTM-3.0 with transform skip max size being set equal to 8x8, which is also suggested to be enabled for CTC sequences.

It was commented that the transform skip size may be coupled with MTS size in JVET-M0464. To be revisited and confirmed together with JVET-M0464. It was commented there is a clear preference from several related contributions that enlarging maximum transform skip size to be larger than 4x4.

JVET-M0846 Cross-check of JVET-M0275 (Non-CE6: On transform skip conditions) [P. Keydel (HHI)]

JVET-M0354 CE6-related: MTS with Haar transform for Screen Contents Coding [K. Naser, F. Galpin, T. Poirier (Technicolor)]

This contribution proposes to introduce Haar transform inside the multiple transform selection for the coding of the screen contents. It is motivated by the fact that the basis functions of Haar transform well matches the properties of the screen contents, where blocks with constant values and sharp edges frequently occurs. Specifically, the DCT8 transform is replaced by Haar transform. The proposal is tested with CTC for class F and TGM. For further analysis, CPR mode was also enabled to study the cross-effect between them. The experimental results show the average BD-rate saving as follows (AI%, RA%, and LDB%):

- Class F (0.46, 0.35, 0.32)

- Class TGM (1.34, 0.83, 0.26)

When CPR is enabled: (AI%, RA% and LDB%):

- Class F (0.25, 0.10, 0.02)

- Class TGM (0.69, 0.45, 0.23)

For natural sequences, it is reported that small loss is observed (~0.1%). It was questioned why Haar transform is helpful for screen content. DCT8 of all available sizes are replaced by Haar transform in the proposal. It was commented that Haar transform may be useful for textures with strong edges, e.g., screen content and depth maps. The proposal is related to other CE6 related proposals in this category, which is mainly targeting for screen content. Further study is encouraged.

JVET-M0869 Cross-check report of JVET-M0354 (CE6-related: MTS with Haar transform for Screen Contents Coding) [M. Koo (LGE)]

JVET-M0464 Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip [B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

This contribution proposes two modifications related to transform type signalling and residual coding for transform skip. The first modification includes aligning the cases in which transform skip (TS) and multiple transform selection (MTS) apply. This limits TS to luma transform blocks as well as extends it to transform block sizes up to 32x32. The second modification constitutes of a modified transform coefficient level coding for the TS residual. Relative to the regular residual coding case, the residual coding for TS includes no signalling of the last x/y position, coded_sub_block_flag coded for every subblock, sig_coeff_flag context modelling with reduced template, a single context model for abs_level_gt1_flag and par_level_flag, context modeling for the sign flag, additional greater than 5, 7, 9 flags, modified Rice parameter derivation for the remainder binarization and a limit for the number of context coded bins per sample.

This contribution was reviewed in track B (CE8 related), and further reviewed in CE6 BoG regarding the changes on transform signaling and related encoder changes.

It was questioned if results without transform skip changes are available, the proponent do not have the results available at the time of presentation. It was commented that the changes related to transform skip and MTS should be separated. It is commented that the proposed scheme needs to couple maximum transform skip size with maximum MTS size. It was commented that transform skip should be kept conceptually the same as HEVC when MTS is disabled, it was clarified by proponent that transform skip behaves the same when MTS is disabled.

It was questioned about the major benefit of the proposed, it is clarified that there is a cleanup of transform skip and MTS related syntax signaling. It was commented that the syntax changes may be not necessary for the encoder speedup.

Three aspects are being proposed

- Extending transform skip to 32x32 (several related proposals)

- Encoder changes related to transform search

- Syntax unifications are proposed (one related proposal JVET-M0201)

It is agreed that a unification/cleanup of the transform signaling may be desired. Regarding 2), the results with inter MTS enabled are also requested since the proposed binarization of MTS index gives different priority to different transform types.

JVET-M0501 CE6 related: Unification of Transform Skip mode and MTS [X. Zhao, X. Li, S. Liu (Tencent)]

In VTM3.0, both Multiple Transform Selection (MTS) and Transform Skip (TS) mode are supporting transform selection but with separate syntax. In this contribution, TS mode and MTS are unified such that TS mode becomes part of MTS by replacing DCT-8 with Identity Transform (IDT) and enabling MTS for chroma. It is reported that with the proposed method, transform skipping can be done with more flexibility, e.g., horizontal and vertical transform can be skipped separately or jointly. Similar to TS mode, the proposed method also supports enabling IDT for different maximum block sizes so that transform skipping can be optimized for different video content. It is reportedly shown that, on top of VTM-3.0, when the maximum size of enabling IDT is 4-point, average -0.15% and -0.09% luma BD-Rate is achieved under All Intra (AI) and Random Access (RA) configurations, respectively, and average -1.04% and -0.86% luma BD-Rate is achieved for Class F under AI and RA configurations, respectively. When the maximum block size of enabling IDT is set as 16-point, average -0.08% and -0.06% luma BD-Rate is achieved under All Intra (AI) and Random Access (RA) configurations, respectively, and average -2.76% and -2.42% luma BD-Rate is achieved for Class F under AI and RA configurations, respectively.

Two aspects have been proposed. First, Transform skip flag is replaced by MTS index (DCT8,DCT8).

Second, transform skip is applied either horizontally or vertically.

It is commented that VVC has different quantization between MTS and TS. The proposed method adjusted/unified quantization part between MTS and TS.

Partial of this method was proposed last meeting. Only 4 point MTS was replaced and luma change only.

For intra MTS, (DCT8, DCT8) combination occurs rarely and it doesn’t give much coding gain. The proposed method reused that part by considering TS mode. Transform candidates in MTS has been modified. Statistics of MTS candidates may be modified by the proposed method.

TS mode is only available in DM mode.

Reusing MTS flag for DM mode was not tested (it was asked)

Enable MTS for Chroma is applied for 4 tests. It provides coding performance improvement about  0.1%(CTC), ,

It was commented that decoding time increment is about 2 to 3%, and it is observed up to 5%. It is commented that decoder runtime increases due to MTS being applied in chroma.

The proposed method requires syntax and semantic changes between MTS and TS.

It was commented (by non-proponent) that the proposed method is not clean-up. Rather M0464 seems more clean in the aspects of syntax change.

The proponent has different opinion because there is a clean-up regarding syntax change.

It was commented that the coding performance may come from syntax modification.

Results reported by several related proposals show that 32x32 transform skip gives additional coding performance, which conflicts with comment.

It was also commented that it is difficult to achieve big gain (2% in ClassF) by syntax modification.

Recommend:

First aspect (TS and MTS unification): to be studied in CE

Second aspect (enabling MTS for chroma): to be studied in CE

It was agreed that only apply MTS for DM mode to be tested.

JVET-M0656 Crosscheck of JVET-M0501 (CE6 related: Unification of Transform Skip mode and MTS) [K. Choi (Samsung)] [late] [miss]

## Transform Selection

JVET-M0480 CE6-related: Implicit transform selection for Multi directional LM [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

This contribution proposes to apply implicit transform selection for multi-directional LM (MDLM). Currently, multiple transform selection (MTS) is applied only to a luma block, where DCT-II, DST-VII, and DCT-VIII are selectively applied for horizontal and vertical direction independently by signalling tu_mts_flag and mts_idx. On the contrary, DCT-II is always applied to a chroma block for both horizontal and vertical direction. The proposed method implicitly selects transform core for chroma block when MDLM is applied, in which either top or left neighbor samples are used for linear model derivation. Experimental results show -0.42% (Cb) and -0.47% (Cr) BD-rate gain for AI, -0.28% (Cb) and -0.21% (Cr) for RA configuration without increase of encoding and decoding time. In version 3 of this contribution, the experimental results shown in Table 1 to Table 3 were corrected since the old tables describe test results of the parameter setting where min/max values of applicable chroma block size set equal to 4 and 32, whereas the setting of min/max recommended by the proponent is 4/16.

The proposed method shows some benefit on chroma coding, may be due to the asymmetric distribution of residual energy in MDLM. For CCLM, only DCT2 is used which is same as VTM-3.0. It was questioned whether there is a parsing dependency, it is clarified by proponent that there is no parsing dependency. Further study in CE (maybe together with other related contributions).

JVET-M0573 Crosscheck of JVET-M0480 (CE6-related: Implicit transform selection for Multi directional LM) [K. Kazui (Fujitsu)] [late] [miss]

JVET-M0482 CE6-related: Implicit transform selection for Multi-hypothesis inter-intra mode [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

This contribution proposes to apply implicit transform selection for multi-hypothesis inter-intra (MHIntra) prediction. Currently, multiple transform selection (MTS) is applied to a luma block, where DCT-II, DST-VII, and DCT-VIII are selectively applied for horizontal and vertical direction independently by signalling tu_mts_flag and mts_idx. The proposed method implicitly selects transform core without signalling mts_idx for blocks to which MHIntra is applied if the tu_mts_flag is true. Experimental results against VTM-3.0 with inter MTS enabled show negligible impact on BD-rate performance with reducing 4% of encoding time.

Together with Inter MTS (w/o fast method), there is -0.47% gain for RA with 139% enc-time. No impact is shown on decoding. During the discussion, it was questioned whether sub-block transform is applied for multi-hypothesis intra inter mode (MHIntra). It was mentioned that sub-block transform does not have (DCT2, DST7), (DST7, DCT2) combinations.

Further study in CE.

JVET-M0569 Crosscheck of JVET-M0482 (CE6-related: Implicit transform selection for Multi-hypothesis inter-intra mode) [T. Chujoh (Sharp)] [late]

## Others

JVET-M0201 CE6-related: Syntax clean-up related to MTS [K. Choi, M. Park, M. W. Park, W. Choi (Samsung)]

This contribution presents a syntax clean-up related to MTS by rearranging a MTS mode syntax from the part of residual coding syntax to the part of transform unit syntax. Testing results on the proposed method show no change of coding performance compared to VTM3.0.

It was commented that it is more reasonable to put transform index signaling ahead of residual coding. There is also a related proposal JVET-M0464 which also relocates the MTS related syntax. To be revisited together with JVET-M0464 when complete results of “CPR on” becomes available.

JVET-M0671 Crosscheck of JVET-M0201 (CE6-related: Syntax clean-up related to MTS) [X. Cao (Hikvision)] [late] [miss]

JVET-M0280 CE6-related: Context selection for entropy coding the MTS flag [S.-T. Hsiang, S.-M. Lei (MediaTek)]

This contribution proposes a modified method for entropy coding the syntax element tu_mts_flag for indicating the use of the multiple primary transform selection. The context modeling scheme for coding tu_mts_flag in VVC working draft 3 is conditioned on the quad-tree depth of the current CU. Some of assigned contexts are not efficiently used under the current CU size constraints. In the proposed method, the context modeling scheme is conditioned on the CU size using 2 contexts. The proposed method reportedly generates -0.01%, -0.01%, and +0.02% luma BD-rates for the AI, RA, and LB settings, respectively, under the VTM3.0 CTCs while reducing the total number of context variables by 4. The proposed method reportedly generates 0.00% and 0.00% luma BD-rates for the RA and LB settings, respectively, under the VTM3.0 CTCs with the Inter MTS mode being enabled.

It was commented that there seems an increasing interest on the context reduction for various syntaxes given the input contributions of this meeting. The context selection is dependent on the block area size (whether it is larger than 256 or not), two contexts are used.

JVET-M0738 Crosscheck of JVET-M0280 (CE6-related: Context selection for entropy coding the MTS flag) [A. Tamse (Samsung)] [late] [miss]

JVET-M0347 CE6-related: On MTS CU flag coding [X. Cao, F. Chen, L. Wang (Hikvision)]

This contribution presents a simplification on MTS CU flag coding, the proposed method unifies the context model and removes the dependency with the depth of CU’s quartree partition for MTS_CU_flag, and the numbers of context models of MTS_CU_flag are reduced from 6 to 1. It is reported that the BD-rate are 0.01%, 0.00% and 0.01% for AI, RA and LB respectively. When enable inter MTS on (EMT=3, EMT Fast=3), the BD-rate are 0.01% and 0.02% for RA and LB respectively.

The proposed method removed the context dependency on the CU quad-tree depth, only one single context is used. It was commented that there is also modification on context selection for MTS CU flag in JVET-M0464, which may have interaction with the proposed method. However, it was commented that there is no change on MTS CU flag context selection in JVET-M0464.

It was questioned whether there is a benefit by reducing contexts, and it was commented that less context and simpler context selection is a preference given the same coding performance. The proposed method is supported by a non-proponent.

Recommend by BoG for Adoption.

JVET-M0655 Crosscheck of JVET-M0347 (CE6-related: Simplification on MTS CU flag coding) [K. Choi (Samsung)] [late] [miss]

JVET-M0361 Non-CE6: Mismatch between text specification and reference software on the signalling root CBF [J. Jung, D. Kim, G. Ko, J. Son, J. Kwak (WILUS)]

In this contribution, it is reported that there is a mismatch between VVC Draft 3 and VTM-3.0 on signalling syntax element cu_cbf. In VVC Draft 3, cu_cbf is signalled for inter-coded blocks when cu_skip_flag is equal to 0. However, in VTM-3.0, cu_cbf is signalled for inter-coded blocks when merge_flag is equal to 0. It is reportedly shown that SW change reflecting the method described in the specification text increases overall BD-rate by 0.25%, 0.44% with 101%, 100% overall encoding time and 100%, 100% overall decoding time under Random Access (RA) and Low Delay B (LDB) configurations, respectively. Therefore, it is recommended to change the specification text to the method used in the reference software.

It was commented by the proponent that the proposed spec text is a bug-fix to the current VVC Draft 3, and it has been confirmed by Editor.

Recommend to include this bug-fix into next VVC Draft.

JVET-M0790 Cross-check of JVET-M0361 (Non-CE6: Mismatch between text specification and reference software on the signalling root CBF) [B. Lee (Chosun Univ.)] [late]

JVET-M0524 CE3/6-related: Unification of RQT-like transform partitioning for intra and inter blocks [H. Egilmez, V. Seregin, A. Said, M. Karczewicz (Qualcomm)]

This contribution presents two unified extensions of the transform unit (TU) partitioning method, tested in CE6.4.2 for inter blocks. The first unification is a trivial extension of the method in CE6.4.2, which is applied to both intra and inter predicted blocks. The second unified design is a more flexible variation of the same method allowing smaller TU partitions.

Over VTM-3.0, the first extension provides

- 0.19% AI, 0.45% RA and 0.65% LDB average luma coding gains;

- 0.19% AI, 0.74% RA and 1.00% LDB average luma coding gains when Inter MTS is also enabled.

The second extension achieves

- 0.29% AI, 0.56% RA and 0.71% LDB average luma coding gains;

- 0.29% AI, 0.83% RA and 1.11% LDB average luma coding gains when Inter MTS is also enabled.

Based on the review output of CE6 Category 4, study on RQT-like sub-block transform is discontinued. Presentation may be not needed.

JVET-M0847 Cross-check of JVET-M0524 (CE3/6-related: Unification of RQT-like transform partitioning for intra and inter blocks) [P. Philippe (bcom Orange)] [late] [miss]