| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0888-v1 |
| --- | --- |

| Title: | CE1-related: Picture boundary handing with VPDU constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Shih-Ta Hsiang, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, shih-ta.hsiang, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two solutions to simplify virtual pipeline data unit (VPDU) processing at picture boundary, where the VPDU size is targeted to be 64x64-luma/32x32-chroma for 4:2:0 chroma sampling format. If one coding tree unit (CTU) of 128x128-luma/64x64-chroma is at picture boundary and is not a complete CTU, solution 1 applies quad-tree (QT) split to partition the CTU in the first depth instead of binary-tree (BT) split. In solution 2, if a 128xN CTU is at the bottom picture boundary, QT split or vertical BT split can be chosen to split the CTU instead of horizontal BT split in the first depth. Similarly, if one Nx128 CTU is at the right picture boundary, QT split or horizontal BT split can be used to split the CTU in the first depth instead of vertical BT split. BD-rate results of these two solutions are reported as follows:

Solution 1:
RA BD-rates = 0.xx% (Y), 0.xx% (U), 0.xx% (V)
LB BD-rates = 0.xx% (Y), 0.xx% (U), 0.xx% (V)

Solution 2:
RA BD-rates = 0.05% (Y), 0.04% (U), 0.06% (V)
LB BD-rates = 0.07% (Y), -0.06% (U), 0.03% (V)

Both solutions ensure that the VPDU constraint is not violated at picture boundaries. It is asserted that the runtime impact is small if accurately measured.

# Introduction

VPDUs are defined as non-overlapping MxM-luma(L)/NxN-chroma(C) units in a picture [1]. The following constraint is proposed.

Cond. 1: For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU.
Cond. 2: For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU.
Proposed constraint: For each CTU, the above two conditions shall not be violated, and the processing order of CUs shall not leave a VPDU and re-visit it later.

Examples in Figure 1 are allowed BT splits and TT splits by the constraint. In VVC, we target M and N equal to 128 and 64, respectively.

Figure 1.  Examples of allowed BT and TT splits for 64x64-L/32x32-C pipelining (VPDUs indicated by dashed lines)

However, in VTM3.0 [2], the CU split at picture boundary could violate the VPDU constraint. For example, in Figure 2, the processing order of CUs shall leave a VPDU and then re-visit it later. Therefore, hardware decoders have to handle exceptions at picture boundaries, which is not desirable.

Figure 2.  An example for the processing order of CUs leaving a VPDU and re-visit it later. (VPDUs indicated by dashed lines, picture boundary is the blue line, the region outside the picture is in gray, vertical BT split is the green line, and the numbers show the processing orders of the four CUs)

# Proposed methods

Two solutions are proposed to simplify VPDU processing at picture boundary.

## Solution 1

If one CTU of 128x128-luma/64x64-chroma is at picture boundary and is not a complete CTU, QT split is applied to partition the CTU in the first depth instead of BT split, as shown in Figure 3.

Figure 3.  The two examples on the left side (i.e., (a) and (c)) for applying QT split to a 128xN/Nx128 CTU in the first depth (Depth = 0), and the two examples on the right side (i.e., (b) and (d)) show the processing order for applying horizontal/vertical BT split in the second depth (Depth = 1). (VPDUs indicated by dashed lines, picture boundary is the blue line, and the region outside the picture is in gray)

## Solution 2

If one 128xN CTU is at the bottom picture boundary, then QT split or vertical BT split can be chosen to split the CTU in the first depth (Depth = 0) instead of horizontal BT split, as shown in Figure 4. Similarly, if one Nx128 CTU is at the right picture boundary, QT split or horizontal BT split can be used to split the CTU in the first depth (Depth = 0) instead of vertical BT split, as shown in Figure 5.

Figure 4.  The two examples on the left side (i.e., (a) and (b)) for applying QT split and vertical BT split to a 128xN CTU in the first depth (Depth = 0), and two examples on the right side (i.e., (c) and (d)) show the processing order for applying horizontal BT split in the second depth (Depth = 1). (VPDUs indicated by dashed lines, picture boundary is the blue line, and the region outside the picture is in gray)

Figure 5.  The two examples on the left side (i.e., (a) and (b)) for applying QT split and horizontal BT split to a Nx128 CTU in the first depth (Depth = 1), and the two examples on the right side (i.e., (c) and (d)) show the processing order for applying vertical BT split in the second depth (Depth = 2). (VPDUs indicated by dashed lines, picture boundary is the blue line, and the region outside the picture is in gray)

# Experimental results

The proposed methods have been implemented on VTM3.0 [2]. The simulations were performed following the JVET common test conditions (CTC) [3]. Table 1 and Table 2 are the results for solution 1 and solution 2, respectively. Runtime percentages are not accurate. It is asserted that the runtime impact is small if accurately measured.

Table 1.  Results of solution 1.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | 0.09% | 0.03% | 100% | 103% |
| Class C | 0.03% | 0.05% | 0.08% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.07% | 0.13% | 0.09% | 98% | 100% |
| Class F | 0.04% | 0.05% | 0.06% | 99% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.03% | 0.15% | 0.08% | 100% | 100% |
| Class E | 0.35% | 0.06% | -0.07% | 99% | 103% |
| Overall |  |  |  |  |  |
| Class D | 0.06% | 0.40% | 0.24% | 101% | 101% |
| Class F |  |  |  |  |  |

Table 2.  Results of solution 2.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.03% | 0.05% | 100% | 113% |
| Class A2 | 0.02% | 0.03% | 0.04% | 100% | 105% |
| Class B | 0.08% | 0.03% | 0.08% | 100% | 104% |
| Class C | 0.02% | 0.07% | 0.05% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.04% | 0.06% | 100% | 105% |
| Class D | 0.06% | 0.07% | 0.03% | 100% | 98% |
| Class F | 0.03% | 0.02% | 0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.17% | 0.01% | 100% | 99% |
| Class C | -0.02% | 0.14% | 0.18% | 100% | 99% |
| Class E | 0.18% | -0.15% | -0.14% | 100% | 105% |
| Overall | 0.07% | -0.06% | 0.03% | 100% | 101% |
| Class D | 0.03% | -0.09% | 0.20% | 100% | 98% |
| Class F | 0.05% | 0.07% | 0.26% | 100% | 99% |

# Conclusions

This contribution proposes two solutions to ensure that the VPDU constraint is satisfied at picture boundaries.

# References

- C.-M. Tsai, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE1.2.1: Constraint for binary and ternary partitions,” Document of Joint Video Experts Team, JVET-L0081.

- VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).