| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M1027-v2 |
| --- | --- |

| Title: | Description of Core Experiment 7 (CE 7): Quantization and coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Heiko Schwarz Muhammed Coban Cheung Auyeung | Email: | heiko.schwarz@hhi.fraunhofer.de mcoban@qti.qualcomm.com cheung.auyeung@huawei.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

The purpose of this core experiment is to explore the coding efficiency and complexity impact of proposed algorithms for quantization and transform coefficient coding. This core experiment evaluates the impact of:

- Context modeling/selection/coding order for syntax elements related to transform coefficients

The objectives of this CE include:

- Investigation of the impact of context selection and signaling schemes for transform coefficient levels on the coding efficiency and complexity.

# Participants List

| Affiliation | Person, Email address |
| --- | --- |
| Qualcomm | Muhammed Coban <mcoban@qti.qualcomm.com> Dmytro Rusanovskyy <dmytror@qti.qualcomm.com> Jie Dong <jiedong@qti.qualcomm.com> Ted Hsieh <thsieh@qti.qualcomm.com> Marta Karczewicz <martak@qti.qualcomm.com> |
| Fraunhofer HHI | Heiko Schwarz <heiko.schwarz@hhi.fraunhofer.de> Tung Nguyen <tung.nguyen@hhi.fraunhofer.de> Christian Helmrich <christian.helmrich@hhi.fraunhofer.de> |
| Samsung | Yinji Piao <yj1003.piao@samsung.com> Kiho Choi <kiho14.choi@samsung.com> |
| Huawei | Alexey Filippov <alexey.filippov@huawei.com> Jianle Chen <Jianle.chen@huawei.com> Cheung Auyeung <cheung.auyeung@huawei.com> Yang Haitao <haitao.yang@huawei.com> Yin Zhao <yin.zhao@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> Biao Wang <biao.wang@huawei.com> Vasily Rufitskiy <vasily.rufitskiy@huawei.com> Alexander Karabutov <karabutov.alexander@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> |
| Orange | Pierrick Philippe <pierrick.philippe@orange.com> Felix Henry <felix.henry@orange.com> Gordon Clare <gordon.clare@orange.com> |
| MediaTek | Mohammed Sarwer <Mohammed.Sarwer@mediatek.com> Olena Chubach <Olena.Chubach@mediatek.com> Yu-Wen Huang <yuwen.huang@mediatek.com> Chih-Wei Hsu <cw.hsu@mediatek.com> Ching-Yeh Chen <chingyeh.chen@mediatek.com> T.-D. Chuang <peter.chuang@mediatek.com> Shih-Ta Hsiang <shih-ta.hsiang@mediatek.com> |
| Sharp | Eiichi Sasaki <eiichi.sasaki@sharp.co.jp> Takeshi Chujoh <chujoh.takeshi@sharp.co.jp> Tomohiro Ikai <ikai.tomohiro@sharp.co.jp> Kiran Misra <misrak@sharplabs.com> Andrew Segall <asegall@sharplabs.com> Frank Bossen <frank.bossen@me.com> |
| Apple | Alexis Tourapis <atourapis@apple.com> Jae Hoon Kim <i.jaehoon.kim@icloud.com> |
| KDDI | Kei Kawamura <ki-kawamura@kddi.com> Kyohei Unno <ky-unno@kddi-research.jp> Yoshitaka Kidani <yo-kidani@kddi.com> |
| Kwai | Yi-Wen Chen <yiwenchen@kwai.com> Xianglin Wang <xianglinwang@kwai.com> |
| Bytedance | Li Zhang <lizhang.idm@bytedance.com> Hongbin Liu <liuhongbin.01@bytedance.com> |
| Nokia | Jani Lainema <jani.lainema@nokia.com> |
| Broadcom | Minhua Zhou <minhua.zhou@broadcom.com> Wade Wan <wade.wan@broadcom.com> Peisong Chen <peisong.chen@broadcom.com> |
| Ericsson | Julien Le Tanou <julien.le.tanou@ericsson.com> |
| Sony | Takeshi Tsukuba <Takeshi.Tsukuba@sony.com> Karl Sharman <karl.sharman@eu.sony.com> |
| InterDigital | Xiaoyu Xiu <xiaoyu.xiu@interdigital.com> Yuwen He <Yuwen.He@InterDigital.com> Rahul Vanam <Rahul.vanam@interdigital.com> |
| Canon | Chris Rosewarne <chris.rosewarne@cisra.canon.com.au> Andrew Dorrell <andrew.dorrell@cisra.canon.com.au> |
| BBC | Andre Seixas Dias <Andre.SeixasDias@bbc.co.uk> Saverio Blasi <saverio.blasi@bbc.co.uk> |
| Technicolor | Fabien Racape <fabien.racape@technicolor.com> Ya Chen <Ya.chen@technicolor.com> Edouard Francois <Edouard.francois@technicolor.com> Philippe de Lagrange <Philippe.delagrange@technicolor.com> Leleannec Fabrice <Fabrice.Leleannec@technicolor.com> |
| HiSilicon | Jianhua Zheng <zhengjianhua@hisilicon.com> Quanhe Yu <yuquanhe@hisilicon.com> |
| ETRI | Jungwon Kang <jungwon@etri.re.kr> Sung-Chang Lim <sclim@etri.re.kr> |
| Panasonic | Kiyofumi Abe <abe.kiyo@jp.panasonic.com> Han Boon Teo <hanboon.teo@sg.panasonic.com> Chong Soon Lim <chongsoon.lim@sg.panasonic.com> |
| LG | Jung-Ah Choi <jungah.choi@lge.com> Jin Heo <jin78.heo@lge.com> Sunmi Yoo <sunmi.yoo@lge.com> Jangwon Choi <jangwon84.choi@lge.com> Ling Li <l.li@lge.com> Seung Hwan Kim <seunghwan3.kim@lge.com> Mehdi Salehifar <mehdi.salehifar@lge.com> Moonmo Koo <moonmo.koo@lge.com> Jaehyun Lim <jaehyun.lim@lge.com> |
| Dolby | Sean McCarthy <smcca@dolby.com> Peng Yin <pyin@dolby.com> |
| Comcast | Dan Grois <grois@ieee.org> |
| Intel | Sabine Roessel <sabine.roessel@intel.com> |
| Tsinghua University | Benben Niu <nbb16@mails.tsinghua.edu.cn> |
| Tencent | Xin Zhao <xinzzhao@tencent.com> Xiang Li <xlxiangli@tencent.com> |
| WILUS | Dongcheol Kim <dongcheol.kim@wilusgroup.com> |
| Foxconn | Yao-Jen Chang <yaojen.chang.foxconn@gmail.com> |

# Tools under Test

The following tools will be considered in this CE. Details are described in sec. 6.

|  | Proponent | Document | Tool description |
| --- | --- | --- | --- |
| 1 | Nokia | JVET-M0305 | Joint coding of Cb and Cr residuals |
| 2 | Tencent | JVET-M0491 | Reduced number of context-coded bins |
| 3 | Samsung | JVET-M0198 | Template-based unified Rice parameter derivation |
| 4 | Qualcomm | JVET-M0558 | Template-based Rice parameter derivation for coding of abs_remainder |

# Test Description

## Software, reference configurations, and test conditions

All tools have to be integrated into the VTM-4 software (outcome of the Marrakech meeting).  Results have to be reported for the VTM-4 software and the common test conditions (standard configuration files).

All tools are tested against the VTM-4 anchor (same set of tools, except the tool to be tested).

Results have to be provided for two ranges of QP values:

- Standard QPs (37, 32, 27, 22): AI, RA, and LD configurations as specified in CTC (all frames);

- Low QPs (17, 12, 7, 2): AI, RA, and LD configurations with 100 frames only (i.e., by using the encoder option “--FramesToBeEncoded=100”).

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-rate). Proponents are encouraged to provide supplementary results.

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the reference configurations);

- Decoding time (compared to the reference configuration).

In addition, proponents are encouraged to provide the following (but not limited to) information for complexity and entropy throughput analysis:

- Number of context models

- Number of context-coded and bypass bins in average case and the worst case

- Table size of scanning paths

- Analysis of the frequency of context switches

- Analysis of the dependency between two consecutive bins

- Analysis of entropy parsing dependency (whether bits to bins conversion depends on complex reconstruction and/or pixel processing process)

- Analysis of entropy de-binarization dependency (whether bins to symbols conversion depends on complex reconstruction and/or pixel processing process)

## Test plan

The core experiment is split into the following tests:

Transform coefficient coding

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| CE7-1 | Jani Lainema (Nokia) | Joint coding of Cb and Cr residual | H. Schwarz (HHI) |
| CE7-2 | Xin Zhao (Tencent) | Reduced number of context-coded bins |  |
| CE7-3 | Yinji Piao (Samsung) | Template-based unified Rice parameter derivation |  |
| CE7-4 | Muhammed Coban (Qualcomm) | Template based Rice parameter derivation for coding of abs_remainder |  |

# Timeline

General timeline for core experiments (as agreed by JVET).

| T1 | 3 weeks after meeting | - Final CE description uploaded |
| --- | --- | --- |
| T2 | 2 weeks after VTM-4 release or March, 4 (whichever is earlier) | - Proponents completed initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | 3 weeks before next JVET meeting or T2 + 1 week (whichever is later) | - Proponents froze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |

Test results and cross check results are reported to CE coordinators (for inclusion into CE report).

# Detailed Description of Test

## Joint coding of Cb and Cr residuals [JVET-M0305]

This test measures performance of joint Cb-Cr coding mode proposed in JVET-M0305 [1]. One bin indicator is signaled in the bitstream to enable the mode. In the case the mode is enabled a joint residual signal is coded in the bitstream. On the decoder side the joint residual is used for Cb component and a negative version of the residual is applied for Cr.

Signaling of the mode is expected to follow the syntax and semantics given below:

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| if( cIdx == 2 && tu_cbf_cb[ x0 ][ y0 ] ) { |  |
| tu_cb_cr_joint_residual[ x0 ][ y0 ] | ae(v) |
| if( tu_cb_cr_joint_residual [ x0 ][ y0 ] ) { |  |
| return |  |
| } |  |
| … |  |

tu_cb_cr_joint_residual[ x0 ][ y0 ] specifies whether indicated Cb residual is used to derive both Cb and Cr residuals. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. tu_cb_cr_joint_residual[ x0 ][ y0 ] equal to 1 specifies that the indicated Cb residual is used to derive the Cr residual. tu_cb_cr_joint_residual[ x0 ][ y0 ] equal to 0 specifies that Cr residual may be present in the bitstream depending on other syntax elements. When tu_cb_cr_joint_residual[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

## Reduced number of context-coded bins [JVET-M0491]

In the coefficient coding of current VVC design, the averaged maximum number of context-coded bins per 4x4 sub-block is 32. In JVET-M0491 [2], to further reduce the averaged maximum number of context-coded bins per 4x4 sub-block, it is proposed to reduce the maximum number of context-coded bins for 4x4 sub-blocks in high frequency region such that the averaged maximum number of context-coded bins per 4x4 sub-block is 30 for the whole CU.

## Template-based unified Rice parameter derivation [JVET-M0198]

The unified rice parameter derivation in JVET-M0198 [3] is similar to the one for completely bypass-coded level, which refers to the neighboring transform coefficients within a given template. Rice parameter is determined not only based on sum of absolute levels of neighboring five transform coefficients in local template, but the corresponding base level is also taken into consideration.

RicePara = RiceParTable[ max(min( 31, sumAbs – 5 * baseLevel), 0) ],

where RiceParTable is the table specified in VTM-4 and baseLevel can be equal to 4 or 0.

For remainder of greater than 3 (gt3Flag), the baselevel is equal to 4; for fully bypassed coded level, the baselevel is equal 0.

## Template based Rice parameter derivation for abs_remainder for unification of Rice parameter derivation  [JVET-M0558]

In JVET-M0558 [4], local neighbour template based Rice parameter derivation scheme used for coding of dec_abs_level (bypass coded coefficients) is extended to the coding of abs_remainder, replacing the VTM-4.0’s HEVC like scheme. The local 5 neighboring coefficients’ sum of absolute values is used to derive the Rice parameter for coding of abs_remainder (representing half of remainder values for context coded coefficients) by

sumAbs=k=04xk

ricePar   = max(0, riceParTable[ min( 31, sumAbs ) ] -  1)

where the riceParTable[] is the existing table that is used to derive the Rice parameter in VTM-4.0 dec_abs_level coding scheme, and xk are the decoded levels of neighboring coefficients.

# References

- J. Lainema, “CE7-related: Joint coding of chroma residuals,” Joint Video Experts Team, doc. JVET-M0305, Jan. 2019.

- M. Gao, X. Li, S. Liu, “CE7-related: Reduced maximum number of context-coded bins for transform coefficient coding,” Joint Video Experts Team, doc. JVET-M0491, Jan. 2019.

- Y. Piao, K. Choi, “CE7-related: Unified rice parameter derivation for coefficient level coding,” Joint Video Experts Team, doc. JVET-M0198, Jan. 2019.

- M. Karczewicz, M. Coban, “CE7-related: Template based Rice parameter derivation,” Joint Video Experts Team, doc. JVET-M0558, Jan. 2019.