| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M1029 |
| --- | --- |

| Title: | Description of Core Experiment 9 (CE9): Decoder Motion Vector Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JVET |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Xiaoyu Xiu | Email: | semih.esenlik@huawei.com xiaoyuxiu@kwai.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

This document is a description of Core Experiment 9 (CE9) on decoder motion vector derivation. The scope of the tools that are to be studied in this CE include decoder motion vector refinement and bi-directional optical flow. In total 11 tests are planned to be performed in two subtests, 2 tests in DMVR subtest and 9 tests in BDOF subtest.

# List of participants

| Participants | Proponent | Cross-checker |
| --- | --- | --- |
| Semih Esenlik (semih.esenlik@huawei.com) Jianle Chen (Jianle.Chen@huawei.com) Haitao Yang (haitao.yang@huawei.com) Huanbang Chen (chenhuanbang@huawei.com) Xiang Ma (maxiang6@huawei.com) | X | X |
| Wei-Jung Chien (wchien@qti.qualcomm.com) Chun-Chi Chen (chunchic@qti.qualcomm.com) Chao-Hsiung Hung (chaohsiu@qti.qualcomm.com) Dmytro Rusanovskyy (dmytror@qti.qualcomm.com) | X | X |
| Xiaoyu Xiu (xiaoyu.xiu@interdigital.com) Yuwen He (yuwen.he@interdigital.com) Daniel Luo (daniel.luo@interdigital.com) Saurav Bandyopadhyay (saurav.bandyopadhyay@interdigital .com) |  |  |
| Meng Xu (mengxxu@tencent.com) Xiang Li (xlxiangli@tencent.com) Sehoon Yea (sehoonyea@tencent.com) |  |  |
| Sriram Sethuraman (Sriram.sethuraman@ittiam.com) |  |  |
| Xu Chen (anderson.chen@hisilicon.com) Jianhua Zheng (zhengjianhua@hisilicon.com) |  |  |
| Hongbin Liu (liuhongbin.01@bytedance.com) Li Zhang (lizhang.idm@bytedance.com) | X | X |
| Fangdong Chen (chenfangdong@hikvision.com) |  |  |
| Tzu-Der Chuang (peter.chuang@mediatek.com) Chih-Wei Hsu (cw.hsu@mediatek.com) Yu-Wen Huang (yuwen.huang@mediatek.com) Ching-Yeh Chen (chingyeh.chen@mediatek.com) Shih-Ta Hsiang (shih-ta.hsiang@mediatek.com) |  |  |
| Minhua Zhou (minhua.zhou@broadcom.com) |  |  |
| Yue Li (lytt@mail.ustc.edu.cn) |  |  |
| Ola Hugosson (Ola.Hugosson@arm.com) |  |  |
| Yin, Peng (pyin@dolby.com) |  |  |
| Kenji Kondo (kenji.kondo@sony.com) | X | X |
| Segall, Andrew (asegall@sharplabs.com) Chujoh, Takeshi (chujoh@sharp.co.jp) Ikai, Tomohiro (ikai.tomohiro@sharp.co.jp) Sasaki, Eiichi (eiichi.sasaki@sharp.co.jp) |  |  |
| Kyohei Unno (ky-unno@kddi-research.jp) Kei Kawamura (ki-kawamura@kddi.com) Yoshitaka Kidani (yo-kidani@kddi.com) |  |  |
| Hahyun Lee (hanilee@etri.re.kr) Jungwon Kang (jungwon@etri.re.kr) Sung-Chang Lim (sclim@etri.re.kr) |  |  |
| Jing Ya LI (jingya.li@sg.panasonic.com) Chong Soon LIM (chongsoon.lim@sg.panasonic.com) Kiyofumi Abe (abe.kiyo@jp.panasonic.com) Ru-Ling Liao (ruling.liao@sg.panasonic.com) | X | X |
| Franck Galpin (franck.galpin@technicoolor.com) Edouard Francois (Edouard.Francois@technicolor.com) |  |  |
| Anish Tamse (tamse.anish@samsung.com) Yinji Piao (yj1003.piao@samsung.com) Kiho Choi (kiho14.choi@samsung.com) Jie Chen (jie1222.chen@samsung.com) |  |  |
| Hong Wang (philcage@163.com) |  |  |
| Gosala Kulupana (gosala.kulupana@bbc.co.uk) |  |  |
| Hyeongmun Jang (hm.jang@lge.com) |  |  |
| Grace Yu (grace.yu@intel.com) |  |  |
| Jie Chen (jiechen.cj@alibaba-inc.com) Yan Ye (yan.ye@alibaba-inc.com) |  |  |

# CE9.1: DMVR

## Description of the tests

### CE9.1.1: JVET-M0516

This test disables the use of 8x8/4xN CUs for DMVR. Extra reference samples are prefetched to fill the padded area prior to bilinear interpolation and final motion compensation.

## Test Specifications

| # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| CE9.1.1.a | Disable 8x8/4xN CUs for DMVR. | C.-C. Chen | S. Esenlik |
| CE9.1.1.b | CE9.1.1.a + Prefetch extra samples to fill padded area | C.-C. Chen | S. Esenlik |

# CE9.2: BDOF

## Description of the tests

### CE9.2.1: JVET-M0890

Disabling of SAD based early termination of BDOF is tested.

### CE9.2.2: JVET-M0073

In VTM-3.0, BDOF has 2-stages early termination for CU level and 4x4 block level. As mentioned in JVET-M0073, the CU level early termination requires large buffer size to store inter prediction blocks. To consider the trade-off for hardware complexity and decoding time, it’s tested that the SAD region for early termination is divided for each NxN size sub-block to reduce buffer size.

1) The SAD region is divided for each NxN sub-blocks for 1st stage of early termination.

2) The SAD region is divided for each NxN sub-blocks for 1st stage of early termination. And an early termination for 2nd stage (4x4 block level) is disabled.

3) The early termination for 1st stage (CU level) is disabled.

### CE9.2.3: JVET-M0249

Modify BDOF in two aspects: first, when a CU contains more than N (different N may be tested) pixels, it is split into multiple sub-blocks and BDOF is performed for each sub-block; second, SAD is calculated only for some pixels instead of all pixels.

### CE9.2.4: JVET-M0284

Splitting BDOF application region into 2 or 4 smaller sub-regions, if the region crosses a 64x64 VPDU boundary.

### CE9.2.5: JVET-M0316

This contribution proposes to replace VTM4’s method of calculating BDOF’s optical flow parameter with following:

s1=[i,j]∊Ω absIx1+Ix0.

s1=[i,j]∊Ω absIx1+Ix0.

s2=[i,j]∊Ω absIy1+Iy0.

s2=[i,j]∊Ω absIy1+Iy0.

s3=[i,j]∊Ω signIx1+Ix0*(I1-I0).

s3=[i,j]∊Ω signIx1+Ix0*(I1-I0).

s4=[i,j]∊Ω signIy1+Iy0*(I1-I0).

s4=[i,j]∊Ω signIy1+Iy0*(I1-I0).

vx =-s3s1.

vx =-s3s1.

vy =-s4s2.

vy =-s4s2.

If S1 > th & s2 < th

Horizontal flow = Vx, Vertical flow = 0

If S2 > th & s2 < th

Horizontal flow = 0, Veritcal flow = Vy,

Else

Horizontal flow = Vx, Veritcal flow = Vy,

The proposed method removes all multiplication operations.

## Test Specifications

| # | description | Tester | Crosschecker |
| --- | --- | --- | --- |
| CE9.2.1 | Disable SAD based early termination | S. Esenlik/K. Kondo |  |
| CE9.2.2.a | The SAD region is divided for each NxN sub-blocks for 1st stage of early termination. | K. Kondo |  |
| CE9.2.2.b | The SAD region is divided for each NxN sub-blocks for 1st stage of early termination. And an early termination for 2nd stage (4x4 block level) is disabled. | K. Kondo |  |
| CE9.2.2.c | The early termination for 1st stage (CU level) is disabled. | K. Kondo |  |
| CE9.2.3.a | When a CU contains more than N (different N may be tested) pixels, it is split into multiple sub-blocks and BDOF is performed for each sub-block. | H. Liu |  |
| CE9.2.3.b | SAD is calculated only for 4 corners (instead of all 16 pixels) of each 4x4 sub-block. | H. Liu |  |
| CE9.2.4 | Implicitly split BDOF application region along VPDU boundaries | S. Esenlik |  |
| CE9.2.5 | Simplification of BDOF’s optical flow parameter derivation | LI Jingya |  |
| CE9.2.6 | CE9.2.1 + CE9.2.4 | S. Esenlik |  |

# Time-line and Responsibilities

| T1 |  | Finalize CE description |
| --- | --- | --- |
| T2 |  | Deliver software packages and initial study by cross-checkers. Proponents can continue to modify the software until T3 |
| T3 |  | Software is finalized. Proponents provide full results and final crosschecking begins. |
| T4 |  | Contribution upload. CE9 summary report is uploaded. |