| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M1029-v7 |
| --- | --- |

| Title: | Description of Core Experiment 9 (CE9): Decoder Motion Vector Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JVET |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Xiaoyu Xiu | Email: | semih.esenlik@huawei.com xiaoyuxiu@kwai.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

This document is a description of Core Experiment 9 (CE9) on decoder motion vector derivation. The scope of the tools that are to be studied in this CE include decoder motion vector refinement and bi-directional optical flow. In total 12 tests are planned to be performed in two subtests, 3 tests in DMVR subtest and 9 tests in BDOF subtest.

# List of participants

| Participants | Proponent | Cross-checker |
| --- | --- | --- |
| Semih Esenlik (semih.esenlik@huawei.com) Jianle Chen (Jianle.Chen@huawei.com) Haitao Yang (haitao.yang@huawei.com) Huanbang Chen (chenhuanbang@huawei.com) Xiang Ma (maxiang6@huawei.com) | X | X |
| Wei-Jung Chien (wchien@qti.qualcomm.com) Chun-Chi Chen (chunchic@qti.qualcomm.com) Chao-Hsiung Hung (chaohsiu@qti.qualcomm.com) Dmytro Rusanovskyy (dmytror@qti.qualcomm.com) Yao-Jun Chang (yjchang@qti.qualcomm.com) | X | X |
| Sriram Sethuraman (Sriram.sethuraman@ittiam.com) | X | X |
| Xu Chen (anderson.chen@hisilicon.com) Jianhua Zheng (zhengjianhua@hisilicon.com) |  | X |
| Hongbin Liu (liuhongbin.01@bytedance.com) Li Zhang (lizhang.idm@bytedance.com) | X | X |
| Kenji Kondo (kenji.kondo@sony.com) Masa Ikeda (Masaru.Ikeda@sony.com) | X | X |
| Takeshi Chujoh (chujoh.takeshi@sharp.co.jp) |  | X |
| Jing Ya LI (jingya.li@sg.panasonic.com) Chong Soon LIM (chongsoon.lim@sg.panasonic.com) Kiyofumi Abe (abe.kiyo@jp.panasonic.com) Che-Wei Kuo (chewei.kuo@sg.panasonic.com) Han Boon Teo (hanboon.teo@sg.panasonic.com) | X | X |

# Test conditions and evaluation criteria

The VVC Test Model (VTM4.0) will be used as anchor. Proposals will be evaluated using BD rates, encoder run time, decoder time with Common Test Conditions (CTC) [1] as agreed at the Marrakech meeting.

In subsection 5 (CE9.2: BDOF specific simplification) the following suggestion from the meeting note should be taken into account: “The maximum size where early termination is used should be blocks of size 256 luma samples.”

# CE9-1: DMVR specific simplification

## Description of the tests

### CE9-1.1: JVET-M0516

This test disables the use of 8x8/4xN CUs for DMVR. In addition, the boundary padding process for  generating the filter input samples within the extended area is removed. As instead, the filter input samples within the extended area are loaded directly from the given reference picture. Specifically, the required number of samples to be loaded for DMVR is changed from (w+7)*(h+7) to (w+7+4)*(h+7+4).

Fig. Filter input samples (i.e. (w+7)*(h+7)) plus padded samples (i.e. plotted as white circles) at the extended area. (Example of 4x4 CU)

### CE9-1.2: JVET-L0174

This test studies the impact of restricting the bit-depth of the interpolated samples used for refinement cost calculations to 8-bits. Specifically, the method proposes to restrict the final output of bilinear interpolation to 8-bits before performing the cost evaluations so that the internal memory requirements are reduced. Also, 8-bit cost evaluation becomes more SIMD-friendly where most software platforms can get a 2x speed-up on the cost evaluations when compared to using the existing bit-depth in the adopted DMVR. In JVET-L0174, it was shown that there was no BDRATE impact and that the decoding time is reduced by 1%. This was scheduled for study in CE9.2.2 at the L-meeting. At the JVET closing plenary at the M-meeting, further study of this was requested and allowed.

## Test Specifications

| # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| CE9-1.1a | Disable 8x8/4xN CUs for DMVR. | C.-C. Chen | S. Esenlik |
| CE9-1.1b | CE9.1.1.a + Remove boundary padding process | C.-C. Chen | S. Esenlik |
| CE9-1.2 | Impact of bit-depth restriction applied to interpolated samples used for refinement in DMVR | S. Sethuraman | C.-C. Chen T. Chujoh |

# CE9-2: BDOF specific simplification

## Description of the tests

### CE9-2.1: JVET-M0890

Disabling of SAD based early termination of BDOF is tested.

### CE9-2.2: JVET-M0073

In VTM-3.0, BDOF has 2-stages early termination for CU level and 4x4 block level. As mentioned in JVET-M0073, the CU level early termination requires large buffer size to store inter prediction blocks. To consider the trade-off for hardware complexity and decoding time, it’s tested that the SAD region for early termination is divided for each 16x16 size sub-block to reduce buffer size.

1) The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination.

2) The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. And an early termination for 2nd stage (4x4 block level) is disabled.

3) The early termination for 1st stage (CU level) is disabled.

### CE9-2.3: JVET-M0249

Modify BDOF in two aspects: first, when a CU contains more than 256 luma samples, it is split into sub-blocks and each sub-block contains 256 luma samples. BDOF is performed for each sub-block. Second, SAD of each 4x4 sub-block is calculated using the 4 corner luma samples instead of all 16 luma samples.

### CE9-2.4: JVET-M0284

Splitting BDOF application region into 16x16 sub-regions, if the region is bigger than 16x16 luma samples in size.

### CE9-2.5: JVET-M0316

This contribution proposes to replace VTM4’s method of calculating BDOF’s optical flow parameter with following:

s1=[i,j]∊Ω absIx1+Ix0.

s2=[i,j]∊Ω absIy1+Iy0.

s3=[i,j]∊Ω signIx1+Ix0*(I1-I0).

s4=[i,j]∊Ω signIy1+Iy0*(I1-I0).

vx =-s3s1.

vy =-s4s2.

If S1 > th & s2 < th

Horizontal flow = Vx, Vertical flow = 0

If S2 > th & s2 < th

Horizontal flow = 0, Veritcal flow = Vy,

Else

Horizontal flow = Vx, Veritcal flow = Vy,

The proposed method removes all multiplication operations.

## Test Specifications

| # | description | Tester | Crosschecker |
| --- | --- | --- | --- |
| CE9-2.1 | Disable SAD based early termination | S. Esenlik/K. Kondo | X. Xiu |
| CE9-2.2a | The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. | K. Kondo | T. Chujoh |
| CE9-2.2b | The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. And an early termination for 2nd stage (4x4 block level) is disabled. | K. Kondo | T. Chujoh |
| CE9-2.2c | The early termination for 1st stage (CU level) is disabled. | K. Kondo | T. Chujoh |
| CE9-2.3a | When a CU contains more than 256 (luma samples, it is split into multiple sub-blocks with 256 luma samples and BDOF is performed for each sub-block. | H. Liu | I. Tomohiro |
| CE9-2.3b | SAD is calculated only for 4 corners (instead of all 16 luma samples) of each 4x4 sub-block. | H. Liu | I. Tomohiro |
| CE9-2.4 | Implicitly split BDOF application region along 16x16 boundaries | S. Esenlik | K. Kondo/X. Xiu |
| CE9-2.5 | Simplification of BDOF’s optical flow parameter derivation | LI Jingya | S. Sethuraman |
| CE9-2.6 | CE9.2.1 + CE9.2.4 | S. Esenlik | K. Kondo/X. Xiu |

# Time-line and Responsibilities

| T1 | February 8, 2019 | Finalize CE description |
| --- | --- | --- |
| T2 | February 26, 2019 | Deliver software packages and initial study by cross-checkers. Proponents can continue to modify the software until T3 |
| T3 | March 4, 2019 | Software is finalized. Proponents provide full results and final crosschecking begins. |
| T4 | 12 March 2019 | Contribution upload. CE9 summary report is uploaded. |

# References

[1]	“JVET common test conditions and software reference configurations for SDR”, Document: JVET-M0449, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring