| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M1032-v2 |
| --- | --- |

| Title: | Description of Core Experiment 12 (CE12): Tile Set Boundary Handling |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Hendry Robert Skupin Wade Wan | Tel: Email: | fnu.hendry@huawei.com robert.skupin@hhi.fraunhofer.de wade.wan@broadcom.com |
| Source: | CE12 Coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to test impact of handling boundaries of tile group based independent coded regions to the compression performance of VVC. Techniques included in this CE are:

- Handling all tile group boundaries as picture boundaries which includes padding to the tile group boundaries and allowing motion vectors to point to samples outside the tile group boundaries in the reference pictures.

- Encoder-side restrictions of tile groups to not use samples or motion vector candidates from outside the tile group.

Participants in this activity are ETRI, Ericsson, HHI, Huawei, Intel, KDDI, Kwai, LG, MediaTek, Nokia, Panasonic, Qualcomm, Samsung, Sharp, and Sony, Tencent, TNO.

# Participants

| Nr. | Name | Company | Email |
| --- | --- | --- | --- |
| 1 | Peisong Chen | Broadcom | peisong.chen@broadcom.com |
| 2 | Wade Wan | Broadcom | wade.wan@broadcom.com |
| 3 | Minhua Zhou | Broadcom | minhua.zhou@broadcom.com |
| 4 | Li Zhang | Bytedance | lizhang.idm@bytedance.com |
| 5 | Gun Bang | ETRI | gbang@etri.re.kr |
| 6 | Jungwon Kang | ETRI | jungwon@etri.re.kr |
| 7 | Hahyun Lee | ETRI | hanilee@etri.re.kr |
| 8 | Rickard. Sjöberg | Ericsson | rickard.sjoberg@ericsson.com |
| 9 | Robert Skupin | HHI | robert.skupin@hhi.fraunhofer.de |
| 10 | Roman Chernyak | Huawei | chernyak.roman@huawei.com |
| 11 | Hendry | Huawei | fnu.hendry@huawei.com |
| 12 | Seungwook Hong | Huawei | seungwook.hong@huawei.com |
| 13 | Jill Boyce | Intel | jill.boyce@intel.com |
| 14 | Kei Kawamura | KDDI | ki-kawamura@kddi.com |
| 15 | Yi-Wen Chen | Kwai | yiwenchen@kwai.com |
| 16 | Tsung-Chuan Ma | Kwai | tsung-chuanma@kwai.com |
| 17 | Shawn Wang | Kwai | xianglinwang@kuaishou.com |
| 18 | Xiaoyu Xiu | Kwai | xiaoyuxiu@kwai.com |
| 19 | Seung Hwan Kim | LG | seunghwan3.kim@lge.com |
| 20 | Ling Li | LG | l.li@lge.com |
| 21 | Jaehyun Lim | LG | jaehyun.lim@lge.com |
| 22 | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com |
| 23 | Tzu-Der Chuang | MediaTek | peter.chuang@mediatek.com |
| 24 | Yu-Wen Huang | MediaTek | cw.hsu@mediatek.com |
| 25 | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com |
| 26 | Alireza Aminlou | Nokia | alireza.aminlou@nokia.com |
| 27 | Virginie Drugeon | Panasonic | virginie.drugeon@eu.panasonic.com |
| 28 | Tadamasa Toma | Panasonic | toma.tadamasa@jp.panasonic.com |
| 29 | Muhammed Coban | Qualcomm | mcoban@qti.qualcomm.com |
| 30 | Vadim Seregin | Qualcomm | vseregin@qti.qualcomm.com |
| 31 | Amith Dsouza | Samsung | amith.ds@samsung.com |
| 32 | Raj Narayana Gadde | Samsung | raj.gadde@samsung.com |
| 33 | Chirag Pujara | Samsung | chirag.p@samsung.com |
| 34 | Sachin Despande | Sharp | sdeshpande@sharplabs.com |
| 35 | Kiran Misra | Sharp | asegall@sharplabs.com |
| 36 | Andrew Segall | Sharp | asegall@sharplabs.com |
| 37 | Takeshi Tsukuba | Sony | takeshi.tsukuba@sony.com |
| 38 | Byeongdoo Choi | Tencent | bdchoi@tencent.com |
| 39 | Alexandre Gabriel | TNO | alexandre.dasilvapratasgabriel@tno.nl |
| 40 | Emmanuel Thomas | TNO | emmanuel.thomas@tno.nl |

# Test conditions and evaluation criteria

Tests in the CE shall be conducted and evaluated by using test conditions and evaluation procedure as described in JVET-M0870. In addition, the following shall apply to all tests in this CE:

All tiles are contained in individual tile groups, i.e. a single VCL NAL unit, and, vice versa, each tile group contains a single tile.

All tile group boundaries are coded in a way that allows independent decoding of tile groups.

# Tests specification

| Test# | Description | Document# |
| --- | --- | --- |
| CE12-1 | Padding tile group boundary and allow motion vectors to point to samples outside the boundaries in the reference pictures as if they are picture boundaries. In this case, each tile group is an independently decodable region and contains one tile. | JVET-M0136 JVET-M0261 JVET-M0530 |
| CE12-2 | Restrict motion vectors on encoder-side to not use sample values or motion vector candidates depending on samples or motion vectors outside the tile group boundaries. | JVET-M0445 |

# CE12-1: Handling independently decodable tile group boundaries as picture boundaries

The proposal is summarized as follows:

- High-level syntax to specify whether each tile group boundary of the tile group is treated the same as picture boundary in the decoding process. For example, a flag, tile_group_boundary_treated_as_pic_boundary_flag, could be added to the tile group header.

- Each tile group is an independenly decodable region and contains one tile.

- When the boundaries of a tile group are indicated to be treated as picture boundaries, the following applies:

- In the derivation process for temporal luma motion vector prediction, the right and bottom picture boundary positions used in the process, indicated by pic_height_in_luma_samples − 1 and pic_width_in_luma_samples − 1, respectively, are replaced with the right and the bottom boundary positions, respectively, of the tile group, in units of luma samples.

- In the luma sample interpolation process, the left, right, top, and bottom picture boundary positions used in the process, indicated by 0, pic_height_in_luma_samples − 1, 0, pic_width_in_luma_samples − 1, respectively, are replaced with the left, right, top, and bottom boundary positions, respectively, of the tile group, in units of luma samples, respectively.

- In the chroma sample interpolation process, the left, right, top, and bottom picture boundary positions used in the process, indicated by 0, pic_height_in_luma_samples / SubWidthC − 1, 0, pic_width_in_luma_samples / SubWidthC − 1, respectively, are replaced with the left, right, top, and bottom boundary positions, respectively, of the tile group, in units of chroma samples, respectively.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE12-1 | Alireza Aminlou alireza.aminlou@nokia.com Hendry fnu.hendry@huawei.com Muhammed Coban mcoban@qti.qualcomm.com JVET-M0136, JVET-M0261, JVET-M0530 | HHI |

# CE12-2: Encoder-side tile group restriction

Motion search is restricted on encoder-side to obey tile boundaries in the following way:

- No sample values outside each identified tile set (i.e., tile or tile group) or outside the picture are referenced for inter prediction.

- No usage of motion vector candidates derived from blocks outside each identified tile set .

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE12-2 | Robert Skupin robert.skupin@hhi.fraunhofer JVET-M0445 | Huawei Qualcomm |

# Expected Gains

It is expected that both tests would have coding penalty against anchor. It is further expected that test 12.1 would provide coding efficiency improvement over test 12.2.

# Time-line and Responsibilities

T1: 2019-February-08:	Final CE description uploaded

T2: 2019-February-25:	Cross-checking begins, proponents provide software

T3: 2019-March-04: 	Final version of CE software, full test results must be provided at this time.

Note: The above time-line may be revised depending on the availability of tiles implementation in the VVC reference software.