| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N_Notes_d2 |
| --- | --- |

| Title: | Meeting Report of the 14th Meeting of the Joint Video Experts Team (JVET), Geneva, CH, 19–27 March 2019 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chairs of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its fourteenth meeting during 19–27 March 2019 at the ITU-T premises in Geneva, CH. The JVET meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.13 of this document. It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of a new standard. The name Versatile Video Coding (VVC) was chosen in April 2018 as the informal nickname for the new standard.

The JVET meeting began at approximately 0900 hours on Tuesday 19 March 2019. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Wednesday 27 March 2019. Approximately XXX people attended the JVET meeting, and approximately XXXqq input documents and 19 AHG reports were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of developing video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or otherwise gives better support regarding the requirements of future application domains of video coding. As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the thirteenth JVET meeting in producing a fourth draft of the VVC standard and the fourth version of the associated VVC test model (VTM). Further important goals were reviewing the results of 13 Core Experiments (CE), reviewing other technical input on novel aspects of video coding technology, and producing the next versions of the VVC draft text and VTM, and plan next steps for further investigation of candidate technology towards the formal standard development.

The JVET produced 18 output documents from the meeting (update):

- JVET-M1001 Versatile Video Coding specification text (Draft 4)

- JVET-M1002 Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4)

- JVET-M1004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 9)

- JVET-M1006 Methodology and reporting template for coding tool testing and for neural network tool testing

- JVET-M1010, JVET-L1011, and JVET-L1012 JVET common test conditions and software reference configurations for SDR, HDR/WCG, and 360° video

- JVET-M1021 through JVET-M1033, Description of Core Experiments 1 through 13

For the organization and planning of its future work, the JVET established XX “ad hoc groups” (AHGs) to progress the work on particular subject areas. At this meeting, XX Core Experiments (CE) were defined. The next four JVET meetings were planned for 3–12 July 2019 under WG 11 auspices in Gothenburg, SE, during 1–9 October 2019 under ITU-T SG16 auspices in Geneva, CH, during 8–17 January 2020 under WG 11 auspices in Brussels, BE, and during 15–24 April 2020 under WG 11 auspices in Alpbach, AT.

The document distribution site http://phenix.it-sudparis.eu/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/SC 29/WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11 held its fourteenth meeting during 19–27 March 2019 at the ITU-T premises in Geneva, CH. The JVET meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of a new standard. The name Versatile Video Coding (VVC) was chosen in April 2018 as the informal nickname for the new standard.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2019_03_N_Geneva/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the thirteenth JVET meeting in producing a fourth draft of the VVC standard and the fourth version of the associated VVC test model (VTM). Further important goals were reviewing the results of 13 Core Experiments (CE), reviewing other technical input on novel aspects of video coding technology, and producing the next versions of draft text and VTM, and plan next steps for further investigation of candidate technology towards the formal standard development.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect the VTM or BMS software but have no normative effect are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in the VTM description (an error, oversight, or messiness) or in the software are marked by the string “Decision (BF):”.

This meeting report is based primarily on notes taken by the JVET chairs. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 12 March 2019. Any documents uploaded after 1159 hours Paris/Geneva time on Wednesday 13 March were considered “officially late”, giving a grace period of 12 hours to accommodate those living in different time zones of the world. The deadline does not apply to AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents.

It was remarked that various CE results documents (both proponent proposal documents and CE summary reports) were arriving late, and that it is especially important for CEs to be thoroughly studied. JVET-N0104 for CE3-3.5 was an example of this (for which the proponent said that a software bug causing a need to regenerate VTM anchors was a problem). The timing of availability of software and text for CEs was also discussed. One of the purposes of a cross-check is to determine whether the technical description and text are correct. Having an earlier deadline for CE proposal documents (not necessarily CE summary documents) was suggested. It was agreed to have a 2-week deadline for CE proposal documents, including draft text.

It was suggested to have CE description documents include a description of how the results are planned to be reported – e.g., the form of the tables to be used for the results data. Complexity analysis characterizations were suggested to be a particular issue where this applies.

All contribution documents with registration numbers higher than JVET-N0500 were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “M0501+” range might include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions. Also, many cross-check reports were uploaded late.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:qq

- JVET-N0XXX (a proposal on …), uploaded 03-XX.

- ….

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline. For example, some of them were proposing combinations or simplifications of other proposals.

The following other document not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-N0XXX (a document on …), uploaded 03-XX.

- ….

All cross-verification reports were both registered late and uploaded late (all with numbers higher than JVET-N0500) and therefore are not specifically identified here, in the interest of brevity. Initial upload times for each document are recorded in Annex A of this report.

The following (X) contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-N0XXX (withdrawn), … .

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then typically be counted as a late contribution). At the current meeting, this situation applied to the initial uploads of documents JVET-N0213 and JVET-N0366.

Contributions that had significant problems with uploaded versions included the following:

- JVET-N0XXX (…)

- …

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision although participants tend to seldom notice what is recorded there.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-M1000, the Versatile Video Coding specification text (Draft 4) JVET-M1001, the Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4) JVET-M1002, the Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 9 JVET-M1004, the Methodology and reporting template for neural network tool testing JVET-M1006, the JVET common test conditions and software reference configurations for SDR video JVET-M1010, and the Description of Core Experiments 1 through 13 (JVET-M1021 through JVET-M1033), had been completed and were approved. The software implementation of VTM (versions 4.0, 4.0.1 and 4.1), and the 360Lib software implementation (version 9.0) were also approved.

The group was initially asked to review the meeting report of the previous meeting for finalization. The meeting report was later approved without modification.

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

## Agenda

The agenda for the meeting was as follows:

- Opening remarks and review of meeting logistics and communication practices

- IPR policy reminder and declarations

- Contribution document allocation

- Review of results of the previous meeting

- Reports of ad hoc group (AHG) activities

- Reports of core experiments planned at the previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of additional video coding technology contributions

- Consideration of information contributions

- Coordination activities

- Approval of output documents and associated editing periods

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director’s AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

“TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation.”

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM software implementation package uses the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the VTM software, and for evaluating proposals for technology to be potentially included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and and HDRtools as well.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jvet/.

It was reminded to send a notice to the chairs in cases of changes to document titles, authors etc.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers was 1129.

For distribution of test sequences, a password-protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI. Accredited members of JVET may contact the responsible JVET coordinators to obtain the password information (but the site is not open for use by others).

## Terminology

Some terminology used in this report is explained below:

(check for completeness with JVET-N0013, and draft text)

- ACT: Adaptive colour transform.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- AMT or MTS: Adaptive multi-core transform, or multiple transform set.

- AMVR: (Locally) adaptive motion vector resolution.

- APS: Adaptation parameter set.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- ARSS: Adaptive reference sample smoothing.

- ATMVP or “subblock-based temporal merging candidates” : Alternative temporal motion vector prediction.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BDOF: Bi-directional optical flow (formerly known as BIO).

- BL: Base layer.

- BMS: Bench-mark set, a compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardzation.

- BoG: Break-out group.

- BCW: Bi-prediction with CU based weighting

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CE: Core Experiment – a coordinated experiment conducted toward assessment of coding technology.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CIIP: Combined Inter/Intra prediction.

- CL-RAS: Cross-layer random-access skip.

- CPMV: Control-point motion vector.

- CPMVP: Control-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DMVR: Decoder-side motion vector refinement.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EMT: Explicit multiple-core transform.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ECV: Extended Colour Volume (up to WCG).

- EL: Enhancement layer.

- ET: Encoding time.

- FRUC: Frame rate up conversion (pattern matched motion vector derivation).

- GBI: …

- HDR: High dynamic range.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HMVP: History based motion vector prediction.

- HyGT: Hyper-cube Givens transform (a type of NSST).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- ISP: Intra subblock partitioning

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LIC: Local illumination compensation.

- LM: Linear model.

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MC: Motion compensation.

- MCP: Motion compensated prediction.

- MDNSST: Mode dependent non-separable secondary transform.

- MMLM: Multi-model (cross component) linear mode.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MPM: Most probable mode (in intra prediction).

- MV: Motion vector.

- MVD: Motion vector difference.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- PDPC: Position dependent (intra) prediction combination.

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- BT: Binary tree.

- TT: Ternary tree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SBT: Subblock transform.

- SbTMVP: Subblock based temporal motion vector prediction.

- SD: Slice data; alternatively, standard-definition.

- SDT: Signal-dependent transform.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SMVD: Symmetric MVD.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- TPM: Triangular partitioning mode

- UCBDS: Unrestricted center-biased diamond search.

- UWP: Unequal weight prediction.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- VTM: VVC Test Model.

- VVC: Versatile Video Coding, the standardization project developed by JVET.

- WAIP: …

- WCG: Wide colour gamut.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size as a CU in the VVC context.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size as a CU in the VVC context.

## Opening remarks

Remarks during the opening session of the meeting 0900 Tuesday 19 March (chaired by GJS and JRO) were as follows.

- The meeting logistics, agenda, working practices, policies, and document allocation were reviewed.

- The results of the previous meeting were reviewed.

- On placeholders – there were a number of cases where there was some description of a concept but no test results (see section 2.4.2).

- The primary goals of the meeting were to review the results of CEs, identify promising technology directions, and adopt proposed technology into the VVC draft text and VTM.

- Due to the high number of input contributions, parallelization and breakout work were planned to be used at the meeting.

- Planning of viewing & equipment setup [1 JVET Viewing room and 1 MPEG Viewing room all in -1 of Montbrillant, M04B and MO3B and possibly a 3rd room in the -2 of the Tower if needed]

- Principles of standards development were discussed.

- Standardization timelines [CD July, DIS October (possibly Jan), Consent & FDIS July; or CD October, DIS Jan, Consent & FDIS July]

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0900–2100+ hours, with coffee and lunch breaks as convenient. Ongoing scheduling refinements were announced on the group email reflector as needed. Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Tue. 19 March, 1st day

  - 0900–1230, 1430-1700 Opening plenary in CICG 1 (chaired by GJS & JRO)

  - 1730 Room A HLS BoG in Room A (JB, starting with 6.18.2 general HLS category)

  - 1730-2200 Track B CE2 Subblock MC in Room C (GJS)

  - 1730-1830, 1845-2200 Track A CEs in CICG 1, then Popov

- Wed. 20 March, 2nd day

  - Track A in Room 1 CICG: 900-1345 CE3,

  - Track A in Room 1 CICG: 1500-1815 CE6, CE7

  - Track A in Room C ITU-T: 1830-2215 CE8

  - Track B in Room K

    - 0900-1100 CE2 Subblock MC

    - 1115-1300, 1430-1640 CE4 Inter pred and MV coding

    - 1645-1900 CE9 Decoder MV derivation

  - 0900-1300 HLS BoG in Room 5

  - 1430 Coded picture regions BoG in Room 5

  - 1530 Intra BoG in Room 13

  - 1900- CE2-related BoG in Room K (Y. He and C.-C. Chen)

  - 1500-??? BoG on CE3 and related (G. v.d.Auwera)

- Thu

  - Track B in Room K

    - 0900 CE10 Combined intra/inter prediction

    - 1000 CE12 Tile set boundary motion comp handling

  - 1430-1545 VCEG (Q6/16) in

- TBD CE4-related BoG (H. Yang)

- TBD CE9-related BoG (S. Esenlik)

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks, and may not be completely accurate):

- AHG reports (19) (section 3) (Plenary)

- Project development (X) (section 4) (Plenary)

  - Text and software development (5)

  - Test conditions (5)

  - Peformance assessment (1)

  - Coding studies on specific use cases (14)

    - Adaptive resolution conversion (6)

    - 360° video coding (4)

    - Chroma formats (4)

  - Test Material (2)

- Core Experiments (xx) (section 5) with subtopics

  - CE1: Post prediction and post reconstruction filtering (9) (section 5.1) (Track A)

  - CE2: Subblock motion compensation (20) (section 5.2) (Track B)

  - CE3: Intra prediction and mode coding (19) (section 5.3) (Track A)

  - CE4: Inter prediction and motion vector coding (18) (section 5.4) (Track B)

  - CE5: Adaptive loop filtering (4) (section 5.5) (Track A)

  - CE6: Transforms and transform signalling (4) (section 5.6) (Track A)

  - CE7: Quantization and coefficient coding (4) (section 5.7) (Track A)

  - CE8: Screen content coding tools (9) (section 5.8) (Track A)

  - CE9: Decoder motion vector derivation (7) (section 5.9) (Track B)

  - CE10: Combined intra/inter prediction (2) (section 5.10) (Track B)

  - CE11: Deblocking (4) (section 5.11) (Track A)

  - CE12: Tile set boundary motion comp handling (2) (section 5.12) (Track B)

  - CE13: Neural network based loop filtering (5) (section 5.13) (Track A)

- Non-CE technology proposals (xx) (section 6) with subtopics

  - CE1 related – Post prediction and post reconstruction filtering (10) (section 6.1) (Track A)

  - CE2 related – Subblock motion compensation (17) (section 6.2) (Track B)

  - CE3 related – Intra prediction and mode coding (54) (section 6.3) (Track A)

  - CE4 related – Inter prediction and motion vector coding (40) (section 6.4) (Track B)

  - CE5 related – Adaptive loop filtering (3) (section 6.5) (Track A)

  - CE6 related – Transforms and transform signalling (26) (section 6.6) (Track A)

  - CE7 related – Quantization and coefficient coding (11) (section 6.7) (Track A)

  - CE8 related – Screen content coding tools (40) (section 6.8) (Track A)

  - CE9 related – Decoder motion vector derivation (33) (section 6.9) (Track B)

  - CE10 related – Combined intra/inter prediction (3) (section 6.10) (Track B)

  - CE11 related – Deblocking (3) (section 6.11) (Track A)

  - CE12 related – Tile set boundary motion comp handling (3) (section 6.12) (Track B)

  - CE13 related – Neural network based loop filtering (1) (section 6.13) (Track A)

  - Quantization (3) (section 6.14) (Track A)

  - Entropy coding (9) (section 6.15) (Track A)

  - Luma mapping with chroma scaling (9) (section 6.16) (Track A)

  - Other coding tools (2) (section 6.17) (Track A)

  - High-level syntax (78) (section 6.18) (Track B)

- Complexity analysis and reduction (2) (section 7) (Track A)

- Encoder optimization (2) (section 8) (Track A)

- Metrics and evaluation criteria (0) (section 9) (Track none)

- Withdrawn (6) (Track none)

- Joint meetings, plenary discussions, BoG reports, Summary of actions (section 10)

- Project planning (section 12)

- Establishment of AHGs (section 13)

- Output documents (section 14)

- Future meeting plans and concluding remarks (section 15)

The document counts above do not include cross-checks and CE summary reports.

Track A (58+174) was generally chaired by JRO, and Track B (49+173) by GJS.

# AHG reports (19)

These reports were discussed Tuesday 19 March XXXX–XXXX (chaired by GJS and JRO).

JVET-N0001 JVET AHG report: Project management (AHG1) [J.-R. Ohm, G. J. Sullivan]

This document reports on the work of the JVET ad hoc group on Project Management, including an overall status report on the VVC standardization project and the progress made during the interim period since the preceding meeting.

The work of the JVET overall had proceeded well in the interim period with a huge number of input documents submitted to the current meeting. Intense discussion had been carried out on the group email reflector, and all output documents from the preceding meeting had been produced.

Output documents from the preceding meeting had been made available at the "Phenix" site (http://phenix.it-sudparis.eu/jvet/) or the ITU-based JVET site (http://wftp3.itu.int/av-arch/jvet-site/2019_01_ M_Marrakech/ ), particularly including the following:

- The meeting report (JVET-M1000) [Posted 2019-03-18]

- Versatile Video Coding (Draft 4) (JVET-M1001) [Posted 2019-02-01, last update 2019-03-17]

- Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4) (JVET-M1002) [Posted 2019-02-16]

- Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 9 (JVET-M1004) [Posted 2019-02-17]

- Methodology and reporting template for coding tool testing (JVET-M1005) [Posted 2019-01-30]

- Methodology and reporting template for neural network coding tool testing (JVET-M1006) [Posted 2019-01-30]

- JVET common test conditions and software reference configurations (JVET-M1010) [Posted 2019-02-07]

- Description of CE 1..13 (JVET-M1021..35) [all first posted 2019-01-18, further updates during the CE definition period of 3 weeks after the meeting, until 2019-02-08]. The following CE description documents had later updates (more than 4 weeks after the meeting):

    - JVET-M1022 [last updated 2019-03-15]

    - JVET-L1023 [last updated 2019-03-14]

    - JVET-L1024 [last updated 2019-03-17]

    - JVET-L1026 [last updated 2019-03-14]

    - JVET-L1028 [last updated 2019-02-28]

    - JVET-L1029 [last updated 2019-03-14]

    - JVET-L1030 [last updated 2019-03-17]

The nineteen ad hoc groups had made progress, and reports from those activities had been submitted.

Software integration of VTM was finalized approximately according to the plan.

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system. That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful.

Roughly 600 input contributions to the current meeting (not counting the AHG and CE summary reports) had been registered for consideration at the meeting. Though topics of Core Experiments and related documents for the development of low-level coding tools reflect the bulk of these documents, around 70 documents were submitted on aspects of high-level syntax, including tile partitioning.

A preliminary basis for the document subject allocation and meeting notes for the 14th meeting had been made publicly available on the ITU-hosted ftp site.

Discussed aspects:

draft text should be marked as draft 4

draft text to be updated

another version of VTM

CE4 and CE9 had some clarifications

78 on HLS

JVET-N0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, J. Chen, J. Boyce, S. Kim, S. Liu, Y. Ye]

This document reports the work of the JVET ad hoc group on draft text and test model algorithm description editing (AHG2) between the 13th meeting in Marrakech, MA (9–18 January 2019) and the 14th meeting in Geneva, CH, (19–27 March 2019).

At the 13th JVET meeting, it was decided to include additional coding features for intra picture-prediction, inter-picture prediction, transform, CABAC engine and de-blocking filter in the fourth draft of Versatile Video Coding (VVC D4) and the VVC Test Model 4 (VTM4) encoding. Draft reference software to implement the VVC decoding process and VTM3 encoding method has also been developed.

The normative decoding process for Versatile Video Coding is specified in the VVC draft 4 text specification document. The VVC Test Model 4 (VTM 4) Algorithm and Encoder Description document provides an algorithm description as well as an encoder-side description of the VVC Test Model 4, which serves as a tutorial for the algorithm and encoding model implemented in the VTM4.0 software.

An issue tracker (https://jvet.hhi.fraunhofer.de/trac/vvc) was used to facilitate the reporting of errata with the VVC documents.

JVET-M1001 VVC specification (Draft 4)

Seven versions of JVET-M1001 were published by the Editing AHG between the 13th meeting in Marrakech, MA (9–18 January 2019) and the 14th meeting in Geneva, CH, (19–27 March 2019).

JVET-M1001 has been established based on JVET-L1001 and now contains the following:

- Incorporated JVET-M0102: Intra subpartitions (ISP).

- Incorporated JVET-M0142: chroma format dependent CCLM downsampling filter.

- Incorporated JVET-M0064: table reduction in CCLM model parameter calculation.

- Incorporated JVET-M0092: intra reference sample filtering cleanup.

- Incorporated JVET-M0238: PDPC linear interpolation on the secondary boundary for adjacent angular modes is changed to nearest neighbour.

- Incorporated JVET-M0407: CPR search range.

- Fixed bug #154 Availability check for CPR/IBC chroma CU reference block is missing.

- Aligned PDPC filtering for INTRA_ANGULAR18 and INTRA_ANGULAR50 to VTM.

- Fixed bug #167 on PDPC size condition.

- Incorporated JVET-M0497: Fast DST-7/DCT-8.

- Incorporated JVET-M0303: Shape adaptive transform selection.

- Incorporated JVET-M0464/JVET-M0201: Combined transform skip(TS) and MTS syntax plus extended TS sizes.

- Incorporated JVET-M0297: Zero-out of last 16 samples for 32 samples DST-7/DCT-8.

- Incorporated JVET-M0140: Subblock transform for inter CUs.

- Incorporated JVET-M0251/M0257: Zero-out of last 32 samples for 64 samples DCT-2 fix.

- Incorporated JVET-M0273/M0240/M0116/M0338/M0204: only using left neighbor for SbTMVP fetching

- Incorporated JVET-M0246: AMVR for affine

- Incorporated JVET-M0145: affine sub-block MV clipping

- Incorporated JVET-M0166/M0228/M0477: remove MV comparison for constructed merge candidates

- Incorporated JVET-M0170: Parallel processing for merge mode

- Incorporated JVET-M0147: Decoder side motion vector refinement

- Incorporated JVET-M0361: fix of cu_cbf for merge mode

- Incorporated JVET-M0487: using integer samples instead of bilinear interpolation for extended region of BDOF.

- Incorporated JVET-M0483: IBC signalled as a separate CU prediction mode.

- Incorporated JVET-M0063: Generalization of BDOF bit-depth.

- Incorporated triangular modifications including:

  - JVET-M0118/M0185/M0190/M207(test 1)/M0216(the first aspect)/M0234 (change corresponding to the result table 7 and 8)/M0317(section 2.2)/M0328: Do not signal the triangular prediction mode flag in cases where the combination is not allowed (MMVD, CIIP),

  - JVET-M0328: always use second weight group in triangular prediction.

- Incorporated JVET-M0883: signaling change of triangular merging candidate which does not need LUT.

- Incorporated JVET-M0193: pairwise average merging candidate reduction.

- Incorporated HMVP modifications including:

  - JVET-M0436 reduce HMVP number from 6 to 5,

  - JVET-M0300 HMVP initialization for parallel processing with tiles,

  - JVET-M0264 GBI weight is also stored in HMVP,

  - JVET-M0126 reduced HMVP candidate pruning.

- Incorporated JVET-M0255: MMVD mode without fractional sample offsets for screen content coding.

- Incorporated JVET-M0171/M0068: remove redundant MV scaling in MMVD.

- Incorporated JVET-M0444: symmetrical MVD coding for L0 to L1.

- Incorporated JVET-M0479: MV clipping to 18 bits.

- Incorporated JVET-M0512: TMVP storage reduction.

- Incorporated JVET-M0192: subblock chroma MV derivation for affine from two luma MVs.

- Incorporated JVET-M0111: weighted prediction (WP) and disable GBI signalling if WP is enabled.

- Incorporated JVET-M0281/M0117: modified AMVP pruning with rounding.

- Fixed bug #175 incorrect derivation of CCLM parameter b.

- Incorporated JVET-M0128: Reference picture management

- Incorporated JVET-M0132: Adaptation parameter set (APS)

- Incorporated JVET-M0853: Adding the support of rectangular tile groups in addition to the existing raster-scan tile groups, and enabling extraction of MCTSs without changing VLC NAL units

- Incorporated JVET-M0160: Adding loop_filter_across_tile_group_enabled_flag to the PPS

- Incorporated JVET-M0101:

  - Replace the existing IRAP_NUT with 3 new NAL unit types: IDR_W_RADL, IDR_N_LP, CRA_NUT (from JVET-M0101).

  - Add external means flag HandleCraAsCvsStartFlag, with similar text as in HEVC. Text provided in a v3 of JVET-M0101.

  - Add a NUT value for step-wise temporal access STSA (from JVET-M0101).

  - Add a NUT value for AUD (from JVET-M0101).

  - Add sps_max_sub_layers_minus1 syntax element to SPS, and decoding process in 8.1.1, 8.1.2 and 8.1.3 of JVET-M0101.

  - Add text of sections 7.4.2.4 to 7.4.2.4.5 on NAL unit order and AU boundary detection from JVET-M0101, which is primarily editorial, but has some technical aspects.

  - Add profile_tier_level( ) syntax structure which includes sub-layer level idc (similar to HEVC but without sub-layer-specific profiles).

  - Add general_non_packed_constraint_flag with semantics as in JVET-M0101 (rename the flag to display_suitability_flag? – that's editorial).

  - Add the temporal scalability sub-bitstream extraction process in JVET-M0101.

  - Add RASL and RADL NUTs

- Incorporated JVET-M0451: Add new constraint flags corresponding to VVC WD 3 tools.

- Incorporated JVET-M0415: Change the sps_ref_wraparound_offset to sps_ref_wraparound_offset_minus1 and changing the units to be MinCbSizeY as in option 1 (minor cleanup).

- Incorporated JVET-M0381: Reduce merge idx ctx coded bins (test 2.2.2a).

- Incorporated JVET-M0502: Add one context for pred_mode_flag (method 2).

- Incorporated JVET-M0453: Modified CABAC probability estimation (5.1.13* + init from 5.1.2).

- Fixed bug #147 on coefficient coding.

- Incorporated JVET-M0470: Limited EGk for abs_rem/ dec_abs_level.

- Incorporated JVET-M0173: Move rem_abs_gt3_flag into first coding pass.

- Incorporated JVET-M0119: Modified dequantization scaling for TS.

- Incorporated JVET-M0685: QP prediction fix for parallel encoding.

- Incorporated JVET-M0113/M0188: Bug fix for quantization group QP signalling.

- Incorporated JVET-M0421: Split-first signalling for partitioning.

- Incorporated JVET-M0446/M0888/M0905: Inferred QT split to avoid 32x128/128x32 partitions at picture boundaries.

- Incorporated JVET-M0427: Picture reconstruction with luma mapping and chroma scaling (LMCS).

The following adoptions have not yet been integrated:

- Incorporated JVET-M0908: Deblocking of CIIP boundaries.

- Incorporated JVET-M0471: Deblocking with long tap filters (also alignment with SBT).

- Incorporated JVET-M0277: Apply pcm_loop_filter_disabled_flag for ALF.

Integration Issues

The following items have been discussed within the AHG:

- Deblocking over ISP internal TU boundaries: Two different approaches have been identified resulting from interaction between JVET-M0102 Intra Subpartitions (ISP) and JVET-M0471 long deblocking:

  - Do not deblock (originally proposed and adopted ISP design)

  - Deblock all internal TU boundaries, i.e. for Subblock Transforms (SBT) and ISP.

The current draft and VTM-4.0.1 incorporates 1. However, it needs to be discussed which solution is preferred. The following document was identified to be related:

JVET-N0473 Non-CE11: On ISP transform boundary deblocking [K. Misra, A. Segall (Sharp Labs of America), M. Ikeda, T. Suzuki (Sony)]

- Last position coding fix for large size transform zero-out:  JVET-M0251 and JVET-M0257 have been adopted to change the following:

  - Last x/y position is coded using the reduced non-zero size, e.g. 32x32 for a 64x64 block (affects binarization)

  - The context derivation of last x/y position still depends on the larger transform size

- The current draft incorporates 1. and 2. while VTM-4.0.1 incorporates only 1. However, it needs to be discussed which solution is preferred. The following documents were identified to be related:

  - JVET-N0189 Non-CE7: Unified last position coding for 32-point transforms [Y. Piao, K. Choi (Samsung)]

  - JVET-N0194 CE6-related: Context selection of last non-zero coefficient position coding based on reduced TU size (related to JVET-M0297 and JVET-M0251/M0257)

- [Check revised AHG report about shared merge list for CPR; text had not been integrated yet]

Model 4 (VTM 4) Algorithm and Encoder Description

Two versions of JVET-M1002 was published by the Editing AHG between the 13th meeting in Marrakech, MA (9–18 January 2019) and the 14th meeting in Geneva, CH, (19–27 March 2019).

JVET-M1002 has been established based on JVET-L1002. It provides the algorithm description for majority of coding tools in VVC. In this editing period, the following changes were included:

- Incorporated JVET-M0427: luma mapping with chroma scaling (previously known as adaptive in-loop reshaper)

- Incorporated JVET-M0102: Intra subpartitions (ISP)

- Incorporated JVET-M0147: Decoder side motion vector refinement

- Incorporated JVET-0471: Long Deblocking

- Incorporated JVET-0483: Intra block copy

- Incorporated JVET-M0453: CABAC core engine

- Incorporated JVET-M0118, JVET-M0328 and JVET-M0883: triangle prediction related changes

- Incorporated JVET-M0487 and JVET-M0063: BDOF related changes

- Incorporated JVET-M0273: SbTMVP related changes

- Incorporated JVET-M0111: BWA related changes:

- Incorporated JVET-M0142: Alternative CCLM downsampling filter

- Incorporated JVET-M0064: Reduced table size of CCLM parameter derivation

- Incorporated JVET-M0238: Simplification of PDPC reference samples

- Incorporated JVET-M0407: IBC reference region modification

- Incorporated JVET-M0297: 32-length DST-7/DCT-8 using zero-out

- Incorporated JVET-M0464: Unified MTS and transform skip syntax

- Incorporated JVET-M0173: rem_abs_gt3_flag in first coding pass

- Incorporated JVET-M0246: Affine AMVR

In this editing period, the descriptions of most new coding tools from Marrakech meeting have been included in the latest release. Refinements of some existing tools in VTM3 have not been integrated. General encoder description, High Level Syntax, tiling mechanism and miscellaneous small coding features (such PCM mode and Delta QP signaling) are still missing at this moment. The VTM editors will work on those aspects in the following meeting circles.

Recommendations

The AHG recommended to:

- Approve the edited JVET-M1001 and JVET-M1002 documents as JVET outputs,

- Continue to edit the VVC draft and Test Model documents to ensure that all agreed elements of VVC are fully described,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification draft and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC WD and Test Model documents,

- Ensure that, when considering the addition of new feature to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC Working Draft is made available in a timely manner.

JVET-N0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, A. Norkin, K. Sührung]

This report summarizes the activities of the AhG3 on test model software development that has taken place between the 13th and 14th JVET meetings.

The mandates given to the AHG are:

- Coordinate development of test model (VTM) software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Discuss and make recommendations on the software development process.

- Propose improvements to the guideline document for developments of the test model software.

- Perform tests of VTM 4 behaviour relative to HEVC and VTM 3 using the VTM common test conditions and the multi-resolution streaming test conditions described in JVET-M0466.

- Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate.

- Coordinate with AHG6 for integration with 360lib software.

The software development continued on the GitLab server. VTM version 3.2 was tagged on Jan. 19., VTM 4.0 on Feb. 12. and VTM 4.0.1 on Feb. 26. VTM 4.1 is expected during the 14th JVET meeting.

For core experiments (CEs) the same development workflow was followed as for the last meeting.

VTM software development

Development was continued on the GitLab server, which allows participants to register accounts and use a distributed development workflow based on git.

The server is located at:

https://vcgit.hhi.fraunhofer.de

The registration and development workflow is documented at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/VVC-Software-Development-Workflow

The VTM software can be found at

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

VTM 3.2 was tagged on Jan 19, 2019.

- Changes relate to VTM 3.1 include:

- Removed JVET-Lxxx macros

- Fix for CU results reuse with dual tree (~7% encoder run time reduction for AI)

- Fix for block statistics generation

After one release candidates, VTM 4.0 was tagged on Feb. 12, 2019.

Changes related to VTM 3.2 include:

- JVET-M0453: modified CABAC core engine

- JVET-M0090: change default C[b,r]QpOffsetDualTree to 0, change C[b,r]QpOffset to 1 for CTC

- JVET-M0421: Split-first signalling for partitioning

- JVET-M0464: UniMTS

- JVET-M0479: 18-bit Mv clip

- JVET-M0502: context for prediction mode flag

- JVET-M0407: CPR range extension

- JVET-M0173: moving gtr2 flag to first pass

- JVET-M0142: CCLM collocated- hroma

- JVET-M0068/M0171: MMVD cleanup

- JVET-M0064: CCLM Simplification

- JVET-M0444: SMVD

- JVET-M0446/M0888/M0905: VPDU constraint at picture boundary

- JVET-M0487: integer positions in extended region

- JVET-M0497: Fast DST-7/DCT-8 with dual implementation support

- JVET-M0170: shared merging list candidate

- JVET-M0409: ATMVP fix when IBC is on

- JVET-M0118/M0185/etc: on merge_triangle_flag redudant signaling when a CU uses MMVD or CIIP

- JVET-M0381 Test CE2.2.2.a: one context for Merge_subblock_idx (AffMergeIdx in VTM software)

- JVET-M0145: Affine MV clip

- JVET-M0512: TMVP storage reduction using floating-point representation

- JVET-M0228/M0166/M0477: Remove CPMV comparisons for construnted affine merge candidates

- JVET-M0265: MV rounding cleanup

- JVET-M0255: MMVD without Fractional Distances for SCC

- JVET-M0273 (change 1)/JVET-M0240/JVET-M0116 (method 1)/JVET-M0338(method 1)/JVET-M0204(method 2):Only use left neighbor for ATMVP offset derivation

- JVET-M0264: Harmonization between HMVP and GBi

- JVET-M0063: BDOF bitDepth bugfix

- JVET-M0328: Only keep the second weight group during TPM MC

- JVET-M0192: modifications - affine chroma MV is average of two luma sub-block MVs

- JVET-M0193: allow only the first pairwise candidate, remove the remaining five

- JVET-M0883: Using regular merge index signaling for triangle mode

- JVET-M0257: Scan only non zero-out regions of large TUs (width or height > 32)

- JVET-M0312/M0255: disable fractional MVD for UHD sequences

- JVET-M0823: encoder optimization for MMVD

- JVET-M0246: adaptive MV precision for affine inter mode

- JVET-M0470: Fixed transition point between GR and TU + EG(k) escape codes for coefficient

- JVET-M0839: increase number of SATD candidates in Affine Merge from 1 to 2

- JVET-M0238: Nearest neighbor instead of linear interpolation in PDPC

- JVET-M0281 test a: perform all AMVP rounding before pruning even if AMVR is off

- JVET-M0436: Reduce HMVP table size

- JVET-M0117: On MVP candidate list generation for AMVP

- JVET-M0247: encoder optimization of affine AMVR

- JVET-M0471: Long deblocking filters

- JVET-M0908: CIIP deblocking

- JVET-M0303: implicit MTS

- JVET-M0427: Picture reconstruction with mapping

- JVET-M0119: remove scaling of ts coefficients

- JVET-M0253: Hash-based motion search

- JVET-M0147: DMVR (Decoder-side Motion Vector Refinement)

- JVET-M0102: Intra Sub-Partitions (ISP)

- JVET-M0126: HMVP merge pruing

- JVET-M0483: IBC

- JVET-M0297: skipping high frequency coefficients in wide/high blocks

- JVET-M0140 Sub-block Transform (SBT) for inter blocks

- JVET-M0854: MMVD precision for UHD (encoder only)

- JVET-M0055: VTM transcoding capabilities/DebugCTU

- removal of SPSNext / SPS alignment with draft text

- CABAC states init retraining

- configuration file for class F

- various bug fixes and cleanups

- software manual updates

VTM 4.0.1 was tagged on Feb. 26, 2019. Changes include:

- Bug fixes

- JVET-M0277: PCM and loop filter

- JVET-M0428: Deblocking optimization

- JVET-M0451: Interoperability syntax

- JVET-M0600: Rate control

- JVET-M0102: encoder speedup for ISP

- Macro to disable MMVD

It should be noted that version 4.0.1 addresses an issue where sequential and parallel encoding modes for the random access (RA) configuration were yielding different results.

VTM 4.1 is expected to be tagged during the 14th JVET meeting. Changes include:

- JVET-M0132: APS implementation

- JVET-M0685: QP parameter predictor derivation

- JVET-M0113/M0188 quantization groups based on area

- JVET-M0111: disable GBI signalling when WP is enabled

- JVET-M0864: 2D cache model

- JVET M0445 MCTS

- JVET-M0091: sub-CTU QPA for <=HD sequences, share mean-value calculations via computeAvg( )

- SPS/SPSNext cleanup

- various bug fixes and cleanups

At the beginning of the 14th meeting, some high level syntax implementations are still pending:

- JVET-L0686 Spec text for the agreed starting point on slicing and tiling (may be superseded by JVET-M0853)

- JVET-M0101: On VVC HLS (merge request submitted)

- JVET-M0415: horizontal reference wrap-around signaling (merge request submitted)

- JVET-M0853: tile groups PPS Signalling implementation (merge request for partial implementation submitted)

Common test conditions performance

The following table shows VTM 4.0 performance over HM 16.20:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -23.74% | -39.31% | -36.64% | 1375% | 170% |
| Class A2 | -22.96% | -25.04% | -19.85% | 2155% | 179% |
| Class B | -19.12% | -24.27% | -31.03% | 2341% | 179% |
| Class C | -19.52% | -21.31% | -24.54% | 3136% | 189% |
| Class E | -22.80% | -23.91% | -27.15% | 1860% | 174% |
| Overall | -21.23% | -26.19% | -28.01% | 2170% | 179% |
| Class D | -16.30% | -18.23% | -19.63% | 3471% | 184% |
| Class F | -33.58% | -36.53% | -38.27% | 3251% | 183% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -31.64% | -43.55% | -45.48% | 717% | 174% |
| Class A2 | -36.35% | -40.36% | -35.20% | 800% | 170% |
| Class B | -30.96% | -42.07% | -42.78% | 780% | 161% |
| Class C | -26.08% | -31.79% | -33.50% | 941% | 173% |
| Class E |  |  |  |  |  |
| Overall | -30.87% | -39.28% | -39.33% | 810% | 169% |
| Class D | -25.20% | -29.25% | -30.34% | 989% | 176% |
| Class F | -35.75% | -40.34% | -41.42% | 498% | 137% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over HM-16.20 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -24.33% | -33.19% | -32.22% | 745% | 150% |
| Class C | -21.17% | -27.80% | -28.90% | 853% | 156% |
| Class E | -24.36% | -30.57% | -33.01% | 382% | 119% |
| Overall | -23.29% | -30.74% | -31.31% | 660% | 143% |
| Class D | -21.07% | -24.72% | -25.38% | 843% | 159% |
| Class F | -31.86% | -38.71% | -39.89% | 421% | 117% |

The following table shows VTM 4.0 performance compared to VTM 3.2:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.66% | -3.82% | -4.59% | 132% | 96% |
| Class A2 | -2.65% | -2.97% | -2.75% | 126% | 103% |
| Class B | -2.18% | -1.75% | -2.17% | 124% | 104% |
| Class C | -2.40% | 1.58% | 1.21% | 124% | 117% |
| Class E | -2.45% | 1.57% | 1.27% | 130% | 111% |
| Overall | -2.43% | -1.01% | -1.35% | 126% | 107% |
| Class D | -2.18% | 0.09% | 0.08% | 117% | 115% |
| Class F | -18.38% | -14.24% | -14.34% | 211% | 111% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over VTM-3.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -5.43% | 6.44% | 3.69% | 147% | 115% |
| Class A2 | -6.83% | 5.50% | 6.18% | 156% | 119% |
| Class B | -5.04% | 5.89% | 5.33% | 153% | 124% |
| Class C | -5.16% | 7.10% | 7.61% | 157% | 127% |
| Class E |  |  |  |  |  |
| Overall | -5.51% | 6.25% | 5.78% | 153% | 122% |
| Class D | -4.90% | 6.63% | 6.93% | 170% | 126% |
| Class F | -17.37% | -9.37% | -9.08% | 164% | 126% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-3.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.15% | 19.69% | 20.48% | 151% | 115% |
| Class C | -4.19% | 14.54% | 15.70% | 157% | 131% |
| Class E | -1.95% | 9.93% | 12.19% | 142% | 124% |
| Overall | -3.61% | 15.53% | 16.81% | 151% | 122% |
| Class D | -4.48% | 15.65% | 18.69% | 168% | 121% |
| Class F | -13.29% | 0.70% | 1.13% | 151% | 125% |

The following table shows VTM 4.0 performance compared to VTM 3.2 under old CTC (no chroma QP adjustment):

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.66% | -3.82% | -4.59% | 132% | 97% |
| Class A2 | -2.66% | -3.00% | -2.77% | 126% | 102% |
| Class B | -2.18% | -1.77% | -2.21% | 124% | 105% |
| Class C | -2.40% | 1.58% | 1.21% | 124% | 117% |
| Class E | -2.45% | 1.57% | 1.27% | 130% | 113% |
| Overall | -2.43% | -1.02% | -1.36% | 126% | 107% |
| Class D | -2.17% | 0.09% | 0.09% | 117% | 114% |
| Class F | -18.38% | -14.24% | -14.33% | 211% | 112% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over VTM-3.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.07% | -2.33% | -3.32% | 148% | 115% |
| Class A2 | -5.27% | -3.04% | -2.29% | 157% | 119% |
| Class B | -4.54% | -3.22% | -2.95% | 154% | 121% |
| Class C | -4.60% | 0.20% | 0.51% | 157% | 127% |
| Class E |  |  |  |  |  |
| Overall | -4.61% | -2.09% | -1.97% | 154% | 121% |
| Class D | -4.39% | 0.41% | 0.73% | 170% | 125% |
| Class F | -16.87% | -12.89% | -12.71% | 165% | 127% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-3.2 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.31% | -1.45% | -0.41% | 152% | 116% |
| Class C | -3.20% | -0.79% | -0.20% | 156% | 129% |
| Class E | -1.48% | -0.78% | 0.28% | 143% | 125% |
| Overall | -2.82% | -1.06% | -0.17% | 151% | 122% |
| Class D | -3.65% | -1.46% | 0.11% | 169% | 122% |
| Class F | -12.22% | -8.78% | -8.63% | 151% | 123% |

The following table shows VTM 4.0.1 performance compared to VTM 4.0:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.01% | 0.07% | 97% | 102% |
| Class A2 | 0.03% | 0.11% | 0.02% | 97% | 101% |
| Class B | 0.03% | -0.01% | 0.06% | 97% | 102% |
| Class C | 0.04% | -0.03% | -0.03% | 96% | 102% |
| Class E | 0.05% | 0.05% | 0.01% | 96% | 103% |
| Overall | 0.04% | 0.02% | 0.03% | 97% | 102% |
| Class D | 0.02% | -0.02% | -0.13% | 96% | 101% |
| Class F | 0.07% | 0.08% | -0.01% | 97% | 103% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | -0.01% | -0.02% | -0.01% | 100% | 102% |
| Class B | 0.00% | -0.10% | -0.15% | 101% | 100% |
| Class C | -0.01% | -0.14% | -0.25% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.08% | -0.12% | 101% | 101% |
| Class D | 0.00% | -0.14% | 0.11% | 100% | 99% |
| Class F | 0.21% | 0.00% | 0.05% | 101% | 102% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.40% | 0.16% | 100% | 101% |
| Class C | -0.01% | 0.46% | 0.08% | 100% | 101% |
| Class E | 0.05% | 1.75% | 0.47% | 101% | 101% |
| Overall | 0.01% | 0.42% | 0.21% | 100% | 101% |
| Class D | -0.02% | 0.06% | 0.38% | 100% | 102% |
| Class F | 0.12% | -0.11% | -0.09% | 100% | 99% |

There is a notable performance drop for random access and low delay in class F. Bug fixes to address this issue have been submitted for inclusion in VTM 4.1.

Full results are attached to the AHG report as Excel files.

Multi-resolution streaming test conditions

An updated method is proposed in JVET-N0446. The contribution reports the following performance data. In the tables below, AS-RA and MR-RA denote test conditions typically used in adaptive streaming, when a source video may be downsampled prior to encoding at a lower bitrate if encoding downsampled video results in a better rate-distortion trade-off than encoding the original video at that bitrate. The choice of resolution-QP pairs in AS-RA is done once per sequence and is based on the anchor, with 4 resolution-QP points in total. The MR-RA corresponds to encoding the video at four spatial resolutions each at 4 QP (total of 16 points), then determining the convex hull for both anchor and test for the BD-rate calculations. The PSNR is calculated between the original source video and upsampled (if needed) reconstructed video. Both test conditions use closed-GOP encoding and 2s intra picture intervals.

BD-rate between VTM 4.0 and HM 16.19 using the JVET CTC, AS-RA and MR-RA test conditions for classes A1, A2 and B.

|  |  | JVET RA CTC |  |  | AS-RA |  |  | MR-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Seq. | Y [%] | U [%] | V [%] | Y [%] | U [%] | V [%] | Y [%] | U [%] | V [%] |
| Class A1 | Tango2 | -34.57 | -49.75 | -47.42 | -29.18 | -43.36 | -37.71 | -28.85 | -48.96 | -43.85 |
|  | FoodMarket4 | -34.05 | -32.94 | -35.92 | -31.05 | -31.39 | -30.98 | -30.14 | -35.07 | -35.31 |
|  | Campfire | -29.34 | -34.61 | -42.86 | -22.14 | -18.47 | -30.87 | -24.32 | -32.28 | -40.95 |
|  | Average | -32.65 | -39.10 | -42.07 | -27.46 | -31.07 | -33.19 | -27.77 | -38.77 | -40.03 |
| Class A2 | CatRobot1 | -40.26 | -46.15 | -39.99 | -40.38 | -45.56 | -40.64 | -39.68 | -45.03 | -39.94 |
|  | DaylightRoad2 | -40.51 | -41.81 | -33.47 | -39.96 | -46.72 | -36.52 | -39.91 | -44.96 | -37.16 |
|  | ParkRunning3 | -31.53 | -18.89 | -16.40 | -25.96 | -22.82 | -19.69 | -23.30 | -19.85 | -17.41 |
|  | Average | -37.43 | -35.62 | -29.95 | -35.44 | -38.37 | -32.28 | -34.29 | -36.61 | -31.50 |
| Class B | MarketPlace | -32.06 | -33.91 | -40.09 | -29.28 | -27.71 | -28.04 | -28.81 | -32.15 | -33.57 |
|  | RitualDance | -27.99 | -30.08 | -37.09 | -27.14 | -29.30 | -31.95 | -26.57 | -31.84 | -34.86 |
|  | Cactus | -32.93 | -39.15 | -31.87 | -36.29 | -44.90 | -41.35 | -32.50 | -39.60 | -34.96 |
|  | BasketballDrive | -32.50 | -41.11 | -41.53 | -35.29 | -43.09 | -39.45 | -29.14 | -40.47 | -37.55 |
|  | BQTerrace | -31.07 | -40.71 | -40.11 | -32.91 | -45.74 | -50.67 | -26.67 | -39.38 | -42.59 |
|  | Average | -31.31 | -36.99 | -38.14 | -32.18 | -38.15 | -38.29 | -28.74 | -36.69 | -36.71 |
| Overall Average |  | -33.35 | -37.19 | -36.98 | -31.78 | -36.28 | -35.26 | -29.99 | -37.24 | -36.19 |

BD-rate between VTM 4.0 and VTM 3.0 using the JVET CTC, AS-RA and MR-RA test conditions for classes A1, A2 and B.

|  |  | JVET RA CTC |  |  | AS-RA |  |  | MR-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Seq. | Y [%] | U [%] | V [%] | Y [%] | U [%] | V [%] | Y [%] | U [%] | V [%] |
| Class A1 | Tango2 | -5.95 | 8.82 | 6.48 | -4.79 | 6.25 | 4.11 | -4.86 | 4.23 | 3.59 |
|  | FoodMarket4 | -4.22 | 4.38 | 3.99 | -4.66 | 4.94 | 6.08 | -4.64 | 3.10 | 2.87 |
|  | Campfire | -6.15 | 6.08 | 0.67 | -1.52 | 1.40 | 4.95 | -3.77 | 5.06 | -0.17 |
|  | Average | -5.44 | 6.43 | 3.71 | -3.66 | 4.20 | 5.05 | -4.42 | 4.13 | 2.10 |
| Class A2 | CatRobot1 | -5.23 | 3.96 | 5.11 | -6.12 | 1.24 | 1.71 | -6.12 | 1.36 | 2.79 |
|  | DaylightRoad2 | -6.09 | 6.55 | 6.86 | -6.63 | 0.64 | 1.75 | -6.43 | 4.89 | 7.02 |
|  | ParkRunning3 | -9.24 | 6.16 | 6.74 | -6.06 | 2.47 | 2.64 | -6.66 | 4.87 | 5.49 |
|  | Average | -6.85 | 5.56 | 6.24 | -6.27 | 1.45 | 2.03 | -6.40 | 3.70 | 5.10 |
| Class B | MarketPlace | -7.33 | 3.58 | 1.94 | -3.71 | 16.74 | 16.56 | -1.78 | 10.99 | 8.51 |
|  | RitualDance | -4.76 | 4.04 | 4.90 | -5.28 | 10.17 | 12.15 | -3.09 | 6.19 | 8.36 |
|  | Cactus | -3.70 | 4.85 | 4.42 | -3.92 | 1.52 | 1.91 | -3.68 | 1.94 | 2.63 |
|  | BasketballDrive | -4.90 | 6.93 | 7.57 | -5.79 | 8.70 | 10.47 | -3.69 | 9.93 | 9.79 |
|  | BQTerrace | -4.60 | 10.99 | 8.22 | -4.39 | 7.67 | 4.84 | -5.40 | 5.01 | 6.65 |
|  | Average | -5.06 | 6.08 | 5.41 | -4.62 | 8.96 | 9.18 | -3.53 | 6.81 | 7.19 |
| Overall Average |  | -5.65 | 6.03 | 5.17 | -4.81 | 5.61 | 6.11 | -4.56 | 5.23 | 5.23 |

Differences in CTC between HM and VTM

A difference in coding conditions between HM and VTM was found as follows:

JCT-VC decided at some point that parameter sets should be repeated at every IRAP picture. With the joint call for proposals for VVC, this was not made a requirement. The VVC CTC were inherited from the CfP conditions. VTM did not have the ability to repeat parameter sets, while the HM CTC configuration files still used the parameter.

A configuration parameter for repetition of parameter sets was implemented on top of VTM 4.0 and included in VTM 4.0.1. The following table shows the performance impact of repeating parameter sets (AI and RA configurations):

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.06% | 0.06% | 100% | 101% |
| Class A2 | 0.03% | 0.03% | 0.03% | 100% | 101% |
| Class B | 0.10% | 0.09% | 0.10% | 100% | 100% |
| Class C | 0.26% | 0.23% | 0.24% | 100% | 99% |
| Class E | 0.34% | 0.30% | 0.31% | 100% | 99% |
| Overall | 0.16% | 0.14% | 0.15% | 100% | 100% |
| Class D | 0.76% | 0.67% | 0.69% | 100% | 97% |
| Class F | 0.26% | 0.29% | 0.19% | 101% | 100% |
|  |  |  | Random Access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.01% | 98% | 100% |
| Class A2 | 0.01% | 0.01% | 0.01% | 98% | 99% |
| Class B | 0.03% | 0.03% | 0.03% | 99% | 97% |
| Class C | 0.08% | 0.07% | 0.07% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.03% | 0.03% | 99% | 99% |
| Class D | 0.22% | 0.21% | 0.21% | 99% | 97% |
| Class F | 0.14% | 0.06% | -0.12% | 99% | 98% |

In the discussion, it was agreed to repeat the parameter sets at random access points in VTM testing.

Software manual

The software manual was updated to reflect general scope and compilation instructions. The encoding and decoding parameter description though still reflects HM, rather than VTM.

Although the software coordinators requested contributions to the software manual for changed parameters twice on the JVET email reflector, not a single input was received.

Update to Guidelines for VVC reference software development

A proposed update for the guidelines was uploaded as JVET-N0503.

CE software

For each CE a group was created in GitLab and CE coordinators were given owner rights to the group. This way they could clone VTM as required, create branches for different tests and assign user access to the group themselves.

The CE development workflow is described at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/Core-experiment-development-workflow

CE read access is available using shared accounts: One account exists for MPEG members, which uses the usual MPEG account data (as announced on the appropriate email lists). A second account exists for VCEG members. The account information for VCEG members is available in the TIES system:

https://www.itu.int/ifa/t/2017/sg16/exchange/wp3/q06/vceg_account.txt

Bug tracking

The bug tracker for VTM and specification text is located at:

https://jvet.hhi.fraunhofer.de/trac/vvc

The bug tracker uses the same accounts as the HM software bug tracker. Users may need to log in again due to the different sub-domain. For spam fighting reasons account registration is only possible at the HM software bug tracker at

https://hevc.hhi.fraunhofer.de/trac/hevc

Please file all issues related to the VVC reference software into the bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

Recommendations

The AHG recommended to:

- Continue to develop the VTM reference software

- Improve documentation, especially the software manual

- Resolve any normative issues resulting from the large number of integrations in the most recent development cycle

- Encourage people to test VTM software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bit-streams/test cases that trigger bugs in VTM.

- Encourage people to submit non-normative changes that reduce encoder run time without significantly sacrificing compression performance

- Review and approve updated guidelines for reference software development

JVET-N0004 JVET AHG report: Test material and visual assessment (AHG4) [T. Suzuki, V. Baroncini, R. Chernyak, P. Hanhart, A. Norkin, J. Ye]

The test sequences used for CfP/CTC are available on ftp://jvet@ftp.ient.rwth-aachen.de in directory “/jvet-cfp” (accredited members of JVET may contact the JVET chairs for login information).

Due to copyright restrictions, the JVET database of test sequences is only available to accredited members of JVET (i.e. members of ISO/IEC MPEG and ITU-T VCEG).

Not much activity was reported.

Regarding establishing a new structure at the test sequence repository.

There was not much progress since the last meeting. There was discussion in the last meeting that the current directory structure of test sequence ftp site is not good for the current activities. The ftp directory was created during the preparation of CfE/CfP and the same directory structure is still used. One possibility is to re-design the directory as follows, for example:

ctc/ : Contains the active test set of the common testing conditions

ahg/ : Contains subdirectories with sequences under consideration. The subfolder might be structured by meeting period (e.g. named by the doc-number of the corresponding meeting report?)

ce/  : Contains subdirectories for data exchange for specific CE (already implemented, see ce/JVET-{K,L}1031_Deblocking

upload : stays as before

During the last meeting, there was a comment that all sequences, all classes should be at the same place. But there is still meaningful to separate SDR, HDR, 360.

At the meeting, it was agreed that the coordinator has discretion over changes to the directory structure as needed.

The following related contributions were submitted.

- JVET-N0386 AOV YUV4:4:4 sequence for VVC standardization and suggestions for class F [X. Xu, J. Ye, X. Li, S. Liu, L. Wu, C. Xie, K. Liu, B. Wang, P. Liu, K. Dong, Y. Kuang, W. Feng (Tencent)]

- JVET-N0502 Five SCC TGM sequences of both YUV4:4:4 and YUV4:2:0 formats [T. Lin, K. Zhou, S. Wang (Tongji), L. Zhao (Shaoxing)] [late]

The AHG recommended:

- To review related contributions

- To discuss further ftp directory structure

- To continue to collect new test sequences available for JVET with licensing statement

JVET-N0005 JVET AHG report: Memory bandwidth consumption of coding tools (AHG5) [R. Hashimoto, T. Ikai, X. Li, D. Luo, H. Yang, M. Zhou]

The document summarizes activities of AHG on memory bandwidth consumption of coding tools between the 13th and the 14th JVET meetings.

Mandates:

- Develop improved software tools for measuring both average and worst case of memory bandwidth, and provide information for usage of these tools.

- Study cache configurations for measuring decoder memory bandwidth consumption.

- Identify coding tools in CEs and VTM with significant memory bandwidth impact.

- Study the impact of memory bandwidth on specific application cases.

Contributions to this meeting are as follows.

- JVET-N0068 “CE2: On restriction of memory bandwidth consumption of affine mode (CE2-4.8)”, M. Zhou (Broadcom)

- JVET-N0195 “CE2: Memory bandwidth reduction for the affine mode (Test 2.4.3)”, 	J. Li, C.-W. Kuo, C. S.Lim (Panasonic)

- JVET-N0199 “CE2-related: Combined test of affine memory bandwidth and complexity reduction (Test2.4.6+Test2.4.8)”, 	J. Li, C.-W. Kuo, C. S.Lim (Panasonic), M. Zhou (Broadcom)

- JVET-N0256 “CE2: Worst-case Memory Bandwidth Reduction for affine (Test 2.4.5)”, 	H. Huang, L. Pham Van, W.-J. Chien, M. Karczewicz (Qualcomm)

- JVET-N0323 “CE2: Reducing worst-case memory bandwidth of affine mode (CE2-4.2.a, CE2-4.2.b, CE2-4.2.c and CE2-4.2.d)”, 	Y.-W. Chen, X. Wang (Kwai Inc.), H. Chen, T. Solovyev, H. Yang, J. Chen (Huawei)

- JVET-N0369 “CE2-related: On restriction of memory bandwidth consumption of affine mode (2x2 variant)”, 	M. Zhou (Broadcom)

- JVET-N0398 “CE2-4.4: Affine block memory bandwidth reduction by MV clip”, 	X. Li, G. Li, X. Xu, S. Liu (Tencent)

- JVET-N0633 “CE2-related: on the memory reduction for affine motion inheritance”, H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)

- JVET-N0166 “Non-CE4: Advanced Multi-hypothesis Inter Prediction for bandwidth reduction in B frame”, Y. Sun, F. Chen, L. Wang (Hikvision)

- JVET-N0397 “AHG16: Memory bandwidth reduction for small CUs”, 	X. Li, G. Li, X. Xu, S. Liu (Tencent)

The AHG recommends to review all related contributions.

It is suggested to further improve the cache model. It is also suggested to perform further study on DMVR.

JVET-N0006 JVET AHG Report: 360 video conversion software development (AHG6) [Y. He, K. Choi]

The mandates of this AHG are as follows:

- Prepare and deliver the 360Lib-9.0 software version and common test condition configuration files according to JVET-L1012.

- Generate CTC VTM anchors according to JVET-L1012, and finalize the reporting template for the common test conditions.

- Produce documentation of software usage for distribution with the software.

Brief summary for the activities:

The 360Lib-9.0 software package included following changes:

Projection format conversion:

- Modified chroma sample location based on chroma sample location type in blending process for PHEC (JVET-M0368).

Configurations:

- Added MV wrap around in the encoding parameter settings for PERP.

Software:

- Fixed the bug between the interaction between DMVR and horizontal MV wrap-around used in PERP coding.

360Lib-9.0 related release:

360Lib-9.0 with support of VTM-4.0 was released on Mar. 4, 2019;

The 360Lib software is developed using a Subversion repository located at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

The released version of 360Lib-9.0 can be found at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-9.0/

360Lib-9.0 testing results can be found at:

ftp.ient.rwth-aachen.de/testresults/360Lib-9.0

360Lib bug tracker

https://hevc.hhi.fraunhofer.de/trac/jem/newticket?component=360Lib

Table 1 is for the projection formats comparison using VTM-4.0 according to 360o video CTC (JVET-L1012). It compares padded hybrid equi-angular cubemap (PHEC) coding and padded equi-rectangular projection (PERP) coding using VTM-4.0.

Table 2 is for PERP coding comparison between VTM-4.0 and HM-16.16. Table 3 is to compare VTM-4.0 with PHEC coding and HM-16.16 with CMP coding.

Table 1. VTM-4.0 PHEC vs PERP (VTM-4.0 PERP as anchor)

|  | PHEC over PERP (VTM-4.0) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -11.14% | -8.91% | -9.36% | -11.06% | -8.79% | -9.30% |
| Class S2 | -5.15% | -6.10% | -6.11% | -5.13% | -5.98% | -6.01% |
| Overall | -8.75% | -7.78% | -8.06% | -8.69% | -7.67% | -7.98% |

Table 2. VTM-4.0 PERP vs HM-16.16 PERP (HM-16.16 PERP as anchor)

|  | VTM-4.0 PERP - Over HM-16.16 PERP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -23.38% | -39.08% | -39.73% | -23.36% | -39.09% | -39.68% |
| Class S2 | -32.35% | -38.20% | -38.19% | -32.34% | -38.23% | -38.24% |
| Overall | -26.97% | -38.73% | -39.12% | -26.95% | -38.75% | -39.10% |

Table 3. VTM-4.0 PHEC vs HM-16.16 CMP (HM-16.16 CMP as anchor)

|  | VTM-4.0 PHEC - Over HM-16.16 CMP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -27.65% | -40.47% | -40.84% | -27.53% | -40.44% | -40.79% |
| Class S2 | -35.15% | -41.54% | -41.51% | -35.14% | -41.53% | -41.52% |
| Overall | -30.65% | -40.90% | -41.11% | -30.57% | -40.88% | -41.08% |

The AHG recommends:

- To integrate the JVET-M0452 (Hemisphere cubemap projection format).

- To continue software development of the 360Lib software package.

- To generate CTC VTM anchors according to 360 video CTC, and provide the reporting template for the common test conditions.

It is noted that there was some misalgnment between the original adoption of M0452 and the submitted software. Currently proponents are working on the final version, which will then be integrated and released as 9.1.

JVET-N0007 JVET AHG report: Coding of HDR/WCG material (AHG7) [A. Segall, E. François, W. Husak, D. Rusanovskyy]

The AHG was established with the following mandates:

- Study and evaluate available HDR/WCG test content.

- Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results of the CfP responses.

- Compare the performance of the VTM and HM for HDR/WCG content.

- Prepare for expert viewing of HDR content at the 14th JVET meeting if feasible.

- Coordinate implementation of HDR anchor aspects in the test model software with AHG3.

- Study additional aspects of coding HDR/WCG content.

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, with an [AHG7] indication on message headers.  The primary activity of the AhG was related to the mandates of comparing the performance of the VTM for HDR/WCG content. This work is described in the following subsection.

During the anchor generation process, it was observed that the definition of the HDR anchors currently does not use the “luma mapping with chroma scaling” tool (LMCS), otherwise known as reshaping. There is no knowledge of this being an intentional decision at the Marrakech meeting, and it appears more due to the following scenario happening:

- January 30th:	LMCS tool committed to the VTM.

VTM configuration files enable luma dQP for coding HDR-PQ sequences

HDR CTC specifies that the configuration files should be used.

- February 4th: 	VTM-4.0rc1 tagged

- February 7th: 	Bug found with the combination of LMCS and luma dQP tool enabled.

Patch submitted by LMCS proponent to disable LMCS when luma dQP is

enabled.

- February 11th: 	VTM-4.0 tagged

VTM configuration files still enable luma dQP

This created a poorly defined situation, as the HDR CTC describes that luma dQP should be enabled as defined in the configuration files, and there was no update for the HDR CTC scheduled as an output of the Marrakech meeting.

Further evaluation showed that the performance of the anchor was not dramatically affected.  And, given that the time between tagging VTM-4.0 and the document release date for the Geneva meeting was less than one month, it was deemed best to defer clarifying and/or correcting this issue at the Geneva meeting.

The AhG performed experiments comparing the performance of VTM 4.0 and VTM 3.0 codecs on HDR content.  A summary of the performance is provided below.

Please note, and to be abundantly clear, results marked as “VTM 4.0 (Luma dQP Disabled)” define the performance of the VTM 4.0 with LMCS enabled and Luma dQP disabled.  “VTM 3.0 (Luma dQP Enabled)” define the existing VTM 3.0 HDR anchor.

VTM 4.0 versus VTM 3.0

VTM 4.0 versus HM 16.18

VTM 4.0 LMCS performance

VTM 4.0 performance – HLG Content

Contributions

There is one contribution related to HDR video coding:

| JVET-N0221 | m46879 | Chroma Quantization Parameter Qpc Table for HDR Signal | T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen(Dolby) |
| --- | --- | --- | --- |

Additional Comments

While not part of any official AhG activity, informal comments related to the AhG activity were made to one or more of the AhG chairs and captured here:

Question: Should the non-PSNR metrics be removed for HLG content? Revisit, possibly BoG later

Question: Should the fixed bit-rate category be removed from the CTC? Revisit, possibly BoG later

The AHG recommends the following:

- Review all input contributions

- Discuss the configuration of the HDR anchor and align VTM 5 and/or reissue the HDR CTC document as needed

To clarify the problem above, the VTM4 software disables LMCS when dQP is enabled. Therefore, it was not possible to test the benefit of LMCS for HDR in the old CTC. Currently, there is no means of testing luma dQP approaches tailored for HDR and LMCS together. This aspect should be further studied, there are no inputs on that. Likely the encoders for the two cases are not aligned.

New CTC for HDR should be issued. Revisit.

JVET-N0008 JVET AHG report: 360° video coding tools and test conditions (AHG8) [J. Boyce, K. Choi, P. Hanhart, J.-L. Lin]

The AHG was established with the following mandates:

- Study the effect on compression and subjective quality of different projections formats, resolutions, and packing layouts.

- Discuss refinements of common test conditions, test sequences, and evaluation criteria.

- Solicit additional test sequences, and evaluate suitability of test sequences on head-mounted displays and normal 2D displays.

- Study coding tools dedicated to 360° video, their impact on compression, and implications to the core codec design.

- Study the effect of viewport resolution, field of view, and viewport speed/direction on visual comfort.

- Study complexity of GPU rendering of projection formats

- Study syntax for signalling of projection formats

There was no AHG email activity on the main jvet reflector, jvet@lists.rwth-aachen.de, with an [AHG8] indication on message headers.

There was no 360º related CE for this meeting. There are 7 input documents related to 360º video coding, which are listed and categorized below. The 3 contributions in the inloop filter disabling category are also related to high-level syntax.

Input contributions:

Reference wraparound

JVET-N0070 AHG16/AHG8: Proposed Cleanup for Reference Wraparound [B. Heng, M. Zhou, W. Wan (Broadcom)]

Projection formats

JVET-N0219 Coding of 360° video in HEC using different numbers of tiles per face [J. Sauer, M. Bläser (RWTH Aachen)]

JVET-N0232 [AHG8] Coding performance of 360° Videos in PHEC format with different face sizes [A. DSouza, C. Pujara, R. Gadde, W. Choi, K. Choi, K.P. Choi (Samsung)] [late]

JVET-N0233 [AHG8] Alternate face arrangement for PHEC [A. DSouza, C. Pujara, R. Gadde, W. Choi, K. Choi, K.P. Choi (Samsung)] [late]

Inloop filter disabling

JVET-N0234 [AHG12/AHG17] Signaling of virtual boundaries [A. DSouza, C. Pujara, R. Gadde, W. Choi, K. Choi, K.P. Choi (Samsung)] [late]

JVET-N0275 AHG14/AHG8/AHG17: Sections for intra refresh and inloop filter disabling [J. Boyce, R. Lei, L. Xu (Intel)]

JVET-N0438 AHG12: Loop filter disabled across virtual boundaries [S.-Y. Lin, L. Liu, J.-L. Lin, Y.-C. Chang, C.-C. Ju (MediaTek), P. Hanhart, Y. He (InterDigital)]

The AHG recommends the following:

- Review input contributions

- Consider in-loop filter disabling contributions in coordination with high-level syntax experts

- Review common test conditions for 360° video, including objective metrics and viewports

- Review 360° video test material, and consider adding or replacing test sequences for common test conditions

In previous meetings, it had been demonstrated that disabling the filters at cube face boundaries provided subjective benefit. There are different solutions that will be discussed in context of HLS. The wraparound contribution is an implementation issue which is not too specific on 360. BoG activity (J. Boyce) on the projection formats, which could have impact on an update of CTC for 360 video.

JVET-N0009 JVET AHG report: Neural Networks in Video Coding (AHG9) [S. Liu, K. Kawamura, Y. Li, L. Wang, P. Wu, H. Yang]

The AHG was established at Marrakech, Morocco with the following mandates:

- Investigate the benefit of using neural networks in video compression such as CNN loop filter, intra prediction, re-sampling in adaptive resolution coding, and encoder side partition mode decisions.

- Investigate the complexity impact of using neural networks in video compression.

- Investigate the complexity measurement of neural network coding tools.

- Investigate the impact of training materials on the performance of neural network coding tools.

- Investigate the impact of the training process on performance and complexity.

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG9] in message headers. Subjects such as software sharing, training data and process, neural network structure and complexity, etc. have been actively discussed among proponents and participants. The reporting template for evaluating neural network coding tools is updated [1]. This AHG worked closely with CE13. Detailed discussions about tool performance and complexity evaluations are included in CE13 report.

AHG9 related input documents for this meeting are summarized as follows.

CE13 contributions

- Y.-L. Hsiao, O. Chubach, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE13-1.1: Convolutional neural network loop filter,” JVET-N0110, Geneva CH, 19–27 March 2019

- J. Yao, L.Wang, “CE13-2.1: Convolutional Neural Network Filter (CNNF) for Intra Frame,” JVET-N0169, Geneva CH, 19–27 March 2019

- Y. Wang, Z. Chen, Y. Li, L. Zhao, S. Liu, X. Li, “CE13: Dense Residual Convolutional Neural Network based In-Loop Filter (Test 2.2 and 2.3),” JVET-N0254, Geneva CH, 19–27 March 2019

- H. Yin, R. Yang, X. Fang, S. Ma, “CE13-1.2: Adaptive convolutional neural network loop filter,” JVET-N0480, Geneva CH, 19–27 March 2019

- Y. Dai, D. Liu, N. Yan, F. Wu, “CE13: Experimental results of CNN-based In-Loop Filter (USTC),” JVET-N0513, Geneva CH, 19–27 March 2019

CE13-related contributions

- S. Wan, M.-Z.Wang, H. Gong, C.-Y. Zou, Y.-Z. Ma, J.-Y. Huo, Y.-F. Yu, Y. Liu, “CE13-related: In-loop filter with only CNN-based filter,” JVET-N0133, Geneva CH, 19–27 March 2019

AHG9-related contributions

- M. Li, P. Wu, “AHG17 & AHG9: Comments on carriage of coding tool parameters in Adaptation Parameter Set,” JVET-N0065, Geneva CH, 19–27 March 2019

The AHG recommends:

- To review all related contributions

- To continue investigating the benefits and complexity of using neural networks in video coding

It is commented that currently no aspect other than LF is investigated in input contributions. Further review with CE13.

JVET-N0010 JVET AHG report: Encoding algorithm optimization (AHG10) [A. Duenas, A. Tourapis, C. Helmrich, S. Ikonin, A. Norkin, R. Sjöberg, T. Toma]

At the 13th JVET meeting, the AHG on Encoding algorithm optimizations was established with the following mandates:

- Study the impact of using techniques such as GOP structures and perceptually optimized adaptive quantization for encoder optimization.

- Study the impact of adaptive quantization on individual tools in the test model.

- Study the quantization adaptation tool in the test model.

- Investigate the feasibility of adding a CTC test category in which adaptive quantization is turned on.

- Study quality metrics for measuring subjective quality using e.g. the CfP response MOS scores.

- Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures, adaptive quantization without signalling, and multi-pass encoding.

- Study methods of rate control and their impact on performance, subjective and objective quality.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). No e-mail related to AHG10 activity was sent to the JVET reflector during the AHG period.

The following input documents were identified to be related to the AHG:

JVET-N0247: Non-CE: An improvement to hash-based motion estimation

This document presents an improvement to hash-based motion estimation. In the proposed method, all rectangle block can perform hash-based motion estimation without building corresponding hash map. In addition, we reuse the MV of hash mode to initialize the MV in the normal motion estimation. For TGM sequences, experimental results report 1.01% and 2.04% coding gain for RA and LD_B, with encoding time reduced to 92% and 92%. The anchor is VTM-4.0 and IBC is off. If IBC is on, 0.65% and 1.43% coding gain is reported, with encoding time reduced to 91% and 95%.

JVET-N0249: Non-CE8: An alternative search area for IBC

This document presents an alternative search area setting for intra block copy when CTU size is 128x128. For class F/4:2:0 TGM, the BD-rate numbers of the proposed setting are -0.90%/-2.12% for AI, -0.74%/-1.74% for RA.

JVET-N0329: CE8-related: Encoder improvements on IBC search

In this contribution, several encoder improvements are proposed to enhance the intra block copy (IBC) search in the VTM-4.0, including 1) enhanced IBC local search by using block vector (BV) candidates from previously IBC coding units (CUs); 2) enhanced predictor-based IBC search by considering the BV candidates derived by history-based BV prediction (HBVP) and extended BV derivation; 3) adaptive SAD and SATD for encoder rate-distortion (R-D) calculation; 4) simplified IBC hash table generation in original sample domain. Compared to CE8 anchors, for Class SCC sequences, the proposed encoder improvements reportedly provide 1.26%, 0.84% and 0.34% BD-rate reductions for AI, RA and LD configurations, respectively, without increasing encoding and decoding time.

JVET-N0506: Disabling raster search in integer-pel ME for common test condition

This contribution reports the performance change in VTM when raster search is turned off. When turning it off, a 0.15 % increase is observed for RA case in Y BD-rate, but the encoding time is reduced to 95 % for RA, on average, in comparison with VTM 4.0. Since turning off raster search as a default can reduce encoding burden for experiments, this contribution recommends disabling raster search for CTC.

Recommendation

The AHG recommends that the related input contributions are reviewed and to further continue the study of encoding algorithm optimizations in JVET.

It is commented that most inputs above relate to further optimization in SCC, except the last one which suggests improved ME.

Further work on other aspects such as quantization should be encouraged.

JVET-N0011 JVET AHG report: Screen Content Coding (AHG11) [S. Liu, J. Boyce, A. Filippov, Y.-C. Sun, J. Xu, M. Zhou]

The AHG was established at Marrakech, Morocco with the following mandates:

- Investigate coding tools targeted at screen content in terms of compression benefit and implementation complexity.

- Identify test materials, discuss testing conditions for screen content coding, and propose associated updated common test conditions.

- Study the impact of loop filters on screen content coding.

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG11] in message headers. There were about a dozen emails exchanged on the JVET reflector with some discussions about testing sequences. There were many email exchanges among CE8 participants under [CE8] with tool specific discussions. During the spec editing of VVC Draft 4, CPR (current picture referencing) was restructured and replaced by IBC (intra block copy) such that the intra-picture block prediction tool is now enabled and signaled at block level. Reference software VTM4 was updated accordingly.

In total there are 57 SCC related technical contributions identified so far, among which 31 IBC related technical contributions, 6 Palette related technical contributions, 12 Transform Skip related technical contributions, 6 other SCC tool (e.g. BDPCM) technical contributions and 2 SCC test material contributions identified for this meeting. They are listed in the next section.

Input documents related to AHG11 are summarized as follows.

IBC related contributions

- JVET-N0094, CE8-related: Context modelling of transform skip mode,  M. G. Sarwer, O. Chubach, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-N0095, CE8-related: Unified method for coding BVD and MVD, S.-T. Hsiang, S.-M. Lei (MediaTek)

- JVET-N0096, CE8-related: A fixed updating order for IBC reference memory, C.-Y. Lai, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-N0173, Non-CE8: IBC Reference Area Rearrange, H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)

- JVET-N0174, Non-CE8: Exclusive Regular Merge And IBC Merge in One Shared Merge List Area, H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)

- JVET-N0175, Non-CE8: IBC Reference Memory for Arbitrary CTU Size, H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)

- JVET-N0176, Non-CE8: IBC Merge List Simplification, H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)

- JVET-N0201, Non-CE8: IBC modifications, J. Li, C.-W. Kuo, C. S.Lim (Panasonic)

- JVET-N0202, AHG16/Non-CE8: Report on conformance check failures of IBC block vectors, J. Li, C.-W. Kuo, C. S.Lim (Panasonic)

- JVET-N0249, Non-CE8: An alternative search area for IBC, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-N0250, Non-CE8: Reference memory reduction for intra block copy, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-N0251, Non-CE8: Intra block copy clean-up, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-N0255, CE8: Combination of MMVD and IBC mode (test 1.3a and 1.3b), Y. Li, Z. Chen (Wuhan Univ.), X. Xu, X. Li, S. Liu (Tencent)

- JVET-N0260, Non-CE8: Disabling fractional MVD search in DMVR for SCC, W. Zhu

- JVET-N0289, CE8-Related: On MVD Coding, M. Salehifar, S. Paluri, S. Kim (LGE)

- JVET-N0315, CE8-related: Unification of Motion Vector Rounding Method for Chroma IBC Mode, Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)

- JVET-N0316, CE8-related: Default Processing for IBC Mode, Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)

- JVET-N0317, CE8-related: Simplification on IBC Merge/Skip Mode, Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)

- JVET-N0318, CE8-related: Block Size Limitation for IBC Mode, Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)

- JVET-N0329, CE8-related: Encoder improvements on IBC search, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)

- JVET-N0382, CE8-related: unified IBC block vector prediction, X. Xu, X. Li, S. Liu (Tencent)

- JVET-N0383, AHG16/Non-CE8: IBC search range adjustment for implementation consideration, X. Xu, X. Li, S. Liu (Tencent)

- JVET-N0384, Non-CE8: IBC search range increase for small CTU sizes, X. Xu, X. Li, S. Liu (Tencent)

- JVET-N0457, CE8-1.1: Block vector prediction for IBC, J. Nam, J. Lim, S. Kim (LGE)

- JVET-N0458, CE8-1.2: Block vector coding for IBC, J. Nam, J. Lim, S. Kim (LGE)

- JVET-N0459, CE8-related: Modified block vector coding for IBC, J. Nam, H. Jang, J. Choi, J. Heo, J. Lim, S. Kim (LGE)

- JVET-N0460, CE8-related: Default candidates for IBC merge mode, J. Nam, H. Jang, J. Lim, S. Kim (LGE)

- JVET-N0461, CE8-related: Signaling on maximum number of candidates for IBC merge mode, J. Nam, H. Jang, J. Lim, S. Kim (LGE)

- JVET-N0466, Non-CE8: The corner case handling regarding mv derivation for Chroma IBC in dual tree structure, H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)

- JVET-N0467, Non-CE8: Experimental result for various size of IBC block with optimized syntax signaling, H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)

- JVET-N0472, Non-CE8: On IBC reference buffer design, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)

Palette related contributions

- JVET-N0258, CE8-related: Palette Mode Coding, W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-N0259, CE8-related: compound palette mode, W. Zhu, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-N0346, Non-CE8: Palette Mode in HEVC for YUV4:4:4 format, Y.-H. Chao, H. Wang, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-N0404, CE8: Palette Mode Improvements (CE8-2.2), Y.-C. Sun, T.-S. Chang, J. Lou (Alibaba), Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm), R. Chernyak, S. Ikonin, J. Chen (Huawei)

- JVET-N0405, CE8-related: Palette Mode Simplification, Y.-C. Sun, T.-S. Chang, J. Lou (Alibaba), Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-N0550, CE8-related: Line-based CG Palette Mode, Y.-H. Chao, T. Hsieh, M. Karczewicz (Qualcomm)

Transform skip related contributions

- JVET-N0094, CE8-related: Context modelling of transform skip mode, M. G. Sarwer, O. Chubach, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-N0122, CE6/CE8-related: Alternative implementation of residual rearrangement for transform skipped blocks, T. Tsukuba, M. Ikeda, T. Suzuki (Sony)

- JVET-N0123, Non-CE6/CE8: Chroma Transform Skip, T. Tsukuba, M. Ikeda, T. Suzuki (Sony)

- JVET-N0280, CE8: Residual Coding for Transform Skip Mode (CE8-4.3a, CE8-4.3b, CE8-4.4a, and CE8-4.4b), B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)

- JVET-N0337, CE8-related: Adaptive disabling of intra sample interpolation for screen content coding, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)

- JVET-N0357, CE8-related: Context Modelling of Sign for TS Residual Coding, B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)

- JVET-N0401, Non-CE3/8: Enable Transform Skip in CUs using ISP, S. De-Luxán-Hernández, T. Nguyen, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (HHI)

- JVET-N0428, CE8-4.1: Rearrangement of the residual block for transform skip, S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim (LGE)

- JVET-N0429, CE8-4.2: Residual Coding for transform skip with various maximum context coded bins, S. Yoo, J. Choi, J. Heo, J. Choi, J. Lim, S. Kim (LGE)

- JVET-N0430, CE8-related: Transform skip restriction, J. Choi, J. Heo, S. Yoo, J. Choi, J. Lim, S. Kim (LGE)

- JVET-N0431, CE8-related: sign flag coding for transform skip residual, S. Yoo, J. Choi, J. Heo, J. Choi, J. Lim, S. Kim (LGE)

- JVET-N0455, CE8-related: Sign context modelling and level mapping for TS residual coding, M. Karczewicz, M. Coban (Qualcomm)

Other SCC tool related contributions

- JVET-N0214, CE8: BDPCM with harmonized residual coding and CCB limitation (CE8-3.1a, CE8-3.1b, CE8-5.1a, CE8-5.1b) , G. Clare (bcom), F. Henry (Orange), B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand (HHI), X. Zhao, X. Li, X. Xu, S. Liu (Tencent)

- JVET-N0247, Non-CE8: an improvement to hash-based motion estimation, J. Li, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance), R. Xiong (Peking Univ.)

- JVET-N0366, CE8-related: Modified limitation on context coded bins for CE8-3.1a and CE8-5.1a, X. Zhao, X. Li, X. Xu, S. Liu

- JVET-N0413, CE8-related: Quantized residual BDPCM, M. Karczewicz, M. Coban (Qualcomm)

- JVET-N0464, Non-CE8: MMVD Motion vector rounding for SCC, H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)

- JVET-N0501, CE8 related: Optimal Scan Order for BDPCM Residues, A. Singh, A. Konda, C. Pujara, R. Gadde, W. Choi, K.P. Choi (Samsung)

Test material contributions

- JVET-N0386, AOV YUV4:4:4 sequence for VVC standardization and suggestions for class F, X. Xu, J. Ye, X. Li, S. Liu, L. Wu, C. Xie, K. Liu, B. Wang, P. Liu, K. Dong, Y. Kuang, W. Feng (Tencent)

- JVET-N0502, Five SCC TGM sequences of both YUV4:4:4 and YUV4:2:0 formats, Tao Lin, Kailun Zhou, Shuhui Wang (Tongji), Liping Zhao (Shaoxing)

The AHG recommends:

- To review all related contributions.

- To continue investigating SCC coding tool performance, complexity and interactions between themselves and with other coding tools.

- To continue evaluating new test materials.

It is emphasized that an update on the SCC test conditions is urgently needed. Revisit.

JVET-N0012 JVET AHG report: High-level parallelism and coded picture regions (AHG12) [S. Deshpande, M. M. Hannuksela, R. Sjöberg, R. Skupin, W. Wan, Y.-K. Wang, S. Wenger]

At the 13th JVET meeting, the AHG on High-level syntax was established with the following mandates:

- Study wavefront processing including the relationship with tiles and low delay characteristics.

- Study flexible loop filter control and tile size restriction, including identifying implications on coding tools and implementation.

- Study flexible tile partitioning (e.g. more flexible than HEVC and tile boundaries not spanning a full picture).

- Study support of independently coded picture regions, including easy rewriting of such regions into a conforming sub-bitstream.

- Prepare software and configurations for the test model to facilitate parallel processing tests.

- Study the coding efficiency impact of parallel processing and coded picture regions.

The regular JVET email reflector was used for discussions (jvet@lists.rwth-aachen.de)

In the JVET email reflector, a kick off message was sent. There was no other discussion on the email reflector regarding AHG12.

Input documents related to AHG12 are listed below. There are a total of 39 documents.

| JVET # | MPEG # | Created | Title | Source |
| --- | --- | --- | --- | --- |
| JVET-N0042 | m46669 | 2019-03-06 14:08:58 | AHG12: Comments on MPEG draft requirements on immersive media access and delivery | M. M. Hannuksela,E. B. Aksu,A. Hourunranta (Nokia) |
| JVET-N0043 | m46670 | 2019-03-06 20:27:31 | AHG12: VVC design goals related to MPEG requirements on immersive media access and delivery | M. M. Hannuksela,E. B. Aksu (Nokia) |
| JVET-N0044 | m46671 | 2019-03-07 18:55:23 | AHG12: Comparison of approaches for independently coded picture regions | M. M. Hannuksela,A. Aminlou,K. Kammachi-Sreedhar (Nokia) |
| JVET-N0045 | m46673 | 2019-03-07 19:43:39 | AHG12: Sub-picture layers for realizing independently coded picture regions | M. M. Hannuksela,K. Kammachi-Sreedhar,A. Aminlou (Nokia) |
| JVET-N0046 | m46674 | 2019-03-07 19:44:50 | AHG12: Sub-picture-based picture partitioning and decoding | M. M. Hannuksela,A. Aminlou (Nokia) |
| JVET-N0047 | m46675 | 2019-03-07 19:46:26 | AHG12/AHG17: Merging IRAP and non-IRAP VCL NAL units into the same coded picture | M. M. Hannuksela (Nokia) |
| JVET-N0055 | m46710 | 2019-03-11 10:30:24 | AHG12: On sub-picture design | B. Choi,S. Wenger,S. Liu (Tencent) |
| JVET-N0056 | m46711 | 2019-03-11 10:33:00 | AHG12: On tile design | B. Choi,S. Wenger,S. Liu (Tencent) |
| JVET-N0057 | m46712 | 2019-03-11 10:33:37 | AHG12: On tile group design | B. Choi,S. Wenger,S. Liu (Tencent) |
| JVET-N0060 | m46715 | 2019-03-11 12:16:13 | AHG12: Improved parallel processing capability with WPP | Y. Fujimoto,M. Ikeda,T. Suzuki(Sony) |
| JVET-N0066 | m46722 | 2019-03-11 17:58:56 | AHG12: On top-to-bottom tile partitioning | Y. He,A. Hamza (InterDigital) |
| JVET-N0071 | m46727 | 2019-03-12 03:23:43 | AHG12: Unified signalling of tile grouping information | L. Chen,C.-Y. Chen,Y.-W. Huang,S.-M. Lei (MediaTek) |
| JVET-N0073 | m46729 | 2019-03-12 03:24:59 | AHG17: [SYS-VVC] Signalling subpicture coded video sequence | L. Chen,C.-Y. Chen,Y.-W. Huang,S.-M. Lei (MediaTek) |
| JVET-N0099 | m46755 | 2019-03-12 05:13:05 | AHG12/AHG17 On sub-picture parameter set | Y. He,A. Hazma (InterDigital) |
| JVET-N0107 | m46763 | 2019-03-12 07:34:47 | AHG12: Sub-picture based video coding | Y.-K. Wang,Hendry,J. Chen (Huawei) |
| JVET-N0108 | m46764 | 2019-03-12 07:35:10 | AHG12/AHG17: Allowing mixed IRAP and non-IRAP NAL unit types within a picture | Y.-K. Wang,Hendry (Huawei) |
| JVET-N0109 | m46765 | 2019-03-12 07:35:36 | CE12/AHG12: Treating boundaries of independent tile groups as picture boundaries | Hendry,S. Hong,J. Chen,Y.-K. Wang (Huawei) |
| JVET-N0111 | m46767 | 2019-03-12 07:35:53 | AHG12: Flexible tiling | Y.-K. Wang,Hendry,M. Sychev (Huawei) |
| JVET-N0124 | m46780 | 2019-03-12 08:02:15 | AHG12: On Tile Group Signalling | S. Deshpande (Sharp) |
| JVET-N0149 | m46807 | 2019-03-12 09:28:50 | AHG12: Signalling on wavefront parallel processing | T. Ikai,S. Deshpande,T. Chujoh,E. Sasaki,T. Aono (Sharp) |
| JVET-N0150 | m46808 | 2019-03-12 09:29:10 | AHG12: One CTU delay wavefront parallel processing | T. Ikai (Sharp) |
| JVET-N0191 | m46849 | 2019-03-12 12:24:12 | [AHG17/AHG12] On APS id for bitstream merging for VVC | N. Ouedraogo,E. Nassor,J. Taquet,G. Kergourlay,F. Maze (Canon) |
| JVET-N0192 | m46850 | 2019-03-12 12:27:25 | [AHG17/AHG12] Bitstream extraction and merging with variable initial Qp | N. Ouedraogo,E. Nassor,G. Kergourlay,F. Maze (Canon) |
| JVET-N0234 | m46892 | 2019-03-12 15:02:19 | [AHG12/AHG17] Signaling of virtual boundaries | A. DSouza,C. Pujara,R. Gadde,W. Choi,K. Choi,K.P. Choi(Samsung) |
| JVET-N0288 | m46948 | 2019-03-12 18:37:31 | AHG12/ AHG17: Comments on High-Level Syntax of VVC | S. Deshpande (Sharp) |
| JVET-N0348 | m47008 | 2019-03-12 20:55:41 | AHG12: Hierarchical tiling for VVC | R. Skupin,Y. Sanchez,K. Suehring,T. Schierl (HHI) |
| JVET-N0354 | m47014 | 2019-03-12 20:56:46 | AHG12: Conformance of temporally independent tile groups | R. Skupin,Y. Sanchez,K. Suehring,T. Schierl (HHI) |
| JVET-N0355 | m47015 | 2019-03-12 20:56:58 | CE12-related: Normative temporally independent tile group coding | R. Skupin,Y. Sanchez,V. George,K. Suehring,T. Schierl (HHI) |
| JVET-N0402 | m47064 | 2019-03-12 23:49:03 | CE12-related: MCTS improvement by modifying motion compensation filter coefficients | A. Aminlou,A. Zare,M. M. Hannuksela (Nokia) |
| JVET-N0403 | m47065 | 2019-03-12 23:53:25 | CE12-related: MCTS improvement by modifying prediction block | A. Aminlou,A. Zare,M. M. Hannuksela (Nokia) |
| JVET-N0356 | m47016 | 2019-03-12 20:57:09 | CE12-2: Encoder-side tile group restriction | R. Skupin,Y. Sanchez,V. George,K. Suehring,T. Schierl (HHI) |
| JVET-N0411 | m47073 | 2019-03-13 00:34:32 | AHG12: Layout signalling of independent coded regions | E. Thomas(TNO) |
| JVET-N0438 | m47100 | 2019-03-13 02:19:07 | AHG12: Loop filter disabled across virtual boundaries | S.-Y. Lin,L. Liu,J.-L. Lin,Y.-C. Chang,C.-C. Ju (MediaTek),P. Hanhart,Y. He (InterDigital) |
| JVET-N0496 | m47158 | 2019-03-13 10:43:36 | AHG12: Rectangular tile group address signaling | R. Sjöberg,M. Damghanian,M. Pettersson (Ericsson) |
| JVET-N0497 | m47159 | 2019-03-13 10:43:39 | AHG12: Dependent tiles | R. Sjöberg,M. Pettersson,M. Damghanian (Ericsson) |
| JVET-N0498 | m47160 | 2019-03-13 10:43:42 | AHG12: On Uniform Tile Partitioning | R. Sjöberg,M. Pettersson,M. Damghanian (Ericsson) |
| JVET-N0528 | m47231 | 2019-03-15 17:14:07 | AAHG12/AHG19/SYS-VVC: Sub-picture decoding design: a comparison of proposed solutions | E. Thomas |
| JVET-N0559 | M47300 | 2019-03-17 02:28:28 | Crosscheck of JVET-N0060 (AHG12: Improved parallel processing capability with WPP) | T. Ikai (Sharp) |
| JVET-N0646 | M47631 | 2019-03-19 07:12:52 | Crosscheck of JVET-N0150 (AHG12: One CTU delay wavefront parallel processing) | M. Ikeda (Sony) |

JVET-N0013 JVET AHG report: Tool reporting procedure (AHG13) [W.-J. Chien, J. Boyce, Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, S. Liu, D. Luo]

This document summarizes the activity of AHG13: “Tool reporting procedure” between the 13th Meeting in Marrakech, MA (9–18 Jan. 2019) and the 14th meeting in Geneva, CH (19–27 March 2019). Tool on/off experimental results vs. VTM anchor are provided for the tools specified in JVET-M1005.

The initial version of JVET-M1005 “Methodology and reporting template for tool testing” was provided on Jan. 30th.  The document contained a reporting template.

All tests described in JVET-M1005 were conducted. VTM tool tests were conducted on VTM-4.0 software with VTM configuration by switching off or on specific tool either in configuration files or macros.

The tested tools, testers, and cross-checkers are listed in the tables below.

List of adoptions included in VTM (Tool off test (unless specified) vs VTM anchor)

| Tool Name | Acronym | Document reference(s) | AI | RA | LD | Tester | Crosscheck |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chroma separate tree | CST | JVET-K0230, JVET-K0556 | X | X | X | Tzu-Der Chuang (peter.chuang@mediatek.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Dependent quantization* | DQ | JVET-L0274, JVET-M0173, JVET-M0251, JVET-M0470, | X | X | X | Tzu-Der Chuang (peter.chuang@mediatek.com) | Daniel Luo (daniel.luo@interdigital.com) |
| Cross-component linear model | CCLM | JVET-L0085, JVET-L0136, JVET-L0191, JVET-L0338, JVET-L0340, | X | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; leolzhao@ tencent.com) |
| multiple transform set | MTS | JVET-L0059, JVET-L0111, JVET-L0118, JVET-L0285, JVET-M0297, JVET-M0464, JVET-M0497 | X | X | X | Kiho Choi (kiho14.choi@samsung.com) | Shan Liu (shanl; xinzzhao@ tencent.com) |
| Adaptive loop filter | ALF | JVET-L0082, JVET-L0083, JVET-L0147, JVET-L0392, JVET-L0664, JVET-M0132 | X | X | X | Wei-Jung Chien (wchien@qti.qualcomm.com) | Daniel Luo (daniel.luo@interdigital.com) |
| Affine motion model | AFF | JVET-L0045, JVET-L0047, JVET-L0142, JVET-L0265, JVET-L0260, JVET-L0271, JVET-L0632, JVET-L0694, JVET-M0145, JVET-M0192 |  | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; guichunli@ tencent.com) |
| subblock-based temporal merging candidates | SbTMVP | JVET-L0055, JVET-L0104, JVET-L0195, JVET-L0257, JVET-L0369, JVET-L0468, JVET-M0273 |  | X | X | Shan Liu (shanl; guichunli@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Adaptive motion vector resolution | AMVR | JVET-L0377, JVET-M0246 |  | X | X | Shan Liu (shanl; guichunli@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Pairwise merge candidate | PMC | JVET-L0090, JVET-M0193 |  | X | X | Tzu-Der Chuang (peter.chuang@mediatek.com) | Roman Chernyak (chernyak.roman@huawei.com) |
| Triangular partition mode | TPM | JVET-L0124, JVET-L0208, JVET-M0328, JVET-M0883 | X | X | X | Kiho Choi (kiho14.choi@samsung.com) | Shan Liu (shanl; leolzhao@ tencent.com) |
| Bi-directional optical flow | BDOF | JVET-L0256, JVET-M0487 |  | X | X | Kiho Choi (kiho14.choi@samsung.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Combined intra/inter prediction | CIIP | JVET-L0100 |  | X | X | Kiho Choi (kiho14.choi@samsung.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Merge with MVD | MMVD | JVET-L0054, JVET-M0068, JVET-M0171, JVET-M0255 |  | X | X | Kiho Choi (kiho14.choi@samsung.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Bi-predictive with CU weights | BCW | JVET-L0646, JVET-M0111, JVET-M0264 |  | X | X | Daniel Luo (daniel.luo@interdigital.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Multi-reference line prediction | MRLP | JVET-L0283 | X | X | X | Shan Liu (shanl; leolzhao@ tencent.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Intra block copy mode** | IBC | JVET-M0407, JVET-M0483 | X | X | X | Shan Liu (shanl; xiaozhongxu@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Intra sub-partitioning | ISP | JVET-M0102 | X | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Decoder motion vector refinement | DMVR | JVET-M0147 |  | X |  | Daniel Luo (daniel.luo@interdigital.com) | Roman Chernyak (chernyak.roman@huawei.com) |
| Sub-block transform | SBT | JVET-M0140 |  | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; xinzzhao@ tencent.com) |
| Luma mapping with chroma scaling | LMCS | JVET-M0427 | X | X | X | Taoran Lu (tlu@dolby.com) | Hyeongmun Jang (hm.jang@lge.com) |
| CABAC | CABAC | JVET-M0453, JVET-M0725 | X | X | X | Yi-Wen Chen (yiwenchen@kwai.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Symmetric motion vector difference | SMVD | JVET-M0444 |  | X | X | Yi-Wen Chen(yiwenchen@kwai.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Transform skip mode | TSM | JVET-M0464 | X | X | X | Shan Liu (shanl; xinzzhao@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Shared merging candidate cist | SML | JVET-M0170 |  | X | X | Shan Liu (shanl; xlxiangli@ tencent.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Shape-adaptive transform selection*** | SATS | JVET-M0303 | X | X | X | Shan Liu (shanl; xinzzhao@ tencent.com) | Kiho Choi (kiho14.choi@samsung.com) |

* Test was conducted by disabling DQ and enabling Sign Data Hiding

** Test was conducted by enabling IBC

*** Test was conducted by disabling MTS and enabling SATS

[integrate terminology]

The results of the tests are summarized in the table below. The attached spreadsheet provides additional data. Scatter plots are also provided for the tested tools in random access configuration, comparing PSNR-Y based bd-rate on the Y axis vs. each of Enc runtime ratio, Dec runtime ratio, and a weighted average of Enc and Dec runtime ratio, (Enc + a*Dec)/(a+1), with a configurable weight, a. The exemplary weighting is set to 6 and can be adjusted in the spreadsheet attached to this report.

Full experimental results and configuration files can be found at the link below:

https://hevc.hhi.fraunhofer.de/svn/svn_VVCTestConfig/branches/VTM-4.0/

There was no bitrate or PSNR differences between testers and cross-checkers.

Encoder and Decoder runtime ratios provided by both the testers and cross-checkers are included in the reporting template, to identify if there were significant runtime differences.

Simulation results in all intra configuration (AI) of VTM tool tests. (VTM anchor)

|  |  |  |  | AI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.29% | 11.28% | 12.41% | 121% | 101% | 121% | 99% |
| DQ | 1.84% | 1.04% | 0.73% | 89% | 104% | 86% | 106% |
| CCLM | 2.08% | 19.05% | 19.23% | 99% | 100% | 99% | 99% |
| MTS | 1.95% | 1.39% | 1.39% | 63% | 96% | 63% | 95% |
| ALF | 2.25% | 2.96% | 3.17% | 100% | 88% | 100% | 89% |
| MRLP | 0.45% | 0.24% | 0.25% | 96% | 99% | 97% | 100% |
| CPR | -0.46% | -0.38% | -0.42% | 148% | 100% | 147% | 100% |
| SAO | 0.24% | 0.33% | 0.62% | 100% | 97% | 100% | 96% |
| ISP | 0.61% | 0.45% | 0.46% | 83% | 95% | 84% | 97% |
| ILRS | 0.87% | -1.17% | -1.00% | 101% | 100% | 100% | 100% |
| CABAC | 1.01% | 1.10% | 1.21% | 95% | 103% | 93% | 97% |
| TSM | 0.26% | 0.35% | 0.34% | 100% | 97% | 100% | 99% |
| SATS | -1.04% | -1.36% | -1.36% | 99% | 103% | 99% | 102% |

Simulation results in random access configuration (RA) of VTM tool tests. (VTM anchor)

|  |  |  |  | RA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.16% | 4.90% | 6.35% | 102% | 99% | 101% | 99% |
| DQ | 1.57% | 0.78% | 0.25% | 99% | 103% | 96% | 102% |
| CCLM | 0.91% | 17.37% | 17.31% | 99% | 100% | 101% | 102% |
| MTS | 0.87% | 0.47% | 0.59% | 97% | 100% | 95% | 97% |
| ALF | 3.84% | 3.53% | 3.11% | 101% | 88% | 101% | 88% |
| AFF | 2.62% | 1.84% | 1.75% | 88% | 99% | 89% | 99% |
| SbTMVP | 0.46% | 0.27% | 0.30% | 100% | 100% | 101% | 100% |
| AMVR | 1.04% | 1.50% | 1.46% | 85% | 101% | 86% | 100% |
| PMC | 0.19% | 0.04% | 0.13% | 102% | 100% | 99% | 99% |
| TPM | 0.39% | 0.61% | 0.61% | 91% | 100% | 92% | 102% |
| BDOF | 0.75% | 0.15% | 0.10% | 96% | 94% | 98% | 93% |
| CIIP | 0.51% | 0.42% | 0.55% | 96% | 99% | 98% | 99% |
| MMVD | 0.53% | 0.48% | 0.52% | 89% | 101% | 88% | 100% |
| BPWA | 0.42% | 0.52% | 0.57% | 93% | 103% | 96% | 101% |
| MRLP | 0.23% | -0.06% | 0.00% | 99% | 100% | 99% | 100% |
| IBC | 0.01% | -0.07% | -0.05% | 99% | 99% | 99% | 100% |
| SAO | 0.57% | 0.87% | 0.92% | 101% | 99% | 100% | 99% |
| ISP | 0.26% | 0.29% | 0.05% | 97% | 98% | 99% | 100% |
| DMVR | 0.73% | 1.05% | 1.08% | 100% | 94% | 99% | 94% |
| SBT | 0.38% | 0.32% | -0.04% | 94% | 99% | 93% | 97% |
| ILRS | 1.38% | -2.46% | -2.37% | 93% | 98% | 94% | 99% |
| CABAC | 0.94% | 1.00% | 0.79% | 100% | 102% | 98% | 98% |
| SMVD | 0.26% | 0.24% | 0.24% | 97% | 104% | 96% | 100% |
| TSM | 0.18% | 0.03% | -0.06% | 98% | 99% | 98% | 99% |
| SML | -0.01% | -0.05% | -0.06% | 101% | 102% | 101% | 101% |
| SATS | -0.53% | -0.50% | -0.65% | 101% | 102% | 101% | 102% |

Simulation results in low delay B configuration (LDB) of VTM tool tests. (VTM anchor)

|  |  |  |  | LDB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.08% | 1.71% | 3.03% | 99% | 101% | 100% | 101% |
| DQ | 1.33% | 1.47% | 0.57% | 96% | 102% | 98% | 102% |
| CCLM | 0.05% | 4.63% | 5.56% | 100% | 99% | 100% | 104% |
| MTS | 0.24% | 0.58% | 0.21% | 100% | 100% | 98% | 97% |
| ALF | 2.56% | 3.33% | 3.03% | 103% | 93% | 100% | 90% |
| AFF | 2.07% | 1.31% | 0.98% | 84% | 96% | 85% | 96% |
| SbTMVP | 0.63% | 0.78% | 0.39% | 101% | 97% | 101% | 101% |
| AMVR | 0.38% | 0.66% | 0.95% | 91% | 102% | 92% | 102% |
| PMC | -0.02% | 0.08% | -0.22% | 99% | 100% | 100% | 100% |
| TPM | 0.95% | 1.34% | 1.27% | 88% | 101% | 88% | 104% |
| CIIP | 0.47% | 0.99% | 1.13% | 95% | 98% | 93% | 100% |
| MMVD | 0.45% | 0.42% | 0.27% | 92% | 100% | 90% | 101% |
| BPWA | 0.29% | 0.31% | 0.23% | 97% | 100% | 97% | 100% |
| MRLP | 0.09% | 0.36% | 0.09% | 98% | 100% | 100% | 101% |
| IBC | 0.05% | 0.24% | -0.05% | 105% | 99% | 103% | 98% |
| SAO | 1.24% | 2.48% | 3.92% | 102% | 97% | 101% | 99% |
| ISP | 0.13% | 0.35% | -0.20% | 99% | 99% | 100% | 101% |
| SBT | 0.66% | 1.22% | -0.20% | 92% | 99% | 90% | 95% |
| ILRS | 0.76% | -1.71% | -2.10% | 93% | 96% | 92% | 98% |
| CABAC | 0.88% | 0.55% | 0.16% | 99% | 100% | 97% | 101% |
| TSM | 0.11% | 0.07% | 0.17% | 97% | 97% | 98% | 102% |
| SML | -0.01% | 0.05% | -0.04% | 103% | 105% | 100% | 100% |
| SATS | -0.13% | -0.01% | 0.00% | 100% | 103% | 102% | 102% |

Pixel usage and memory bandwidth results of VTM tool “off” test. (VTM anchor)

|  | AI |  | RA |  |  | LDB |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | Pixel usage | Pixel usage | Ave mem BW | Max mem BW | Pixel usage | Ave mem BW | Max mem BW |
| CCLM | 50.87% | 3.34% |  |  | 0.81% |  |  |
| ALF | 99.00% | 55.66% |  |  | 44.87% |  |  |
| AFF |  | 20.40% |  |  | 26.99% |  |  |
| SBTMC |  | 12.34% |  |  | 13.81% |  |  |
| AMVR |  | 4.91% |  |  | 2.12% |  |  |
| TAP |  | 2.52% |  |  | 6.02% |  |  |
| BDOF |  | 43.80% |  |  |  |  |  |
| CIIP |  | 1.30% |  |  | 1.65% |  |  |
| MMVD |  | 8.70% |  |  | 9.12% |  |  |
| BPWA |  | 10.17% |  |  | 8.54% |  |  |
| MRLP | 6.41% | 0.65% |  |  | 0.31% |  |  |
| SAO | 54.71% | 11.13% |  |  | 18.41% |  |  |
| DMVR |  | 40.21% |  |  |  |  |  |
| SBT |  | 2.53% |  |  | 4.15% |  |  |
| SMVD |  | 3.00% |  |  |  |  |  |
| TSM | 1.61% | 0.58% |  |  | 1.01% |  |  |

PSNR-Y vs encoding runtime ratio of VTM with VTM tool tests (VTM anchor)

PSNR-Y vs decoding runtime ratio of VTM with VTM tool tests (VTM anchor)

PSNR-Y vs weighted runtime ratio (a = 6) of VTM with VTM tool tests (VTM anchor)

It was asked whether there are some features we could turn off in the LB configuration to save runtime. AMVR was suggested as a potential candidate (0.4% gain for 10% runtime in the LB case; helps more for high-resolution RA).

Related contributions:

JVET-N0123	AHG13/Non-CE6/CE8: Chroma Transform Skip	T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)

Additional tests on transform skip mode (TSM) were conducted in the contribution.

- TSM: Disable transform skip on top of VTM-4.0 (AhG13 Test, JVET-M0464 enabled)

- TSM1: Restrict transform skip size up to 4x4 on top of VTM-4.0 (JVET-M0464 enabled)

  - Encoder option: --MTS=1 --TransformSkipLog2MaxSize=2

  - Chroma Transform skip: N/A

  - Configuration for TSM_1 are found in config_TSM_1.

- TSM2: Restrict transform skip size up to 4x4 on top of VTM-4.0 with JVET-M0464 disabled (including ticket#185)

  - Encoder option: --EMT=1 --EMTFast=1 --TransformSkipLog2MaxSize=2 with JVET-M0464 macro disabled

  - Chroma Transform skip: 2x2/4x2/2x4/4x4 can be used

  - Configuration and source changes for TSM_2 are found in config_TSM_2.

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over TM- 4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| TSM | 0.18% | 0.03% | -0.06% | 98% | 99% |
| TSM1 | 0.17% | -0.07% | -0.07% | 100% | 100% |
| TSM2 | 0.00% | -0.10% | -0.42% | 104% | 101% |

JVET-N0238	AHG13: ISP Tool-off Tests in VTM-4.0.1	S. De-Luxán-Hernández, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (HHI)

Additional tests on intra sub-partitioning (ISP) were conducted in the contribution.

- ISP: Disable ISP on top of VTM-4.0 (AhG13 Test)

  - Anchor: VTM-4.0 with ISPFast set to 0.

- ISP1: Disable ISP on top of VTM-4.0.1

  - Anchor: VTM-4.0.1 with ISPFast set to 0.

- ISP2: Disable ISP on top of VTM-4.0.1

  - Anchor: VTM-4.0.1 with ISPFast set to 1.qq

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over TM- 4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| ISP | 0.61% | 0.45% | 0.46% | 83% | 95% |
| ISP1 | 0.61% | 0.45% | 0.46% | 83% | 96% |
| ISP2 | 0.58% | 0.44% | 0.43% | 87% | 96% |

The AHG recommends the following:

- Consider the reported tool test results during tool adoption decision making

- Review related contributions

- Refine list of tested tools and test methodology for the next meeting cycle

  - Consider the reported tool test results as a benchmark for CE tests

  - Consider including reporting of compute system information for testers and cross-checkers

  - Consider additional performance or complexity metrics

JVET-N0014 JVET AHG report: Progressive intra refresh (AHG14) [J.-M. Thiesse, A. Duenas, K. Kazui, R. Sjöberg, A. Tourapis]

This document summarizes activities of AhG on Progressive intra refresh between the 13th and the 14th JVET meetings.

There were several mail exchanges on the main JVET email reflector (jvet@lists.rwth-aachen.de) with an [AHG14] indication on message headers. Three topics have been discussed on these mails:

- First, the relevance of intra refresh for low delay has been discussed. Thanks to the early sharing of a latency study by Huawei, the appropriateness of intra refresh was demonstrated without or together with DU.

- Second, there were emails regarding the opportunity of having normative changes to support intra refresh. One expert indicates that the potential additional checks for the decoder may be complex to support and this should be justified by a very high performance benefit threshold. Another expert indicated a use-case where having a smaller latency could be relevant as well as having a granularity to balance between latency and coding efficiency.

Another expert said that practical implementation using intra refresh are complex for handling of entertainment/broadcasting applications. In particular, scene changes and commercial insertion were mentioned. Also, this expert and other expert commented on the complexity due to camera motions which are not in the same direction than the intra refresh scanning.

- Third, there was a discussion on modifying the test condition which leads to the following conditions being agreed and proposed:

- Basis = Low-Delay B Main10 configuration.

- Do not use a pyramidal B structure (=> GOPSize = 1) in order to have all pictures with a somewhat similar bits sizes.

- Use a QP offset of +1 between the first I and the first B picture (the B picture having a higher QP than the I picture).

- Set the intra refresh period equal to 1 second, meaning that the stream shall be refreshed after a cycle of 1 second (can be less).

- PSNR and bitrates should be computed without the first I picture + the first cycle minus 1 (meaning that the first 30 pictures are not considered in 30Hz, 60 in 60Hz…)

An updated version of the encoder only implementation has been shared on the AHG 14 Gitlab (https://vcgit.hhi.fraunhofer.de/jvet-l-ahg-14/VVCSoftware_VTM/tree/AHG14-IntraRefresh_EncOnly).

The following contributions were identified for the AHG.

- Contribution on general aspects

  - JVET-N0114, “AHG14: A delay analysis for IRAP and GDR”, Hendry, Y.-K. Wang, M. Sychev (Huawei)

- Contributions on normative intra refresh handling including software

  - JVET-N0080, “AHG14: Revised software for ultra low-latency encoding”, K. Kazui(Fujitsu)

  - JVET-N0391, “AHG14: Updates on intra refresh proposal”, J.-M. Thiesse, D. Gommelet, D. Nicholson (VITEC)

- Contributions on normative intra refresh handling

  - JVET-N0115, “AHG14/AHG17: GDR - gradual decoding refresh”, Hendry, Y.-K. Wang, J. Chen, S. Hong (Huawei)

  - JVET-N0116, “AHG14/AHG17: DDR - distributed decoding refresh”, Y.-K. Wang, M. Sychev, Hendry (Huawei)

  - JVET-N0275, “AHG14/AHG8/AHG17: Sections for intra refresh and inloop filter disabling”, J. Boyce, R. Lei, L. Xu

  - JVET-N0310, “AHG14: On gradual decoding refresh”, M. M. Hannuksela, K. Kammachi-Sreedhar, A. Aminlou (Nokia)

- Contributions on clean random access in case of intra refresh

  - JVET-N0101, “AHG14/ AHG17: On Gradual and Clean Random Access”, S. Deshpande (Sharp)

  - JVET-N0495, “AHG14: Recovery point indication NAL unit”, R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)

The AhG recommended:

- To review all related contributions.

- To integrate the encoder-only modifications into the next VTM software, update the anchor appropriately, and to continue investigating the benefits of normative progressive intra refresh solutions.

- To consider updating the common test conditions for integrating the intra refresh scenario.

JVET-N0015 JVET AHG report: Bitstream decoding properties signalling (AHG15) [J. Boyce, J. Chen, S. Deshpande, M. Karczewicz, A. Tourapis, Y.-K. Wang, S. Wenger]

This document summarizes the activity of AHG15: Bitstream decoding properties signaling, between the between the 13th meeting in Marrakesh, MA (9–18 Jan. 2019) and the 14th meeting in Geneva, CH (19-27 Mar. 2019).

There was no email activity on the reflector during this period.

Two related input contributions were identified:

- JVET-N0276 AHG15: On interoperability point signalling [J. Boyce (Intel)]

  - JVET-N0276 proposes adding a sub-profile indicator to the profile_tier_level( ) syntax structure which would be managed by a 3rd party registration authority, and suggests parent body level discussion.

- JVET-N0278 AHG17/AHG15: On VVC HLS relevant to MPEG requirements on immersive media delivery and access (N18134) [J. Boyce (Intel)]

  - Both JVET-N0276 and JVET-N0278 have interactions with other high-level syntax contributions.

In the discussion it was noted that JVET-N0276 proposes a fixed length of 3 bytes for the sub-profile identifier, structured as in T.35 (assuming a one-byte country code and a one-byte terminal provider code, this would provide one additional byte for the subprofile ID within a profile for each terminal provider). To the extent of awareness of those present, only one byte has been needed for the country code and one byte for the terminal provider code.

The AHG chair also described the recently formed MC-IF organization, which is considering activity to develop sub-profiles.

The BoG recommends:

Review contributions

Coordinate with high-level syntax experts on high-level syntax location(s) and scope for tool restriction syntax

Initiate parent body discussion about 3rd party registration authority for sub-profile indicator

JVET-N0016 JVET AHG report: Implementation studies (AHG16) [M. Zhou, J. An, E. Chai, K. Choi, S. Sethuraman, T. Hsieh, X. Xiu]

This document summarizes the activity of AHG16: implementation studies, between the 13th JVET meeting in Marrakesh, MA (9–18 January 2019) and the 14th JVET meeting in Geneva, CH (19–27 March 2019).

There were seome, but not many, email exchanges on the main JVET email reflector in the interim period.

Comments posted on the email reflector before the finalization of CE descriptions

- Small block size (e.g. 2x2) chroma intra-prediction is a known issue when the QTBTT was adopted. Processing four times of intra chroma blocks (compared to 4x4 minimum chroma block size) alone is not something affordable for high performance VVC decoder designs. The CCLM has made the situation worse because the luma and chroma reconstruction can no longer be made parallel.  It is desirable to have a consistent solution to resolve this issue. Potential solutions for the separate tree case were proposed at the last meeting; more difficult might be to find a solution for the combined tree case.

- Memory bandwidth remains an issue to be resolved for motion compensation of both the regular inter prediction mode and the affine mode. For the regular inter prediction mode, the HEVC design (i.e. bi-pred 8x8, uni-pred 8x4/4x8) might be a straightforward solution if no better options can be found. For the affine mode where 4x4 sub-block size is used for motion compensation, designs should consider not only the constraint of the worst case memory bandwidth consumption and the worst number of reference block requests out of the cache memory, but also the computational complexity reduction of the 4x4 block based motion compensation process.

- Separate luma/chroma partitioning tree introduces a long latency for chroma intra prediction when the CCLM is on. In the worst case, the chroma intra prediction cannot be kicked off until the whole 64x64 luma block is reconstructed (e.g. when a 64x64 luma block is split horizontally while the chroma blocks are split vertically, and vice versa). It might be interesting to see if the CCLM can be disabled for 4x4 blocks without harming coding efficiency.

- Having hard coded limits on the maximum number of context coded bins per 4x4 CG does not appear to be useful for implementation, replacing them with implicit limits is preferable as it simplifies design and avoids potential quality issues.

- The quantization noise can increase the dynamic range of inverse transform output. It is recommended to clip the output of the non-separable secondary inverse transforms (NSST) 16-bit range before fed into the primary inverse transforms, to avoid potential impact on the primary inverse transforms in terms of the intermediate precisions and intermediate transform buffer size.

- Sub-partitions of 1xN and 2xN in the newly adopted sub-partition intra prediction create implementation difficulties; it is desirable to remove those elements from the current design.

- Intra Block Copy (IBC) is now treated a separate mode in VTM4.0. The separate IBC merge/AMVP list derivation is down to 4x4 block level, the IBC merge list derivation should be simplified significantly to meet the throughput requirement. Moreover, the current IBC design is optimized for 128x128 CTU size only in terms of the local buffer usage (~23 Kbytes). If the CTU size is reduced to 64x64 for instance, only ¼ of the 23 Kbytes local memory gets used and the rest of local buffer gets idle. This should be fixed.

- It is desirable to split the large block size local illumination compensation (LIC) into multiple of 16x16 blocks (similar to what has been done for the DMVR), to minimize the buffer usage and processing overhead.  In the current design of inter prediction loop, the motion compensation, DMVR and BDOF can all be processed on a 16x16 block basis. This additional LIC variant is tested in CE1.

- It might be more straightforward to design a de-blocking filter which operates on 4x4 grids and uses a line buffer of e.g. 6 luma lines (increase the line buffer size for chroma accordingly if it helps), to minimize the number of conditional checks and avoid inconsistent operations along different edges (e.g. inside the CTU and along the top CTU boundary).

Other feedback provided to the proponents

- For palette mode suggested the following changes to reduce the buffer size and improve the throughput:

  - Limit the number of palette_index_idc to a maximum (e.g. 256). For every 256 index runs (or some other number), interleave palette_index_idc and copy_above_palette_index_flag/palette_run. In which case the palette_index_idc only need to be buffered 256 entries.

  - And signal the quantized sample values immediately after the “escape code” to avoid multiple-pass CU block reconstruction.

  - Restrict the palette size especially for small size PUs to constrain the worst case bin rate.

  - Fix the maximum palette size for the standard (currently it is signalled in SPS).

- For newly adopted luma in-loop reshaping (aka DRA), suggested the following changes to reduce the implementation cost and latency:

  - Avoid computing the luma average value at TU level to reduce the latency. In the current design, the chroma residual scaling cannot take place until the whole TU size of the luma prediction block is generated and the average is computed, resulting in high latency in the chroma reconstruction even in the shared tree case for large TUs (e.g. 64x64).

  - Avoid storing the chroma residual in high-precision (currently 16-bit) for the chroma residual scaling purpose. (roughly 2.5 Kbytes local buffer saving if it is reduced to 11-bit for 10-bit video).

  - Signal the temporal domain reshaping parameters using picture-level parameter sets to avoid potential negative impact on “trick mode”.

- For newly adopted intra sub-partition prediction (ISP), identified that the current design breaks the 64x64 decoder pipeline, because for e.g. 128x64 or 64x128 CUs, the luma part is split but the chroma part is not, which creates 64x32 or 64x32 size chroma TUs. For the 64x64 block based decoder pipeline, the chroma TU size can only be up to 32x32. This was communicated with the proponents.

- For Intra Block Copy (IBC), suggested to disallow the IBC local copy from beyond the most recently decoded three (preceding) VDPUs and the already decoded area of the current VDPU.  It is asserted that reading and writing data from the same memory is not desirable, as it may create a race condition in terms of memory access.  It is also desirable to add assertions to the decoder side to make sure that the IBC copies the data from the valid reference area.

The first CU of the current VDPU (i.e. VDPU7) can copy from VDPU3 in the current IBC design, as shown below:

It was discussed whether this was intended to be allowed. It was commented that this was intended to be prohibited, and is prohibited in the meeting notes, and is described as prohibited in the VTM document but is not prohibited in the draft text or software. It was reported that contribution JVET-N0383 shows that the coding efficiency impact of prohibiting this is minor (0.2% for TGM in AI, less in RA) and provides text.

Decision (BF): Fix this bug

Memory bandwidth study for VTM4.0

Broadcom conducted a memory bandwidth study by running both the VTM4.0 and VTM3.0 for CTC with a commercial motion compensation cache model integrated. The summary results of the random access configuration are provided in the table below (for informational purposes).

The DMVR is the tool which may cause the difference in memory bandwidth consumption between the VTM4.0 and VTM3.0.  For RA configuration, this tool appears to increase the average memory bandwidth consumption by roughly 3% (see ABW_diff column) relative to VTM3.0, but the increase in the peak memory bandwidth (see MBW_diff column) is very limited (roughly 1% in class A1).

Memory bandwidth comparison (VTM4.0 vs.VTM3.0, RA):

|  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VTM4.0 Over VTM-3.0 |  |  |  |  |  |  |  |
|  | Y | U | V | TCM_diff | ABW_diff | MBW_diff | EncT | DecT |
| Class A1 | -5.44% | 6.42% | 3.71% | 2.77% | 2.82% | 0.97% | 155% | 100% |
| Class A2 | -6.85% | 5.56% | 6.23% | 2.43% | 2.42% | 0.00% | 146% | 90% |
| Class B | -5.06% | 6.08% | 5.41% | 1.48% | 1.47% | -1.10% | 151% | 100% |
| Class C | -5.14% | 6.98% | 7.68% | 2.66% | 2.81% | 0.23% | 155% | 112% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -5.52% | 6.28% | 5.84% | 3.12% | 3.34% | 3.19% | 152% | 101% |
| Class D | -4.91% | 6.22% | 6.88% | 3.68% | 3.81% | 3.19% | 165% | 114% |
| Class F (optional) | -17.34% | -9.43% | -9.17% | 5.71% | 6.70% | 0.63% | 149% | 113% |

Where

- TCM_diff : Total cache misses (over all the frames coded), percentage increase relative to VTM3.0.

- ABW_diff: Average memory bandwidth (over all the frames coded), percentage increase relative to VTM3.0.

- MBW_diff : Worst case (Max) memory bandwidth (among all the frames coded), percentage increase relative to VTM3.0.

MBW_diff is the most important measure, which shows the percentage of the worst case memory bandwidth increase of VTM4.0 relative to VTM3.0.

Bin to bit ratio in VTM4.0 and HM16.19

It has been observed that the bin to bit ratio in VTM4.0 is double digits percentage higher than that of HM16.19 for CTC. The summary results of the RA configuration are provided in Table 2 for informational purposes (data from JVET-N0049).

BD-binrate and bin to bit ratio (VTM4.0 vs.VTM3.0):

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.19 |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-bitrate |  |  | BD-binrate (average, weighted) |  |  | Bin2bit ratio (peak, weighted) |  |  | Bin2bit ratio (peak, unweighted) |  |  |
|  | Y | U | V | Y | U | V | HM16.19 | VTM4.0 | Diff (%) | HM16.19 | VTM4.0 | Diff (%) |
| Class A1 | -32.65% | -39.10% | -42.07% | -22.00% | -29.31% | -32.78% | 0.94 | 1.13 | 20.49% | 1.22 | 1.43 | 16.67% |
| Class A2 | -37.43% | -35.62% | -29.96% | -28.45% | -25.82% | -19.33% | 0.95 | 1.13 | 17.94% | 1.23 | 1.42 | 15.18% |
| Class B | -31.31% | -36.99% | -38.14% | -21.90% | -27.91% | -29.16% | 0.93 | 1.08 | 15.90% | 1.22 | 1.37 | 12.32% |
| Class C | -26.50% | -27.38% | -29.19% | -14.87% | -15.71% | -17.78% | 0.88 | 1.03 | 16.42% | 1.18 | 1.33 | 12.20% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -31.52% | -34.58% | -34.90% | -21.36% | -24.52% | -24.88% | 0.93 | 1.09 | 17.39% | 1.21 | 1.38 | 13.75% |
| Class D | -25.61% | -25.09% | -26.33% | -15.20% | -14.46% | -15.91% | 0.86 | 1.02 | 18.44% | 1.17 | 1.32 | 13.32% |
| Class F (optional) | -36.12% | -38.21% | -39.22% | -22.29% | -24.45% | -25.49% | 0.84 | 1.03 | 23.01% | 1.16 | 1.31 | 12.98% |

For a given minimum compression ratio (MinCR) of a level, a higher bin to bit ratio implies that a CABAC engine needs to process bins at a faster pace in order to deal with the peak bit-rate in real time.

In the table above, a bypass bin is counted as 0.25 context coded bins in the weighted measurements; and a bypass bin and a context coded bin carry an equal weight (1:1) in the unweighted measurements

The following contributions were identified for the AHG.

- Contributions relating to the small block size chroma intra prediction

  - JVET-N0453, “AHG16/Non-CE3: on chroma 2xN and Nx2 intra prediction in single tree”, Y. Zhao, A. Karabutov, H. Yang, J. Chen (Huawei)

  - JVET-N0465, “CE3-related: Restrict 2xN, Nx2 chroma processing for dual tree structure”, H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)

  - JVET-N0396, “CE3-related: Enabling parallel reconstruction of small intra-coded blocks’, L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, H. Huang, M. Karczewicz (Qualcomm)

- Contribution relating to the memory bandwidth reduction

  - JVET-N0397, “AHG16: Memory bandwidth reduction for small CUs”, X. Li, G. Li, X. Xu, S. Liu (Tencent)

- Contributions aiming for simplifications and cleanups of the in-loop luma reshaper

  - JVET-N0069, “AHG16/AHG17: Proposed Cleanup for Reshaper High Level Syntax”, B. Heng, M. Zhou, W. Wan (Broadcom)

  - JVET-N0117, “AHG16/AHG17: Signalling of reshaper parameters in APS”, Y.-K. Wang, Hendry, J. Chen (Huawei), P. Yin, T. Lu, F. Pu, S. McCarthy (Dolby)

  - JVET-N0113, “AHG16: Subblock-based chroma residual scaling”, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

  - JVET-N0220, “AHG16: Simplification of Reshaper Implementation”, T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen(Dolby)

  - JVET-N0389, “Chroma residual scaling with separate luma/chroma tree”, E. Francois, C. Chevance (Technicolor)

- Contributions aiming for addressing the 64x64 decoder pipeline and 1xN/2xN sub-partitions issues of the intra sub-partition prediction (ISP)

  - JVET-N0308, “AHG16/Non-CE3: Restriction of the maximum CU size for ISP to 64x64”, S. De-Luxan-Hernandez, B. Bross, T. Nguyen, V. George, B. Stabernack, H. Schwarz, D. Marpe, T. Wiegand (HHI)

  - JVET-N0469, “CE3-related : Disable ISP mode for 128xN, Nx128 Block”, H. Jang, J. Heo, J. Nam, J. Lim, S. Kim (LGE)

  - JVET-N0372: “Non-CE3: ISP with independent sub-partitions for certain block sizes”, S. De-Luxán-Hernández, B. Bross, T. Nguyen, V. George, B. Stabernack, H. Schwarz, D. Marpe, T. Wiegand (HHI)

  - JVET-N0432, “AHG16/Non-CE3: On 1xN and 2xN subpartitions in intra subpartition coding”, A. K. Ramasubramonian, G. Van der Auwera, T. Hsieh, V. Seregin, L. Pham Van, M. Karczewicz

- Contributions aiming for reducing the chroma reconstruction latency in the separate tree case

  - JVET-N0376, “AHG16/CE3-related: CCLM mode restriction for increasing decoder throughput”, J.-Y. Jung, Y.-L. Lee (Sejong Univ.), D.-Y. Kim, W. J. Jeong (Chips&Media), S.-C. Lim, J. Kang (ETRI)

  - JVET-N0390, “AHG16/non-CE3: Study of CCLM restrictions in case of separate luma/chroma tree”, E. Francois, F. Galpin, F. Le LÃ©annec, T. Poirier (Technicolor)

- Contributions aiming for addressing the IBC related issues

  - JVET-N0383, “AHG16/Non-CE8: IBC search range adjustment for implementation consideration”, X. Xu, X. Li, S. Liu (Tencent)

  - JVET-N0384, “Non-CE8: IBC search range increase for small CTU sizes”, X. Xu, X. Li, S. Liu (Tencent)

  - JVET-N0202, “AHG16/Non-CE8: Report on conformance check failures of IBC block vectors”, J. Li, C.-W. Kuo, C. S.Lim (Panasonic)

  - JVET-N0201, “Non-CE8: IBC modifications”, J. Li, C.-W. Kuo, C. S.Lim (Panasonic)

- Contribution relating to cleanups for the reference wraparound

  - JVET-N0070, “AHG16/AHG8: Proposed Cleanup for Reference Wraparound”, B. Heng, M. Zhou, W. Wan (Broadcom)

- Contribution relating to CABAC bin to bit ratio study

  - JVET-N0049, “AHG16/Non-CE7: A study of bin to bit ratio in VTM4.0 and HM16.19”, M. Zhou (Broadcom)

- Contribution relating to MPM list derivation

  - JVET-N0183, “AHG16/Non-CE3: Shared MPM list”, A. M. Kotra, S. Esenlik, J. Chen, B. Wang, H. Gao (Huawei)

Recommendations

- The AHG recommended reviewing the input contributions.

JVET-N0017 JVET AHG report: High-level syntax (AHG17) [R. Sjöberg, S. Deshpande, M. M. Hannuksela, R. Skupin, Y.-K. Wang, S. Wenger, H. Yu]

This document summarizes the activities of the AHG on High-level syntax (HLS) between the 13th JVET meeting in Marrakesh, MA (9-18 Jan. 2019) and the 14th meeting in Geneva, CH (19-27 Mar. 2019).

It is reported that the amount of input contributions related to the mandates of this AHG has increased from 22 in Marrakesh to 42 for this meeting. It can be noted that 11 of these 42 were also marked with another AHG in their titles.

For the wider category of high-level syntax contributions including also AHG12/CE12, AHG14 and AHG15, the number of input contributions is reported to have increased from 50 in Marrakesh to 78 for this meeting.

It is reported that of the 12 JCT-VC meetings for HEVC version 1, 3 meetings had a higher number of high-level syntax contributions than this JVET meeting with a peak of 96 of high-level syntax contributions at the 11th meeting in Stockholm 2012.

An informal analysis of the Geneva and Marrakesh input document titles as well as Macau and Ljubljana meeting reports shows that the number of documents in the broad high-level syntax category has increased from 8 to 78 in three meeting cycles.

Relevant contributions were listed in the AHG report. 42 of the input documents registered by March 17 had been registered with AHG17 relevance identified in their titles.

No e-mail related to AHG17 was sent to the JVET reflector during the AHG period except a kick-off message.

JVET-N0018 JVET AHG report: Quantization control (AHG18) [R. Chernyak, E. François, C. Helmrich, A. Segall]

This document summarizes the activity of AHG18: Quantization control between the 13th meeting in Marrakech, MA (9–18 Jan. 2019) and the 14th meeting in Geneva, CH (19–27 Mar. 2019).

No e-mail related to AHG18 activity was sent to the JVET reflector during the AHG period.

Input documents related to AHG18 were summarized as follows.

- JVET-N0090, CE7-related: Support of signalling default and user-defined scaling matrices, O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-N0204, AHG18: Support of quantization matrices, T. Toma, K. Abe (Panasonic), S.-C. Lim, J. Kang (ETRI)

- JVET-N0221, Chroma Quantization Parameter Qpc Table for HDR Signal, T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen (Dolby)

- JVET-N0246, Modified dequantization scaling, K. Sharman, S. Keating (Sony)

- JVET-N0341, On reporting combined YUV BD rates, F. Bossen (Sharp)

Contributions specific to luma-based chroma residual scaling

- JVET-N0113, AHG16: Subblock-based chroma residual scaling, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-N0299, On Luma Dependent Chroma Residual Scaling of In-loop Reshaper, J. Zhao, S. Kim (LGE)

- JVET-N0389, Chroma residual scaling with separate luma/chroma tree, E. Francois, C. Chevance (Technicolor)

- JVET-N0417, Simplified luma dependant chroma residual scaling of in-loop reshaper, N. Hu, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-N0477, On luma mapping with chroma scaling, Y. Ye, J. Chen, R. Liao (Alibaba)

The AHG recommended:

- To review all related contributions

It was commented that in v1 of HEVC, there were distinct QP offsets for the two chroma components at the slice level (but at the CU level, delta QP was applied to all three components, with chroma also going through a mapping table). In RExt, we added separate chroma QP control at the CU level.

JVET-N0019 JVET AHG report: Layered coding and resolution adaptivity (AHG19) [S. Wenger, A. Segall, M.M. Hannuksela, Hendry, S. McCarthy, Y.-C. Sun]

AHG19 was established by the 13th meeting (Marrakech) to study layered coding and resolution adaptivity. Little reflector activity took place; besides the kickoff message, JVET chair Gary Sullivan summarized certain activities during the Marrakech meeting and requested information on planned documents for the 14th (Geneva) meeting, and two organizations answered in public. Ten documents were submitted to JVET that include the AHG19 marker in their respective titles. The ten documents were summarized in the AHG report. The AHG recommended that those contributions be studied during the meeting.

There was limited activity of the AHG between the Marrakech and the Geneva meeting. A total of four emails included the [AHG19] marker. Besides the kick-off message, JVET cochair Gary Sullivan summarized certain activities related to the AHG’s subject matter during the Marrakech meeting, and requested information on planned documents for the 14th (Geneva) meeting. Per this request Alibaba/Huawei responded that they are working towards a contribution on the subject of ARC for the Geneva meeting. Similarly, the University for Science and Technology, China (USTC) suggested that they “hopefully” will contribute, but only to the July meeting.

In the discussion of the AHG report, there was some discussion of potential use of reduced-resolution update, and local spatial adaptivity of the resolution of the picture components – e.g., spatially localized switching of picture resolution or inter-picture switching between 4:2:0 and 4:4:4.

Bit depth was also suggested as something that could change adaptively.

It was mentioned that mixing of different bitstreams could cause issues – e.g., extraction and rewriting could cause mixtures of bit depth and chroma format (and perhaps colour space). It was commented that some of this might best be handled by a system outside of the video decoder.

The AHG recommended reviewing the related input contributions and to study layered coding and resolution adaptivity aspects.

# Project development (X)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

## Text and software development (5)

JVET-N0468 AhG2: Editorial suggestion on the text specification for inter and IBC mode [H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)]

JVET-N0470 AhG2: Mismatch between text specification and reference software on SMVD [H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)]

JVET-N0474 Comments on algorithm description document for VTM [S.-H. Park, J.-W. Kang (Ewha W. Univ.)]

JVET-N0503 Proposed update to Guidelines for VVC reference software development [F. Bossen, X. Li, K. Sühring] [late]

JVET-N0512 AhG2: Editorial Suggestion on the text specification for intra with interaction part between intra and IBC [H. Jang, J. Heo, J. Nam, S. Yoo, J. Choi, J. Lim, S. Kim (LGE)] [late]

## Test conditions (5)

JVET-N0320 On improving reports of software encoder run times [F. Bossen (Sharp)]

JVET-N0331 On interpolation for BD rate computation [F. Bossen (Sharp)]

JVET-N0341 On reporting combined YUV BD rates [F. Bossen (Sharp)]

(Also relevant to quantization and contains a normative technical proposal.)

JVET-N0446 New Adaptive Streaming Test Conditions for the VVC development [M. Afonso, A. Norkin, J. Sole, K. Swanson, A. Aaron (Netflix), J. Kim, K. Kolarov, D. Singer, A. Tourapis (Apple), Y. Ye, W. Jiang (Alibaba)]

JVET-N0506 Disabling raster search in integer-pel ME for common test condition [S.-H. Park, J.-W. Kang (Ewha W. Univ.)] [late]

## Performance assessment (1)

JVET-N0605 Comparative study of video coding solutions VVC, AV1, EVC versus HEVC [Y. Chen, E. François, F. Galpin, R. Jullian, M. Kerdranvat (Technicolor)] [late]

## Coding studies on specific use cases (14)

### Adaptive resolution conversion (6)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0048 AHG19: On adaptive resolution changing [M. M. Hannuksela, A. Aminlou (Nokia)]

JVET-N0052 [AHG19] On Signaling of Adaptive Resolution Change [S. Wenger, B. Choi, S. Liu (Tencent)]

JVET-N0118 AHG19: Adaptive resolution change (ARC) support in VVC [Hendry, S. Hong, Y.-K. Wang, J. Chen (Huawei), Y.-C Sun, T.-S Chang, J. Lou (Alibaba)]

JVET-N0279 AHG19: Adaptive resolution change [Peisong Chen, Tim Hellman, Brian Heng, Wade Wan, Minhua Zhou (Broadcom)]

JVET-N0422 AHG19: Support of adaptive resolution change (ARC) in VVC [H. Wang, N. Hu, M. Coban, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-N0532 AHG19: Resampling filters for adaptive resolution change (ARC) in VVC [Pankaj Topiwala, Madhu Krishnan, Wei Dai (FastVDO)] [late]

JVET-N0669 Crosscheck of JVET-N0532 (AHG19: Resampling filters for adaptive resolution change (ARC) in VVC) [?? (??)] [late]

### 360° video coding (4)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0070 AHG16/AHG8: Proposed Cleanup for Reference Wraparound [B. Heng, M. Zhou, W. Wan (Broadcom)]

JVET-N0219 Coding of 360° video in HEC using different numbers of tiles per face [J. Sauer, M. Bläser (RWTH Aachen)]

JVET-N0232 [AHG8] Coding performance of 360° Videos in PHEC format with different face sizes [A. DSouza, C. Pujara, R. Gadde, W. Choi, K. Choi, K.P. Choi (Samsung)] [late]

JVET-N0233 [AHG8] Alternate face arrangement for PHEC [A. DSouza, C. Pujara, R. Gadde, W. Choi, K. Choi, K.P. Choi (Samsung)] [late]

### Chroma sampling and chroma formats (4)

JVET-N0225 Various chroma format support in VVC [L. Li, J. Heo, J. Nam, M. Koo, J. Lim, S. Kim (LGE)] [miss] [late]

JVET-N0367 Support of chroma 4:4:4 format in VVC [X. Zhao, X. Li, X. Xu, S. Liu (Tencent)]

JVET-N0392 4:4:4 and 4:2:2 chroma formats support for VVC [A. Filippov, V. Rufitskiy, T. Solovyev, R. Chernyak, J. Chen (Huawei)] [late]

JVET-N0414 Modifications to support the YUV 4:4:4 chroma format [H. Wang, Y.-H. Chao, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-N0671 Support of 4:4:4 and 4:2:2 chroma formats in VVC [A. Filippov, V. Rufitskiy, T. Solovyev, R. Chernyak, J. Chen (Huawei), X. Zhao, X. Li, X. Xu, S. Liu (Tencent), H. Wang, Y.-H. Chao, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)] [miss] [late]

## Test material (2)

JVET-N0386 AOV YUV4:4:4 sequence for VVC standardization and suggestions for class F [X. Xu, J. Ye, X. Li, S. Liu, L. Wu, C. Xie, K. Liu, B. Wang, P. Liu, K. Dong, Y. Kuang, W. Feng (Tencent)]

JVET-N0502 Five SCC TGM sequences of both YUV4:4:4 and YUV4:2:0 formats [T. Lin, K. Zhou, S. Wang (Tongji), L. Zhao (Shaoxing)] [late]

# Core Experiments

Tuesday 1730- Initial discussion in track A how to organize subjective tests

– CE1: 1 test session on CE1-6 (12 cases), no mutual comparison in initial round

– CE5: set of up to 4 test sessions (48 cases)

– CE11: separate test sessions for ALF on/off cases, but mix CE11-1 and CE11-2. Sets of up to 3 test sessions (34 cases) in each

Up to 5 groups of 3 experts needed for each complete set of test sessions

M. Wien will set up a plan/timeline to run the tests, could likely be started on Wed evening or Thursday. Sequence of CEs as above.

## CE1: Post-prediction and post-reconstruction filtering (9)

Contributions in this category were discussed Tuesday 19 March 1930–2130 in Track A (chaired by JRO).

JVET-N0021 CE1: Summary Report on Post-prediction and post-reconstruction filtering [J. Ström, S. Ikonin, V. Seregin]

The tests are of two types: post-prediction and post-reconstruction filters (CE1-1.1, CE1-1.2, CE1-1.3, CE1-1.7, CE1-2.1, CE1-2.4, all CE1-4 tests, all CE1-5 tests and all CE1-6 tests), and traditional loop-filters (CE1-1-5, CE1-2.3). The restrictions are different for these two types of filters.

Restrictions for post-prediction and post-reconstruction filters:

The size of the smallest processed block is 64 luma samples.

Processing should not be applied on intra blocks, nor should it be applied to combined intra/inter blocks (CIIP blocks) or intra block copy (IBC) / current picture reference (CPR) blocks.

For all tests, the following three constraint conditions may be tested:

- No restriction on using samples to the left and above the block (mandatory)

- Samples to the left and above can be used, but only if they come from an inter block (i.e., not from an intra block or a CIIP block or an IBC/CPR block). (mandatory)

- No samples outside the block can be used. (non-mandatory)

For CE1-1.6, these constraints are relaxed:

- No restriction to apply for intra blocks or intra/inter blocks and thus also no restriction to use samples to the left or above the block (same as intra prediction in VTM).

For loop-filter tests (CE1-1.5 and CE1-2.3), different constraints are used:

- There is no restriction on the smallest processed block.

- There is no restriction to apply filtering on intra blocks, i.e., both intra and inter blocks can be filtered.

- There is no restriction on using samples to the left or above the block, as long as the used samples reside in the same CTU as the filtered pixels.

In these latter tests, RDO was employed (in a similar fashion as used for deblocking in JVET-M0428, i.e. a RD decision would know that such a loop filter would be applied). In a cross-check document (N0617), it is reported that the method of CE1-2.3 without such RDO mechanism would result in small loss.

The table below provides a summary of the tested technologies. For a more detailed description, please refer to Section 5.

Table 1.

| Test# | Description | Document# |
| --- | --- | --- |
| CE1-1 | Reduced complexity bilateral filter | JVET-M0885, JVET-M0884 |
| CE1-2 | Hadamard transform domain filter | JVET-M0468 |
| CE1-4 | Uniform Luma Inter Prediction Filter | JVET-M0042 |
| CE1-5 | Local illumination compensation | JVET-M0088, M0115, M0182, M0224, M0450, M0500 |
| CE1-6 | Intra Reference Sample Deblocking | JVET-M0138 |
| CE1-7 | Combined tests | Combinations of above documents |

The table below gives a comparison between the technologies.

Table 2.

| Test | filter shape | Comp. complex. per sample* | Precis. of mult | Sequential operations to get one sample processed | Operations before block processing | Parallel friendly | Latency (in clock cycles) | Memory. required (bytes) | How to derive filter coeffs | Min. and max. filtered CU size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE1-1.1a CE1-1.1b CE1-1.1c | 5 pixel “plus”-shape; For inter, 5x5 area is used to calculate filter weights. | 4 mult 23 adds (of which 1 rounding) 8 checks 5 shifts (of which 4 fixed) | 2×8 and 10×11 | 4 to get dNL 3 to get weight 1 to get weight*diff 1 to sum and round 7 in total | lookup s and k using qp calc m and c1 (1 variable shift each) calc c2 (1 add) | yes | Estimation: 3 clock cycles. | 30 | max0, m-k*dNL | Min: 8x8 Max: Intra: 64x64 Inter: 16x64, 64x16 |
| CE1-1.2a CE1-1.2b CE1-1.2c | —”— | 2 mult 25 adds (of which 3 rounding) 8 checks 7 shifts (of which 6 fixed) | 6x9 | As above, plus one more to get sI from I. 8 in total | —”— | yes | Estimation: 3 clock cycles. | 24 | —”— | —”— |
| CE1-1.3a CE1-1.3b CE1-1.3c | 5 pixel “plus”-shape | 2 mult 13 adds 4 checks 5 shifts (of which 4 fixed) | 7x9 | —”— | —”— | yes | Estimation: 2 clocks cycles | 24 | —”— | —”— |
| CE1-1.5 | 5 pixel “plus”-shape; For inter, 5x5 area is used to calculate filter weights. | intra: 2 mult 13 adds 4 checks 5 shifts (of which 4 fixed) inter: +12 adds +4 checks +2 shifts (both fixed) | 7x9 | —”— | —”— | yes | Estimation: Intra: 2 clock cycles Inter: 3 clock cycles | 47 | intra: max0, m-k*ΔsI inter: max0, m-k*dNL | Min: 4x4 Max: Intra: 64x64 Inter: 16x64, 64x16 |
| CE1-1.7 | 5 pixel “plus”-shape; For inter, 5x5 area is used to calculate filter weights. | 4 mult 23 adds 10 checks 5 shifts (of which 3 fixed) | 8×10 and 8×12 |  |  | yes | Estimation: 3 clock cycles. | 120 | max0, m-k*dNL | Min: 8x8 Max: Intra: 64x64 Inter: 16x64, 64x16 |
| CE1-2.1a CE1-2.1b CE1-2.1c | 3x3 | 18+4 l-bit add for round 8 shifts 6 checks | n/a | 5 add; 1 look-up table (16*7bits); 2 checks | Load 16*7bits per CU if the QP is switched to a different range | yes | 1 clock: @770MHz 16nm @450MHz 28nm 2 clocks: @770MHz 28nm | 70 bytes (16 7-bit values per qp group) | Pre-calculated in LUT | min: 8x8 max: 16xN or Nx16 (N<128) |
| CE1-2.3 | —”— | —”— | —”— | —”— | —”— | —”— | —”— | —”— | —”— | Min: no restrictions Max: Intra: 64x64 Inter: 16x64, 64x16 |
| CE1-2.4a CE1-2.4b CE1-2.4c | —”— | —”— | —”— | —”— | —”— | —”— | —”— | 84 bytes (16 7-bit values per qp group) | —”— | min: 8x8 max: 16xN or Nx16 (N<128) |
| CE1-4.2a CE1-4.2c | 5 pixel “plus” shape or 9tab filter (horizontal or vertical) | 6 adds 4 shifts |  |  |  | yes |  |  |  | Inter Min: 8x8 Max: 64x64 |
| CE1-4.3a CE1-4.3c | —”— | —”— | —”— | —”— | —”— | —”— | —”— | —”— | —”— | —”— |
| CE1-5.1a CE1-5.1b | 1 sample | 1 mult 1 add |  |  | Linear regression. Operations per CU: Mult: 4 * min(width, height) + 12 Add 8 * min(width, height) + 12 Shifts: 36 | yes |  | 128 for LUT | Linear regression. Operations per CU: Mult: 4 * min(width, height) + 12 Add 8 * min(width, height) + 12 Shifts: 36 | Inter Min: 64 samples Max: 64x64 |
| CE1-5.3a CE1-5.3b |  |  |  |  |  |  |  |  |  |  |
| CE1-5.4a CE1-5.4b |  |  |  |  |  |  |  |  |  |  |
| CE1-5.5a CE1-5.5b |  |  |  |  |  |  |  |  |  |  |
| CE1-5.6a CE1-5.6b | 1 sample | 1 mult 1 add |  |  | Linear regression. Operations per CU: Mult: 4 * min(width, height) + 12 Add 8 * min(width, height) + 12 Shifts: 36 | yes |  | 128 for LUT | Linear regression. Operations per CU: Mult: 4 * min(width, height) + 12 Add 8 * min(width, height) + 12 Shifts: 36 | Inter Min: 64 samples Max: 64x64 |
| CE1-5.7a CE1-5.7b | 1 sample | 1 mult 1 add |  |  | Linear regression. Operations per CU: Mult: 4 * min(8, width, height) + 12 Add 8 * min(8, width, height) + 12 Shifts: 36 | yes |  | 128 for LUT | Linear regression. Operations per CU: Mult: 4 * min(8, width, height) + 12 Add 8 * min(8, width, height) + 12 Shifts: 36 | Inter Min: 64 samples Max: 128x128 |
| CE1-6.1 | 2-taps | Multi 0.891 Add 2.979 shift 0.554 checks 0.166 | 6×10 | For filter decision: 3 multi / 7 add / 2 shift / 3 check For filter operation: 1 multi / 16 add / 2 shift | Get tc, beta from LUT based on slice QP | Yes | Estimation: 2 clock cycle | 7 values of 6-bit unsigned integer for filter coefficients | Derive from LUT | Width >= 32 and Height >= 32 |
| CE1-6.2 | 2-taps | multi 1.980 add 3.992 shift 0.996 checks 0.039 | 7×10 | For filter decision: 1 multi / 3 add / 1 shift / 3 check For filter operation: 1 multi / 3 add / 1 shift | None | yes | Estimation: 2 clock cycle | none | Linear combination of distance to corner samples | Width >= 32 and Height >= 32 |

## * max/min/abs are counted as checks

Below is a summary of the test results for the non-combination tests.

*: SIMD optimization for the proposed technology

**: running time unreliable

***: different SIMD settings (other than default settings) are used

Table 3a

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE1-1.1a |  |  |  |  |  | -0.35% | -0.13% | -0.06% | 102% | 100% | -0.35% | 0.65% | 0.81% | 101% | 101% |
| CE1-1.1b |  |  |  |  |  | -0.34% | -0.10% | -0.05% | 101% | 100% | -0.34% | 0.68% | 0.56% | 101% | 99% |
| CE1-1.1c |  |  |  |  |  | -0.31% | -0.08% | -0.05% | 102% | 102% | -0.31% | 0.64% | 0.74% | 100% | 98% |
| CE1-1.2a |  |  |  |  |  | -0.33% | -0.13% | -0.10% | 102% | 100% | -0.23% | 0.74% | 0.52% | 100% | 98% |
| CE1-1.2b |  |  |  |  |  | -0.31% | -0.13% | -0.12% | 103% | 102% | -0.20% | 0.84% | 0.68% | 100% | 98% |
| CE1-1.2c |  |  |  |  |  | -0.27% | -0.12% | -0.11% | 102% | 100% | -0.19% | 0.70% | 0.56% | 101% | 99% |
| CE1-1.3a |  |  |  |  |  | -0.25% | -0.09% | -0.07% | 102% | 101% | -0.09% | 0.88% | 0.41% | 100% | 97% |
| CE1-1.3b |  |  |  |  |  | -0.24% | -0.09% | -0.07% | 103% | 101% | -0.09% | 0.74% | 0.57% | 100% | 98% |
| CE1-1.3c |  |  |  |  |  | -0.21% | -0.08% | -0.09% | 100% | 100% | -0.04% | 0.93% | 0.60% | 99% | 97% |
| CE1-1.5 | -0.16% | 0.25% | 0.24% | 107% | 108% | -0.44% | -0.09% | -0.11% | 105% | 102% | -0.26% | 0.99% | 0.64% | 104% | 103% |
| CE1-1.7a |  |  |  |  |  | -0.35% | -0.04% | -0.01% | 90% | 89% | -0.39% | 0.54% | 0.50% | 88% | 94% |
| CE1-1.7c |  |  |  |  |  | -0.31% | -0.03% | -0.03% | 89% | 89% | -0.40% | 0.57% | 0.38% | 83% | 93% |
| CE1-2.1a |  |  |  |  |  | -0.32% | -0.13% | -0.06% | 102% | 100% | -0.34% | 0.64% | 0.59% | 101% | 100% |
| CE1-2.1b |  |  |  |  |  | -0.31% | -0.11% | -0.09% | 102% | 100% | -0.35% | 0.66% | 0.59% | 101% | 101% |
| CE1-2.1c |  |  |  |  |  | -0.30% | -0.09% | -0.06% | 102% | 100% | -0.36% | 0.75% | 0.72% | 101% | 100% |
| CE1-2.3 | -0.18% | 0.39% | 0.35% | 108% | 105% | -0.45% | -0.05% | -0.19% | 104% | 100% | -0.34% | 0.99% | 0.54% | 103% | 101% |
| CE1-2.4a |  |  |  |  |  | -0.32% | -0.58% | -0.56% | 109% | 100% | -0.30% | 0.38% | 0.44% | 114% | 100% |
| CE1-2.4b |  |  |  |  |  | -0.29% | -0.51% | -0.46% | 110% | 99% | -0.29% | 0.50% | 0.17% | 115% | 100% |
| CE1-2.4c |  |  |  |  |  | -0.30% | -0.42% | -0.41% | 109% | 99% | -0.27% | 0.57% | 0.34% | 114% | 100% |
| CE1-4.2a |  |  |  |  |  | -0.35% | -0.70% | -0.69% | 114% | 101% | -0.15% | 0.42% | 0.23% | 125% | 102% |
| CE1-4.2c |  |  |  |  |  | -0.35% | -0.59% | -0.52% | 112% | 98% | -0.12% | 0.55% | 0.47% | 125% | 102% |
| CE1-4.3a |  |  |  |  |  | -0.33% | -0.67% | -0.66% | 111% | 101% | -0.14% | 0.42% | 0.19% | 118% | 101% |
| CE1-4.3c |  |  |  |  |  | -0.33% | -0.49% | -0.50% | 109% | 98% | -0.08% | 0.61% | 0.43% | 118% | 101% |
| CE1-5.1a |  |  |  |  |  | -0.43% | -0.17% | -0.16% | 113% | 100% | -0.42% | -0.04% | -0.36% | 111% | 101% |
| CE1-5.1b |  |  |  |  |  | -0.35% | -0.28% | -0.30% | 115% | 100% | -0.36% | -0.11% | -0.30% | 114% | 101% |
| CE1-5.3a |  |  |  |  |  | -0.56% | -0.25% | -0.25% | 122% | 101% | -0.59% | -0.49% | -0.60% | 119% | 100% |
| CE1-5.3b |  |  |  |  |  | -0.47% | -0.40% | -0.46% | 125% | 101% | -0.54% | -0.38% | -0.57% | 121% | 101% |
| CE1-5.4a |  |  |  |  |  | -0.52% | -0.17% | -0.19% | 123% | 100% | -0.56% | -0.35% | -0.65% | 127% | 102% |
| CE1-5.4b |  |  |  |  |  | -0.43% | -0.38% | -0.38% | 124% | 101% | -0.49% | -0.29% | -0.40% | 129% | 102% |
| CE1-5.5a |  |  |  |  |  | -0.42% | -0.16% | -0.11% | 108% | 101% | -0.50% | -0.11% | -0.42% | 108% | 101% |
| CE1-5.5b |  |  |  |  |  | -0.30% | -0.29% | -0.24% | 109% | 102% | -0.45 | -0.19 | -0.48 | 110% | 102% |
| CE1-5.6a |  |  |  |  |  | -0.43% | -0.25% | -0.23% | 112% | 99% | -0.42% | -0.09% | -0.36% | 111% | 101% |
| CE1-5.6b |  |  |  |  |  | -0.35% | -0.29% | -0.37% | 114% | 100% | -0.34% | 0.10% | -0.37% | 113% | 102% |
| CE1-5.7a |  |  |  |  |  | -0.39% | -0.24% | -0.22% | 113% | 100% | -0.33% | -0.08% | -0.34% | 112% | 101% |
| CE1-5.7b |  |  |  |  |  | -0.30% | -0.33% | -0.32% | 114% | 100% | -0.30% | -0.18% | -0.35% | 114% | 101% |
| CE1-6.1 | -0.03% | 0.01% | -0.02% | 93%** | 100% | -0.02% | -0.01% | -0.09% | 94%** | 100% | 0.00% | 0.52% | 0.51% | 98%** | 99% |
| CE1-6.2 | 0.02% | 0.05% | 0.01% | 96%** | 101% | 0.00% | 0.02% | -0.04% | 104%** | 100% | -0.01% | 0.25% | -0.03% | 101%** | 100% |

Below is a summary of the test results for the combination tests.

Table 3b

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE1-7.1 | -0.16% | 0.25% | 0.24% | 105% | 106% | -0.69% | -0.46% | -0.48% | 117% | 100% | -0.27% | 1.47% | 1.04% | 125% | 103% |
| CE1-7.2 |  |  |  |  |  | -0.59% | -0.49% | -0.41% | 115% | 101% | -0.36% | 1.01% | 0.89% | 123% | 104% |
| CE1-7.3b | -0.18% | 0.39% | 0.35% | 108% | 105% | -0.72% | -0.40% | -0.54% | 114% | 100% | -0.35% | 1.41% | 1.25% | 122% | 100% |
| CE1-7.4b |  |  |  |  |  | -0.58% | -0.47% | -0.46% | 111% | 100% |  |  |  |  |  |
| CE1-7.5 |  |  |  |  |  | -0.74% | -0.62% | -0.56% | 125% | 102% | -0,45% | 0,51% | 0,34% | 131% | 101% |
| CE1-7.6 |  |  |  |  |  | -0.72% | -0.64% | -0.64% | 123% | 100% |  |  |  |  |  |
| CE1-7.7.1a |  |  |  |  |  | -0.75% | -0.17% | -0.15% | 114% | 100% | -0.76% | 0.56% | 0.23% | 111% | 100% |
| CE1-7.7.1b |  |  |  |  |  | -0.66% | -0.34% | -0.32% | 116% | 101% | -0.71% | 0.59% | 0.24% | 114% | 101% |
| CE1-7.7.2a |  |  |  |  |  | -0.74% | -0.67% | -063% | 123% | 99% | -0.70% | 0.37% | 0.31% | 126% | 100% |
| CE1-7.7.2b |  |  |  |  |  | -0.65% | -0.74% | -0.73% | 125% | 99% | -0.63% | 0.26% | 0.00% | 129% | 100% |
| CE1-7.7.3a |  |  |  |  |  | -0.87% | -0.63% | -0.61% | 125% | 100% | -0.75% | 0.85% | 0.33% | 127% | 100% |
| CE1-7.7.3b |  |  |  |  |  | -0.79% | -0.73% | -0.74% | 127% | 100% | -0.63% | 0.77% | 0.50% | 126% | 101% |
| CE1-7.8a |  |  |  |  |  | -0.44% | -0.53% | -0.50% | 111% | 99% | -0.34% | 1.01% | 0.69% | 115% | 100% |
| CE1-7.8b |  |  |  |  |  | -0.44% | -0.47% | -0.42% | 111% | 100% | -0.33% | 0.73% | 0.72% | 116% | 100% |
| CE1-7.8c |  |  |  |  |  | -0.43% | -0.40% | -0.38% | 111% | 99% | -0.31% | 0.86% | 0.76% | 115% | 100% |

From the discussion:

All approaches are only applied to inter coded blocks (except the “loop filters” which are decoupled from the immediate prediction/reconstruction processes)

There are two aspects of complexity, first using intra reconstructed samples (which are the cases 1-x.ya) that causes interdependency such that for performing the prediction or reconstruction of an inter coded block we have to wait for the neighbored intra block to be reconstructed, and additional processing can start only then. These “a” cases were introduced as an additional reference to understand how large the loss of a given tool would be if it was made more implementation friendly and avoid possible latency.

The second aspect is the processing complexity as such, which is probably different for the different methods (as per table above). This applies to cases b which use reconstructed samples from inter coded neighbors only, c does not use any neighboring blocks at all.

The additional worst case processing complexity (in terms of number of operations) is exactly the same also for the loop filter versions. The advantage would be that it comes as an independent stage, and therefore does not require the restrictions a-c, and can be applied to intra coded blocks as well. On the other hand, it would be undesirable to have possible impact on the pipeline of loop filters. Some impact on encoder as well.

It is further pointed out that at least LIC has some dependency with LMCS, since the reference picture is stored in the original domain, whereas the reconstruction of neighbored blocks is in reshaped domain. The related LUT access is not in the table above, but seems not to be a big deal.

For bilateral (CE1-1), Hadamard (CE1-2) and uniform (CE1-4), the differences between cases a,b,c are low (<0.05%), all in the range of around 0.3%. The additional logic e.g. for doing boundary padding for cases b and c is not considered in the tables above, but seems not to be a big deal.

CE1-4 appears to be the most regular design among the methods investigated in CE1. It is asked how the gain would relate to that of adaptive interpolation filters reported elsewhere, or if it would be possible to combine in one filter stage with variable coefficients, because that might be more desirable in terms of decoder complexity. Currently, no information available on this aspect.

For LIC (CE1-5), the differences between a and b are in the range of 0.1%, overall for case b the gains are in the range 0.3%-0.4%, depending on configuration with bi pred, etc.

For the methods that use additional filtering in the prediction (CE1-5x, CE1-4x, CE1-2.4 which is Hadamard as post-pred. filter), encoding time increase of 8-25% is observed, due to additional RD checks.

For hardware implementation, processing in 16x16 block units would be desirable. For LIC this was investigated (CE1-5.7x vs. CE1-5.1x) which results in approx. 0.05% loss. For CE1-2.4.x and CE1-4.x, it was not tested but could be expected (comparing .b and .c results) that the loss might be marginal. Implementation-wise, it would be desirable, and should be specified accordingly in case that such technology would be adopted.

For the loop filter variants of Hadamard and bilateral, gain of approx. 0.45% is reported for RA, lower for LB (whereas in post rec. configuration, similar gains for both RA and LB). This gain is pretty low for justifying another stage in the chain of loop filters. Furthermore, it is noted that the gain is lower (or even loss in case of Hadamard) when not tested in RDO.

For LIC, it is remarked that the gain is relatively high for sequences with illumination changes. However there are no typical cases for that in CTC (gains between 0.2% and 0.8% per sequence in RA). For sequences with illumination changes, weighted prediction may also help.

All methods investigated in this CE come either with not-insignificant additional building blocks, or not-insignificant increase in encoder runtime, or both. Compared to that, the benefit in compression is relatively low, no good enough tradeoff for adoption of any of these.

JVET-N0059 CE1: Uniform Luma Inter Prediction Filter [J. Rasch, A. Henkel, J. Pfaff, M. Albrecht, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-N0074 CE1-5.5: Simplification of local illumination compensation [C.-M. Tsai, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0521 Crosscheck of JVET-N0074 (CE1-5.5) [P. Bordes (Technicolor)] [miss] [late]

JVET-N0639 Cross-check of JVET-N0074 (CE1-5.5: Simplification of local illumination compensation) [S. Bandyopadhyay (InterDigital)] [late]

JVET-N0205 CE1: Pipeline restriction and DBF modification on LIC (test 1.5.6, test 1.5.7) [K. Abe, T. Toma, J. Li, C.-W. Kuo, V. Drugeon (Panasonic)]

JVET-N0636 Cross-check of JVET-N0205 (CE1-5.6: Pipeline restriction and DBF modification on LIC) [S. Bandyopadhyay (InterDigital)] [late]

JVET-N0283 CE1-5.4 mutual exclusion of LIC and BPWA [P. Bordes, F. Urban, T. Poirier (Technicolor)]

JVET-N0343 CE1: Unidirectional illumination compensation (CE1-5.1) [V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz (Qualcomm)]

JVET-N0632 Crosscheck of JVET-N0343 (CE1: Unidirectional illumination compensation (CE1-5.1)) [K. Abe (Panasonic)] [late]

JVET-N0370 CE1: Intra reference sample deblocking (CE1.6) [Z. Zhang, K. Andersson, R. Sjöberg, J. Ström (Ericsson)]

JVET-N0406 CE1: LIC applied after bi-prediction (CE1-5.3) [S. Bandyopadhyay, Y. He (InterDigital)]

JVET-N0478 CE1: Hadamard transform domain filter (CE1-2) [S. Ikonin, V. Stepin, A. Karabutov, J. Chen (Huawei)]

JVET-N0617 Crosscheck of JVET-N0478 (CE1: Hadamard transform domain filter): additional test results for HTDF at the “loop filter” position (CE1-2.3) [?? (??)] [miss] [late]

JVET-N0489 CE1: Bilateral filter tests [J. Ström, P. Wennersten, J. Enhorn (Ericsson), K. Reuze, D. Rusanovskyy, M. Karczewicz (Qualcomm)]

## CE2: Subblock motion compensation (20)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0022 CE2: Summary report on sub-block based motion prediction [C.-C. Chen, Y. He, H. Liu]

Discussed Tues 1730-2200, Wed 0900-1100 (GJS)

The goal of Core Experiment 2 (CE2) is to investigate sub-block based motion prediction techniques on top of VTM-4.0. It comprises 5 categories, as follows:

CE 2-1: Affine motion compensation and affine MV coding (2)

CE 2-2: Affine merge mode (2)

CE 2-3: Sub-block based Merge Mode (1)

CE 2-4: Memory bandwidth reduction for sub-block modes (10)

CE 2-5: Storage reduction for sub-block modes (6)

In each test, a comparative study along with related tests is conducted, results and complexity analysis are provided. Crosschecking reports of all tests are integrated in this document as well.

CE2-1: Affine Motion Compensation and Affine MV Coding

| Test# | Description | Document# |
| --- | --- | --- |
| 2-1.1a | Interweaved prediction for affine motion compensation for luma only | JVET-N0261 |
| 2-1.1b | Test 2-1.1a + chroma |  |
| 2-1.1c | Test 2-1.1a + Test 2-4.6 |  |
| 2-1.1d | Test 2-1.1a + Test 2-4.8a |  |
| 2-1.2a | SMVD with signaling MVDs of all control points for L0 | JVET-N0319 |
| 2-1.2b | SMVD with signaling MVDs of TL control point for L0 and set TR and BL control point MVD for L0 to 0 |  |

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2-1.1a | -0.30% | 0.02% | -0.02% | 100% | 100% | -0.14% | 0.04% | -0.15% | 101% | 101% |
| 2-1.1b | -0.32% | -0.21% | -0.18% | 100% | 99% | -0.14% | -0.13% | -0.18% | 101% | 100% |
| 2-1.1c | -0.28% | 0.03% | 0.03% | 101% | 100% | -0.11% | 0.10% | 0.03% | 101% | 102% |
| 2-1.1d | -0.28% | 0.01% | 0.02% | 101% | 101% | -0.13% | -0.13% | 0.22% | 101% | 101% |
| 2-1.2a | -0.10% | -0.06% | -0.06% | 106% | 100% |  |  |  |  |  |
| 2-1.2b | -0.07% | -0.05% | -0.05% | 105% | 100% |  |  |  |  |  |

The gains are larger for affine-friendly test sequences. Cactus had 1.25% coding gain. For the Slideshow Class F sequence, the LB gain was reported as 2.86%. It was commented that the worst case is not affected, since this is applied as a uni-pred mode. Memory access is also reportedly not affected, since the same memory region in the reference picture is fetched for both prediction signals. A position-dependent weighting function is applied with a 4x4 weighting pattern.

The spirit of this is sort of to get some of the advantage that would be gained by using 2x2 subblocks for affine MC rather than 4x4 subblocks. (There is also a proposal to just use 2x2 subblocks, JVET-N0273.)

It was commented that another related contribution JVET-N0399 reported that using ordinary averaging could provide roughly the same gain.

It was commented that the grid shifting requires some delay in the processing pipeline and some line buffering, e.g., as contrasted with ordinary sub-block bipred, in which case the results of each block can be written out block by block without delay.

It was commented that also JVET-N0399 uses higher precision in the intermediate stage prior to the averaging (which is how the current biprediction operates).

A non-CE contribution JVET-N0236 was also mentioned, which combines predictions from a sample-based optical flow and affine.

Due to implementation concerns and other potential approaches that may interact with this, it was agreed that further study would be needed.

The 2-1.2x proposals provide very little gain and significantly increased encoding time and require some additional logic and syntax, so did not seem appropriate for action.

CE2-2: Affine Merge Mode

| Test# | Description | Document# |
| --- | --- | --- |
| 2-2.1 | Only select the MV with reference index equal to 0 as CPMV | JVET-N0075 |
| 2-2.2a | Affine MMVD with POC-based offset mirroring and a distance table {1/2, 1, 2, 4, 8} | JVET-N0378 |
| 2-2.2b | Affine MMVD with POC-based offset scaling |  |
| 2-2.2c | Test 2-2.2a + Affine MMVD with resolution-based distance tables: {1/2, 1, 2, 4, 8}/N, where N=1,4,8 |  |

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2-2.1 | 0.02% | -0.01% | 0.00% | 100% | 99% | 0.02% | 0.04% | -0.08% | 100% | 101% |
| 2-2.2a | -0.13% | -0.15% | -0.10% | 101% | 100% | 0.01% | -0.09% | -0.26% | 103% | 99% |
| 2-2.2b | -0.12% | -0.17% | -0.19% | 102% | 100% | -0.06% | -0.05% | -0.26% | 104% | 99% |
| 2-2.2c | -0.15% | -0.11% | -0.10% | 101% | 100% | -0.03% | -0.02% | 0.09% | 103% | 99% |

2-2.1 eliminates reference picture index logic for affine merge, proposing to always use reference picture index 0 for affine merge. This reportedly reduces the worst case number of needed comparisons for determining the validity and matching of reference picture indexes from about 60 to 47. It was asked what is the frequency of picking non-zero reference indexes and commented that QP control could affect whether the non-zero reference indexes would be desirable to use or not. The LB case was suggested to consider, since in this case we use a QP hierarchy without reordering reference picture lists, and may in such a case tend to use more nonzero indexes for referencing. It was commented that two test sequences showed some loss in that case (0.2% in one case and 0.4% in another).

Another contribution JVET-N0203 was also reported to show potential gain for using a non-zero reference indexes more often (e.g. in the LB case).

The benefit here did not seem very significant. No action.

2-2.2x tests related to affine MMVD offsets. In the current spec, MMVD is not applied to affine mode. This would add some additional logic and syntax parsing that is different for affine than in other modes. The gain seemed too small for action on this.

The 2-2.3 category was withdrawn.

CE2-4: Memory bandwidth reduction for sub-block modes

| Test# | Description | Document# |
| --- | --- | --- |
| 2-4.1 | Withdrawn | n/a |
| 2-4.2a | Affine sub-block size setting: 4x4 for UNI, 8x4 for BI | JVET-N0323 |
| 2-4.2b | Affine sub-block size setting: 4x4 for UNI, 4x8 for BI |  |
| 2-4.2c | Affine sub-block size setting: 4x4 for UNI, adaptive 8x4/4x8 for BI |  |
| 2-4.2d | Affine sub-block size setting: 4x4 for UNI, 8x8 for BI |  |
| 2-4.3 | Clip sub-block MVs based on the bounding box pointed to by top-left one | JVET-N0195 |
| 2-4.4a | Clip sub-block MVs based on limiting max differences among 4x4 MVs within an 8x8 block are no larger than Tx =1 and Ty=0 | JVET-N0398 |
| 2-4.4b | Test 2-4.4a with Tx =1 and Ty=1 |  |
| 2-4.4c | Test 2-4.4a with Tx =1 and Ty=1 and distance-based MVmin MVmax determination off |  |
| 2-4.5a | Reduce worst-case memory bandwidth by switching sub-block size: 4x4, 8x8 | JVET-N0256 |
| 2-4.5b | Reduce worst-case memory bandwidth by switching interpolation filter: 4-, 8-tap |  |
| 2-4.5c | Reduce worst-case memory bandwidth by imposing encoder bitstream conformance |  |
| 2-4.6 | Complexity and bandwidth reduction for 4x4 sized partition using 6-tap filter | JVET-N0196 |
| 2-4.7a | Motion compensation with padding for 4x4 UNI and 4x8/8x4/4x16/16x4 BI | JVET-N0253 |
| 2-4.7b | Based on Test 2-4.7a, padding with 1 less column or row for 4x8 BI and 4x4 UNI |  |
| 2-4.8a | Encoder/decoder conformance, the fallback mode is not triggered, δx = δy = 2 | JVET-N0068 |
| 2-4.8b | Decoder conformance, the fallback mode is triggered, δx = δy = 2 |  |
| 2-4.9a | Withdrawn | JVET-N0210 |
| 2-4.9b-1 | Integer motion compensation for regular inter mode 4x4/4x8/8x4 |  |
| 2-4.9b-2 | Test 2-4.9b-1 + Rounded MVs for storage |  |
| 2-4.10a | Affine MVp range limit on constructed affine candidates with parameter threshold=2 | JVET-N0379 |
| 2-4.10b | Test 2-4.10a with parameter threshold=1 |  |
| 2-4.10c | Affine MVp range limit on all affine blocks with parameter threshold=2 |  |

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2-4.1 |  |  |  |  |  |  |  |  |  |  |
| 2-4.2a | 0.10% | 0.02% | 0.04% | 99% | 100% | 0.11% | 0.14% | 0.15% | 101% | 98% |
| 2-4.2b | 0.11% | 0.04% | 0.03% | 102% | 103% | 0.12% | 0.12% | -0.05% | 98% | 97% |
| 2-4.2c | 0.10% | 0.03% | 0.06% | 101% | 102% | 0.11% | 0.11% | 0.05% | 99% | 97% |
| 2-4.2d | 0.21% | 0.10% | 0.12% | 100% | 102% | 0.18% | 0.08% | -0.01% | 99% | 98% |
| 2-4.3 | 0.01% | -0.06% | 0.00% | 101% | 100% | -0.03% | 0.05% | -0.26% | 101% | 100% |
| 2-4.4a | 0.09% | -0.01% | 0.00% | 102% | 100% | -0.01% | 0.10% | -0.17% | 103% | 99% |
| 2-4.4b | 0.01% | -0.03% | -0.05% | 102% | 99% | -0.02% | -0.14% | -0.28% | 103% | 100% |
| 2-4.4c | 0.01% | -0.03% | -0.05% | 104% | 102% | -0.02% | -0.14% | -0.28% | 105% | 103% |
| 2-4.5a | 0.04% | -0.06% | -0.03% | 102% | 104% | 0.00% | 0.21% | -0.22% | 100% | 100% |
| 2-4.5b | 0.07% | 0.05% | 0.07% | 101% | 104% | 0.02% | -0.09% | -0.10% | 100% | 101% |
| 2-4.5c | 0.03% | 0.01% | 0.06% | 103% | 104% | 0.04% | 0.09% | -0.28% | 100% | 98% |
| 2-4.6 | 0.08% | -0.05% | -0.01% | 101% | 100% | 0.11% | 0.16% | -0.15% | 101% | 100% |
| 2-4.7a | 0.01% | -0.04% | -0.08% | 98% | 99% | 0.00% | -0.05% | -0.01% | 99% | 99% |
| 2-4.7b | 0.00% | -0.05% | -0.07% | 99% | 99% | -0.01% | 0.07% | -0.05% | 99% | 100% |
| 2-4.8a | 0.02% | 0.00% | 0.03% | 95%* | 97%* | 0.00% | 0.18% | -0.19% | 92%* | 94%* |
| 2-4.8b | -0.01% | -0.01% | 0.01% | 98%* | 99%* | 0.01% | 0.17% | -0.26% | 102%* | 100%* |
| 2-4.9b-1 | 0.09% | 0.11% | 0.18% | 102% | 96% | 0.09% | 0.35% | 0.12% | 101% | 96% |
| 2-4.9b-2 | 0.06% | 0.15% | 0.21% | 98% | 100% | 0.07% | 0.17% | 0.17% | 100% | 102% |
| 2-4.10a | -0.01% | 0.02% | 0.00% | 100% | 100% | -0.02% | 0.21% | 0.01% | 101% | 100% |
| 2-4.10b | -0.01% | -0.02% | -0.04% | 100% | 100% | 0.00% | 0.10% | 0.08% | 100% | 99% |
| 2-4.10c | 0.00% | 0.01% | 0.00% | 100% | 100% | -0.01% | 0.13% | 0.04% | 99% | 99% |
| * It is reported from proponents that the encoding and decoding runtime of respective tests may not be reliable. |  |  |  |  |  |  |  |  |  |  |

One category of these tests is reducing computation for subblock MC (e.g. using shorter filters on small blocks), others affect memory bandwidth and the number of fetches from cache memory.

- 2-4.6 (uses shorter filters for all 4x4 subblocks for regular inter and affine)

- 2-4.7x (uses padding for all 4xN & Nx4 subblocks with N<32 only for regular inter)

- 2-4.9b-x (applying integer MC for 4x8 and 8x4 bipred and enable 4x4 integer MC bipred in regular inter) use shorter filters for smaller blocks.

It was commented that the loss in high resolutions for 2-4.6 is very small.

It was commented that eliminating the use of 4x4 for affine might cause visual artefacts.

It was commented that 4x4 bipred with short filters requires less computation than 8x8 bipred.

It was commented that for some non-CTC “heavily affine” test sequences (e.g., SpinCalendar), using shorter filters has a significant penalty.

It was noted that DMVR has some padding, although only for large blocks.

For 2-4.9b-x, it was commented that removing the 4x4 bipred from it has negligible loss (e.g., 0.01%).

It was said that actually just removing 4x4 unipred, and 4x8/8x4 bipred regular inter modes (as in HEVC) is reported in JVET-N0266 and has a very similar penalty (0.08% for RA, 0.11% for LB) as the integer MC proposal 2-4.9b-x. However, it was commented that this could have a penalty for SCC.

Decision (complexity reduction): Adopt JVET-N0266 test 2, to remove 4x4 unipred, and 4x8/8x4 bipred regular inter modes (as in HEVC) (pending text check).

It was commented that adopting this might remove the need for shared merge candidates from the regular merge list. The coding efficiency impact of this was reported to be negligible. Decision (cleanup): Remove the use of for shared merge candidates in the regular merge list. (The text impact is a simplification and will be provided in a revision of JVET-N0266.)

Shared merge candidates are still used for the IBC merge list.

This action does not address the computational issue with affine mode.

Further testing of 2-4.6 during the meeting, relative to the JVET-N0266 anchor and relative to a JVET-N0266 anchor with affine entirely disabled, was suggested. Such results were requested.

Testing for non-CTC sequences (e.g., SpinCalendar) was also suggested. Some participants thought the CTC adequate, as the CTC includes some affine-benefitting sequences with rotation (CatRobot and Cactus) or other affine effects (e.g. MarketPlace, ParkRunning3, DaylightRoad2).

It was agreed that if test results for some non-CTC sequences also become available, they could also be reviewed.

Revisit for review of test results.

(Note that IBC still supports 4x4. Note that the current name is IBC, not CPR.)

Moving on to memory bandwidth consideration, the above action helps already. Memory bandwidth approaches for affine mode were then further discussed.

Complexity analysis

| Test# | Bandwidth measurement for uni/bi-prediction | Operations of SB MV derivation per 8x8 block (e.g. clipping) | Complexity of MC per 8x8 bi-prediction: Multiplication, addition | Others (additional operations per CU to determine SB size, MC fetching unit) Multiplication, addition, comparison, look-up size |
| --- | --- | --- | --- | --- |
| Inter | 10.69 / 14.69 | n/a | 2944+352=3296 Mul. 2704+296=3000 Add | no |
| Affine | 9.09 / 18.19 | no clipping | 960*4+352=4192 Mul. 872*4+296=3784 Add | no |
| 2-4.1 |  |  |  |  |
| 2-4.2a | 9.09 / 13.38 | same | 1920*2+352=4192 Mul. 1744*2+296=3784 Add | 1 comparison per CU (1 for uni/bi) |
| 2-4.2b | 9.09 / 13.38 | same | 1472*2+352=3296 Mul. 1352*2+296= 3000 Add | 1 comparison per CU (1 for uni/bi) |
| 2-4.2c | 9.09 / 13.38 | same | 1920*2+352=4192 Mul. 1744*2+296=3784 Add | 2 comparisons per CU (1 for width/height + 1 for uni/bi) |
| 2-4.2d | 9.09 / 11.06 | same | 2944+352=3296 Mul. 2704+296=3000 Add | 1 comparison per CU (1 for uni/bi) |
| 2-4.3 | 6.59 /13.19 | same | 4*624+352 = 2848 Mul. 4*552+296 = 2504 Add | same |
| 2-4.4a | 5.28 /10.56 | 15 comparisons for each 8x8 block, may be performed in parallel | same | no |
| 2-4.4b | 5.53 / 11.06 | 15 comparisons for each 8x8 block, may be performed in parallel | same | no |
| 2-4.4c | 5.53 / 11.06 | 14 comparisons for each 8x8 block, may be performed in parallel | same | no |
| 2-4.5a | 9.09 / 11.06 | no change | same | 13 comparisons per CU (1 for uni/bi + 12 for restriction) |
| 2-4.5b | 9.09 / 11.07 | no change | same | 14 comparisons per CU (1 for uni/bi + 12 for restriction) |
| 2-4.5c | 9.09 / 11.08 | no change | same | no |
| 2-4.6 |  |  |  |  |
| 2-4.7a | 15.25 / 20.19 | no change | same | no |
| 2-4.7b | 15.25 / 22.25 | no change | same | no |
| 2-4.8a | See JVET-N0068 |  |  |  |
| 2-4.8b | See JVET-N0068 |  |  |  |
| 2-4.9a | See JVET-N0068 |  |  |  |
| 2-4.9b-1 | 10.69 / 10.09 | no change | same as 8x8 bi-prediction | no change |
| 2-4.9b-2 | 10.69 /10.09 | no change | same as 8x8 bi-prediction | no change |
| 2-4.10a | no change | no change | same | no |
| 2-4.10b | no change | no change | same | no |
| 2-4.10c | no change | no change | same | no |

Relevant approaches were suggested to be 2-4.2, 2-4.3, 2-4.4, 2-4.5, 2-4.8

The CE plan said to use the memory cache analysis model, but the data provided for the complexity analysis tabulated above did not do that.

2-4.8b computes a bounding box and prevents affine motion from referencing outside the box by replacing the affine model with a single MV for the whole CU if a constraint is violated.

2-4.4 limits the range of motion vectors within each 8x8 area so the integer part differs by at most 1

2-4.5 is similar in spirit to 2-4.8 but has a different fallback

2-4.10 imposes an encoder constraint

It seemed clear that some approach in this area can effectively fix the memory bandwidth problem that could be caused in affine mode with “evil” data, with little loss of coding efficiency.

It was said that 2-4.8 and 2-4.4 are roughly similar for 8x8 PUs. For larger PUs one of them uses a whole-PU operation where the other operates separately in each 8x8 region. The worst case is the 8x8 case regardless.

It was commented that with a sufficiently “evil” encoder, the MVs of different 8x8 regions in a larger PU could be very disjoint, and that the behaviour of 2-4.8 can prevent that, while 2-4.4 does not.

However, there would still be a worst case with 8x8 bipred PUs either way.

There was substantial nonproponent support for 2-4.8b.

Decision (complexity reduction): Adopt 2-4.8b (with a max difference of 2 integer positions).

Without a cache model the worst case memory bandwith per luma sample for this was reported to be 17x17/64.

One more subsection remained in CE2 for review as of Tuesday 2200, targeted at memory reduction.

Discussion resumed Wednesday 0900.

There was discussion of the allowed range of MVD. No action appeared to be needed, as the draft already constrains this. The resulting MVs are also modulo wrapped to prevent them from having too large a range.

CE2-5: Storage Reduction for Sub-block Modes

| Test# | Description | Document# |
| --- | --- | --- |
| 2-5.1a | Replace spatial affine inheritance with inheritance from affine HMVP | JVET-N0377 |
| 2-5.1b | Test 2-5.1a + affine inheritance from line buffer |  |
| 2-5.2a | Directly derive sub-block MV by using affine inheritance merge candidate | JVET-N0257 |
| 2-5.2b | Test 2-5.2a + Affine inheritance using sub-block MVs |  |
| 2-5.2c | Test 2-5.2b + Shifting affine CPMVs by (2,2) |  |
| 2-5.2d | Test 2-5.2d + Changing neighboring block checking order for top-right and bottom-left CPMVs prediction in the constructed affine MVP |  |
| 2-5.2e | Test 2-5.2d + 16x16 grid affine inheritance |  |
| 2-5.3 | 2 or 3 sub-block MVs with half CU height or half CU width for affine inheritance | JVET-N0076 |
| 2-5.4 | Affine parameters storage to replace CPMVs and the block dimensions storage | JVET-N0272 |
| 2-5.5a | Parameter-based HMVP affine candidates | JVET-N0263 |
| 2-5.5b | Test 2-5.5a + 2.5.1.a |  |
| 2-5.5c | Test 2-5.5a + 2.5.1.b |  |
| 2-5.6a | Test 2-5.3 + Two subblock MVs with a fixed distance (4) for affine inheritance | JVET-N0078 |
| 2-5.6b | Test 2-5.4 + Two subblock MVs with a fixed distance (8) for affine inheritance |  |
| 2-5.6c | Test 2-5.4 + Two subblock MVs with a fixed distance (16) for affine inheritance |  |

For CE2-5.3, it was noted that the CE description had an error in a diagram, as the proposal had later been modified to operate differently from what was shown in the figure.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2-5.1a | 0.08% | 0.00% | 0.02% | 100% | 100% | 0.09% | 0.10% | -0.22% | 100% | 98% |
| 2-5.1b | 0.03% | 0.00% | -0.02% | 99% | 100% | 0.05% | 0.05% | 0.08% | 100% | 98% |
| 2-5.2a | 0.00% | -0.06% | -0.02% | 100% | 100% | 0.00% | 0.18% | -0.39% | 100% | 100% |
| 2-5.2b | 0.01% | -0.08% | -0.05% | 100% | 100% | 0.01% | -0.05% | -0.10% | 100% | 99% |
| 2-5.2c | 0.03% | -0.04% | 0.05% | 100% | 100% | 0.04% | 0.22% | -0.17% | 101% | 99% |
| 2-5.2d | 0.01% | 0.01% | 0.02% | 1001% | 1001% | 0.01% | -0.04% | -0.11% | 100% | 99% |
| 2-5.2e | 0.02% | -0.01% | 0.05% | 100% | 100% | 0.01% | 0.04% | -0.04% | 100% | 100% |
| 2-5.3 | 0.01% | -0.07% | -0.02% | 100% | 100% | 0.02% | 0.13% | -0.32% | 99% | 99% |
| 2-5.4 | 0.02% | -0.04% | 0.01% | 100% | 100% | 0.00% | 0.02% | -0.14% | 100% | 100% |
| 2-5.5a | -0.06% | -0.07% | -0.08% | 101% | 101% | -0.02% | -0.13% | -0.12% | 100% | 101% |
| 2-5.5b | 0.00% | -0.04% | -0.02% | 100% | 100% | 0.10% | 0.07% | -0.07% | 100% | 99% |
| 2-5.5c | -0.05% | -0.11% | -0.12% | 100% | 100% | 0.02% | -0.02% | -0.13% | 99% | 99% |
| 2-5.6a | 0.02% | -0.05% | -0.04% | 100% | 100% | 0.01% | -0.12% | 0.03% | 99% | 101% |
| 2-5.6b | 0.00% | -0.08% | 0.04% | 100% | 100% | 0.02% | -0.02% | 0.10% | 100% | 99% |
| 2-5.6c | 0.00% | -0.04% | -0.01% | 100% | 99% | -0.02% | 0.03% | -0.25% | 100% | 101% |

Memory reduction is summarized in the table below.

| Test# | Local storage (bits per CTU: 36 bits per CPMV, 3 bits for CU width, 3 bits for CU height, 10 bits for CU top-left x and y, 1 bit for affine type) | Line buffer for (bits per 8x8) | Additional logic |
| --- | --- | --- | --- |
| VTM4.0 | 11072 = 128/8 * (36*3*2+5+10+1) + 128/8 * 2*(36*3*2+3*1+10+1) | 8 (3 bits for width, 5 bits for CU x position) |  |
| 2-5.1a | 1165 = (5*(36*3*2+3*2+10+1)) | 0 bit | Affine HMVP table update |
| 2-5.1b | 1165 = (5*(36*3*2+3*2+10+1)) | Same | Affine HMVP table update |
| 2-5.2a | 11072 =128/8 * (36*3*2+5+10+1) + 128/8 * 2*(36*3*2+3*1+10+1) | Same | No |
| 2-5.2b | 3152 = ((128/8*2+2) * 36*2)+128/8 * (5+10+1) + 128/8 * 2*(3*1+10+1) | Same | No |
| 2-5.2c | 3152 = ((128/8*2+2) * 36*2)+128/8 * (5+10+1) + 128/8 * 2*(3*1+10+1) | Same | No |
| 2-5.2d | 3152 = ((128/8*2+2) * 36*2)+128/8 * (5+10+1) + 128/8 * 2*(3*1+10+1) | Same | No |
| 2-5.2e | 1648 = (128/16*2+2) * 36*2)+128/16 * (5+10+1) + 128/16 * 2*(3*1+10+1) | 8 per 16x16 | No |
| 2-5.3 | 4160 =128/8 * (36*2+5+10+1) + 128/8 * 2*(36*2+3*1+10+1) | Same | No |
| 2-5.4 | 3072 = 48*(8*4*2) | Same | No |
| 2-5.5a | 11392 = 320 + 128/8 * (36*3*2+5+10+1) + 128/8 * 2*(36*3*2+3*1+10+1) | Same | Affine HMVP table update (no pruning |
| 2-5.5b | 320 = (10*4*8) | 0 | Affine HMVP table update (no pruning |
| 2-5.5c | 320 = (10*4*8) | Same | Affine HMVP table update (no pruning |
| 2-5.6a | Same | 0 | No |
| 2-5.6b | Same |  |  |
| 2-5.6c | Same | 0 | No |

The current VTM 4 requires about 50% extra memory (~10k bits / 1384 bytes) of local buffer for affine CPMV storage in addition to what is needed for motion data storage without affine. The number would be larger for software implementations ~7888 bytes of local cache, perhaps 75% rather than 50%.

Additionally, 2-5.1a, 2-5.5b and 2-5.6x remove usage of some information about the above CTU, which may reduce line buffering storage, depending on implementation.

Some proposals have extra logic introduced.

Revisit after offline study of the following:

- Three alternatives to each other:

  - 2-5.2b or a variant that removes the 2-5.2a part of it was suggested for focus. It reduces the memory needed and was suggested to have a consistent structure.

  - 2-5.5c was also suggested for focus. It has a larger memory reduction but has some additional logic, increasing the number of candidates and availability checks.

  - 2-5.1b was also suggested for focus, as it has a larger memory reduction than 2-5.2b and was said to have less additional logic than 2-5.5c.

- An orthogonal concept: 2-5.6a or b

It was also said that the current design is acceptable and “clean”/consistent, so no action may really be needed. The amount of memory reduction available from these schemes is not especially large relative to total requirements.

JVET-N0068 CE2: On restriction of memory bandwidth consumption of affine mode (CE2-4.8) [M. Zhou (Broadcom)]

JVET-N0075 CE2-2.1: Simplification of constructed affine merging candidate derivation [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0076 CE2-5.3: Simplifications for inherited affine merging candidates [Y.-L. Hsiao, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0078 CE2-5.6: Simplifications for inherited affine merging candidates at CTU row boundaries [Y.-L. Hsiao, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek), K. Zhang, L. Zhang, H. Liu, J. Xu (Bytedance)]

JVET-N0637 Crosscheck of JVET-N0078 (CE2-5.6: Simplifications for inherited affine merging candidates at CTU row boundaries) [H. Huang (Qualcomm)] [late]

JVET-N0195 CE2: Memory bandwidth reduction for the affine mode (Test 2.4.3) [J. Li, C.-W. Kuo, C. S.Lim (Panasonic)]

JVET-N0196 CE2: Using the shorter-tap filter for 4x4 sized partitions (Test 2.4.6) [J. Li, C.-W. Kuo, C. S.Lim (Panasonic)]

JVET-N0210 CE2-4.9: Integer motion compensation for regular inter mode [X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)]

JVET-N0253 CE2-4.7: Motion compensation with padded samples for small coding units [H. Liu, L. Zhang, J. Chon, H. Chuang, K. Zhang, J. Xu, Y. Wang (Bytedance)]

JVET-N0256 CE2: Worst-case Memory Bandwidth Reduction for affine (Test 2.4.5) [H. Huang, L. Pham Van, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-N0257 CE2: Alignment of affine control-point motion vector and subblock motion vector (Test 2.5.2) [H. Huang, W.-J. Chien, V. Seregin, H. Wang, M. Karczewicz (Qualcomm)]

JVET-N0616 Crosscheck of JVET-N0257 (CE2: Alignment of affine control-point motion vector and subblock motion vector (Test 2.5.2)) [H. Chen (Huawei)] [late]

JVET-N0261 CE2-1.1: Interweaved Prediction for Affine Motion Compensation [K. Zhang (Bytedance)]

JVET-N0687 Cross-check of JVET-N0261 (CE2-1.1d: Interweaved Prediction for Affine Motion Compensation) [J. Li, C.-W. Kuo, C. S.Lim (Panasonic)] [miss] [late]

JVET-N0263 CE2-5.5: History-parameter-based affine model inheritance [K. Zhang (Bytedance)]

JVET-N0272 CE2-5.4: Parameter-based affine model inheritance [K. Zhang (Bytedance)]

JVET-N0319 CE2: Symmetric MVD for affine bi-prediction coding (CE2-1.2) [H. Yang, J. Luo, Y. He (InterDigital)]

JVET-N0323 CE2: Reducing worst-case memory bandwidth of affine mode (CE2-4.2.a, CE2-4.2.b, CE2-4.2.c and CE2-4.2.d) [Y.-W. Chen, X. Wang (Kwai Inc.), H. Chen, T. Solovyev, H. Yang, J. Chen (Huawei)]

JVET-N0377 CE2-5.1: Affine HMVP with modified affine inheritance [G. Li, X. Xu, X. Li, S. Liu (Tencent), J. Zhao, S. Kim (LGE)]

JVET-N0378 CE2-2.2: Affine merge with prediction offset [G. Li, X. Xu, X. Li, S. Liu (Tencent)]

JVET-N0379 CE2-4.10: Constraint on constructed affine MV range [G. Li, X. Xu, X. Li, S. Liu (Tencent)]

JVET-N0529 Cross-check of JVET- N0379 (CE2-4.10: Constraint on constructed affine MV range) [J. Zhao (LGE)] [late]

JVET-N0398 CE2-4.4: Affine block memory bandwidth reduction by MV clip [X. Li, G. Li, X. Xu, S. Liu (Tencent)]

## CE3: Intra prediction and mode coding (19)

Contributions in this category were discussed Wednesday 20 March 0900–1345 (Track A chaired by JRO).

JVET-N0023 CE3: Summary Report on Intra Prediction and Mode Coding [G. Van der Auwera, L. Li, A. Filippov]

This is the summary report of the third Core Experiment (CE3) [1]. The goal of CE3 is to study intra prediction tools, including mode coding, for potential inclusion into the VVC standard.

The following is the list of defined sub-tests in CE3:

- CE3-1: Cross-component prediction (18 tests)

- CE3-2: Small block size restrictions (5 tests)

- CE3-3: MPM list harmonization (12 tests)

- CE3-4: Affine linear weighted intra prediction (2 tests)

This document summarizes the objective results (BD-rates, runtimes), cross-check reports, and related input contributions.

CTC conditions as agreed at the 13th JVET meeting (Marrakech, MA) for SDR (JVET-M1010) are followed for the evaluation of objective performance. The anchor is the ‘VVC Test Model’ or VTM. Results are provided for the ‘All Intra’ and ‘Random Access’ test configurations for all sub-tests, and additionally the ‘Low Delay B’ configuration for sub-test CE3-2. The VTM version 4.0 software is used with ‘vtm’ configuration files to produce the VTM anchor. Tools are implemented into the VTM version 4.0 software.

In addition to VTM-4.0, a modified codebase is also used for evaluating the tests in CE3-3. In the codebase, a macro is added [1] on top of VTM-4.0 to always include all MPMs (6 MPMs in VTM-4.0) for full encoder R-D checking in intra mode decision. For evaluating the coding performance of each CE3-3 test, following Test A and Test B results are reported:

Test A:

- Anchor: VTM-4.0

- Tested: VTM-4.0 with test method

Test B:

- Anchor: Codebase

- Tested: Codebase with test method

Separate test results (A and B) are provided for modifications to the encoder search algorithm other than the one described above.

Proposals are compared with respect to objective compression efficiency while complexity is also assessed.

3-1, category on CCLM simplifications:

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-1.1.1 | Selection of the up to 6 samples is applied to the down-sampled luma samples | JVET-N0387 (Technicolor) |
| 3-1.2.1 | Reduction of reference samples to N points for CCLM (CCLM_LT, CCLM_T, and CCLM_L), using an offset, with N being set according to the size of the block | JVET-N0131 (Xidian Univ., NPU, OPPO) |
| 3-1.2.2 | Reduction of reference samples to N points for CCLM (CCLM_LT, CCLM_T, and CCLM_L), using an offset, with N being set fixed (i.e., uniform for all block sizes) |  |
| 3-1.3.1 | Skipping initial P reference samples for CCLM_L and CCLM_T, with P being set according to the size of the block | JVET-N0130 (NPU, Xidian Univ., OPPO) |
| 3-1.3.2 | Test 3-1.3.1 + Test 3-1.2.1 |  |
| 3-1.3.3 | Test 3-1.3.1 + Test 3-1.2.2 |  |
| 3-1.4.1 | CCLM parameter calculation with half number of reference samples | JVET-N0228 (LGE) |
| 3-1.4.2 | CCLM parameter calculation with half and upper-limited reference samples |  |
| 3-1.5 | CCLM model derived from four neighbouring luma samples | JVET-N0271 (Bytedance, Peking Univ.) |
| 3-1.6.1 | Identify the min/max luma samples without down-sampling process; apply the 6-taps filter on the identified min/max samples to generate the filtered (down-sampled) min/max luma samples for the derivation of α and β. | JVET-N0321 (Kwai) |
| 3-1.6.2 | Combination of test 3-1.6.1 with JVET-M0108 (test 3-1.1.1) |  |
| 3-1.6.3 | Combination of test 3-1.6.1 with JVET-M0211 (test 3-1.2.2) |  |
| 3-1.6.4 | Combination of test 3-1.6.1 with JVET-M0211+JVET-M0212 (test 3-1.3.3) |  |
| 3-1.6.5 | Combination of test 3-1.6.1 with JVET-M0219 (test 3-1.4.2) |  |
| 3-1.6.6 | Combination of test 3-1.6.1 with JVET-M0274 (test 3-1.5) |  |

An overview of the tested methods for reducing reference samples used in calculation of CCLM model parameters is provided in following table:

| Test # | #ref samples LT_CCLM | #ref_samples T_CCLM | #ref_samples L_CCLM | Locations | #comparison operations |
| --- | --- | --- | --- | --- | --- |
| VTM4 | Up to 2*min(W,H) (if avail.) | Up to 2*W (if avail.) | Up to 2*H (if avail.) | Note: W, H are chroma TB dimensions | 2*#ref_samples |
| 3-1.1.1 | 6 (3 top line, 3 left) | 3 | 3 | #ref samples LT_CCLM = nS_T + nS_L #ref_samples T_CCLM = nS_T #ref_samples L_CCLM = nS_L Top: (0, – 1), (nS_T – 1, – 1), (nS_T/2, – 1) Left: (– 1, 0), (– 1, nS_L – 1), (– 1, nS_L/2) |  |
| 3-1.2.1 | 2 (min(W,H) == 2) or 4 | 2 (W == 2) or 4 | 2 (H == 2) or 4 | Example: -Original reference samples number: L=16 -Reduced reference samples number: N=4 -Sampling interval: ∆=L/N=4 -Starting offset = ∆/2 =2 | 4 (min(W,H) == 2) or 8 |
| 3-1.2.2 | 4 | 4 | 4 |  | 8 |
| 3-1.3.1 | Idem VTM4 | Up to ¾ (2*W) | Up to ¾ (2*H) | Skipping initial quarter of reference samples for CCLM_L and CCLM_T (VTM4) | -LT_CCLM: Idem VTM4 -L_CCLM: Up to 3W -T_CCLM: Up to 3H |
| 3-1.3.2 | 4 | 2 (if after offset, #samples remaining is <4) or 4 | 2 (if after offset, #samples remaining is <4) or 4 | -LT_CCLM: 4 -L_CCLM and _T_CCLM: Initial offset is set as the first quarter of the reference samples (skipped). Then, reference samples are selected according to the method in 3-1.2.1 | Up to 8 |
| 3-1.3.3 | 4 | 4 | 4 | -LT_CCLM: 4 -L_CCLM and _T_CCLM: Initial offset is set as the first quarter of the reference samples (skipped). Then, reference samples are selected according to the method in 3-1.2.2 | 8 |
| 3-1.4.1 | Up to min(W,H) (if avail.) | Up to W (if avail.) | Up to H (if avail.) | Half of VTM4 reference samples are used | #ref_samples |
| 3-1.4.2 | Up to min(W,H) (if avail.) (max. 8) | Up to W (if avail.) (max. 8) | Up to H (if avail.) (max. 8) | Half of VTM4 reference samples are used, up to 8 max. | #ref_samples (up to 16) |
| 3-1.5 | 4 | 4 | 4 | - LT_CCLM: W’=W, H’=H - T_CCLM: W’=W+H - L_CCLM: H’=H+W 4 samples are selected as: - LT_CCLM: S[W’/4, -1], S[3W’/4, -1], S[-1, H’/4], S[-1, 3H’/4] - T_CCLM: S[W’/8, -1], S[3W’/8, -1], S[5W’/8, -1], S[7W’/8, -1] - L_CCLM: S[-1, H’/8], S[-1, 3H’/8], S[-1, 5H’/8], S[-1, 7H’/8] 4 neighbouring luma samples at the selected positions are down-sampled and compared four times to find two smaller values: x0A and x1A, and two larger values: x0B and x1B. Corresponding chroma sample values are denoted as y0A, y1A, y0B and y1B. Then xA, xB, yA and yB are derived as for model calc.: xA=(x0A + x1A +1)>>1; xB=(x0B + x1B +1)>>1; yA=(y0A + y1A +1)>>1; yB=(y0B + y1B +1)>>1. | 4 |
| 3-1.6.1 | Method applied to VTM4 |  |  | Reconstructed luma samples without down-sample filtering to find the min/max luma samples, followed by filtering before model parameter calc. | Same as VTM4 |
| 3-1.6.2 | Method applied to test 3-1.1.1 |  |  |  | Same as 3-1.1.1 |
| 3-1.6.3 | Method applied to test 3-1.2.2 |  |  |  | Same as 3-1.2.2 |
| 3-1.6.4 | Method applied to test 3-1.3.3 |  |  |  | Same as 3-1.3.3 |
| 3-1.6.5 | Method applied to test 3-1.4.2 |  |  |  | Same as 3-1.4.2 |
| 3-1.6.6 | Method applied to test 3-1.5 |  |  |  | Same as 3-1.5 |

Continuation of above table:

| Test # | # comparison operation in worst case | # downsampling samples in worst case |
| --- | --- | --- |
| VTM4 | 8 | 4 |
| 3-1.1.1 | 5 | 4 |
| 3-1.2.1 | 4 | 4 |
| 3-1.2.2 | 8 | 4 |
| 3-1.3.1 | 8 | 4 |
| 3-1.3.2 | 4 | 4 |
| 3-1.3.3 | 8 | 4 |
| 3-1.4.1 | 4 | 2 |
| 3-1.4.2 | 4 | 2 |
| 3-1.5 | 4 | 4 |
| 3-1.6.1 | 8 | 2 |
| 3-1.6.2 | 8 | 2 |
| 3-1.6.3 | 8 | 2 |
| 3-1.6.4 | 8 | 2 |
| 3-1.6.5 | 4 | 2 |
| 3-1.6.6 | 4 | 0 |

The worst case above relates to a unit of 2x2 chroma block.

This would also be resolved by disallowing 2x2 chroma blocks (as investigated in CE3-2).

|  | All Intra Main10 - Over VTM-4.0 |  |  |  |  | Random Access Main10 - Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-1.1.1 | 0.08% | 0.14% | 0.17% | 98% | 97% | 0.02% | -0.02% | 0.01% | 100% | 99% |
| 3-1.2.1 | -0.03% | -0.11% | -0.21% | 100% | 100% | -0.01% | -0.24% | -0.27% | 100% | 100% |
| 3-1.2.2 | -0.03% | -0.24% | -0.31% | 100% | 99% | -0.01% | -0.35% | -0.46% | 100% | 100% |
| 3-1.3.1 | -0.02% | -0.06% | -0.13% | 100% | 99% | -0.01% | -0.11% | -0.20% | 100% | 100% |
| 3-1.3.2 | -0.02% | 0.02% | -0.08% | 100% | 99% | 0.01% | -0.07% | -0.18% | 100% | 100% |
| 3-1.3.3 | -0.03% | -0.06% | -0.17% | 100% | 99% | -0.01% | -0.23% | -0.40% | 100% | 100% |
| 3-1.4.1 | 0.00% | 0.07% | 0.03% | 100% | 100% | 0.00% | 0.08% | -0.04% | 101% | 101% |
| 3-1.4.2 | 0.00% | 0.04% | 0.01% | 101% | 100% | 0.01% | 0.04% | -0.12% | 101% | 101% |
| 3-1.5 | -0.05% | -0.30% | -0.38% | 99% | 100% | -0.05% | -0.36% | -0.63% | 100% | 100% |
| 3-1.6.1 | 0.02% | 0.19% | 0.22% | 99% | 97% | 0.02% | 0.22% | 0.14% | 98% | 98% |
| 3-1.6.2 | 0.05% | 0.32% | 0.33% | 99% | 98% | 0.02% | 0.16% | 0.16% | 99% | 99% |
| 3-1.6.3 | -0.02% | -0.07% | -0.13% | 100% | 98% | 0.00% | -0.18% | -0.27% | 100% | 99% |
| 3-1.6.4 | -0.02% | 0.08% | -0.05% | 100% | 99% | 0.00% | -0.02% | -0.13% | 99% | 98% |
| 3-1.6.5 | 0.01% | 0.18% | 0.13% | 100% | 100% | 0.02% | 0.11% | 0.08% | 99% | 99% |
| 3-1.6.6 | 0.00% | 0.20% | 0.08% | 101% | 100% | 0.00% | 0.20% | -0.01% | 100% | 100% |

There is not really a problem in terms of complexity, but simplifying things and unifying logic across different block sizes may be desirable.

Switching to different logic for different block sizes may not always be desirable

There is an issue in VTM with large block sizes in that the number of comparisons scales up with the number of reference samples, which may not be necessary

3-1.2.2 and 3-1.5 are using the same (or at least similar) locations of filtering (4 positions), are unifications and simplifications according to the aspects above, where 3-1.2.2 has more comparisons, and 3-1.5 replaces some comparison by additional averaging operation. 3-1.5 has a very small gain.

3-1.6.1 through 3-1-6.5 switches min/max and filtering, which may be undesirable due to the fact that the result of min/max imposes which samples to access for filtering.

3-1.6.6 completely omits the filtering, and combines it with the averaging operation of 3.1.5. Some concern expressed this may not be desirable, as there is some indication of minor losses of 0.05%, and the filtering is very regular operation, no concern of complexity)

3-1.6.6 is the simplest solution (no filtering), 3-1.2.2 and 3.1.5 are also clear simplifications relative to current VTM, have comparable complexity.

3-1.5 is the best solution in this group (candidate for adoption).

Decision: Adopt JVET-N0271 (Revisit: Cross-checker to confirm appropriateness of text)

CE3-1, category on multi-model LM:

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-1.7 | Multiple-model LM with small block size restriction | JVET-N0264 (Foxconn, Qualcomm) |
| 3-1.8.1 | Multiple-model LM using piecewise linear model and block size restriction | JVET-N0241 (Canon) |
| 3-1.8.2 | More stringent block size restriction on CCLMs with JVET-M0384 included. |  |

Summary of the tested methods:

|  | CCLM | CE3-1.7 | CE3-1.8.1 | CE3-1.8.2 |
| --- | --- | --- | --- | --- |
| Signalling | 3 modes | 6 modes | 3 modes (CCLM or MMLM depending on block size) | 3 modes (CCLM or MMLM depending on block size) |
| Number of lines used for model derivation | Y: 2 lines above, 3 lines left, C: 1 line above and left); 1 line above at CTU boundary | idem | idem | idem |
| Chroma block sizes applicable | nSamples >= 4 (all block sizes) | CCLM: nSamples >= 4 (all sizes) MMLM: nSamples > 16 | CCLM: 4 <= nSamples < 16 MMLM: nSamples >= 16 | CCLM: 16 <= nSamples < 64 MMLM: nSamples >= 64 |

|  | All Intra Main10 - Over VTM-4.0 |  |  |  |  | Random Access Main10 - Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-1.7 | -0.27% | -1.56% | -2.13% | 100% | 100% | -0.13% | -1.29% | -1.75% | 100% | 100% |
| 3-1.8.1 | -0.30% | -0.97% | -1.45% | 100% | 102% | -0.16% | -0.97% | -1.39% | 100% | 111% |
| 3-1.8.2 | -0.26% | -0.63% | -0.99% | 100% | 104% | -0.11% | -0.47% | -0.70% | 100% | 111% |

MMLM requires computation of two models, plus an additional classification of samples to determine the two groups of samples from which the models are computed.

Also during prediction, a classification is necessary to determine which model is applied to a given chroma sample.

To keep the worst case complexity, MMLM is only applied to larger blocks (larger than 16 in 3-1.7, larger/equal 16 in 3-1.8). 3-1.8 reuses the current signalling, enforces MMLM always to be used for larger blocks.

The additional gain over CCLM is not large enough to justify the additional complexity. No action on multi-model.

CE3-2: Small block size restrictions

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-2.1.1 | Disable 2x2/4x2/2x4 in dual tree only | JVET-N0137 (Sharp) |
| 3-2.1.2 | Disable 2x2/4x2/2x4 in dual tree and use DC only for 2x2/4x2/2x4 intra chroma prediction |  |
| 3-2.1.3 | 3-2.1.1 + 3-2.2 | JVET-N0140 (Sharp, MediaTek) |
| 3-2.2 | Share reference samples for at least 16 chroma samples in both dual and shared tree | JVET-N0081 (MediaTek) |
| 3-2.3 | Disable intra chroma 2x2, 2x4 and 4x2 in dual tree. And use a 2x8 or 8x2 coefficient group size instead of a 2x2 coefficient group size on remaining intra chroma blocks which has width or height size as 2. | JVET-N0102 (Canon) |

Summary table of the methods per tree type:

| Test | Shared trees | Separated trees |
| --- | --- | --- |
| 3-2.1.1 |  | Disables 2x2, 2x4, and 4x2 |
| 3-2.1.2 | DC mode for small blocks | Disables 2x2, 2x4, and 4x2 |
| 3-2.1.3 | Shared reference sample in parallel regions with minimum size of 16 | Disables 2x2, 2x4, and 4x2 |
| 3-2.2 | Shared reference sample such that a region with minimum size of 16 can be processed in parallel |  |
| 3-2.3 | Blocks of width or height of 2 use a 2x8 or 8x2 coefficient group size, respectively | Disables 2x2, 2x4, and 4x2 |

|  | All Intra Main10 - Over VTM-4.0 |  |  |  |  | Random Access Main10 - Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-2.1.1 | 0.02% | 0.37% | 0.49% | 100% | 100% | 0.01% | 0.30% | 0.36% | 101% | 101% |
| 3-2.1.2 | 0.02% | 0.37% | 0.49% | 100% | 100% | 0.10% | 0.61% | 0.74% | 100% | 101% |
| 3-2.1.3 | 0.02% | 0.37% | 0.49% | 101% | 101% | 0.06% | 0.46% | 0.62% | 101% | 102% |
| 3-2.2 | 0.03% | 0.30% | 0.33% | 100% | 101% | 0.06% | 0.29% | 0.48% | 101% | 101% |
| 3-2.3 | 0.02% | 0.33% | 0.47% | 96% | 94% | 0.02% | 0.24% | 0.32% | 100% | 99% |

|  | Low Delay B Main10 - Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT |
| 3-2.1.1 | 0.00% | 0.18% | 0.15% | 100% | 101% |
| 3-2.1.2 | 0.07% | 0.66% | 0.53% | 100% | 101% |
| 3-2.1.3 | 0.04% | 0.44% | 0.42% | 101% | 101% |
| 3-2.2 | 0.02% | 0.43% | -0.20% | 100% | 100% |
| 3-2.3 | -0.02% | 0.22% | 0.24% | 100% | 100% |

From the solutions investigated in CE, 3-2.1.3 is the best approach resolving the dependency issues of small chroma blocks. The compression loss would be in an acceptable range. There are however CE related contributions which might come with less loss. Revisit: Decide after reviewing CE realted contributions if adoption of 3-2.1.3 would be appropriate, or if it is better to wait for another CE cycle. Definitely, it is agreed that the issue needs to be resolved.

CE3-3: Harmonization of MPM list

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-3.1.1 | Unified MPM list generation with changed order to include tools such as MRL, intra sub-partition coding, etc. | JVET-N0184 (Huawei, LGE) |
| 3-3.1.2 | Harmonization of luma intra mode coding (including MRL, intra sub-partitioning, etc.) | JVET-N0451 (LGE, Huawei) |
| 3-3.2.1 | Simplification of MPM derivation | JVET-N0134 (ETRI) |
| 3-3.2.2 | Simplification of MPM derivation with changed order described in JVET-M0295 |  |
| 3-3.2.3 | Withdrawn |  |
| 3-3.3.1 | Unification of MPM list generation with prioritization of Planar mode in MPM list | JVET-N0393 (Tencent) |
| 3-3.3.2 | Unification of MPM list generation with Planar and DC modes being first two MPMs of zero reference line, each of the first two bins of MPM index is coded using one context |  |
| 3-3.3.3 | Unification of MPM list generation with Planar and DC modes being first two MPMs of zero reference line, the first bin of MPM index is coded using more than one context |  |
| 3-3.4.1 | Planar/DC coded as separate modes | JVET-N0436 (Sharp) |
| 3-3.4.2 | Same as previous test, but with modified encoder search described in JVET-M0528 |  |
| 3-3.4.3 | Planar/DC coded as separate modes, use 4MPMs described in JVET-M0494 |  |
| 3-3.5.1 | Explicitly signal Planar&DC MPM list and intra_luma_ref_idx signalling only for angular mode | JVET-N0104 (Fujitsu) Comment: uploaded late on 17 Mar. |
| 3-3.5.2 | Test 3.3.5.1, use 4MPMs described in JVET-M0494 |  |

It was commented on the CE3 participants email list that full test results were due by T3 (4 March 2019), and the following is an overview of lateness as provided by one test proponent:

CE3-3.1.1: March 13 (9 days late), uploaded document

CE3-3.1.2: March 17 (13 days late), uploaded document

CE3-3.2: March 6 (2 days late), uploaded to gitlab; bug fixed version (Mar. 20).

CE3-3.3: March 10 (6 days late), uploaded to gitlab; bug fixed version (Mar. 20).

CE3-3.4: March 4 (on time), uploaded to gitlab

CE3-3.5: March 17 (13 days late), uploaded document

The following table provides a summary of some aspects of the CE3-3 methods.

|  | VTM-4.0 | 3.1.1 | 3.1.2 | 3.2.1 | 3.2.2 | 3.3.1 | 3.3.2 | 3.3.3 | 3.4.1 | 3.4.2 | 3.4.3 | 3.5.1 | 3.5.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # MPMs (regular) | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 2+6 | 2+6 | 2+4 | 2+4 | 2+4 |
| # MPMs (MRL) | 6 | 5 | 4 | 6 | 6 | 4 | 4 | 4 | 6 | 6 | 4 | 6 | 4 |
| # MPMs (ISP) | 6 | 6 | 5 | 6 | 6 | 5 | 5 | 5 | 1+6 | 1+6 | 1+4 | 1+5 | 1+4 |
| Re-index if fewer | No | Yes | Yes | No | No | Yes | Yes | Yes | NA | NA | NA | NA | NA |
| Context-coded | No | Yes (*) (1st bin) | No | No | No | No | Yes (*) (1st, 2nd bins) | Yes (*) (1st, 2nd bins) | Yes (**) | Yes (**) | Yes (**) | Yes (**) | Yes (**) |
| # contexts added | - | 3 | - | - | - | - | 1+1 | 3+1 | 10+2 | 10+2 | 10+2 | 5+1 | 5+1 |

(*) mpm_idx

(**) angular flags and planar_or_dc_flag

| Test # | All Intra Main10 - Over VTM-4.0 |  |  |  |  | Random Access Main10 - Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-3.1.1 | -0.08% | -0.05% | -0.07% | 101% | 100% | -0.04% | 0.00% | -0.09% | 101% | 99% |
| 3-3.1.2 | -0.04% | 0.02% | 0.00% | 98% | 99% | 0.01% | 0.00% | -0.08% | 100% | 99% |
| 3-3.2.1 | 0.10% | 0.12% | 0.10% | 102% | 100% | 0.03% | 0.01% | -0.05% | 100% | 100% |
| 3-3.2.2 | -0.05% | 0.03% | 0.00% | 102% | 100% | -0.02% | 0.00% | -0.09% | 100% | 100% |
| 3-3.3.1 | -0.02% | 0.04% | 0.02% | 99% | 100% | 0.01% | 0.00% | -0.09% | 99% | 100% |
| 3-3.3.2 | -0.07% | -0.01% | 0.00% | 99% | 100% | -0.02% | -0.02% | -0.04% | 99% | 100% |
| 3-3.3.3 | -0.11% | -0.08% | -0.05% | 100% | 100% | -0.02% | -0.05% | 0.00% | 99% | 100% |
| 3-3.4.1 | -0.11% | -0.08% | -0.09% | 99% | 101% | -0.02% | -0.02% | -0.13% | 100% | 100% |
| 3-3.4.2 | -0.29% | -0.20% | -0.17% | 103% | 100% | -0.08% | -0.01% | -0.07% | 100% | 100% |
| 3-3.4.3 | -0.05% | -0.05% | -0.02% | 97% | 100% | 0.00% | -0.01% | -0.04% | 99% | 99% |
| 3-3.5.1 | -0.09% | -0.06% | -0.07% | 100% | 98% | -0.03% | -0.01% | -0.07% |  |  |
| 3-3.5.2 | -0.05% | -0.05% | -0.02% | 100% | 98% |  |  |  |  |  |

Currently, three independent processes are defined in spec and software for constructing MPM in cases of regular, multi-line and ISP intra prediction. All proposals are unifying this to a single MPM list construction, but some aspects may still be different, e.g. substitutions.

All proposals are then making the MPM index coding in some regard different for the three cases, e.g. by using different number of indexes, re-indexing, separating DC/planar signalling.

Some of the proposals are introducing additional context coded bins, e.g. for the additional DC/planar flag, or for MPM index.

In terms of results, two versions were investigated: “Test A” (called VTM4 intra search), “Test B” (called VTM4 intra search + 6 MPM in full RD check list). However, the original intent of using exactly the same encoder search for all proposals turned out to be not easy to follow, as obviously the way of constructing and coding an MPM list, and the decision of which modes would not be checked are interrelated. The appropriate way of resolving this might have been full search which is not practical. Therefore, Test B was defined believing that it would be closer to full search, but there is some debate among proponents about this, because some believe that the additional MPMs are not necessarily optimum for their proposals. So far, it has not been analysed in detail to which extent proposal-specific encoder optimization was used, and to which extent proposals have used their own MPMs preferentially, etc.

From the current results in compression performance, it is difficult to draw conclusions.

Currently, no proposal on the table is fulfilling the desirable amount of harmonization.

Ideally, a harmonization should use identical method of MPM list construction and identical method of MPM coding for all three cases, and come with no loss in compression performance and no increase in complexity.

There are various non-CE proposals which are verbally claimed to provide better solutions. Necessary to continue this CE, but also with better definition of test conditions to make proposals comparable, and complexity assessment.

BoG (G.v.d.Auwera) to further discuss this, and review the related proposals.

CE3-4 Affine weighted intra prediction

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-4.1 | Affine linear weighted intra prediction, i.e., to store all matrices and offsets needed, is restricted to be not more than 8 Kilobyte | JVET-N0217 (HHI) |
| 3-4.2 | Architectural variations of the affine linear weighted intra prediction will be studied which do not exceed the total memory requirement of 8 Kilobyte |  |

|  | All Intra Main10 - Over VTM-4.0 |  |  |  |  | Random Access Main10 - Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-4.1 | -0.79% | -0.27% | -0.28% | 138% | 99% | -0.42% | -0.63% | -0.64% | 104% | 99% |
| 3-4.2 | -0.82% | -0.24% | -0.32% | 141% | 99% | -0.44% | -0.54% | -0.63% | 105% | 101% |

It is noted that 3-4.2 is further optimization that was not in the original CE description.

No more than 4 multiplications per sample

Most concerns of hardware experts seem to be resolved

Further review necessary, some experts would like to study this further, including text, software, etc.

To be clarified how the tables should be conveyed in the spec.

Presentation should also report result and method of reduced-complexity encoder version

Revisit (presentation in plenary) – adopt or stop further investigating it.

JVET-N0081 CE3-2.2: Shared reference samples for multiple chroma intra CBs [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0102 CE3-2.3: Chroma block coding and size restriction [C. Rosewarne, J. Gan (Canon)]

JVET-N0104 CE3-3.5 Explicitly signal non angular modes in intra mode coding [J. Yao, J. Zhu, W. Cai, K. Kazui (Fujitsu)] [late]

JVET-N0130 CE3: Reduced reference samples range for CCLM_L and CCLM_T (Tests 1.3.1, 1.3.2 and 1.3.3) [S. Wan (NPU), Q.-H. Ran, X.-W. Li, J.-Y. Huo, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), Y.-F. Yu, Y. Liu (OPPO)]

JVET-N0131 CE3: Offset-based reference sample reduction for CCLM (Tests 1.2.1 and 1.2.2) [J.-Y. Huo, X.-W. Li, J.-L. Wang, X.-Y. Chai, Y.-Z. Ma, F.-Z. Yang, B.-Z. Shen (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

JVET-N0134 CE3: Simplification of MPM derivation (CE3-3.2) [J. Lee, H. Lee, S.-C. Lim, J. Kang, H. Y. Kim (ETRI)]

JVET-N0137 CE3-2.1: Intra chroma partitioning and prediction [T. Zhou, T. Ikai (Sharp)]

JVET-N0140 CE3-2.1.3: Combination of CE3-2.1.1 and CE3-2.2 [T. Zhou, T. Ikai (Sharp), Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0184 CE3-3.1.1: Unified MPM list generation with changed order to include tools such as MRL, intra sub-partition coding, etc. [B. Wang, A. M. Kotra, S. Esenlik, H. Gao, J. Chen (Huawei), L. Li, J. Heo, J. Choi, S. Yoo, J. Lim, S. Kim (LGE)]

JVET-N0217 CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2) [J. Pfaff, B. Stallenberger, M. Schäfer, P. Merkle, P. Helle, T. Hinz, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-N0582 Crosscheck of JVET-N0217 (CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)) [F. Racapé (Technicolor)] [late]

JVET-N0228 CE3: Reduced number of reference samples for CCLM parameter calculation (CE3-1.4.1 and CE3-1.4.2) [J. Choi, J. Heo, S. Yoo, L. Li, J. Choi, J. Lim, S. Kim (LGE)]

JVET-N0241 CE3: Results of Test CE3-1.8 on multiple model LM using piecewise linear model [C. Gisquet, G. Laroche, P. Onno, J. Taquet (Canon)]

JVET-N0264 CE3-1.7: Multiple-model LM with small block size restriction [P.-H. Lin (Foxconn), A. K. Ramasubramonian, G. Van der Auwera, T. Hsieh, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-N0271 CE3-1.5: CCLM derived from four neighbouring samples [M. Wang, K. Zhang (Bytedance)]

JVET-N0321 CE3: Simplification of LM mode (CE3-1.6) [Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-N0577 Crosscheck of JVET-N0321: CE3: Simplification of LM mode (Tests CE3-1.6.3 and CE3-1.6.5) [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, H. Huang, M. Karczewicz (Qualcomm)] [late]

JVET-N0387 CE3: report of CE3-1.1 on reduction of the number of reference samples in CCLM mode [E. François (Technicolor)]

JVET-N0393 CE3-3.3: MPM list harmonization [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-N0436 CE3-3.4.1, CE3-3.4.2, CE3-3.4.3: Results for unified MPM list construction process [F. Bossen, K. Misra (Sharp Labs of America)]

JVET-N0451 CE3-3.1.2: Harmonization on MPM list [J. Heo, L. Li, J. Choi, J. Choi, S. Yoo, J. Lim, S. Kim (LGE)]

## CE4: Inter prediction and motion vector coding (18)

JVET-N0024 CE4: Summary report on inter prediction and motion vector coding [H. Yang, G. Li, K. Zhang]

Discussed Wednesday 20 March 1115-1300 (GJS).

This contribution provides a summary report of Core Experiment 4 on inter prediction and motion vector coding. CE4 comprises 4 categories,

- Merge mode signalling

- STMVP

- MMVD modification

- TPM merge list modification

All techniques are implemented on top of and test against VTM-4.0. Simulation results and crosschecking reports of each test specified in this document are provided.

CE4-1: Merge mode signaling

| Test# | Source | Description |
| --- | --- | --- |
| 4-1.1 | JVET-N0141 | Signalling MMVD syntax elements when merge_flag equal to 0. |
| 4-1.2.a | JVET-N0324 | Signal a regular merge flag right after the merge flag and skip flag to indicate whether the regular merge/skip mode is used by the current CU |
| 4-1.2.b | JVET-N0324 | Combination of test 4-1.2.a with 4-1.1 |
| 4-1.3 | JVET-N0237 | Change the MMVD flag position, and remove unnecessary syntax element |
| 4-1.4.a | JVET-N0252 | Signal a merge_subblock_flag and a merge_triangle_flag consecutively |
| 4-1.4.b | JVET-N0252 | Combination of test 4-1.4.a with 4-1.1 |
| 4-1.4.c | JVET-N0252 | Combination of test 4-1.4.a with 4-1.3 |
| 4-1.5.a | JVET-N0212 | Remove TMVP merge candidate for the block size with samples <= 32 |
| 4-1.5.b | JVET-N0212 | Remove TMVP merge candidate for the block size with samples <= 64 |
| 4-1.5.c | JVET-N0212 | Remove TMVP merge candidate for the block size of 4xN / Nx4 and 8x8 |

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-1.1 | -0.03% | -0.01% | 0.02% | 99% | 101% | -0.06% | 0.02% | -0.15% | 102% | 101% |
| 4-1.2.a | -0.01% | -0.07% | -0.06% | 102% | 102% | -0.07% | 0.30% | 0.15% | 101% | 99% |
| 4-1.2.b | 0.09% | 0.07% | 0.15% | 100% | 101% | 0.04% | 0.31% | 0.17% | 101% | 99% |
| 4-1.3 | -0.02% | -0.17% | -0.11% | 101% | 100% | -0.01% | 0.08% | -0.29% | 101% | 100% |
| 4-1.4.a | -0.01% | -0.09% | -0.03% | 102% | 102% | -0.03% | 0.02% | -0.10% | 95% | 89% |
| 4-1.4.b | -0.01% | 0.06% | 0.06% | 102% | 102% | -0.05% | 0.01% | -0.05% | 96% | 87% |
| 4-1.4.c | -0.02% | -0.10% | -0.06% | 103% | 103% | -0.02% | -0.07% | -0.39% | 92% | 82% |
| 4-1.5.a | -0.01% | -0.04% | -0.06% | 104% | 98% | -0.01% | 0.11% | 0.03% | 81% | 100% |
| 4-1.5.b | -0.03% | -0.07% | -0.06% | 100% | 100% | 0.01% | 0.10% | -0.08% | 100% | 66% |
| 4-1.5.c | -0.03% | -0.06% | -0.04% | 100% | 102% | 0.04% | 0.04% | -0.26% | 81% | 97% |

Test 4-1.1 moves MMVD syntax from merge_data() to coding_unit(), trying to improve coding efficiency in high QP condition. As a coding efficiency proposal, the gain from this did not seem sufficient to justify a change. The proponent indicated that coding efficiency was not the main goal of this, but rather it is to reduce the maximum conditioning depth of branching conditions. Some other participants said that since MMVD is conceptually part of merging in an encoder, it would be less conceptually logical and confusing for encoder design to put the associated data somewhere else in the syntax. It was also commented that the maximum depth is not really a problem. So no action was taken on this.

Test 4-1.2 assigns shortest codeword to regular merge mode among all the 5 modes of merge candidates. The modified signalling order is: Regular merge, MMVD, subblock, CIIP, and TPM, rather than MMVD, subblock, CIIP, TPM, and then regular merge. The coding efficiency benefit from this was also very small, but it was commented that the test did verify that the most commonly chosen merge mode was not the one that had been first in the list. It was also commented that the current order is different from what would be expected in terms of wanting to put the modes that are most likely to be implemented/tested by an encoder first. It was commented that high-level disabling of features is another way to effectively remove entries in the list. This is a minor matter, but the current design does not seem logical. Decision (cleanup): Adopt.

Test 4-1.3 proposes removing mmvd_merge_flag and its context model, and the parsing of MMVD syntax being conditioned on merge_idx being equal to 0 or 1.

There is an assumption in the current MMVD design that the merge list has at least two candidates, otherwise parsing mmvd_merge_flag is a waste. However, there is no checking to ensure this. The proposal removes this unnecessary syntax. This is a minor matter, but the current design does not seem logical. However, it was commented that there may be other “corner case” aspects that should be considered – for example, when considering the interaction between the triangle mode and regular merge candidates (which is discussed in some non-CE contributions). This can be further discussed in relation to some other such issues.

Test 4-1.4 divides all merge candidates into to three groups, CU merge candidates, subblock merge candidates, and triangle merge candidates, according to the type of block partitioning for motion compensation. It is proposed to put together the syntax of each group, and reorder the syntax of the three groups as subblock candidates, triangle candidates, and CU candidates. This has an interaction with the action taken on Test 4-1.2. With the action taken on 4-1.2, there is a grouping, although in a different order. So no action was needed on 4-1.4.

Test 4-1.5 proposes not using TMVP in merge for small CUs. Three criteria for identifying small CUs are tested. With the action taken in response to CE2, 4x4 inter CUs are no longer in the design and therefore don’t need to be considered. The motivation for this was said to be for improving hardware throughput by skipping some of the logic needed for list construction when small blocks are used. Different sizes were tested and that coding efficiency benefit remained negligible. Decision (cleanup): Adopt 4-1.5.c.

It was said that 4-1.5b and 4-1.5c have some interaction with work to simplify the MV list construction for triangle mode, since some of these are based on trying to use a similar process as in regular merge list construction with spatial and TMVP candidates. Revisit for consideration of the triangle mode.

Discussion adjourned at 1300 and was then resumed at 1430.

CE4-2: STMVP

| Test# | Source | Description |
| --- | --- | --- |
| 4-2.2.a | JVET-N0285 | STMVP merge candidate, consisting in an averaged motion vector between the TMVP merge candidate and some other motion vectors predictors. |

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-2.2.a | -0.13% | -0.10% | -0.10% | 101% | 101% | 0.00% | 0.23% | 0.07% | 102% | 103% |

A single STMVP (spatial and temporal averaged) motion vector candidate, if available, is proposed to be put into regular merge list before the top-left spatial motion vector candidate. The derivation of the STMVP candidate uses one temporal candidate and up to two spatial candidates in the regular merge list construction. This is a proposed additional candidate for the merge list. The coding efficiency benefit for this change seemed too small to justify the change.

CE4-3: MMVD modification

| Test# | Source | Description |
| --- | --- | --- |
| 4-3.1.b | JVET-N0125 | Binarization method change |
| 4-3.1.c | JVET-N0125 | Multiple distance list concept |
| 4-3.1.d | JVET-N0125 | Multiple distance list concept + binarization method change |
| 4-3.3.a | JVET-N0441 | MMVD distance table is reduced 4 and is refined based on the resolution of base motion vectors |
| 4-3.3.b | JVET-N0441 | 4-3.3.a + 4-3.4.a |
| 4-3.3.c | JVET-N0441 | 4-3.3.a + 4-1.1 |
| 4-3.3.d | JVET-N0441 | 4-3.3.b + 4-1.1 |
| 4-3.4.a | JVET-N0142 | Diagonal direction candidate for MMVD |
| 4-3.4.b | JVET-N0142 | 4-3.4.a + 4-1.1 |

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-3.1.b | -0.02% | -0.12% | -0.12% | 100% | 98% | 0.00% | 0.13% | -0.28% | 100% | 101% |
| 4-3.1.c | -0.01% | -0.02% | -0.02% | 98% | 95% | 0.00% | 0.01% | -0.05% | 103% | 102% |
| 4-3.1.d | -0.02% | -0.13% | -0.12% | 100% | 98% | 0.00% | 0.10% | -0.28% | 100% | 103% |
| 4-3.3.a | 0.01% | 0.07% | 0.05% | 96% | 101% | 0.01% | 0.19% | 0.00% | 96% | 101% |
| 4-3.3.b | -0.08% | 0.09% | 0.10% | 97% | 101% | -0.05% | 0.25% | 0.21% | 96% | 100% |
| 4-3.3.c | -0.02% | 0.13% | 0.13% | 96% | 101% | -0.03% | 0.25% | 0.02% | 97% | 101% |
| 4-3.3.d | -0.11% | 0.11% | 0.17% | 97% | 101% | -0.12% | 0.20% | 0.05% | 98% | 101% |
| 4-3.4.a | -0.07% | 0.00% | 0.03% | 100% | 100% | -0.03% | 0.31% | 0.25% | 100% | 100% |
| 4-3.4.b | -0.11% | 0.02% | 0.12% | 100% | 101% | -0.08% | 0.13% | -0.14% | 101% | 100% |

Test 4-3.1.b changes the binarization of mmvd_distance_idx, making the codeword shorter for larger MVDs. The coding efficiency change was negligible and the change did not seem desirable for sufficient other reasons.

Test 4-3.1.c replaces the tile group header flag tile_group_fpel_mmvd_enabled_flag by a syntax symbol that could represent 4 alternative resolutions of MMVD distance instead of 2 in the current VTM design. The coding efficiency change was negligible and the change did not seem desirable for sufficient other reasons.

Test 4-3.3.a proposes reducing the number of candidates in distance list from 8 to 4, keeping the first four smaller MVD offsets. In addition, the decoded distance is further modified according to the resolution of the base motion vector. If base MV falls on the grid of 2-pel resolution, the distance is enlarged 4x. If base mv falls on the grid of integer-pel resolution, the distance is enlarged 2x. The coding efficiency change was very small and the change did not seem desirable for sufficient other reasons.

Test 4-3.4.a proposes diagonal direction MVD offsets. With this modification, four points with the same MMVD distance index forms a diamond shape. The proposed method doubles the number of MVDs. The coding efficiency change was very small and the change did not seem desirable for sufficient other reasons.

CE4-4: TPM merge list modification

| Test# | Source | Description |
| --- | --- | --- |
| 4-4.1.a | JVET-N0083 | Use regular merge mode candidate list and two signaled indices. Candidate 1 select L0 MV if exist; otherwise select L1 MV. Candidate 2 select L1 MV if exist; otherwise select L0 MV. |
| 4-4.1.c | JVET-N0083 | 4-4.1.a + MV pruning and adding offset MV if necessary |
| 4-4.2.a | JVET-N0128 | Use first 3 candidates of regular merge candidate list for triangle list construction |
| 4-4.2.b | JVET-N0128 | Use first 4 candidates of regular merge candidate list for triangle list construction |
| 4-4.2.c | JVET-N0128 | 4-4.2.a + spatial temporal candidate only restriction |
| 4-4.2.d | JVET-N0128 | 4-4.2.c + simplified pruning |
| 4-4.2.e | JVET-N0128 | Use regular merge list with spatial temporal restriction |
| 4-4.2.f | JVET-N0128 | 4-4.2.e + simplified pruning |
| 4-4.3.a | JVET-N0322 | Share regular merge candidate list for triangle partition merge prediction. For RA, L0 and L1 motion vectors from the regular candidate list are selected in an interleaving manner. For LDB, L0 motion vectors are selected, followed by uni-prediction zero vectors. |
| 4-4.3.b | JVET-N0322 | Share regular merge candidate list for triangle partition merge prediction. For RA, L0 and L1 motion vectors from the regular candidate list are selected in an interleaving manner. For LDB, L0 motion vectors are selected, followed by L1 motion vectors. |
| 4-4.4.a | JVET-N0269 | Partial of the regular merge list construction process is reused. |
| 4-4.5.a | JVET-N0226 | Reduce the pruning operations in TPM merge list construction |
| 4-4.5.b | JVET-N0226 | Remove the pruning operations in TPM merge list construction |
| 4-4.6.a | JVET-N0454 | Derive triangular merge list based on the candidates of regular merge list, using same scan order. If candidate is bi-predictive, L0 part is used. |
| 4-4.6.b | JVET-N0454 | Derive triangular merge list based on the candidates of regular merge list, using same scan order. If candidate is bi-predictive, pruning is applied. |
| 4-4.7.c | JVET-N0197 | The reference index of L1 is set equal to be the same as that of L0, mirroring process is applied after offset addition |
| 4-4.7.d | JVET-N0197 | The reference index of L1 is set equal to be the same as that of L0, mirroring process is applied before offset addition |
| 4-4.8.a | JVET-N0211 | Reuse regular merge list derivation for triangle merging candidate list construction |
| 4-4.8.b | JVET-N0211 | Use candidates before HMVP in the regular merge list as the candidates for triangle merging candidate list construction |
| 4-4.9.a | JVET-N0418 | Derive merge list based on the regular list, bi-MV is split into uni-MVs and pruning is applied. |
| 4-4.9.b | JVET-N0418 | 4-4.9.a with no pruning in the final TPM merge list. |
| 4-4.9.c | JVET-N0418 | Merge list is derived based on the spatial and temporal MV candidates from the regular merge list, bi-MV is split into uni-MVs and pruning is applied |
| 4-4.9.d | JVET-N0418 | 4-4.9.c with no pruning in the final TPM merge list. |

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-4.1.a | 0.06% | 0.00% | 0.06% | 101% | 100% | 0.15% | 0.19% | -0.01% | 100% | 100% |
| 4-4.1.c | 0.05% | 0.04% | 0.07% | 101% | 99% | 0.11% | 0.08% | -0.08% | 100% | 100% |
| 4-4.2.a | -0.03% | -0.07% | -0.11% | 100% | 100% | 0.08% | 0.12% | -0.28% | 100% | 100% |
| 4-4.2.b | -0.01% | -0.10% | -0.09% | 100% | 100% | -0.01% | -0.10% | -0.19% | 100% | 100% |
| 4-4.2.c | -0.03% | -0.04% | -0.06% | 100% | 100% | 0.04% | 0.01% | -0.09% | 100% | 100% |
| 4-4.2.d | -0.01% | -0.03% | 0.00% | 100% | 100% | 0.16% | 0.28% | -0.05% | 100% | 100% |
| 4-4.2.e | -0.03% | -0.04% | -0.09% | 100% | 100% | 0.05% | 0.16% | -0.08% | 100% | 101% |
| 4-4.2.f | 0.00% | -0.01% | 0.02% | 100% | 100% | 0.13% | 0.11% | 0.25% | 100% | 99% |
| 4-4.3.a | -0.02% | -0.09% | -0.06% | 101% | 99% | 0.16% | 0.33% | 0.03% | 100% | 100% |
| 4-4.3.b | -0.02% | -0.09% | -0.07% | 101% | 101% | 0.17% | 0.29% | 0.20% | 99% | 99% |
| 4-4.4.a | -0.02% | -0.09% | -0.11% | 100% | 100% | 0.09% | 0.20% | -0.19% | 100% | 100% |
| 4-4.5.a | 0.01% | -0.01% | 0.01% | 100% | 100% | 0.02% | 0.19% | -0.15% | 101% | 100% |
| 4-4.5.b | 0.03% | 0.01% | 0.02% | 100% | 100% | 0.14% | 0.37% | -0.04% | 101% | 101% |
| 4-4.6.a | 0.12% | 0.10% | 0.16% | 101% | 102% | 0.16% | 0.19% | 0.18% | 101% | 103% |
| 4-4.6.b | 0.00% | -0.07% | -0.09% | 101% | 102% | 0.11% | 0.12% | -0.01% | 101% | 103% |
| 4-4.7.c | 0.07% | 0.03% | 0.06% | 102% | 100% | 0.05% | -0.11% | -0.22% | 103% | 101% |
| 4-4.7.d | 0.06% | 0.03% | 0.08% | 103% | 100% | 0.05% | -0.11% | -0.22% | 104% | 100% |
| 4-4.8.a | 0.07% | 0.03% | 0.06% | 96% | 109% | 0.04% | 0.27% | 0.00% | 101% | 111% |
| 4-4.8.b | -0.03% | -0.04% | -0.09% | 96% | 110% | 0.05% | 0.16% | -0.08% | 101% | 111% |
| 4-4.9.a | -0.04% | -0.14% | -0.14% | 100% | 100% | -0.03% | 0.10% | -0.01% | 100% | 101% |
| 4-4.9.b | -0.01% | -0.08% | -0.06% | 100% | 100% | 0.31% | 0.48% | 0.06% | 100% | 102% |
| 4-4.9.c | -0.02% | -0.04% | -0.04% | 100% | 100% | 0.13% | 0.19% | 0.08% | 100% | 101% |
| 4-4.9.d | 0.00% | -0.05% | -0.07% | 101% | 101% | 0.34% | 0.45% | 0.30% | 100% | 101% |

In VTM4.0 TPM, the MV candidate list construction comprises two steps.

Step 1. Construct the source candidate list

- Obtain regular motion candidates from 5 spatial neighbouring blocks with full pruning.

- Obtain regular merge candidates from two temporal blocks.

Step 2. Construct the TPM candidate list with uni-directional MVs, with full pruning

- Loop #1: For each regular motion candidates, if it is uni-prediction (either from list 0 or list 1), it is directly added to the merge list as a TPM candidate.

- Loop #2: For each motion candidate, if it is bi-prediction, the motion information from list 0 is added as a new TPM candidate.

- Loop #3: For each motion candidate, if it is bi-prediction, the motion information from list 1 is added as a new TPM candidate.

- Loop #4: For each motion candidates, if it is bi-prediction, the motion information of list 1 is firstly scaled to list 0 reference picture, and the average of the two MVs is added as a new TPM candidate.

- Add default motion vector candidates.

The complexity is higher than regular merge candidate list construction. All tests target at reducing complexity of TPM list construction.

Most of the tests construct the regular merge list, full or partial, and use it as the source list. This is referred to as stage 1 of TPM list construction. Tests differs in stage 2 where a uni-prediction candidate list is constructed, trying to reduce the complexity in three ways, 1) reduce the number of iterations when checking the source candidate list, 2) reduce the number potential candidates, and 3) reduce the number of prunings when adding a new candidate to the list.

Test 4-4.5 is different from others in that regular merge list is not used in stage 1 TPM list construction.

Test 4-4.7 is different from others in that it firstly construct a TPM list with bi-pred MVs, by converting a uni-pred MV to a bi-pred one. And then derive a pair of uni-pred MVs from two signaled TPM candidates. Additional syntax indicating prediction direction is signaled. It removes operations in motion data storage stage by deriving bi-pred MV in the stage of TPM list construction.

A few tests do not even construct a TPM list in stage 2, and remove the complexity regarding stage 2 list construction. These tests are 4-4.1.a, 4-4.1.c, 4-4.6.a, 4-4.7.c, 4-4.7.d.

All tests are summarized below according to key elements in the design.

| Test # | Stage 1 of TPM list construction |  |  | Stage 2 of TPM list construction |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Candidate derivation | Number of candidates | Candidate pruning | Source list checking iterations | Conversion of Bi-pred MV | Candidate pruning | Others |
| 4-4.1.a | Regular merge | 5 | Regular merge | - | - | - | - |
| 4-4.1.c | Regular merge | 5 | Regular merge | - | - | 1 | Add MV offsets conditionally |
| 4-4.2.a | Regular merge | 3 | Regular merge | 4 | Same as VTM4 | Full | - |
| 4-4.2.b | Regular merge | 4 | Regular merge | 4 | Same as VTM4 | Full | - |
| 4-4.2.c | Regular merge (spatial, temporal candidates) | 4 (max) | Regular merge | 4 | Same as VTM4 | Full | - |
| 4-4.2.d | Regular merge (spatial, temporal candidates) | 4 (max) | Regular merge | 4 | Same as VTM4 | Simplified pruning to only the last one | - |
| 4-4.2.e | Regular merge (spatial, temporal candidates) | 5 (max) | Regular merge | 4 | Same as VTM4 | Full | - |
| 4-4.2.f | Regular merge (spatial, temporal candidates) | 5 (max) | Regular merge | 4 | Same as VTM4 | Simplified pruning to only the last one | - |
| 4-4.3.a | Regular merge | 5 | Regular merge | 1 | Split to L0 and L1 MV | No | Different handling for RA and LDB |
| 4-4.3.b | Regular merge | 5 | Regular merge | 1 (RA) 2 (LDB) | Split to L0 and L1 MV | No | Different handling for RA and LDB |
| 4-4.4.a | Regular merge (spatial, temporal, HMVP candidates) | 5 (max) | Regular merge | 1 | Split to L0 and L1 MV | Simplified pruning, controlled by a threshold | - |
| 4-4.5.a | TPM source list | 7 (max) | Simplified pruning | 4 | Same as VTM4 | Simplified | - |
| 4-4.5.b | TPM source list | 7 (max) | Simplified pruning | 4 | Same as VTM4 | No | - |
| 4-4.6.a | Regular merge | 5 | Regular merge | 1 | Use L0 MV only | No | - |
| 4-4.6.b | Regular merge | 5 | Regular merge | 1 | Split to L0 and L1 MV | L0 vs. L1 pruning | - |
| 4-4.7.c | Regular merge | 4 | Regular merge | 1 | - | No | - |
| 4-4.7.d | Regular merge | 4 | Regular merge | 1 | - | No | - |
| 4-4.8.a | Regular merge | 6 | Regular merge | 4 | Same as VTM4 | Full | - |
| 4-4.8.b | Regular merge (spatial, temporal candidates) | 5 (max) | Regular merge | 4 | Same as VTM4 | Full | - |
| 4-4.9.a | Regular merge | 6 | Regular merge | 1 | Split to L0 and L1 MV | Full | - |
| 4-4.9.b | Regular merge | 6 | Regular merge | 1 | Split to L0 and L1 MV | No | - |
| 4-4.9.c | Regular merge (spatial, temporal candidates) | 5 (max) | Regular merge | 1 | Split to L0 and L1 MV | Full | - |
| 4-4.9.d | Regular merge (spatial, temporal candidates) | 5 (max) | Regular merge | 1 | Split to L0 and L1 MV | No | - |

Complexity analysis of list construction

| Test# | Merge list size | Max number of potential candidates (stage1+stage2) | Max number of candidate comparison (stage1+stage2) | Max number of MV scaling (stage1+stage2) | Max number of temporal candidates (stage1+stage2) | Max number of MV average (stage1+stage2) | Max number of encoder MCs (stage1+stage2) | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Regular merge | 6 | 13 | 9 | 2 | 2 | 2 | 12 |  |
| TPM | 5 | 28 (7+21) | 84 (10+74) | 11 (4+7) | 2 (2+0) | 7 (0+7) | 5 |  |
| 4-4.1.a | 5 | 13 | 9 | 2 | 2 | 2 | 10 |  |
| 4-4.1.c | 5 | 13 | 10 (9+1) | 2 | 2 | 2 | 10 | One additional MV addition (adding (1/4, 0) on second candidate) |
| 4-4.2.a | 5 | 22 (13+9) | 35 | 5 | 2 | 5 | 5 |  |
| 4-4.2.b | 5 | 25 (13+12) | 47 | 6 | 2 | 6 | 5 |  |
| 4-4.2.c | 5 | 19 (7+12) | 43 | 6 | 2 | 4 | 5 |  |
| 4-4.2.d | 5 | 19 (7+12) | 16 | 6 | 2 | 4 | 5 |  |
| 4-4.2.e | 5 | 25 (7+18) | 67 | 8 | 2 | 6 | 5 |  |
| 4-4.2.f | 5 | 25 (7+18) | 22 | 8 | 2 | 8 | 5 |  |
| 4-4.3.a | 5 | 13 | 9 | 2 | 2 | 2 | 5 |  |
| 4-4.3.b | 5 | 13 | 9 | 2 | 2 | 2 | 5 |  |
| 4-4.4.a | 5 | 15 (5+10) | 12 (9+3) | 1 | 1 | 0 | 5 |  |
| 4-4.5.a | 5 | 28 (+0) | 6+46 (-4-28) | 4+7 (+0+0) | 2 (+0) | 7 (+0) | 5 (+0) | / |
| 4-4.5.b | 5 | 28 (+0) | 6+0 (-4-74) | 4+7 (+0+0) | 2 (+0) | 7 (+0) | 5 (+0) | / |
| 4-4.6.a | 5 | 13 (13+0) | 9 (9+0) | 2 | 2 | 2 | 5 |  |
| 4-4.6.b | 5 | 23 (13+10) | 13 (9+4) | 2 | 2 | 2 | 5 |  |
| 4-4.7.c | 4 | 13 (13+0) | 9 | 2 | 2 | 2 | 8 | No mapping logic |
| 4-4.7.d | 4 | 13 (13+0) | 9 | 2 | 2 | 2 | 8 | No mapping logic |
| 4-4.8.a | 6 | 31 (13+18) | 71 (9+62) | 8 (2+6) | 2 (2+0) | 8 (2+6) | 5 |  |
| 4-4.8.b | 5 | 22 (7+15) | 55 (5+50) | 7 (2+5) | 2 (2+0) | 5 (0+5) | 5 |  |
| 4-4.9.a | 5 | 25 (13+12) | 47 (9+38) | 2 (2+0) | 2 (2+0) | 2 (2+0) | 5 |  |
| 4-4.9.b | 5 | 25 (13+12) | 9 (9+0) | 2 (2+0) | 2 (2+0) | 2 (2+0) | 5 |  |
| 4-4.9.c | 5 | 17 (7+10) | 35 (5+30) | 2 (2+0) | 2 (2+0) | 2 (2+0) | 5 |  |
| 4-4.9.d | 5 | 17(7+10) | 5 (5+0) | 2 (2+0) | 2 (2+0) | 2 (2+0) | 5 |  |

To prune the number of variants to consider, it was agreed to rule out those with 4 source list checking iterations. And 4-4.9.b, 4-4.9.d, 4-4.6.a, 4-4.1.a for coding efficiency loss in LB without sufficient other rationale. 4-4.3.a, 4-4.3.b also had some loss in LB but were suggested to further consider due eliminating pruning.

4-4.7c and 4-4.7d add additional syntax and additional encoder motion compensations (resulting in 3-4% encoder runtime increase), but reduce some decoder logic associated with MV storage. Thus, these were ruled out.

4-4.1.c was also eliminated, due to having more motion compensations than any of the other remaining variants and also showing some (small) loss.

4-4.1.c was also eliminated, due to having more motion compensations than any of the other remaining variants and also showing some (small) loss.

| Test # | Stage 1 of TPM list construction |  |  | Stage 2 of TPM list construction |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Candidate derivation | Number of candidates | Candidate pruning | Source list checking iterations | Conversion of Bi-pred MV | Candidate pruning | Others |
| 4-4.3.a | Regular merge | 5 | Regular merge | 1 | Split to L0 and L1 MV | No | Different handling for RA and LDB |
| 4-4.3.b | Regular merge | 5 | Regular merge | 1 (RA) 2 (LDB) | Split to L0 and L1 MV | No | Different handling for RA and LDB |
| 4-4.4.a | Regular merge (spatial, temporal, HMVP candidates) | 5 (max) | Regular merge | 1 | Split to L0 and L1 MV | Simplified pruning, controlled by a threshold | - |
| 4-4.6.b | Regular merge | 5 | Regular merge | 1 | Split to L0 and L1 MV | L0 vs. L1 pruning | - |
| 4-4.9.a | Regular merge | 6 | Regular merge | 1 | Split to L0 and L1 MV | Full | - |
| 4-4.9.c | Regular merge (spatial, temporal candidates) | 5 (max) | Regular merge | 1 | Split to L0 and L1 MV | Full | - |

4-4.3.b has decoder logic that treats low-delay (i.e., all reference pictures are temporally preceding) as a special case, which was suggested to be undesirable customization. However, it was noted that there is logic for this already in the current standard (used for TMVP and BCW).

It was commented that there are relevant non-CE proposals as well (esp. JVET-N0340). There was general agreement that none of the remaining 6 variants may be as good as a non-CE proposal, so further discussion of them was not needed.

JVET-N0083 CE4-4.1: Simplification of triangle merging candidate list derivation [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0125 CE4-3.1: MMVD binarization [S. Jeong, M. W. Park, K. Choi (Samsung)]

JVET-N0128 CE4-4.2: Triangle Prediction Mode Harmonization [A. Tamse, M. W. Park, K. Choi (Samsung)]

JVET-N0584 Cross-check of JVET-N0128: CE4-4.2: Triangle Prediction Mode Harmonization [H. Wang (Qualcomm)] [miss] [late]

JVET-N0141 CE4-1.1: Syntax Change of MMVD [E. Sasaki, T. Chujoh, T. Ikai (Sharp)]

JVET-N0142 CE4-3.4: Diagonal direction candidate for MMVD [T. Hashimoto, E. Sasaki, T. Ikai (Sharp)]

JVET-N0197 CE4: Triangle merge candidate list simplification (Test 4.4.7) [J. Li, C.-W. Kuo, C. S.Lim (Panasonic)]

JVET-N0211 CE4-4.8: Triangle merge candidate list derivation simplification [X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)]

JVET-N0688 Cross-check of JVET-N0211 (CE4-4.8b: Simplification of triangle prediction merging candidate list derivation) [J. Li, C.-W. Kuo, C. S.Lim (Panasonic)] [miss] [late]

JVET-N0212 CE4-1.5: Remove TMVP merge candidate for the specified blocksizes [S.H. Wang (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)]

JVET-N0226 CE4-4.5: Pruning reduction in Triangle Merge mode [A. Robert, T. Poirier, F. Le Léannec, F. Galpin (Technicolor)]

JVET-N0237 CE4-1.3: Modification of merge data syntax [G. Ko, D. Kim, J. Jung, J. Son, J. Kwak (WILUS)]

JVET-N0252 CE4-1.4: Syntax changes of merge data [Y. Ahn, D. Sim (Digital Insights)]

JVET-N0269 CE4-4.4: Merge list construction for triangular prediction mode [L. Zhang (Bytedance)]

JVET-N0285 CE4-2.2: Pairwise extension with STMVP [F. Le Léannec, A. Robert, T. Poirier (Technicolor)]

JVET-N0322 CE4: Triangle prediction merge list construction (CE4-4.3) [X. Wang, Y.-W. Chen (Kwai Inc.)]

JVET-N0585 Cross-check of JVET-N0322: CE4: Triangle prediction merge list construction (CE4-4.3) [H. Wang (Qualcomm)] [miss] [late]

JVET-N0324 CE4: Regular merge flag coding (CE4-1.2.a and CE4-1.2.b) [Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-N0418 CE4: Using regular merge candidate list for triangular PU mode (CE4-4.9) [H. Wang, V. Seregin, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-N0635 Cross-check of JVET-N0418 (CE4-4.9: Using regular merge candidate list for triangular PU mode) [X. Wang (Kwai Inc.)] [miss] [late]

JVET-N0441 CE4-3.3: Candidates optimization on MMVD [N. Park, H. Jang, J. Nam, J. Lim, S. Kim (LGE)]

JVET-N0454 CE4-4.6: Simplification for merge list derivation in triangular prediction mode [T. Solovyev, S. Esenlik, S. Ikonin, J. Chen (Huawei)]

## CE5: Adaptive loop filtering (4)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0025 CE5: Summary Report on Adaptive Loop Filter [V. Seregin, C.-Y. Chen]

JVET-N0088 CE5.1: Adaptive loop filter with virtual boundary processing [C.-Y. Chen, T.-D. Chuang, C.-Y. Lai, Z.-Y. Lin, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0180 CE5-2: Loop filter line buffer reduction [A. M. Kotra, S. Esenlik, J. Chen, B. Wang, H. Gao (Huawei)]

JVET-N0242 CE5: Results of tests CE5-3.1, CE5-3.2, CE5-3.3 and CE5-3.4 on Non-Linear Adaptive Loop Filter [J. Taquet, P. Onno, C. Gisquet, G. Laroche (Canon)]

JVET-N0415 CE5: Coding tree block based adaptive loop filter (CE5-4) [N. Hu, V. Seregin, H. E. Egilmez, M. Karczewicz (Qualcomm)]

## CE6: Transforms and transform signalling (4)

Contributions in this category were discussed Wednesday 20 March 1500–1650 (Track A chaired by JRO).

JVET-N0026 CE6: Summary Report on Transforms and Transform Signaling [X. Zhao, H.E. Egilmez]

This contribution summarizes the activities of Core Experiment (CE) on Transforms and Transform Signaling. The goal of this CE is to study transform design and signaling for the VVC standard. The CE studies were divided into three categories, including:

(1) CE6-1: Unification between MTS and Transform Skip mode (4 tests, 3 tests withdrawn)

(2) CE6-2: Reduced transform candidates of MTS (13 tests, 3 tests withdrawn)

(3) CE6-3: Secondary transform (5 tests)

In this CE all experiments were done using based on the VTM-4.0 SW. This document summarizes the test results, brief experiment definition, cross-check reports and complexity measurements, and also related contributions.

Core experiments (CEs) are organized according to the following five categories, and the following table lists all the experiments in each category, and the corresponding input document to the Geneva meeting. In total 16 tests were studied in this CE, and 6 tests were withdrawn from the original CE6 plan.

The 16 tests reported in this CE6 summary are listed below.

| Test # | Doc # | Description | Tester | Cross-checker |
| --- | --- | --- | --- | --- |
| CE6-1.1a | JVET-N0360 | One dimensional TS (IDT used as horizontal or vertical transform) is applied up to 32-point. | X. Zhao (Tencent) | T. Tsukuba (Sony) |
| CE6-2.1a | JVET-N0053 | 2-mode MTS without implicit transform selection, Allowed intra picture 64x64 CU splits: QT, TT, BT | J. Lainema (Nokia) | C. Hollmann (Ericsson) |
| CE6-2.1b |  | 2-mode MTS without implicit transform selection, Allowed intra picture 64x64 CU splits: QT, TT | J. Lainema (Nokia) | C. Hollmann (Ericsson) |
| CE6-2.1c |  | 2-mode MTS without implicit transform selection, Allowed intra picture 64x64 CU splits: QT | J. Lainema (Nokia) | C. Hollmann (Ericsson) |
| CE6-2.2a | JVET-N0208 | Non-normative change with 3 modes | K. Choi (Samsung) | X. Zhao (Tencent) |
| CE6-2.2b |  | Non-normative change with 2 modes | K. Choi (Samsung) | X. Zhao (Tencent) |
| CE6-2.2c |  | Normative change with 3 modes | K. Choi (Samsung) | X. Zhao (Tencent) |
| CE6-2.2d |  | Normative change with 2 modes | K. Choi (Samsung) | X. Zhao (Tencent) |
| CE6-2.3a | JVET-N0490 | Simplified transform selection with reduced modes and no size-32 DCT-8, Number of evaluated candidates: 3 | C. Hollmann (Ericsson) | J. Lainema (Nokia) |
| CE6-2.3b |  | Simplified transform selection with reduced modes and no size-32 DCT-8, Number of evaluated candidates: 4 | C. Hollmann (Ericsson) | J. Lainema (Nokia) |
| CE6-2.3c |  | Simplified transform selection with reduced modes and no size-32 DCT-8, Number of evaluated candidates: 5 | C. Hollmann (Ericsson) | J. Lainema (Nokia) |
| CE6-3.1a | JVET-N0193 | ( 4 transform sets. 2 transforms per set ) | M. Koo (LGE) | P. Philippe (Orange) |
| CE6-3.1b |  | ( 4 transform sets. 2 transforms per set ) + (Secondary transform uses at most maximum 8 multiplications/sample ) | M. Koo (LGE) | P. Philippe (Orange) |
| CE6-3.1c |  | ( 4 transform sets. 2 transforms per set ) + ( Secondary transform uses at most maximum 8 multiplications/sample ) + ( Secondary transform is disabled for 4x4 TU ) | M. Koo (LGE) | P. Philippe (Orange) |
| CE6-3.1d |  | ( 4 transform sets. 2 transforms per set ) + ( Secondary transform uses at most maximum 8 multiplications/sample ) + ( 16x48 matrices are employed instead of 16x64 ones ) | M. Koo (LGE) | P. Philippe (Orange) |
| CE6-3.1e |  | ( 4 transform sets. 1 transforms per set ) | M. Koo (LGE) | P. Philippe (Orange) |

The following table summarizes the results of CE6 tests using CTC configuration and VTM-4.0 as anchor.

|  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-1.1a | 0.19% | -0.20% | -0.21% | 95% | 99% | 0.08% | -0.13% | -0.12% | 100% | 100% | 0.04% | -0.04% | 0.18% | 100% | 101% |
| CE6-2.1a | -0.32% | -1.81% | -1.83% | 107% | 100% | 0.04% | -0.72% | -0.83% | 98% | 99% | 0.16% | -0.07% | -0.47% | 99% | 100% |
| CE6-2.1b | -0.02% | -1.48% | -1.51% | 89% | 100% | 0.11% | -0.78% | -0.74% | 97% | 100% | 0.17% | -0.04% | -0.65% | 99% | 100% |
| CE6-2.1c | 0.20% | -0.25% | -0.26% | 75% | 100% | 0.18% | -0.13% | -0.25% | 97% | 100% | 0.14% | 0.34% | -0.31% | 99% | 99% |
| CE6-2.2a* | 0.00% | 0.00% | 0.00% | 99% | 101% | 0.00% | -0.02% | -0.06% | 100% | 101% | 0.01% | 0.04% | -0.12% | 101% | 100% |
| CE6-2.2b* | 0.11% | -0.06% | -0.08% | 87% | 101% | 0.05% | -0.12% | -0.06% | 100% | 101% | 0.03% | 0.06% | -0.42% | 100% | 101% |
| CE6-2.2c* | 0.00% | 0.01% | 0.02% | 99% | 101% | 0.00% | -0.08% | -0.07% | 100% | 101% | -0.04% | 0.12% | -0.14% | 100% | 101% |
| CE6-2.2d* | 0.09% | -0.08% | -0.11% | 87% | 100% | 0.04% | -0.05% | -0.15% | 98% | 100% | 0.04% | 0.16% | 0.15% | 99% | 100% |
| CE6-2.3a | -0.03% | -0.01% | 0.03% | 100% | 100% |  |  |  |  |  |  |  |  |  |  |
| CE6-2.3b | -0.02% | -0.01% | -0.01% | 90% | 100% |  |  |  |  |  |  |  |  |  |  |
| CE6-2.3c | -0.02% | -0.01% | -0.03% | 90% | 100% |  |  |  |  |  |  |  |  |  |  |
| CE6-3.1a | -1.56% | -0.64% | -1.09% | 124% | 96% | -0.84% | -1.35% | -1.89% | 110% | 98% | -0.27% | -0.37% | -0.95% | 107% | 98% |
| CE6-3.1b | -1.40% | -0.51% | -1.04% | 125% | 98% | -0.73% | -1.25% | -1.81% | 111% | 99% | -0.24% | -0.50% | -1.09% | 108% | 99% |
| CE6-3.1c | -1.28% | -0.35% | -0.80% | 123% | 98% | -0.68% | -1.03% | -1.59% | 111% | 99% | -0.23% | -0.71% | -0.78% | 107% | 99% |
| CE6-3.1d | -1.34% | -0.40% | -0.87% | 126% | 98% | -0.69% | -1.13% | -1.75% | 111% | 99% | -0.22% | -0.19% | -0.83% | 108% | 99% |
| CE6-3.1e | -1.21% | 0.13% | -0.17% | 101% | 96% | -0.61% | -0.90% | -1.35% | 105% | 98% | -0.19% | -0.06% | -0.40% | 104% | 98% |

*Results provided after contribution deadline.

CE6-1 shows loss for natural video, but come gain -0.8%,-0.6% for AI/RA in TGM screen content. Compared to gains shown by other SCC tools, this is very small gain, whereas the loss in natural is of some concern. No action.

CE6-2.1x intends to show the benefit of reducing number of transforms to 2. For AI, this reduces encoder complexity, or gives small gain when increasing complexity. For RA and LDB, luma losses are observed, probably due to the abandoning of DCT-VIII. For decoder implementation, no real advantage, as DCT-VIII and DST-VII are practically the same (flipped basis).

For SBT, the proposal substituted DCT-VIII by DCT-II.

Benefit not clear with diverging results in intra and inter cases. No action.

CE6-2.2x is targeting reduction of encoder runtime by a non-normative change of testing less modes. 6-2.2b reaches comparable encoder runtime for AI as 6-2.1b (though the modes tested are not the same as the ones that 6-2.1b would leave in a normative change). For RA and LB, less luma losses are observed than for the normative change of 6-2.1b. The luma loss in AI is 0.1%. It is discussed if it would be useful to adopt this to CTC for AI (as faster encoder), some experts expressed support for that. However, the AI simulations are not the most critical bottleneck. For real-world implemetations of fast AI encoders, this might be useful, no action for CTC

CE6-2.3x (JVET-N0490) had a very late update (03-20) after making a bug fix, RA data are not available yet. No cross-check available yet. The number of combinations that can be signalled is reduced, e.g. no combination of DCT-VIII/DCT-VIII, disallowing DCT-VIII for length 32. Results show no gain and no encoder run time reduction for CTC, but when compared against another non-CTC encoder which checks more modes, some 10% encoder runtime reduction is reported for AI. No clear benefit.

CE6-3 is abot secondary transform. CE6-3.1a and CE6-3.1e have worst case 16 mul/sample, the other versions have 8 mul/sample. Memory requirements for transform coefficient tables and other complexity aspects are analysed as follows:

| Operation counts | Primary transform: Same as VTM Secondary transform: RST4x4: 256 (M), 240(A), 16(S) RST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per sample: 16 | Primary transform: Same as VTM Secondary transform: RST4x4: 256 (M), 240(A), 16(S) RST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per sample: 8 | Primary transform: Same as VTM Secondary transform: RST4x4: 256 (M), 240(A), 16(S) RST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per sample: 8 | Primary transform: Same as VTM Secondary transform: RST4x4: 256 (M), 240(A), 16(S) RST8x8: 768 (M), 720(A), 48(S) Worst Case multiplication per sample: 8 | Primary transform: Same as VTM Secondary transform: RST4x4: 256 (M), 240(A), 16(S) RST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per sample: 16 |
| --- | --- | --- | --- | --- | --- |
| Memory requirements | RST8x8: 16*64*8 /(8*1024)= 1 KB per kernel RST4x4: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 2*4*(1+ 0.25) = 10 KB | RST8x8: 16*64*8 /(8*1024)= 1 KB per kernel RST4x4: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 2*4*(1+ 0.25) = 10 KB | RST8x8: 16*64*8 /(8*1024)= 1 KB per kernel RST4x4: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 2*4*(1+ 0.25) = 10 KB | RST8x8: 16*48*8 /(8*1024)= 0.75 KB per kernel RST4x4: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 2*4*(0.75+ 0.25) = 8 KB | RST8x8: 16*64*8 /(8*1024)= 1 KB per kernel RST4x4: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 4*(1+ 0.25) = 5 KB |
| Bit-precision to represent transform coefficients | 8 bits | 8 bits | 8 bits | 8 bits | 8 bits |
| Precision of arithmetic operations during transform computation | 8 bits x 16 bits mult. | 8 bits x 16 bits mult. | 8 bits x 16 bits mult. | 8 bits x 16 bits mult. | 8 bits x 16 bits mult. |
| Bit-precision for storing intermediate data representation | Same as VTM | Same as VTM | Same as VTM | Same as VTM | Same as VTM |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. | No | No | No | No | No |
| Other operations and memory requirements | N/A | N/A | N/A | N/A | N/A |
| List of all combinations of transform types | Two RST Kernel combined with primary transform in VTM | Two RST Kernel combined with primary transform in VTM | Two RST Kernel combined with primary transform in VTM | Two RST Kernel combined with primary transform in VTM | One RST Kernel combined with primary transform in VTM |
| List of all combinations of transforms block size and transform type used for secondary transformation | For blocks with min(W,H)<8 : RST4x4 For blocks with min(W,H)>=8 : RST8x8 | For blocks with min(W,H)<8 : RST4x4 For blocks with min(W,H)>=8 : RST8x8 For simplification (option (B), 8 mult/sample): for 4x4 block: Half RST4x4 for 8x8 block: Half RST8x8 for 4*8 and 8*4: only apply on one 4x4 corner | For blocks with min(W,H)<8 : RST4x4 For blocks with min(W,H)>=8 : RST8x8 For simplification (option (B), 8 mult/sample): for 4x4 block: No RST for 8x8 block: Half RST8x8 for 4*8 and 8*4: only apply on one 4x4 corner | For blocks with min(W,H)<8 : RST4x4 For blocks with min(W,H)>=8 : RST8x8 For simplification (option (B), 8 mult/sample): for 4x4 block: Half RST4x4 for 8x8 block: Half RST8x8 for 4*8 and 8*4: only apply on one 4x4 corner | For blocks with min(W,H)<8 : RST4x4 For blocks with min(W,H)>=8 : RST8x8 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). | Primary transform: Same as VTM Secondary transform: Reduced matrix multiplications | Primary transform: Same as VTM Secondary transform: Reduced matrix multiplications | Primary transform: Same as VTM Secondary transform: Reduced matrix multiplications | Primary transform: Same as VTM Secondary transform: Reduced matrix multiplications | Primary transform: Same as VTM Secondary transform: Reduced matrix multiplications |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. | N/A | N/A | N/A | N/A | N/A |

Among the different versions, 6-3.1d appears to have the best complexity/performance tradeoff, though it has a larger requirement for storage compared to e, the worst case number of multiplications is reduced.

It is noted that the secondary transform does not have impact on decoder latency (as the inverse transform can be performed out of loop); however an encoder might face some latency issue depending on its implementation, and would also not be required to use it.

It provides 1.3% rate reduction for AI with 25% encoder runtime increase, and 0.7% with 11% encoder runtime increase for RA. For LB, the gain of 0.2% does not justify an increase of 8% encoder runtime.

Decision: Adopt JVET-N0193, version from CE6-3.1d. CTC for AI and RA, not LB.

Revisit: Specification should be amended by introducing clipping at the output of the output of the secondary transform (software has it). Cross-checkers are asked to study the matching of software and specification text, including the transform table.

Highlevel flag for disabling. There should be a better name than “secondary transform”, since it is the first transform executed after decoding the transform coefficients.

BoG (H. Egilmez) to make an initial review of CE related contributions, and identify the aspects to be investigated in the upcoming CE.

JVET-N0053 CE6: 2-mode MTS (CE6-2.1) [J. Lainema (Nokia)]

JVET-N0193 CE6: Reduced Secondary Transform (RST) (CE6-3.1) [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-N0360 CE6-1: Unification between MTS and Transform Skip mode [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-N0490 CE6: Transform Simplification (CE6-2.3a-c) [C. Hollmann, D. Saffar, J. Ström, P. Wennersten (Ericsson)]

## CE7: Quantization and coefficient coding (4)

Contributions in this category were discussed Wednesday 20 March 1700–1815 (Track A chaired by JRO).

JVET-N0027 CE7: Summary report on quantization and coefficient coding [H. Schwarz, M. Coban]

The purpose of this core experiment is to explore the coding efficiency and complexity impact of proposed algorithms for quantization and transform coefficient coding.

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| CE7-1 | Jani Lainema (Nokia) | Joint coding of Cb and Cr residual | H. Schwarz (HHI) |
| CE7-2 | Xin Zhao (Tencent) | Reduced number of context-coded bins | Y. Piao (Samsung) |
| CE7-3 | Yinji Piao (Samsung) | Template-based unified Rice parameter derivation | Y.-H. Chao (Qualcomm) |
| CE7-4 | Yung-Hsuan Chao (Qualcomm) | Template based Rice parameter derivation for coding of abs_remainder | Y. Piao (Samsung) |

| document | author(s) | title |
| --- | --- | --- |
| JVET-N0054 | J. Lainema (Nokia) | CE7: Joint coding of chrominance residuals (CE7-1) |
| JVET-N0188 | Y. Piao, K. Choi (Samsung) | CE7-7.3: Unified rice parameter derivation for coefficient level coding |
| JVET-N0345 | Y.-H. Chao, M. Coban, M. Karczewicz (Qualcomm) | CE7: Template based Rice parameter derivation for coding of abs_remainder (CE7-4) |
| JVET-N0361 | X. Zhao, X. Li, S. Liu (Tencent) | CE7-2: Reduced number of context-coded bins |

| CE 7 |  | Common test conditions |  |  |  |  | low QPs (2, 7, 12, 17) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| Joint residual coding for Cb and Cr |  |  |  |  |  |  |  |  |  |  |  |
| AI | CE7-1 | -0.38% | -0.49% | -1.52% | 103% | 100% | -0.28% | 0.57% | 0.27% | 108% | 99% |
| RA | CE7-1 | -0.26% | -0.81% | -0.92% | 101% | 100% | -0.19% | 0.24% | -0.10% | 102% | 100% |
| LD | CE7-1 | -0.07% | -2.99% | -4.76% | 100% | 100% | -0.18% | 0.09% | -0.58% | 103% | 100% |
| Reduction of number of context-coded bins |  |  |  |  |  |  |  |  |  |  |  |
| AI | CE7-2 | 0.03% | 0.07% | 0.04% | 99% | 100% | 0.11% | 0.16% | 0.16% | 99% | 99% |
| RA | CE7-2 | 0.01% | -0.03% | -0.10% | 100% | 99% | 0.15% | 0.22% | 0.24% | 100% | 100% |
| LD | CE7-2 | 0.02% | 0.36% | 0.00% | 100% | 99% | 0.13% | 0.22% | 0.19% | 101% | 100% |
| Unification of Rice parameter derivation |  |  |  |  |  |  |  |  |  |  |  |
| AI | CE7-3 | 0.00% | 0.01% | 0.00% | 102% | 101% | 0.00% | -0.02% | -0.02% | 102% | 102% |
|  | CE7-4 | 0.00% | 0.02% | 0.03% | 103% | 100% | -0.02% | -0.02% | -0.01% | 104% | 103% |
| RA | CE7-3 | -0.02% | 0.03% | -0.11% | 101% | 100% | 0.00% | -0.01% | -0.02% | 102% | 103% |
|  | CE7-4 | 0.00% | -0.05% | -0.10% | 101% | 100% | 0.01% | -0.01% | 0.00% | 103% | 122% |
| LD | CE7-3 | -0.01% | 0.19% | -0.20% | 100% | 100% | 0.00% | -0.01% | 0.00% | 102% | 102% |
|  | CE7-4 | -0.04% | -0.01% | 0.23% | 101% | 101% | 0.02% | 0.02% | 0.03% | 103% | 101% |

CE7-1 is introducing a mode signalled per TB where Cb and Cr are jointly coded, and the reconstructed residual samples of the two compoents are just sign inverted against each other. This might exploit a similar effect as cross-component prediction (which also would give roughly similar as as per previous tool/core experiments). However, the new method has the advantage that it allows non-sequential decoding.

Slight increase of encoding time due to the need of checking that mode. Some fast decision is already used.

It is noted that at low QPs there might be a small drop of chroma PSNR, but still clear evidence of bit rate reduction.

There are CE related contributions (JVET-N0282, JVET-N0347) which seem to be extensions of this method rather than replacements.

Decision: Adopt JVET-N0054. Cross-checker confirmed that it is matching the implementation.

CE7-2 targets reducing the number of context coded bins in transform coding from 32 per 4x4 subblock to 30, which is achieved by formulating the budget of the limit differently for different subblock position, but in a way that it never exceeds Nx30, where N is the number of subblocks in a given transform block. In total, this may reduce the worst case number of context coded bins by around 5%, which does not appear to give a big deal. On the other hand, the definition of the limitation becomes more complicated, as it would be variable for different subblocks. There is only small loss in CTC, and some loss slightly above 0.1% average for low QP range.

No action, as the benefit is very low, and the specification text would be slightly more complicated.

CE7-3 and CE7-4 target some unification between the methods of bypass coding and remainder coding (by using the absolute value sum of the neighborhood template. CE7-3 has a minor advantage in compression (in the range of 0.01-0.05%, depending on class).

Decision: Adopt JVET-N0188. Alignment between text and software confirmed by cross-checker.

JVET-N0054 CE7: Joint coding of chrominance residuals (CE7-1) [J. Lainema (Nokia)]

JVET-N0188 CE7-7.3: Unified rice parameter derivation for coefficient level coding [Y. Piao, K. Choi (Samsung)]

JVET-N0345 CE7: Template based Rice parameter derivation for coding of abs_remainder (CE7-4) [Y.-H. Chao, M. Coban, M. Karczewicz (Qualcomm)]

JVET-N0361 CE7-2: Reduced number of context-coded bins [X. Zhao, X. Li, S. Liu (Tencent)]

## CE8: Screen content coding tools (9)

Contributions in this category were discussed Wednesday 20 March 1830–2215 (Track A chaired by JRO).

JVET-N0028 CE8: Summary Report on Screen Content Coding Tools [X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu]

| Test | Tester | Document | Tool description | Cross checker |
| --- | --- | --- | --- | --- |
| CE8-1.1a | J. Nam (LGE) | JVET-N0457 | Block vector prediction for merge mode and AMVP mode using default positions | X. Xu (Tencent) |
| CE8-1.1b | J. Nam (LGE) | JVET-N0457 | Block vector prediction for merge mode and AMVP mode using default positions with pruning invalid candidates | X. Xu (Tencent) |
| CE8-1.2b | J. Nam (LGE) | JVET-N0458 | Change binarization process for abs_mvd_minus2 syntax | K. Choi (Samsung) |
| CE8-1.2c | J. Nam (LGE) | JVET-N0458 | Add context models for abs_mvd_greater0_flag[] and/or abs_mvd_greater1_flag[] syntaxes | K. Choi (Samsung) |
| CE8-1.2d | J. Nam (LGE) | JVET-N0458 | Combination of 8.1.2b + 8.1.2c | K. Choi (Samsung) |
| CE8-1.3a | Y. Li (Wuhan U.) | JVET-N0255 | IBC with integer MMVD offsets | G. Venugopal (HHI) |
| CE8-1.3b | Y. Li (Wuhan U.) | JVET-N0255 | IBC with integer MMVD offsets and negative directions | G. Venugopal (HHI) |
| CE8-2.1 | Y.-H. Chao (Qualcomm) Y.-C. Sun (Alibaba) | JVET-N344 | Palette mode in HEVC SCC (except that separate palettes are used in case of separate luma/chroma trees) | R. Chernyak (Huawei) |
| CE8-2.2a | Y.-C. Sun (Alibaba) Y.-H. Chao (Qualcomm) R. Chernyak (Huawei) | JVET-N0404 | Separate palette coding for luma and chroma | B. Bross (HHI) |
| CE8-2.2c | Y.-C. Sun (Alibaba) | JVET-N0404 | Palette mode and simplified intra mode combination | C.-Y. Lai (MediaTek) |
| CE8-2.2e | Y.-C. Sun (Alibaba) Y.-H. Chao (Qualcomm) R. Chernyak (Huawei) | JVET-N0404 | Combine 8.2.2a and 8.2.2c | S. Yoo (LGE) |
| CE8-3.1a | F. Henry (Orange) T. Nguyen (HHI) X. Li (Tencent) | JVET-N0280 | Block-DPCM with H/V predictor from JVET-M0058 with residual coding from JVET-M0464 and context coded bins limitation from JVET-M0449 (max 2 context coded bins/sample) | P. Onno (Canon) |
| CE8-3.1b | F. Henry (Orange) T. Nguyen (HHI) X. Li (Tencent) | JVET-N0280 | Block-DPCM with H/V predictor from JVET-M0058 with residual coding from JVET-M0464 and context coded bins limitation from JVET-M0449 (max 3 context coded bins/sample) | P. Onno (Canon) |
| CE8-4.1a | S. Yoo (LGE) | JVET-N0428 | Rearrangement of the residual block according to intra mode for transform skip by flipping | B. Bross (HHI) |
| CE8-4.1b | S. Yoo (LGE) | JVET-N0428 | Rearrangement of the residual block of transform skip by rotating | X. Zhao (Tencent) K. Choi (Samsung) |
| CE8-4.1c | S. Yoo (LGE) | JVET-N0428 | Rearrangement of the residual block adaptively between rotation and flip for transform skip | M. Ikeda (Sony) |
| CE8-4.2a | S. Yoo (LGE) | JVET-N0429 | Residual coding for transform skip including context coded sign information and bitstream restriction for context coded bins limitation with max 2 context coded bins per sample | F. Henry (Orange) |
| CE8-4.2b | S. Yoo (LGE) | JVET-N0429 | Residual coding for transform skip including context coded sign information and bitstream restriction for context coded bins limitation with max 3 context coded bins per sample | Y.-C. Sun (Alibaba) |
| CE8-4.3a | T. Nguyen (HHI) | JVET-N0280 | Residual coding for transform skip and bitstream restriction for context coded bins limitation with max 2 context coded bins per sample | X. Xu (Tencent) |
| CE8-4.3b | T. Nguyen (HHI) | JVET-N0280 | Residual coding for transform skip and bitstream restriction for context coded bins limitation with max 3 context coded bins per sample | X. Xu (Tencent) |
| CE8-4.4a | T. Nguyen (HHI) X. Li (Tencent) | JVET-N0280 | Residual coding for transform skip and context coded bins limitation from JVET-M0449 with max 2 context coded bins per sample | S. Yoo (LGE) |
| CE8-4.4b | T. Nguyen (HHI) X. Li (Tencent) | JVET-N0280 | Residual coding for transform skip and context coded bins limitation from JVET-M0449 with max 3 context coded bins per sample | Y.-C. Sun (Alibaba) |
| CE8-5.1a | F. Henry (Orange) B. Bross (HHI) X. Li (Tencent) | JVET-N0214 | Combination of CE8-3.1a and CE8-4.4a (both use max 2 context coded bins per sample) | A. Filippov (Huawei) |
| CE8-5.1b | F. Henry (Orange) B. Bross (HHI) X. Li (Tencent) | JVET-N0214 | Combination of CE8-3.1b and CE8-4.4b (both use max 3 context coded bins per sample) | S. Yoo (LGE) |

Results:

|  |  | AI Over VTM-4.0 |  |  |  |  | RA Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CTC | CE8-1.1a | 0.00% | 0.00% | -0.02% | 101% | 99% | -0.03% | -0.04% | -0.08% | 102% | 101% |
|  | CE8-1.1b | -0.04% | -0.03% | -0.06% | 103% | 103% | -0.04% | -0.06% | -0.03% | 103% | 100% |
|  | CE8-1.2b | -0.01% | -0.01% | 0.00% | 100% | 99% | 0.00% | 0.02% | 0.00% | 100% | 100% |
|  | CE8-1.2c | 0.00% | -0.01% | -0.02% | 100% | 99% | 0.00% | 0.04% | -0.07% | 100% | 100% |
|  | CE8-1.2d | -0.01% | -0.02% | -0.03% | 100% | 99% | 0.00% | -0.03% | -0.04% | 99% | 99% |
|  | CE8-1.3a | -0.02% | -0.06% | -0.04% | 108% | 100% | -0.01% | -0.02% | -0.10% | 102% | 100% |
|  | CE8-1.3b | -0.01% | -0.04% | -0.02% | 108% | 100% | 0.00% | 0.07% | -0.02% | 101% | 100% |
|  | CE8-2.1 | 0.06% | 0.02% | 0.05% | 103% | 104% | 0.04% | -0.02% | -0.04% | 103% | 102% |
|  | CE8-2.2a | 0.06% | 0.02% | 0.05% | 101% | 98% | 0.05% | 0.02% | 0.00% | 102% | 99% |
|  | CE8-2.2c | 0.06% | 0.02% | 0.04% | 105% | 103% | 0.04% | -0.08% | -0.03% | 102% | 100% |
|  | CE8-2.2e | 0.06% | 0.02% | 0.04% | 106% | 102% | 0.04% | -0.04% | -0.02% | 101% | 98% |
|  | CE8-3.1a | -0.04% | -0.02% | -0.02% | 106% | 102% | 0.02% | 0.03% | -0.10% | 101% | 101% |
|  | CE8-3.1b | -0.04% | -0.01% | -0.03% | 106% | 102% | 0.02% | -0.07% | 0.00% | 101% | 100% |
|  | CE8-4.1a | -0.02% | -0.01% | 0.00% | 100% | 100% | -0.02% | -0.05% | -0.02% | 100% | 99% |
|  | CE8-4.1b | -0.07% | -0.05% | -0.02% | 100% | 99% | -0.04% | 0.04% | -0.11% | 99% | 99% |
|  | CE8-4.1c | -0.08% | -0.08% | -0.01% | 100% | 99% | -0.03% | -0.08% | -0.07% | 100% | 100% |
|  | CE8-4.2a | -0.12% | -0.04% | 0.01% | 101% | 105% | -0.02% | -0.06% | 0.00% | 101% | 102% |
|  | CE8-4.2b | -0.13% | -0.03% | 0.02% | 101% | 103% | -0.04% | -0.03% | -0.08% | 102% | 102% |
|  | CE8-4.3a | -0.11% | -0.02% | 0.02% | 100% | 102% | -0.02% | 0.01% | -0.08% | 100% | 101% |
|  | CE8-4.3b | -0.12% | -0.03% | 0.02% | 100% | 101% | -0.02% | -0.03% | -0.01% | 100% | 101% |
|  | CE8-4.4a | -0.12% | -0.03% | 0.02% | 100% | 101% | -0.02% | 0.02% | -0.05% | 100% | 100% |
|  | CE8-4.4b | -0.12% | -0.02% | 0.02% | 100% | 101% | -0.03% | -0.02% | -0.05% | 100% | 101% |
|  | CE8-5.1a | -0.12% | -0.03% | 0.03% | 106% | 102% | 0.02% | 0.00% | -0.03% | 101% | 100% |
|  | CE8-5.1b | -0.12% | -0.04% | 0.05% | 106% | 102% | 0.02% | 0.03% | 0.05% | 101% | 99% |
| Class F | CE8-1.1a | 0.02% | -0.03% | -0.09% | 101% | 100% | -0.02% | -0.04% | -0.08% | 100% | 100% |
|  | CE8-1.1b | -0.20% | -0.22% | -0.32% | 102% | 100% | -0.18% | -0.27% | -0.31% | 101% | 99% |
|  | CE8-1.2b | -0.21% | -0.17% | -0.15% | 100% | 100% | -0.21% | -0.28% | -0.26% | 100% | 100% |
|  | CE8-1.2c | -0.10% | -0.05% | -0.18% | 101% | 100% | -0.15% | -0.22% | -0.19% | 100% | 101% |
|  | CE8-1.2d | -0.32% | -0.36% | -0.38% | 100% | 101% | -0.26% | -0.44% | -0.33% | 100% | 100% |
|  | CE8-1.3a | -0.03% | -0.03% | -0.08% | 111% | 101% | -0.04% | -0.03% | -0.19% | 103% | 101% |
|  | CE8-1.3b | -0.01% | -0.07% | -0.19% | 108% | 101% | -0.08% | -0.09% | -0.13% | 102% | 101% |
|  | CE8-2.1 | -2.38% | -2.20% | -2.28% | 100% | 102% | -1.44% | -2.13% | -2.07% | 103% | 101% |
|  | CE8-2.2a | -2.38% | -2.20% | -2.28% | 98% | 96% | -1.44% | -1.97% | -1.89% | 100% | 98% |
|  | CE8-2.2c | -2.63% | -2.73% | -2.87% | 106% | 100% | -1.79% | -2.61% | -2.61% | 105% | 101% |
|  | CE8-2.2e | -2.63% | -2.73% | -2.87% | 106% | 96% | -1.75% | -2.44% | -2.30% | 105% | 101% |
|  | CE8-3.1a | -2.63% | -1.72% | -1.70% | 104% | 100% | -1.74% | -1.26% | -1.29% | 101% | 101% |
|  | CE8-3.1b | -2.78% | -1.74% | -1.84% | 104% | 100% | -1.81% | -1.45% | -1.35% | 101% | 101% |
|  | CE8-4.1a | -0.37% | -0.20% | -0.36% | 100% | 100% | -0.34% | -0.37% | -0.23% | 100% | 100% |
|  | CE8-4.1b | -0.58% | -0.30% | -0.42% | 100% | 100% | -0.47% | -0.27% | -0.28% | 100% | 100% |
|  | CE8-4.1c | -0.74% | -0.47% | -0.48% | 100% | 101% | -0.57% | -0.33% | -0.51% | 100% | 101% |
|  | CE8-4.2a | -2.03% | -0.82% | -0.79% | 104% | 104% | -1.10% | -0.32% | -0.27% | 103% | 104% |
|  | CE8-4.2b* | -2.91% | -1.55% | -1.71% | 103% | 105% | -1.92% | -1.22% | -1.10% | 103% | 105% |
|  | CE8-4.3a | -1.83% | -0.75% | -0.79% | 101% | 96% | -0.96% | -0.18% | -0.35% | 100% | 98% |
|  | CE8-4.3b | -2.67% | -1.41% | -1.56% | 100% | 96% | -1.77% | -1.05% | -1.01% | 100% | 100% |
|  | CE8-4.4a | -2.57% | -1.39% | -1.36% | 99% | 96% | -1.74% | -0.99% | -0.92% | 99% | 100% |
|  | CE8-4.4b | -2.79% | -1.52% | -1.70% | 99% | 98% | -1.93% | -1.22% | -1.18% | 99% | 100% |
|  | CE8-5.1a | -3.77% | -2.26% | -2.31% | 103% | 100% | -2.65% | -1.71% | -1.77% | 101% | 102% |
|  | CE8-5.1b | -3.94% | -2.38% | -2.59% | 104% | 101% | -2.80% | -1.82% | -1.96% | 101% | 101% |
| SCC TGM | CE8-1.1a | 0.02% | -0.01% | -0.01% | 100% | 100% | -0.06% | -0.01% | 0.00% | 99% | 99% |
|  | CE8-1.1b | -0.72% | -0.67% | -0.68% | 101% | 101% | -0.51% | -0.45% | -0.54% | 100% | 98% |
|  | CE8-1.2b | -0.58% | -0.55% | -0.54% | 100% | 100% | -0.25% | -0.21% | -0.25% | 100% | 100% |
|  | CE8-1.2c | -0.24% | -0.27% | -0.25% | 100% | 100% | -0.12% | -0.11% | -0.08% | 100% | 100% |
|  | CE8-1.2d | -0.85% | -0.80% | -0.81% | 100% | 100% | -0.35% | -0.20% | -0.32% | 100% | 101% |
|  | CE8-1.3a | -0.16% | -0.17% | -0.17% | 112% | 101% | -0.06% | 0.00% | -0.04% | 105% | 100% |
|  | CE8-1.3b | -0.16% | -0.17% | -0.17% | 110% | 100% | -0.12% | -0.05% | -0.06% | 104% | 100% |
|  | CE8-2.1 | -6.85% | -5.66% | -5.29% | 91% | 94% | -3.30% | -3.16% | -3.03% | 100% | 100% |
|  | CE8-2.2a | -6.85% | -5.66% | -5.29% | 90% | 93% | -3.18% | -2.80% | -2.52% | 98% | 97% |
|  | CE8-2.2c | -8.09% | -6.55% | -6.16% | 99% | 94% | -3.87% | -3.62% | -3.48% | 102% | 100% |
|  | CE8-2.2e | -8.09% | -6.55% | -6.16% | 99% | 91% | -3.68% | -3.36% | -3.01% | 101% | 98% |
|  | CE8-3.1a | -3.51% | -2.25% | -2.20% | 103% | 99% | -1.61% | -0.81% | -0.87% | 100% | 101% |
|  | CE8-3.1b | -3.78% | -2.41% | -2.35% | 102% | 99% | -1.68% | -0.92% | -0.92% | 100% | 101% |
|  | CE8-4.1a | -0.45% | -0.30% | -0.30% | 100% | 100% | -0.38% | -0.22% | -0.21% | 100% | 101% |
|  | CE8-4.1b | -0.57% | -0.34% | -0.33% | 100% | 99% | -0.31% | -0.07% | -0.07% | 100% | 100% |
|  | CE8-4.1c | -0.74% | -0.48% | -0.48% | 100% | 100% | -0.53% | -0.18% | -0.19% | 100% | 100% |
|  | CE8-4.2a | -2.13% | -0.04% | -0.16% | 106% | 99% | -1.05% | 0.67% | 0.72% | 102% | 100% |
|  | CE8-4.2b* | -4.38% | -2.03% | -2.13% | 104% | 102% | -2.78% | -0.87% | -0.82% | 102% | 102% |
|  | CE8-4.3a | -1.85% | 0.14% | 0.01% | 103% | 99% | -0.90% | 0.74% | 0.81% | 99% | 100% |
|  | CE8-4.3b | -4.00% | -1.77% | -1.84% | 101% | 99% | -2.58% | -0.72% | -0.64% | 99% | 101% |
|  | CE8-4.4a | -3.87% | -1.71% | -1.81% | 100% | 99% | -2.67% | -0.74% | -0.63% | 98% | 101% |
|  | CE8-4.4b | -4.37% | -2.12% | -2.19% | 100% | 100% | -2.96% | -0.98% | -0.97% | 98% | 101% |
|  | CE8-5.1a | -5.83% | -3.24% | -3.25% | 102% | 98% | -3.54% | -1.23% | -1.13% | 99% | 101% |
|  | CE8-5.1b | -6.23% | -3.63% | -3.63% | 102% | 97% | -3.77% | -1.56% | -1.48% | 99% | 101% |

Table 4: CE8 test results with VTM-4.0+IBC=0

|  |  | AI Over VTM-4.0 |  |  |  |  | RA Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CTC | CE8-2.1 | 0.06% | 0.06% | 0.04% | 102% | 105% | 0.04% | -0.04% | -0.03% | 104% | 104% |
|  | CE8-2.2a | 0.06% | 0.06% | 0.04% | 102% | 103% | 0.04% | -0.04% | -0.03% | 102% | 102% |
|  | CE8-2.2c | 0.06% | 0.06% | 0.04% | 102% | 107% | 0.03% | 0.00% | 0.02% | 105% | 105% |
|  | CE8-2.2e | 0.06% | 0.06% | 0.04% | 102% | 105% | 0.03% | 0.00% | 0.02% | 103% | 102% |
|  | CE8-3.1a | -0.06% | -0.01% | 0.03% | 109% | 101% | 0.02% | 0.03% | -0.10% | 101% | 101% |
|  | CE8-3.1b | -0.06% | -0.01% | 0.00% | 109% | 102% | 0.01% | -0.01% | -0.13% | 102% | 101% |
|  | CE8-4.1a | -0.03% | -0.04% | -0.03% | 100% | 100% | -0.02% | -0.06% | -0.03% | 101% | 101% |
|  | CE8-4.1b | -0.08% | -0.03% | -0.06% | 100% | 100% | -0.04% | -0.02% | -0.08% | 100% | 100% |
|  | CE8-4.1c | -0.08% | -0.02% | -0.06% | 100% | 101% | -0.04% | -0.02% | -0.22% | 101% | 101% |
|  | CE8-4.2a | -0.12% | -0.05% | -0.02% | 100% | 103% | -0.02% | -0.03% | -0.13% | 103% | 103% |
|  | CE8-4.2b* | -0.13% | -0.05% | -0.05% | 99% | 98% | -0.02% | -0.03% | -0.02% | 100% | 99% |
|  | CE8-4.3a | -0.12% | -0.04% | -0.01% | 100% | 99% | -0.03% | -0.04% | -0.10% | 99% | 99% |
|  | CE8-4.3b | -0.13% | -0.04% | -0.03% | 100% | 99% | -0.03% | -0.02% | -0.10% | 99% | 99% |
|  | CE8-4.4a | -0.13% | -0.05% | -0.03% | 100% | 100% | -0.04% | 0.00% | -0.04% | 99% | 99% |
|  | CE8-4.4b | -0.13% | -0.04% | -0.02% | 100% | 100% | -0.02% | -0.01% | -0.06% | 99% | 98% |
|  | CE8-5.1a | -0.14% | 0.00% | 0.02% | 109% | 102% | 0.02% | 0.01% | -0.13% | 101% | 100% |
|  | CE8-5.1b | -0.14% | -0.01% | 0.02% | 109% | 102% | 0.01% | 0.00% | -0.12% | 101% | 100% |
| Class F | CE8-2.1 | -8.58% | -7.71% | -7.86% | 94% | 99% | -6.72% | -7.66% | -7.47% | 103% | 103% |
|  | CE8-2.2a | -8.58% | -7.71% | -7.86% | 94% | 96% | -6.72% | -7.52% | -7.22% | 103% | 101% |
|  | CE8-2.2c | -8.79% | -7.85% | -8.08% | 103% | 101% | -7.22% | -8.13% | -7.97% | 105% | 103% |
|  | CE8-2.2e | -8.79% | -7.85% | -8.08% | 103% | 98% | -7.21% | -7.96% | -7.75% | 105% | 101% |
|  | CE8-3.1a | -5.90% | -4.52% | -4.51% | 109% | 97% | -4.04% | -3.48% | -3.29% | 102% | 100% |
|  | CE8-3.1b | -6.26% | -4.90% | -4.82% | 108% | 98% | -4.37% | -3.76% | -3.69% | 102% | 100% |
|  | CE8-4.1a | -1.06% | -0.93% | -0.85% | 100% | 100% | -0.88% | -0.73% | -0.60% | 101% | 101% |
|  | CE8-4.1b | -0.86% | -0.66% | -0.70% | 100% | 100% | -0.68% | -0.43% | -0.42% | 101% | 101% |
|  | CE8-4.1c | -1.11% | -1.01% | -0.99% | 98% | 102% | -0.88% | -0.64% | -0.55% | 101% | 102% |
|  | CE8-4.2a | -3.80% | -2.36% | -2.33% | 106% | 102% | -2.12% | -1.37% | -1.20% | 103% | 103% |
|  | CE8-4.2b | -5.52% | -3.87% | -3.90% | 104% | 100% | -3.60% | -2.59% | -2.40% | 104% | 103% |
|  | CE8-4.3a | -3.47% | -2.19% | -2.25% | 103% | 98% | -1.96% | -1.30% | -1.22% | 101% | 101% |
|  | CE8-4.3b | -5.25% | -3.64% | -3.70% | 102% | 96% | -3.41% | -2.56% | -2.35% | 101% | 101% |
|  | CE8-4.4a | -4.70% | -2.98% | -3.14% | 101% | 97% | -2.94% | -2.08% | -1.78% | 101% | 101% |
|  | CE8-4.4b | -5.48% | -3.84% | -3.85% | 101% | 96% | -3.65% | -2.69% | -2.44% | 100% | 101% |
|  | CE8-5.1a | -7.19% | -5.37% | -5.42% | 108% | 94% | -4.95% | -3.92% | -3.77% | 101% | 99% |
|  | CE8-5.1b | -7.80% | -5.97% | -5.94% | 108% | 96% | -5.50% | -4.37% | -4.26% | 101% | 99% |
| SCC TGM | CE8-2.1 | -27.12% | -25.34% | -25.16% | 75% | 81% | -13.17% | -13.41% | -13.18% | 99% | 100% |
|  | CE8-2.2a | -27.12% | -25.34% | -25.16% | 75% | 81% | -13.19% | -12.80% | -12.27% | 99% | 98% |
|  | CE8-2.2c | -29.14% | -26.79% | -26.63% | 87% | 83% | -14.51% | -14.35% | -14.25% | 101% | 100% |
|  | CE8-2.2e | -29.14% | -26.79% | -26.63% | 87% | 81% | -14.55% | -13.85% | -13.47% | 101% | 98% |
|  | CE8-3.1a | -15.10% | -12.15% | -12.24% | 108% | 93% | -7.09% | -5.25% | -5.29% | 101% | 99% |
|  | CE8-3.1b | -16.09% | -13.20% | -13.29% | 106% | 94% | -7.72% | -5.89% | -5.86% | 100% | 99% |
|  | CE8-4.1a | -2.19% | -1.90% | -1.92% | 100% | 98% | -1.17% | -1.00% | -0.98% | 101% | 101% |
|  | CE8-4.1b | -1.84% | -1.68% | -1.67% | 99% | 99% | -0.77% | -0.72% | -0.72% | 101% | 101% |
|  | CE8-4.1c | -2.20% | -2.00% | -1.99% | 97% | 100% | -1.11% | -0.98% | -0.97% | 100% | 101% |
|  | CE8-4.2a | -9.61% | -6.94% | -7.24% | 109% | 96% | -4.28% | -2.36% | -2.36% | 102% | 100% |
|  | CE8-4.2b | -14.31% | -10.94% | -11.16% | 106% | 94% | -7.12% | -4.77% | -4.73% | 102% | 100% |
|  | CE8-4.3a | -8.75% | -6.33% | -6.59% | 107% | 92% | -3.81% | -2.08% | -2.09% | 100% | 99% |
|  | CE8-4.3b | -13.76% | -10.44% | -10.68% | 104% | 92% | -6.81% | -4.44% | -4.47% | 100% | 98% |
|  | CE8-4.4a | -12.60% | -9.23% | -9.46% | 101% | 89% | -6.27% | -3.93% | -3.90% | 99% | 99% |
|  | CE8-4.4b | -14.35% | -10.94% | -11.15% | 101% | 91% | -7.27% | -4.88% | -4.88% | 99% | 100% |
|  | CE8-5.1a | -17.91% | -14.15% | -14.34% | 107% | 90% | -9.29% | -6.39% | -6.41% | 99% | 99% |
|  | CE8-5.1b | -19.10% | -15.39% | -15.56% | 106% | 91% | -10.08% | -7.25% | -7.25% | 99% | 98% |

* Results are updated after contribution deadline.

Discussion in context of CE review for CE8-1

CE8-1.x are all targeting improvement of compression performance of IBC vector coding, but do not achieve much (relatively) compared to the overall compression benefit of IBC for SC sequences (0.8% compared to > 40% for TGM class).

The question is if the main design goal of IBC specific coding of vectors should be simplification or (significant) improvement of compression (or both). It is agreed that the main design goal should be significant simplification, without affecting or (if possible) increasing compression. However, methods that would significantly increase compression without overly adding complexity would also be considered. In the context of SC sequences (in partcular TGM class) significant probably means several percent. The current proposals in CE8-1 do not fulfill these criteria.

As a matter of fact, and aspects that might be relevant to consider in that context

- IBC has clear benefit from 4x4 blocks (whereas motion comp is currently giving up 4x4 in regular inter mode) (for example, it is reported informally by one expert that disallowing 4x4, 4x8 and 8x4 in IBC would increase the average rate by approx. 18% for the TGM class).

- There is a recursive dependency in IBC vector coding (in merge and prediction stages), which is currently solved by using shared merge lists

- Is the merge list derivation more complicated than MPM derivation in conventional intra (where the same kind of recursion occurs, but is not seen as a problem). If not, we do not urgently need to target simplification.

- The sample prediction itself is not a problem, as it is integer based (simpler than other intra modes)

- Possibly, if a sufficiently simplified merge would be designed, the sharing concept could be given up (provided that it provides benefit in terms of compression performance)

It is noted that nobody knows if there is impact of merge list sharing for IBC performance in screen content. This aspect should be investigated in next CE if there would be a target of simplified coding methods. (revisit - possibly JVET-N0093 answers this question – if yes, delete this sentence)

CE8-2: Palette mode

8-2.2a is implementing separate palettes for luma and chroma, regardless if it is combined or separate luma/chroma trees. The results indicate that this has a loss (not very significant) in particular for chroma in random access, compared to the default palette mode of 8-2.1 (for intra, it is identical, because separate trees are used by default). This seems to indicate that separate palettes for luma and chroma are not beneficial in case of combined tree. This behaviour is independent from enabling or disabling IBC.

8-2.2c is using a special palette entry that indicates to copy a sample from a neighbored block, where a CU level flag indicates whether this sample is the boundary of the same row in the left neighbor block, or the same column in the block above. This provides an additional gain of 1.2%/0.7% for TGM class and 0.25%/0.35% for class F, compared to 8-2.1 in AI/RA configuration, for the case where IBC is enabled. Decoder-complexity wise this is simple, but encoder runtime increases compared to 8-2.1.

8-2.2e is combination of 8-2.2a/c, which does not show benefit over c standalone

As a general conclusion, palette mode is a non-insignificant additional building block which should be justified by significant improvement of compression performance. The best version currently known (CE8-2.2c) provides 2.6%/1.8% for class F, and 8%/3.9% for TGM, in AI/RA mode. This is asserted to be not sufficient to justify inclusion. It is however noted that contributions in the context of 4:4:4 support indicate that palette mode has much bigger advantage in combination with IBC, when operated on 4:4:4 screen content.

It is noted that the gains are somewhat higher in low QP range which was tested additionally

The discussion on 4:4:4 aspects has to be conducted in the plenary. Based on the outcome, it might be desirable to include testing of 4:4:4 in the next version of CE8. Revisit.

8-3 is using a modified residual coding from 8-4.4 proposal, and therefore will be discussed after

About the different 8-4 variants:

8-4.1a is using VTM TS residual coding, but performs flipping of 4x4 blocks based on intra prediction mode, either hor, or vert, or not.

8-4.1b is using VTM TS residual coding, but performs 180 rotation (always), similar as HEVC but for all block sizes.

8-4.1c is combining a/b, where 4x4 blocks are rotated if they have not been flipped, Other blocks are always rotated.

8-4.2a/b is based on 8-4.3a/b, but it can switch to another method of sign coding based on the number of nonzero coefficients.

8-4.3a/b is the proposal for alternative coding of TS residual as from JVET-M0464, with restriction of average number of context coded bins per sample, implemented as bitstream restriction

8-4.4a/b is derived from 8-4.3a/b, where however the bitstream restriction is replaced by a counter based limitation (from JVET-M0449), where a decoder would know that bypass coding is used as soon as the counter limit is reached.

a/b above means that the limit is 2 resp. 3 context coded bins per sample (for 8-4.2 through 8-4.4). This limit applies per TU.

The differences compared to VTM are:

- no last signif. Position

- modification of subblock CBF inference

- context coding of sign flag

- sign coded at beginning

- scan direction is changed (forward)

Benefit is observed only for screen content, so it should be considered as SCC tool. However, it would be undesirable if different residual coding would be used for TS in screen and natural content. There are no losses (and perhaps some small gain such as 0.1% for AI) for natural content as well.

Gain is 4.3%/3% for AI/RA in TGM, and 2.8%/1.9% in class F (with IBC enabled). This is for the max 3 context coded bins. For 2 context coded bins, the gain is 3.8%/2.6% for AI/RA class TGM, 2.5%/1.7% for class F. (these results relate to CE8-4.4, which is the conceptually best solution w.r.t limitation of CC bins)

The method uses similar building blocks as the current TS (resp. transform coeff.) coding, also similar level of complexity (at least for the case of max two context coded bins). The necessary modifications for implementing this additionally are quite common, such as replacing bypass by context coding, changing scan orders, etc. All this seems acceptable complexity-wise, and the gain is in an interesting range for the screen content cases.

Decision: Adopt JVET-N0280, method 8-4.4a, counter-based limitation with 2 context coded bins. Revisit: Conformance of text with software to be confirmed by cross-checker.

JVET-N0214 CE8: BDPCM with harmonized residual coding and CCB limitation (CE8-3.1a, CE8-3.1b, CE8-5.1a, CE8-5.1b) [G. Clare (bcom), F. Henry (Orange), B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand (HHI), M. Xu, X. Li, X. Xu, M. Gao, S. Liu (Tencent)]

JVET-N0255 CE8: Combination of MMVD and IBC mode (test 1.3a and 1.3b) [Y. Li, Z. Chen (Wuhan Univ.), X. Xu, X. Li, S. Liu (Tencent)]

JVET-N0527 Crosscheck of JVET-N0255 (CE8-1.3a and CE8-1.3b) [G. Venugopal (HHI)] [miss] [late]

JVET-N0280 CE8: Residual Coding for Transform Skip Mode (CE8-4.3a, CE8-4.3b, CE8-4.4a, and CE8-4.4b) [B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-N0344 CE8: Palette Mode in HEVC (CE8-2.1) [Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm), Y.-C. Sun, T.-S. Chang, J. Lou (Alibaba)]

JVET-N0404 CE8: Palette Mode Improvements (CE8-2.2) [Y.-C. Sun, T.-S. Chang, J. Lou (Alibaba), Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm), R. Chernyak, S. Ikonin, J. Chen (Huawei)]

JVET-N0428 CE8-4.1: Rearrangement of the residual block for transform skip [S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim (LGE)]

JVET-N0429 CE8-4.2: Residual Coding for transform skip with various maximum context coded bins [S. Yoo, J. Choi, J. Heo, J. Choi, J. Lim, S. Kim (LGE)]

JVET-N0596 Cross-chreck of JVET-N0429 (CE8-4.2: Residual Coding for transform skip with various maximum context coded bins) [F. Henry (Orange)] [miss] [late]

JVET-N0457 CE8-1.1: Block vector prediction for IBC [J. Nam, J. Lim, S. Kim (LGE)]

JVET-N0458 CE8-1.2: Block vector coding for IBC [J. Nam, J. Lim, S. Kim (LGE)]

## CE9: Decoder motion vector derivation (7)

JVET-N0029 CE9: Summary report on decoder motion vector derivation [S. Esenlik, X. Xiu]

This was discussed in Track B Wednesday 20 March 1645 (GJS).

The tools in the scope of this CE include bi-directional optical flow (BDOF) and decoder motion vector refinement (DMVR).

The core experiment summary report is organized into 2 sub-tests as follows:

- CE9-1: DMVR Specific Simplification (3 tests)

- CE9-2: BDOF Specific Simplification (9 tests)

This report summarises the status of each experiment. Crosscheck results are integrated in the report.

CE9-1: DMVR simplifications

| # | Description | Document |
| --- | --- | --- |
| CE9-1.1a | Disable 8x8/4xN CUs for DMVR | JVET-N0407 |
| CE9-1.1b | CE9-1.1.a + Remove boundary padding process | JVET-N0407 |
| CE9-1.2 | Impact of bit-depth restriction applied to interpolated samples used for refinement in DMVR. | JVET-N0291 |

Summary of results

| Test | Description | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| CE9-1.1a | Disable 8x8/4xN CUs for DMVR. | 0.04% | 0.07% | 0.03% | 100% | 99% |
| CE9-1.1b | CE9-1.1.a + Remove boundary padding process | 0.04% | 0.04% | 0.05% | 99% | 99% |
| CE9-1.2 | Impact of bit-depth restriction applied to interpolated samples used for refinement in DMVR | -0.01% | -0.04% | -0.04% | 101% | 101% |

CE9-1.1b somewhat increases average memory access. There was discussion of whether the worst case is affected (vs 8x16 biprediction). In hardware the padding is probably preferable, whereas in software the padding may impose an extra burden. Padding is also used in BDOF. The argument for eliminating the padding is primarily design consistency (not having some different process for MC prediction depending on whether it’s for DMVR or not). The opinions about this were mixed – seemingly a matter of hardware versus software perspectives – so this aspect was retained as in the existing draft (i.e., keep the padding).

Decision (complexity reduction): Adopt CE-9-1.1a (draft text in JVET-N0407).

CE9-1.2 uses 8-bit SAD rather than 10-bit SAD (just discarding two LSBs), motivated by the belief that this would be easier in SIMD. It was commented that this is not so helpful for newer devices (AVX2 and various DSP chips), as they have support for wider bit depths.

The cross-checker said that the SIMD code had not been modified to take advantage of the bit depth reduction.

There was no loss observed, which was suggested to be because the LSBs are noisy.

Aside from reduced computation, this was said to have a reduced memory requirement – e.g., 40 bytes in hardware (not very much).

The potential benefit from this seemed very small, so no action was taken.

CE9-2: BDOF simplifications

| # | Description | Document |
| --- | --- | --- |
| CE9-2.1 | Disable SAD based early termination | JVET-N0177 |
| CE9-2.2a | The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. | JVET-N0187 |
| CE9-2.2b | The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. And an early termination for 2nd stage (4x4 block level) is disabled. | JVET-N0187 |
| CE9-2.2c | The early termination for 1st stage (CU level) is disabled. | JVET-N0187 |
| CE9-2.3a | When a CU contains more than 256 luma samples, it is split into multiple sub-blocks with 256 luma samples and BDOF is performed for each sub-block. | JVET-N0270 |
| CE9-2.3b | SAD is calculated only for 4 corners (instead of all 16 luma samples) of each 4x4 sub-block. | JVET-N0270 |
| CE9-2.4 | Implicitly split BDOF application region along 16x16 boundaries | JVET-N0178 |
| CE9-2.5 | Simplification of BDOF’s optical flow parameter derivation | JVET-N0198 |
| CE9-2.6 | CE9.2.1 + CE9.2.4 | JVET-N0178 |

| Test | Description | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| CE9-2.1 | Disable SAD based early termination | -0.02% | -0.01% | 0.00% | 101% | 104% |
| CE9-2.2a | The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. | 0.03% | -0.01% | -0.03% | 100% | 102% |
| CE9-2.2b | The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. And an early termination for 2nd stage (4x4 block level) is disabled. | 0.03% | 0.00% | -0.02% | 101% | 102% |
| CE9-2.2c | The early termination for 1st stage (CU level) is disabled. | 0.00% | -0.01% | 0.01% | 101% | 103% |
| CE9-2.3a | When a CU contains more than 256 (luma samples, it is split into multiple sub-blocks with 256 luma samples and BDOF is performed for each sub-block. | 0.01% | -0.01% | -0.02% | 100% | 100% |
| CE9-2.3b | SAD is calculated only for 4 corners (instead of all 16 luma samples) of each 4x4 sub-block. | 0.01% | -0.03% | -0.03% | 99% | 100% |
| CE9-2.4 | Implicitly split BDOF application region along 16x16 boundaries | 0.01% | 0.00% | -0.01% | 101% | 100% |
| CE9-2.5 | Simplification of BDOF optical flow parameter derivation | 0.10% | 0.04% | 0.06% | 99% | 100% |
| CE9-2.6 | CE9.2.1 + CE9.2.4 | 0.00% | 0.00% | -0.01% | 102% | 104% |

The current design is not compatible with the VPDU concept, which was considered a significant problem and is why some of these “simplification” proposals have increased runtimes. All of the proposals fix that problem except CE9-2.5, which is addressing a different issue.

CE9-2.5 was considered unrelated to the others; and it was agreed to discuss the others first.

It was suggested to focus on CE9-2.1 and CE9-2.4. (It was remarked that CE9-2.4 includes the basic concept of CE9-2.2.)

CE9-2.2a is only implicitly splitting the SAD computation region, whereas 2.4 is splitting the whole BDOF application region (e.g., applying padding around 16x16 padding), following the same splitting as when BDOF is applied after DMVR. It was remarked (and seemed generally agreed) that CE9-2.4 makes the BDOF design more consistent with DMVR. CE9-2.4 has two early termination stages – at the 16x16 and 4x4 levels. Some participants suggested removing one of the early termination stages but not both of them. The relationship to the SAD performed at the 16x16 level for DMVR was also discussed, with the suggestion to use that SAD measure for the early termination.

JVET-N0097, JVET-N0148, JVET-N0158, JVET-N0296 and JVET-N0507 are related non-CE contributions.

It was suggested to focus on JVET-N0296, which removes the 4x4-level early termination and uses the SAD for DMVR for 16x16-level early termination when DMVR is enabled, and when DMVR is not enabled, does not use 16x16-level early termination. It seemed odd that if an encoder chooses not to use DMVR, the decoder would not benefit from early termination.

JVET-N0097 has some similarities while keeping a form of early termination with DMVR disabled, but uses a different sample source for the SAD calculation in that case. There was discussion of whether it was desirable to need to support the different SAD for that case.

C9-2.3 was discussed, but the irregularity of the memory access was questioned.

Decision (complexity/cleanup): Adopt CE9-2.4.

CE9-2.5 modifies the BDOF equations by replacing multiplies with adds and subtracts. It was commented that there are many other multiplies needed in the design since BDOF is built upon conventional biprediction (which requires a substantial number of multiplies) and the savings is not so much to justify doing something different in this part. Also some loss was observed (as much as 0.48% on some sequences, although averaging 0.1%). So no action was taken on this.

JVET-N0177 CE9: Disabling SAD based early termination of BDOF (CE9-2.1) [S. Esenlik, B. Wang, A. M. Kotra, H. Gao, J. Chen (Huawei), K. Kondo, M. Ikeda, T. Suzuki (Sony)]

JVET-N0178 CE9: Implicit splitting of BDOF application region (CE9-2.4 and CE9-2.6) [H. Chen, X. Ma, S. Esenlik, H. Yang, J. Chen (Huawei)]

JVET-N0187 CE9-2.2: On early termination of BDOF [K. Kondo, M. Ikeda, T. Suzuki (Sony)]

JVET-N0198 CE9: Simplification of BDOF’s optical flow parameter derivation (Test 9.2.5) [J. Li, C.-W. Kuo, C. S.Lim (Panasonic)]

JVET-N0270 CE9-2.3: Modifications on BDOF [H. Liu, L. Zhang (Bytedance)]

JVET-N0291 CE9: Results of DMVR related Tests CE9-1.2 [S. Sethuraman (Ittiam)]

JVET-N0407 CE9: Removal of 4xN/8x8 CUs and Boundary Padding Process from DMVR (Test 9.1.1) [C.-C. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)]

## CE10: Combined intra/inter prediction (X)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0030 CE10: Summary Report on Combined Inter and Intra Prediction [C.-W. Hsu, M. Winken]

JVET-N0298 CE10: CIIP using explicit signaling of weights (CE10-1.2) [A. Seixas Dias, G. Kulupana, S. Blasi (BBC)]

JVET-N0302 CE10: CIIP with position-independent weights (Test CE10-1.1) [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)]

## CE11: Deblocking (4)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0031 CE11: Summary Report on Deblocking [A. Norkin, A. M. Kotra]

JVET-N0098 CE11-2.1: Deblocking for 4xN, Nx4 and 8xN and Nx8 block boundaries not aligned with 8x8 grids [K. Andersson, J. Enhorn, Z. Zhang, R. Sjöberg (Ericsson), A. M. Kotra, J. Chen, S. Esenlik, B. Wang, H. Gao, Y. Zhao (Huawei), C.-M. Tsai, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0181 CE11-2.3: Disabling of sub-pu deblocking [A. M. Kotra, S. Esenlik, J. Chen, B. Wang, H. Gao (Huawei)]

JVET-N0463 CE11-2.2 Disable deblocking filter for 4xN for vertical edge and Nx4 for Horizontal edge on 4x4 grid [H. Jang, J. Nam, J. Lim, S. Kim (LGE)]

JVET-N0595 CE11-1.1: Further information on conditionally signaled very strong deblocking [C. Helmrich (HHI)] [late]

## CE12: Tile set boundary motion compensation handling (2)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0032 CE12: Summary report on Tile Set Boundary Handling [Hendry, R. Skupin, W. Wan]

JVET-N0109 CE12/AHG12: Treating boundaries of independent tile groups as picture boundaries [Hendry, S. Hong, J. Chen, Y.-K. Wang (Huawei)]

JVET-N0356 CE12-2: Encoder-side tile group restriction [R. Skupin, Y. Sanchez, V. George, K. Sühring, T. Schierl (HHI)] [late]

## CE13: Neural-network based loop filtering (5)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0033 CE13: Summary Report on Neural Network based Filter for Video Coding [Y. Li, S. Liu, K. Kawamura]

JVET-N0110 CE13-1.1: Convolutional neural network loop filter [Y.-L. Hsiao, O. Chubach, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0653 Crosscheck of JVET-N0110 (CE13-1.1: Convolutional neural network loop filter) [H. Yin, R. Yang, X. Fang, S. Ma (??)] [late]

JVET-N0169 CE13-2.1: Convolutional Neural Network Filter (CNNF) for Intra Frame [?? (??)]

JVET-N0254 CE13: Dense Residual Convolutional Neural Network based In-Loop Filter (Test 2.2 and 2.3) [Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao, S. Liu, X. Li (Tencent)]

JVET-N0480 CE13-1.2: Adaptive convolutional neural network loop filter [H. Yin, R. Yang, X. Fang, S. Ma (??)]

JVET-N0513 CE13: Experimental results of CNN-based In-Loop Filter [Y. Dai, D. Liu, N. Yan, F. Wu (USTC)] [miss] [late]

JVET-N0710 CE13-2.6/CE13-2.7: Evaluation results of CNN based in-loop filtering [K. Kawamura, Y. Kidani, S. Naito (KDDI)] [late]

# Non-CE Technology proposals

## CE1 related – Post-prediction and post-reconstruction filtering (10)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0144 Non-CE1: Simplification of division calculation in local illumination compensation [Y. Yasugi, T. Ikai (Sharp)]

JVET-N0556 Crosscheck of JVET-N0144 (Non-CE1: Simplification of division calculation in local illumination compensation) [K. Abe (Panasonic)] [miss] [late]

JVET-N0171 CE1-related: Combination of LIC and affine [H. Chen, X. Ma, H. Yang, J. Chen (Huawei)]

JVET-N0720 Crosscheck of JVET-N0171 (CE1-related: Combination of LIC and affine) [H. Huang (Qualcomm)] [miss] [late]

JVET-N0206 CE1-related: Improvement of the neighboring reference samples restriction on LIC test [K. Abe, T. Toma (Panasonic)]

JVET-N0570 Crosscheck of JVET-N0206 (CE1-related: Improvement of the neighboring reference samples restriction on LIC test) [V. Seregin (Qualcomm)] [miss] [late]

JVET-N0304 CE1-related: combined bi-directional LIC and post-reconstruction filters mutually exclusive [P. Bordes, F. Urban, F. Galpin (Technicolor)]

JVET-N0552 Crosscheck of JVET-N0304 (CE1-related: combined bi-directional LIC and post-reconstruction filters mutually exclusive) [T. Chujoh, T. Ikai (Sharp)] [miss] [late]

JVET-N0620 Crosscheck of JVET-N0304 (CE1-related: combined bi-directional LIC and post-reconstruction filters mutually exclusive) [C.-M. Tsai (MediaTek)] [miss] [late]

JVET-N0718 Crosscheck of JVET-N0304 (CE1-related: combined bi-directional LIC and post-reconstruction filters mutually exclusive) [S. Ikonin (Huawei)] [late]

JVET-N0306 non-CE1: simplification of LIC parameters derivation [P. Bordes, F. Urban, F. Galpin (Technicolor)]

JVET-N0538 Crosscheck of JVET-N0306 (non-CE1: simplification of LIC parameters derivation) [Y. He (InterDigital)] [miss] [late]

JVET-N0307 CE1-related: on LIC reference samples selection [P. Bordes, F. Urban, F. Galpin (Technicolor)]

JVET-N0410 CE1-related: Simplification of LIC parameter derivation and its unification with CCLM [A. Filippov, V. Rufitskiy, J. Chen (Huawei)]

JVET-N0648 Cross-check of JVET-N0410 (CE1-related: Simplification of LIC parameter derivation and its unification with CCLM) [J. Chen (Alibaba)] [miss] [late]

JVET-N0663 Cross-check of JVET-N0410 (CE1-related: Simplification of LIC parameter derivation and its unification with CCLM) [S.-C. Lim, H. Lee, J. Lee, J. Kang (ETRI)] [miss] [late]

JVET-N0679 Crosscheck of JVET-N0410 (CE1-related: Simplification of LIC parameter derivation and its unification with CCLM) [J. Lainema (Nokia)] [miss] [late]

JVET-N0717 Crosscheck of JVET-N0410 (CE1-related: Simplification of LIC parameter derivation and its unification with CCLM) [S.-T. Hsiang (MediaTek)] [miss] [late]

JVET-N0727 Crosscheck of JVET-N0410 (CE1-related: Simplification of LIC parameter derivation and its unification with CCLM) [N. Choi (Samsung)] [miss] [late]

JVET-N0479 CE1-related: Hadamard transform domain in-loop filter also using bottom/right reconstructed samples [S. Ikonin, V. Stepin, A. Karabutov, J. Chen (Huawei)]

JVET-N0625 Crosscheck of JVET-N0479 (CE1-related: Hadamard transform domain in-loop filter also using bottom/right reconstructed samples) [K. Zhang (Bytedance)] [miss] [late]

JVET-N0482 CE1-related: Additional test results for CE1-7 tests [D. Rusanovskyy, K. Reuze, M. Karczewicz (Qualcomm)]

JVET-N0634 Crosscheck of JVET-N0482 (CE1-related: Additional test results for CE1-1.7 tests) [M. Abdoli, T. Guionnet (ATEME)] [late]

JVET-N0493 CE1-related: Multiplication-free bilateral loop filter [J. Ström, P. Wennersten, J. Enhorn, R. Sjöberg (Ericsson)]

JVET-N0716 CE1-related: Simplified and robust LIC parameter derivation unified with CCLM [A. Filippov, V. Rufitskiy, J. Chen (Huawei), C. Helmrich, H. Schwarz, D. Marpe, T. Wiegand (HHI)] [miss] [late]

## CE2 related – Subblock motion compensation (19)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0079 CE2-related: Applying SMR to subblock-based merging candidate list [Y.-C. Lin, C.-C. Chen, M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0599 Crosscheck of JVET-N0079 (CE2-related: Applying SMR to subblock-based merging candidate list) [Y. Han, W.-J. Chien (Qualcomm)] [miss] [late]

JVET-N0199 CE2-related: combined test of affine memory bandwidth complexity reduction (Test2.4.6+Test2.4.8) [J. Li, C.-W. Kuo, C. S.Lim (Panasonic), M. Zhou (Broadcom)]

JVET-N0623 Crosscheck of JVET-N0199 (CE2-related: Combined test of affine memory bandwidth complexity reduction (Test2.4.6+Test2.4.8)) [K. Zhang (Bytedance)] [miss] [late]

JVET-N0235 CE2-related: Symmetric MVD signalling [J. Luo, Y. He (InterDigital)]

JVET-N0618 Crosscheck of JVET-N0235 on improvements on bi-directional optical flow [X. Li (Tencent)] [miss] [late]

JVET-N0769 Crosscheck of JVET-N0235 (non-CE4: Symmetric MVD signaling) [H. Chen (Huawei)] [miss] [late]

JVET-N0236 CE2-related: Prediction refinement with optical flow for affine mode [J. Luo, Y. He (InterDigital)]

JVET-N0626 Crosscheck of JVET-N0236 (CE2-related: Prediction refinement with optical flow for affine mode) [K. Zhang (Bytedance)] [miss] [late]

JVET-N0651 Crosscheck of JVET-N0236 (CE2-related: Prediction refinement with optical flow for affine mode) test 1-2 [G. Li (Tencent)] [miss] [late]

JVET-N0736 Crosscheck of JVET-N0236 test 6 (CE2-related: Prediction refinement with optical flow for affine mode) [T. Ikai (Sharp)] [late]

JVET-N0266 CE2-related: Disabling bi-prediction or inter prediction for small blocks [H. Liu, L. Zhang (Bytedance)]

JVET-N0645 Cross-chreck of JVET-N0266 (CE2-related: Disabling bi-prediction or inter prediction for small blocks) [J. Chen (Alibaba)] [miss] [late]

JVET-N0268 CE2-related: Remove above-left affine inherited motion vector predictor [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-N0546 Crosscheck of JVET-N0268 (CE2-related: Remove above-left affine inherited motion vector predictor) [H. Chen (Huawei)] [miss] [late]

JVET-N0273 CE2-related: Affine motion compensation using 2x2 subblock [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-N0547 Crosscheck of JVET-N0273 (CE2-related: Affine motion compensation using 2x2 subblock) [H. Chen (Huawei)] [miss] [late]

JVET-N0334 CE2/4/9-related: Overflow prevention in motion field storage [X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-N0681 Crosscheck of JVET-N0334 (CE2/4/9-related: Overflow prevention in motion field storage) [P. Hanhart (InterDigital)] [miss] [late]

JVET-N0335 CE2/4-related: unification of MV rounding [X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-N0621 Crosscheck of JVET-N0335 (CE2/4-related: unification of MV rounding) [C.-M. Tsai (MediaTek)] [miss] [late]

JVET-N0336 CE2/4-related: On motion field storage reduction for spatial motion prediction [X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-N0515 Crosscheck of JVET-N0336 (CE2/4-related: On motion field storage reduction for spatial motion prediction) [Y.-L. Hsiao (MediaTek)] [miss] [late]

JVET-N0721 Crosscheck of JVET-N0336 (CE8-related: Modified limitation on context coded bins for CE8-4.3a and CE8-5.1a) [G. Clare, F. Henry (Orange)] [late]

JVET-N0369 CE2-related: On restriction of memory bandwidth consumption of affine mode (2x2 variant) [M. Zhou (Broadcom)]

JVET-N0575 Crosscheck of JVET-N0369 (CE2-related: On restriction of memory bandwidth consumption of affine mode (2x2 variant)) [K. Zhang (Bytedance)] [late]

JVET-N0399 CE2-related: Simplifications of interweaved affine mode [X. Li, G. Li, X. Xu, S. Liu (Tencent)]

JVET-N0624 Crosscheck of JVET-N0399 (CE2-related: Simplifications of interweaved affine mode) [K. Zhang (Bytedance)] [miss] [late]

JVET-N0456 CE2-related: Affine motion model derivation method for affine merge mode [W. Chen, J. Luo, Y. He (InterDigital)]

JVET-N0481 Non-CE2: Gbi inheritance for constructed affine merge candidate [J. Chen, R.-L. Liao, Y. Ye (Alibaba)]

JVET-N0655 Crosscheck of JVET-N0481 (Non-CE2: Gbi inheritance for constructed affine merge candidate) [H. Liu (Bytedance)] [miss] [late]

JVET-N0659 Crosscheck of JVET-N0481: Non-CE2: Gbi inheritance for constructed affine merge candidate [J. Luo (InterDigital)] [miss] [late]

JVET-N0504 CE2-related: simplification of 2x2 subblock based affine motion compensation [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

JVET-N0597 Crosscheck of JVET-N0504 (CE2-related: Simplified prediction refinement for affine motion compensation) [R.-L. Liao (Alibaba)] [miss] [late]

JVET-N0510 CE2-related: Phase-variant affine subblock motion compensation [T.-D. Chuang, C.-Y. Chen, C.-Y. Lai, Y.-W. Huang, S.-M. Lei (MediaTek)] [late]

JVET-N0676 Crosscheck of JVET-N0510 (CE2-related: Phase-variant affine subblock motion compensation) [Y.-C. Sun (Alibaba)] [miss] [late]

JVET-N0633 CE2-related: on the memory reduction for affine motion inheritance [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)] [miss] [late]

JVET-N0709 CE2-related: Computation reductions of interweaved affine mode [K. Kawamura, S. Naito (KDDI)] [late]

JVET-N0737 Non-CE2: Combination of affine MV clip and prediction refinement with optical flow [X. Li, G. Li, X. Xu, S. Liu (Tencent)] [late]

JVET-N0752 CE2-related: Supplementary results for CE2-5.5.c [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang (Bytedance)] [late]

## CE3 related – Intra prediction and mode coding (54)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0082 CE3-related: Constrained partitioning of chroma intra CBs [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0693 Crosscheck of JVET-N0082 (CE3-related: Constrained partitioning of chroma intra CBs) [C. Rosewarne (Canon)] [miss] [late]

JVET-N0129 Non-CE3: PDPC simplification [N. Choi, M. W. Park, K. Choi (Samsung)]

JVET-N0644 Cross-check of JVET-N0129 (Non-CE3: PDPC simplification) [A. K. Ramasubramonian (Qualcomm)] [miss] [late]

JVET-N0132 Non-CE3: Simplification of DC Prediction [J.-Y. Huo, J.-L. Wang, X.-W. Li, Y.-Z. Ma, W. Zhang, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

JVET-N0657 Cross-check of JVET-N0132 (Non-CE3: Simplification of DC Prediction) [X. Ma (Huawei)] [miss] [late]

JVET-N0139 CE3-related: Intra Angular Prediction and Modified PDPC Based on Two Reference Lines [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-N0143 Non-CE3: Simplification of multi-line intra [T. Hashimoto, E. Sasaki, T. Ikai (Sharp)]

JVET-N0758 Cross check of Non-CE3: Simplification of multi-line intra (JVET-N0143) [S. Jeong, K. Choi (Samsung)] [late]

JVET-N0151 CE3-related: Modified Chroma Derived Mode [D. Jiang, J. Lin, F. Zeng, C.Fang (Dahua)]

JVET-N0154 Non-CE3: Chroma Intra Default Modes Modification Based on Order of Luminance Modes [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-N0155 Non-CE3: Chroma Intra Default Modes Modification Based on Statistics of Luminance Modes [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-N0157 Non-CE3: MPM List Modification for Zero Reference Line [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-N0159 Non-CE3: Relaxation method of processing dependency in ISP for small blocks [K. Unno, K. Kawamura, Y. Kidani, S. Naito (KDDI)]

JVET-N0530 Crosscheck of JVET-N0159 (Non-CE3: Relaxation method of processing dependency of ISP for small blocks) [S. De- Luxán-Hernández (HHI)] [miss] [late]

JVET-N0767 Cross-check of JVET-N0159 (Non-CE3: Relaxation method of processing dependency in ISP for small blocks) [S. De-Luxán-Hernández (HHI)] [late]

JVET-N0164 CE3-related: Size restriction for CCLM [F. Chen, L. Wang (Hikvision)]

JVET-N0563 Crosscheck of JVET-N0164 (CE3-related: Size restriction for CCLM) [J. Choi (LGE)] [miss] [late]

JVET-N0170 CE3-related: Explicitly signal non angular modes with encoding changes [J. Yao, J. Zhu, W. Cai, K. Kazui (Fujitsu)] [late]

JVET-N0745 Crosscheck of JVET-N0170 (CE3-related: Explicitly signal non angular modes with encoding changes) [S. Iwamura (NHK)] [miss] [late]

JVET-N0183 Non-CE3: Shared MPM list [A. M. Kotra, S. Esenlik, J. Chen, B. Wang, H. Gao (Huawei)]

JVET-N0641 Crosscheck of JVET-N0183 (AHG16/Non-CE3: Shared MPM list) [Y.-C. Lin (MediaTek)] [miss] [late]

JVET-N0185 CE3-related: A unified MPM list for intra mode coding [B. Wang, A. M. Kotra, S. Esenlik, H. Gao, J. Chen (Huawei)]

JVET-N0750 Crosscheck of JVET-N0185 (CE3-related: A unified MPM list for intra mode coding) [M. G. Sarwer (MediaTek)] [miss] [late]

JVET-N0186 CE3-related: Simplification on Intra sub-partition coding mode [B. Wang, A. M. Kotra, S. Esenlik, H. Gao, J. Chen (Huawei)]

JVET-N0728 Crosscheck of JVET-N0186 (AHG16/Non-CE3: Simplification on Intra sub-partition coding mode) [Y.-W. Chen (Kwai Inc.)] [miss] [late]

JVET-N0215 Non-CE3: Intra prediction mode restriction in small blocks [D. Y. Lee, T. H. Kim, G. H. Park (KHU)]

JVET-N0660 Cross check of Non-CE3: Intra prediction mode restriction in small blocks (JVET-N0215) [M. W. Park (Samsung)] [late]

JVET-N0216 Non-CE3: Determination of wide-angle mode using the size of a coding block [D. Kim, J. Jung, G. Ko, J. Son, J. Kwak (WILUS)]

JVET-N0218 CE3-related: Chroma Intra Mode Coding with Lumped Luma Directions and Replacement for DM mode [J. Park, B. Jeon (SKKU)]

JVET-N0723 Cross check of JVET-N0218 (CE3-related: Chroma Intra Mode Coding with Lumped Luma Directions and Replacement for DM mode) [N. Choi (Samsung)] [miss] [late]

JVET-N0222 CE3-related: CIIP intra mode propagation to intra block [L. Li, J. Nam, J. Heo, J. Lim, S. Kim (LGE)]

JVET-N0223 CE3-related: reference sample filtering simplification [L. Li, J. Heo, J. Lim, S. Kim (LGE)]

JVET-N0224 CE3-related: ISP support for 4:2:2 and 4:4:4 [L. Li, M. Koo, J. Heo, J. Nam, J. Lim, S. Kim (LGE)]

JVET-N0229 Non-CE3: CCLM prediction for 4:2:2 and 4:4:4 color format [J. Choi, J. Heo, J. Lim, S. Kim (LGE)]

JVET-N0230 Non-CE3: Simplified intra mode candidates for ISP [J. Choi, J. Heo, S. Yoo, L. Li, J. Choi, J. Lim, S. Kim (LGE)]

JVET-N0665 Crosscheck of JVET-N0230 (Non-CE3: Simplified intra mode candidates for ISP) [J. Lee, S.-C. Lim, H. Lee, J. Kang (ETRI)] [miss] [late]

JVET-N0248 Non-CE3: Combined-Hypothesis Intra-Prediction with Unified Intra Mode Coding [G. Kulupana, A. Seixas Dias, S. Blasi (BBC)]

JVET-N0303 CE3-related: Simplified unified luma intra mode coding [S. Blasi, A. Seixas Dias, G. Kulupana (BBC)]

JVET-N0764 Cross check of JVET-N0303 [K. Misra (Sharp Labs of America)] [late]

JVET-N0766 Cross check of JVET-N0303 (Non-CE6: Further cost reduction for primary transform) [K. Choi (Samsung)] [miss] [late]

JVET-N0305 CE3-related: Luma intra mode coding with restricted MPM derivation [S. Blasi, A. Seixas Dias, G. Kulupana (BBC)]

JVET-N0531 Crosscheck of JVET-N0305 (CE3-related: Luma intra mode coding with restricted MPM derivation) [S. De-Luxán-Hernández (HHI)] [miss] [late]

JVET-N0308 AHG16/Non-CE3: Restriction of the maximum CU size for ISP to 64x64 [S. De-Luxán-Hernández, B. Bross, T. Nguyen, V. George, B. Stabernack, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-N0672 Cross-check of JVET-N0308 (AHG16/Non-CE3: Restriction of the maximum CU size for ISP to 64x64) [X. Cao (Hikvision)] [miss] [late]

JVET-N0313 CE3-related: Low latency intra sub-partitions [R. Vanam, Y. He (InterDigital)]

JVET-N0719 Crosscheck of JVET-N0313 (Low latency intra sub-partitions) [R. Jullian (Technicolor)] [late]

JVET-N0330 Non-CE3: Intra sub-partition coding without thin partitions [T.-C. Ma, X. Xiu, Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-N0333 CE3-related: MMLM only cross-component prediction [H.-J. Jhu, Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-N0607 Cross-check of JVET-N0333, "CE3-related: MMLM only cross-component prediction" [?? (??)] [miss] [late]

JVET-N0733 Cross-check of JVET-N0333 (CE3-related: MMLM only cross-component prediction) [A. K. Ramasubramonian (Qualcomm)] [miss] [late]

JVET-N0339 Non-CE3: Unification on WAIP for normal and ISP intra prediction [T.-C. Ma, X. Xiu, Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-N0581 Crosscheck of JVET-N0339 (Non-CE3: Unification on WAIP for normal and ISP intra prediction) [J. Heo, H. Jang, J. Lim, S. Kim (LGE)] [late]

JVET-N0342 Non-CE3: Decoder-side Intra Mode Derivation with Prediction Fusion [M. Abdoli, E. Mora, T. Guionnet, M. Raulet (ATEME)]

JVET-N0590 Crosscheck of JVET-N0342 (Decoder-side Intra Mode Derivation with Prediction Fusion) [?? (??)] [miss] [late]

JVET-N0358 Non-CE3: Harmonization of CBF coding for intra sub-partition coding mode [Z. Zhang, K. Andersson, R. Sjöberg (Ericsson)]

JVET-N0371 Non-CE3: directional intra prediction with varying angle [G. Rath, F. Urban, F. Racapé (Technicolor)]

JVET-N0591 Crosscheck of JVET-N0371 (Non-CE3: directional intra prediction with varying angle) [P. Merkle (HHI)] [late]

JVET-N0372 Non-CE3: ISP with independent sub-partitions for certain block sizes [S. De-Luxán-Hernández, B. Bross, T. Nguyen, V. George, B. Stabernack, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-N0536 Crosscheck of JVET-N0372 (Non-CE3: ISP with independent sub-partitions for certain block sizes) [X. Zhao (Tencent)] [miss] [late]

JVET-N0376 AHG16/CE3-related: CCLM mode restriction for increasing decoder throughput [J.-Y. Jung, Y.-L. Lee (Sejong Univ.), D.-Y. Kim, W. J. Jeong (Chips&Media), S.-C. Lim, J. Kang (ETRI)]

JVET-N0705 Cross-check of JVET-N0376 (AHG16/CE3-related: CCLM mode restriction for increasing decoder throughput) [G. Laroche (Canon)] [miss] [late]

JVET-N0390 AHG16/non-CE3: Study of CCLM restrictions in case of separate luma/chroma tree [E. François, F. Galpin, F. Le Léannec, T. Poirier (Technicolor)]

JVET-N0394 CE3-related: Unified MPM list based on CE3-3.3 and CE3-3.5.1 [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-N0583 Cross-check of JVET-N0394: CE3-related: Unified MPM list based on CE3-3.3 and CE3-3.5.1 [X. Xiu (Kwai Inc.)] [miss] [late]

JVET-N0396 CE3-related: Enabling parallel reconstruction of small intra-coded blocks [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, H. Huang, M. Karczewicz (Qualcomm)]

JVET-N0682 Crosscheck of JVET-N0396 (CE3-related: Enabling parallel reconstruction of small intra-coded blocks) [?? (??)] [miss] [late]

JVET-N0401 Non-CE3/Non-CE8: Enable Transform Skip in CUs using ISP [S. De-Luxán-Hernández, T. Nguyen, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-N0629 Crosscheck of JVET-N0401 (Non-CE3/Non-CE8: Enable Transform Skip in CUs using ISP) [T.-C. Ma (Kwai Inc.)] [miss] [late]

JVET-N0412 CE3-related: Modification on the intra luma mode coding process [Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI)]

JVET-N0749 Crosscheck of JVET-N0412 (CE3-related: Modification on the intra luma mode coding process) [S. Iwamura (NHK)] [miss] [late]

JVET-N0426 Non-CE3: History-based intra MPM default angular modes derivation [Z. Zhang, P. Wennersten, R. Yu, J. Ström, R. Sjöberg (Ericsson)]

JVET-N0427 CE3-related: Harmonization between ISP and WAIP [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)] [late]

JVET-N0627 Crosscheck of JVET-N0427 (CE3-related: Harmonization between ISP and WAIP) [T.-C. Ma (Kwai Inc.)] [miss] [late]

JVET-N0432 AHG16/Non-CE3: On 1xN and 2xN subpartitions in intra subpartition coding [A. K. Ramasubramonian, G. Van der Auwera, T. Hsieh, V. Seregin, L. Pham Van, M. Karczewicz]

JVET-N0726 Crosscheck of JVET-N0432 (AHG16/Non-CE3: On 1xN and 2xN subpartitions in intra subpartition coding) [N. Choi (Samsung)] [miss] [late]

JVET-N0433 CE3-related: Unification of MPM derivation for luma intra modes [A. K. Ramasubramonian, G. Van der Auwera, L. Pham Van, M. Karczewicz (Qualcomm)]

JVET-N0562 Crosscheck of JVET-N0433 (CE3-related: Unification of MPM derivation for luma intra modes) [M. G. Sarwer (MediaTek)] [miss] [late]

JVET-N0435 Non-CE3: Harmonization between WAIP and intra smoothing filters [P.-H. Lin, Y.-C. Yang (Foxconn)]

JVET-N0437 Non-CE3: On allowing non-MPM modes for ISP and non-zero reference line [F. Bossen, K. Misra (Sharp Labs of America)]

JVET-N0450 CE3-3.4-related: unified MPM list construction [F. Bossen, K. Misra (Sharp)]

JVET-N0704 Crosscheck of JVET-N0450 (CE3-3.4-related: unified MPM list construction) [S. Blasi, A. Seixas Dias, G. Kulupana (BBC)] [miss] [late]

JVET-N0452 Non-CE3: Harmonization between WAIP and ISP [J. Heo, H. Jang, L. Li, J. Choi, J. Lim, S. Kim (LGE)]

JVET-N0628 Crosscheck of JVET-N0452 (Non-CE3 : Harmonization between WAIP and ISP) [T.-C. Ma (Kwai Inc.)] [miss] [late]

JVET-N0453 AHG16/Non-CE3: on chroma 2xN and Nx2 intra prediction in single tree [Y. Zhao, A. Karabutov, H. Yang, J. Chen (Huawei)]

JVET-N0686 Crosscheck of JVET-N0453 Test 1 (AHG16/Non-CE3: on chroma 2xN and Nx2 intra prediction in single tree) [C. Rosewarne (Canon)] [late]

JVET-N0465 CE3-related: Restrict 2xN, Nx2 chroma processing for dual tree structure [H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)]

JVET-N0588 Crosscheck of JVET-N0465 (CE3-related: Restrict 2xN, Nx2 chroma processing for dual tree structure) [T. Zhou (Sharp)] [miss] [late]

JVET-N0469 CE3-related: Disable ISP mode for 128xN, Nx128 Block [H. Jang, J. Heo, J. Nam, J. Lim, S. Kim (LGE)]

JVET-N0475 Non-CE3/6: Enabling Transform Skip for ISP [T.-C. Ma, X. Xiu, Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-N0524 Non-CE3: Modified beta derivation in CCLM [M. Ikeda, T. Suzuki (Sony)] [miss] [late]

JVET-N0694 Crosscheck of JVET-N0524 (Non-CE3: Modified beta derivation in CCLM) [Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI)] [late]

## CE4 related – Inter prediction and motion vector coding (40)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0084 CE4-related: Motion compression for HMVP buffers and MV line buffer [Y.-L. Hsiao, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0085 CE4-related: MV rounding unification [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0578 Cross-check of JVET-N0085: CE4-related: MV rounding unification [X. Xiu (Kwai Inc.)] [miss] [late]

JVET-N0086 CE4-related: Reduction of interactions between bi-prediction coding tools [C.-C. Chen, M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0539 Crosscheck of JVET-N0086: CE4-related: Reduction of interactions between bi-prediction coding tools [X. Xu (Tencent)] [miss] [late]

JVET-N0586 Cross-check of JVET-N0086: CE4-related: Reduction of interactions between bi-prediction coding tools [H. Wang (Qualcomm)] [miss] [late]

JVET-N0087 CE4-related: Simplification of triangle merging candidate list derivation on top of CE4-4.1c [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0126 CE4-related: Signaling MMVD syntax elements [S. Jeong, M. W. Park, K. Choi (Samsung)]

JVET-N0127 CE4-related: MMVD enabling signaling in SPS [S. Jeong, M. Park, K. Choi (Samsung)]

JVET-N0165 CE4-related: On spatial candidate list construction [L. Xu, F. Chen, L. Wang (Hikvision)]

JVET-N0656 Crosscheck of JVET-N0165 (Non-CE4: On spatial candidate list construction) [S. H. Wang (PKU)] [miss] [late]

JVET-N0166 CE4-related: Advanced Multi-hypothesis Inter Prediction for bandwidth reduction in B frame [Y. Sun, F. Chen, L. Wang (Hikvision)]

JVET-N0699 Crosscheck of JVET-N0166 (Non-CE4: Advanced multi-hypothesis inter prediction for bandwidth reduction in B frame) [M.-S. Chiang (MediaTek)] [miss] [late]

JVET-N0167 CE4-related: Advanced Multi-hypothesis Inter Prediction [Y. Sun, F. Chen, L. Wang (Hikvision)]

JVET-N0700 Crosscheck of JVET-N0167 (Non-CE4: Advanced multi-hypothesis inter prediction) [M.-S. Chiang (MediaTek)] [miss] [late]

JVET-N0168 CE4-related: AMVR cost calculation modification in motion estimation stage [Y. Sun, F. Chen, L. Wang (Hikvision)]

JVET-N0200 Non-CE4: Simplification of inter MVP list generation for 4x4 block [J. Li, C.-W. Kuo, C. S.Lim (Panasonic)]

JVET-N0557 Crosscheck of JVET-N0200 (Non-CE4: Simplification of inter MVP list generation for 4x4 block) [Y. Yasugi (Sharp)] [miss] [late]

JVET-N0203 Non-CE4: Modification of the temporal merging candidate for the inter merge mode [G. Lee, G. Kim, S. Cha, D. Nam, J. Han (Sejong University)]

JVET-N0567 Crosscheck of JVET-N0203 (Non-CE4: Modification of the temporal merging candidate for the inter merge mode) [H. Lee, S. -C. Lim, J. Lee, J. Kang (ETRI)] [miss] [late]

JVET-N0213 CE4-related: Remove TMVP merge and AMVP candidate for the specified blocksizes [S. H. Wang (PKU), X. Zheng (DJI), S. S. Wang, S. W. Ma (PKU)] [placeholder] [late]

JVET-N0650 Crosscheck of JVET-N0213 (CE4-related: Remove TMVP merge and AMVP candidate for the specified block sizes) [G. Li (Tencent)] [miss] [late]

JVET-N0240 Non-CE4: Simplification of merge MVD derivation [G. Ko, D. Kim, J. Jung, J. Son, J. Kwak (WILUS)]

JVET-N0572 Crosscheck of JVET-N0240 (Non-CE4: Simplification of merge MVD derivation) [Y.-W. Chen (Kwai Inc.)] [miss] [late]

JVET-N0265 CE4-related: Simplification of HMVP in AMVP mode [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-N0756 Crosscheck of JVET-N0265 (CE4-related: Simplification of HMVP in AMVP mode) [Franck Galpin (Technicolor)] [late]

JVET-N0267 Non-CE4: shared merge list without double HMVP tables [L. Zhang (Bytedance)]

JVET-N0542 Crosscheck of JVET-N0267: Non-CE4: shared merge list without double HMVP tables [X. Xu (Tencent)] [miss] [late]

JVET-N0277 Non-CE4: Parallel friendly pair-wise merge candidate derivation [K. Zhang (Bytedance)]

JVET-N0560 Crosscheck of JVET-N0277 (Non-CE4: Parallel-friendly pairwise average merge candidate derivation) [T. Hashimoto (Sharp)] [miss] [late]

JVET-N0286 CE4-related: Simplified coding of the BPWA index [F. Le Léannec, Y. Chen, T. Poirier, A. Robert (Technicolor)]

JVET-N0517 Crosscheck of JVET-N0286 (CE4-related: Simplified coding of the BPWA index) [S.-T. Hsiang (MediaTek)] [miss] [late]

JVET-N0287 CE4-related: Simplified coding of the MVP index [F. Le Léannec, Y.Chen, T. Poirier, A. Robert (Technicolor)]

JVET-N0602 Crosscheck of JVET-N0287 (CE4-related: Simplified coding of the MVP index) [Y. Han, W.-J. Chien (Qualcomm)] [miss] [late]

JVET-N0309 Non-CE4: Switched half-pel interpolation filter [A. Henkel, B. Bross, M. Winken, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-N0707 Crosscheck of JVET-N0309 (Non-CE4: Switched half-pel interpolation filter) [N. Hu (Qualcomm)] [miss] [late]

JVET-N0332 Non-CE4: MVD scaling issue for LTRPs [Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-N0680 Crosscheck of JVET-N0332 (Non-CE4: MVD scaling issue for LTRPs) [G. Ko, D. Kim, J. Jung, J. Son, J. Kwak (WILUS)] [miss] [late]

JVET-N0340 CE4-related: An improved method for triangle merge list construction [X. Wang, Y.-W. Chen, X. Xiu, T.-C. Ma (Kwai Inc.)]

This was discussed in Track B Wednesday 20 March 1625 (GJS).

In this contribution, a method is proposed to derive uni-prediction merge list for triangle prediction. In this method, regular merge candidate list is re-used for triangle partition merge prediction with no extra motion vector pruning. For each merge candidate in the regular merge candidate list, one and only one of its L0 or L1 motion vector is used for triangle prediction. In addition, the order of selecting the L0 vs. L1 motion vector is based on its merge index parity. With the proposed scheme, the regular merge list can be directly used and there is no need to explicitly generate the uni-prediction merge candidate list for triangle prediction. Simulation results based on CTC reportedly show that the proposed scheme incurs an average 0.01% and 0.09% luma coding efficiency loss respectively for random access and low delay B configuration, and no chroma coding efficiency loss relative to VTM-4.0.

Complexity properties equivalent to what was collected for the CE are shown in the table below:

| Merge list size |  | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Others |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 13 | 9 | 2 | 2 | – |  |

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | 0.00% | 99% | 98% |
| Class A2 | -0.05% | -0.06% | 0.01% | 98% | 98% |
| Class B | 0.00% | -0.08% | -0.21% | 101% | 101% |
| Class C | 0.04% | 0.09% | -0.06% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.03% | -0.08% | 100% | 100% |
| Class D | 0.00% | 0.08% | 0.26% | 101% | 102% |
| Class F | 0.07% | 0.08% | 0.02% | 102% | 103% |

|  |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.00% | -0.15% | 101% | 103% |
| Class C | 0.12% | 0.27% | -0.08% | 99% | 97% |
| Class E | 0.13% | -0.43% | 0.08% | 96% | 98% |
| Overall | 0.09% | -0.02% | -0.07% | 99% | 100% |
| Class D | 0.20% | 0.38% | -0.24% | 100% | 104% |
| Class F | 0.04% | -0.15% | -0.04% | 101% | 99% |

Decision (complexity reduction / cleanup): Adopt.

JVET-N0587 Cross-check of JVET-N0340: CE4-related: An improved method for triangle merge list construction [H. Wang (Qualcomm)] [miss] [late]

JVET-N0373 Non-CE4: HMVP unification between the Merge and MVP list [H. Lee, J. Kang, S.-C. Lim, J. Lee, H. Y. Kim (ETRI)]

JVET-N0674 Crosscheck of JVET-N0373 (Non-CE4: HMVP unification between the Merge and MVP list) [R. Yu (Ericsson)] [miss] [late]

JVET-N0380 CE4-related: Fix of MMVD signalling [G. Li, X. Li, X. Xu, S. Liu (Tencent)]

JVET-N0760 Cross check of CE4-related: Fix of MMVD signalling (JVET-N0380) [S. Jeong, K. Choi (Samsung)] [late]

JVET-N0385 CE4-related: modification of HMVP update process inside the shared merging candidate list region [X. Xu, X. Li, G. Li, S. Liu (Tencent)]

JVET-N0606 Crosscheck of JVET-N0385 (Non-CE4: modification of HMVP update process inside the shared merging candidate list region) [N. Zhang, H. Liu (Bytedance)] [miss] [late]

JVET-N0400 CE4-related: Signaling of maximum number of triangle candidates [X. Li, G. Li, X. Xu, S. Liu (Tencent)]

JVET-N0647 Crosscheck of JVET-N0400 (CE4-related: Signaling of maximum number of triangle candidates) [Y.-J. Chang (Qualcomm)] [miss] [late]

JVET-N0434 Non-CE4: Unified context model of AMVR and Affine AMVR [Y.-C. Yang, P.-H. Lin (Foxconn)]

JVET-N0751 Crosscheck of JVET-N0434 (Non-CE4: Unified context model of AMVR and Affine AMVR) [Z.-Y. Lin (MediaTek)] [late]

JVET-N0439 CE4-related: supplementary information regarding CE4-4.4 [L. Zhang, K. Zhang, H. Liu, J. Xu, Y. Wang (Bytedance)]

JVET-N0447 Non-CE4: On triangle merge mode [Y.-J. Chang, C.-C. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-N0622 Crosscheck of JVET-N0447 on triangle merge mode [X. Li (Tencent)] [miss] [late]

JVET-N0448 Non-CE4: On MMVD signaling [Y.-J. Chang, W.-J. Chien, C.-C. Chen, M. Karczewicz (Qualcomm)]

JVET-N0761 Cross check of Non-CE4: On MMVD signaling (JVET-N0448) [S. Jeong, K. Choi (Samsung)] [miss] [late]

JVET-N0449 CE4-related: Simplification on MMVD distance table [Y. Zhang, Y.-J. Chang, C.-C. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-N0670 Crosscheck of JVET-N0449 (CE4-related: Simplification on MMVD distance table) [T.-H. Li, Y.-C. Yang (Foxconn)] [miss] [late]

JVET-N0462 Non-CE4: Mismatch between text specification and reference software on inter prediction mode [J. Nam, H. Jang, N. Park, J. Lim, S. Kim (LGE)]

JVET-N0471 Non-CE4: Simplication of decoding process for SMVD reference indices [H. Lee, J. Kang, S.-C. Lim, J. Lee, H. Y. Kim (ETRI)]

JVET-N0476 Non-CE4: Constraints of picture size in VVC [Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-N0483 Non-CE4: Simplification of triangle partition and SBT combination [R.-L. Liao, Y. Ye, J. Chen (Alibaba)]

JVET-N0598 Crosscheck of JVET-N0483: Non-CE4: Simplification of triangle partition and SBT combination [C.-H. Hung, W.-J. Chien (Qualcomm)] [late]

JVET-N0486 CE4-related: Simplification for share merge list and HMVP harmonization process [S. H. Wang (PKU), X. Zheng (DJI), S. S. Wang, S. W. Ma (PKU)]

JVET-N0643 Crosscheck of JVET-N0486 (CE4-related: Simplification for shared merge list and HMVP harmonization process) [Y.-C. Lin (MediaTek)] [miss] [late]

JVET-N0487 CE4-related: Further simplification of triangle prediction merging candidate list derivation [X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)]

JVET-N0738 Crosscheck of JVET-N0487 (CE4-related: Further simplification of triangle prediction merging candidate list derivation) [F. Chen (Hikvision)] [miss] [late]

JVET-N0499 CE4-related: Simplification for merge list derivation in triangular prediction mode on top of test CE4-4.6.b [T. Solovyev, S. Esenlik, S. Ikonin, J. Chen (Huawei)]

JVET-N0500 Non-CE4: Restrictions on triangular merge list size [T. Solovyev, S. Esenlik, S. Ikonin, J. Chen (Huawei)] [late]

JVET-N0525 CE4-related: CE4-4.5 pruning reduction in TPM using regular merge candidates [A. Robert, T. Poirier, F. Le Léannec, F. Galpin (Technicolor)] [late]

## CE5 related – Adaptive loop filtering (3)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0243 Non-CE5: Complementary results of tests CE5-3 on Non-Linear ALF [J. Taquet, P. Onno, C. Gisquet, G. Laroche (Canon)]

JVET-N0662 Cross-check of JVET-N0243 (Non-CE5: Complementary results of tests CE5-3 on Non-Linear ALF) [S.-C. Lim, H. Lee, J. Lee, J. Kang (ETRI)] [miss] [late]

JVET-N0416 CE5-related: Unification of picture boundary and line buffer handling for ALF [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-N0545 Crosscheck of JVET-N0416 (CE5-related: Unification of picture boundary and line buffer handling for ALF) [C.-Y. Chen (MediaTek)] [miss] [late]

JVET-N0488 CE5-related: Syntax redundancy removal in adaptive loop filter [X.W. Meng (PKU), X. Zheng (DJI), C.M. Jia, S.S. Wang, S.W. Ma (PKU)]

JVET-N0569 Crosscheck of JVET-N0488 (CE5-related: Syntax redundancy removal in adaptive loop filter) [W. Choi (Samsung)] [miss] [late]

## CE6 related – Transforms and transform signalling (26)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0089 CE6-related: Unified implicit MTS between ISP mode and regular intra mode [M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0565 Crosscheck of JVET-N0089 (CE6-related: Unified implicit MTS between ISP mode and regular intra mode) [Y. Sun, F. Chen, L. Wang (Hikvision)] [miss] [late]

JVET-N0105 CE6-related: RST binarisation [C. Rosewarne, J. Gan (Canon)]

JVET-N0121 Non-CE6: MTS simplification [T. Tsukuba, Y. Yagasaki, T. Suzuki (Sony)]

JVET-N0747 Crosscheck of JVET-N0121 (Non-CE6: MTS simplification) [S. Iwamura (NHK)]	[miss] [late]

JVET-N0122 CE6/CE8-related: Alternative implementation of residual rearrangement for transform skipped blocks [T. Tsukuba, M. Ikeda, T. Suzuki (Sony)] [placeholder] [late]

JVET-N0123 Non-CE6/CE8: Chroma Transform Skip [T. Tsukuba, M. Ikeda, T. Suzuki (Sony)]

JVET-N0534 Crosscheck of JVET-N0123 (AHG13/Non-CE6/CE8: Chroma Transform Skip) [X. Zhao (Tencent)] [miss] [late]

JVET-N0160 CE6-related: Simplification on transform selection for Intra Sub-Partitions [X. Cao, L. Xu, F. Chen, L. Wang (Hikvision)]

JVET-N0535 Crosscheck of JVET-N0160 (Non-CE6: Simplification on transform selection for Intra Sub-Partitions) [X. Zhao (Tencent)] [miss] [late]

JVET-N0161 CE6-related: Implicit transform selection based on intra mode [X. Cao, F. Chen, L. Wang (Hikvision)]

JVET-N0731 Cross-check of JVET-N0161 (Non-CE6: Implicit transform selection based on intra mode) [S. De-Luxán-Hernández (HHI)] [miss] [late]

JVET-N0172 Non-CE6: Unification of Implicit Transform Core Selection [H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)]

JVET-N0190 Non-CE6: Modifications on sub-block transform [J. Jung, D. Kim, G. Ko, J. Son, J. Kwak (WILUS)]

JVET-N0713 Crosscheck of JVET-N0190 (Non-CE6: Modifications on sub-block transform) [X. Zhao (Tencent)] [miss] [late]

JVET-N0208 CE6-6.2: Simplification related to MTS with reduced modes [K. Choi, M. Park, M. W. Park, W. Choi (Samsung)]

JVET-N0338 Non-CE6: Unified transform selection for implicit MTS [T.-C. Ma, X. Xiu, Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-N0362 Non-CE6: Configurable max transform size in VVC [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-N0729 Cross-check of JVET-N0362 (Non-CE6: Configurable max transform size in VVC) [S. De-Luxán-Hernández (HHI)] [miss] [late]

JVET-N0363 CE6-related: Modified encoder decision for transform skip [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-N0667 Cross-check of JVET-N0363 (CE6-related: Modified encoder decision for transform skip) [X. Cao (Hikvision)] [miss] [late]

JVET-N0364 Non-CE6: Further cost reduction for primary transform [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-N0365 CE6-related: MPM based non-separable secondary transform [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-N0518 Crosscheck of JVET-N0365 (CE6-related: MPM based non-separable secondary transform) [P. Philippe (Orange bcom)] [miss] [late]

JVET-N0375 Non-CE6: Simplification on implicit transform selection in ISP mode [S.-C. Lim, J. Kang, J. Lee, H. Lee, H. Y. Kim (ETRI)]

JVET-N0678 Crosscheck of JVET-N0375 (Non-CE6: Simplification on implicit transform selection in ISP mode) [J. Lainema (Nokia)] [miss] [late]

JVET-N0388 Non-CE6: Shape Adaptive Transform Selection for ISP, SBT and MTS [K. Naser, T. Poirier, F. Le Léannec (Technicolor)]

JVET-N0743 Crosscheck of JVET-N0388 (Non-CE6: Shape Adaptive Transform Selection for ISP, SBT and MTS) [P. Philippe (Orange bcom)] [miss] [late]

JVET-N0419 Non-CE6: A Simplification of Implicit MTS [H. E. Egilmez, A. Said, N. Hu, C.-H. Hung, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-N0420 Non-CE6: A Simplification of Implicit Transform Selection in Intra-subblock Partitioning [H. E. Egilmez, A. K. Ramasubramonian, A. Said, N. Hu, V. Seregin, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-N0730 Cross-check of JVET-N0420 (Non-CE6: A Simplification of Implicit Transform Selection in Intra-subblock Partitioning) [S. De-Luxán-Hernández (HHI)] [miss] [late]

JVET-N0424 CE6-related: Simplification of MTS for intra-CUs [C.-H. Hung, H. E. Egilmez, N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-N0519 Crosscheck of JVET-N0424 (CE6-related: An Explicit MTS Design with Fast Encoder) [P. Philippe (Orange bcom)] [miss] [late]

JVET-N0445 Non-CE6: MTS kernel derivation for efficient memory usage [S. Shrestha, A. Kumar, B. Lee (Chosun Univ.), Y. Lee, J. Park (Humax)]

JVET-N0668 Crosscheck of JVET-N0445 (Non-CE6: MTS kernel derivation for efficient memory usage) [J. Kim (??)] [miss] [late]

JVET-N0485 Non-CE6: Efficient separable Multi-Transform-Signalling (MTS) without zero-out [A. Said, H.E. Egilmez, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-N0762 Crosscheck of JVET-N0485 (Non-CE6: Efficient separable Multi-Transform-Selection (MTS) without zero-out) [P. Philippe (Orange bcom)] [miss] [late]

JVET-N0491 CE6-related: Transform Candidate Ordering [C. Hollmann, D. Saffar, J. Ström, P. Wennersten (Ericsson)]

JVET-N0509 CE6-Related: NSST with 8 Coefficients Computation [K. Naser, G. Rath, P. de Lagrange (Technicolor)] [late]

JVET-N0695 Crosscheck of JVET-N0509 (CE6-Related: NSST with 8 Coefficients Computation) [?? (??)] [miss] [late]

JVET-N0555 CE6-related: Simplification of the Reduced Secondary Transform [M. Siekmann, M. Winken, H. Schwarz, D. Marpe, T. Wiegand (HHI)] [late]

## CE7 related – Quantization and coefficient coding (11)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0090 CE7-related: Support of signalling default and user-defined scaling matrices [O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0091 CE7-related: TB-level constraints on context-coded bins for coefficient coding [T.-D. Chuang, S.-T. Hsiang, Z.-Y. Lin, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0638 Crosscheck of JVET-N0091 (CE7-related: TB-level constraints on context-coded bins for coefficient coding) [H. Huang (Qualcomm)] [miss] [late]

JVET-N0092 CE7-related: Unified Rice parameter derivation of abs_remainder and dec_abs_level syntax elements [M. G. Sarwer, O. Chubach, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0732 Cross-check of JVET-N0092 (CE7-related: Unified Rice parameter derivation of abs_remainder and dec_abs_level syntax elements) [A. K. Ramasubramonian (Qualcomm)] [miss] [late]

JVET-N0103 CE7-related: Coefficient group size harmonisation [C. Rosewarne, J. Gan (Canon)]

JVET-N0106 CE7-related: TU level limits on context-coded bins for coefficient coding [M. Coban, M. Karczewicz (Qualcomm)]

JVET-N0765 Crosscheck of JVET-N0106 (CE7-related: TU level limits on context-coded bins for coefficient coding) [C.-Y. Lai (MediaTek)] [miss] [late]

JVET-N0194 CE6/7-related: Context selection of last non-zero coefficient position coding based on reduced TU size (related to JVET-M0297 and JVET-M0251/M0257) [M. Koo, J. Choi, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-N0189 Non-CE7: Unified last position coding for 32-point transforms [Y. Piao, K. Choi (Samsung)]

JVET-N0533 Crosscheck of JVET-N0189 (Non-CE7: Unified last position coding for 32-point transformsNon-CE7: Unified last position coding for 32-point transforms) [X. Zhao (Tencent)] [miss] [late]

JVET-N0281 CE7-related: Constraint on the number of regular bins per subblock [S.-T. Hsiang, T.-D. Chuang, S.-M. Lei (MediaTek)]

JVET-N0592 Cross-check of JVET-N0281 (CE7-related: Constraint on the number of regular bins per subblock) [F. Le Léannec, T. Poirier (Technicolor)] [miss] [late]

JVET-N0282 CE7-related: Joint chroma residual coding with multiple modes [C. Helmrich, C. Rudat, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-N0326 Non-CE7: Simplification of cbf coding [Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-N0652 Crosscheck of JVET-N0326 (Non-CE7: Simplification of cbf coding) [G. Li (Tencent)] [miss] [late]

JVET-N0347 CE7-related: Joint coding of chroma residuals [G. Van der Auwera, A. K. Ramasubramonian, M. Coban, M. Karczewicz (Qualcomm)]

JVET-N0573 Crosscheck of JVET-N0347 (CE7-related: Joint coding of chroma residuals) [Y.-W. Chen (Kwai Inc.)] [miss] [late]

JVET-N0725 Crosscheck of JVET-N0347 (CE7-related: Joint coding of chroma residuals) [N. Choi (Samsung)] [miss] [late]

JVET-N0492 CBF flags signalling in VVC [R. Chernyak, Y. Zhao, S. Ikonin, J. Chen (Huawei)]

JVET-N0685 Crosscheck of JVET-N0492 (CBF flags signalling in VVC) [P. Hanhart (InterDigital)] [miss] [late]

## CE8 related – Screen content coding tools (41)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0093 CE8-related: Disallowing coexistence of regular merge and IBC merge in SMR [Y.-C. Lin, C.-C. Chen, M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0601 Crosscheck of JVET-N0093 (CE8-related: Disallowing coexistence of regular merge and IBC merge in SMR) [Y. Han, W.-J. Chien (Qualcomm)] [miss] [late]

JVET-N0094 CE8-related: Context modelling of transform skip mode [M. G. Sarwer, O. Chubach, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0768 Crosscheck of JVET-N0094 (CE8-related: Context modelling of transform skip mode associated Resources) [B. Wang (Huawei)] [miss] [late]

JVET-N0095 CE8-related: Unified method for coding BVD and MVD [S.-T. Hsiang, S.-M. Lei (MediaTek)]

JVET-N0677 Crosscheck of JVET-N0095 (CE8-related: Unified method for coding BVD and MVD) [Y.-C. Sun (Alibaba)] [miss] [late]

JVET-N0096 CE8-related: A fixed updating order for IBC reference memory [C.-Y. Lai, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0702 Crosscheck of JVET-N0096 (CE8-related: A fixed updating order for IBC reference memory) [T.-S Chang (Alibaba)] [miss] [late]

JVET-N0173 Non-CE8: IBC Reference Area Rearrange [H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)]

JVET-N0611 Cross-check of JVET-N0173 [?? (??)] [miss] [late]

JVET-N0174 Non-CE8: Exclusive Regular Merge and IBC Merge in One Shared Merge List Area [H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)]

JVET-N0640 Crosscheck of JVET-N0174 (Non-CE8: Exclusive regular merge and IBC merge in one shared merge list area) [Y.-C. Lin (MediaTek)] [miss] [late]

JVET-N0175 Non-CE8: IBC Reference Memory for Arbitrary CTU Size [H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)]

JVET-N0540 Crosscheck of JVET-N0175: Non-CE8: IBC Reference Memory for Arbitrary CTU Size [X. Xu (Tencent)] [miss] [late]

JVET-N0176 Non-CE8: IBC Merge List Simplification [H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)]

JVET-N0683 Crosscheck of JVET-N0176 (Non-CE8: IBC Merge List Simplification) [J. Nam, J. Lim, S. Kim (LGE)] [miss] [late]

JVET-N0201 Non-CE8: IBC modifications [J. Li, C.-W. Kuo, C. S.Lim (Panasonic)]

JVET-N0516 Crosscheck of JVET-N0201 (Non-CE8: IBC modifications) [S.-T. Hsiang (MediaTek)] [miss] [late]

JVET-N0202 AHG16/non-CE8: Report on conformance check failures of IBC block vectors [J. Li, C.-W. Kuo, C. S.Lim (Panasonic)] late]

JVET-N0249 Non-CE8: An alternative search area for IBC [J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

JVET-N0250 Non-CE8: Reference memory reduction for intra block copy [J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

JVET-N0684 Crosscheck of JVET-N0250 (Non-CE8: Reference memory reduction for intra block copy) [J. Nam, J. Lim, S. Kim (LGE)] [miss] [late]

JVET-N0251 Non-CE8: Intra block copy clean-up [J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

JVET-N0541 Crosscheck of JVET-N0251: Non-CE8: Intra block copy clean-up [X. Xu (Tencent)] [miss] [late]

JVET-N0258 CE8-related: Palette Mode Coding [W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu, Y. Wang (Bytedance)]

JVET-N0701 Crosscheck of JVET-N0258 (CE8-related: Palette Mode Coding) [T.-S. Chang (Alibaba)] [miss] [late]

JVET-N0259 CE8-related: compound palette mode [W. Zhu, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

JVET-N0260 Non-CE8: Disabling fractional MVD search in DMVR for SCC [W. Zhu, H. Liu, K. Zhang, L. Zhang, J. Xu, Y. Wang (Bytedance)]

JVET-N0739 Crosscheck of JVET-N0260 (Non-CE8: Disabling fractional MVD search in DMVR for SCC) [S. Ye (Hikvision)] [miss] [late]

JVET-N0289 CE8-Related: On MVD Coding [M. Salehifar, S. Paluri, S. Kim (LGE)]

JVET-N0722 Crosscheck of JVET-N0289 (CE8-Related: On MVD Coding) [Y. Han, W.-J. Chien (Qualcomm)] [miss] [late]

JVET-N0316 CE8-related: Default Processing for IBC Mode [Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-N0558 Crosscheck of JVET-N0316 (CE8-related: Default Processing for IBC Mode) [T. Zhou (Sharp)] [miss] [late]

JVET-N0317 CE8-related: Simplification on IBC Merge/Skip Mode [Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-N0593 Cross-check of JVET-N0317 (CE8-related: Simplification on IBC Merge/Skip Mode) [F. Le Léannec, A. Robert (Technicolor)] [miss] [late]

JVET-N0318 CE8-related: Block Size Limitation for IBC Mode [Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-N0642 Crosscheck of JVET-N0318 (CE8-related: block size limitation for IBC mode) [Y.-C. Lin (MediaTek)] [miss] [late]

JVET-N0329 CE8-related: Encoder improvements on IBC search [X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-N0543 Crosscheck of JVET-N0329: CE8-related: Encoder improvements on IBC search [X. Xu (Tencent)] [miss] [late]

JVET-N0337 CE8-related: Adaptive disabling of intra sample interpolation for screen content coding [X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-N0661 Crosscheck of JVET-N0337 (CE8-related: Adaptive disabling of intra sample interpolation for screen content coding) [H. Yang (InterDigital)] [miss] [late]

JVET-N0346 Non-CE8: Palette Mode in HEVC for YUV4:4:4 format [Y.-H. Chao, H. Wang, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-N0690 Crosscheck of JVET-N0346 (Non-CE8: Palette Mode in HEVC for YUV4:4:4 format) [C.-Y. Lai (MediaTek)] [miss] [late]

JVET-N0357 CE8-related: Context Modelling of Sign for TS Residual Coding [B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-N0366 CE8-related: Modified limitation on context coded bins for CE8-3.1a and CE8-5.1a [X. Zhao, X. Li, X. Xu, S. Liu (Tencent)] [placeholder] [late]

JVET-N0382 CE8-related: unified IBC block vector prediction [X. Xu, X. Li, S. Liu (Tencent)]

JVET-N0520 Crosscheck of JVET-N0382 (CE8-related: unified IBC block vector prediction) [C.-C. Chen (MediaTek)] [miss] [late]

JVET-N0383 Non-CE8: IBC search range adjustment for implementation consideration [X. Xu, X. Li, S. Liu (Tencent)]

JVET-N0610 Cross-check of JVET-N0383 [?? (??)] [miss] [late]

JVET-N0384 Non-CE8: IBC search range increase for small CTU sizes [X. Xu, X. Li, S. Liu (Tencent)]

JVET-N0405 CE8-related: Palette Mode Simplification [Y.-C. Sun, T.-S. Chang, J. Lou (Alibaba), Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-N0608 Crosscheck of JVET-N0405 (CE8-related: Palette Mode Simplification) [W. Zhu (Bytedance)] [miss] [late]

JVET-N0413 CE8-related: Quantized residual BDPCM [M. Karczewicz, M. Coban (Qualcomm)]

JVET-N0703 Crosscheck of JVET-N0413 (CE8-related: Quantized residual BDPCM) [T.-S Chang (Alibaba)] [miss] [late]

JVET-N0430 CE8-related: Transform skip restriction [J. Choi, J. Heo, S. Yoo, J. Choi, J. Lim, S. Kim (LGE)]

JVET-N0455 CE8-related: Sign context modelling and level mapping for TS residual coding [M. Karczewicz, M. Coban (Qualcomm)]

JVET-N0594 Cross-chreck of JVET-N0455 (CE8-related: Sign context modelling and level mapping for TS residual coding) [F. Le Léannec, T. Poirier (Technicolor)] [miss] [late]

JVET-N0609 Crosscheck of JVET-N0455: CE8-related: Sign context modelling and level mapping for TS residual coding [T. Nguyen (HHI)] [miss] [late]

JVET-N0459 CE8-related: Modified block vector coding for IBC [J. Nam, H. Jang, J. Choi, J. Heo, J. Lim, S. Kim (LGE)]

JVET-N0460 CE8-related: Default candidates for IBC merge mode [J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

JVET-N0461 CE8-related: Signaling on maximum number of candidates for IBC merge mode [J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

JVET-N0630 Crosscheck of JVET-N0461: CE8-related: Signaling on maximum number of candidates for IBC merge mode [L. Pham Van (Qualcomm)] [miss] [late]

JVET-N0464 Non-CE8: MMVD Motion vector rounding for SCC [H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)]

JVET-N0576 Crosscheck of JVET-N0464: Non-CE8: MMVD Motion vector rounding for SCC [X. Xu (Tencent)] [miss] [late]

JVET-N0466 Non-CE8: The corner case handling regarding mv derivation for Chroma IBC in dual tree structure [H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)]

JVET-N0603 Crosscheck of JVET-N0466 (Non-CE8: The corner case handling regarding mv derivation for Chroma IBC in dual tree structure) [Y. Han, W.-J. Chien (Qualcomm)] [miss] [late]

JVET-N0467 Non-CE8: Experimental result for various size of IBC block with optimized syntax signaling [H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)]

JVET-N0604 Crosscheck of JVET-N0467 (Non-CE8 : Experimental result for various size of IBC block with optimized syntax signaling) [Y. Han, W.-J. Chien (Qualcomm)] [miss] [late]

JVET-N0472 Non-CE8: On IBC reference buffer design [J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

JVET-N0550 CE8-related: Subblock based Palette Mode [Y.-H. Chao, T. Hsieh, M. Karczewicz (Qualcomm)] [late]

JVET-N0580 Cross-check of JVET-N0550: CE8-related: Line-based CG Palette Mode [X. Xiu (Kwai Inc.)] [miss] [late]

JVET-N0742 CE8-related: Joint test of JVET-N0094 and CE8-4.1 [M. G. Sarwer, O. Chubach, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim (LGE)] [late]

JVET-N0763 CE8-related: Joint proposal for transform skip (CE8-4.1c, JVET-M0279 and JVET-N0094) [S. Yoo, J. Choi, J. Heo, J. Choi, J. Lim, S. Kim (LGE), M. G. Sarwer, O. Chubach, C. -W. Hsu, Y. -W. Huang, S.-M. Lei (MediaTek)] [late]

## CE9 related – Decoder motion vector derivation (34)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0097 CE9-related: Simplification of cascading DMVR and BDOF processes [C.-Y. Chen, T.-D. Chuang, C.-C. Chen, C.-Y. Lai, Z.-Y. Lin, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0571 Crosscheck of JVET-N0097 (CE9-related: Simplification of cascading DMVR and BDOF processes) [Y.-W. Chen (Kwai Inc.)] [miss] [late]

JVET-N0145 Non-CE9: Harmonization of DMVR and MMVD [E. Sasaki, T. Hashimoto, T. Chujoh, T. Ikai (Sharp)]

JVET-N0759 Cross check of Non-CE9: Harmonization of DMVR and MMVD (JVET-N0145) [S. Jeong, K. Choi (Samsung)] [late]

JVET-N0146 Non-CE9: On conditions for DMVR and BDOF [T. Chujoh, T. Ikai (Sharp)]

JVET-N0522 Crosscheck of JVET-N0146 [P. Bordes (Technicolor)] [miss] [late]

JVET-N0147 CE9-related: Simplification of BDOF [T. Chujoh, T. Ikai (Sharp)]

JVET-N0673 Crosscheck of JVET-N0147 (CE9-related: Simplification of BDOF) [R. Yu (Ericsson)] [miss] [late]

JVET-N0148 CE9-related: Early termination for BDOF [T. Chujoh, T. Ikai (Sharp)]

JVET-N0549 Crosscheck of JVET-N0148 (CE9-related: Early termination for BDOF) [K. Kondo, M. Ikeda (Sony)] [late]

JVET-N0152 Non-CE9: On motion refinement parameter derivation in BDOF [R. Yu, D. Liu (Ericsson)]

JVET-N0654 Crosscheck of JVET-N0152 (Non-CE9: On motion refinement parameter derivation in BDOF) [H. Liu (Bytedance)] [miss] [late]

JVET-N0153 Non-CE9: On DMVR and GBI [R. Yu, D. Liu (Ericsson)]

JVET-N0566 Crosscheck of JVET-N0153 (non-CE9: on DMVR and GBI) [H. Lee, S. -C. Lim, J. Lee, J. Kang (ETRI)] [miss] [late]

JVET-N0158 CE9-related: Alternative method of SAD based early termination for BDOF [K. Unno, K. Kawamura, S. Naito (KDDI)]

JVET-N0551 Crosscheck of JVET-N0158 (CE9-related: Alternative method of SAD based early termination for BDOF) [T. Chujoh, T. Ikai (Sharp)] [miss] [late]

JVET-N0162 CE9-related: Modified enabling condition for DMVR [F. Chen, L. Wang (Hikvision)]

JVET-N0692 Crosscheck of JVET-N0162 (Non-CE9: Modified enabling condition for DMVR and BDOF) [S. Esenlik (Huawei)] [miss] [late]

JVET-N0163 CE9-related: Simplified refinement process for DMVR [F. Chen, L. Wang (Hikvision)]

JVET-N0179 Non-CE9: Reducing bi-linear interpolation buffer requirement for DMVR [S. Esenlik, H. Gao, A. M. Kotra, B. Wang, J. Chen (Huawei)]

JVET-N0734 Crosscheck of JVET-N0179 (Non-CE9: Reducing bi-linear interpolation buffer requirement for DMVR) [C.-C. Chen, W.-J. Chien (Qualcomm)] [miss] [late]

JVET-N0209 CE9-related: Simplification of BDOF based on CE9-2.5 [Y. Kato, K. Abe, T. Toma (Panasonic)]

JVET-N0239 CE9-related: BDOF-BWA unification [F. Galpin, T. Poirier, P. Bordes (Technicolor)]

JVET-N0658 Crosscheck of JVET-N0239: CE9-related: BDOF-BWA unification [J. Luo (InterDigital)] [miss] [late]

JVET-N0262 CE9-related: Disable DMVR if BPWA is not using default weight [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-N0514 Crosscheck of JVET-N0262 (CE9-related: Disabling DMVR for non equal weight BPWA) [Y.-L. Hsiao (MediaTek)] [late]

JVET-N0292 CE9-related: Reference sample access constraints at VPDU level for DMVR [S. Sethuraman (Ittiam)]

JVET-N0294 CE9-related: Adaptive search pattern for DMVR [S. Sethuraman (Ittiam)]

JVET-N0735 Crosscheck of JVET-N0294 (CE9-related: Adaptive search pattern for DMVR) [C.-C. Chen, W.-J. Chien (Qualcomm)] [miss] [late]

JVET-N0295 CE9-related: Using extended samples during DMVR SAD cost evaluations [S. Sethuraman (Ittiam)]

JVET-N0568 Crosscheck of JVET-N0295 (CE9-related: Using extended samples during DMVR SAD cost evaluations) [H. Lee, S. -C. Lim, J. Lee, J. Kang (ETRI)] [miss] [late]

JVET-N0296 CE9-related: DMVR costs based early termination for BDOF in CE9-2.6 [S. Sethuraman (Ittiam)]

JVET-N0708 Crosscheck of JVET-N0296 (CE9-related: DMVR costs based early termination for BDOF in CE9-2.6) [Y. Kato, K. Abe, T. Toma (Panasonic)] [miss] [late]

JVET-N0312 CE9-related: A SIMD-friendly simplification for BDOF [S. Sethuraman (Ittiam)]

JVET-N0711 Crosscheck of JVET-N0312 (CE9-related: A SIMD-friendly simplification for BDOF) [Y. Kato, K. Abe, T. Toma (Panasonic)] [miss] [late]

JVET-N0314 CE9-related: Results for use of DMVR refined MVs from top and top-left CTUs for spatial MV prediction and CU boundary de-blocking [S. Sethuraman (Ittiam)]

JVET-N0325 CE9-related: Improvements on bi-directional optical flow (BDOF) [X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-N0328 Non-CE9: Mutually exclusive DMVR/BDOF at CU level [Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-N0544 Crosscheck of JVET-N0328 (Non-CE9: Mutually exclusive DMVR/BDOF at CU level) [C.-Y. Chen (MediaTek)] [miss] [late]

JVET-N0374 Non-CE9: On early termination for DMVR [H. Lee, S.-C. Lim, J. Lee, J. Kang, H. Y. Kim (ETRI)]

JVET-N0740 Non-CE9: Cross-check of JVET-N0374 (Non-CE9: On early termination for DMVR) [S. Sethuraman (Ittiam)] [miss] [late]

JVET-N0408 Non-CE9: Early Termination Techniques for DMVR [C.-C. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-N0409 CE9-related: Horizontal and Vertical Boundary Padding Process of DMVR [C.-C. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-N0691 Crosscheck of JVET-N0409 (CE9-related: Horizontal and Vertical Boundary Padding Process of DMVR) [S. Esenlik (Huawei)] [miss] [late]

JVET-N0440 CE9-related: Disabling DMVR and BDOF when underlying assumptions are false [H. Liu, L. Zhang, K. Zhang, J. Xu, W. Zhu, Y. Wang (Bytedance)]

JVET-N0675 Crosscheck of JVET-N0440 (CE9-related: Disabling DMVR and BDOF when underlying assumptions are false) [R. Yu (Ericsson)] [miss] [late]

JVET-N0442 Non-CE9: Conditions fix for DMVR and BDOF [N. Park, H. Jang, J. Nam, J. Lim, S. Kim (LGE)]

JVET-N0553 Crosscheck of JVET-N0442 (Non-CE9: Conditions fix for DMVR and BDOF) [T. Chujoh, T. Ikai (Sharp)] [miss] [late]

JVET-N0443 Non-CE9: BDOF processing considering assumption for the optical flow [N. Park, H. Jang, J. Nam, J. Lim, S. Kim (LGE)]

JVET-N0561 Crosscheck of JVET-N0443 (Non-CE9: BDOF processing considering assumption for the optical flow) [E. Sasaki (Sharp)] [miss] [late]

JVET-N0589 Crosscheck of JVET-N0443 (Non-CE9: BDOF processing considering assumption for the optical flow) [?? (??)] [miss] [late]

JVET-N0444 Non-CE9: Mismatch beween test specification and reference software on BDOF and DMVR [N. Park, H. Jang, J. Nam, J. Lim, S. Kim (LGE)]

JVET-N0484 Non-CE9: Simplification of DMVR and BDOF combination [R.-L. Liao, Y. Ye, J. Chen (Alibaba)]

JVET-N0574 Crosscheck of JVET-N0484 (Non-CE9: Simplification of DMVR and BDOF combination) [Y.-W. Chen (Kwai Inc.)] [miss] [late]

JVET-N0505 Non-CE9: Simplification of parametric motion vector refinement in DMVR [E. Sasaki, T. Zhou, T. Ikai (Sharp)] [late]

JVET-N0748 Crosscheck of JVET-N0505 (Non-CE9: Simplification of parametric motion vector refinement in DMVR) [S. Iwamura (NHK)]	[miss] [late]

JVET-N0507 CE9-related: Early termination of BDOF with DMVR cost [K. Kondo, M. Ikeda (Sony)] [late]

JVET-N0554 Crosscheck of JVET-N0507 (CE9-related: Early termination of BDOF with DMVR cost) [T. Chujoh, T. Ikai (Sharp)] [miss] [late]

JVET-N0548 CE9-related: Additional experimental results of JVET-N0187 (CE9-2.2) [K. Kondo, M. Ikeda (Sony)] [late]

JVET-N0757 CE9-related: combination of PROF for affine and BDOF-BWA [F. Galpin, T. Poirier (Technicolor), J. Luo, Y. He (InterDigital)] [late]

## CE10 related – Combined intra/inter prediction (3)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0327 CE10-related: Simplification on combined inter and intra prediction (CIIP) [X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-N0537 Crosscheck of JVET-N0327 (CE10-related: Simplification on combined inter and intra prediction (CIIP)) [L. Zhao (Tencent)] [miss] [late]

JVET-N0395 CE10-related: Simplification and Improvement of combined intra-inter prediction mode [L. Zhao, X. Li, X. Zhao, S. Liu (Tencent)]

JVET-N0579 Cross-check of JVET-N0395: CE10-related: Simplification and Improvement of combined intra-inter prediction mode [X. Xiu (Kwai Inc.)] [miss] [late]

JVET-N0508 CE10-related: A combination of JVET-N0302 (CE10.1.1) and JVET-N0327 [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm), X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)] [miss] [late]

## CE11 related – Deblocking (3)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0182 CE11-related: Simplification of sub-pu deblocking and longer tap filter [A. M. Kotra, S. Esenlik, J. Chen, B. Wang, H. Gao (Huawei)]

JVET-N0619 Crosscheck of JVET-N0182 (CE11-related: Simplification of sub-pu deblocking and longer tap filter) [C.-M. Tsai (MediaTek)] [miss] [late]

JVET-N0359 CE11-related: On MV threshold for deblocking [K. Andersson, J. Enhorn (Ericsson)]

JVET-N0473 Non-CE11: On ISP transform boundary deblocking [K. Misra, A. Segall (Sharp Labs of America), M. Ikeda, T. Suzuki (Sony)]

## CE12 related – Tile set boundary motion compensation handling (3)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0355 CE12-related: Normative temporally independent tile group coding [R. Skupin, Y. Sanchez, V. George, K. Sühring, T. Schierl (HHI)]

JVET-N0402 CE12-related: MCTS improvement by modifying motion compensation filter coefficients [A. Aminlou, A. Zare, M. M. Hannuksela (Nokia)]

JVET-N0403 CE12-related: MCTS improvement by modifying prediction block [A. Aminlou, A. Zare, M. M. Hannuksela (Nokia)]

## CE13 related – Neural-network based loop filtering (1)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0133 CE13-related: In-loop filter with only CNN-based filter [S. Wan, M.-Z.Wang, H. Gong, C.-Y. Zou (NPU), Y.-Z. Ma, J.-Y. Huo (Xidian Univ.), Y.-F. Yu, Y. Liu (OPPO)]

JVET-N0712 CE13-related: Adaptive CNN based in-loop filtering with boundary weights [K. Kawamura, Y. Kidani, S. Naito (KDDI)] [miss] [late]

## Quantization (3)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0204 AHG18: Support of quantization matrices [T. Toma, K. Abe (Panasonic), S.-C. Lim, J. Kang (ETRI)]

JVET-N0631 Crosscheck of JVET-N0204 (AHG18: Support of quantization matrices) [H. Sakurai, M.Ikeda (Sony)] [miss] [late]

JVET-N0221 Chroma Quantization Parameter Qpc Table for HDR Signal [T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen (Dolby)]

JVET-N0246 Modified dequantization scaling [K. Sharman, S. Keating (Sony)]

## Entropy coding (9)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0112 Simplification of context modeling for coding CU split decisions [S.-T. Hsiang, S.-M. Lei (MediaTek)]

JVET-N0523 Crosscheck of JVET-N0112 [P. Bordes, Y. Chen (Technicolor)] [miss] [late]

JVET-N0689 Cross-check of JVET-N0112 (test 2 and 3) [J. Li, C.-W. Kuo, C. S.Lim (Panasonic)] [miss] [late]

JVET-N0207 AHG14: CABAC skip mode [K. Abe, T. Toma (Panasonic)]

JVET-N0666 Crosscheck of JVET-N0207 (AHG14: CABAC skip mode) [C. Rosewarne (Canon)] [late]

JVET-N0301 Simplification of the initialization process for context variables [H. Kirchhoffer, J. Stegemann, D. Marpe, H. Schwarz, T. Wiegand (HHI)]

JVET-N0697 Crosscheck of JVET-N0301 (Simplification of the initialization process for context variables) [J. Dong (Qualcomm)] [miss] [late]

JVET-N0746 Crosscheck of JVET-N0301 and JVET-N0425 (CABAC initialization) [F. Bossen (Sharp)] [miss] [late]

JVET-N0311 Context Modeling Simplification and Reduction in VVC [Y. Chen, F. Le Léannec, T. Poirier (Technicolor)]

JVET-N0381 Native CABAC initialization [F. Bossen (Sharp)]

JVET-N0698 Crosscheck of JVET-N0381 (Native CABAC initialization) [J. Dong (Qualcomm)] [miss] [late]

JVET-N0714 Crosscheck of JVET-N0381 (Native CABAC initialization) [H. Kirchhoffer (HHI)] [miss] [late]

JVET-N0425 Simplification of CABAC initialization process [J. Dong, A. Said, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-N0715 Crosscheck of JVET-N0425 (Simplification of CABAC initialization process) [H. Kirchhoffer (HHI)] [miss] [late]

JVET-N0600 Context reduction for inter and split syntax elements [Y. Chen, F. Le Léannec, T. Poirier (Technicolor)] [miss] [late]

JVET-N0696 Availability based context modelling for mtt_split_cu_vertical_flag [A. Wieckowski, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)] [late]

## Luma mapping with chroma scaling (9)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0113 AHG16: Subblock-based chroma residual scaling [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)] [late]

JVET-N0755 Crosscheck of JVET-N0113 (AHG16: Subblock-based chroma residual scaling) [J. Lee, S.-C. Lim, H. Lee, J. Kang (ETRI)] [miss] [late]

JVET-N0220 Simplification of Reshaper Implementation [T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen (Dolby)]

JVET-N0526 Crosscheck of JVET-N0220 (Simplification of Reshaper Implementation) [C. Chevance (Technicolor)] [miss] [late]

JVET-N0274 Non-CE: Cleanups on in-loop reshaping [K. Zhang (Bytedance)]

JVET-N0612 Crosscheck of JVET-N0274 (Non-CE: Cleanups on in-loop reshaping) [T. Lu (Dolby)] [miss] [late]

JVET-N0299 On Luma Dependent Chroma Residual Scaling of In-loop Reshaper [J. Zhao, S. Kim (LGE)]

JVET-N0649 Crosscheck of JVET-N0299 (On Luma Dependent Chroma Residual Scaling of In-loop Reshaper) [G. Li (Tencent)] [miss] [late]

JVET-N0300 On High Bit Depth Signaling of In-loop Reshaper [J. Zhao, S. Kim (LGE)]

JVET-N0613 Crosscheck of JVET-N0300 (On High Bit Depth Signaling of In-loop Reshaper) [T. Lu (Dolby)] [miss] [late]

JVET-N0389 Chroma residual scaling with separate luma/chroma tree [E. François, C. Chevance (Technicolor)]

JVET-N0614 Crosscheck of JVET-N0389 (Chroma residual scaling with separate luma/chroma tree) [T. Lu (Dolby)] [miss] [late]

JVET-N0417 Simplified luma dependant chroma residual scaling of in-loop reshaper [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-N0664 Cross-check of JVET-N0417 (Simplified luma dependant chroma residual scaling of in-loop reshaper) [S.-C. Lim, H. Lee, J. Lee, J. Kang (ETRI)] [miss] [late]

JVET-N0477 On luma mapping with chroma scaling [Y. Ye, J. Chen, R. Liao (Alibaba)]

JVET-N0615 Crosscheck of JVET-N0477 (On luma mapping with chroma scaling) [T. Lu (Dolby)	[miss] [late]

## Other coding tools (2)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0368 An implementation of adaptive color transform in VVC [X. Zhao, X. Xu, X. Li, S. Liu (Tencent)]

JVET-N0421 Non-CE: Unsymmetrical quadtree partitioning [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang (Bytedance)]

## High-level syntax (78)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

### Interfacing with MPEG Systems (1)

JVET-N0041 Summary of the status of systems-related issues for VVC [Y. Lim (Samsung), E. Thomas (TNO)]

### General high-level syntax (31)

#### NAL unit header and genearl apsects of parameter sets (6)

JVET-N0050 AHG17: On Forbidden-Zero Bit in NAL Unit Header [S. Wenger, B. Choi, S. Liu (Tencent)]

JVET-N0051 AHG17: On NAL Unit header design for VVC [S. Wenger, B. Choi, S. Liu (Tencent)]

JVET-N0067 AHG17: On the first byte of the NAL unit header [M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia)] [late]

JVET-N0278 AHG17: On VVC HLS relevant to MPEG requirements on immersive media delivery and access (N18134) [J. Boyce (Intel)] [late]

JVET-N0511 AHG17: Signalling leading picture information in the NAL unit header structure [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)] [late]

JVET-N0349 AHG17: On Parameter Set Design for VVC [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

#### Reference picture management (4)

JVET-N0058 AHG17: On decoded picture buffer management [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-N0100 AHG17: On Reference Picture List Signalling [S. Deshpande (Sharp)]

JVET-N0135 AHG17: On reference picture list [Y. Fujimoto, T. Suzuki (Sony)] [late]

JVET-N0136 AHG17: On non-referenced picture [Y. Fujimoto, T. Suzuki (Sony)] [late]

#### APS and tile group header (8)

JVET-N0065 AHG17 & AHG9: Comments on carriage of coding tool parameters in Adaptation Parameter Set [M. Li, P. Wu (ZTE)]

JVET-N0069 AHG16/AHG17: Proposed Cleanup for Reshaper High Level Syntax [B. Heng, M. Zhou, W. Wan (Broadcom)]

JVET-N0117 AHG17: Signalling of reshaper parameters in APS [Y.-K. Wang, Hendry, J. Chen (Huawei), P. Yin, T. Lu, F. Pu, S. McCarthy (Dolby)]

JVET-N0138 AHG17: Carriage of reshaper model parameters [W. Choi, K. Choi, K. Choi (Samsung)]

JVET-N0284 AHG17: On ALF and Reshaper Signaling [S.Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

JVET-N0290 AHG17: Conditional Signaling of ALF and In-Loop Reshaper Model [S.Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

JVET-N0293 AHG17: Reshaper Reset Signaling [S. Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

JVET-N0297 AHG17: Reshaper Model Signaling for Intra Coded Tile Group [S. Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

#### Interoperability and capability points definition and signalling (1)

JVET-N0276 AHG15: On interoperability point signalling [J. Boyce (Intel)]

#### High efficiency random access (4)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0119 AHG17: EDR - external decoding refresh [Y.-K. Wang (Huawei)] [late]

JVET-N0244 AHG17: Video coding based on cross RAP referencing (CRR) [H. Yu, L. Yu (Zhejiang Univ.)] [miss] [late]

JVET-N0494 AHG17: Dependent random access point pictures in VVC [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

JVET-N0072 AHG17: New NAL unit types for VVC [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

The DRAP part of this document belongs to this agenda item.

#### Miscellaneous general HLS topics (8)

JVET-N0061 AHG19: Signalling temporal IDs and levels for HFR backwards compatible bitstreams [V. Drugeon (Panasonic)]

JVET-N0063 AHG17: Separate essential display information from non-essential display information in the Video Usability Information [V. Drugeon (Panasonic)]

JVET-N0120 AHG17: Misc. HLS clean-ups [Y.-K. Wang (Huawei)] [late]

JVET-N0227 Maximum transform size signaling in HLS [L. Li, M. Koo, J. Nam, J. Heo, J. Lim (LGE)] [miss] [late]

JVET-N0234 [AHG12/AHG17] Signaling of virtual boundaries [A. DSouza, C. Pujara, R. Gadde, W. Choi, K. Choi, K.P. Choi (Samsung)] [late]

JVET-N0438 AHG12: Loop filter disabled across virtual boundaries [S.-Y. Lin, L. Liu, J.-L. Lin, Y.-C. Chang, C.-C. Ju (MediaTek), P. Hanhart, Y. He (InterDigital)]

JVET-N0744 Crosscheck of JVET-N0438 (AHG12: Loop filter disabled across virtual boundaries) [A. Konda, C. Pujara (Samsung)] [miss] [late]

JVET-N0288 AHG17: Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

JVET-N0352 AHG17: Conformance Window [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

### Coded picture regions (30)

#### Tiles and tile groups (11)

JVET-N0056 AHG12: On tile design [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-N0057 AHG12: On tile group design [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-N0064 AHG17: Signalling of only one tile group per picture [V. Drugeon (Panasonic)]

JVET-N0066 AHG12: On top-to-bottom tile partitioning [Y. He, A. Hamza (InterDigital)]

JVET-N0071 AHG12: Unified signalling of tile grouping information [L. Chen, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0111 AHG12: Flexible tiling [Y.-K. Wang, Hendry, M. Sychev (Huawei)]

JVET-N0124 AHG12: On Tile Group Signalling [S. Deshpande (Sharp)]

JVET-N0348 AHG12: Hierarchical tiling for VVC [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

JVET-N0496 AHG12: Rectangular tile group address signaling [R. Sjöberg, M. Damghanian, M. Pettersson (Ericsson)]

JVET-N0497 AHG12: Dependent tiles [R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)]

JVET-N0498 AHG12: On Uniform Tile Partitioning [R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)]

#### Independently coded picture regions (16)

JVET-N0042 AHG12: Comments on MPEG draft requirements on immersive media access and delivery [M. M. Hannuksela, E. B. Aksu, A. Hourunranta (Nokia)]

JVET-N0043 AHG12: VVC design goals related to MPEG requirements on immersive media access and delivery [M. M. Hannuksela, E. B. Aksu (Nokia)]

JVET-N0044 AHG12: Comparison of approaches for independently coded picture regions [M. M. Hannuksela, A. Aminlou, K. Kammachi-Sreedhar (Nokia)]

JVET-N0045 AHG12: Sub-picture layers for realizing independently coded picture regions [M. M. Hannuksela, K. Kammachi-Sreedhar, A. Aminlou (Nokia)]

JVET-N0046 AHG12: Sub-picture-based picture partitioning and decoding [M. M. Hannuksela, A. Aminlou (Nokia)]

JVET-N0055 AHG12: On sub-picture design [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-N0073 AHG17: [SYS-VVC] Signalling subpicture coded video sequence [L. Chen, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-N0099 AHG12/AHG17 On sub-picture parameter set [Y. He, A. Hazma (InterDigital)]

JVET-N0107 Sub-picture based video coding [Y.-K. Wang, Hendry, J. Chen (Huawei)]

JVET-N0191 [AHG17/AHG12] On APS id for bitstream merging for VVC [N. Ouedraogo, E. Nassor, J. Taquet, G. Kergourlay, F. Maze (Canon)]

JVET-N0192 [AHG17/AHG12] Bitstream extraction and merging with variable initial Qp [N. Ouedraogo, E. Nassor, G. Kergourlay, F. Maze (Canon)]

JVET-N0354 AHG12: Conformance of temporally independent tile groups [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

JVET-N0411 AHG12: Layout signalling of independent coded regions [E. Thomas (TNO)]

JVET-N0528 AHG12/AHG19/SYS-VVC: Sub-picture decoding design: a comparison of proposed solutions [E. Thomas (TNO)] [late]

JVET-N0047 AHG12/AHG17: Merging IRAP and non-IRAP VCL NAL units into the same coded picture [M. M. Hannuksela (Nokia)]

JVET-N0108 AHG12/AHG17: Allowing mixed IRAP and non-IRAP NAL unit types within a picture [Y.-K. Wang, Hendry (Huawei)]

JVET-N0753 AHG12: Design decision questions on independently coded picture regions [M. M. Hannuksela (Nokia), S. Deshpande (Sharp)] [late]

#### Wavefront parallel processing (3)

JVET-N0060 AHG12: Improved parallel processing capability with WPP [Y. Fujimoto, M. Ikeda, T. Suzuki (Sony)]

JVET-N0559 Crosscheck of JVET-N0060 (AHG12: Improved parallel processing capability with WPP) [T. Ikai (Sharp)] [late]

JVET-N0149 AHG12: Signalling on wavefront parallel processing [T. Ikai, S. Deshpande, T. Chujoh, E. Sasaki, T. Aono (Sharp)] [late]

JVET-N0150 AHG12: One CTU delay wavefront parallel processing [T. Ikai (Sharp)]

JVET-N0646 Crosscheck of JVET-N0150 (AHG12: One CTU delay wavefront parallel processing) [M. Ikeda (Sony)] [late]

### Low latency random access (10)

JVET-N0072 AHG17: New NAL unit types for VVC [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

The GDR part of this document belongs to this agenda item.

JVET-N0080 AHG14: Revised software for ultra low-latency encoding [K. Kazui (Fujitsu)]

JVET-N0101 AHG14/ AHG17: On Gradual and Clean Random Access [S. Deshpande (Sharp)]

JVET-N0114 AHG14: A delay analysis for IRAP and GDR [Hendry, Y.-K. Wang, M. Sychev (Huawei)]

JVET-N0115 AHG14/AHG17: GDR - gradual decoding refresh [Hendry, Y.-K. Wang, J. Chen, S. Hong (Huawei)]

JVET-N0116 AHG14/AHG17: DDR - distributed decoding refresh [Y.-K. Wang, M. Sychev, Hendry (Huawei)]

JVET-N0275 AHG14/AHG8/AHG17: Sections for intra refresh and inloop filter disabling [J. Boyce, R. Lei, L. Xu (Intel)]

JVET-N0310 AHG14: On gradual decoding refresh [M. M. Hannuksela, K. Kammachi-Sreedhar, A. Aminlou (Nokia)] [late]

JVET-N0391 AHG14: Software updates for intra refresh [J.-M. Thiesse, D. Gommelet, D. Nicholson (VITEC)] [late]

JVET-N0495 AHG14: Recovery point indication NAL unit [R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)]

### HRD (7)

JVET-N0062 AHG19: Signalling HRD models for HFR backwards compatible bitstreams [V. Drugeon (Panasonic)]

JVET-N0077 Hypothetical Reference Decoder in VVC [K. Kazui (Fujitsu)]

JVET-N0350 AHG17: HRD starting point [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

JVET-N0351 AHG17: HRD parameters for temporal scalability [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

JVET-N0353 AHG17: Parsing of HRD related SEI messages [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

JVET-N0423 AHG17: Hypothetical Reference Decoder [S. Deshpande (Sharp)]

JVET-N0706 AHG17: Decoded Picture Hash SEI [K. Sühring, R. Skupin, Y. Sanchez (HHI)] [late]

# Complexity analysis and reduction (2)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0049 AHG16: A study of bin to bit ratio in VTM4.0 and HM16.19 [M. Zhou (Broadcom)]

JVET-N0397 AHG16: Memory bandwidth reduction for small CUs [X. Li, G. Li, X. Xu, S. Liu (Tencent)]

JVET-N0564 Crosscheck of JVET-N0397 (AHG16: Memory bandwidth reduction for small CUs) [Y. He (InterDigital)] [miss] [late]

# Encoder optimization (3)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0238 AHG13: ISP Tool-off Tests in VTM-4.0.1 [S. De-Luxán-Hernández, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-N0247 Non-CE: An improvement to hash-based motion estimation [J. Li, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

JVET-N0741 AHG10: Enable Rate Control with In-Loop Reshaping [Z. Liu, Z. Chen, Y. Li (Wuhan Univ.), X. Li, S. Liu (Tencent)] [late]

# Metrics and evaluation criteria (X)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

# Withdrawn (6)

JVET-N0156 Withdrawn

JVET-N0231 Withdrawn

JVET-N0245 Withdrawn

JVET-N0315 Withdrawn

JVET-N0431 Withdrawn

JVET-N0501 Withdrawn

# Plenary meetings, joint meetings, BoG reports, and summary of actions taken

## Plenary meeting XXday XX March XXXX-

Reports of the tracks were presented as follows:

Track A:

Track B:

## Joint meeting XXday XX March XXXX-

JVET with … .

## Closing plenary sessions

## BoGs (X)

JVET-N0724 BoG report on general high level syntax [J. Boyce]

JVET-N0754 BoG Report on Coded Picture Regions [S. Deshpande (Sharp)]

## List of actions taken affecting Draft 3 of VVC, VTM 3, and 360Lib

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the VVC draft text, VTM or 360Lib description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part.

# Project planning

## Core experiment planning

…

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- VTM algorithm description text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be XXday XX June 2019.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized during the opening plenary on January 9 that those rules which had been set up or refined during the 12th meeting should be observed. In particular, for some CEs, results were available late, and some changes in the experimental setup (particularly in CE4) were not discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the draft standard by the next meeting.

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure tha the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials; the method for members to obtain the credentials is TBA on the reflector).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-M1010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g. not referring to methods as “improved”, “optimized”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation is requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- All substantial communications about a CE, other than logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the main JVET reflector. In the case that large amounts of data are to be distributed is recommended to send an announcement to the JVET reflector without attaching the materials, and send the materials to those who have requested it directly, or provide a link to it, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector.

T2 = Test model software release + 2 weeks or X XX, whichever is earlier: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time and announced on the JVET reflector. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description by this time. Full test results must be provided at this time (at least for proposals targeting to be promoted to the draft standard at the next meeting).

CE reports may contain additional information about tests of straightforwared combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable). [Search/remove obsolete references to BMS.]

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, we would like see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of value of the tool given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology.

Availability of spec text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the spec will be. There must also be sufficient time to study it in detail. CE contributions without sufficiently mature draft spec text in the CE input document should not be considered for adoption.

Plans for the CEs to be conducted were established Thursday 18 January (GJS); CE plan documents were reviewed Friday 19 January (GJS & JRO).

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

## Software development and anchor generation (update)

The planned timeline for software releases was established as follows:

- VTM4.0 will be released by 2019-02-11. VTM4.1 with non-CTC adoptions will be released later. (If necessary, VTM4.0 may not include final tuning of context initialization values.)

- Further versions of VTM may be released for additional bug fixing, as appropriate.

- Preparation of the VTM software will include immediate removal of macros that were added in the previous meeting cycle. The software coordinator has the discretion to retain some such macros.

- Timeline of 360lib9.0: 1 week after the release of VTM4.0 (2019-02-18). Further versions may be released as appropriate for bug fixing.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise CE and AHG studies. - Report on project status to JVET reflector. - Provide a report to next meeting on project coordination status. | J.-R. Ohm, G. J. Sullivan (co-chairs) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize JVET-M1001 VVC text specification draft 4. - Produce and finalize JVET-M1002 VVC Test Model 4 (VTM 4) Algorithm and Encoder Description. - Gather and address comments for refinement of these documents. - Coordinate with test model software development AhG to address issues relating to mismatches between software and text. | B. Bross, J. Chen (co-chairs), J. Boyce, S. Kim, S. Liu, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test model (VTM) software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Discuss and make recommendations on the software development process. - Propose improvements to the guideline document for developments of the test model software. - Perform tests of VTM 4 behaviour relative to HEVC and VTM 3 using the VTM common test conditions and the multi-resolution streaming test conditions described in JVET-M0466. - Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Coordinate with AHG6 for integration with 360lib software. | F. Bossen, X. Li, A. Norkin, K. Sühring (co-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of the VVC standard. - Identify and recommend appropriate test materials for use in the development of the VVC standard. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Evaluate new test sequences, particularly including the material recently submitted by the Blender Foundation / Blender Animation Studio and Twitch. - Propose a new structure for the test sequence repository. - Facilitate availability of viewing equipment and facilities arrangements for the next meeting and pre-meeting testing as feasible. | T. Suzuki (chair), V. Baroncini, R. Chernyak, P. Hanhart, A. Norkin, J. Ye (vice-chairs) | N |
| Memory bandwidth consumption of coding tools (AHG5) (jvet@lists.rwth-aachen.de) - Develop improved software tools for measuring both average and worst case of memory bandwidth, and provide information for usage of these tools. - Study cache configurations for measuring decoder memory bandwidth consumption. - Identify coding tools in CEs and VTM with significant memory bandwidth impact. - Study the impact of memory bandwidth on specific application cases. | R. Hashimoto (chair), T. Ikai, X. Li, D. Luo, H. Yang, M. Zhou (vice-chairs) | N |
| 360° video conversion software development (AHG6) (jvet@lists.rwth-aachen.de) - Prepare and deliver the 360Lib-9.0 software version and common test condition configuration files according to JVET-M1012. - Generate CTC (PHEC) anchors and PERP results for VTM according to JVET-M1012, and finalize the reporting template for the common test conditions. - Produce documentation of software usage for distribution with the software. | Y. He, K. Choi (co-chairs) | N |
| Coding of HDR/WCG material (AHG7) (jvet@lists.rwth-aachen.de) - Study and evaluate available HDR/WCG test content. - Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results of the CfP responses. - Compare the performance of the VTM and HM for HDR/WCG content. - Prepare for expert viewing of HDR content at the 14th JVET meeting if feasible. - Coordinate implementation of HDR anchor aspects in the test model software with AHG3. - Study additional aspects of coding HDR/WCG content. | A. Segall (chair), E. François, W. Husak, D. Rusanovskyy (vice-chairs) | N |
| 360° video coding tools and test conditions (AHG8) (jvet@lists.rwth-aachen.de) - Study the effect on compression and subjective quality of different projections formats, resolutions, and packing layouts. - Discuss refinements of common test conditions, test sequences, and evaluation criteria. - Solicit additional test sequences, and evaluate suitability of test sequences on head-mounted displays and normal 2D displays. - Study coding tools dedicated to 360° video, their impact on compression, and implications to the core codec design. - Study the effect of viewport resolution, field of view, and viewport speed/direction on visual comfort. - Study complexity of GPU rendering of projection formats - Study syntax for signalling of projection formats, cubeface layouts, spherical rotations | J. Boyce (chair), K. Choi, P. Hanhart, J.-L. Lin (vice-chairs) | N |
| Neural networks in video coding (AHG9) (jvet@lists.rwth-aachen.de) - Investigate the benefit of using neural networks in video compression such as CNN loop filter, intra prediction, re-sampling in adaptive resolution coding, and encoder side partition mode decisions. - Investigate the complexity impact of using neural networks in video compression. - Investigate the complexity measurement of neural network coding tools. - Investigate the impact of training materials on the performance of neural network coding tools. - Investigate the impact of the training process on performance and complexity. | S. Liu (chair), B. Choi, K. Kawamura, Y. Li, L. Wang, P. Wu, H. Yang (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as GOP structures and perceptually optimized adaptive quantization for encoder optimization. - Study the impact of adaptive quantization on individual tools in the test model. - Study the quantization adaptation tool in the test model. - Investigate the feasibility of adding a CTC test category in which adaptive quantization is turned on. - Study quality metrics for measuring subjective quality using e.g. the CfP response MOS scores. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures, adaptive quantization without signalling, and multi-pass encoding. - Study methods of rate control and their impact on performance, subjective and objective quality. | A. Duenas, A. Tourapis (co-chairs), C. Helmrich, S. Ikonin, A. Norkin, R. Sjöberg, T. Toma (vice-chairs) | N |
| Screen content coding (AHG11) (jvet@lists.rwth-aachen.de) - Investigate coding tools targeted at screen content in terms of compression benefit and implementation complexity. - Identify test materials, discuss testing conditions for screen content coding, and propose associated updated common test conditions. - Study the impact of loop filters on screen content coding | S. Liu (chair), J. Boyce, A. Filippov, Y.-C. Sun, J. Xu, M. Zhou (vice-chairs) | N |
| High-level parallelism and coded picture regions (AHG12) (jvet@lists.rwth-aachen.de) - Study wavefront processing including the relationship with tiles and low delay characteristics. - Study flexible loop filter control and tile size restriction, including identifying implications on coding tools and implementation. - Study flexible tile partitioning (e.g. more flexible than HEVC and tile boundaries not spanning a full picture). - Study support of independently coded picture regions, including easy rewriting of such regions into a conforming sub-bitstream. - Prepare software and configurations for the test model to facilitate parallel processing tests. - Study the coding efficiency impact of parallel processing and coded picture regions. | S. Deshpande (chair), M. M. Hannuksela, R. Sjöberg, R. Skupin, W. Wan, Y.-K. Wang S. Wenger (vice-chairs) | N |
| Tool reporting procedure (AHG13) (jvet@lists.rwth-aachen.de) - Prepare output document JVET-M1005, which describes the methodology of tool-off testing and a list of tools to be tested by identified testers. - Provide configurations files, bitstreams, and results of tool-on/tool-off testing. - Use the tool usage counts and memory bandwidth usage to study the decoder complexity of features in on/off testing. - Prepare a report with results of the tests. | W.-J. Chien, J. Boyce (co-chairs), Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, S. Liu, D. Luo (vice-chairs) | N |
| Progressive intra refresh (AHG14) (jvet@lists.rwth-aachen.de) - Define relevant test conditions for the study of progressive intra refresh for random access without intra frames - Update the implementation of encoder-only intra refresh in the VTM model in the AHG14 fork of the software repository. - Evaluate different ways to produce intra refresh within VVC and characterize their coding efficiency impact, subjective quality, and delay characteristics, including encoder-only approaches and normative approaches - Consider the use of constrained intra prediction and tile-based approaches - Study recovery point handling, including practical implementation issues and perfect-versus-approximate decoded picture recovery. - Consider the potential need for starting a coded video sequence without an intra picture. | J.-M. Thiesse (chair), A. Duenas, K. Kazui, R. Sjöberg, A. Tourapis (vice-chairs) | N |
| Bitstream decoding properties signalling (AHG15) (jvet@lists.rwth-aachen.de) - Study syntax alternatives for interoperability point signalling - Study selection of constraint flags to be included in the VTM and their impact on syntax, semantics, and decoding process | J. Boyce (chair), J. Chen, S. Deshpande, M. Karczewicz, A. Tourapis, Y.-K. Wang, S. Wenger (vice-chairs) | N |
| Implementation studies (AHG16) (jvet@lists.rwth-aachen.de) - Study draft and proposed coding tools to identify implementation issues relating to decoder pipelines, decoder throughput, and other aspects of implementation difficulty. - Solicit hardware analysis of complex tools. - Particularly consider intra reconstruction throughput for small blocks. - Provide feedback on potential solutions to address identified issues. | M. Zhou (chair), J. An, E. Chai, K. Choi, S. Ethuraman, T. Hsieh, X. Xiu (vice-chairs) | N |
| High-level syntax (AHG17) (jvet@lists.rwth-aachen.de) - Study NAL unit header, sequence parameter set, picture parameter set, adaptation parameter set, and tile group header syntax designs - Study the proposed picture header designs and alternatives - Study reference picture buffering and list construction - Study random access signalling and random access approaches, including approaches with reference pictures provided by external means - Assist in software development and text drafting for the high-level syntax in the VVC design. | R. Sjöberg (chair), S. Deshpande, M. M. Hannuksela, R. Skupin, Y.-K. Wang, S. Wenger, H. Yu (vice-chairs) | N |
| Quantization control (AHG18) (jvet@lists.rwth-aachen.de) - Identify methods for quantization step size control for luma and chroma, including spatially and frequency-adaptive approaches - Develop methods for evaluating quantization step size control operation - Study the impact of MTS transforms on quantization matrices and the need for default matrices - Study the interaction between in-loop “reshaping” and quantization step size control - Develop testing conditions for evaluating QP signalling improvements including rate control and perceptual optimization strategies as appropriate - Evaluate the performance of the current VVC QP design using the two adaptive quantization control techniques currently available in the VTM | R. Chernyak (chair), E. François, C. Helmrich, A. Segall (vice-chairs) | N |
| Layered coding and resolution adaptivity (AHG19) (jvet@lists.rwth-aachen.de) - Study adaptive-resolution coding approaches for real-time communication, adaptive streaming, and 360-degree viewport-dependent streaming, including filters for resampling, reference picture management, and related scope and signalling - Study approaches for temporal scalability to avoid temporal judder when temporal scalability sub-bitstream extraction is used for achieving lower frame rate, and consider whether this should have a normative impact. - Identify related test conditions, test sequences, and evaluation techniques (including subjective assessment techniques) - Study potential approaches for support of layered coding scalability including spatial, temporal, quality, and view scalability | S. Wenger and A. Segall (co-chairs), M. M. Hannuksela, Hendry, S. McCarthy, Y.-C. Sun (vice-chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as MPEG output document, a separate version under the MPEG document header should be generated. This version should be sent to GJS and JRO for upload.

JVET-M1000 Meeting Report of the 13th JVET Meeting [G. J. Sullivan, J.-R. Ohm] (2019-03-08, near next meeting)

Initial versions of the meeting notes (d0 … d8) were made available on a daily basis during the meeting.

JVET-M1001 Versatile Video Coding (Draft 4) [B. Bross, J. Chen, S. Liu] [WG 11 N 18274] (2019-03-08)

(Initial version planned to be made available by 2019-02-01.)

See the list of elements under section Error! Reference source not found., as agreed by the Wed. 18 October plenary.

JVET-M1002 Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4) [J. Chen, Y. Ye, S. Kim] [WG 11 N 18725] (2019-03-08)

(Initial version planned to be made available by 2019-02-15.)

See the list of elements under section Error! Reference source not found., as agreed by the Wed. 18 October plenary.

Remains valid – not updated: JVET-K1003 Guidelines for VVC reference software development [K. Sühring] (2018-07-31)

New version?

JVET-M1004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 9) [Y. Ye, J. Boyce] (2019-02-15)

Remains valid – not updated: JVET-L1005 Methodology and reporting template for coding tool testing [W.-J. Chien and J. Boyce] (2018-10-26)

JVET-M1006 Methodology and reporting template for neural network coding tool testing [Y. Li, S. Liu, K. Kawamura] (2019-02-01)

This output was produced to capture aspects specific to enable study of neural network techniques.

JVET-M1010 JVET common test conditions and software reference configurations for SDR video [F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring] (2019-02-01)

Update regarding CPR and hash search, used only for class F.

Enable inter MTS for lower-resoluitons? Perhaps in a CE, but not in CTC.

Remains valid – not updated: JVET-L1011 JVET common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, S. Iwamura, D. Rusanovskyy] (2018-10-26)

Remains valid – not updated: JVET-L1012 JVET common test conditions and evaluation procedures for 360° video [P. Hanhart, J. Boyce, K. Choi, J.-L. Lin] (2018-10-26)

JVET-M1021 Description of Core Experiment 1 (CE 1): Post-prediction and post-reconstruction filtering [J. Ström, S. Ikonen, V. Seregin]

JVET-M1022 Description of Core Experiment 2 (CE2): Subblock motion compensation [C.-C. Chen, Y. He, H. Liu]

JVET-M1023 Description of Core Experiment 3 (CE3): Intra prediction and mode coding [G. Van der Auwera, L. Li, A. Filippov]

JVET-M1024 Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding [H. Yang, G. Li, K. Zhang]

JVET-M1025 Description of Core Experiment 5 (CE5): Adaptive loop filtering [C.-Y. Chen, V. Seregin]

JVET-M1026 Description of Core Experiment 6 (CE6): Transforms and transform signalling [X. Zhao, H. E. Egilmez]

JVET-M1027 Description of Core Experiment 7 (CE 7): Quantization and coefficient coding [H. Schwarz, M. Coban, C. Auyeung]

Coordination between CE7 and CE8 is desired for TS coefficient coding evaluation.

JVET-M1028 Description of Core Experiment 8 (CE8): Screen Content Coding Tools [X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu]

Transform skip coefficient coding should be tested in CE8, and should be tested with low QP as well as with CTC.

Coordination between CE7 and CE8 is desired for TS coefficient coding evaluation.

JVET-M1029 Description of Core Experiment 9 (CE9): Decoder Motion Vector Derivation [S. Esenlik, X. Xiu]

JVET-M1030 Description of Core Experiment 10 (CE10): Combined inta/inter prediction [C.-W. Hsu, M. Winken]

JVET-M1031 Description of Core Experiment 11 (CE11): Deblocking [A. Norkin, A. M. Kotra]

JVET-M1032 Description of Core Experiment 12 (CE12): Tile set boundary motion compensation handling [Hendry, R. Skupin, W. Wan]

JVET-M1033 Description of Core Experiment 13 (CE13): Neural-network based loop filtering [Y. Li, S. Liu, K. Kawamura]

Potentially obsolete notes: New CEs which may fill gaps in above numbering:

Adaptive loop filter [V. Seregin, …]

Post prediction/reconstruction filtering (include BF, HF, LIC, DIF) [J. Ström, S. Ikonin, …]

Neural network based loop filters [Y. Li, …]

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Tuesday of the first week and closing it on the Wednesday of the second week of the SG 16 meeting – a total of 9 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday prior to such meetings and closing it at lunchtime on the last day of the WG 11 meeting – a total of 9.5 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined.

Some specific future meeting plans (to be confirmed) were established as follows:

- Wed. 3 – Fri. 12 July 2019, 15th meeting under WG 11 auspices in Gothenburg, SE.

- Tue. 1 – Wed. 9 October 2019, 16th meeting under ITU-T auspices in Geneva, CH.

- Wed. 8 – Fri. 17 January 2020, 17th meeting under WG 11 auspices in Brussels, BE.

- Wed. 15 – Fri. 24 April 2020, 18th meeting under WG 11 auspices in Alpbach, AT.

The agreed document deadline for the 15th JVET meeting was planned to be XXday XX June 2019. Plans for scheduling of agenda items within that meeting remained TBA.

ITU was thanked for the excellent hosting of the 14th meeting of the JVET.

XXXX and XXXX were thanked for providing viewing equipment used during the 14th JVET meeting. XXXX was thanked for designing and coordinating the subjective test efforts related to CE5 and CE11. The experts who helped in preparing and conducting the test, or participated in the role as test subjects, were also thanked.

The 14th JVET meeting was closed at approximately 1318 hours on Friday 18 January 2019.

# Annex A to JVET report:
List of documents

# Annex B to JVET report:
List of meeting participants

The participants of the fourteenth meeting of the JVET, according to a sign-in sheet circulated during the meeting sessions (approximately XXX people in total), were as follows: