| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0002-v2 |
| --- | --- |

| Title: | JVET AHG report: Draft text and test model algorithm description editing (AHG2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Benjamin Bross Jianle Chen Jill Boyce Seung-Hwan Kim Shan Liu Yan Ye | Email: | benjamin.bross@hhi.fraunhofer.de jianle.chen@huawei.com jill.boyce@intel.com seunghwan3.kim@lge.com shanl@tencent.com yan.ye@alibaba-inc.com |
| Source: | AHG2 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on draft text and test model algorithm description editing (AHG2) between the 13th meeting in Marrakech, MA (9–18 January 2019) and the 14th meeting in Geneva, CH, (19–27 March 2019).

# Mandates

At the 13th JVET meeting, AHG2 on draft text and test model algorithm description editing was established with the following mandates:

- Produce and finalize JVET-M1001 Versatile Video Coding (Draft 4).

- Produce and finalize JVET-M1002 VVC Test Model 4 (VTM 4) Algorithm and Encoder Description.

- Gather and address comments for refinement of these documents.

- Coordinate with Test model software development AhG to address issues relating to mismatches between software and text.

# Background

At the 13th JVET meeting, it was decided to include additional coding features for intra picture-prediction, inter-picture prediction, transform, CABAC engine and de-blocking filter in the fourth draft of Versatile Video Coding (VVC D4) and the VVC Test Model 4 (VTM4) encoding. Draft reference software to implement the VVC decoding process and VTM3 encoding method has also been developed.

The normative decoding process for Versatile Video Coding is specified in the VVC draft 4 text specification document [1]. The VVC Test Model 4 (VTM 4) Algorithm and Encoder Description document provides an algorithm description as well as an encoder-side description of the VVC Test Model 4, which serves as a tutorial for the algorithm and encoding model implemented in the VTM4.0 software [2].

An issue tracker (https://jvet.hhi.fraunhofer.de/trac/vvc) was used to facilitate the reporting of errata with the VVC documents.

# JVET-M1001 VVC specification (Draft 4)

## List of Integrations

Seven versions of JVET-M1001 were published by the Editing AHG between the 13th meeting in Marrakech, MA (9–18 January 2019) and the 14th meeting in Geneva, CH, (19–27 March 2019).

JVET-M1001 has been established based on JVET-L1001 and now contains the following:

- Incorporated JVET-M0102: Intra subpartitions (ISP).

- Incorporated JVET-M0142: chroma format dependent CCLM downsampling filter.

- Incorporated JVET-M0064: table reduction in CCLM model parameter calculation.

- Incorporated JVET-M0092: intra reference sample filtering cleanup.

- Incorporated JVET-M0238: PDPC linear interpolation on the secondary boundary for adjacent angular modes is changed to nearest neighbour.

- Incorporated JVET-M0407: CPR search range.

- Fixed bug #154 Availability check for CPR/IBC chroma CU reference block is missing.

- Aligned PDPC filtering for INTRA_ANGULAR18 and INTRA_ANGULAR50 to VTM.

- Fixed bug #167 on PDPC size condition.

- Incorporated JVET-M0497: Fast DST-7/DCT-8.

- Incorporated JVET-M0303: Shape adaptive transform selection.

- Incorporated JVET-M0464/JVET-M0201: Combined transform skip(TS) and MTS syntax plus extended TS sizes.

- Incorporated JVET-M0297: Zero-out of last 16 samples for 32 samples DST-7/DCT-8.

- Incorporated JVET-M0140: Subblock transform for inter CUs.

- Incorporated JVET-M0251/M0257: Zero-out of last 32 samples for 64 samples DCT-2 fix.

- Incorporated JVET-M0273/M0240/M0116/M0338/M0204: only using left neighbor for SbTMVP fetching

- Incorporated JVET-M0246: AMVR for affine

- Incorporated JVET-M0145: affine sub-block MV clipping

- Incorporated JVET-M0166/M0228/M0477: remove MV comparison for constructed merge candidates

- Incorporated JVET-M0170: Parallel processing for merge mode

- Incorporated JVET-M0147: Decoder side motion vector refinement

- Incorporated JVET-M0361: fix of cu_cbf for merge mode

- Incorporated JVET-M0487: using integer samples instead of bilinear interpolation for extended region of BDOF.

- Incorporated JVET-M0483: IBC signalled as a separate CU prediction mode.

- Incorporated JVET-M0063: Generalization of BDOF bit-depth.

- Incorporated triangular modifications including:

  - JVET-M0118/M0185/M0190/M207(test 1)/M0216(the first aspect)/M0234 (change corresponding to the result table 7 and 8)/M0317(section 2.2)/M0328: Do not signal the triangular prediction mode flag in cases where the combination is not allowed (MMVD, CIIP),

  - JVET-M0328: always use second weight group in triangular prediction.

- Incorporated JVET-M0883: signaling change of triangular merging candidate which does not need LUT.

- Incorporated JVET-M0193: pairwise average merging candidate reduction.

- Incorporated HMVP modifications including:

  - JVET-M0436 reduce HMVP number from 6 to 5,

  - JVET-M0300 HMVP initialization for parallel processing with tiles,

  - JVET-M0264 GBI weight is also stored in HMVP,

  - JVET-M0126 reduced HMVP candidate pruning.

- Incorporated JVET-M0255: MMVD mode without fractional sample offsets for screen content coding.

- Incorporated JVET-M0171/M0068: remove redundant MV scaling in MMVD.

- Incorporated JVET-M0444: symmetrical MVD coding for L0 to L1.

- Incorporated JVET-M0479: MV clipping to 18 bits.

- Incorporated JVET-M0512: TMVP storage reduction.

- Incorporated JVET-M0192: subblock chroma MV derivation for affine from two luma MVs.

- Incorporated JVET-M0111: weighted prediction (WP) and disable GBI signalling if WP is enabled.

- Incorporated JVET-M0281/M0117: modified AMVP pruning with rounding.

- Fixed bug #175 incorrect derivation of CCLM parameter b.

- Incorporated JVET-M0128: Reference picture management

- Incorporated JVET-M0132: Adaptation parameter set (APS)

- Incorporated JVET-M0853: Adding the support of rectangular tile groups in addition to the existing raster-scan tile groups, and enabling extraction of MCTSs without changing VLC NAL units

- Incorporated JVET-M0160: Adding loop_filter_across_tile_group_enabled_flag to the PPS

- Incorporated JVET-M0101:

  - Replace the existing IRAP_NUT with 3 new NAL unit types: IDR_W_RADL, IDR_N_LP, CRA_NUT (from JVET-M0101).

  - Add external means flag HandleCraAsCvsStartFlag, with similar text as in HEVC. Text provided in a v3 of JVET-M0101.

  - Add a NUT value for step-wise temporal access STSA (from JVET-M0101).

  - Add a NUT value for AUD (from JVET-M0101).

  - Add sps_max_sub_layers_minus1 syntax element to SPS, and decoding process in 8.1.1, 8.1.2 and 8.1.3 of JVET-M0101.

  - Add text of sections 7.4.2.4 to 7.4.2.4.5 on NAL unit order and AU boundary detection from JVET-M0101, which is primarily editorial, but has some technical aspects.

  - Add profile_tier_level( ) syntax structure which includes sub-layer level idc (similar to HEVC but without sub-layer-specific profiles).

  - Add general_non_packed_constraint_flag with semantics as in JVET-M0101 (rename the flag to display_suitability_flag? – that's editorial).

  - Add the temporal scalability sub-bitstream extraction process in JVET-M0101.

  - Add RASL and RADL NUTs

- Incorporated JVET-M0451: Add new constraint flags corresponding to VVC WD 3 tools.

- Incorporated JVET-M0415: Change the sps_ref_wraparound_offset to sps_ref_wraparound_offset_minus1 and changing the units to be MinCbSizeY as in option 1 (minor cleanup).

- Incorporated JVET-M0381: Reduce merge idx ctx coded bins (test 2.2.2a).

- Incorporated JVET-M0502: Add one context for pred_mode_flag (method 2).

- Incorporated JVET-M0453: Modified CABAC probability estimation (5.1.13* + init from 5.1.2).

- Fixed bug #147 on coefficient coding.

- Incorporated JVET-M0470: Limited EGk for abs_rem/ dec_abs_level.

- Incorporated JVET-M0173: Move rem_abs_gt3_flag into first coding pass.

- Incorporated JVET-M0119: Modified dequantization scaling for TS.

- Incorporated JVET-M0685: QP prediction fix for parallel encoding.

- Incorporated JVET-M0113/M0188: Bug fix for quantization group QP signalling.

- Incorporated JVET-M0421: Split-first signalling for partitioning.

- Incorporated JVET-M0446/M0888/M0905: Inferred QT split to avoid 32x128/128x32 partitions at picture boundaries.

- Incorporated JVET-M0427: Picture reconstruction with luma mapping and chroma scaling (LMCS).

The following adoptions have not yet been integrated and will be integrated in an eight version (v8):

- JVET-M0908: Deblocking of CIIP boundaries.

- JVET-M0471: Deblocking with long tap filters (also alignment with SBT).

- JVET-M0277: Apply pcm_loop_filter_disabled_flag for ALF.

- JVET-M0265: Clean-up on MV rounding.

## Integration Issues

The following items have been discussed within the AHG:

- Deblocking over ISP internal TU boundaries: Two different approaches have been identified resulting from interaction between JVET-M0102 Intra Subpartitions (ISP) and JVET-M0471 long deblocking:

  - Do not deblock (originally proposed and adopted ISP design)

  - Deblock all internal TU boundaries, i.e. for Subblock Transforms (SBT) and ISP.

The current draft and VTM-4.0.1 incorporates 1. However, it needs to be discussed which solution is preferred. The following document was identified to be related:

JVET-N0473 Non-CE11: On ISP transform boundary deblocking [K. Misra, A. Segall (Sharp Labs of America), M. Ikeda, T. Suzuki (Sony)]

- Last position coding fix for large size transform zero-out:  JVET-M0251 and JVET-M0257 have been adopted to change the following:

  - Last x/y position is coded using the reduced non-zero size, e.g. 32x32 for a 64x64 block (affects binarization)

  - The context derivation of last x/y position still depends on the larger transform size

The current draft incorporates 1. and 2. while VTM-4.0.1 incorporates only 1. However, it needs to be discussed which solution is preferred. The following document was identified to be related:

JVET-N0189 Non-CE7: Unified last position coding for 32-point transforms [Y. Piao, K. Choi (Samsung)]

JVET-N0194 CE6-related: Context selection of last non-zero coefficient position coding based on reduced TU size (related to JVET-M0297 and JVET-M0251/M0257) [M. Koo, J. Choi, M. Salehifar, J. Lim, S. Kim (LGE)]

- Combination of shared merge list (JVET-M0170) and IBC (JVET-M0483):

  - Ticket #192 provides a merge request to applies JVET-M0170 shared merge candidate list also to the IBC merge list. The merge request for merged in VTM-4.0.1.

  - Text was provided to the editors to align the draft with VTM-4.0 which will be integrated in an upcoming version of the draft.

# Model 4 (VTM 4) Algorithm and Encoder Description.

Two versions of JVET-M1002 was published by the Editing AHG between the 13th meeting in Marrakech, MA (9–18 January 2019) and the 14th meeting in Geneva, CH, (19–27 March 2019).

JVET-M1002 has been established based on JVET-L1002. It provides the algorithm description for majority of coding tools in VVC. In this editing period, the following changes were included:

- Incorporated JVET-M0427: luma mapping with chroma scaling (previously known as adaptive in-loop reshaper)

- Incorporated JVET-M0102: Intra subpartitions (ISP)

- Incorporated JVET-M0147: Decoder side motion vector refinement

- Incorporated JVET-0471: Long Deblocking

- Incorporated JVET-0483: Intra block copy

- Incorporated JVET-M0453: CABAC core engine

- Incorporated JVET-M0118, JVET-M0328 and JVET-M0883: triangle prediction related changes

- Incorporated JVET-M0487 and JVET-M0063: BDOF related changes

- Incorporated JVET-M0273: SbTMVP related changes

- Incorporated JVET-M0111: BWA related changes:

- Incorporated JVET-M0142: Alternative CCLM downsampling filter

- Incorporated JVET-M0064: Reduced table size of CCLM parameter derivation

- Incorporated JVET-M0238: Simplification of PDPC reference samples

- Incorporated JVET-M0407: IBC reference region modification

- Incorporated JVET-M0297: 32-length DST-7/DCT-8 using zero-out

- Incorporated JVET-M0464: Unified MTS and transform skip syntax

- Incorporated JVET-M0173: rem_abs_gt3_flag in first coding pass

- Incorporated JVET-M0246: Affine AMVR

In this editing period, the descriptions of most new coding tools from Marrakech meeting have been included in the latest release. Refinements of some existing tools in VTM3 have not been integrated. General encoder description, High Level Syntax, tiling mechanism and miscellaneous small coding features (such PCM mode and Delta QP signaling) are still missing at this moment. The VTM editors will work on those aspects in the following meeting circles.

# Recommendations

The AHG recommends to:

- Approve the edited JVET-M1001 and JVET-M1002 documents as JVET outputs,

- Continue to edit the VVC draft and Test Model documents to ensure that all agreed elements of VVC are fully described,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification draft and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC WD and Test Model documents,

- Ensure that, when considering the addition of new feature to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC Working Draft is made available in a timely manner.

# References

- B. Bross, J, Chen, S. Liu, “Versatile Video Coding (Draft 4),” document JVET-M1001, 13th JVET meeting: Marrakech, MA, 9–18 Jan. 2010.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4),” document JVET-M1002, 13th JVET meeting: Marrakech, MA, 9–18 Jan. 2010.