| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0053 |
| --- | --- |

| Title: | CE6: 2-mode MTS (CE6-2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jani Lainema | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document provides results for core experiment CE6-2.1 which was set up to study effectiveness of different configurations of a 2-mode MTS (multiple transform selection) approach including only DCT2 and DST7 transforms.

The proposed 2-mode MTS limits the maximum value of the tu_mts_idx syntax element to 1 and uses only DCT2 and DST7 transforms. In the case of the proposed 2-mode MTS, tu_mts_idx equal to 0 indicates that DCT2-DCT2 transform pair is used, and tu_mts_idx equal to 1 indicates that DST7-DST7 transform pair is used.

As part of the experiment, allowing different maximum sizes for TT and BT splits for intra tile groups were studied. In the current common test conditions TT and BT splitting is not used for 64x64 CUs in intra tile groups. The tests a, b and c differ from each other in that regard as follows:

- CE6-2.1a: QT, TT and BT splits are allowed for 64x64 CUs in intra tile groups

- CE6-2.1b: QT, TT splits are allowed for 64x64 CUs in intra tile groups

- CE6-2.1c: QT splits are allowed for 64x64 CUs in intra tile groups (as in CTC)

# Proposed tool

The proposed 2-mode MTS limits the maximum value of the tu_mts_idx syntax element to 1 and uses only DCT2 and DST7 transforms. In the case of the proposed 2-mode MTS, tu_mts_idx equal to 0 indicates that DCT2-DCT2 transform pair is used, and tu_mts_idx equal to 1 indicates that DST7-DST7 transform pair is used.

As DCT8 is disabled by the tool, the transform pairs used by sub-block transform (SBT) coding are also updated by substituting DCT8 transforms by DCT2 transforms.

The splitting thresholds for 64x64 intra CUs were controlled with the following VTM-4 compile time parameters:

#if CE6_MAX_SIZE_BT_I_SLICE_64

static const int    MAX_BT_SIZE   = 64;

#else

static const int    MAX_BT_SIZE   = 32;

#endif

#if CE6_MAX_SIZE_TT_I_SLICE_64

static const int    MAX_TT_SIZE   = 64;

static const int    MAX_TT_SIZE_C = 64;

#else

static const int    MAX_TT_SIZE   = 32;

static const int    MAX_TT_SIZE_C = 32;

#endif

# Simulation results

The following table summarizes the results for common test conditions (CTC) and for the Inter MTS.  Detailed results for both categories of tests are available in the attached Excel spreadsheets.

| CTC |  | 2.1a | 2.1b | 2.1c |
| --- | --- | --- | --- | --- |
| Split |  | QT,TT,BT | QT,TT | QT |
|  | Y | -0.32 % | -0.02 % | 0.20 % |
|  | U | -1.81 % | -1.48 % | -0.25 % |
| AI | V | -1.83 % | -1.51 % | -0.26 % |
|  | Enc | 107 % | 89 % | 75 % |
|  | Dec | 100 % | 100 % | 100 % |
|  | Y | 0.04 % | 0.11 % | 0.18 % |
|  | U | -0.72 % | -0.78 % | -0.13 % |
| RA | V | -0.83 % | -0.74 % | -0.25 % |
|  | Enc | 98 % | 97 % | 97 % |
|  | Dec | 99 % | 100 % | 100 % |
|  | Y | 0.16 % | 0.17 % | 0.14 % |
|  | U | -0.07 % | -0.04 % | 0.34 % |
| LB | V | -0.47 % | -0.65 % | -0.31 % |
|  | Enc | 99 % | 99 % | 99 % |
|  | Dec | 100 % | 100 % | 99 % |

| Inter-MTS |  | 2.1a | 2.1b | 2.1c |
| --- | --- | --- | --- | --- |
| Split |  | QT,TT,BT | QT,TT | QT |
|  | Y | 0.19 % | 0.26 % | 0.32 % |
|  | U | -0.84 % | -0.88 % | -0.23 % |
| RA | V | -0.91 % | -0.86 % | -0.35 % |
|  | Enc | 89 % | 89 % | 89 % |
|  | Dec | 100 % | 100 % | 100 % |
|  | Y | 0.26 % | 0.30 % | 0.29 % |
|  | U | -0.66 % | -0.31 % | -0.25 % |
| LB | V | -0.74 % | -0.75 % | -0.53 % |
|  | Enc | 90 % | 90 % | 90 % |
|  | Dec | 100 % | 100 % | 100 % |

# Specification text

8.7.4	Transformation process for scaled transform coefficients

Table 8-13 – Specification of trTypeHor and trTypeVer depending on tu_mts_idx[ x ][ y ]

| tu_mts_idx[ x0 ][ y0 ] | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| trTypeHor | 0 | 1 | 2 | 1 | 2 |
| trTypeVer | 0 | 1 | 1 | 2 | 2 |

Table 8-14 – Specification of trTypeHor and trTypeVer depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag

| cu_sbt_horizontal_flag | cu_sbt_pos_flag | trTypeHor | trTypeVer |
| --- | --- | --- | --- |
| 0 | 0 | 0 2 | 1 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 2 |
| 1 | 1 | 1 | 1 |

9.5.3	Binarization process

Table 9-4 – Syntax elements and associated binarizations

| tu_mts_idx[ ][ ] | TR | cMax = 1 4, cRiceParam = 0 |
| --- | --- | --- |

9.5.4.2	Derivation process for ctxTable, ctxIdx and bypassFlag

Table 9-10 – Assignment of ctxInc to syntax elements with context coded bins

| tu_mts_idx[ ][ ] | cqtDepth | na 6 | na 7 | na 8 | na | na |
| --- | --- | --- | --- | --- | --- | --- |

8.7.4.3	Transformation matrix derivation process

- Otherwise, if trType is equal to 2 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] =

{

{  84   74   55   29 }

{  74    0  −74  −74 }

{  55  −74  −29   84 }

{  29  −74   84  −55 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] =

{

{  86   85   78   71   60   46   32   17 }

{  85   60   17  −32  −71  −86  −78  −46 }

{  78   17  −60  −86  −46   32   85   71 }

{  71  −32  −86  −17   78   60  −46  −85 }

{  60  −71  −46   78   32  −85  −17   86 }

{  46  −86   32   60  −85   17   71  −78 }

{  32  −78   85  −46  −17   71  −86   60 }

{  17  −46   71  −85   86  −78   60  −32 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] =

{

{  88   88   87   85   81   77   73   68   62   55   48   40   33   25   17    8 }

{  88   81   68   48   25    0  −25  −48  −68  −81  −88  −88  −81  −68  −48  −25 }

{  87   68   33   −8  −48  −77  −88  −81  −55  −17   25   62   85   88   73   40 }

{  85   48   −8  −62  −88  −77  −33   25   73   88   68   17  −40  −81  −87  −55 }

{  81   25  −48  −88  −68    0   68   88   48  −25  −81  −81  −25   48   88   68 }

{  77    0  −77  −77    0   77   77    0  −77  −77    0   77   77    0  −77  −77 }

{  73  −25  −88  −33   68   77  −17  −88  −40   62   81   −8  −87  −48   55   85 }

{  68  −48  −81   25   88    0  −88  −25   81   48  −68  −68   48   81  −25  −88 }

{  62  −68  −55   73   48  −77  −40   81   33  −85  −25   87   17  −88   −8   88 }

{  55  −81  −17   88  −25  −77   62   48  −85   −8   88  −33  −73   68   40  −87 }

{  48  −88   25   68  −81    0   81  −68  −25   88  −48  −48   88  −25  −68   81 }

{  40  −88   62   17  −81   77   −8  −68   87  −33  −48   88  −55  −25   85  −73 }

{  33  −81   85  −40  −25   77  −87   48   17  −73   88  −55   −8   68  −88   62 }

{  25  −68   88  −81   48    0  −48   81  −88   68  −25  −25   68  −88   81  −48 }

{  17  −48   73  −87   88  −77   55  −25   −8   40  −68   85  −88   81  −62   33 }

{   8  −25   40  −55   68  −77   85  −88   88  −87   81  −73   62  −48   33  −17 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15

transMatrixCol0to15 =

{

{  90   90   89   88   87   86   85   84   82   80   78   77   74   72   68   66 }

{  90   87   84   78   72   63   53   42   30   17    4   −9  −21  −34  −46  −56 }

{  89   84   74   60   42   21    0  −21  −42  −60  −74  −84  −89  −89  −84  −74 }

{  88   78   60   34    4  −26  −53  −74  −86  −90  −82  −66  −42  −13   17   46 }

{  87   72   42    4  −34  −66  −85  −89  −77  −50  −13   26   60   82   90   80 }

{  86   63   21  −26  −66  −87  −85  −60  −17   30   68   88   84   56   13  −34 }

{  85   53    0  −53  −85  −85  −53    0   53   85   85   53    0  −53  −85  −85 }

{  84   42  −21  −74  −89  −60    0   60   89   74   21  −42  −84  −84  −42   21 }

{  82   30  −42  −86  −77  −17   53   89   68    4  −63  −90  −60    9   72   88 }

{  80   17  −60  −90  −50   30   85   74    4  −68  −87  −38   42   88   66   −9 }

{  78    4  −74  −82  −13   68   85   21  −63  −87  −30   56   89   38  −50  −90 }

{  77   −9  −84  −66   26   88   53  −42  −90  −38   56   87   21  −68  −82   −4 }

{  74  −21  −89  −42   60   84    0  −84  −60   42   89   21  −74  −74   21   89 }

{  72  −34  −89  −13   82   56  −53  −84    9   88   38  −68  −74   30   90   17 }

{  68  −46  −84   17   90   13  −85  −42   72   66  −50  −82   21   90    9  −86 }

{  66  −56  −74   46   80  −34  −85   21   88   −9  −90   −4   89   17  −86  −30 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..15

transMatrixCol16to31 =

{

{  63   60   56   53   50   46   42   38   34   30   26   21   17   13    9    4 }

{ −66  −74  −80  −85  −88  −90  −89  −86  −82  −77  −68  −60  −50  −38  −26  −13 }

{ −60  −42  −21    0   21   42   60   74   84   89   89   84   74   60   42   21 }

{  68   84   90   85   72   50   21   −9  −38  −63  −80  −89  −87  −77  −56  −30 }

{  56   21  −17  −53  −78  −90  −84  −63  −30    9   46   74   88   86   68   38 }

{ −72  −89  −82  −53   −9   38   74   90   80   50    4  −42  −77  −90  −78  −46 }

{ −53    0   53   85   85   53    0  −53  −85  −85  −53    0   53   85   85   53 }

{  74   89   60    0  −60  −89  −74  −21   42   84   84   42  −21  −74  −89  −60 }

{  50  −21  −78  −85  −38   34   84   80   26  −46  −87  −74  −13   56   90   66 }

{ −77  −84  −26   53   90   56  −21  −82  −78  −13   63   89   46  −34  −86  −72 }

{ −46   42   90   53  −34  −88  −60   26   86   66  −17  −84  −72    9   80   77 }

{  78   74  −13  −85  −63   30   89   50  −46  −90  −34   60   86   17  −72  −80 }

{  42  −60  −84    0   84   60  −42  −89  −21   74   74  −21  −89  −42   60   84 }

{ −80  −60   50   85   −4  −87  −42   66   77  −26  −90  −21   78   63  −46  −86 }

{ −38   74   63  −53  −80   26   89    4  −87  −34   77   60  −56  −78   30   88 }

{  82   42  −77  −53   68   63  −60  −72   50   78  −38  −84   26   87  −13  −90 }

},

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).