| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0063/m46719 |
| --- | --- |

| Title: | [JVET AHG17] [SYS-VVC] Separate essential display information from non-essential display information in the Video Usability Information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Monzastr. 4c 63225 Langen Germany | Tel: Email: | +49-6103-766240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This proposal suggests including and reorganising a VUI-like syntax structure for VVC, although not necessarily described within the VVC specification itself. It is proposed that essential display information should be signalled somewhere within the video elementary stream and that this essential information contains only the following type of information: Sample aspect ratio, Colour description, Chroma location, Fields coding or frame coding for interlaced content, and Timing information. Additional information about the content may be signalled, but separately from the above mentioned essential information.

# Introduction

AVC and HEVC contain a structure called VUI (Video Usability Information) included into the Sequence Parameter Set that contain similar information in both standards (although the syntax is not exactly the same in both standards). The information contained in the VUI syntax structure is not required for decoding the luma and chroma samples of the video signal. However, some of the information is absolutely essential for proper rendering of the signal on a display. One example of such information is the colour information, which contains the syntax element transfer_characteristics. This syntax element contains information about the transfer function used to prepare the video signal (such as PQ or HLG for HDR content). If this information is not signalled, it has to be guessed by the display, which can create a terrible display experience if the receiver assumes that PQ content uses the standard gamma transfer function, even if the quality of the encoded video itself was very high.

Some of the information contained in the VUI is essential and is becoming even more essential in today’s world, where video can be HDR or SDR, use the SD, HD or UHD colour primaries, use the BT. 709 or the BT. 2020 chroma samples location with respect to luma samples location. Given how essential this information is to proper rendering of the video content, some application standards, such as DVB in ETSI TS 101 154 [1] mandates the presence of the VUI syntax structure and the presence and value of some of the syntax element, as in the excerpt from ETSI TS 101 154 below:

Therefore, such information has to be signalled somewhere, and it has to be signalled in a frame accurate way, which means that signalling of such information at the Systems level may be unreliable. All previously developed video coding standards enabled to signal such information in a frame accurate way at the video elementary stream level. The author is aware of discussions about specifying such information into another document than the VVC specification and supports this endeavour, as long as the signalling is still possible and frame accurate. However, modifying the location of this syntax structure would not change the basic idea proposed in the rest of this document. For simplicity, some of the syntax proposed in this document assumes that the information is signalled in the SPS as a placeholder, but it should not be understood as a proposal to put it there.

The main topic discussed in this proposal relates to the exact information to be signalled. As documented in ETSI TS 101 154 [1], some of the syntax elements are essential for the receiver to read and for the display to act upon (such as aspect_ratio_idc, colour_primaries, or transfer_characteristics), whereas others are meant only as information and can be safely ignored in most applications (such as video_full_range_flag, overscan_appropriate_flag, or the default display window). Unfortunately, in the HEVC syntax for the VUI, essential information is mixed with non-essential one that content providers may not know or may not want to signal. One such example is that signalling of colour_primaries and transfer_characteristics requires signalling of video_format and video_full_range_flag, which may be unknown and is not as essential for proper display. As a result, essential information about colour description is missing in some real-world bitstreams, forcing receivers to guess the information.

The current document proposes:

- To signal essential display information somewhere within the video elementary stream (although it does not have to be described within the VVC specification itself)

- That this essential information contains only the following type of information:

  - Sample aspect ratio

  - Colour description

  - Chroma location

  - Fields coding or frame coding for interlaced content

  - Timing information

- That additional information about the content should be signalled separately from the above-mentioned essential information (either in a different syntax structure, or at the end of the same syntax structure after the essential information)

# New syntax structure: essential video information

## Introduction

The basic idea is to signal the following information in a completely new syntax structure called EDI (Essential Display Information):

- Sample aspect ratio

- Colour description

- Chroma location

- Fields coding or frame coding for interlaced content

- Timing information

The VUI syntax structure may still exist to signal non-essential information about the content.

## Modifications to SPS syntax and semantics (as a placeholder)

The following syntax shows an example of how the two EDI and VUI syntax structures may be signalled in the SPS, but should not be understood as a proposal to signal them in the SPS. Any other level that would enable frame accurate signalling within the video elementary stream is acceptable.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| edi_parameters_present_flag | u(1) |
| if( edi_parameters_present_flag ) { |  |
| edi_parameters() |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters() |  |
| } |  |
| … |  |
| } |  |

edi_parameters_present_flag equal to 1 specifies that the edi_parameters( ) syntax structure as specified in TBD is present and that the vui_parameters() syntax structure as specified in TBD may be present. edi_parameters_present_flag equal to 0 specifies that the edi_parameters() and the vui_parameters( ) syntax structures are not present.

vui_parameters_present_flag equal to 1 specifies that the vui_parameters( ) syntax structure as specified in TBD is present. vui_parameters_present_flag equal to 0 specifies that the vui_parameters( ) syntax structure as specified in TBD is not present.

## New EDI syntax structure

The new Essential Display Information syntax structure uses the same syntax elements names and semantics as the VUI of HEVC for the relevant syntax elements.

| edi_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = EXTENDED_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coeffs | u(8) |
| } |  |
| field_seq_flag | u(1) |
| frame_field_info_present_flag | u(1) |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| if( field_seq_flag ) { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| else |  |
| chroma_sample_loc_type | ue(v) |
| } |  |
| vui_timing_info_present_flag | u(1) |
| if( vui_timing_info_present_flag ) { |  |
| vui_num_units_in_tick | u(32) |
| vui_time_scale | u(32) |
| vui_poc_proportional_to_timing_flag | u(1) |
| if( vui_poc_proportional_to_timing_flag ) |  |
| vui_num_ticks_poc_diff_one_minus1 | ue(v) |
| vui_hrd_parameters_present_flag | u(1) |
| if( vui_hrd_parameters_present_flag ) |  |
| hrd_parameters( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| } |  |

In terms of semantics, it is proposed to keep the exact same text as in HEVC Annex E for all syntax elements that are listed in the EDI above. The only syntax element that is new in the syntax presented above is chroma_sample_loc_type for progressive content. chroma_sample_loc_type should be understood as having the exact same semantics as chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field.

## Modifications to VUI syntax structure

The VUI syntax structure is modified to remove the information already present in the EDI syntax structure proposed above.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| video_signal_type_present_flag | u(1) |
| if( video_signal_type_present_flag ) { |  |
| video_format | u(3) |
| video_full_range_flag | u(1) |
| } |  |
| neutral_chroma_indication_flag | u(1) |
| default_display_window_flag | u(1) |
| if( default_display_window_flag ) { |  |
| def_disp_win_left_offset | ue(v) |
| def_disp_win_right_offset | ue(v) |
| def_disp_win_top_offset | ue(v) |
| def_disp_win_bottom_offset | ue(v) |
| } |  |
| // Other non-essential video information such as, for example, restriction flags |  |
| } |  |

In terms of semantics, it is proposed to keep the exact same text as in HEVC Annex E for all syntax elements that are listed in the VUI above.

# Reorganising the HEVC VUI syntax structure to signal the essential display information first

## Introduction

The basic idea is to signal all the information (essential and non-essential) within the same data structure, but separate the essential information by signalling it at the beginning, and include a flag for all the non-essential information signalled after the essential information.

## Modifications to VUI syntax structure

The proposed VUI mostly uses the same syntax elements names and the same semantics as in HEVC Annex E.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = EXTENDED_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coeffs | u(8) |
| } |  |
| field_seq_flag | u(1) |
| frame_field_info_present_flag | u(1) |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| if( field_seq_flag ) { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| else |  |
| chroma_sample_loc_type | ue(v) |
| } |  |
| vui_timing_info_present_flag | u(1) |
| if( vui_timing_info_present_flag ) { |  |
| vui_num_units_in_tick | u(32) |
| vui_time_scale | u(32) |
| vui_poc_proportional_to_timing_flag | u(1) |
| if( vui_poc_proportional_to_timing_flag ) |  |
| vui_num_ticks_poc_diff_one_minus1 | ue(v) |
| vui_hrd_parameters_present_flag | u(1) |
| if( vui_hrd_parameters_present_flag ) |  |
| hrd_parameters( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| additional_info_present_flag | u(1) |
| if( additional_info_present_flag ) { |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| video_signal_type_present_flag | u(1) |
| if( video_signal_type_present_flag ) { |  |
| video_format | u(3) |
| video_full_range_flag | u(1) |
| } |  |
| neutral_chroma_indication_flag | u(1) |
| default_display_window_flag | u(1) |
| if( default_display_window_flag ) { |  |
| def_disp_win_left_offset | ue(v) |
| def_disp_win_right_offset | ue(v) |
| def_disp_win_top_offset | ue(v) |
| def_disp_win_bottom_offset | ue(v) |
| } |  |
| // Other non-essential video information such as, for example, restriction flags |  |
| } |  |
| } |  |

In terms of semantics, it is proposed to keep the exact same text as in HEVC Annex E for all syntax elements that are listed in the VUI above. There are two syntax elements that are new in the syntax presented above: chroma_sample_loc_type for progressive content and additional_info_present_flag. chroma_sample_loc_type should be understood as having the exact same semantics as chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field.

additional_info_present_flag equal to 1 specifies that the VUI syntax structure contains more syntax elements about the video content that are not essential for proper rendering of the video content. additional_info_present_flag equal to 0 specifies that the VUI syntax structure does not contain any more syntax elements.

# Conclusion

The VUI syntax structure of AVC and HEVC contains some essential information for rendering a video properly on a display and some information that is less essential although potentially interesting for post-processing and rendering. This proposal suggests:

- To signal essential display information somewhere within the video elementary stream (although it does not have to be described within the VVC specification itself)

- That this essential information contains only the following type of information:

  - Sample aspect ratio

  - Colour description

  - Chroma location

  - Fields coding or frame coding for interlaced content

  - Timing information

- That additional information about the content should be signalled separately from the above-mentioned essential information (either in a different syntax structure, or at the end of the same syntax structure after the essential information).

# References

- ETSI TS 101 154 “Digital Video Broadcasting (DVB); Specification for the use of Video and Audio Coding in Broadcast and Broadband Applications” https://www.etsi.org/deliver/etsi_ts/101100_101199/101154/02.05.01_60/ts_101154v020501p.pdf

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).