| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, Switzerland, 19–27 March 2019 | Document: JVET-N0071-v2 |
| --- | --- |

| Title: | AHG12: Unified signalling of tile grouping information |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a unified signalling for rectangular tile grouping and raster scan tile grouping. The proposed text changes simplify the syntax, semantics, and derivations of variables. The related editorial fixes are also made in the proposed text change.

# Introduction

In JVET-M1001-v6 [1], the rectangular tile grouping support is included as highlighted.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag ) |  |
| num_tiles_in_tile_group_minus1 | ue(v) |
| tile_group_type | ue(v) |
| tile_group_pic_order_cnt_lsb | u(v) |
| … |  |
| if( NumTilesInCurrTileGroup > 1 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumTilesInCurrTileGroup − 1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

General tile group data syntax

| tile_group_data( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < NumTilesInCurrTileGroup; i++, tileIdx++ ) { |  |
| ctbAddrInTs = FirstCtbAddrTs[ TgTileIdx[ i ] ] |  |
| for( j = 0; j < NumCtusInTile[ TgTileIdx[ i ] ]; j++, ctbAddrInTs++ ) { |  |
| if( ( j % ColWidth[ TgTileIdx[ i ] ] ) = = 0 ) { |  |
| NumHmvpCand = 0 |  |
| NumHmvpIbcCand = 0 |  |
| } |  |
| CtbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| } |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < NumTilesInCurrTileGroup − 1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |

# Proposed method

For applications like subpicture extraction and merging, it is desired that the required changes relating to the tile groups (tile groups may form a subpicture) appear as much as possible at the higher level, i.e., the picture parameter set and the least changes remain at the tile group level if necessary. Thus this contribution proposes a unified signalling for rectangular tile grouping and raster scan tile grouping. The proposed text changes simplify the syntax, semantics, and derivations of variables. The related editorial changes are also made in the proposed text change. All changes are also highlighted in Cyan.

Proposal 1

remove the condition to allow a rectangular tile group is positioned as specified including one tile group in a picture. Currently it is assumed always at (0, 0) position.

| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| --- | --- |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |

Proposal 2

single_tile_per_tile_group equal to 1 specifies that each tile group that refers to this PPS includes one tile. single_tile_per_tile_group equal to 0 specifies that a tile group that refers to this PPS may include more than one tile. When single_tile_in_pic_flag is equal to 1, single_tile_per_tile_group is inferred to be equal to 1.

Proposal 3

Editorial fixes such as

signalled_tile_group_indexid_flag;

tile_group_id[ i ] specifies the tile group ID of the i-th tile group. The length of the tile_group_id[ i ] syntax element is signaled_tile_group set_id_length_minus1 + 1 bits.  When not present, tile_group_id[ i ] is inferred to be equal to i, for each i in the range of 0 to num_tile_groups_in_pic_minus1, inclusive.

Proposal 4

Unifying signalling for rectangular tile grouping and raster scan tile grouping.

| single_tile_per_tile_group_flag | u(1) |
| --- | --- |
| if( !single_tile_per_tile_group_flag ) { |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| if( signalled_tile_group_id_flag ) { |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| pps_extension_flag | u(1) |

| tile_group_pic_parameter_set_id | ue(v) |
| --- | --- |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag ) |  |
| num_tiles_in_tile_group_minus1 | ue(v) |
| tile_group_type | ue(v) |

The aggregated changes are suggested as follows.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag ) { |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| if( signalled_tile_group_id_flag ) { |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag ) |  |
| num_tiles_in_tile_group_minus1 | ue(v) |
| tile_group_type | ue(v) |
| tile_group_pic_order_cnt_lsb | u(v) |
| … |  |
| if( NumTilesInCurrTileGroup > 1 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumTilesInCurrTileGroup − 1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

The semantics for modified elements shown below are also highlighted in Cyan.

#### Picture parameter set RBSP semantics

…

single_tile_per_tile_group equal to 1 specifies that each tile group that refers to this PPS includes one tile. single_tile_per_tile_group equal to 0 specifies that a tile group that refers to this PPS may include more than one tile. When single_tile_in_pic_flag is equal to 1, single_tile_per_tile_group is inferred to be equal to 1.

rect_tile_group_flag equal to 0 specifies that tiles within each tile group are in raster scan orderand the tile group information is not signalled in PPS. rect_tile_group_flag equal to 1 specifies that tiles within each tile group cover a rectangular region of the picture and the tile group information is signalled in the PPS. When single_tile_per_tile_group_flag is equal to 1 rect_tile_group_flag is inferred to be equal to 1.

num_tile_groups_in_pic_minus1 plus 1 specifies the number of tile groups in each picture referring to the PPS. The value of num_tile_groups_in_pic_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive. When not present and single_tile_per_tile_group_flag is equal to 1, the value of num_tile_groups_in_pic_minus1 is inferred to be equal to NumTilesInPic − 1.

top_left_tile_idx[ i ] specifies the tile index of the tile located at the top-left position corner of the i-th tile group in tile scan order. The value of top_left_tile_idx[ i ] shall not be equal to the value of top_left_tile_idx[ j ] for any i not equal to j. When not present, the value of top_left_tile_idx[ i ] is inferred to be equal to i. The length of the top_left_tile_idx[ i ] syntax element is Ceil( Log2(  NumTilesInPic ) bits.

bottom_right_tile_idx[ i ] specifies the tile index of the tile located at the bottom-right position corner of the i-th tile group in tile scan order. When single_tile_per_tile_group_flag is equal to 1 bottom_right_tile_idx[ i ] is inferred to be equal to top_left_tile_idx[ i ]. The length of the bottom_right_tile_idx[ i ] syntax element is Ceil( Log2( NumTilesInPic) ) bits.

A tile with the tile index not in the range of top_left_tile_idx[ i ] and bottom_right_tile_idx[ i ] is considered not coded. When rect_tile_group_flag equal to 1, top_left_tile_idx[ i ] specifies the tile index of the tile located at the top-left corner of the i-th rectangular tile group, and bottom_right_tile_idx[ i ] specifies the tile index of the tile located at the bottom-right corner of the i-th rectangular tile group. When rect_tile_group_flag equal to 0,   top_left_tile_idx[ i ] specifies the tile index of the left-most tile in the top row of the i-th tile group, and bottom_right_tile_idx[ i ] specifies the tile index of the right-most tile in the bottom row of the i-th tile group.

It is a requirement of bitstream conformance that any particular tile shall only be included in one tile group.

The variable NumTilesInTileGroup[ i ], which specifies the number of tiles in the i-th tile group, and related variables, are derived as follows:

if (rect_tile_group_flag){

deltaTileIdx = bottom_right_tile_idx[ i ] – top_left_tile_idx[ i ]
NumTileRowsInTileGroupMinus1[ i ] = deltaTileIdx / ( num_tile_columns_minus1 + 1 )
NumTileColumnsInTileGroupMinus1[ i ] = deltaTileIdx % ( num_tile_columns_minus1 + 1 )	(7-33)
NumTilesInTileGroup[ i ] = ( NumTileRowsInTileGroupMinus1[ i ] + 1 ) *
	  ( NumTileColumnsInTileGroupMinus1[ i ] + 1 )

}else

NumTilesInTileGroup[ i ] = bottom_right_tile_idx[ i ] – top_right_tile_idx[ i ] + 1

signalled_tile_group_id_flag equal to 1 specifies that the tile group ID for each tile group is signalled. signalled_tile_group_indexid_flag equal to 0 specifies that tile group IDs are not signalled. When rect_tile_group_flag is equal to 0, the value of signalled_tile_group_index_flag is inferred to be equal to 0.

signalled_tile_group_id_length_minus1 plus 1 specifies the number of bits used to represent the syntax element tile_group_id[ i ] when present, and the syntax element tile_group_address in tile group headers. The value of signalled_tile_group_indexid_length_minus1 shall be in the range of 0 to 15, inclusive. When not present, the value of signalled_tile_group_indexid_length_minus1 is inferred to be equal to Ceil( Log2( num_tile_groups_in_pic_minus1 + 1 ) ) − 1.

tile_group_id[ i ] specifies the tile group ID of the i-th tile group. The length of the tile_group_id[ i ] syntax element is signaled_tile_group set_id_length_minus1 + 1 bits.  When not present, tile_group_id[ i ] is inferred to be equal to i, for each i in the range of 0 to num_tile_groups_in_pic_minus1, inclusive.

#### General tile group header semantics

…

tile_group_address specifies the tile address of the first tile in the tile group. When not present, the value of tile_group_address is inferred to be equal to 0.

If rect_tile_group_flag is equal to 0, the following applies:

- The tile address is the tile ID as specified by Equation 6-7.

- The length of tile_group_address is Ceil( Log2 ( NumTilesInPic ) ) bits.

- The value of tile_group_address shall be in the range of 0 to NumTilesInPic − 1, inclusive.

Otherwise (rect_tile_group_flag is equal to 1), the following applies:

- The tile address is the tile group ID of the tile group.

- The length of tile_group_address is signalled_tile_group_idindex_length_minus1 + 1 bits.

- If signalled_tile_group_id_flag is equal to 0, the value of tile_group_address shall be in the range of 0 to num_tile_groups_in_pic_minus1, inclusive. Otherwise, the value of tile_group_address shall be in the range of 0 to 2( signalled_tile_group_id index_length_minus1 + 1 ) − 1, inclusive.

It is a requirement of bitstream conformance that the following constraints apply:

- The value of tile_group_address shall not be equal to the value of tile_group_address of any other coded tile group NAL unit of the same coded picture.

- The tile groups of a picture shall be in increasing order of their tile_group_address values.

- The shapes of the tile groups of a picture shall be such that each tile, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded tile(s).

num_tiles_in_tile_group_minus1, when present, specifies the number of tiles in the tile group minus 1. The value of num_tiles_in_tile_group_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive. When not present, the value of num_tiles_in_tile_group_minus1 is inferred to be equal to 0.

The variable NumTilesInCurrTileGroup, which specifies the number of tiles in the current tile group, and TgTileIdx[ i ], which specifies the tile index of the i-th tile in the current tile group, are derived as follows:

if( rect_tile_group_flag ) {
	tileGroupIdx = 0
	while( tile_group_address  !=  rect_tile_group_id[ tileGroupIdx ] )
		 tileGroupIdx++
	NumTilesInCurrTileGroup = NumTilesInTileGroup[ tileGroupIdx ]
if( rect_tile_group_flag ) {

tileIdx = top_left_tile_idx[ tileGroupIdx ]
	for( j = 0, tIdx = 0; j < NumTileRowsInTileGroupMinus1[ tileGroupIdx ] + 1;	(7-35)
					j++, tileIdx  +=  num_tile_columns_minus1 + 1 )
		for( i = 0, currTileIdx = tileIdx; i < NumTileColumnsInTileGroupMinus1[ tileGroupIdx ] + 1;
						i++, currTileIdx++, tIdx++ )
			TgTileIdx[ tIdx ] = currTileIdx
} else {
	NumTilesInCurrTileGroup = num_tiles_in_tile_group_minus1 + 1
	TgTileIdx[ 0 ] = tile_group_address
	for( i = 1; i < NumTilesInCurrTileGroup; i++ )
		TgTileIdx[ i ] = TgTileIdx[ i − 1 ] + 1
}

...

Some examples of tile group partitioning

In the examples, the top_left_tile_idx[i] and bottom_right_tile_idx[i] are used for unified specification of  both rectangular tile group and tile scan tile group.

- Two rectangular tile groups

- One tile group in the middle of picture

- One tile group with tile scan

# References

- “Versatile Video Coding (Draft 4)”, JVET-M1001-v6, January 2019, Marrakesh, Morocco.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).