| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0093-v2 |
| --- | --- |

| Title: | CE8-related: Disallowing coexistence of regular merge and IBC merge in SMR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Cheng Lin, Chun-Chia Chen, Man-Shu Chiang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yucheng.lin, chunchia.chen, man-shu.chiang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

A shared merging candidate list is applied to regular merge in VTM4.0, and another different shared merging candidate list is applied to intra block copy (IBC) merge in the VTM4.0 fix of Ticket #193. Therefore, in one shared merging candidate list region (SMR), two shared merging candidate lists may have to be generated. This contribution proposes to disallow coexistence of regular merge and IBC merge in the SMR, so only one merging candidate list has to be generated for one SMR. It is reported that proposed method results in negligible BD-rate and runtime changes.

# Introduction

A shared merging candidate list is applied to regular merge in VTM4.0 as proposed in JVET-M0170 [1], and another different shared merging candidate list is applied to intra block copy (IBC) merge in the VTM4.0 fix of Ticket #193 [2]. Based on [1] and [2], in the worst case, there exists two shared merging candidate lists for one SMR of 32 luma samples. As shown in Figure 1, when Prediction Mode A of a leaf coding unit (CU) in the SMR is IBC merge mode and Prediction Mode B of another leaf CU in the SMR is regular merge mode, two shared merging candidate lists for the SMR, instead of one, have to be generated, which is not desirable.

Prediction Mode A

Prediction Mode B

Shared Merging Candidate List Region (SMR)

Prediction Mode C

Figure 1.  Illustration of SMR

# Proposed method

The purpose is to generate up to only one shared merging candidate list in one SMR. For leaf CUs in one SMR, coexistence of regular merge mode and IBC merge mode are disallowed:

- When a leaf CU in the SMR is coded with regular merge mode, all other leaf CUs in the SMR shall not be coded with IBC merge mode.

- When a leaf CU in the SMR is coded with IBC merge mode, all other leaf CUs in the SMR shall not be coded with regular merge mode.

The proposed method is currently implemented as an encoder-side constraint. It can be easily converted to a constraint on both encoder and decoder sides to save syntax elements if the JVET group prefers this way.

# Experimental results

The proposed method has been implemented on top of VTM4.0 [3]. All the tests are conducted according to common test conditions (CTCs) [4] except that the anchor enables IBC. Additional tests for class F and TGM sequences are also conducted, and HashME is enabled for IBC when testing these sequences. Simulation results of the VTM4.0 fix of Ticket #193 are shown in Table 1, where the anchor is VTM4.0 + IBC and the test is anchor + fix of Ticket #193. Simulation results of proposed method are shown in Table 2, where the anchor is VTM4.0 + IBC and the test is anchor + fix of Ticket #193 + the proposed method. It can be observed that the results in Table 1 and those in Table 2 are very close, meaning the proposed method has negligible impact on coding efficiency and runtime changes. In our proposal, only one merging candidate list has to be generated for one SMR.

Table 1.  Results of the fix of Ticket #193; anchor: VTM4.0 + IBC; test: anchor + fix of Ticket #193

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.01% | 100% | 99% |
| Class A2 | 0.01% | -0.03% | -0.02% | 100% | 93% |
| Class B | 0.00% | 0.01% | -0.08% | 99% | 100% |
| Class C | -0.01% | 0.05% | 0.02% | 100% | 97% |
| Class E | 0.01% | -0.04% | 0.11% | 100% | 100% |
| Overall | 0.00% | 0.00% | -0.01% | 100% | 98% |
| Class D | 0.00% | 0.07% | 0.02% | 101% | 97% |
| Class F | 0.03% | -0.02% | -0.10% | 101% | 101% |
| TGM | 0.05% | 0.01% | 0.01% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 100% | 97% |
| Class A2 | 0.01% | -0.10% | 0.01% | 100% | 100% |
| Class B | 0.02% | -0.23% | -0.14% | 100% | 99% |
| Class C | -0.01% | 0.23% | 0.06% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.04% | -0.03% | 100% | 99% |
| Class D | -0.02% | -0.09% | 0.08% | 100% | 99% |
| Class F | -0.01% | 0.02% | -0.01% | 100% | 99% |
| TGM | -0.04% | 0.01% | -0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.09% | -0.41% | 100% | 102% |
| Class C | 0.00% | -0.08% | 0.03% | 100% | 101% |
| Class E | 0.06% | -0.80% | 0.59% | 99% | 100% |
| Overall | 0.02% | -0.19% | -0.01% | 100% | 101% |
| Class D | -0.05% | 1.11% | 0.20% | 99% | 100% |
| Class F | -0.02% | 0.10% | -0.73% | 100% | 101% |
| TGM | -0.01% | 0.13% | 0.08% | 99% | 102% |

Table 2.  Results of the proposed method; anchor: VTM4.0 + IBC; test: anchor + fix of Ticket #193 + the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.01% | 100% | 98% |
| Class A2 | 0.01% | -0.03% | -0.02% | 100% | 93% |
| Class B | 0.00% | 0.01% | -0.08% | 100% | 99% |
| Class C | -0.01% | 0.05% | 0.02% | 100% | 97% |
| Class E | 0.01% | -0.04% | 0.11% | 101% | 104% |
| Overall | 0.00% | 0.00% | -0.01% | 100% | 98% |
| Class D | 0.00% | 0.07% | 0.02% | 101% | 97% |
| Class F | 0.03% | -0.02% | -0.10% | 100% | 100% |
| TGM | 0.05% | 0.01% | 0.01% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.01% | 100% | 100% |
| Class A2 | 0.01% | -0.14% | 0.02% | 100% | 101% |
| Class B | 0.01% | -0.19% | -0.08% | 100% | 100% |
| Class C | 0.00% | 0.23% | 0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | 0.01% | 100% | 100% |
| Class D | -0.01% | -0.06% | -0.04% | 100% | 99% |
| Class F | -0.02% | 0.00% | -0.02% | 100% | 101% |
| TGM | 0.13% | 0.20% | 0.14% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.11% | 0.06% | 100% | 101% |
| Class C | -0.05% | -0.01% | 0.12% | 100% | 100% |
| Class E | 0.06% | -0.80% | 0.59% | 99% | 100% |
| Overall | -0.01% | -0.16% | 0.22% | 100% | 101% |
| Class D | -0.04% | 1.19% | 0.33% | 99% | 99% |
| Class F | -0.09% | 0.02% | -0.24% | 100% | 101% |
| TGM | 0.11% | 0.17% | 0.14% | 99% | 99% |

# Conclusions

In this contribution, it is proposed to disallow coexistence of regular merge and IBC merge in one SMR. It is reported that proposed method results has negligible impact on coding efficiency and runtime. It is asserted that the complexity is reduced because only one merging candidate list, instead of two, has to be generated for each SMR.

# References

[1] C.-C. Chen, Y.-C. Lin, M.-S. Chiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, and S.-M. Lei, “CE4.3.1: Shared merging candidate list,” Document of Joint Video Experts Team, JVET-M0170.

[2] JVET VVC Trac Ticket #193, available at https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/193

[3] VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-4.0

[4] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).