| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0109-v2 |
| --- | --- |

| Title: | CE12/AHG12: Treating boundaries of independent tile groups as picture boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seungwook Hong Jianle Chen Ye-Kui Wang 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | fnu.hendry@huawei.com seungwook.hong@huawei.com jianle.chen@huawei.com yekui.wang@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution describes test results for CE12 test 1. The test investigates the effect of treating MCTS boundaries as picture boundaries to compression performance.

The performance of test 1 is reported against the agreed common anchor (as described in JVET-M0870 with some changes communicated and agreed by CE participants via mailing list) for test 1 and test 2 in CE12. Comparison between test 1 and test 2 is reported in CE12 summary report.

The proposal is summarized as follows:

- A flag is added to the PPS for each rectangular tile group, to specify whether the tile group is a motion-constrained tile group.

- When a tile group is indicated as a motion-constrained tile group, its boundaries are treated same as picture boundaries in the decoding process excluding in-loop filtering operations.

Text changes are provided in the Annex of this contribution.

JVET-N0107 on sub-pictures has an overlap with this proposal. If JVET-N0107 is adopted, then this proposal would be essentially covered and no action is needed for this proposal.

# Introduction

At the 13th JVET meeting in Marrakech, it was agreed to investigate whether treating MCTC boundaries as picture boundaries would provide coding performance improvement. The CE was designed to have two tests to study the impact of treating boundaries of MCTS based independently coded regions to the compression performance of VVC. The tests are summarized as follows:

- Test 1: treating all tile group boundaries as picture boundaries, including padding at tile group boundaries and allowing motion vectors to point to samples outside the tile group boundaries in the reference pictures.

- Test 2: encoder-side restrictions of tile groups to not use samples or motion vector candidates from outside the tile groups.

This contribution provide simulation results for test 1 as well as our proposal.

# Simulation results from CE12-1 and observations

The test was designed to test the proposal in JVET-M0136, which is summarized as follows:

- High-level syntax to specify whether tile group boundaries of the tile group is treated the same as picture boundaries in the decoding process. For example, a flag, tile_group_boundary_treated_as_pic_boundary_flag, could be added to the tile group header.

- Each tile group is an independently decodable region and contains one tile.

- When the boundaries of a tile group are indicated to be treated as picture boundaries, the following applies:

- In the derivation process for temporal luma motion vector prediction, the right and bottom picture boundary positions used in the process, indicated by pic_height_in_luma_samples − 1 and pic_width_in_luma_samples − 1, respectively, are replaced with the right and the bottom boundary positions, respectively, of the tile group, in units of luma samples.

- In the luma sample interpolation process, the left, right, top, and bottom picture boundary positions used in the process, indicated by 0, pic_height_in_luma_samples − 1, 0, pic_width_in_luma_samples − 1, respectively, are replaced with the left, right, top, and bottom boundary positions, respectively, of the tile group, in units of luma samples, respectively.

- In the chroma sample interpolation process, the left, right, top, and bottom picture boundary positions used in the process, indicated by 0, pic_height_in_luma_samples / SubWidthC − 1, 0, pic_width_in_luma_samples / SubWidthC − 1, respectively, are replaced with the left, right, top, and bottom boundary positions, respectively, of the tile group, in units of chroma samples, respectively.

For the simulation, the following procedure was agreed and described in JVET-M0870:

- For a given input 360° video, conversion is applied first to convert source projection format into CMP format.

- The resulting CMP format YUV files will be used as input YUVs for running the anchor encoding / decoding.

- For sub-picture boundary padding evaluation, the sub-picture YUV regions will be cropped from CMP format YUV files and the resulting cropped YUV files will be used as input YUVs for sub-picture evaluation.

For reporting the sub-picture evaluation results, the following procedure to metric computation was agreed:

- JVET CTC is used to for evaluation of coding methods.

- PSNR (Y, Cb, Cr) metric between the input and output of the encoder is used as distortion metric.

- For computation of bitrate for sub-picture configuration, the extra bitrate from having multiple bitstreams is accounted. Extra parameter set data will be deducted from the total bitrate. Tile entry point and configuration info shall be included in the total bitrate.

- When using separate encoders to encode sub-pictures, the average distortion per picture shall be calculated in MSE domain before averaging the resulting PSNR values over pictures of a sequence.

Table 1 and Table 2 tabulate test 1 results against anchor results for the 24-MCTS and 96-MCTS cases, respectively.

Table 1 Performance of test 1 against anchor – RA configuration with 24 MCTSs

| Test sequence | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| ChairLift | 3.79% | 5.18% | 4.87% |
| GasLamp | 2.15% | 1.40% | 1.76% |
| Harbor | 1.04% | 1.20% | 0.41% |
| KiteFlite | 0.51% | 1.10% | 1.09% |
| SkateboardInLot | 7.79% | 9.88% | 10.73% |
| Trolley | 0.54% | 0.68% | 1.26% |
| Average | 2.64% | 3.24% | 3.35% |

Table 2 Performance of test 1 against anchor – RA configuration with 96 MCTSs

| Test sequence | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| ChairLift | 15.82% | 19.84% | 18.53% |
| GasLamp | 10.41% | 10.13% | 10.42% |
| Harbor | 7.52% | 8.32% | 7.99% |
| KiteFlite | 3.02% | 4.27% | 4.29% |
| SkateboardInLot | 19.30% | 24.14% | 27.19% |
| Trolley | 3.93% | 4.70% | 5.61% |
| Average | 10.00% | 11.90% | 12.34% |

# Proposal

We propose that MCTS boundaries are treated as picture boundaries, as follows:

- A flag is added to the PPS for each rectangular tile group, to specify whether the tile group is a motion-constrained tile group. When a tile group is indicated as a motion-constrained tile group, its boundaries are treated same as picture boundaries in the decoding process excluding in-loop filtering operations.

- When it is indicated that the boundaries of a tile group are treated as picture boundaries, the following applies:

- In the derivation process for temporal luma motion vector prediction, the right and bottom picture boundary positions used in the process, indicated by pic_height_in_luma_samples − 1 and pic_width_in_luma_samples − 1, respectively, are replaced with the right and the bottom boundary positions, respectively, of the tile or slice, in units of luma samples.

- In the luma sample interpolation process, the left, right, top, and bottom picture boundary positions used in the process, indicated by 0, pic_height_in_luma_samples − 1, 0, pic_width_in_luma_samples − 1, respectively, are replaced with the left, right, top, and bottom boundary positions, respectively, of the tile or slice, in units of luma samples, respectively.

- In the chroma sample interpolation process, the left, right, top, and bottom picture boundary positions used in the process, indicated by 0, pic_height_in_luma_samples / SubWidthC − 1, 0, pic_width_in_luma_samples / SubWidthC − 1, respectively, are replaced with the left, right, top, and bottom boundary positions, respectively, of the tile or slice, in units of chroma samples, respectively.

The detailed text changes for the proposal are provided in the Annex of this contribution.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex

### Picture parameter set RBSP syntax and semantics

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| ... |  |
| single_tile_per_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| if( rect_tile_group_flag ) |  |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| motion_constrained_tile_group_flag[ i ] | u(1) |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| ... |  |
| } |  |

motion_constrained_tile_group_flag[ i ] equal to 1 specifies that the i-th rectangular tile group is a motion constrained tile group. motion_constrained_tile_group_flag[ i ] equal to 0 specifies that the i-th rectangular tile group is not a motion constrained tile group.

When a rectangular tile gruop is a motion constrained tile group, its boundary ise treated the same as picture boundary in the decoding process described in clause 8 for pictures referring to the PPS.

### General tile group data semantics

end_of_tile_one_bit shall be equal to 1.

For each tile, the variables LeftBoundaryPos, TopBoundaryPos, RightBoundaryPos and BotBoundaryPos are derived as follows:

–	if motion_constrained_tile_group_flag is equal to TRUE, the following applies:

tileGroupIdx = 0
while( tile_group_address  !=  rect_tile_group_id[ tileGroupIdx ] )
		tileGroupIdx++

tgFirstTileColIdx = top_left_tile_idx[ tileGroupIdx ] % ( num_tile_columns_minus1 + 1 )	(7-xx)

tgFirstTileRowIdx = top_left_tile_idx[ tileGroupIdx ] / ( num_tile_columns_minus1 + 1 )	(7-xx)

tgLastTileColIdx = bottom_right_tile_idx[ tileGroupIdx ] % ( num_tile_columns_minus1 + 1 )	(7-xx)

tgLastTileRowIdx = bottom_right_tile_idx[ tileGroupIdx ] / ( num_tile_columns_minus1 + 1 )	(7-xx)

LeftBoundaryPos = ColBd[ tgFirstTileColIdx ]  <<  CtbLog2SizeY	(7-xx)

RightBoundaryPos = ( ( ColBd[ tgLastTileColIdx ] + ColWidth[ tgLastTileColIdx ] )  <<
							CtbLog2SizeY ) − 1		(7-xx)

TopBoundaryPos = RowBd[ tgFirstTileRowIdx ]  <<  CtbLog2SizeY	(7-xx)

BotBoundaryPos = ( ( RowBd[ tgLastTileRowIdx ] + RowHeight[ tgLastTileRowIdx ] )  <<
							CtbLog2SizeY ) −1		(7-xx)

–	Otherwise (i.e., motion_constrained_tile_group_flag is equal to FALSE), the following applies:

LeftBoundaryPos = 0		(7-xx)

RightBoundaryPos = pic_width_in_luma_samples − 1	(7-xx)

TopBoundaryPos = 0		(7-xx)

BotBoundaryPos = pic_height_in_luma_samples − 1	(7-xx)

### 8.3.2.11 Derivation process for temporal luma motion vector prediction

...

- If yCb  >>  CtbLog2SizeY is equal to yColBr  >>  CtbLog2SizeY, yColBr is less than pic_height_in_luma_samples or equal to BotBoundaryPos and xColBr is less than pic_width_in_luma_samples or equal to RightBoundaryPos, the following applies:

- The variable colCb specifies the luma coding block covering the modified location given by ( ( xColBr  >>  3 )  <<  3, ( yColBr  >>  3 )  <<  3 ) inside the collocated picture specified by ColPic.

- The luma location ( xColCb, yColCb ) is set equal to the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic.

- The derivation process for collocated motion vectors as specified in clause 8.3.2.9 is invoked with currCb, colCb, ( xColCb, yColCb ), refIdxLX , and control parameter controlParaFlag set equal to 0 as inputs, and the output is assigned to mvLXCol and availableFlagLXCol.

...

8.3.6.3.2	Luma sample 8-tap interpolation filtering process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL.

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

–	The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 8-9.

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

–	If motion_constrained_tile_group_flag is equal to 1, the following applies:

xInti = Clip3( LeftBoundaryPos, RightBoundaryPos, xIntL + i − 3 )	(8-673)

yInti = Clip3( TopBoundaryPos, BotBoundaryPos, yIntL + i − 3 )	(8-674)

–	Otherwise (i.e., motion_constrained_tile_group_flag is equal to 0), the following applies:

xInti = sps_ref_wraparound_enabled_flag  ?	(8-673)
	ClipH( sps_ref_wraparound_offset, picW, ( xIntL + i − 3 ) ):  Clip3( 0, picW − 1, xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )		(8-674)

The predicted luma sample value predSampleLXL is derived as follows:

–	If both xFracLand yFracL are equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt3 ][ yInt3 ] << shift3	(8-675)

–	Otherwise, if xFracL is not equal to 0 and yFracL is equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fL[ xFracL ][ i ] * refPicLXL[ xInti ][ yInt3 ]   >>  shift1	(8-676)

–	Otherwise, if xFracL is equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fL[ yFracL ][ i ] * refPicLXL[ xInt3 ][ yInti ]   >>  shift1	(8-677)

–	Otherwise, if xFracL is not equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

- The sample array temp[ n ] with n = 0..7, is derived as follows:

temp[ n ] =  i=07fL[ xFracL ][ i ] * refPicLXL[ xInti ][ yIntn ]   >>  shift1	(8-678)

- The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL =  i=07fL[ yFracL ][ i ] * temp[ i ]   >>  shift2	(8-679)

...

### 8.3.6.3.3	Luma sample bilinear interpolation filtering process

Inputs to this process are:

- a luma location in full-sample units ( xIntL, yIntL ),

- a luma location in fractional-sample units ( xFracL, yFracL ),

- the luma reference sample array refPicLXL,

Output of this process is a predicted luma sample value predSampleLXL

The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 8-10.

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If motion_constrained_tile_group_flag is equal to 1, the following applies:

xInti = Clip3( LeftBoundaryPos, RightBoundaryPos, xIntL + i )	(8-673)

yInti = Clip3( TopBoundaryPos, BotBoundaryPos, yIntL + i )	(8-674)

–	Otherwise (i.e., motion_constrained_tile_group_flag is equal to 0), the following applies:

xInti = sps_ref_wraparound_enabled_flag  ? 		(8-680)
	ClipH( sps_ref_wraparound_offset, picW, ( xIntL + i − 3 ) ) : Clip3( 0, picW − 1, xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )		(8-681)

...

### 8.3.6.3.4	Chroma sample interpolation process

##### Chroma sample interpolation process

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in 1/32 fractional-sample units ( xFracC, yFracC ),

–	the chroma reference sample array refPicLXC.

Output of this process is a predicted chroma sample value predSampleLXC

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthC − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthC ).

–	The variable picWC is set equal to pic_width_in_luma_samples / SubWidthC and the variable picHC is set equal to pic_height_in_luma_samples / SubHeightC.

The luma interpolation filter coefficients fC[ p ] for each 1/32 fractional sample position p equal to xFracC or  yFracC are specified in Table 8-11.

The variable xOffset is set equal to sps_ref_wraparound_offset / SubWidthC.

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

–	If motion_constrained_tile_group_flag is equal to 1, the following applies:

xInti = Clip3( LeftBoundaryPos/SubWidthC, RightBoundaryPos/SubWidthC, xIntL + i )	(8-687)

yInti = Clip3( TopBoundaryPos/SubHeightC, BotBoundaryPos/SubHeightC, yIntL + i )	(8-687)

–	Otherwise (i.e., motion_constrained_tile_group_flag is equal to 0), the following applies:

xInti = sps_ref_wraparound_enabled_flag  ?  ClipH( xOffset, picWC, ( xIntC + i − 1 ) )	(8-687)
	:  Clip3( 0, picWC − 1, xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )		(8-688)

...