| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0117-v1 |
| --- | --- |

| Title: | AHG17: Signalling of reshaper parameters in APS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry 10180 Telesis Court San Diego, CA, 92121, USA Jianle Chen 2330 Central Expy Santa Clara, CA 95050, USA Peng Yin, Taoran Lu, Fangjun Pu, Sean McCarthy 432 Lakeside Drive, Sunnyvale, CA 94085, USA | Tel: Email: | +1-908-803-3888 yekui.wang@huawei.com +1-858-375-8065 fnu.hendry@huawei.com +1-858-999-4243 jianle.chen@huawei.com pyin@dolby.com |
| Source: | Huawei Technologies Co., Ltd., Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution asserts that the properties of the reshaper parameters can be characterized as follows:

- The size of a set of reshaper parameters contained in the tile_group_reshaper_model( ) syntax structure is typically around 60 to 100 bits for video coded with a 10-bit internal bit depth.

- The reshaper model is typically updated by the encoder about once per second.

- Within a bitstream, many sets of reshaper parameters may be applied.

Based on the above asserted properties of the reshaper parameters, it is asserted that signalling of the reshaper parameters in the APS is more desirable than in any of the following: SPS; PPS; picture header (if it exists); and tile group header, and therefore it is proposed to signal the reshaper parameters in the APS.

In addition, to avoid the partial-update problem, it is also proposed that an APS NAL unit shall not contain more than one type of parameters; i.e., either ALF parameters or reshaper parameters, never both.

# Introduction

The reshaper feature was adopted from JVET-M0427-v3. The reshaper syntax and semantics in JVET-M0427-v3 are as follows (additions and changed parts are highlighted in yellow):

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_reshaper_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

sps_reshaper_enabled_flag equal to 1 specifies that reshaper is used in the coded video sequence (CVS). sps_reshaper_enabled_flag equal to 0 specifies that reshaper is not used in the CVS.

| tile_group_header( ) { | Descriptor |
| --- | --- |
| … | . |
| if( sps_reshaper_enabled_flag) { |  |
| tile_group_reshaper_model_present_flag | u(1) |
| if(tile_group_reshaper_model_present_flag ) |  |
| tile_group_reshaper_model ( ) |  |
| tile_group_reshaper_enable_flag | u(1) |
| if( tile_group_reshaper_enable_flag && (!( qtbtt_dual_tree_intra_flag && tile_group_type == I ) ) ) |  |
| tile_group_reshaper_chroma_residual_scale_flag | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

tile_group_reshaper_model_present_flag equal to 1 specifies tile_group_reshaper_model() is present in tile group header. tile_group_reshaper_model_present_flag equal to 0 specifies tile_group_reshaper_model() is not present in tile group header. When tile_group_reshaper_model_present_flag is not present, it is inferred to be equal to 0.

tile_group_reshaper_enabled_flag equal to 1 specifies that reshaper is enabled for the current tile group. tile_group_reshaper_enabled_flag equal to 0 specifies that reshaper is not enabled for the current tile group. When tile_group_reshaper_enable_flag is not present, it is inferred to be equal to 0.

tile_group_reshaper_chroma_residual_scale_flag equal to 1 specifies that chroma residual scaling is enabled for the current tile group. tile_group_reshaper_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling is not enabled for the current tile group. When tile_group_reshaper_chroma_residual_scale_flag is not present, it is inferred to be equal to 0.

| tile_group_reshaper_model () { | Descriptor |
| --- | --- |
| reshaper_model_min_bin_idx | ue(v) |
| reshaper_model_delta_max_bin_idx | ue(v) |
| reshaper_model_bin_delta_abs_cw_prec_minus1 | ue(v) |
| for ( i = reshaper_model_min_bin_idx; i <= reshaper_model_max_bin_idx; i++) { |  |
| reshaper_model_bin_delta_abs_CW[ i ] | u(v) |
| if( reshaper_model_bin_delta_abs_CW[ i ] ) > 0 ) |  |
| reshaper_model_bin_delta_sign_CW_flag[ i ] | u(1) |
| } |  |
| } |  |

reshaper_model_min_bin_idx specifies the minimum bin (or piece) index to be used in the reshaper construction process. The value of reshaper_model_min_bin_idx shall be in the range of 0 to MaxBinIdx, inclusive. The value of MaxBinIdx shall be equal to 15.

reshaper_model_delta_max_bin_idx specifies the maximum allowed bin (or piece) index MaxBinIdx minus the maximum bin index to be used in the reshaper construction process. The value of reshaper_model_max_bin_idx is set equal to MaxBinIdx – reshaper_model_delta_max_bin_idx.

reshaper_model_bin_delta_abs_cw_prec_minus1 plus 1 specifies the number of bits used for the representation of the syntax element reshaper_model_bin_delta_abs_CW[ i ].

reshaper_model_bin_delta_abs_CW[ i ] specifies the absolute delta codeword value for the i-th bin.

reshaper_model_bin_delta_sign_CW_flag[ i ] specifies the sign of reshaper_model_bin_delta_abs_CW[ i ] as follows:

–	If reshaper_model_bin_delta_sign_CW_flag[ i ] is equal to 0, the corresponding variable RspDeltaCW[ i ] is a positive value.

–	Otherwise (reshaper_model_bin_delta_sign_CW_flag[ i ] is not equal to 0), the corresponding variable RspDeltaCW[ i ] is a negative value.

When reshaper_model_bin_delta_sign_CW_flag[ i ] is not present, it is inferred to be equal to 0.

The variable RspDeltaCW[ i ] is set equal to (1 − 2*reshaper_model_bin_delta_sign_CW[ i ]) * reshaper_model_bin_delta_abs_CW[ i ].

The variable RspCW[ i ] is derived as follows:

The variable OrgCW is set equal to (1 << BitDepthY ) / ( MaxBinIdx + 1).

–	If reshaper_model_min_bin_idx < = i <= reshaper_model_max_bin_idx

RspCW[ i ] = OrgCW + RspDeltaCW[ i ].

–	Otherwise, RspCW[ i ] = 0.

The value of RspCW [ i ] shall be in the range of 32 to 2 * OrgCW − 1 if the value of BitDepthY is equal to 10.

The variables InputPivot[ i ] with i in the range of 0 to MaxBinIdx + 1, inclusive are derived as follows:

InputPivot[ i ] = i * OrgCW

The variable ReshapePivot[ i ] with i in the range of 0 to MaxBinIdx + 1, inclusive, the variable ScaleCoef[ i ] and InvScaleCoeff[ i ]with i in the range of 0 to MaxBinIdx , inclusive, are derived as follows:

shiftY = 14
ReshapePivot[ 0 ] = 0;

for( i = 0; i <= MaxBinIdx ; i++) {
	ReshapePivot[ i + 1 ] = ReshapePivot[ i ] + RspCW[ i ]

ScaleCoef[ i ] = ( RspCW[ i ] * (1 << shiftY) + (1 << (Log2(OrgCW) - 1))) >> (Log2(OrgCW))
	if ( RspCW[ i ] == 0 )
		InvScaleCoeff[ i ] = 0
	else
		InvScaleCoeff[ i ] = OrgCW * (1 << shiftY) / RspCW[ i ]
}

The variable ChromaScaleCoef[ i ] with i in the range of 0 to MaxBinIdx, inclusive, are derived as follows:

ChromaResidualScaleLut[64] = {16384, 16384, 16384, 16384, 16384, 16384, 16384, 8192, 8192, 8192, 8192, 5461, 5461, 5461, 5461, 4096, 4096, 4096, 4096, 3277, 3277, 3277, 3277, 2731, 2731, 2731, 2731, 2341, 2341, 2341, 2048, 2048, 2048, 1820, 1820, 1820, 1638, 1638, 1638, 1638, 1489, 1489, 1489, 1489, 1365, 1365, 1365, 1365, 1260, 1260, 1260, 1260, 1170, 1170, 1170, 1170, 1092, 1092, 1092, 1092, 1024, 1024, 1024, 1024};

shiftC = 11

–	if ( RspCW[ i ] == 0 )

ChromaScaleCoef [ i ] = (1 << shiftC)

–	 Otherwise (RspCW[ i ] != 0),

ChromaScaleCoef[ i ] = ChromaResidualScaleLut[RspCW[ i ] >> 1]

# Discussion

The properties of the reshaper parameters can be characterized as follows:

- The size of a set of reshaper parameters contained in the tile_group_reshaper_model( ) syntax structure is typically around 60 to 100 bits for video coded with a 10-bit internal bit depth.

- The reshaper model is typically updated by the encoder about once per second.

- Within a bitstream, many sets of reshaper parameters may be applied.

Based on the above characteristics of the reshaper parameters, the following can be concluded:

- It is not desirable to signal the reshaper parameters in the SPS because doing so would require generating a new SPS and a new IRAP picture whenever an update to the reshaper model is needed. This is particularly not good for low-delay application environments for which frequent IRAP pictures are not needed. Inclusion of reshaper parameters in the SPS would also preclude out-of-band transmission of SPSs.

- It is also not desirable to signal the reshaper parameters in the PPS because doing so would preclude out-of-band transmission of PPSs.

- It is also not desirable to signal the reshaper parameters in a picture header (if it exists) because a set of reshaper parameters typically applies to a sequence of pictures.

- It is also not desirable to signal the reshaper parameters in the tile group header (as in the current VVC design) because a set of reshaper parameters typically applies to a sequence of pictures.

Given the preceding, the most appropriate place for signalling of the reshaper parameters is in the APS. Therefore, it is proposed to signal the reshaper parameters in the APS.

In the current VVC design, the APS only carries the ALF parameters. Signalling of multiple types of parameters in one APS NAL unit would invoke the notorious partial-update problem. (Recall that the partial-update problem was a headache when designing the APS during HEVC development years ago and that an agreeable technical solution was never found. Instead, the problem became moot when the APS feature was removed from the HEVC draft text.)

To avoid the partial-update problem in VVC, we further propose that an APS NAL unit shall not contain more than one type of parameters; i.e., either ALF parameters or reshaper parameters, never both.

# Proposal

The proposed syntax and semantics for signalling of reshaper parameters in the APS are as follows, where additions and changed parts are highlighted in yellow, while removals are strikethroughs in red fonts):

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_reshaper_enabled_flag | u(1) |
| ... |  |
| } |  |

sps_reshaper_enabled_flag equal to 1 specifies that reshaper is used in the coded video sequence (CVS). sps_reshaper_enabled_flag equal to 0 specifies that reshaper is not used in the CVS.

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_APS ) // 0 |  |
| alf_data( ) |  |
| else if ( aps_params_type = = MAP_APS ) // 1 |  |
| reshaper_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

aps_params_type specifies the type of APS parameters carried in the APS as specified in the following table:

Table 7-x – APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | MAP_APS | In-loop mapping (i.e., reshaper) parameters |
| 2..7 | Reserved | Reserved |

| tile_group_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( sps_alf_enabled_flag ) { |  |
| tile_group_alf_enabled_flag | u(1) |
| if( tile_group_alf_enabled_flag ) |  |
| tile_group_alf_aps_id | u(5) |
| } |  |
| ... |  |
| if( sps_reshaper_enabled_flag) { |  |
| tile_group_reshaper_model_present_flag | u(1) |
| if(tile_group_reshaper_model_present_flag ) |  |
| tile_group_reshaper_model ( ) |  |
| tile_group_reshaper_enable_flag | u(1) |
| if( tile_group_reshaper_enable_flag ) { |  |
| tile_group_reshaper_aps_id | u(5) |
| if( !( qtbtt_dual_tree_intra_flag && tile_group_type = = I ) ) |  |
| if( tile_group_reshaper_enable_flag && ( ! ( qtbtt_dual_tree_intra_flag && tile_group_type = = I ) ) |  |
| tile_group_reshaper_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| ... |  |
| if( NumTilesInCurrTileGroup > 1 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumTilesInCurrTileGroup − 1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

Adding the following definitions to clause 3:

ALF APS: An APS that has aps_params_type equal to ALF_APS.

reshaper APS: An APS that has aps_params_type equal to MAP_APS.

Make the following semantics changes:

...

tile_group_alf_aps_id specifies the adaptation_parameter_set_id of the ALF APS that the tile group refers to. The TemporalId of the ALF APS NAL unit having adaptation_parameter_set_id equal to tile_group_alf_aps_id shall be less than or equal to the TemporalId of the coded tile group NAL unit.

When multiple ALF APSs with the same value of adaptation_parameter_set_id are referred to by two or more tile groups of the same picture, the multiple ALF APSs with the same value of adaptation_parameter_set_id shall have the same content.

...

tile_group_reshaper_model_present_flag equal to 1 specifies tile_group_reshaper_model() is present in tile group header. tile_group_reshaper_model_present_flag equal to 0 specifies tile_group_reshaper_model() is not present in tile group header. When tile_group_reshaper_model_present_flag is not present, it is inferred to be equal to 0.

tile_group_reshaper_enabled_flag equal to 1 specifies that reshaper is enabled for the current tile group. tile_group_reshaper_enabled_flag equal to 0 specifies that reshaper is not enabled for the current tile group. When tile_group_reshaper_enable_flag is not present, it is inferred to be equal to 0.

tile_group_reshaper_aps_id specifies the adaptation_parameter_set_id of the reshaper APS that the tile group refers to. The TemporalId of the reshaper APS NAL unit having adaptation_parameter_set_id equal to tile_group_reshaper_aps_id shall be less than or equal to the TemporalId of the coded tile group NAL unit.

When multiple reshaper APSs with the same value of adaptation_parameter_set_id are referred to by two or more tile groups of the same picture, the multiple reshaper APSs with the same value of adaptation_parameter_set_id shall have the same content.

tile_group_reshaper_chroma_residual_scale_flag equal to 1 specifies that chroma residual scaling is enabled for the current tile group. tile_group_reshaper_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling is not enabled for the current tile group. When tile_group_reshaper_chroma_residual_scale_flag is not present, it is inferred to be equal to 0.

...

| reshaper_data( ) { | Descriptor |
| --- | --- |
| reshaper_model_min_bin_idx | ue(v) |
| reshaper_model_delta_max_bin_idx | ue(v) |
| reshaper_model_bin_delta_abs_cw_prec_minus1 | ue(v) |
| for ( i = reshaper_model_min_bin_idx; i <= reshaper_model_max_bin_idx; i++) { |  |
| reshaper_model_bin_delta_abs_CW[ i ] | u(v) |
| if( reshaper_model_bin_delta_abs_CW[ i ] ) > 0 ) |  |
| reshaper_model_bin_delta_sign_CW_flag[ i ] | u(1) |
| } |  |
| } |  |

reshaper_model_min_bin_idx specifies the minimum bin (or piece) index to be used in the reshaper construction process. The value of reshaper_model_min_bin_idx shall be in the range of 0 to MaxBinIdx, inclusive. The value of MaxBinIdx shall be equal to 15. [Ed. (YK): Probably just replace all instances of "MaxBinIdx" with "15", unless it could have a different value in the standard.]

reshaper_model_delta_max_bin_idx specifies the maximum allowed bin (or piece) index MaxBinIdx minus the maximum bin index to be used in the reshaper construction process. The value of reshaper_model_delta_max_bin_idx shall be in the range of 0 to MaxBinIdx, inclusive.

The variable ReshaperMaxBinIdx is set equal to MaxBinIdx − reshaper_model_delta_max_bin_idx. The value of ReshaperMaxBinIdx shall be greater than or equal to reshaper_model_min_bin_idx.

reshaper_model_bin_delta_abs_cw_prec_minus1 plus 1 specifies the number of bits used for the representation of the syntax element reshaper_model_bin_delta_abs_CW[ i ]. The value of eshaper_model_bin_delta_abs_cw_prec_minus1 shall be in the range of 0 to BitDepthY − 2, inclusive.

reshaper_model_bin_delta_abs_CW[ i ] specifies the absolute delta codeword value for the i-th bin. The reshaper_model_bin_delta_abs_CW[ i ] syntax element is represented by reshaper_model_bin_delta_abs_cw_prec_minus1 + 1 bits.

reshaper_model_bin_delta_sign_CW_flag[ i ] specifies the sign of reshaper_model_bin_delta_abs_CW[ i ] as follows:

...

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. and Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).