| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0141-v1 |
| --- | --- |

| Title: | CE4-1.1: Syntax Change of MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Eiichi Sasaki Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81-50-5213-6277 eiichi.sasaki@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes an alternative syntax structure where the syntax elements of MMVD are signaled when both cu_skip_flag and merge_flag are equal to 0 and CuPredMode is equal to MODE_INTER, i.e. the position before the AMVP and Inter Affine syntax elements. It is reported that BD-rate gain is 0.03% and 0.06% in RA and LB condition, respectively.

Introduction

In the current VVC WD, MMVD [1] syntax elements are signaled when cu_skip_flag is equal to 1 or merge_flag is equal to 1, i.e. the position before the other merge modes. However, in high QP condition, MMVD is less effective than the other merge modes because relatively large number of bins are necessary to express MVD. To improve the coding efficiency in high QP condition without increasing parsing complexity, we propose alternative syntax structure that the syntax elements of MMVD are signaled when both cu_skip_flag and merge_flag are equal to 0 and CuPredMode is equal to MODE_INTER.

Proposed Syntax Structure

The proposed syntax structure is shown in the following.

Table 1. Proposed coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( tile_group_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && tile_group_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( tile_group_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( tile_group_type != I && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) ) && sps_ibc_enabled_flag ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| … |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_data( x0, y0, cbWidth, cbHeight ) |  |
| } else if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| … |  |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( tile_group_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| … | ae(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

Table 2. Proposed merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| ciip_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| ciip_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && tile_group_type = = B && ciip_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight >= 64 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Experimental Results

The proposal is implemented on VTM-4.0 and Table 3 shows the results under CTC[2]. As an additional information, Table 4 shows the results under high QP condition (QPs are 42, 47, 52 and 57).

Table 3. Simulation results under CTC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.08% | 0.14% | 99% | 101% |
| Class A2 | -0.04% | 0.08% | 0.15% | 99% | 100% |
| Class B | -0.01% | 0.01% | 0.03% | 99% | 101% |
| Class C | -0.04% | -0.04% | -0.16% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.01% | 0.02% | 99% | 101% |
| Class D | 0.05% | -0.08% | 0.07% | 100% | 101% |
| Class F | -0.08% | -0.03% | -0.09% | 99% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.14% | -0.32% | 102% | 101% |
| Class C | -0.03% | 0.16% | 0.20% | 101% | 101% |
| Class E | -0.09% | 0.09% | -0.32% | 102% | 101% |
| Overall | -0.06% | 0.02% | -0.15% | 102% | 101% |
| Class D | 0.02% | 0.28% | -0.83% | 101% | 101% |
| Class F | -0.17% | 0.13% | -0.18% | 101% | 100% |

Table 4. Simulation results under high QP condition

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.60% | -0.22% | 99% | 101% |
| Class A2 | -0.14% | -0.07% | -0.11% | 99% | 99% |
| Class B | -0.36% | -0.46% | -0.32% | 100% | 97% |
| Class C | -0.14% | -0.39% | -0.33% | 100% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.39% | -0.26% | 100% | 98% |
| Class D | -0.06% | -0.29% | -0.24% | 100% | 97% |
| Class F | -0.09% | -0.18% | -0.10% | 99% | 96% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.28% | -0.54% | -0.46% | 102% | 94% |
| Class C | -0.02% | -0.84% | -0.22% | 102% | 95% |
| Class E | -0.67% | -1.61% | -1.07% | 102% | 94% |
| Overall | -0.29% | -0.91% | -0.53% | 102% | 94% |
| Class D | 0.07% | -0.23% | -0.21% | 103% | 96% |
| Class F | -0.24% | -0.77% | -2.13% | 102% | 93% |

References

- M. W. Park, Y. Piao, M. Park, K. Choi, “CE4 Ultimate motion vector expression,” JVET-L0054, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).