| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0146-v1 |
| --- | --- |

| Title: | Non-CE9: On conditions for DMVR and BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-43-299-8526 chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

DMVR is a method to improve motion vectors by using bilateral matching of bi-directional prediction and BDOF is a method to improve bi-directional prediction efficiency by using gradient information. Both methods have the assumption of constant brightness. Therefore, those tools may not be useful when weighted prediction tools considering variable brightness apply. In this contribution, some additional conditions for DMVR and BDOF are proposed. Since BDOF has already applied when bi-prediction with CU weights (GBI) selects default weights, the same condition is introduced when DMVR applies. Also, when the luminance flags of weighted coefficients and offsets of WP on the L0 and L1 reference pictures are equal to zero, DMVR and BDOF apply. As an experimental result on CTC, the average of encoding and decoding times became 1% faster. The loss of coding efficiency was 0.00% on average.

Introduction

DMVR (decoder side motion vector refinement) [1] is a method to improve motion vectors by using bilateral matching of bi-directional prediction and since SAD (sum of absolute differences) of the matching cost function is assumed the constant brightness. BDOF (bi-directional optical flow) [2] improves bi-directional prediction efficiency by using gradient information and the derivation of optical flows is assumed the constant brightness. Therefore, when WP (weighted prediction) or bi-prediction with CU weights (GBI) don’t apply, DMVR and BDOF should apply.

In this contribution, when DMVR applies, the following condition is introduced.

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0.

Also, when BDOF applies, the following condition is introduced.

- luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0.

Since the introduction of additional conditions are reduced the processing, the experiment has been shown to reduce the encoding and decoding time with no loss of coding efficiency.

Proposal changes of specification

The working draft [3] should be changed by the followings.

- 8.5	Decoding process for coding units coded in inter prediction mode

- 8.5.1	General decoding process for coding units coded in inter prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

Output of this process is a modified reconstructed picture before in-loop filtering.

The derivation process for quantization parameters as specified in clause 8.7.1 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight, and the variable treeType as inputs.

The decoding process for coding units coded in inter prediction mode consists of the following ordered steps:

- The variable dmvrFlag is set equal to 0.

- The motion vector components and reference indices of the current coding unit are derived as follows:

- If merge_triangle_flag[ xCb ][ yCb ], inter_affine_flag[ xCb ][ yCb ] and merge_subblock_flag[ xCb ][ yCb ] are all equal to 0, the following applies:

- The derivation process for motion vector components and reference indices as specified in clause 8.5.2.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], and the bi-prediction weight index gbiIdx as outputs.

- When all of the following conditions are true, dmvrFlag is set equal to 1:

- sps_dmvr_enabled_flag is equal to 1

- merge_flag[ xCb ][ yCb ] is equal to 1

- both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

- mmvd_flag[ xCb ][ yCb ] is equal to 0

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 64

- 8.7.5	Decoding process for inter blocks

- 8.7.5.1	General

This process is invoked when decoding a coding unit coded in inter prediction mode.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- variables numSbX and numSbY specifying the number of luma coding subblocks in horizontal and vertical direction,

- the motion vectors mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the refined motion vectors refMvL0[ xSbIdx ][ ySbIdx ] and refMvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the bi-prediction weight index gbiIdx,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an array predSamples of prediction samples.

Let predSamplesL0L, predSamplesL1L and predSamplesIntraL be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesL0Cb, predSamplesL1Cb, predSamplesL0Cr and predSamplesL1Cr, predSamplesIntraCb, and predSamplesIntraCr be (cbWidth / 2)x(cbHeight / 2) arrays of predicted chroma sample values.

The width and the height of the current coding sublock subCbWidth, subCbHeight in luma samples are derived as follows:

sbWidth  =  cbWidth / numSbX		(8-793)

sbHeight  =  cbHeight / numSbY		(8-794)

For each coding subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1, the following applies:

- The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

( xSb, ySb )  =  ( xCb + xSbIdx * sbWidth, yCb + ySbIdx * sbHeight )	(8-795)

- The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

- If all of the following conditions are true, bdofFlag is set equal to TRUE.

- sps_bdof_enabled_flag is equal to 1.

- predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

- MotionModelIdc[ xCb ][ yCb ] is equal to 0.

- merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0.

- cIdx is equal to 0.

Otherwise, bdofFlag is set equal to FALSE.

Experimental results

The proposed method has been implemented on VTM-4.0 software. The simulation was based on VTM configuration JVET-M1010 [4]. The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

As summary of experimental results, the average of encoding time became 1% faster and the average of decoding time became 1% faster. There was no loss on average.

Table 1  Summary of experimental results on CTC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.06% | 99% | 99% |
| Class A2 | 0.02% | 0.10% | 0.10% | 99% | 100% |
| Class B | -0.01% | 0.03% | -0.01% | 99% | 99% |
| Class C | 0.00% | -0.01% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | 0.02% | 99% | 99% |
| Class D | 0.02% | -0.09% | -0.08% | 100% | 100% |
| Class F | -0.10% | -0.07% | -0.08% | 99% | 100% |

Conclusion

In this contribution, when weighted prediction tools apply, some additional conditions for DMVR and BDOF are introduced. Since additional conditions reduce the processing, the average of encoding and decoding times became 1% faster and there is no loss of coding efficiency on average. Finally, we would like to recommend that the working draft and reference software should support this specification.

Reference

[1]	S. Sethuraman, CE9: Results of DMVR related Tests CE9.2.1 and CE9.2.2, Joint Video Experts Team Document, JVET-M0147, Marrakesh, Jan. 2019.

[2]	X. Xiu, Y. He and Y. Ye, CE9-related: Complexity reduction and bit-width control for bi-directional optical flow (BIO), Joint Video Experts Team Document, JVET-L0256, Macao, Oct. 2018.

[3]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 4), Joint Video Experts Team Document, JVET-M1001-v6, Marrakesh, Mar. 2019.

[4]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-M1010, Marrakesh, Jan. 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).