| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0148-v3 |
| --- | --- |

| Title: | CE9-related: Early termination for BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-43-299-8526 chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The early termination for BDOF can reduce complexity, however a buffing delay problem of the CU based early termination has been indicated. In this contribution, an early termination for BDOF by using SAD value of DMVR is proposed. The bilateral matching with SAD has been already introduced on DMVR. Therefore, there is no additional matching processing. As an experimental result on CTC, the average of encoding and decoding times became 1% faster and there is no loss of coding efficiency on average. In addition, when the early termination of BDOF is disabled, the average gain is -0.02% and the encoding and decoding times does not increase.

Introduction

At the Marrakesh meeting, a buffing delay problem of the CU based early termination for BDOF has been indicated [1] and several CEs [2] has been established.

DMVR is a method to improve motion vectors by using bilateral matching of bi-directional prediction and since SAD (sum of absolute differences) of the matching cost function has been already introduced.

In this contribution, an early termination for BDOF by using SAD value of DMVR without additional matching process is proposed.

The following two steps in DMVR process apply.

- If the initial matching cost sadList[ 4 ] is less than a threshold, DMVR has been skipped and BDOF also does not apply.

- If the best matching cost sadList[ bestIdx ] is less than a threshold, BDOF does not apply.

The experiment has been shown to reduce the encoding and decoding time with no loss of coding efficiency.

Proposal changes of specification

The working draft should be changed by the followings.

- 8.5.3	Decoder side motion vector refinement process

- 8.5.3.1	General

Inputs to this process are:

- a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture,

- a variable sbWidth specifying the width of the current coding subblock in luma samples,

- a variable sbHeight specifying the height of the current coding subblock in luma samples,

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0 and mvL1,

- the selected luma reference picture sample arrays refPicL0L and refPicL1L.

Outputs of this process are:

- delta luma motion vectors dMvL0 and dMvL1.

The variable subPelFlag is set to 0. and the variables srRange, offsetH0, offsetH1, offsetV0, and offsetV1 are all set equal to 2.

Both components of the delta luma motion vectors dMvL0 and dMvL1 are set equal to zero and modified as follows:

- For each X being 0 or 1, the ( sbWidth + 2 * srRange ) x ( sbHeight + 2 * srRange ) array predSamplesLXL of prediction luma sample values is derived by invoking the fractional sample bilinear interpolation process specified in 8.5.3.2.1 with the luma location ( xSb, ySb ), the prediction block width set equal to ( sbWidth + 2 * srRange ), the prediction block height set equal to ( sbHeight + 2 * srRange ), the reference picture sample array refPicLXL, the motion vector mvLX and the refinement search range srRange as inputs.

- The list sadList[ i ] with i = 0..8 is derived by invoking the sum of absolute differences calculation process specified in 8.5.3.3 with sbWidth, sbHeight, offsetH0, offsetH1, offsetV0, offsetV1, predSamplesL0L and predSamplesL1L as inputs.

- If sadList[ 4 ] is greater than or equal to sbHeight * sbWidth, the following applies:

- The variable bestIdx is derived by invoking the array entry selection process specified in clause 8.5.3.4 with the list sadList[ i ] with i = 0..8 as input.

- If sadList[ bestIdx ] is greater than or equal to sbHeight * sbWidth * 2, bdofApply[ xSb ][ ySb ] is set equal to 1.

- Otherwise bdofApply[ xSb ][ ySb ] is set equal to 0.

- If bestIdx is equal to 4, subPelFlag is set equal to 1.

- Otherwise, the following applies:

dX = bestIdx % 3 − 1		(8-436)

dY = bestIdx / 3 − 1		(8-437)

dMvL0[ 0 ] += 16 * dX		(8-438)

dMvL0[ 1 ] += 16 * dY		(8-439)

offsetH0 += dX		(8-440)

offsetV0 += dY		(8-441)

offsetH1 −= dX		(8-442)

offsetV1 −= dY		(8-443)

- The list sadList[ i ] with i = 0..8 is modifed by invoking the sum of absolute differences calculation process specified in 8.5.3.3 with sbWidth, sbHeight, offsetH0, offsetH1, offsetV0, offsetV1, predSamplesL0L and predSamplesL1L as inputs.

- The variable bestIdx is modified by invoking the array entry selection process specified in clause 8.5.3.4 with the list sadList[ i ] with i = 0..8 as input.

- If bestIdx is equal to 4, subPelFlag is set equal to 1

- Otherwise (bestIdx is not equal to 4), the following applies:

dMvL0[ 0 ] += 16 * ( bestIdx % 3 − 1 )		(8-444)

dMvL0[ 1 ] += 16 * ( bestIdx / 3 − 1 )		(8-445)

- When subPelFlag is equal to 1, the parametric motion vector refinement process specified in clause 8.5.3.5 is invoked with the list sadList[ i ] with i = 0..8, and the delta motion vector dMvL0 as inputs and the modified dMvL0 as output.

- The delta motion vector dMvL1 is derived as follows:

dMvL1[ 0 ] = −dMvL0[ 0 ]		(8-446)

dMvL1[ 1 ] = −dMvL0[ 1 ]		(8-447)

- Otherwise bdofApply[ xSb ][ ySb ] is set equal to 1.

- 8.5.7	Decoding process for inter blocks

- 8.5.7.1	General

This process is invoked when decoding a coding unit coded in inter prediction mode.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- variables numSbX and numSbY specifying the number of luma coding subblocks in horizontal and vertical direction,

- the motion vectors mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the refined motion vectors refMvL0[ xSbIdx ][ ySbIdx ] and refMvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the bi-prediction weight index gbiIdx,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an array predSamples of prediction samples.

Let predSamplesL0L, predSamplesL1L and predSamplesIntraL be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesL0Cb, predSamplesL1Cb, predSamplesL0Cr and predSamplesL1Cr, predSamplesIntraCb, and predSamplesIntraCr be (cbWidth / 2)x(cbHeight / 2) arrays of predicted chroma sample values.

The width and the height of the current coding sublock subCbWidth, subCbHeight in luma samples are derived as follows:

sbWidth  =  cbWidth / numSbX		(8-793)

sbHeight  =  cbHeight / numSbY		(8-794)

For each coding subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1, the following applies:

- The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

( xSb, ySb )  =  ( xCb + xSbIdx * sbWidth, yCb + ySbIdx * sbHeight )	(8-795)

- The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

- If all of the following conditions are true, bdofFlag is set equal to TRUE.

- sps_bdof_enabled_flag is equal to 1.

- predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

- MotionModelIdc[ xCb ][ yCb ] is equal to 0.

- merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- cIdx is equal to 0.

- bdofApply[ xSbIdx ][ ySbIdx ] is equal to 1.

- Otherwise, bdofFlag is set equal to FALSE.

Experimental results

The proposed method has been implemented on VTM-4.0 software. The simulation was based on VTM configuration JVET-M1010 [2]. The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

As summary of experimental results, the average of encoding time became 1% faster and the average of decoding time became 1% faster. There was no loss on average.

Table 1  Summary of experimental results on CTC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.01% | 99% | 99% |
| Class A2 | 0.00% | 0.05% | 0.03% | 99% | 99% |
| Class B | 0.01% | -0.04% | -0.01% | 99% | 99% |
| Class C | 0.00% | -0.05% | -0.05% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.01% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.01% | 100% | 97% |

Additional experimental results are shown, when the early termination of BDOF is disabled. The average gain is -0.02% and the encoding and decoding times are not changed. Compared to VTM-4.0 without the early termination of BDOF (same as CE9.2.1 condition), the average gain is almost same, and the decoding time become 8% faster.

Table 2  Summary of experimental results of proposed method when the early termination of BDOF does not apply (test 2)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.03% | 0.01% | 101% | 99% |
| Class A2 | -0.03% | -0.01% | 0.06% | 100% | 99% |
| Class B | -0.01% | 0.00% | -0.01% | 100% | 100% |
| Class C | -0.01% | 0.00% | -0.07% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.01% | -0.01% | 100% | 100% |
| Class D | 0.00% | -0.08% | -0.10% | 101% | 100% |
| Class F | -0.01% | -0.02% | -0.01% | 100% | 97% |

Table 3  Summary of experimental results of VTM-4.0 when the early termination of BDOF is disabled (test 3)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.04% | 0.02% | 100% | 107% |
| Class A2 | -0.04% | 0.02% | 0.05% | 100% | 107% |
| Class B | -0.02% | -0.03% | 0.01% | 100% | 108% |
| Class C | -0.01% | -0.04% | -0.05% | 100% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.01% | 0.00% | 100% | 108% |
| Class D | -0.02% | -0.01% | -0.07% | 100% | 106% |
| Class F | -0.01% | 0.00% | 0.04% | 101% | 116% |

Conclusion

In this contribution, an early termination for BDOF by using SAD value of DMVR without additional matching process is proposed. As experimental results, the average of encoding and decoding times became 1% faster and there is no loss of coding efficiency on average. Finally, we would like to recommend that the working draft and reference software should support this specification.

Reference

[1]	K. Kondo, M. Ikeda and T. Suzuki, Non-CE9: On early termination for BDOF, Joint Video Experts Team Document, JVET-M0073, Marrakesh, Jan. 2019.

[2]	S. Esenlik, and X. Xiu, Description of Core Experiment 9 (CE9): Decoder Motion Vector Derivation, Joint Video Experts Team Document, JVET-M1024, Marrakesh, Jan. 2019.

[2]	F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-M1010, Marrakesh, Jan. 2019.

[3]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 4), Joint Video Experts Team Document, JVET-M1001-v6, Marrakesh, Mar. 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).