| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0157 |
| --- | --- |

| Title: | Non-CE3: MPM List Modification for Zero Reference Line |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheng Fang, Dong Jiang, Jucai Lin, and Feiyang Zeng | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com fang_cheng1@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution proposes five modifications to modify the MPM list order for zero reference line. All the proposed methods maintain almost the same MPM structure as VTM4.0, but give the priority to the Planar mode or DC mode under certain conditions.

Experimental results of all the methods are shown in below, the anchor is VTM4.0 under AI configuration:

- The average BD-rate gains of the first experiment are 0.01%, 0.00% and 0.01% for Y, U, V

- The average BD-rate gains of the second experiment are 0.03%, -0.01% and 0.00% for Y, U, V

- The average BD-rate gains of the third experiment are 0.00%, 0.00% and -0.01% for Y, U, V

- The average BD-rate gains of the fourth experiment are 0.02%, -0.02% and 0.03% for Y, U, V

- The average BD-rate gains of the fifth experiment are 0.04%, -0.01% and 0.00% for Y, U, V

Algorithm description

The proposed methods change the MPM list order for the following cases for zero reference line (non-ISP):

- If one neighboring mode is DC and the other neighboring mode is an angular mode a.

- If one neighboring mode is Planar and the other neighboring mode is an angular mode a, and a equals to either 2 or 34 or 66.

The MPM list order for other cases is the same as VTM4.0[1].

For the first case, the MPM list has two different construction methods:

I. mpm[0] = DC_IDX mpm[1] = a mpm[2] = Planar_IDX mpm[3] = 2 + ( ( maxAB + 61 ) % 64 ) mpm[4] = 2 + ( ( maxAB − 1 ) % 64 ) mpm[5] = 2 + ( ( maxAB + 60 ) % 64 ) II. mpm[0] = DC_IDX mpm[1] = Planar_IDX mpm[2] = a mpm[3] = 2 + ( ( maxAB + 61 ) % 64 ) mpm[4] = 2 + ( ( maxAB − 1 ) % 64 ) mpm[5] = 2 + ( ( maxAB + 60 ) % 64 )

For the second case, MPM list is derived as follows:

III. mpm[0] = PLANAR_IDX mpm[1] = a mpm[2] = DC_IDX mpm[3] = 2 + ( ( maxAB + 61 ) % 64 ) mpm[4] = 2 + ( ( maxAB − 1 ) % 64 ) mpm[5] = 2 + ( ( maxAB + 60 ) % 64 )

Experimental results

According to the methods mentioned above, five experiments Test 1~5 are designed:

- Test 1: method I

- Test 2: method II

- Test 3: method III

- Test 4: combine method I and method III

- Test 5: combine method II and method III

All the experiments are implemented on top of VTM4.0 software. The simulations are performed on Linux cluster using VTM AI configurations under the common test condition described in JVET-M1010 [2].

Table 1~5 show the experimental results of Test 1~5 respectively.

Table 1 Coding performance of Test 1 over VTM-4.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.07% | 0.02% | 100% | 101% |
| Class A2 | -0.02% | 0.11% | 0.00% | 100% | 100% |
| Class B | -0.01% | 0.01% | 0.02% | 100% | 100% |
| Class C | 0.00% | -0.04% | -0.08% | 100% | 101% |
| Class E | 0.00% | -0.03% | -0.04% | 100% | 100% |
| Overall | -0.01% | 0.00% | -0.01% | 100% | 100% |
| Class D | -0.03% | 0.01% | -0.11% | 100% | 101% |
| Class F | 0.00% | 0.06% | 0.05% | 100% | 101% |

Table 2 Coding performance of Test 2 over VTM-4.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.03% | -0.02% | 100% | 101% |
| Class A2 | -0.04% | 0.10% | 0.00% | 100% | 100% |
| Class B | -0.04% | 0.02% | 0.09% | 100% | 101% |
| Class C | 0.00% | -0.02% | -0.13% | 100% | 100% |
| Class E | -0.04% | -0.02% | 0.04% | 100% | 100% |
| Overall | -0.03% | 0.01% | 0.00% | 100% | 100% |
| Class D | -0.01% | 0.07% | -0.15% | 100% | 100% |
| Class F | 0.01% | 0.06% | -0.07% | 100% | 100% |

Table 3: Coding performance of Test 3 over VTM-4.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.06% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.08% | -0.02% | 100% | 100% |
| Class B | -0.01% | -0.02% | 0.05% | 100% | 100% |
| Class C | 0.02% | -0.03% | -0.02% | 100% | 100% |
| Class E | -0.01% | 0.02% | 0.02% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class D | 0.00% | -0.12% | -0.14% | 100% | 100% |
| Class F | -0.01% | 0.02% | 0.05% | 100% | 100% |

Table 4: Coding performance of Test 4 over VTM-4.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.07% | -0.01% | 100% | 101% |
| Class A2 | -0.02% | 0.14% | 0.02% | 100% | 100% |
| Class B | -0.02% | 0.00% | 0.02% | 100% | 101% |
| Class C | -0.01% | -0.07% | -0.08% | 100% | 100% |
| Class E | -0.02% | 0.01% | -0.08% | 100% | 100% |
| Overall | -0.02% | 0.02% | -0.03% | 100% | 100% |
| Class D | 0.00% | -0.01% | -0.12% | 100% | 101% |
| Class F | 0.00% | 0.02% | 0.02% | 100% | 100% |

Table 5: Coding performance of Test 5 over VTM-4.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.04% | -0.02% | 99% | 100% |
| Class A2 | -0.03% | 0.10% | -0.04% | 100% | 100% |
| Class B | -0.04% | 0.02% | 0.03% | 100% | 100% |
| Class C | -0.01% | -0.11% | -0.01% | 100% | 100% |
| Class E | -0.03% | 0.05% | 0.04% | 100% | 101% |
| Overall | -0.04% | 0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.02% | -0.10% | 100% | 100% |
| Class F | 0.01% | 0.00% | -0.13% | 100% | 100% |

Conclusion

In this contribution, five modifications are proposed to modify the MPM list. Combining the method II and method III can achieve 0.04%, -0.01% and 0.00% average BD-rate gains for luminance and both chrominance components with negligible increasing of encoding and decoding complexity on top of VTM4.0 under the AI configuration. It is proposed to adopt this proposal into VTM.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Dahua may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).