| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0164-v3 |
| --- | --- |

| Title: | CE3-related: Size restriction for CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

Separate luma chroma partitioning tree introduces a long latency for chroma intra prediction when the cross component linear mode (CCLM) is enabled. In this contribution, CCLM is used with size restriction. First, CCLM is disabled for 2x2 chroma blocks. Second, CCLM is disabled for 32x32 chroma blocks. Third, CCLM is disabled for 2x2, 2x4, 4x2 chroma blocks. It is reported that the first restriction only brings 0.00%, 0.03%, 0.03% loss on Y,U,V components for AI configuration compared to VTM-4.0, respectively. And the second restriction only brings 0.05%, 0.42%, 0.35 % loss on Y,U,V components for AI configuration compared to VTM-4.0, respectively. And the third restriction only brings 0.01%, 0.27%, 0.27% loss on Y,U,V components for AI configuration compared to VTM-4.0, respectively.

Introduction

Separate luma chroma partitioning tree introduces a long latency for chroma intra prediction when the CCLM is on. In the worst case, the chroma intra prediction cannot be kicked off until the whole 64x64 luma block is reconstructed. Moreover, the time is tight for small blocks, this latency make it worse. As a results, several size restrictions are proposed to reduce the latency:

Restriction 1: CCLM is disabled for 2x2 chroma blocks.

Restriction 2: CCLM is disabled for chroma blocks with size larger or equal to 32x32.

Restriction 3: CCLM is disabled for 2x2, 2x4, 4x2 chroma blocks (additional information).

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-4.0 is served as the experimental platform. The random access configuration (RA) specified in [1], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

Here, we have three tests:

Test A: Apply Restriction 1;

Test B: Apply Restriction 2;

Test C: Apply Restriction 3;

The results are given in Table 1 to Table 3. It is reported that the first restriction only brings 0.00%, 0.03%, 0.03% loss on Y,U,V components for AI configuration compared to VTM-4.0, respectively. And the second restriction only brings 0.05%, 0.42%, 0.35% loss on Y,U,V components for AI configuration compared to VTM-4.0, respectively. And the third restriction only brings 0.01%, 0.27%, 0.27% loss on Y,U,V components for AI configuration compared to VTM-4.0, respectively.

Table 1 Results of Test A (Restriction 1)

Table 2 Results of Test B (Restriction 2)

Table 3 Results of Test C (Restriction 3)

Conclusion

In this contribution, we have two modifications on CCLM enabling condition. These modifications bring no coding loss and help to reduce the latency. Based on the above results, it is suggested to adopt them into the VTM.

Reference

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex: modified text based on Draft 4 v7

Input to this process is a request for a binarization for the syntax element intra_chroma_pred_mode, the current transform block width nTbW and the current transform block height nTbH.

MinCCLMSize is 6 for Restriction 1 and 8 for Restriction 3.

Table 9-13 – Binarization for intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 1

| Value of intra_chroma_pred_mode | Bin string |  |
| --- | --- | --- |
|  | (nTbW + nTbH) > =MinCCLMSize | (nTbW + nTbH) < MinCCLMSize |
| 7 | 0 | 0 |
| 4 | 10 | - |
| 5 | 1110 | - |
| 6 | 1111 | - |
| 0 | 11000 | 100 |
| 1 | 11001 | 101 |
| 2 | 11010 | 110 |
| 3 | 11011 | 111 |

| Table 9-15 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … |  |  |  |  |  |  |
| intra_luma_mpm_idx[ ][ ] | bypass | bypass | bypass | bypass | bypass | na |
| intra_luma_mpm_remainder[ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 ¦¦ (nTbW + nTbH) < MinCCLMSize | 0 | bypass | bypass | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 && (nTbW + nTbH) > =MinCCLMSize | 0 | 1 | 2 | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 1 && (nTbW + nTbH) >= MinCCLMSize | 0 | 1 | 2 | 2 | na | na |
| merge_subblock_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| … |  |  |  |  |  |  |