| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0165-v2 |
| --- | --- |

| Title: | Non-CE4: On spatial candidate list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liying Xu Fangdong Chen Li Wang | Tel: Email: | +86-18100191186 xuliying5@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

Firstly, this contribution proposes a candidate list reorder method for normal merge normal AMVP, Triangle merge, IBC merge, ATMVP，IBC AMVP, affine merge and affine AMVP. Instead of using the left candidate in the front, the proposed method will take the above candidates in front in candidate list construction order. Compared with VTM4.0, the results reportedly show that the proposed swapping method achieves 0.13%, and 0.04% BD rate reduction for, LB, and RA configurations, respectively, while brings negligible increase of encoding and decoding complexity.

Secondly, this contribution proposes a spatial candidate list simplification method for normal merge normal AMVP, Triangle merge, IBC merge, IBC AMVP, affine merge and affine AMVP. The proposed method removes checking the above left candidate (B2). Compared with VTM4.0, the results reportedly show that the proposed method only gets XX%, and XX% BD loss for, LB, and RA configurations, respectively when removing B2 from the merge lists.

Introduction

Merge mode

The checking order of spatial merge candidate is left (A1), above (B1), above-right (B0), bottom-left (A0), above-left (B2), as shown in Fig. 1.

Instead of just checking whether a neighboring block is available and contains motion information, some additional redundancy checks are performed before taking all the motion data of the neighboring block as a merge candidate, such as avoiding having candidates with redundant motion data in the list.

The original checking order is listed as follows.

Table 1

| Normal merge | Triangle merge | IBC merge | Affine merge | ATMVP |
| --- | --- | --- | --- | --- |
| A1-B1-B0-A0-B2 | A1-B1-B0-A0-B2 | A1-B1-B0-A0-B2 | A0-A1-B0-B1-B2 | A1 |

Fig.1 An illustration of the first four candidate construction for Merge mode

AMVP mode

The locations of the spatial candidate blocks are the same for both AMVP and inter-prediction block merging. The AMVP candidate list construction is checked by the spatial order as follows.

Table 2

| Normal AMVP | IBC AMVP | Affine AMVP |
| --- | --- | --- |
| Check left: A0-A1-(scaledA0-ScaledA1), Then Check above: B0-B1-B2-(scaledB0-ScaledB1-ScaledB2) | Check left: A0-A1, Then Check above: B0-B1-B2 | Check left: A0-A1, Then Check above: B0-B1-B2 |

Part1 Proposed Modification

Merge Mode

The reordered checking order in listed as follows.

Table 3

| Normal merge | Triangle merge | IBC merge | Affine merge | ATMVP |
| --- | --- | --- | --- | --- |
| B1-A1-A0-B0-B2 | B1-A1-A0-B0-B2 | B1-A1-A0-B0-B2 | B0-B1-B2-A0-A1 | B1 |

AMVP Mode

The reordered checking order in listed as follows.

Table 4

| Normal AMVP | IBC AMVP | Affine AMVP |
| --- | --- | --- |
| Check above: B0-B1-B2-(scaledB0-ScaledB1-ScaledB2) Then Check left: A0-A1-(scaledA0-ScaledA1), | Check above: B0-B1-B2, Then Check left: A0-A1 | Check above: B0-B1-B2, Then Check left: A0-A1 |

Part2-Proposed Modification

The simplified method of removing checking B2 is list as follows.

Method1

Merge Mode

The new spatial merge list without B2 is listed as follows.

Table 5

| Normal merge | Triangle merge | IBC merge | Affine merge |
| --- | --- | --- | --- |
| A1-B1-B0-A0 | A1-B1-B0-A0 | A1-B1-B0-A0 | A0-A1-B0-B1 |

AMVP Mode

The new spatial AMVP list without B2 is listed as follows.

Table 6

| Normal AMVP | IBC AMVP | Affine AMVP |
| --- | --- | --- |
| Check left: A0-A1-(scaledA0-ScaledA1), Then Check above: B0-B1-(scaledB0-ScaledB1) | Check left: A0-A1, Then Check above: B0-B1 | Check left: A0-A1, Then Check above: B0-B1 |

Method2

Merge Mode

The new spatial merge list without B2 is listed as follows. In this part, B2 in affine merge keeps unchanged.

Table 7

| Normal merge | Triangle merge | IBC merge | Affine merge |
| --- | --- | --- | --- |
| A1-B1-B0-A0 | A1-B1-B0-A0 | A1-B1-B0-A0 | A0-A1-B0-B1-B2 |

AMVP Mode

The new spatial AMVP list without B2 is listed as follows. In this part, B2 in affine AMVP keeps unchanged.

Table 8

| Normal AMVP | IBC AMVP | Affine AMVP |
| --- | --- | --- |
| Check left: A0-A1-(scaledA0-ScaledA1), Then Check above: B0-B1-(scaledB0-ScaledB1) | Check left: A0-A1, Then Check above: B0-B1 | Check left: A0-A1, Then Check above: B0-B1-B2 |

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-4.0 is served as the experimental platform. The CTC specified in [1], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

The proposed technique is implemented on VTM4.0 and tested under Low-delay-B, and Random Access configurations. The results are shown in Table 9 to Table 14. It can be observed that the proposed reordering method can achieve gain on with negligible increase of encoding and decoding complexity. Also, it can be observed that the proposed simplified method only has small loss on RA and LB configurations.

Part1

Test1: Full reorder for merge and AMVP listed as Table3 and Table4.

Table 9

Part2

Test2: Full remove B2 for merge list as Table5

Test3: Full remove B2 for merge and AMVP list as Table 5 and Table6.

Test4: Partially remove B2 for merge list as Table7.

Test5: Partially remove B2 for merge and AMVP list as Table7 and Table8.

Table 10

|  | Test2 |  |  | Test3 |  |  | Test4 |  |  | Test5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA |  |  |  |  |  |  | 0.04% | 0.03% | 0.01% | 0.04% | -0.02% | 0.02% |
| LB |  |  |  |  |  |  | 0.00% | 0.28% | -0.04% | 0.06% | 0.14% | -0.17% |

Table11

Table12

Table13

Table14

Conclusion

This contribution proposes a candidate list reordering method and a simplified spatial checking method for merge mode and AMVP mode on the platform of VTM4.0. It is verified that the proposed method can improve the coding efficiency with negligible increasing of encoding and decoding complexity. Besides, the proposed simplified spatial checking method can simplify the checking scheme and no need to access the motion information of B2 with a little loss. Therefore it is recommended to adopt the proposed method in the next version of reference software.

Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Patents

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).