| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0183-v3 |
| --- | --- |

| Title: | AHG16/Non-CE3: Shared MPM list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra, Semih Esenlik, Jianle Chen, Biao Wang, Han Gao, | Tel: Email: | anand.meher.kotra@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes to extend the “shared merge, IBC” list concept to intra luma most probable mode (MPM) list construction.  This feature enables to share the same MPM list for all leaf coding units of one ancestor node in the CU split tree. Therefore the shared MPM list feature relaxes the worst case scenario of construction of luma MPM list for blocks as small as 4 x 4 samples. Moreover it enables parallel intra mode derivation of blocks belonging to a given shared region.

Objective results are as follows:

Over VTM-4.0.1 Anchor under CTC conditions, negligible BD-Rate loss is observed for both AI and RA configurations with no change in EncT and DecT.

# Introduction

VTM-4.0 [1] uses a concept called shared merge, IBC list wherein the same merging candidate list is used for all leaf coding units (CUs) of one ancestor node in the CU split tree for enabling parallel processing of small skip/merge-coded CUs. The ancestor node is named merge sharing node. It is reported that the shared merge candidate list can effectively reduce difficulty for real-time 8K hardware decoding.

VTM-4.0 also uses a most probable mode (MPM) list to derive the intra mode of a given coding block. The MPM list construction uses the information of the neighboring blocks intra mode to derive the most probable mode.  The MPM list construction needs to be performed to derive intra mode for blocks whose size is as small as 4 x 4 samples. This reportedly places a stringent restriction on the throughput and any complexity increase in the MPM list construction therefore needs to be carefully quantified. VTM-4.0 currently uses different MPM list construction techniques for different tools like intra sub-partition tool (ISP), Multi reference line index (MRL) etc. Currently CE3-3[4] is investigating different proposals which harmonize the different MPM list construction into one single harmonized solution.

The current proposal solves the worst case scenario of constructing an MPM list for blocks whose size is 4x4 samples. Furthermore, it is to be reported that the current shared MPM list technique can be used on top of any intra MPM list construction logic.

# Proposal

The current proposal uses the same logic as shared merge candidate list as proposed in [5]. A given node is considered to be a shared MPM node if it satisfies the following two conditions:

- The MPM sharing node size is equal to or larger than the size threshold

- In the MPM sharing node, one of the child CU size is smaller than the size threshold

When the above conditions are satisfied then all the child CU nodes share the same MPM list as of the shared MPM node. The threshold size used is 32 which is the same for shared merge and IBC lists.

# Experimental results

The proposed method is implemented on top of VTM-4.0.1 [2] and the simulation is performed under common test conditions as described in JVET-M1010 [3].

## Test1:

In the first test, the threshold size is set to 32 samples and all the nodes in the shared MPM region share the same spatial neighbors. However since VTM-4.0.1 uses different MPM list construction for different tools like MRL > 0, ISP, in the worst case 3 lists per 32 samples may have to be constructed. This scenario has the advantage that all the blocks in a given shared MPM region can still construct the MPM lists in parallel.

Table1 below shows the result for Test1.  Anchor used is VTM-4.0.1and the test uses proposed modifications on top of VTM-4.0.1 but the configuration files used are based on VTM-4.0 where ISPFast is set to 0.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.05% | -0.07% | 104% | 100% |
| Class A2 | -0.04% | -0.04% | -0.03% | 104% | 100% |
| Class B | -0.03% | 0.03% | 0.05% | 104% | 100% |
| Class C | -0.01% | 0.05% | 0.02% | 106% | 101% |
| Class E | -0.05% | 0.04% | -0.06% | 106% | 101% |
| Overall | -0.03% | 0.03% | -0.01% | 105% | 101% |
| Class D | 0.00% | -0.02% | 0.06% | 106% | 100% |
| Class F | -0.03% | -0.04% | 0.02% | 104% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.10% | 0.04% | 100% | 100% |
| Class A2 | -0.03% | -0.07% | 0.02% | 100% | 100% |
| Class B | -0.01% | 0.16% | 0.09% | 100% | 100% |
| Class C | 0.02% | 0.19% | 0.20% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.11% | 0.10% | 101% | 100% |
| Class D | -0.01% | -0.06% | -0.13% | 102% | 104% |
| Class F | -0.01% | 0.17% | 0.01% | 101% | 100% |

## Test2:

In the second test, the threshold size is also set to 32 samples and all the nodes in the shared MPM region share the same spatial neighbors. Moreover, when the given child node belongs to the shared MPM region, then the MPM list constructed defaults to the regular 6 MPM construction which is used for blocks without MRL and 6 MPM. This reportedly gives an added advantage that all the blocks can share the same MPM list and the worst case of 1MPM list per 32 samples is also satisfied. Table 2 below shows the results for this case

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.09% | -0.05% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | -0.02% | 101% | 100% |
| Class B | 0.01% | 0.00% | 0.03% | 101% | 101% |
| Class C | 0.03% | 0.10% | 0.09% | 101% | 101% |
| Class E | 0.02% | 0.11% | -0.02% | 101% | 101% |
| Overall | 0.01% | 0.05% | 0.01% | 101% | 101% |
| Class D | 0.03% | -0.04% | -0.04% | 102% | 100% |
| Class F | 0.05% | -0.05% | 0.05% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | 0.02% | 100% | 100% |
| Class A2 | 0.02% | -0.02% | -0.03% | 100% | 100% |
| Class B | 0.00% | 0.10% | 0.01% | 100% | 101% |
| Class C | 0.03% | 0.25% | 0.21% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.09% | 0.06% | 100% | 101% |
| Class D | 0.03% | -0.02% | -0.12% | 101% | 104% |
| Class F | 0.03% | 0.07% | 0.02% | 100% | 101% |

## Test3:

Test 3 is basically same as Test1 where the “test” also uses the configuration files based on VTM-4.0.1. Therefore ISPFast is also set to 1 in this configuration

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | -0.02% | 102% | 103% |
| Class A2 | 0.00% | 0.02% | -0.04% | 99% | 98% |
| Class B | 0.01% | 0.02% | 0.04% | 101% | 100% |
| Class C | 0.04% | -0.01% | 0.07% | 101% | 99% |
| Class E | 0.00% | 0.09% | -0.01% | 100% | 99% |
| Overall | 0.01% | 0.03% | 0.01% | 101% | 100% |
| Class D | 0.04% | 0.10% | 0.03% | 102% | 100% |
| Class F | 0.02% | -0.01% | -0.01% | 99% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.01% | 99% | 100% |
| Class A2 | 0.02% | 0.07% | 0.05% | 100% | 98% |
| Class B | 0.00% | 0.19% | 0.01% | 102% | 102% |
| Class C | 0.03% | 0.27% | 0.23% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.15% | 0.08% | 100% | 101% |
| Class D | -0.01% | -0.09% | -0.22% | 100% | 102% |
| Class F | 0.04% | 0.11% | -0.03% | 100% | 99% |

# Conclusion

This contribution proposes to extend the logic of shared merge, IBC list to also cover the intra luma most probable mode (MPM) list. In VTM-4.0, the MPM list needs to be constructed for blocks as small as 4 x 4 samples which may place stringent requirements on clock cycles for hardware implementations. The proposed solution relaxes this worst case by sharing the same MPM list for all leaf coding units of a given shared MPM node. Moreover the intra mode derivation can parallelly be performed for all CUs present in a given shared region. Objective results show that the proposed technique does not result in any BD-Rate loss without any increase in EncT and DecT. The specification text changes are reported to be very minimal and is attached along with the contribution. It is suggested to adopt this technique into the next version of VVC software and working draft text.

# References

- B. Bross, J. Chen, S. Liu, "Versatile Video Coding (Draft 4)," Document of Joint Video Experts Team, JVET-M1001.

- VTM-4.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0.1

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-M1010.

- G.V. Auwera et al, "Description of Core Experiment 3 (CE3): Intra prediction and mode coding," Document of Joint Video Experts Team, JVET-M0123.

- CE4.3.1: Shared merging candidate list, Document JVET-M0170, C.C. Chen et al.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).