| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0184-r1 |
| --- | --- |

| Title: | CE3-3.1.1: Unified MPM list generation. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | B. Wang, A. M. Kotra, S. Esenlik H. Gao, J. Chen (Huawei) L. Li, J. Heo, J. Choi, S. Yoo J. Lim, S. Kim (LGE) | Tel: Email: | biao.wang@huawei.com, l.li@lge.com, |
| Source: | Huawei Technologies Co., Ltd., LGE |  |  |

_____________________________

# Abstract

The current contribution document reports the result of the CE test CE3-3.1.1. A unified 6-MPM (Most Probable Mode) list generation is proposed and tested. Compared to VTM4.0, the proposal achieve 0.08% coding gain in AI, with 101% encoding and decoding time.

# Introduction

During the 13th JVET meeting in Marrakech, it was requested to test a unified MPM list generation for different coding tools, such as Multiple Reference Line (MRL), Intra sub-partition (ISP) mode coding. The current contribution document reports the result of CE test CE3-3.1.1 where a unified MPM list generation is used for different coding tools including MRL and ISP.

# Problem Definition

In VTM4.0, three different 6-MPM list generations are used for different coding tools, including:

- A 6-MPM list for intra blocks with MRL mode enabled, where planar and DC mode are excluded

- A 6-MPM list for intra blocks with ISP mode enabled, where DC mode is excluded

- A 6-MPM list for normal intra block, where planar and DC are always included.

For each of above 6-MPM list, different 6-MPM construction is used and this makes the MPM list construction complicated. A complex MPM list construction might compromise the throughput of the decoder, in particular for small blocks such as 4x4 samples.

# Proposed Solutions

This CE test uses a unified 6-MPM list construction for both normal intra blocks and intra blocks with ISP enabled. This unified 6-MPM list always include planar but always excluding DC. For intra blocks with MRL mode enabled, Planar mode is excluded and the remaining five modes excluding Planar in the unified 6-MPM list is used. As an example, considering a MPM list constructed with intra modes {Planar, 38, 37, 39, 36, 40} when MRL mode is not enabled (i.e. reference line index equals to zero). When MRL mode is enabled (i.e. the reference line index is not equal to zero), a 5-MPM list {38, 37, 39, 36, 40} is used.

In addition, a supplementary test is provided. It uses a unified 6-MPM list construction for both normal intra blocks and intra blocks with ISP enabled. This unified 6-MPM list always include planar and DC. For intra blocks with MRL mode enabled, Planar and DC modes are excluded and the remaining four modes excluding Planar and DC in the unified 6-MPM list is used. As an example, considering a unified MPM list is constructed with intra modes {38, Planar, DC, 37, 39, 36} when MRL mode is not enabled (i.e. reference line index equals to zero). When MRL mode is enabled (i.e. the reference line index is not equal to zero), a 4-MPM list {38, 37, 39, 36} is used.

For both the CE test and the supplementary result, the first bin of the mpm index codeword is CABAC context coded. In total three contexts are used, corresponding to whether the current intra block is MRL enabled, ISP enabled, or a normal intra block.

# Experimental Results

The simulations are performed according to CTC described in [1]. AI and RA results are reported.

Two tests for each solution are performed, as agreed in CE3.3 test description [2].

- Test A

  - Anchor: VTM4.0 with no changes in full encoder RD check

  - Test: Anchor + Proposed test

- Test B

  - Anchor: VTM4.0 includes all MPM modes in full encoder RD check (i.e. Codebase).

  - Test: Anchor + Proposed test

## Simulation result of CE3-3.1.1, Test A

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.31% | -0.24% | 99% | 101% |
| Class A2 | -0.01% | 0.06% | -0.08% | 100% | 100% |
| Class B | -0.04% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.04% | 0.00% | -0.05% | 101% | 100% |
| Class E | -0.14% | -0.05% | -0.06% | 101% | 101% |
| Overall | -0.08% | -0.05% | -0.07% | 101% | 101% |
| Class D | -0.06% | -0.02% | -0.21% | 100% | 98% |
| Class F | 0.13% | 0.08% | 0.12% | 100% | 100% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.08% | -0.21% | 102% | 101% |
| Class A2 | -0.01% | 0.05% | 0.09% | 102% | 101% |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.01% | 0.08% | -0.09% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.01% | -0.19% | -0.18% | 100% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

## Simulation result of CE3-3.1.1, Test B

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Codebase |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.26% | -0.16% | 101% | 100% |
| Class A2 | 0.11% | -0.07% | -0.04% | 102% | 100% |
| Class B | 0.09% | -0.02% | 0.02% | 102% | 100% |
| Class C | 0.10% | 0.06% | 0.15% | 102% | 100% |
| Class E | 0.00% | -0.02% | -0.12% | 101% | 100% |
| Overall | 0.05% | -0.05% | -0.01% | 102% | 100% |
| Class D | 0.03% | 0.02% | 0.15% | 102% | 101% |
| Class F | 0.15% | 0.00% | 0.02% | 101% | 100% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Codebase |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

## Simulation result of Supplementary Test, Test A

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.25% | -0.22% | 99% | 101% |
| Class A2 | -0.06% | 0.06% | -0.07% | 101% | 101% |
| Class B | -0.07% | -0.10% | 0.00% | 100% | 100% |
| Class C | 0.05% | -0.05% | 0.04% | 100% | 100% |
| Class E | -0.05% | 0.06% | 0.00% | 101% | 100% |
| Overall | -0.08% | -0.06% | -0.04% | 100% | 100% |
| Class D | -0.08% | -0.11% | -0.30% | 100% | 99% |
| Class F | 0.15% | 0.17% | 0.05% | 99% | 99% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.20% | -0.11% | 101% | 100% |
| Class A2 | -0.01% | -0.05% | 0.08% | 101% | 100% |
| Class B | 0.00% | -0.02% | -0.18% | 100% | 100% |
| Class C | 0.04% | -0.15% | -0.13% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.09% | -0.10% | 100% | 100% |
| Class D | -0.01% | -0.23% | -0.33% | 100% | 98% |
| Class F | 0.08% | -0.12% | -0.10% | 100% | 99% |

## Simulation result of Supplementary Test, Test B

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Codebase |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.20% | -0.04% | 100% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.02% | 0.04% | 0.01% | 101% | 100% |
| Class C | 0.06% | 0.04% | 0.09% | 100% | 100% |
| Class E | 0.06% | 0.03% | 0.04% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.05% | 0.14% | 0.04% | 101% | 102% |
| Class F | 0.18% | 0.19% | 0.18% | 99% | 100% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Codebase |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusion

The current contribution document reports the results of the CE test CE3-3.1.1. With a unified MPM list construction 0.08% luma coding gain is achieved with 101% encoding and decoding time.

# References

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring.

- G. Van der Auwera, L. Li, and A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-M1023, Jan 2019

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).