| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0188_r1 |
| --- | --- |

| Title: | CE7-7.3 : Unified rice parameter derivation for coefficient level coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yinji Piao Kiho Choi | Email: | [yj1003.piao@samsung.com] [kiho14.choi@samsung.com] |
| Source: | Samsung Electronics Co.,Ltd |  |  |

_____________________________

# Abstract

This contribution reports CE7-7.3 test result related to the rice parameter derivation method proposed in last meeting. A unified solution for rice parameter derivation of bypass-coded level is proposed for coefficient coding. Instead of two different rice parameter derivation methods for remaining level and fully bypass-coded level, the rice parameter is derived similar to the one for completely bypass-coded level by referring to neighboring coefficient levels with consideration of corresponding base level. Proposed unification provides only 0.00% and -0.02% BD-rate performance change in AI and RA configurations, respectively.

# Introduction

In last meeting, it was decided to include gt3_flag into the first pass of regular-coded bins together with sig_flag, gt1_flag and par_flag in maximum 32 bins. The remainder of coefficient which is regular coded partially is to be coded in bypass mode of arithmetic coding engine using Golomb-Rice code with base level equal to 4. Coefficient which is not scanned in the first pass is completely coded in bypass mode using Golomb-Rice code.

In current VTM4.0, the rice parameter for the remainder of partially regular coded coefficient is derived with previously coded level similar to HEVC, while the rice parameter for the completely bypass-coded level is derived based on the sum of absolute levels of neighboring transform coefficients similar to JEM s/w. To simplify the derivation process of rice parameter for two different types of bypass-coded levels, a unified solution for rice parameter derivation is proposed in this contribution.

# Proposed method

The unified rice parameter derivation is similar to the one for completely bypass-coded level, which refers to the neighboring transform coefficients within a given template. Rice parameter is determined not only based on sum of absolute levels of neighboring five transform coefficients in local template, but the corresponding base level is also taken into consideration.

RicePara = RiceParTable[ max(min( 31, sumAbs – 5 * baseLevel), 0) ],

where baseLevel can be equal to 4, 0.

In comparison to current rice parameter derivation for two types of bypass-coded levels, proposed unification in table I shows more unified and consistent design. By removing previous level dependent rice parameter update, proposed method also enables parallel derivation of rice parameter for multiple positions locate on the same diagonal line as illustrated in Fig.1.

Table I. Comparison of current and proposed rice parameter derivation

| Bypass coded level | Current Rice parameter derivation | Unified Rice parameter derivation |
| --- | --- | --- |
| 2nd pass (bypass coding of remainder, baseLevel=4) for(k=startScanIdx; k< startIdxBypass; k++) { if( gt2_flag[ k ] != 0 ) { decode remainder[k] coeff[k] = coeff[k] + 2 * remainder[k] } } | if(ricePar < 3 && rem > (3<<ricePar)-1) { ricePar++; } | riceParTable [max(min(31,sumAbs– baseLevel*5 ), 0) ] , where base_level = 4 |
| 3rd pass (bypass coding of complete coefficients) for(k=startIdxBypass; k<=endScanIdx; k++) { decode absLevel[k] coeff[k] = absLevel[k] } | riceParTable [ min( 31, sumAbs ) ] | riceParTable [max(min(31,sumAbs– baseLevel*5 ), 0) ] where baseLevel = 0 (same with current) |

Fig 1. Illustration of enabled parallel derivation for rice parameter

# Results

Proposed unification was implemented on top of VTM 4.0. Tests were conducted under VTM common test condition [2] as well as under low QP conditions described in [3]. More details can be found in accompanying excel sheets.

Table I. Test results at CTC and lowQP condition

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  | Over VTM-4.0_lowQP |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.01% | 102% | 99% | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Class A2 | -0.01% | 0.11% | -0.01% | 100% | 101% | -0.03% | -0.07% | -0.06% | 102% | 102% |
| Class B | 0.00% | -0.01% | 0.08% | 102% | 101% | -0.01% | -0.01% | -0.04% | 102% | 102% |
| Class C | 0.00% | 0.01% | -0.07% | 101% | 101% | 0.03% | -0.03% | 0.00% | 102% | 103% |
| Class E | -0.01% | 0.00% | -0.03% | 101% | 102% | -0.02% | 0.00% | -0.02% | 103% | 102% |
| Overall | 0.00% | 0.01% | 0.00% | 102% | 101% | 0.00% | -0.02% | -0.02% | 102% | 102% |
| Class D | 0.00% | -0.01% | -0.08% | 102% | 101% | 0.06% | -0.01% | 0.05% | 102% | 101% |
| Class F | 0.05% | 0.11% | 0.03% | 102% | 102% | -0.05% | -0.03% | -0.03% | 102% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  | Over VTM-4.0_lowQP |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | 0.01% | 100% | 100% | 0.00% | -0.01% | -0.01% | 100% | 102% |
| Class A2 | -0.05% | 0.12% | 0.02% | 100% | 101% | -0.01% | -0.02% | -0.02% | 101% | 103% |
| Class B | -0.02% | 0.16% | -0.18% | 101% | 99% | 0.00% | 0.00% | 0.00% | 102% | 104% |
| Class C | -0.01% | -0.11% | -0.20% | 103% | 101% | 0.01% | -0.02% | -0.07% | 102% | 103% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.02% | 0.03% | -0.11% | 101% | 100% | 0.00% | -0.01% | -0.02% | 102% | 103% |
| Class D | -0.03% | -0.17% | -0.36% | 104% | 102% | 0.01% | -0.03% | -0.09% | 103% | 103% |
| Class F | 0.01% | -0.13% | -0.17% | 102% | 101% | -0.11% | -0.08% | -0.04% | 102% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  | Over VTM-4.0_lowQP |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.01% | -0.26% | -0.48% | 100% | 101% | 0.00% | 0.01% | 0.00% | 101% | 102% |
| Class C | 0.01% | 0.32% | -0.10% | 99% | 97% | -0.01% | 0.00% | 0.03% | 102% | 103% |
| Class E | -0.05% | 0.77% | 0.12% | 100% | 101% | 0.01% | -0.05% | -0.03% | 102% | 102% |
| Overall | -0.01% | 0.19% | -0.20% | 100% | 100% | 0.00% | -0.01% | 0.00% | 102% | 102% |
| Class D | -0.02% | 0.12% | 0.10% | 100% | 98% | -0.01% | -0.01% | -0.05% | 101% | 103% |
| Class F | -0.01% | 0.46% | 0.04% | 87% | 97% | -0.08% | 0.08% | 0.02% | 102% | 103% |

# Conclusions

This document proposes a unified solution for rice parameter derivation of bypass-coded levels in coefficient coding. Since unified method also enables parallel derivation of rice parameters for some positions without performance loss, we suggest inclusion of proposed modification into next version of VTM s/w.

# Reference

- JVET, “CE7-related: Unified rice parameter derivation for coefficient level coding”, JVET-M0198, 13th JVET Meeting, Marrakech, MA, 9-18 Jan., 2019

- JVET, “JVET common test conditions and software reference configurations”, JVET-M1010, 13th JVET Meeting, Marrakech, MA, 9-18 Jan., 2019

- JVET, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding”, JVET-M1027, 13th JVET Meeting, Marrakech, MA, 9-18 Jan., 2019

# Patent rights declaration(s)

Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).