| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0197-v3 |
| --- | --- |

| Title: | CE4: Triangle merge candidate list simplification (Test 4.4.7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Che-Wei Kuo Chong Soon Lim | Tel: Email: | +65-65505423 jingya.li@sg.panasonic.com chewei.kuo@sg.panasonic.com chongsoon.lim@sg.panasonic.com |
| Source: | Panasonic corporation |  |  |

_____________________________

# Abstract

In this contribution, following modifications are proposed to simplify triangle merge candidate list generation. (1) Reuse normal merge candidate list for generation of triangle merge candidates. (2) No pruning or scaling processes for triangle merge candidate list generation. (3) Two triangular partitions must come from difference reference list. It is reported that BD rate change is 0.06% and 0.05% in RA and LDB configurations.

# Introduction

In VTM4.0, triangle merge candidate list is generated separately from normal merge candidate list with full pruning process and scaling process, which brings additional burden to hardware in terms of complexity and number of cycles. Moreover; two triangular partitions are allowed to predict from same reference list, which creates additional complexity for motion vector storage.

# Proposed technology

Following process is proposed to simplify triangle merge list generation:

Firstly, the first four candidates in regular merge candidate list are directly used to construct triangle merge candidate list. The construction process is as following:

For each of the four candidates, if it is bi-prediction, directly copy it to the triangular candidate list. Otherwise if it is uni-prediction, a bi-prediction motion vector is generated and added to triangle merge list by following process:

Let motion vector of the uni-prediction is MVX of LX direction, X = 0, 1; a bi-prediction motion vector is generated consisting of a first motion vector (MVX) and a second motion vector (MVX’) of L(1-X) direction. Two variations of MVX’ are tested.

(test 1) MVX’.x = sign*(MVX.x + offset); MVX’.y = sign*(MVX.y + offset);

(test 2) MVX’.x = sign*(MVX.x) + offset; MVX’.y = sign*(MVX.y) + offset;

* sign = 1 if two reference directions are the same, otherwise sign = -1

* offset = 4 (quarter pel)

Secondly, the first triangle partition selects a first candidate from the triangle candidate list; an additional flag is signaled to indicate the reference direction of the first triangle partition; and motion vector of the first triangle partition is determined as the motion vector of the indicated reference direction from the first candidate. The second triangle partition selects a second candidate from the triangle candidate list, wherein the second candidate is different from the first candidate, and the reference direction of the second triangle partition is derived as not same as the first triangle partition. And motion vector of the second triangle partition is determined as the motion vector of the derived reference direction from the second candidate.

# Complexity analysis

The following table shows the complexity analysis as suggested in N0024. Note for 4.4.7.a and 4.4.7.b, the max number of potential candidates is calculated by summing up:

(stage1) Constructing bi-prediction candidates from the regular merge list

(stage2) Constructing L0-MVs, L1-MVs, and average MVs from stage1

| Test# | Merge list size | Max number of potential candidates (stage1+stage2) | Max number of candidate comparison (stage1+stage2) | Max number of MV scaling (stage1+stage2) | Max number of temporal candidates (stage1+stage2) | Max number of MV average (stage1+stage2) | Max number of encoder MCs (stage1+stage2) | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Regular merge | 6 | 13(5+2+5+1) | 9(5+0+4+0) | 2 | 2 | 2 | 12 |  |
| TPM | 5 | 28(7+21) | 84(10+74) | 11(4+7) | 2(2+0) | 7(0+7) | 5 |  |
| 4.4.7.a | 4 | 20(13+(4+3+0)) | 9 | 2 | 2 | 2 | 8 | No mapping logic |
| 4.4.7.b | 4 | 20(13+(4+3+0)) | 9 | 2 | 2 | 2 | 8 | No mapping logic |

# Simulation results

The following experiments were conducted using the VTM-4.0 software with the JVET common test conditions [1] to collect the B-D rate, encoding and decoding time of the proposed techniques in the Random Access and Low Delay B configurations. Table 1 to Table 2 show the results of the two variations.

Table 1: Results of test1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.07% | 0.17% | 102% | 101% |
| Class A2 | -0.02% | -0.05% | 0.01% | 103% | 100% |
| Class B | 0.05% | -0.03% | 0.05% | 102% | 100% |
| Class C | 0.15% | 0.14% | 0.03% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.03% | 0.06% | 102% | 100% |
| Class D | 0.05% | 0.16% | 0.01% | 103% | 100% |
| Class F | 0.12% | 0.15% | 0.07% | 103% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.13% | -0.23% | 103% | 103% |
| Class C | 0.04% | 0.03% | -0.07% | 103% | 100% |
| Class E | 0.10% | -0.27% | -0.39% | 104% | 100% |
| Overall | 0.05% | -0.11% | -0.22% | 103% | 101% |
| Class D | 0.01% | 0.12% | -0.63% | 103% | 100% |
| Class F | -0.10% | 0.27% | -0.36% | 104% | 101% |

Table 2: Results of test2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.08% | 0.21% | 103% | 99% |
| Class A2 | -0.02% | -0.07% | 0.02% | 103% | 100% |
| Class B | 0.04% | -0.04% | 0.04% | 103% | 100% |
| Class C | 0.15% | 0.17% | 0.07% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.03% | 0.08% | 103% | 100% |
| Class D | 0.05% | 0.12% | -0.01% | 103% | 110% |
| Class F | 0.11% | 0.16% | 0.04% | 103% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.13% | -0.23% | 104% | 101% |
| Class C | 0.04% | 0.03% | -0.07% | 103% | 99% |
| Class E | 0.10% | -0.27% | -0.39% | 105% | 100% |
| Overall | 0.05% | -0.11% | -0.22% | 104% | 100% |
| Class D | 0.01% | 0.12% | -0.63% | 104% | 101% |
| Class F | -0.10% | 0.27% | -0.36% | 104% | 100% |

# Conclusion

This contribution proposes three modifications to simplify triangle merge candidate list generation. It is reported that in test2, 0.06% and 0.05% B-D rate reduction is achieved in random access configuration and low delay B configuration, respectively. Simulation results show that there is no significant impact on encoding and decoding time. It is suggested to adopt proposed method to the next VVC working draft.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Macao, October 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Specification text

#### Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| ciip_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| ciip_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && tile_group_type = = B && ciip_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight >= 64 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_first_partition_dir[x0][y0] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### Derivation process for luma motion vectors for merge triangle mode

This process is only invoked when merge_triangle_flag[ xCb ][ yCb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvLA and mvLB,

- the reference indices refIdxLA and refIdxLB,

- the prediction list flags predFlagLA and predFlagLB.

The motion vectors mvLA and mvLB, the reference indices refIdxLA and refIdxLB and the prediction flags predFlagLA and predFlagLB are derived by the following ordered steps:

- 1. The derivation process for merging candidates from neighbouring coding units as specified in clause 8.5.2 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, and the luma coding block height cbHeight as inputs, and the output being merge candidate list.

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- The following applies:

  - The variables m and n, being the candidates in the triangle merging candidate list mergeTriangleCandList, are derived using merge_triangle_idx0[ xCb ][ yCb ] and as merge_triangle_idx1[ xCb ][ yCb ] follows:

m = merge_triangle_idx0[ xCb ][ yCb ]		(8-491)

n = merge_triangle_idx1[ xCb ][ yCb ] + ( merge_triangle_idx1[ xCb ][ yCb ] >= m ) ? 1 : 0	(8-492)

  - The following assignments are made with M and N being the candidates at position m and n in the triangle merging candidate list mergeTriangleCandList ( M = mergeTriangleCandList[ m ] and N = mergeTriangleCandList[ n ] ):

predFlagLA = (merge_triangle_first_partition_dir[x0][y0]  = =  0 ) ? 0 : 1	(8-493)

predFlagLB = (merge_triangle_first_partition_dir[x0][y0]  = =  0 ) ? 1 : 0	(8-494)

refIdxLA = (merge_triangle_first_partition_dir[x0][y0]  = =  0) ? refIdxL0M : refIdxL1M	(8-495)

refIdxLB = (merge_triangle_first_partition_dir[x0][y0]  = =  0) ? refIdxL0N : refIdxL1N	(8-496)

mvLA[ 0 ] = (merge_triangle_first_partition_dir[x0][y0]  = =  0) ? mvL0M[ 0 ] : mvL1M[ 0 ]	(8-497)

mvLA[ 1 ] = (merge_triangle_first_partition_dir[x0][y0]  = =  0) ? mvL0M[ 1 ] : mvL1M[ 1 ]	(8-498)

mvLB[ 0 ] = (merge_triangle_first_partition_dir[x0][y0]  = =  0) ? mvL0N[ 0 ] : mvL1N[ 0 ]	(8-499)

mvLB[ 1 ] = (merge_triangle_first_partition_dir[x0][y0]  = =  0) ? mvL0N[ 1 ] : mvL1N[ 1 ]

-

-

-

-

-

-

-

#### Derivation process for bi-prediction triangle merging candidates

Inputs to this process are:

- merging candidate list mergeCandList.

Outputs of this process are:

- triangle merging candidate list mergeTriangleCandList,

The following steps are performed:

- For Candk being equal to each of first four candidates in mergeCandList the following applies:

  - If refIdxL0Candk != -1 and refIdxL1Candk != -1, add Candk to mergeTriangleList;

  - Else if refIdxL0Candk != -1 and refIdxL1Candk == -1,

      - Add motion vector Candk’to mergeTriangleList, wherein Candk’ is derived as follows:

currPocDiffL0  =  DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0Candk ] )

currPocDiffL1  =  DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL0Candk ] )

sign = 	currPocDiffL0* currPocDiffL1>0?1:-1

Candk’ = Candk

refIdxL1Candk’ = refIdxL0Candk’

mvL1Candk’[0] = sign*(mvL0Candk[0]) + 4

mvL1Candk’[1] = sign*(mvL0Candk[0]) + 4

  - Else if refIdxL0Candk == -1 and refIdxL1Candk != -1,

      - Add motion vector Candk’to mergeTriangleList, wherein Candk’ is derived as follows:

currPocDiffL0  =  DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL1Candk ] )

currPocDiffL1  =  DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1Candk ] )

sign = 	currPocDiffL0* currPocDiffL1>0?1:-1

Candk’ = Candk

refIdxL0Candk’ = refIdxL1Candk’

mvL0Candk’[0] = sign*(mvL1Candk[0]) + 4

mvL0Candk’[1] = sign*(mvL1Candk[0]) + 4

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

#### Motion vector storing process for triangle merge mode

This process is invoked when decoding a coding unit with merge_triangle_flag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable triangleDir specifying the partition direction,

- the luma motion vectors in 1/16 fractional-sample accuracy mvLA and mvLB,

- the reference indices refIdxLA and refIdxLB,

- the prediction list flags predFlagLA and predFlagLB.

The variables numSbX and numSbY specifying the number of 4x4 blocks in the current coding block in horizontal and vertical direction are set equal to numSbX = cbWidth >> 2 and numSbY = cbHeight >> 2.

The variable minSb is set equal to min( numSbX, numSbY ).

The variable cbRatio is derived as follows:

cbRatio = ( cbWidth > cbHeight ) ? ( cbWidth / cbHeight ) : ( cbHeight / cbWidth )	(8-896)

For each 4x4 subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the following applies:

- The variables xIdx and yIdx are derived as follows:

xIdx = ( cbWidth > cbHeight )  ?  ( xSbIdx / cbRatio ) : xSbIdx	(8-897)

yIdx = ( cbWidth > cbHeight )  ?  ySbIdx  :  ( ySbIdx / cbRatio )	(8-898)

- The variable sType is derived as follows:

- If triangleDir is equal to 0, the following applies:

sType = ( xIdx = = yIdx )  ?  2  :  ( ( xIdx > yIdx )  ?  0  :  1 )	(8-899)

- Otherwise (triangleDir is equal to 1), the following applies:

sType = ( xIdx + yIdx = = minSb )  ?  2  :  ( ( xIdx + yIdx < minSb )  ?  0  :  1 )	(8-900)

- Depending on the value of sType, the following assignments are made:

- If sType is equal to 0, the following applies:

predFlagL0 = ( predFlagLA = = 0 )  ?  1  :  0		(8-901)

predFlagL1 = ( predFlagLA = = 0 )  ?  0  :  1		(8-902)

refIdxL0 = ( predFlagLA = = 0 )  ?  refIdxLA : −1	(8-903)

refIdxL1 = ( predFlagLA = = 0 )  ?  −1  :  refIdxLA	(8-904)

mvL0[ 0 ] = ( predFlagLA = = 0 )  ?  mvLA[ 0 ]  :  0	(8-905)

mvL0[ 1 ] = ( predFlagLA = = 0 )  ?  mvLA[ 1 ]  :  0	(8-906)

mvL1[ 0 ] = ( predFlagLA = = 0 )  ?  0  :  mvLA[ 0 ]	(8-907)

mvL1[ 1 ] = ( predFlagLA = = 0 )  ?  0  :  mvLA[ 1 ]	(8-908)

- Otherwise, if sType is equal to 1, the following applies:

predFlagL0 = ( predFlagLB = = 0 )  ?  1  :  0		(8-909)

predFlagL1 = ( predFlagLB = = 0 )  ?  0  :  1		(8-910)

refIdxL0 = ( predFlagLB = = 0 )  ?  refIdxLB : −1	(8-911)

refIdxL1 = ( predFlagLB = = 0 )  ?  −1  :  refIdxLB	(8-912)

mvL0[ 0 ] = ( predFlagLB = = 0 )  ?  mvLB[ 0 ]  :  0	(8-913)

mvL0[ 1 ] = ( predFlagLB = = 0 )  ?  mvLB[ 1 ]  :  0	(8-914)

mvL1[ 0 ] = ( predFlagLB = = 0 )  ?  0  :  mvLB[ 0 ]	(8-915)

mvL1[ 1 ] = ( predFlagLB = = 0 )  ?  0  :  mvLB[ 1 ]	(8-916)

- Otherwise (sType is equal to 2), the following applies:

predFlagL0 = 1		(8-917)

predFlagL1 = 1		(8-918)

refIdxL0 = ( predFlagLA = = 0 )  ?  refIdxLA  :  refIdxLB	(8-919)

refIdxL1 = ( predFlagLA = = 0 )  ?  refIdxLB  :  refIdxLA	(8-920)

mvL0[ 0 ] = ( predFlagLA = = 0 )  ?  mvLA[ 0 ]  :  mvLB[ 0 ]	(8-921)

mvL0[ 1 ] = ( predFlagLA = = 0 )  ?  mvLA[ 1 ]  :  mvLB[ 1 ]	(8-922)

mvL1[ 0 ] = ( predFlagLA = = 0 )  ?  mvLB[ 0 ]  :  mvLA[ 0 ]	(8-923)

mvL1[ 1 ] = ( predFlagLA = = 0 )  ?  mvLB[ 1 ]  :  mvLA[ 1 ]	(8-924)

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

- The following assignments are made for x = 0..3 and y = 0..3:

MvL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL0	(8-941)

MvL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL1	(8-942)

RefIdxL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL0	(8-943)

RedIdxL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL1	(8-944)

PredFlagL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL0	(8-945)

PredFlagL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL1