| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0210-v2 |
| --- | --- |

| Title: | CE2-4.9: Integer motion compensation for regular inter mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Xiaozhen Zheng, Shanshe Wang, Siwei Ma | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd., |  |  |

_____________________________

- Abstract

In current VVC standard, 4x4 bi-prediction inter CUs are disallowed because of the memory bandwidth issue. Moreover, the use of 4x8/8x4 bi-prediction inter CUs cause the increase of memory bandwidth. This contribution proposes an improvement method that use integer motion compensation for 4x8/8x4 bi-prediction, which allows 4x8/8x4 bi-prediction and avoid the increase of memory bandwidth compare to HEVC. The proposed method can reduce the worst-case memory access bandwidth by about 32% compared to the current VVC, with 0.06% and 0.07% bit-rate increase and xxx% encoding and xxx% decoding time reduction respectively under RA and LDB cases. Compare to disallow 4x8/8x4 bi-prediction, the proposed method can achieve better performance in Class F and almost the same performance in other SDR test classes.

Introduction

Motion compensation (MC) is one of the largest memory bandwidth consumers in decoder side. Providing a reasonable limit on MC bandwidth requirement is important for the implementations of video coding standard.

Memory bandwidth requirement for MC is usually determined by motion compensation block size and the type of prediction (e.g. uni-directional or bi-directional). In the current VVC (VTM-4.0), the 4x4 bi-directional inter CUs are disallowed because of the more than 2x memory bandwidth is required compared to HEVC. However, there is no limitation for 4x8 and 8x4 bi-directional inter CUs. As shown in Figure 1, VTM-4.0 can increase 47% memory bandwidth in worst case compared to HEVC.

Considering that the increase of memory bandwidth for 4x8/8x4 bi-directional inter CUs is caused by fractional pixel motion compensation, this contribution provides another approach that avoids to increase memory bandwidth as well as allows 4x8/8x4 bi-prediction by using 1- pixel and 4-pixel motion vector resolutions compensation at AMVR and 1-pixel at merge, MMVD. It can improve coding efficiency and unify inter prediction syntax for inter CUs.

Figure 1: HEVC and VVC worst-case reference sample number

Proposed method

In VTM-4.0, AMVR has three motion vectors resolutions: fractional-pixel, full-pixel and four-pixel resolutions. If current CU is motion compensated by full-pixel or four-pixel, the interpolation process is not needed and the motion compensated block size is same as the size of current CU. Therefore, the memory bandwidth doesn’t increase.

The proposal suggests using 1-pixel and 4-pixel motion compensation for 4x8 / 8x4 bi-prediction at AMVR mode and 1-pixel motion compensation for 4x8 / 8x4 bi-prediction at Merge, MMVD mode.

There are four solutions to achieve the proposed method:

- Select 1-pixel and 4-pixel candidates at AMVR mode, and round merge candidates to 1-pixel resolution when 4x8 / 8x4 bi-prediction is used; Enable 4x4 bi-prediction with integer motion compensation at AMVR, Merge and MMVD modes.

- Select 1-pixel and 4-pixel candidates at AMVR mode, and round motion vector to 1-pixel resolution at motion compensation process when 4x8 / 8x4 bi-prediction is used; Enable 4x4 bi-prediction with integer motion compensation at AMVR, Merge and MMVD modes.

- Select 1-pixel and 4-pixel candidates at AMVR mode, and round merge candidates to 1-pixel resolution when 4x8 / 8x4 bi-prediction is used;

- Select 1-pixel and 4-pixel candidates at AMVR mode, and round motion vector to 1-pixel resolution at motion compensation process when 4x8 / 8x4 bi-prediction is used.

The above methods can reduce further worst-case memory bandwidth and keep the coding performance. There is no need to modify the current standard specification.

Simulation results

The simulations are conducted following the common test conditions [3] on top of reference software VTM-4.0. The encoding time is unbelievable. The tested solutions are summarized as below:

Test 2.4.9-1: Select 1-pixel and 4-pixel candidates at AMVR mode, and round merge candidates to 1-pixel resolution when 4x8 / 8x4 bi-prediction is used; Enable 4x4 bi-prediction with integer motion compensation at AMVR, Merge and MMVD modes.

Test 2.4.9-2: Select 1-pixel and 4-pixel candidates at AMVR mode, and round motion vector to 1-pixel resolution at motion compensation process when 4x8 / 8x4 bi-prediction is used; Enable 4x4 bi-prediction with integer motion compensation at AMVR, Merge and MMVD modes.

Test 2.4.9-3: Select 1-pixel and 4-pixel candidates at AMVR mode, and round merge candidates to 1-pixel resolution when 4x8 / 8x4 bi-prediction is used;

Test 2.4.9-4: Select 1-pixel and 4-pixel candidates at AMVR mode, and round motion vector to 1-pixel resolution at motion compensation process when 4x8 / 8x4 bi-prediction is used.

Table 1. Performance of proposed method over VTM4.0 (Test2.4.9-1)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.08% | 0.09% | 120% | 96% |
| Class A2 | 0.06% | 0.07% | 0.16% | 91% | 96% |
| Class B | 0.07% | 0.16% | 0.19% | 101% | 96% |
| Class C | 0.19% | 0.23% | 0.22% | 100% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.11% | 0.18% | 102% | 96% |
| Class D | 0.28% | 0.35% | 0.33% | 100% | 95% |
| Class F | 0.05% | 0.08% | 0.02% | 100% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | 0.07% | 0.15% | 100% | 96% |
| Class C | 0.18% | 0.52% | 0.30% | 101% | 96% |
| Class E | 0.00% | 0.58% | -0.16% | 101% | 96% |
| Overall | 0.09% | 0.35% | 0.12% | 101% | 96% |
| Class D | 0.30% | 0.57% | -0.06% | 102% | 95% |
| Class F | -0.05% | 0.34% | -0.27% | 71% | 96% |

Table 2. Performance of disallow 4x4/4x8/8x4 bi-directional inter CUs (Test2.4.9-2)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.03% | 0.06% | 102% | 100% |
| Class A2 | 0.04% | 0.10% | 0.20% | 81% | 100% |
| Class B | 0.06% | 0.19% | 0.34% | 100% | 100% |
| Class C | 0.14% | 0.23% | 0.17% | 106% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.15% | 0.21% | 98% | 100% |
| Class D | 0.22% | 0.39% | 0.49% | 101% | 100% |
| Class F | 0.01% | 0.06% | -0.01% | 106% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.11% | 0.36% | 100% | 105% |
| Class C | 0.18% | 0.36% | 0.35% | 101% | 100% |
| Class E | -0.02% | 0.04% | -0.38% | 100% | 100% |
| Overall | 0.07% | 0.17% | 0.17% | 100% | 102% |
| Class D | 0.33% | 0.83% | -0.14% | 102% | 100% |
| Class F | -0.10% | -0.03% | 0.17% | 69% | 100% |

Conclusions

This contribution proposes a harmonization method between 4x4/4x8/8x4 bi-directional inter CU and full pixel and 4-pixel motion vector resolution AMVR mode. It can further reduce the worst-case memory access bandwidth compared to the current VVC, with 0.06% and 0.07% bit-rate increase and no encoding and decoding time increase respectively under RA and LDB cases. The performance is better in ClassF than disallowing 4x4/4x8/4x8 bi-prediction directly. Meanwhile, it can unify syntax for all inter CUs. And there is no need to do the subpixel interpolation process for 4x4/4x8/8x4 bi-prediction in the proposed method. Therefore, we recommend to integrate the proposed method into the next version of VVC standard and VTM software.

References

- Yi-Wen Chen, Xianglin Wang, JVET-L0104, “AHG5: Reducing VVC worst-case memory bandwidth by restricting bi-directional 4x4 inter CUs/Sub-blocks”, Macao, October 2018.

- Xuewei Meng, Xiaozhen Zheng, Shanshe Wang, Siwei Ma, JVET-M0348, “ CE2/CE4-related: Further reducing VVC memory bandwidth worst case by combining 4x4/4x8/8x4 bi-prediction with AMVR”, Marrakech, January 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-M1010, “JVET common test conditions and software reference configurations for SDR video”, Marrakech, January 2019.

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).