| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0211-v2 |
| --- | --- |

| Title: | CE4-4.8 : Simplification of triangle prediction merging candidate list derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Xiaozhen Zheng, Shanshe Wang, Siwei Ma | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd., |  |  |

_____________________________

- Abstract

This contribution proposes a simplification method of triangle prediction merge list construction in VTM-4.0[1]. The proposed method reusing inter merge list as triangle prediction’s merge list. It is reported that as compared to VTM-4.0, 0.07% B-D rate increase is achieved in random access configuration. And 0.04% B-D rate increase is achieved in low delay B configuration when using the whole merge list. When using only the spatial and temporal merge list, 0.03% coding gain and 0.05 coding loss is achieved in random access and low delay B configuration respectively.

Introduction

The uni-prediction candidate list of triangle prediction consists of five uni-prediction motion vector candidates. It is derived from seven neighboring blocks including five spatial neighboring blocks (1 to 5) and two temporal co-located blocks (6 to 7), as shown in Figure 1. The motion vectors of the seven neighboring blocks are collected and put into the uni-prediction candidate list according in the order of uni-prediction motion vectors, L0 motion vector of bi-prediction motion vectors, L1 motion vector of bi-prediction motion vectors, and averaged motion vector of the L0 and L1 motion vectors of bi-prediction motion vectors. If the number of candidates is less than five, zero motion vector is added to the list.

Figure 1: Position of the neighboring blocks

Proposed method

Considering both inter merge mode and triangle mode need to construct merge list and derive motion information from the lists, this contribution proposes to reuse current inter merge list as the list for triangle mode as it can remove triangle merge list construction and simply motion information derivation of triangle mode. To further reduction motion derivation complexity for triangle mode, this contribute also proposes to use candidates before HMVP in the inter merge list, which means only four spatial candidates and tmvp candidates in the list are used at triangle motion information derivation.

Simulation results

The simulations are conducted following the common test conditions [1] on top of reference software VTM-4.0. The encoding time is unbelievable.

Test 4.4.8.a: Use the whole merge list as triangle merge list.

Teat 4.4.8.b: Only use the spatial and TMVP merge candidates as triangle merge list

Table 1. Performance of proposed method 1 over VTM4.0 (CE4-4.8a)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.06% | 0.09% | 102% | 100% |
| Class A2 | 0.02% | 0.03% | 0.07% | 80% | 100% |
| Class B | 0.05% | -0.01% | 0.00% | 100% | 100% |
| Class C | 0.15% | 0.14% | 0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.03% | 0.06% | 96% | 100% |
| Class D | 0.11% | 0.15% | 0.20% | 100% | 101% |
| Class F | 0.06% | 0.12% | 0.06% | 102% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.04% | -0.16% | 101% | 105% |
| Class C | 0.11% | 0.39% | 0.37% | 100% | 100% |
| Class E | -0.05% | 0.49% | -0.21% | 101% | 100% |
| Overall | 0.04% | 0.27% | 0.00% | 101% | 102% |
| Class D | 0.05% | 0.63% | -0.52% | 101% | 101% |
| Class F | -0.09% | -0.46% | -0.10% | 70% | 100% |

Table 2. Performance of proposed method 2 over VTM4.0 (CE4-4.8b)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.05% | -0.03% | 101% | 96% |
| Class A2 | -0.05% | -0.05% | -0.10% | 81% | 96% |
| Class B | -0.01% | 0.01% | -0.10% | 100% | 97% |
| Class C | -0.02% | -0.10% | -0.11% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.04% | -0.09% | 96% | 97% |
| Class D | 0.00% | -0.12% | -0.13% | 102% | 98% |
| Class F | -0.01% | 0.02% | 0.00% | 100% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.08% | -0.15% | 101% | 101% |
| Class C | 0.04% | 0.30% | 0.15% | 101% | 96% |
| Class E | 0.07% | 0.11% | -0.28% | 101% | 96% |
| Overall | 0.05% | 0.16% | -0.08% | 101% | 98% |
| Class D | 0.04% | 0.04% | -0.51% | 101% | 97% |
| Class F | 0.00% | 0.44% | 1.03% | 70% | 96% |

Conclusions

In this proposal, a simplified triangle prediction merging candidate list derivation method is proposed. It harmonizes merge list construction between inter merge mode and triangle mode. The proposed method can reduce the complexity of motion information derivation for triangle mode. Compared to VTM4.0 anchor, minor coding lost is observed. Moreover, there are some coding gain in random access configuration when using only spatial and tmvp candidates.

References

- Ru-Ling Liao, Chong Soon Lim, JVET-L0124, “CE10.3.1.b: Triangular prediction unit mode”, Macao, October 2018.

- Xuewei Meng, Xiaozhen Zheng, Shanshe Wang, Siwei Ma, JVET-M0349, “ CE10-related: Simplification of triangle prediction merging candidate list derivation”, Marrakech, January 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-M1010, “JVET common test conditions and software reference configurations for SDR video”, Marrakech, January 2019.

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).