| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0225-v2 |
| --- | --- |

| Title: | Various chroma format support in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li Junghak Nam Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {l.li, junghak.nam, jin78.heo, jaehyun.lim, seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this contribution, several aspects are proposed to support various chroma formats in VVC.

Introduction

In current VVC Draft 4, only chroma 4:2:0 are mainly considered. It is desirable for VVC to support various chroma formats. Several aspects are proposed to support that.

Proposed Aspect 1

It is proposed to parse colour_plane_id in tile_group_header when separate_colour_plane_flag is equal to 1.

Table 1 signaling of colour_plane_id in tile_group_header

| tile_group_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| … |  |
| } |  |

colour_plane_id specifies the colour plane associated with the current tile RBSP when separate_colour_plane_flag is equal to 1. The value of colour_plane_id shall be in the range of 0 to 2, inclusive. colour_plane_id values 0, 1 and 2 correspond to the Y, Cb and Cr planes, respectively.

NOTE – : There is no dependency between the decoding processes of pictures having different values of colour_plane_id.

Proposed Aspect 2

It is proposed to parse qtbtt_dual_tree_intra_flag when ChromaArrayType is not equal to 0.

Table 2 signaling of implicit split flag for dual tree of I tile group

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType != 0 ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| … |  |
| } |  |

qtbtt_dual_tree_intra_flag equal to 1 specifies that for I tile groups, each CTU is split into coding units with 64x64 luma samples using an implicit quadtree split and that these coding units are the root of two separate coding_tree syntax structure for luma and chroma. qtbtt_dual_tree_intra_flag equal to 0 specifies implicit qualtree split is not used for I tiel groups. When qtbtt_dual_tree_intra_flag is not present, it is inferred to be equal to 0

Proposed Aspect 3

It is proposed to parse sps_cclm_enabled_flag when ChromaArrayType is not equal to 0.

Table 3 signaling of sps_cclm_enabled_flag

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType != 0 ) |  |
| sps_cclm_enabled_flag | u(1) |
| … |  |
| } |  |

sps_cclm_enabled_flag equal to 0 specifies that the cross-component linear model intra prediction from luma component to chroma component is disabled. sps_cclm_enabled_flag equal to 1 specifies that the cross-component linear model intra prediction from luma component to chroma componenent is enabled. When sps_cclm_enabled_flag is not present, it is inferred to be equal to 0

Proposed Aspect 4

It is proposed to parse alf_chroma_idc when ChromaArrayType is not equal to 0.

Table 4 signaling of alf_chroma_idc

| alf_data( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType != 0 ) |  |
| alf_chroma_idc | tu(v) |
| … |  |
| } |  |

alf_chroma_idc equal to 0 specifies that the adaptive loop filter is not applied to Cb and Cr colour components. alf_chroma_idc equal to 1 indicates that the adaptive loop filter is applied to the Cb colour component. alf_chroma_idc equal to 2 indicates that the adaptive loop filter is applied to the Cr colour component. alf_chroma_idc equal to 3 indicates that the adaptive loop filter is applied to Cb and Cr colour components. When alf_chroma_idc is not present, it is inferred to be equal to 0

Proposed Aspect 5

It is proposed to parse intra_chroma_pred_mode when ChromaArrayType is not equal to 0.

Table 5 signaling of intra_chroma_pred_mode

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType != 0 ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| … |  |
| } |  |

intra_chroma_pred_mode[ x0 ][ y0 ] specifies the intra prediction mode for chroma samples. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When intra_chroma_pred_mode is not present, it is inferred to be equal to 0

Proposed Aspect 6

It is proposed to parse pcm_sample_chroma when ChromaArrayType is not equal to 0.

Table 6 Signaling of pcm_sample_chroma

| pcm_sample( cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if (ChromaArrayType !=0 ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA) { |  |
| for( i = 0; i < 2 * ( ( cbWidth * cbHeight ) / ( SubWidthC * SubHeightC ) ); i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

Proposed Aspect 7

It is proposed to parse tu_cbf_cb and tu_cbf_cr when ChromaArrayType is not equal to 0.

Table 7 Signaling of tu_cbf_cb and tu_cbf_cr

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| … |  |
| if(chromaArrayType !=0) { |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

Proposed Specification Text

Please see attached specification text for detail.

Summary

In this contribution several aspects are proposed to support various chroma formats in VVC. It is recommended to adopt this proposal so that VVC could support various chroma formats.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).