| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0232_v2 |
| --- | --- |

| Title: | [AHG8] Coding performance of 360° Videos in PHEC format with different face sizes |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Amith DSouza Chirag Pujara Raj Narayana Gadde Woong-Il Choi Kiho Choi Kwang Pyo Choi | Tel: Email: | amith.ds@samsung.com chirag.p@samsung.com raj.gadde@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

Omnidirectional videos are one of the categories of videos considered for evaluating the performance of Versatile Video Coding. Cubemap based formats are shown to perform better than other projection formats for encoding 360° Videos. In this contribution, the coding performance of PHEC is computed at different face sizes. It is observed that the coding efficiency improves as face size increases. It is also noted that the bitrate and decoding complexity also increase linearly as face size increases.

# Introduction

Encoding 360° videos requires that the videos are converted into a projection format. The AHG6 on 360° video coding tools and test conditions use PHEC with face size of 1280x1280 as the anchor. It is shown here that the face size of 1280x1280 is not the optimal face size for encoding 360° videos for PHEC. It is observed that larger face sizes show higher compression efficiency. However, the decoding complexity and bitrate of 360° videos increases linearly as the face size increases.

# Results

A brief summary of the results at different face sizes when compared with the AHG anchor is shown below. The results have been generated using 360Lib8.0 and VTM3.0. However, we believe that VTM4.0 would also show similar characteristics.

|  | End-to-end WS-PSNR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT* | DecT | Bitrate |
| 1280 vs 1152 | 2.49% | 3.07% | 2.82% | 80% | 81% | 88% |
| 1280 vs 1184 | 1.91% | 2.55% | 2.44% | 88% | 87% | 91% |
| 1280 vs 1216 | 1.10% | 1.59% | 1.37% | 92% | 91% | 94% |
| 1280 vs 1248 | 0.68% | 1.15% | 1.08% | 94% | 96% | 97% |
| 1280 vs 1280 | 0.00% | 0.00% | 0.00% | 100% | 100% | 100% |
| 1280 vs 1312 | -0.18% | -0.13% | -0.07% | 120% | 105% | 103% |
| 1280 vs 1344 | -0.60% | -1.08% | -1.00% | 132% | 110% | 106% |
| 1280 vs 1376 | -0.67% | -1.32% | -1.16% | 132% | 115% | 109% |
| 1280 vs 1408 | -1.11% | -2.39% | -2.20% | 137% | 120% | 112% |
| 1280 vs 1440 | -1.02% | -2.44% | -2.14% | 143% | 126% | 115% |
| 1280 vs 1472 | -1.25% | -3.27% | -2.92% | 150% | 132% | 118% |
| 1280 vs 1504 | -1.21% | -3.50% | -3.32% | 156% | 138% | 122% |
| 1280 vs 1536 | -1.52% | -4.23% | -3.89% | 147% | 143% | 125% |
| 1280 vs 1568 | -1.29% | -4.45% | -4.00% | 146% | 150% | 128% |

Table 1.Coding efficiency at different face sizes as compared to 1280x1280

*Encoder times are unreliable due to cluster computing.

Fig 1. Plot of average coding efficiency at different face sizes for all sequences

Fig 2. Plot of average coding efficiency for different face sizes for Class S1 sequences.

Fig 3. Plot of average coding efficiency for different face sizes for Class S2 sequences

# Conclusion

It is observed that the coding efficiency of 360° videos is highly dependent on the face size. Using face size of 1536x1536 shows 1.5% higher coding efficiency at 25% bitrate increase and 43% higher decoding complexity.

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).