| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0237 |
| --- | --- |

| Title: | CE4-1.3: Modification of merge data syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geonjung Ko, Dongcheol Kim, Jason Jung, Ju-Hyung (John) Son, Jin Sam Kwak | Tel: Email: | +82-31-712-0523 greg.ko@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

# Abstract

This contribution reports test results of CE4-1.3 that was presented in JVET-M0359. The current MMVD uses only the first or second merge candidate as a base candidate. Therefore, it is proposed that mmvd_flag is parsed only when the merge candidate index is equal to 0 or 1. Since mmvd_merge_flag, which specifies the merge candidate index for MMVD, is unnecessary in the proposed method, mmvd_merge_flag and its context model are removed. It reportedly shows -0.02%/-0.17%/-0.11% (Y/U/V) average BD-rate change with RA configuration and -0.01%/0.08%/-0.29% (Y/U/V) average BD-rate change with LDB configuration.

# Proposed method

The proposed method was presented in JVET-M0359 [1]. It is proposed that mmvd_flag is parsed when merge_idx is equal to 0 or 1. This is because the current MMVD uses the first or second merge candidate as a base candidate [2]. Instead of mmvd_merge_flag, merge_idx can specify the merge candidate index for MMVD in the proposed method, and there is no need for separate mmvd_merge_flag and its context model. By the proposed modification, mmvd_flag is not parsed when a merge mode other than MMVD is used. Table 1 shows the modified merge data syntax. The changes are highlighted in yellow and strikethrough.

Table 1. The modified merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| ciip_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| ciip_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && tile_group_type = = B && ciip_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight >= 64 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( merge_idx[ x0 ][ y0 ] < 2 && !ciip_flag[ x0 ][ y0 ] && !merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Experimental results

CE4-1.3 was implemented on top of VTM-4.0 and tested under the common test conditions [3]. Table 2 and Table 3 summarize experimental results for random access and low delay B configurations, respectively.

Table 2. Experimental results of the proposed method with RA configuration

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.15% | -0.02% | 101% | 100% |
| Class A2 | -0.01% | -0.18% | 0.04% | 101% | 100% |
| Class B | -0.02% | -0.15% | -0.14% | 101% | 100% |
| Class C | -0.04% | -0.21% | -0.27% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.17% | -0.11% | 101% | 100% |
| Class D | -0.01% | -0.09% | -0.28% | 100% | 102% |
| Class F | -0.08% | -0.08% | -0.17% | 100% | 101% |

Table 3. Experimental results of the proposed method with LDB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.06% | 0.05% | 101% | 101% |
| Class C | 0.00% | 0.18% | 0.02% | 101% | 101% |
| Class E | -0.03% | 0.19% | -1.26% | 101% | 98% |
| Overall | -0.01% | 0.08% | -0.29% | 101% | 100% |
| Class D | -0.02% | 0.28% | -0.67% | 101% | 100% |
| Class F | -0.11% | -0.25% | -0.50% | 100% | 101% |

# Conclusion

This contribution reports test results of CE4-1.3 and proposes to modify merge data syntax to reduce signaling overhead when MMVD is not used. The proposal shows -0.02%/-0.17%/-0.11% (Y/U/V) average BD-rate change with RA configuration and -0.01%/0.08%/-0.29% (Y/U/V) average BD-rate change with LDB configuration. Furthermore, the proposal reduces the number of syntax elements and context models. It is suggested to adopt the proposed change in the next version of VVC draft text and the reference software.

# References

[1] G. Ko, D. Kim, J. Jung, J. Son, J. Kwak, “Non-CE4: Modification of merge data syntax,” JVET-M0359, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

[2] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

[3] F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).