| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0239 |
| --- | --- |

| Title: | CE9-related: BDOF-BWA unification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin, T. Poirier, P. Bordes | Email: | franck.galpin@technicolor.com tangi.poirier@technicolor.com philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution unified the usage of BDOF and BWA in VTM-4.0. The reported simulation results show an average luma BD-rate gain of -0.08% in RA configuration, for a decoder complexity of 102%.

# Introduction

In the current VTM-4.0, usage of Bi-Directional Optical Flow (BDOF a.k.a BIO) and Bi-prediction with weighted averaging (BWA aka GBI) are mutually exclusive.

This contribution proposes to allow the usage of BDOF when BWA is enabled.

# Description

The BDOF process consists in correcting the bi-prediction by adding a correction term as follow [1]:

predBDOFx,y=I0x,y+I1x,y+b(x,y)+οoffset≫shift (1)

Where b(x,y) is the correction term depending on the optical flow solved for each 4x4 sub-block.

The BWA consists in computing a weighted average of the bi-prediction as follow [1]:

Pbi-pred=8-w*P0+w*P1+4≫3 (2)

This proposal unified the 2 equations as:

predBDOFx,y=8-wI0x,y+w I1x,y+bx,y+οoffset≫shift

The bit depth for the computation is the same as in BWA and the correction term b(x,y) computation is changed accordingly. The bitdepth of intermediate stored data is not changed. In practice, the core loop operations are changed as follow:

Original BDOF pixel based process

b = tmpx * (gradX0[x] - gradX1[x]) + tmpy * (gradY0[x] - gradY1[x]); b = ((b + 1) >> 1); dst[x] = ClipPel(rightShift((src0[x] + src1[x] + b + offset), shift), clpRng);

BDOF-BWA pixel based process

int b = tmpx * (gradX0[0] - gradX1[0]) + tmpy * (gradY0[0] - gradY1[0]); dst[x] = ClipPel(rightShift(( (w0*src0[x] + w1*src1[x])+b+offset),shift), clpRng);

Overall, 1 addition and 1 shift operation are removed while 2 multiplications are added (note that these multiplications can be done with shift and addition as the set of weights is known and limited).

A scalar and a SIMD implementation are provided, similar to the BDOF implementation.

# Simulation results

Table below show the simulation results of the proposed method in random access. In both configurations, the proposed method allows improvements of the BD-rate performances. Encoding times are less reliable (turbo mode activated).

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.13% | 0.09% | 105% | 102% |
| Class A2 | -0.10% | 0.04% | 0.06% | 104% | 103% |
| Class B | -0.07% | 0.06% | 0.00% | 104% | 102% |
| Class C | -0.11% | -0.02% | -0.03% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | 0.05% | 0.02% | 104% | 102% |
| Class D | -0.16% | -0.04% | -0.05% | 101% | 101% |
| Class F | 0.02% | 0.03% | 0.03% | 104% | 103% |

The usage of BDOF is changed as follow:

|  | BDOF usage |  |
| --- | --- | --- |
|  | VTM-4.0 | BDOF-BWA |
| Class A1 | 39.8% | 49.0% |
| Class A2 | 46.4% | 54.5% |
| Class B | 40.8% | 50.1% |
| Class C | 39.3% | 44.5% |
| Class E |  |  |
| Overall | 42.3% | 51.1% |
| Class D | 42.1% | 45.0% |
| Class F | 39.0% | 42.4% |

Overall, the BWA usage on CTC conditions goes up from 13.7% to 14.8%.

# Conclusion

This contribution proposes to unify BDOF and BWA as it improved the gains of BDOF without adding complexity.  Both BDOF and BWA tools get a greater usage ratio. It is suggested to adopt the unification into the reference test model VTM-5.0.

# References

- "Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3)”, document JVET-L1002, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, SI, Oct.  2018.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).