| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0251_v2 |
| --- | --- |

| Title: | Non-CE8: intra block copy clean-up |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Li Zhang, Kai Zhang, Hongbin Liu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document presents several clean-ups for the current intra block copy (IBC) design to clarify reference buffer size, reference area restriction and reference area for small CTU size.

# Introduction

Intra block copy, i.e. IBC (or current picture referencing, i.e. CPR previously) coding mode, was adopted into VTM at the 12th JVET meeting.

In the current draft (VVC draft 4), the reference area for IBC is defined as

There are several issues in the current design. One is that it might need more than 4 64x64 memory. Second, for smaller CTU setting like 64x64 or 32x32, the reference area is not so efficient because the decoder still needs to maintain a 128x128 reference buffer, given that the decoder can decode bitstreams with CTU size being 128x128.

# Proposed clean-up

To address the issues described above, we propose several clean-ups for the current IBC design.

## IBC reference buffer size

We propose to define the reference buffer size as a fixed size of samples (e.g. 128x128 being the maximum CTU size) instead of a CTU.

## IBC reference buffer for different CTU sizes

We propose to use the previous (128x128/CTUsize) CTUs in the same CTU row as references for IBC coding mode. For example, 4 previous reconstructed CTUs before loop-filtering in the same CTU row are stored in the reference buffer when the CTU size is 64x64. For 64x64 CTU, the reference buffer is considered as 256x64 or a row of 4 CTUs. Similarly, for 32x32 CTU, the buffer is considered as 512x32 or a row of 16 CTUs. For smaller CTU setting, the memory reuse should be based on CTU so that when the current CTU uses a block of memory for decoding, the CTU previously stored in that memory should not be used.

For 64x64 CTU setting, Figure 1 shows the valid reference area. CTU i is the current CTU. The valid reference area includes CTU i-3, i-2 and i-1 in the same CTU row.

Figure 1. IBC reference area when CTU size is 64x64.

So the reference area should be defined as the reconstructed N-1 CTUs plus the reconstructed area in the current CTU, where N is (128x128/CTUsize).

The specific constrains is

( xCb + ( mvL[ 0 ] >> 4 ) ) >> CtbLog2SizeY > ( xCb >> CtbLog2SizeY ) – (128*128 >> CtbLog2SizeY )

## Reference area restriction

To avoid potential conflicts when decoding a CU in a collocated VPDU memory, it is not allowed to reference collocated 64x64 area in the previous CTU. The current text sets the bistream conformance constrain as that ((refX+128)>>6<<6, refY>>6<<6) shall not be reconstructed, which is also implemented in the VTM-4.0 software.

Suppose that the current CU locates at the upper-corner of a CTU, like shown in Fig. 2, because all pixels in the current CTU (CTU i+1) have not been reconstructed, according to the current design, all pixels in the previous CTU can be used as IBC reference.

Figure 2. An example of IBC reference that is valid in the current design. Each block denotes a 64x64 area.

Thus, it is proposed to set the constrain as

((refX>>6) + 2 = = cuX>>6) && (refY>>6 = = cuY>>6) shall be false.

The constrain together with constrains that restricts reference shall be within the current or the previous CTUs in a CTU row clearly define the reference area that can be implemented by memory reuse.

It should be noted that the above constrain is only applied to 128x128 CTU.

## Local search range fix

In the current VTM-4.0 software, the search range for local search is restricted to [-128, 128]. But because a CU can reference a block of previous CTU, a possible BVx can be much beyond -128. We believe that is because of careless but not intention. We propose to fix the lower bound of the local search range to be –(128+(CUx%128)).

# Experimental results

Table 1. IBC performance of VTM4.0 when CTU size is 64x64. Anchor is VTM4.0 w/ 64x64 CTU w/o IBC, the test is VTM4.0 w/ 64x64 CTU w/ IBC for all sequences.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 64x64 CTU w/ IBC off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | -0.03% | 129% | 100% |
| Class A2 | -0.31% | -0.26% | -0.31% | 128% | 101% |
| Class B | -0.35% | -0.29% | -0.35% | 136% | 99% |
| Class C | -0.13% | -0.08% | -0.17% | 136% | 97% |
| Class E | -0.80% | -0.64% | -0.36% | 139% | 98% |
| Overall | -0.31% | -0.24% | -0.25% | 134% | 99% |
| Class D | -0.31% | -0.31% | -0.24% | 136% | 106% |
| Class F | -11.08% | -10.80% | -11.03% | 147% | 102% |
| 4:2:0 TGM | -32.17% | -31.73% | -31.98% | 137% | 97% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 64x64 CTU w/ IBC off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.07% | 0.12% | 98% | 100% |
| Class A2 | 0.08% | 0.18% | 0.16% | 95% | 100% |
| Class B | 0.00% | -0.26% | -0.25% | 96% | 100% |
| Class C | 0.13% | -0.01% | 0.14% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | -0.07% | 0.01% | 97% | 100% |
| Class D | 0.14% | -0.14% | 0.20% | 99% | 97% |
| Class F | -8.75% | -8.79% | -8.81% | 103% | 98% |
| 4:2:0 TGM | -18.37% | -18.10% | -18.45% | 109% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-4.0 64x64 CTU w/ IBC off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.01% | 0.07% | 105% | #NUM! |
| Class C | 0.06% | 0.07% | -0.40% | 106% | 95% |
| Class E | -0.10% | 0.71% | -0.19% | 101% | 97% |
| Overall | 0.02% | 0.20% | -0.15% | 104% | #NUM! |
| Class D | 0.00% | 0.30% | 0.53% | 108% | 102% |
| Class F | -4.67% | -5.26% | -4.71% | 104% | 97% |
| 4:2:0 TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2. IBC performance of the proposed when CTU size is 64x64. Anchor is VTM4.0 w/ 64x64 CTU w/o IBC, the test is the proposed setting.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 64x64 CTU w/ IBC off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.01% | -0.05% | 159% | 100% |
| Class A2 | -0.32% | -0.28% | -0.34% | 157% | 99% |
| Class B | -0.42% | -0.38% | -0.37% | 174% | 100% |
| Class C | -0.19% | -0.14% | -0.32% | 167% | 94% |
| Class E | -1.08% | -0.97% | -0.70% | 173% | 100% |
| Overall | -0.40% | -0.34% | -0.35% | 167% | 98% |
| Class D | -0.37% | -0.36% | -0.31% | 158% | 106% |
| Class F | -15.56% | -15.28% | -15.37% | 166% | 99% |
| 4:2:0 TGM | -42.13% | -41.79% | -42.03% | 134% | 92% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 64x64 CTU w/ IBC off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.04% | 0.09% | 103% | 100% |
| Class A2 | 0.11% | 0.16% | 0.20% | 102% | 99% |
| Class B | -0.05% | -0.32% | -0.31% | 103% | 100% |
| Class C | 0.11% | 0.04% | 0.22% | 108% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.06% | 0.01% | 104% | 99% |
| Class D | 0.06% | -0.08% | 0.14% | 108% | 97% |
| Class F | -12.16% | -12.19% | -12.15% | 110% | 98% |
| 4:2:0 TGM | -25.22% | -24.93% | -25.40% | 112% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-4.0 64x64 CTU w/ IBC off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.04% | -0.23% | -0.32% | 118% | 97% |
| Class E | 0.10% | -0.73% | 0.21% | 109% | 94% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | 0.39% | 0.15% | 120% | 101% |
| Class F | -6.44% | -6.40% | -5.91% | 114% | 99% |
| 4:2:0 TGM | -14.46% | -13.72% | -14.01% | 117% | 94% |

Table 3. IBC performance with the proposed cleanup (restriction described in sec.2.3) under CE8 CTC.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.03% | 102% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class B | 0.00% | 0.00% | 0.01% | 102% | 106% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 114% |
| Class E | 0.00% | 0.00% | 0.00% | 103% | 107% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 106% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class F | 0.07% | 0.08% | -0.01% | 102% | 109% |
| 4:2:0 TGM | 0.22% | 0.22% | 0.20% | 102% | 103% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.01% | 0.00% | 0.01% | 102% | 104% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 111% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 107% |
| Class F | 0.03% | 0.13% | -0.03% | 101% | 106% |
| 4:2:0 TGM | 0.07% | 0.09% | 0.06% | 102% | 105% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.16% | -0.09% | 100% | #NUM! |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 103% |
| Overall | 0.00% | 0.07% | -0.04% | 100% | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 109% |
| Class F | -0.15% | 0.20% | -0.16% | 101% | 102% |
| 4:2:0 TGM | 0.14% | 0.21% | 0.24% | 102% | 100% |

Table IV. Performance of the encoder fix, including the restriction described in sec. 2.3.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.01% | 127% | 103% |
| Class A2 | -0.01% | 0.01% | -0.03% | 130% | 100% |
| Class B | -0.03% | -0.01% | -0.03% | 132% | 104% |
| Class C | -0.03% | 0.03% | -0.03% | 127% | 111% |
| Class E | -0.12% | -0.16% | -0.11% | 132% | 105% |
| Overall | -0.04% | -0.03% | -0.04% | 130% | 105% |
| Class D | -0.02% | -0.01% | 0.07% | 124% | 96% |
| Class F | -0.09% | -0.01% | -0.21% | 113% | 110% |
| 4:2:0 TGM | -0.24% | -0.25% | -0.27% | 101% | 102% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.08% | 0.02% | 107% | 100% |
| Class A2 | -0.02% | -0.06% | -0.05% | 110% | 100% |
| Class B | -0.02% | -0.18% | -0.32% | 111% | 102% |
| Class C | -0.03% | 0.13% | -0.01% | 115% | 109% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.05% | -0.12% | 111% | 103% |
| Class D | 0.03% | 0.03% | -0.02% | 116% | 112% |
| Class F | -0.12% | -0.07% | -0.13% | 108% | 102% |
| 4:2:0 TGM | -0.76% | -0.62% | -0.65% | 106% | 103% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.02% | -0.18% | -0.08% | 119% | 103% |
| Class E | -0.01% | 0.05% | -0.20% | 107% | 93% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.06% | 0.85% | 0.16% | 121% | 106% |
| Class F | -0.44% | -0.21% | -0.79% | 111% | #NUM! |
| 4:2:0 TGM | -0.73% | -0.60% | -0.62% | 107% | #NUM! |

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).