| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0261-v1 |
| --- | --- |

| Title: | CE2-1.1: Interweaved Prediction for Affine Motion Compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, an interweaved prediction approach is proposed for affine motion compensation (AMC). With the interweaved prediction, a coding block is divided into sub-blocks with two different dividing patterns. Then two auxiliary predictions are generated by AMC with the two dividing patterns. The final prediction is calculated as a weighted-sum of the two auxiliary predictions. Interweaved prediction is only applied on uni-prediction and cannot cross a VPDU boundary. Four tests are conducted in this contribution. In CE2-1.1.a, interweaved prediction is applied on luma component only. In CE2-1.1.b, interweaved prediction is applied on both luma and chroma components. In CE2-1.1.c, a combined test of CE2-1.1.a and short-tap interpolation filtering proposed in CE2-4.6 is conducted. In CE2-1.1.d, a combined test of CE2-1.1.a and bandwidth constraint method proposed in CE2-4.8.a is conducted. Simulation results are reported as below:

CE2-1.1.a: RA: -0.30%, 100%, 100%; LB: -0.14%, 101%, 101%.

CE2-1.1.b: RA: -0.32%, 100%, 99%; LB: -0.14%, 101%, 100%.

CE2-1.1.c: RA: -0.28%, 101%, 100%; LB: -0.11%, 101%, 102%.

CE2-1.1.d: RA: -0.28%, 101%, 101%; LB: -0.13%, 101%, 101%.

# Introduction

Interweaved prediction was proposed in JVET-M0268 [1] as shown in Fig. 1. With the interweaved prediction, a coding block is divided into sub-blocks with two different dividing patterns. Then two auxiliary predictions are generated by AMC with the two dividing patterns. The final prediction is calculated as a weighted-sum of the two auxiliary predictions. To avoid motion compensation with 2×H or W×2 block size, the interweaved prediction is only applied on regions where the size of sub-blocks is 4×4 for both the two dividing patterns as shown in Fig. 2.

# Proposed methods

Four CE tests of interweaved prediction are conducted on VTM-4.0. In all the tests, interweaved prediction is only applied on uni-prediction. Besides, interweaved prediction is constrained to be done within each VPDU. In other words, interweaved prediction is not applied on a sub-block with the second division if it crosses two VPDU. The four CE tests are listed as below:

CE2-1.1.a: interweaved prediction is applied on luma component only.

CE2-1.1.b: interweaved prediction is applied on both luma and chroma components.

CE2-1.1.c: CE2-1.1.a is ported on the base of short-tap interpolation filtering proposed in CE2-4.6 [2].

CE2-1.1.d: CE2-1.1.a is ported on the base of bandwidth constraint method proposed in CE2-4.8.a [3].

# Experimental Results

The proposed interweaved prediction has been implemented on top of VTM-4.0. Results of the four tests under RA/LB configurations are summarized in Table 1-Table 4. The simulations were performed following the JVET CTC [4]. More detail results can be found in the attached spreadsheets.

Table 1: Results for the CE2-1.1.a

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.08% | 0.01% | 100% | 101% |  |  |  |  |  |
| Class A2 | -0.53% | 0.17% | 0.16% | 100% | 101% |  |  |  |  |  |
| Class B | -0.37% | -0.14% | -0.07% | 100% | 99% | -0.15% | -0.03% | -0.13% | 100% | 101% |
| Class C | -0.18% | 0.05% | -0.10% | 100% | 98% | -0.15% | 0.08% | -0.12% | 101% | 103% |
| Class E |  |  |  |  |  | -0.12% | 0.08% | -0.21% | 103% | 99% |
| Overall | -0.30% | 0.02% | -0.02% | 100% | 100% | -0.14% | 0.04% | -0.15% | 101% | 101% |
| Class D | -0.28% | -0.13% | -0.05% | 100% | 103% | -0.17% | -0.06% | -0.70% | 100% | 100% |
| Class F | -0.53% | -0.27% | -0.23% | 99% | 97% | -0.79% | -0.70% | -1.01% | 100% | 100% |

Table 2: Results for the CE2-1.1.b

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.01% | 0.02% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.62% | -0.30% | -0.19% | 100% | 101% |  |  |  |  |  |
| Class B | -0.37% | -0.35% | -0.30% | 100% | 99% | -0.16% | -0.39% | -0.19% | 101% | 101% |
| Class C | -0.18% | -0.11% | -0.16% | 100% | 96% | -0.15% | -0.08% | -0.25% | 101% | 101% |
| Class E |  |  |  |  |  | -0.09% | 0.25% | -0.06% | 103% | 98% |
| Overall | -0.32% | -0.21% | -0.18% | 100% | 99% | -0.14% | -0.13% | -0.18% | 101% | 100% |
| Class D | -0.31% | -0.07% | -0.01% | 100% | 101% | -0.15% | -0.03% | -0.36% | 100% | 98% |
| Class F | -0.53% | -0.51% | -0.51% | 99% | 97% | -0.84% | -0.78% | -0.84% | 99% | 99% |

Table 3: Results for the CE2-1.1.c (CE2-4.6 as anchor)

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.07% | 0.06% | 100% | 99% |  |  |  |  |  |
| Class A2 | -0.50% | 0.30% | 0.12% | 101% | 99% |  |  |  |  |  |
| Class B | -0.34% | -0.10% | -0.01% | 101% | 99% | -0.11% | -0.03% | 0.11% | 102% | 100% |
| Class C | -0.18% | -0.02% | -0.01% | 100% | 100% | -0.12% | 0.07% | -0.09% | 100% | 104% |
| Class E |  |  |  |  |  | -0.10% | 0.36% | 0.08% | 102% | 101% |
| Overall | -0.28% | 0.03% | 0.03% | 101% | 100% | -0.11% | 0.10% | 0.03% | 101% | 102% |
| Class D | -0.25% | 0.01% | 0.02% | 100% | 99% | -0.05% | 0.35% | 0.43% | 100% | 98% |
| Class F | -0.53% | -0.30% | -0.30% | 101% | 100% | -0.78% | -0.41% | -0.39% | 102% | 97% |

Table 4: Results for the CE2-1.1.d (CE2-4.8 as anchor)

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.11% | 0.06% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.50% | 0.14% | 0.16% | 100% | 100% |  |  |  |  |  |
| Class B | -0.37% | -0.06% | -0.05% | 101% | 102% | -0.15% | -0.15% | 0.41% | 100% | 101% |
| Class C | -0.17% | 0.07% | -0.02% | 101% | 103% | -0.07% | 0.16% | -0.17% | 101% | 101% |
| Class E |  |  |  |  |  | -0.17% | -0.46% | 0.41% | 104% | 100% |
| Overall | -0.28% | 0.01% | 0.02% | 101% | 101% | -0.13% | -0.13% | 0.22% | 101% | 101% |
| Class D | -0.27% | 0.09% | -0.01% | 100% | 102% | -0.10% | 0.47% | -0.21% | 102% | 104% |
| Class F | -0.52% | -0.25% | -0.24% | 101% | 104% | -0.76% | -0.44% | -0.41% | 101% | 100% |

Fig. 1 Interweaved prediction.

Fig. 2 Interweaved prediction is not applied on the shaded regions.

# Conclusions

Considering the coding gain of interweaved prediction and no conflicting with other tools for bandwidth /computational complexity reduction, it is suggested that CE2-1.1.a be adopted into VVC.

# References

- K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong, “Non-CE2: Interweaved Prediction for Affine Motion Compensation,” JVET-M0268, Jan. 2019.

- J. Li, C.-W. Kuo, C. S.Lim, “CE2: Using the shorter-tap filter for 4x4 sized partitions (Test 2.4.6),” JVET-N0196, March 2019.

- M. Zhou, “CE2: On restriction of memory bandwidth consumption of affine mode (CE2-4.8),” JVET-N0068, March 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Jan. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).