| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 17–28 Mar. 2019 | Document: JVET-N0330 |
| --- | --- |

| Title: | Non-CE3: Intra Sub-Partitions Coding without Thin Partitions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-chuan Ma Xiao-yu Xiu Yi-Wen Chen Xianglin Wang | Email: | tsung-chuanma@kwai.com xiaoyuxiu@kwai.com yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to disable 1xN, 2xN, and/or Nx1 subblock partitions for intra sub-partition (ISP) mode to improve the decoding throughput of the ISP mode. The simulation results of the proposal are summarized as follows:

- When disabling 1×N ISP sub-partitions, the BD-rate increases are 0.04 %, 0.02% and 0.00% for AI, RA and LD, respectively.

- When disabling 1×N and 2×N ISP sub-partitions, the BD-rate increases are 0.14%, 0.07% and 0.03% for AI, RA and LD, respectively.

- When disabling 1×N and N×1 ISP sub-partitions, the BD-rate increases are 0.08 %, 0.04% and 0.01% for AI, RA and LD, respectively.

# Introduction

In VTM-4.0 [1], the ISP mode splits current coding unit into 4 or 2 sub-blocks according to the size of current block [2]. When the ISP is enabled, depending on the selected intra direction, the reconstructed samples of one ISP subblock can be used as references to predict the samples of its following subblock. In other words, the intra prediction of one ISP subblock cannot be started until its preceding ISP subblock (according to the ISP coding order) is fully reconstructed. Hence, such dependency causes decoding throughput issue, especially for the ISP CUs with small sub-partitions, e.g., 1xN, 2xN, and Nx1. For example, ISP with 4xN block consumes 4 times processing cycles than non-ISP 4xN block because it decodes each 1xN sub-block sequentially and therefore the decoding throughput of ISP 4xN block is roughly one-quarter of that of non-ISP (normal intra mode) 4xN block.

In this contribution, it is proposed to disable the ISP mode for the CUs with small sub-partitions. Specifically, the following three options are proposed in the contribution for a reasonable balance between the improved decoding throughput and the BD-rate performance loss:

- First option, only removing ISP blocks that introduces 1xN sub-blocks.

- Second option, removing ISP blocks that introduces 1xN or 2xN sub-blocks.

- Third option, removing ISP blocks that introduces 1xN, or Nx1 sub-blocks.

# Results

The proposed modification is integrated into VTM-4.0 [1] and the results of the proposed implementations are shown below against the VTM4.0 under CTC [3]. Table 1 shows the BD-rate increasing of three options. For more detail in each setting, please refer to the attached spreadsheets. Meanwhile, in Table 1, items highlighted with Still running or SR means that the result is not ready yet and will be updated when it is done.

Table 1. Results of proposed settings

|  | AI | RA | LDB | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Remove 1xN | 0.04%/0.05%/0.04% | 0.02%/0.05%/0.03% | 0.00%/0.19%/0.12% | 100% | 99% |
| Remove 1xN, 2xN | 0.14%/0.16%/0.13% | 0.07%/0.10%/-0.02% | 0.03%/0.31%/0.02% | 99% | 101% |
| Remove 1xN, Nx1 | 0.08%/0.11%/0.06% | 0.04%/0.06%/-0.10% | 0.01%/0.08%/0.08% | 99% | 99% |

# Conclusion

In this proposal, we provide several options to simplify ISP in order to increase the VVC throughput. As shown in Table 1, removing only 1xN causes less RD-rate impact while removing both 1xN and 2xN cause worst RD-rate loss. However, for the sake of throughput, it is recommended to adopt one of the proposed modifications into VVC.

# References

| [1] | "VTM-4.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM". |
| --- | --- |
| [2] | S. De-Luxán-Hernández, H. Schwarz, D. Marpe and T. Wiegand, "CE3: Line-based intra coding mode (Tests 1.1.1 and 1.1.2)," Document JVET-M0102, Marrakesh , MA, 2019. |
| [3] | F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, "JVET common test conditions and software reference configurations for SDR video," 2018. |

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).