| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0331 |
| --- | --- |

| Title: | On interpolation for BD rate computation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 fbossen@sharplabs.com |
| Source: | Sharp Electronics of Canada Ltd. |  |  |

_____________________________

# Abstract

A BD rate number represents the average distance between two rate-distortion curves. Such rate-distortion curves are interpolated from a few rate-distortion points using cubic or piecewise cubic methods. It is asserted that such interpolating curves can diverge significantly from actual rate-distortion curves, and that commonly reported BD rate numbers may be inaccurate. It is proposed to increase the sampling rate of rate-distortion curves such as to improve accuracy.

# Problem Statement

BD rate numbers are currently computed using piecewise cubic interpolation and cubic interpolation with the former being reported in the summary sheet provided in [1]. Issues with the cubic interpolation were previously discussed in [2], leading to the adoption of piecewise cubic interpolation. The accuracy of the piecewise cubic interpolation is largely dependent on the accuracy of the slopes that are estimated for each data point. The estimated slopes may substantially differ from actual slopes, in particular at the lower and higher QP values where fewer data points are available for estimation (each end point has only one neighbour instead of two). The graph below shows an example of a RD curve comprising 16 QP values and the piecewise cubic interpolating curve derived from the standard four QP values.

The following table summarizes the BD rate difference between the piecewise cubic function obtained from 4 QP points (22, 27, 32, 37) and the piecewise linear function obtained from 16 QP points (22..37).

| Sequence | Average | Min | Max | Sequence | Average | Min | Max |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tango | -1.42% | -5.62% | 0.84% | BQMall | 0.01% | -0.75% | 0.99% |
| FoodMarket | -0.05% | -1.14% | 2.49% | PartyScene | -0.01% | -0.68% | 0.54% |
| Campfire | -4.08% | -14.42% | 2.19% | RaceHorsesC | -0.44% | -2.27% | 1.09% |
| CatRobot | -1.01% | -2.73% | 0.52% | BasketballPass | -0.16% | -0.86% | 0.27% |
| DaylightRoad | -2.24% | -13.32% | 6.60% | BQSquare | -0.01% | -0.59% | 0.78% |
| ParkRunning | -0.90% | -2.88% | 2.06% | BlowingBubbles | -0.13% | -0.65% | 0.40% |
| MarketPlace | -0.17% | -0.83% | 0.36% | RaceHorses | -0.06% | -0.54% | 0.63% |
| RitualDance | -0.15% | -0.76% | 0.60% | BasketballDrillText | -0.10% | -0.85% | 0.47% |
| Cactus | -0.92% | -3.72% | 0.88% | ArenaOfValor | -0.34% | -1.73% | 0.48% |
| BasketballDrive | -0.82% | -3.64% | 0.70% | SlideEditing | 0.17% | -0.26% | 0.91% |
| BQTerrace | -4.37% | -19.70% | 4.11% | SlideShow | -0.05% | -1.17% | 0.88% |
| BasketballDrill | -0.18% | -0.97% | 0.45% | All | -0.76% | -19.70% | 6.60% |

When evaluating proposals, BD rate differences are considered. Even if the interpolating curves are “incorrect”, differences between them may still be “correct” if a same approximation error is present in both curves.

However, the approximation error is not always consistent between two such curves, as demonstrated in the graph below, where tested QP values are increased or decreased by 1, while keeping everything else the same. The BD rate difference between the 3 curves is clearly not zero, even though no changes are made to any coding tools.

A fundamental issue here is that sampling is not dense enough, i.e. the gap between QP values being tested is too large.

# Proposed solution

It is proposed to extend the number of data points from 4 to 6, and compute results for the following QP values: 21, 24, 28, 31, 35, 38. The denser sampling results in the piecewise cubic interpolation tracking the fully sampled curve much more closely, as shown in the graph below.

While differences between the fully sampled and the subsampled curves remain, they are significantly reduced, as summarized in the table below.

| Sequence | Average | Min | Max | Sequence | Average | Min | Max |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tango | -0.27% | -5.28% | 5.28% | BQMall | -0.07% | -0.43% | 0.57% |
| FoodMarket | -0.04% | -0.63% | 1.40% | PartyScene | -0.08% | -0.50% | 0.55% |
| Campfire | 0.41% | -2.28% | 3.91% | RaceHorsesC | 0.09% | -0.74% | 1.50% |
| CatRobot | -0.36% | -3.25% | 2.22% | BasketballPass | 0.06% | -0.31% | 0.56% |
| DaylightRoad | -1.83% | -10.44% | 0.71% | BQSquare | -0.1% | -0.81% | 0.30% |
| ParkRunning | 0.25% | -2.04% | 3.09% | BlowingBubbles | -0.09% | -0.67% | 0.46% |
| MarketPlace | -0.02% | -0.46% | 0.72% | RaceHorses | 0.03% | -0.64% | 0.53% |
| RitualDance | -0.01% | -0.32% | 0.66% | BasketballDrillText | -0.12% | -0.61% | 0.65% |
| Cactus | 0.04% | -1.34% | 3.86% | ArenaOfValor | -0.13% | -0.67% | 0.90% |
| BasketballDrive | -0.05% | -2.66% | 3.63% | SlideEditing | -0.15% | -0.64% | 0.24% |
| BQTerrace | 0.16% | -3.48% | 3.96% | SlideShow | -0.32% | -1.02% | 0.26% |
| BasketballDrill | -0.05% | -0.59% | 0.93% | All | -0.12% | -10.44% | 5.28% |

When considering QP alterations of +/- 1, the resulting interpolating curves are much some similar to each other.

## Related work

An increase from four to eight data points has been proposed in [2]. At the time, the outcome was a shift from cubic to piecewise-cubic interpolation.

Increasing the number of QP points was also discussed at the JVET meeting in July 2019, as captured in the meeting notes: “QPs for CTC (add another or space apart by 7)?” (section 11.2). However, the possibility of increasing the number of QP points while reducing the spacing does not seem to have been considered.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, February 2019

- J. Wang, X. Yu, D. He, “On BD-rate calculation”, JCTVC-F0270, July 2011

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).