| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0345 |
| --- | --- |

| Title: | CE7: Template based Rice parameter derivation for coding of abs_remainder (CE7-4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Muhammed Coban, Marta Karczewicz (Qualcomm) | Tel: Email: | yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

# Abstract

This contribution proposes a unified template-based Rice parameter derivation for coding of abs_remainder and dec_abs_level syntax elements by introducing template-based Rice parameter derivation that is used for coding of dec_abs_level syntax element for coding of abs_remainder syntax element instead of current local HEVC style Rice parameter derivation. The simulations results show 0.00% AI, 0.00% RA, -0.04% LB BD-Rate versus the VTM-4.0 anchor for common test conditions for Rice parameter derivation method, and -0.02% AI, 0.01% RA, 0.02% LB BD-Rate versus VTM4.0 anchor under low QP condition.

# Introduction

In VTM-4.0 the rice parameter for coding the coefficients that are completely coded in bypass mode, i.e. dec_abs_level syntax element, is derived based on the sum of absolute levels in a local template around the current transform coefficient level. The Rice parameter for coding the remainder (for coefficients that are not completely coded in bypass mode), i.e. abs_remainder syntax element, is derived as in HEVC without referring to neighboring transform coefficient level. In this proposal, a unified local template-based Rice parameter derivation for coding both abs_remainder and dec_abs_level is proposed.

# Proposal

In this proposal, for coding the non-binary syntax element abs_remainder [k], the sum of absolute values sumAbs in a local template is determined. The variable ricePar is determined by the table that is used for template-based Rice parameter derivation for coding of dec_abs_level syntax element in VTM-4.0. Since abs_remainder is half of remaining level after coding of sig_coeff_flag, abs_level_gt1_flag, par_level_flag, abs_level_gt3_flag, the Rice parameter from look-up table for coding of full levels in dec_abs_level coding is decremented by 1. This can be described by:

ricePar   = riceParTable[ min( 31, sumAbs ) ] 
ricePar = ricePar > 0 ? ricePar -1 : 0

where the riceParTable[] is given by

riceParTable[32] = {0,0,0,0,0,0,0,1,1,1,1,1,1,1,2,2,2,2,2,2,2,2,2,2,2,2,2,2,3,3,3,3};

# Simulation results

The proposed scheme is implemented on top of VTM-4.0 software.  The simulations were carried out under the common test condition [1] and   CE7 description [2]. Table 1 shows the results for QP range defined in the common test condition and Table 2 shows the results for low QP. The encoding/decoding time for low QP test may not be accurate.

Table 1: Results of CE7-4 for QP range in CTC (QP = 22, 27, 32, 37)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.05% | 103% | 103% |
| Class A2 | 0.01% | 0.09% | 0.00% | 103% | 104% |
| Class B | 0.00% | 0.00% | 0.04% | 103% | 99% |
| Class C | 0.02% | 0.03% | 0.06% | 103% | 99% |
| Class E | -0.01% | 0.00% | -0.03% | 102% | 96% |
| Overall | 0.00% | 0.02% | 0.03% | 103% | 100% |
| Class D | 0.02% | -0.05% | 0.06% | 103% | 103% |
| Class F | 0.02% | 0.02% | -0.03% | 102% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM4.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | -0.04% | 100% | 101% |
| Class A2 | -0.01% | 0.04% | -0.01% | 100% | 98% |
| Class B | 0.01% | -0.03% | -0.09% | 100% | 99% |
| Class C | 0.02% | -0.15% | -0.23% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.05% | -0.10% | 101% | 100% |
| Class D | 0.01% | -0.42% | -0.36% | 102% | 101% |
| Class F | -0.05% | -0.18% | -0.11% | 101% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM4.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.28% | -0.09% | 101% | 99% |
| Class C | -0.02% | 0.16% | 0.20% | 102% | 103% |
| Class E | -0.14% | 0.22% | 0.79% | 101% | 102% |
| Overall | -0.04% | -0.01% | 0.23% | 101% | 101% |
| Class D | -0.04% | -0.07% | -0.08% | 101% | 97% |
| Class F | -0.10% | 0.41% | 0.40% | 101% | 101% |

Table 2: Results of CE7-4 for low QP (QP = 2, 7, 12, 17)

|  | Over VTM4.0 low QP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.05% | 0.04% | 104% | 103% |
| Class A2 | -0.01% | -0.02% | -0.03% | 104% | 100% |
| Class B | -0.02% | -0.01% | -0.02% | 103% | 104% |
| Class C | -0.05% | -0.09% | -0.05% | 104% | 103% |
| Class E | -0.07% | -0.01% | 0.00% | 104% | 103% |
| Overall | -0.02% | -0.02% | -0.01% | 104% | 103% |
| Class D | -0.03% | -0.04% | -0.01% | 103% | 101% |
| Class F | -0.06% | -0.06% | -0.07% | 103% | 102% |
|  | Over VTM4.0 low QP |  |  |  |  |
|  | Y | c | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.02% | 103% | 119% |
| Class A2 | 0.01% | 0.02% | 0.02% | 103% | 206% |
| Class B | 0.02% | 0.03% | 0.04% | 102% | 101% |
| Class C | 0.00% | -0.10% | -0.10% | 103% | 105% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.00% | 103% | 122% |
| Class D | 0.00% | -0.02% | -0.07% | 102% | 100% |
| Class F | -0.09% | -0.07% | -0.03% | 101% | 100% |
|  | Over VTM4.0 low QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.07% | 0.06% | 103% | 103% |
| Class C | 0.02% | 0.04% | 0.06% | 103% | 101% |
| Class E | 0.00% | -0.10% | -0.04% | 103% | 100% |
| Overall | 0.02% | 0.02% | 0.03% | 103% | 101% |
| Class D | 0.00% | -0.01% | 0.08% | 102% | 103% |
| Class F | -0.04% | -0.06% | -0.02% | 103% | 102% |

# Reference

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, 13th meeting, Marrakech, MA, January 2019

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding”, JVET-M1027, 13th meeting, Marrakech, MA, January 2019

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).