| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0348 |
| --- | --- |

| Title: | AHG12: Hierarchical tiling for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Sühring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The authors propose a hierarchical tiling concept in which a tile of a first tiling grid can further be selectively split uniformly into multiple tiles using a second tiling grid.

# Introduction

The current VVC specification defines two types of tiles: rectangular tiles and scan order tiles. Tile boundaries in both approaches are required to split the whole coded picture.

Among the applications relying on tiles are ROI based coding, 360° video streaming or conversational scenario. For instance, viewport dependent 360° video streaming according to OMAF is based on tiles. One scenario is based on re-assembling content tiles of varying resolution/size as illustrated in Figure 1 taken from the Annex D of the OMAF specification.

Figure 1: Multiple resolution tiles for viewport dependent streaming of 360° video.

It can be seen that in order to combine tiles of different sizes in the exemplary arrangement, horizontal HEVC slice boundaries are used as illustrated on the right-hand side of the figure. With the absence of CTU granularity slices in VVC, the need for another mechanism allowing such arrangements arises.

# Proposal

The authors propose a hierarchical tiling concept in which a tile of a first tiling grid can further be selectively split uniformly into multiple tiles using a second tiling grid.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| partial_columns_and_rows_present_flag | u(1) |
| if (partial_columns_and_rows_present_flag){ |  |
| for( i = 0; i < num_tile_columns_minus1 * tile_row_height_minus1; i++ ) { |  |
| split_flag[ i ] | u(1) |
| if( split_flag ) { |  |
| num_partial_columns_minus1[ i ] | ue(v) |
| num_partial_rows_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |
| […] |  |
| rbsp_trailing_bits( ) |  |
| } |  |

partial_columns_or_rows_present_flag specifies that one or more tile colums and/or tile rows are further uniformly divided by partial columns or rows.

split_flag[ i ] specifies that the i-th tile region specified by tile columns and tile rows is divided by one or more partial columns or rows into two or more tiles.

num_partial_columns_minus1[ i ] plus 1 specifies the number of partial tile columns partitioning the i-th tile region specified by tile columns and tile rows. num_partial_columns_minus1[ i ] shall be in the range of 0 to ColWidth[ i ], inclusive. When not present, the value of num_partial_columns_minus1[ i ] is inferred to be equal to 0.

num_partial_rows_minus1[ i ] plus 1 specifies the number of partial tile rows partitioning the i-th tile region specified by tile columns and tile rows. num_partial_rows_minus1[ i ] shall be in the range of 0 to RowHeight[ i ], inclusive. When not present, the value of num_partial_rows_minus1[ i ] is inferred to be equal to 0.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).