| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0366-v3 |
| --- | --- |

| Title: | CE8-related: Modified limitation on context coded bins for CE8-4.4a and CE8-5.1a |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Xiaozhong Xu, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, xiaozhongxu, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In CE8-4.4a and CE8-5.1a, the number of context coded bins for entropy coding Block-DPCM and Transform Skip residuals is limited to be average 2 per coefficient. In this contribution, an alternative approach for achieving same number of context coded bins is proposed. The proposed method relocates the signaling of parity flag to be after gt2 flag and allows more context-coded bins for gt2 flag given the total budget of context coded bins. Experimental results show that average 0.xx%, 0.07% and 0.xx% coding gain can be achieved on top of CE8-4.4a for AI, RA and LDB configurations, respectively, and average 0.36%, 0.16% and 0.xx% coding gain can be achieved on top of CE8-5.1a for AI, RA and LDB configurations, respectively.

# Introduction

In CE8-3.1a, the residual coding of Block-DPCM (BDPCM) with H/V predictor from JVET-M0058 is performed with the methods from JVET-M0464, together with limitations on the context coded bins from JVET-M0449 (up to average 2 context coded bins/sample). The limitation on the context coded bins is reported to have 0.54% and 0.27% penalty on the coding gain for AI and RA configurations, respectively. In this contribution, an alternative method for limiting the number of context coded bins is proposed, with same constraint of max number of context coded bins per sample. In CE8-5.1a, the modified residual coding scheme proposed in JVET-M0464 is applied to both Block-DPCM and Transform Skip mode.

The CE8-3.1a, CE8-4.4a and CE8-5.1a tests are described in the following table from JVET-M1028.

| Test | Document | Tool description |
| --- | --- | --- |
| CE8-3.1a | JVET-M0058 JVET-M0464 JVET-M0449 | Block-DPCM with H/V predictor from JVET-M0058 with residual coding from JVET-M0464 and context coded bins limitation from JVET-M0449 (max 2 context coded bins/sample) |
| CE8-4.4a | JVET-M0464 | Residual coding for transform skip and context coded bins limitation from JVET-M0449 with max 2 context coded bins per sample . |
| CE8-5.1a | JVET-M0058 JVET-M0464 JVET-M0449 | Combination of 8-3.1a and 8-4.4a (both use max 2 context coded bins per sample) |

# Proposal

In the residual coding applied in CE8-3.1a, CE8-4.4a and CE8-5.1a, there are 6 syntax elements related to residual coding that may be context coded, including coefficient group flag, significance flag, gt1 flag, sign flag, parity flag and gt2 flag. It is observed that the context coding of parity flag contributes the less coding gain (versus bypass coding) comparing to the remaining 5 syntax elements.

Based on this observation, it is proposed to move parity flag from the first pass to be after gt2 flag coding, such that the remaining 5 syntax elements coding are given higher priority for the consumption of the total budget of context coded bins. In addition, gt2 flags can have more context coded bins by changing the following macro is CE8-3.1a SW.

#define MAX_NUM_GT_BINS_TSRC                              8

The proposed limitation on context coded bins can be applied to both CE8-3.1a, CE8-4.4a and CE8-5.1a, and the difference is whether the modified residual coding scheme is applied for Transform Skip mode or not.

# Experiment results

The experiment is performed on top of VTM-4.0 using both CTC condition. Two tests are performed, the anchor used for the first test is CE8-4.4a and the test method is CE8-4.4a with proposed method, the anchor used for the second test is CE8-5.1a and the test method is CE8-5.1a with proposed method. Detailed results for each test are reported in Table 1 and Table 2, respectively. It is observed that the proposed method has almost no impact on the encoder or decoder run-time.

Table 1: Coding gain of proposed method versus CE8-4.4a.

|  | All Intra Main10 |  |  |  |  |  | Random Access Main 10 |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.01% | 102% | 101% | 0.00% | 0.00% | -0.04% | 100% | 99% |  |  |  |  |  |
| Class A2 | -0.01% | -0.01% | 0.02% | 102% | 102% | 0.01% | 0.10% | -0.07% | 101% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.02% | 0.03% | 100% | 99% | -0.01% | -0.03% | -0.06% | 100% | 100% | 0.02% | 0.19% | 0.05% | 100% | 100% |
| Class C | 0.00% | -0.02% | -0.07% | 102% | 101% | 0.04% | -0.15% | -0.04% | 100% | 100% | 0.02% | -0.22% | 0.14% | 100% | 101% |
| Class E | 0.01% | -0.05% | 0.04% | 100% | 100% |  |  |  |  |  | -0.09% | 1.12% | 0.00% | 103% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.01% | -0.03% | -0.05% | 100% | 100% | -0.01% | 0.29% | 0.07% | 101% | 101% |
| Class D | -0.01% | -0.08% | 0.05% | 100% | 101% | 0.00% | -0.07% | -0.02% | 101% | 99% | 0.00% | 0.26% | 0.36% | 101% | 102% |
| Class F | -0.10% | -0.01% | -0.17% | 100% | 99% | -0.11% | -0.15% | -0.03% | 101% | 101% | -0.17% | -0.26% | 0.00% | 100% | 99% |
| Class TGM | -0.39% | -0.41% | -0.37% | 101% | 102% | -0.07% | -0.12% | -0.18% | 101% | 101% | -0.08% | -0.10% | -0.06% | 100% | 100% |

Table 2: Coding gain of proposed method versus CE8-5.1a.

|  | All Intra Main10 |  |  |  |  |  | Random Access Main 10 |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.01% | 98% | 100% | 0.00% | 0.01% | 0.01% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.01% | 0.03% | -0.01% | 100% | 101% | 0.00% | 0.19% | 0.00% | 99% | 100% |  |  |  |  |  |
| Class B | 0.01% | 0.00% | -0.03% | 100% | 100% | -0.01% | 0.10% | 0.04% | 100% | 100% |  |  |  |  |  |
| Class C | 0.02% | -0.01% | 0.02% | 99% | 96% | -0.04% | -0.05% | -0.03% | 100% | 102% | 0.07% | 0.02% | 0.16% | 100% | 100% |
| Class E | -0.01% | -0.04% | 0.03% | 100% | 100% |  |  |  |  |  | 0.05% | -0.11% | 0.38% | 102% | 101% |
| Overall | 0.00% | -0.01% | 0.00% | 99% | 99% | -0.01% | 0.06% | 0.01% | 100% | 101% |  |  |  |  |  |
| Class D | 0.00% | -0.04% | -0.06% | 101% | 100% | 0.03% | -0.02% | 0.11% | 100% | 100% | 0.04% | 0.23% | 0.52% | 99% | 101% |
| Class F | -0.11% | -0.15% | -0.16% | 99% | 97% | -0.07% | -0.03% | -0.30% | 100% | 100% | -0.02% | 0.41% | -0.17% | 97% | 99% |
| Class TGM | -0.36% | -0.43% | -0.39% | 100% | 100% | -0.16% | -0.14% | -0.25% | 101% | 100% | -0.02% | -0.02% | 0.00% | 100% | 99% |

# Conclusions

This contribution proposes an alternative approach for limiting the number of context coded bins for entropy coding of Block based DPCM and Transform Skip prediction residuals. With the proposed method, parity flags are coded after gt2 flags. It is recommended to adopt the proposed changes on top of CE8-4.4a and/or CE8-5.1a.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).