| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0375_r1 |
| --- | --- |

| Title: | Non-CE6: Simplification on implicit transform selection in ISP mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim Jungwon Kang Jinho Lee Hahyun Lee Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5772 sclim@etri.re.kr jungwon@etri.re.kr jinosoul@etri.re.kr hanilee@etri.re.kr hykim5@etri.re.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution presents a simplification on the implicit transform selection in the intra sub-partitions (ISP) mode. In the ISP mode, the horizontal and vertical transforms are implicitly selected based on the intra prediction mode, the width of sub-partition, and the height of sub-partition. The proposed method removes the dependency on intra prediction mode for transform selection in the ISP mode. Thus, DCT-2 and DST-7 transforms are implicitly selected based on the width of sub-partition and the height of sub-partition. By using the proposed method, the intra prediction mode is not necessary to be used for the transform and inverse transform process. Compared to VTM-4.0.1, it is reported that the test 1 of the proposed method shows average BD-rates of -0.01%, 0.00%, and 0.01% for AI, RA, and LDB cases, respectively. And, the test 2 of the proposed method shows average BD-rates of -0.01%, -0.01%, and -0.04% for AI, RA, and LDB cases, respectively.

Introduction

In the intra sub-partitions (ISP) mode adopted in the last JVET meeting, luma intra-predicted blocks are split vertically or horizontally into 2 or 4 sub-partitions depending on the block size [1]. For each sub-partition, by adding the reconstructed residual to the prediction signal, reconstructed signal is generated. The reconstructed residual is obtained by entropy decoding, inverse quantization, and inverse transform. And, the reconstructed signal of each sub-partition is available to generate the prediction of the next sub-partition. For inverse transform, the horizontal and vertical transforms are implicitly selected based on the intra prediction mode, the width of sub-partition, and the height of sub-partition as shown in Table I.

Table I. Implicit transform selection in the ISP mode

| Luma intra prediction mode | Horizontal transform | Vertical transform |
| --- | --- | --- |
| INTRA_PLANAR, INTRA_ANGULAR31, INTRA_ANGULAR32, INTRA_ANGULAR34, INTRA_ANGULAR36, INTRA_ANGULAR37 | (W >= 4 && W <= 16) ? DST-7 : DCT-2 | (H >= 4 && H <= 16) ? DST-7 : DCT-2 |
| INTRA_ANGULAR33, INTRA_ANGULAR35 | DCT-2 | DCT-2 |
| INTRA_ANGULAR2, INTRA_ANGULAR4,…,INTRA_ANGULAR28, INTRA_ANGULAR30, INTRA_ANGULAR39, INTRA_ANGULAR41,…,INTRA_ANGULAR63, INTRA_ANGULAR65 | (W >= 4 && W <= 16) ? DST-7 : DCT-2 | DCT-2 |
| INTRA_ANGULAR3, INTRA_ANGULAR5,…, INTRA_ANGULAR27, INTRA_ANGULAR29, INTRA_ANGULAR38, INTRA_ANGULAR40,…,INTRA_ANGULAR64, INTRA_ANGULAR66 | DCT-2 | (H >= 4 && H <= 16) ? DST-7 : DCT-2 |

Thus, to select the horizontal and vertical transforms for sub-partitions in the ISP mode, the intra prediction mode shall be used for the transform and inverse transform process.

Proposed method

The proposed method removes the dependency on intra prediction mode for transform selection in the ISP mode. Thus, only DCT-2 and DST-7 transforms are implicitly selected based on the width of sub-partition and the height of sub-partition as described in the following test 1 and test 2.

For the test 1, as shown in Table II, when the width of sub-partition is larger than equal to 4 and smaller than equal to 16, DST-7 is selected as the horizontal transform. Otherwise, DCT-2 is selected as the horizontal transform. And, when the height of sub-partition is larger than equal to 4 and smaller than equal to 16, DST-7 is selected as the vertical transform. Otherwise, DCT-2 is selected as the vertical transform.

Table II. Proposed implicit transform selection in the ISP mode (Test 1)

| Horizontal transform | Vertical transform |
| --- | --- |
| (W >= 4 && W <= 16) ? DST-7 : DCT-2 | (H >= 4 && H <= 16) ? DST-7 : DCT-2 |

For the test 2, as shown in Table III, when the width of sub-partition is larger than equal to 4 and smaller than equal to 8, DST-7 is selected as the horizontal transform. Otherwise, DCT-2 is selected as the horizontal transform. And, when the height of sub-partition is larger than equal to 4 and smaller than equal to 8, DST-7 is selected as the vertical transform. Otherwise, DCT-2 is selected as the vertical transform.

Table III. Proposed implicit transform selection in the ISP mode (Test 2)

| Horizontal transform | Vertical transform |
| --- | --- |
| (W >= 4 && W <= 8) ? DST-7 : DCT-2 | (H >= 4 && H <= 8) ? DST-7 : DCT-2 |

Since the horizontal and vertical transforms are selected regardless of the intra prediction mode, the intra prediction mode is not needed for the transform and inverse transform process.

Experiment results

The experiment was performed based on VTM-4.0.1 software [2] with VTM configuration files under JVET common test condition [3]. And, the experiment was conducted on the platform of Intel Xeon E5-2690 v2 (3.0GHz) and 64GB RAM with GCC 7.3.1 compiler under CentOS Linux.

The experimental results of the proposed method compared to VTM-4.0.1 anchor are summarized in Table IV and Table V. For the test 1, the average BD-rates of -0.01%, 0.00%, and 0.01% are observed for AI, RA, and LDB cases, respectively. And, for the test 2, the average BD-rates of -0.01%, -0.01%, and -0.04% are observed for AI, RA, and LDB cases, respectively. Thus, it can be noted that the proposed method introduces no coding performance impact in AI, RA, LDB cases. And, the detailed results can be found in accompanied Excel files.

Table IV. Experimental result of the proposed method (Test 1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.10% | 100% | 100% |
| Class A2 | -0.03% | -0.01% | -0.10% | 100% | 100% |
| Class B | -0.01% | -0.02% | -0.04% | 100% | 100% |
| Class C | 0.02% | 0.00% | 0.03% | 100% | 100% |
| Class E | -0.05% | -0.11% | -0.12% | 100% | 100% |
| Overall | -0.01% | -0.03% | -0.06% | 100% | 100% |
| Class D | 0.01% | -0.01% | -0.09% | 100% | 100% |
| Class F | 0.02% | -0.06% | -0.10% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.16% | -0.04% | 100% | 100% |
| Class A2 | -0.01% | 0.09% | -0.02% | 100% | 100% |
| Class B | 0.00% | 0.04% | -0.06% | 100% | 100% |
| Class C | 0.01% | 0.17% | 0.19% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.05% | 0.02% | 100% | 100% |
| Class D | -0.03% | -0.25% | -0.40% | 100% | 100% |
| Class F | 0.03% | 0.00% | -0.15% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.16% | -0.74% | 100% | 100% |
| Class C | -0.01% | -0.32% | -0.22% | 100% | 100% |
| Class E | 0.04% | -0.78% | -0.54% | 100% | 100% |
| Overall | 0.01% | -0.24% | -0.52% | 100% | 100% |
| Class D | 0.03% | 0.64% | -0.21% | 100% | 100% |
| Class F | 0.02% | 0.09% | -0.37% | 100% | 100% |

Table V. Experimental result of the proposed method (Test 2)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.08% | 0.02% | 100% | 100% |
| Class A2 | -0.02% | -0.04% | -0.04% | 100% | 100% |
| Class B | -0.01% | 0.05% | -0.02% | 100% | 100% |
| Class C | 0.02% | -0.02% | -0.01% | 100% | 100% |
| Class E | -0.04% | -0.05% | -0.17% | 100% | 100% |
| Overall | -0.01% | 0.01% | -0.04% | 100% | 100% |
| Class D | -0.01% | -0.01% | -0.06% | 100% | 100% |
| Class F | 0.00% | -0.06% | 0.04% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.07% | -0.05% | 100% | 100% |
| Class A2 | -0.02% | 0.10% | 0.08% | 100% | 100% |
| Class B | -0.01% | 0.13% | -0.07% | 100% | 100% |
| Class C | -0.01% | 0.27% | 0.27% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.12% | 0.06% | 100% | 100% |
| Class D | -0.01% | -0.25% | -0.29% | 100% | 100% |
| Class F | 0.03% | 0.03% | 0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.15% | -0.12% | 100% | 100% |
| Class C | -0.01% | -0.10% | -0.15% | 100% | 100% |
| Class E | -0.13% | -0.11% | -0.11% | 100% | 100% |
| Overall | -0.04% | 0.00% | -0.13% | 100% | 100% |
| Class D | 0.01% | 0.66% | -1.04% | 100% | 100% |
| Class F | 0.09% | 0.62% | -0.11% | 100% | 100% |

Conclusions

In this contribution, a simplification on the implicit transform selection in the intra sub-partitions (ISP) mode is proposed. The proposed method removes the dependency on intra prediction mode for transform selection in the ISP mode. Thus, DCT-2 and DST-7 transforms are implicitly selected based on the width of sub-partition and the height of sub-partition. By using the proposed method, the intra prediction mode is not used for the transform and inverse transform process. The experimental result reportedly shows that the proposed method introduces no coding performance impact in AI, RA, LDB cases. Therefore, it is recommended to adopt this proposal to clean up the current transform selection design.

References

- S. De-Luxán-Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe, and T. Wiegand, “CE3: Intra Sub-Partitions Coding Mode (Tests 1.1.1 and 1.1.2),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M0102, Jan. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0.1

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1010, Jan. 2019

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- Annex: Proposed draft text changes based on Draft 4 v6

- 8.7.4	Transformation process for scaled transform coefficients

- 8.7.4.1	General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are both equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT, trTypeHor and trTypeVer are derived as follows: specified in Table 8-15 depending on intraPredMode.

[Test 1]

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? 1 : 0	(8-xxxx)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? 1 : 0	(8-xxxx)

[Test 2]

trTypeHor = ( nTbW >= 4  &&  nTbW <= 8 ) ? 1 : 0	(8-xxxx)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 8 ) ? 1 : 0	(8-xxxx)

- Otherwise, if cu_sbt_flag is equal to 1, trTypeHor and trTypeVer are specified in Table 8-14 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise (sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are equal to 0), trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16  &&  nTbW <= nTbH ) ? 1 : 0	(8-1029)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16  &&  nTbH <= nTbW ) ? 1 : 0	(8-1030)

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-13 depending on tu_mts_idx[ xTbY ][ yTbY ].

The variables nonZeroW and nonZeroH are derived as follows:

nonZeroW = Min( nTbW, ( trTypeHor > 0 )  ?  16  :  32 )	(8-1031)

nonZeroH = Min( nTbH, ( trTypeVer > 0 )  ?  16  :  32 )	(8-1032)

The (nTbW)x(nTbH) array r of residual samples is derived as follows:

- When nTbH is greater than 1, each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nonZeroH − 1 is transformed to e[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.7.4.2 for each column x = 0..nonZeroW − 1 with the height of the transform block nTbH, the non-zero height of the scaled transform coefficients nonZeroH, the list d[ x ][ y ] with y = 0..nonZeroH − 1 and the transform type variable trType set equal to trTypeVer as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbH − 1.

- When nTbH and nTbW are both greater than 1, the intermediate sample values g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 are derived as follows:

g[ x ][ y ] = Clip3( CoeffMin, CoeffMax, ( e[ x ][ y ] + 64 )  >>  7 )	(8-1033)

- When nTbW is greater than 1, each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 is transformed to r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.7.4.2 for each row y = 0..nTbH − 1 with the width of the transform block nTbW, the non-zero width of the resulting array g[ x ][ y ] nonZeroW, the list g[ x ][ y ] with x = 0..nonZeroW − 1 and the transform type variable trType set equal to trTypeHor as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbW − 1.

Table 8-13 – Specification of trTypeHor and trTypeVer depending on tu_mts_idx[ x ][ y ]

| tu_mts_idx[ x0 ][ y0 ] | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| trTypeHor | 0 | 1 | 2 | 1 | 2 |
| trTypeVer | 0 | 1 | 1 | 2 | 2 |

Table 8-14 – Specification of trTypeHor and trTypeVer depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag

| cu_sbt_horizontal_flag | cu_sbt_pos_flag | trTypeHor | trTypeVer |
| --- | --- | --- | --- |
| 0 | 0 | 2 | 1 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 2 |
| 1 | 1 | 1 | 1 |

Table 8-15 – Specification of trTypeHor and trTypeVer depending on predModeIntra

| predModeIntra | trTypeHor | trTypeVer |
| --- | --- | --- |
| INTRA_PLANAR, INTRA_ANGULAR31, INTRA_ANGULAR32, INTRA_ANGULAR34, INTRA_ANGULAR36, INTRA_ANGULAR37 | ( nTbW >= 4 && nTbW <= 16 ) ? 1 : 0 | ( nTbH >= 4 && nTbH <= 16 ) ? 1 : 0 |
| INTRA_ANGULAR33, INTRA_ANGULAR35 | 0 | 0 |
| INTRA_ANGULAR2, INTRA_ANGULAR4,…,INTRA_ANGULAR28, INTRA_ANGULAR30, INTRA_ANGULAR39, INTRA_ANGULAR41,…,INTRA_ANGULAR63, INTRA_ANGULAR65 | ( nTbW >= 4 && nTbW <= 16 ) ? 1 : 0 | 0 |
| INTRA_ANGULAR3, INTRA_ANGULAR5,…, INTRA_ANGULAR27, INTRA_ANGULAR29, INTRA_ANGULAR38, INTRA_ANGULAR40,…,INTRA_ANGULAR64, INTRA_ANGULAR66 | 0 | ( nTbH >= 4 && nTbH <= 16 ) ? 1 : 0 |