| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0407-v2 |
| --- | --- |

| Title: | CE9: Removal of 4xN/8x8 CUs and Boundary Padding Process from DMVR (Test 9.1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen1, Wei-Jung Chien2, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +886-3-566-1980 1chunchic@qti.qualcomm.com, 2wchien@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution studies the performance impact on Decoder-Side Motion Vector Derivation (DMVR) in the context of Core Experiment 9 (JVET-M1029) when 4xN/8x8 CUs and  the boundary padding process are removed. Specifically, the first test imposes an additional constraint on DMVR to prevent it from accessing CUs of size 4xN and 8x8, where N denotes power-of-2 integers. Thus, the minimal processing unit of DMVR is increased from 64 pixels to 128. On top of the first test, the second one removes the boundary padding process that is used to extend the size of reference blocks with 2 extra pixels at each side. As instead, it loads corresponding pixels from reference pictures directly to fill out the extended area. Experiments were conducted on top of the VTM-4.0 with Random-Access testing condition. The performance results are summarized as follows.

CE9-1.1a vs. VTM: (Y) 0.04%, (U) 0.07%, (V) 0.03%, (Enc.) 100%, (Dec.) 99%.

CE9-1.1b  vs. VTM: (Y) 0.04%, (U) 0.04%, (V) 0.05%, (Enc.)   99%, (Dec.) 99%.

# Introduction

Decoder-side Motion Vector Refinement (DMVR) [1] is a bi-directional prediction technique for Merge blocks (i.e. not sub-block merge and not triangle merge) with two initially signalled motion vectors (MV) that can be further refined by using bilateral matching. A CU-size constraint is imposed on DMVR, so DMVR performs only for CUs with 64 pixels at least and 8 pixels in height at least. All CUs that satisfy the CU-size constraint would be further split down to equally-partitioned sub-blocks and then DMVR performs on a sub-block by sub-block basis. The maximal unit size of sub-blocks is 16x16, so that both of the sub-block width and sub-block height must fall below or equal to 16 pixels (e.g. four 16x16 sub-blocks for a 32x32 CU; two 16x8 sub-blocks for a 32x8 CU). After motion refinement, motion compensation (MC) performs based on the reference blocks pointed to by the original MVs from bi-directional reference pictures and the refined MVs. As the refined MVs may point outside the area of reference blocks, a boundary padding process is enforced to extend the size of reference blocks to cover the non-overlapping area between refined MVs’ reference blocks and the original ones. Figure 1 gives an example that when one of the original MVs is shifted, e.g., 2 pixels toward top-left direction, the corresponding reference block may contain some pixels that do not overlap with the original reference block. Thus, a boundary padding process is required to pad out those non-overlapping pixels in order to let MV operate properly as if the reference block of the refine MV were loaded from the corresponding reference picture.

In CE9-1.1, DMVR is tested with (Test a) 4xN and 8x8 CUs disabled, where N represents power-of-2 integers, and (Test b) removing boundary padding process by using extra samples loaded located at the mentioned non-overlapping area from reference pictures. Technical details of each respective tests will be presented in the next section. Performance results and complexity analysis (i.e. memory bandwidth and SAD comparisons) will be presented in Section 3.

| (a) Reference block of original MV | (b) Reference block of refined MV |
| --- | --- |

- Supposing current CU size is WxH in the middle square, the size of reference block pointed to by an original MV is of (w+7)x(H+7). After the original MV is refined, the corresponding reference block pointed to by the refined MV may be located somewhat different from that of the original MV. (Blue circle indicates the reference samples loaded from reference picture; white circle represents missing samples that have to be padded out from blue ones at boundaries.)

# Technical Description

## Removal of 4xN and 8x8 CUs

This coding tool imposes an additional CU-size constraint on DMVR to avoid using CUs of size 4xN and 8x8 for DMVR. Table 1 indicates the differences of CU-size constraint explicitly between VTM-4.0 and the proposed one. As can be seen, the minimal unit size of DMVR is increased from 64 pixels to 128.

- CU-size constraint of VTM-4.0 and the proposed method.

|  | VTM-4.0 | Proposed |
| --- | --- | --- |
| Minimal CU width | 4 pixels | 8 pixels |
| Minimal CU height | 8 pixels | 8 pixels |
| Minimal CU size | 64 pixels (4x16 and 8x8) | 128 pixels (8x16 and 16x8) |

## Removal of Boundary Padding Process

This coding tool removes the boundary padding process that is used in VTM-4.0 to extend the size of reference blocks with 2 extra pixels at most at each side (Figure 1). As instead, it loads corresponding pixels at the extended area from reference pictures directly to fill out the extended area (i.e. the white circles in Figure 1). Specifically, as illustrated in Figure 2, VTM-4.0 only loads the reference block (i.e. (W+7)x(H+7)) pointed by the original MV and pad out missing pixels at extended area using the boundary pixels of the original reference block. The proposed method removes the boundary padding process, and thus in additional to the original reference block it has to load extra samples from each reference picture for filling out those sitting on the extended area. In total, this method requires loading (W+7+4)x(H+7+4) pixels from each reference picture per reference list.

To ensure that the worst-case memory bandwidth is not getting worse, small CUs as specified in Sec. 2.1 are always disabled for DMVR.

- Illustration of the original reference block (i.e. blue circles) pointed to by the original MV and the extended area for padded pixels (i.e. white circles).

# Experimental Results

The experiments for DMVR with the proposed methods were conducted on top of the VTM-4.0 software with the common test conditions [3] to collect BD-rate numbers, encoding-time ratio and decoding-time ratio. The VTM anchor and testing results were generated using the VTM configurations with --PrintHexPSNR=1 and --SIMD=SSE42 for encoder and --SIMD=SSE42 for decoder. Macro settings as specified in the VTM-4.0 software are not changed. Parallel encoding and decoding were applied to Random Access case for both anchors and tests.

## Coding Performance

Two tests were conducted. The respective performance results are summarized in Table 2. In addition, the performance results of DMVR off as recorded from AHG13 [4] report are also provided in Table 3 for comparison.

(CE9-1.1a) Removing 8x8/4xN CUs from DMVR;

(CE9-1.1b) CE9-1.1.a + Removing boundary padding process from DMVR.

- Performance results of CE9-1.1a and CE9-1.1b relative to VTM anchor.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE9-1.1a vs. VTM-4.0 |  |  |  |  | CE9-1.1b vs. VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.07% | -0.02% | 99% | 99% | 0.01% | 0.07% | 0.02% | 99% | 100% |
| Class A2 | 0.02% | 0.10% | 0.13% | 100% | 100% | 0.03% | 0.07% | 0.12% | 98% | 98% |
| Class B | 0.04% | 0.06% | 0.03% | 100% | 99% | 0.03% | 0.03% | 0.02% | 99% | 99% |
| Class C | 0.07% | 0.06% | 0.01% | 100% | 100% | 0.07% | 0.00% | 0.06% | 100% | 99% |
| Overall | 0.04% | 0.07% | 0.03% | 100% | 99% | 0.04% | 0.04% | 0.05% | 99% | 99% |
| Class D | 0.12% | 0.08% | 0.07% | 100% | 100% | 0.11% | 0.04% | 0.04% | 100% | 100% |
| Class F | 0.02% | 0.06% | 0.01% | 100% | 100% | 0.02% | 0.02% | 0.00% | 100% | 99% |

- Performance results of DMVR off relative to VTM anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DMVR off vs. VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.72% | 0.99% | 1.17% | 99% | 94% |
| Class A2 | 1.27% | 1.71% | 1.74% | 100% | 94% |
| Class B | 0.53% | 0.84% | 0.83% | 100% | 96% |
| Class C | 0.61% | 0.85% | 0.83% | 99% | 92% |
| Overall | 0.73% | 1.05% | 1.08% | 100% | 94% |
| Class D | 0.66% | 0.85% | 0.74% | 100% | 91% |
| Class F | 0.18% | 0.33% | 0.29% | 95% | 90% |

## Computational Complexity

The computational complexity of DMVR is measured by the worst-case number of comparisons introduced from bilateral matching. During the matching process, DMVR requires to search over 25 positions on its search range, and thus 24 comparisons are incurred per sub-block in order to find the best one from them. For example, in VTM-4.0, a CTU may be split equally down to 4x16 CUs, so the number of comparisons per CTU is 128/4 * 128/16 * 24 = 6144. Specifically, this can be formulated as follows:

C(W,H) = 128/W * 128/H * 24,

where W and H represents the width and height of a given CU, respectively.

- Number of comparisons per CTU.

|  | (VTM-4.0) 4x16 DMVR | (VTM-4.0) 8x8 DMVR | (CE9-1.1a) 8x16/16x8 DMVR | (CE9-1.1b) 8x16/16x8 DMVR |
| --- | --- | --- | --- | --- |
| C(W,H) | 6144 | 6144 | 3072 | 3072 |

## Memory Bandwidth

The memory bandwidth of DMVR is measured by using the same methodology as introduced in previous CE9 summary report (JVET-M0029) to calculate the number of required reference samples for MC, as follows:

F(W, H, Taps, SR) = ((W + 2 * SR) + Taps - 1 ) * ((H + 2 * SR) + Taps - 1 ) * 2,

where W and H represents the width and height of a given CU, respectively, Taps is the interpolation filter length of MV, SR denotes the search range (i.e. 2) of DMVR and the trailing 2 is for bi-prediction. To understand the full impact on bandwidth when introducing a new coding tool to DMVR, this contribution takes the memory bandwidth of Y/U/V channels into account. Besides, in order to compare the memory bandwidth among different CU sizes, this contribution normalizes the memory bandwidth numbers by the number of pixels of a given CU, as follows:

Fyuv1(W, H) = ( F(W, H, 8, 0) + F(W>>1, H>>1, 4, 0) + F(W>>1, H>>1, 4, 0) ) / (W * H),

Fyuv2(W, H) = ( F(W, H, 8, 2) + F(W>>1, H>>1, 4, 1) + F(W>>1, H>>1, 4, 1) ) / (W * H).

- Memory bandwidth analysis.

|  | (VTM-4.0) 4x16/8x8 DMVR | (VTM-4.0) 4x8/4x16/8x8 BI | (CE9-1.1a) 8x16/16x8 DMVR | (CE9-1.1b) 8x16/16x8 DMVR |
| --- | --- | --- | --- | --- |
| Bandwidth Analysis | Fyuv1(4,16) = 11.3 Fyuv1(8,8) = 10.1 | Fyuv1(4, 8) = 14.7 Fyuv1(4,16) = 11.3 Fyuv1(8, 8) =10.1 | Fyuv1(8,16) = 7.8 Fyuv1(16,8) = 7.8 | Fyuv2(8,16) = 11.7 Fyuv2(16,8) = 11.7 |

It is reported that CE9-1.1b increases the average memory bandwidth of the VTM-4.0 DMVR (i.e. 4x16 DMVR); however, when considering the size of memory access pattern (e.g. 4x2, 4x4, 8x4, 8x8), CE9-1.1b can achieve lower memory bandwidth than that of 4x16 DMVR in VTM-4.0. This can be formulated as

P(M, N, m, n)=ceiling( ( m-1+M+L-1 ) / m ) * ceiling( ( n-1+N+L-1 ) / n ),

where m and n represent a m-by-n memory access pattern, M and N denote the block width and height and L is the length of interpolation filter. This P value is the number of memory block fetched from reference pictures. In order to compare the 4x16 DMVR in VTM-4.0 with the 8x16/16x8 DMVR in CE9-1.1b, the P value is further normalized to 8x16.

P4x16_VTM(M, N, m, n) = 2 * P(M, N, m, n),

P8x16_CE9.1.1(M, N, m, n) = 1 * P(M+4, N+4, m, n).

- Memory bandwidth analysis with various memory access pattern.

| m x n | (VTM-4.0) 4x16 DMVR | (CE9-1.1b) 8x16/16x8 DMVR |
| --- | --- | --- |
| 8x2 | 144 | 112 / 100 |
| 8x4 | 84 | 64 / 60 |
| 4x2 | 192 | 168 / 160 |
| 4x4 | 112 | 96 / 96 |
| 8x8 | 48 | 40 / 40 |
| 16x16 | 24 | 18 / 18 |

# References

- J. Chen, Y. Ye and S. Kim, “Algorithm Description for Versatile Video Coding and Test Model 4 (VTM 4),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1002, Jan. 2019.

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 4),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1001, Jan. 2019.

- F. Bossen, J. Boyce and et. al., “JVET Common Test Conditions and Software Reference Configurations for SDR Video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1010, Jan. 2019.

- W.-J. Chien, J. Boyce and et. al., “JVET AHG Report: Tool Reporting Procedure (AHG13),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N0013, Mar. 2019.

# Proposed Draft Text

The proposed texts are drafted based on JVET-M1001 (v6) [2].

## CE9-1.1a

### 8.5.1 General decoding process for coding units coded in inter prediction mode

…

The decoding process for coding units coded in inter prediction mode consists of the following ordered steps:

- The variable dmvrFlag is set equal to 0.

- The motion vector components and reference indices of the current coding unit are derived as follows:

- If merge_triangle_flag[ xCb ][ yCb ], inter_affine_flag[ xCb ][ yCb ] and merge_subblock_flag[ xCb ][ yCb ] are all equal to 0, the following applies:

- The derivation process for motion vector components and reference indices as specified in clause 8.5.2.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], and the bi-prediction weight index gbiIdx as outputs.

- When all of the following conditions are true, dmvrFlag is set equal to 1:

- sps_dmvr_enabled_flag is equal to 1

- merge_flag[ xCb ][ yCb ] is equal to 1

- both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

- mmvd_flag[ xCb ][ yCb ] is equal to 0

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

- cbWidth is greater than or equal to 8

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 64128

….

## CE9-1.1b

### 8.5.1 General decoding process for coding units coded in inter prediction mode

….

The decoding process for coding units coded in inter prediction mode consists of the following ordered steps:

- The variable dmvrFlag is set equal to 0.

- The motion vector components and reference indices of the current coding unit are derived as follows:

- If merge_triangle_flag[ xCb ][ yCb ], inter_affine_flag[ xCb ][ yCb ] and merge_subblock_flag[ xCb ][ yCb ] are all equal to 0, the following applies:

- The derivation process for motion vector components and reference indices as specified in clause 8.5.2.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], and the bi-prediction weight index gbiIdx as outputs.

- When all of the following conditions are true, dmvrFlag is set equal to 1:

- sps_dmvr_enabled_flag is equal to 1

- merge_flag[ xCb ][ yCb ] is equal to 1

- both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

- mmvd_flag[ xCb ][ yCb ] is equal to 0

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

- cbWidth is greater than or equal to 8

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 64128

….

##### 8.5.7.3.2 Luma sample 8-tap interpolation filtering process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL,

–	a list padVal[ dir ] with dir = 0,1 specifying reference sample padding direction and amount.

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

–	The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 8-9.

The luma interpolation filter coefficients fPadL[ 0 ] are set equal to fL[ xFracL ] and fPadL[ 1 ] are set equal to fL[ yFracL ]. and modified depending on padVal[ dir ] as follows for dir being equal to 0 and 1:

–	If padVal[ dir ] is equal to − 2, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 2 ] = fPadL[ xFracL ][ 2 ] + fPadL[ xFracL ][ 1 ] + fPadL[ xFracL ][ 0 ]	(8-821)

fPadL[ dir ][ 0 ] = 0		(8-822)

fPadL[ dir ][ 1 ] = 0		(8-823)

–	Otherwise, if padVal[ dir ] is equal to − 1, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 1 ] = fPadL[ dir ][ 1 ] + fPadL[ dir ][ 0 ]		(8-824)

fPadL[ dir ][ 0 ] = 0		(8-825)

–	Otherwise, if padVal[ dir ] is equal to 1, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 6 ] = fPadL[ dir ][ 6 ] + fPadL[ dir ][ 7 ]		(8-826)

fPadL[ dir ][ 7 ] = 0		(8-827)

–	Otherwise, if padVal[ dir ] is equal to 2, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 5 ] = fPadL[ dir ][ 5 ] + fPadL[ dir ][ 6 ] + fPadL[ dir ][ 7 ] 	(8-828)

fPadL[ dir ][ 6 ] = 0		(8-829)

fPadL[ dir ][ 7 ] = 0		(8-830)

….

##### 8.5.7.3.4 Chroma sample interpolation process

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in 1/32 fractional-sample units ( xFracC, yFracC ),

–	the chroma reference sample array refPicLXC.

Output of this process is a predicted chroma sample value predSampleLXC

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthC − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthC ).

–	The variable picWC is set equal to pic_width_in_luma_samples / SubWidthC and the variable picHC is set equal to pic_height_in_luma_samples / SubHeightC.

The chroma interpolation filter coefficients fC[ p ] for each 1/32 fractional sample position p equal to xFracC or  yFracC are specified in Table 8-10.

The chroma interpolation filter coefficients fPadC[ 0 ] are set equal to fC[ xFracL ] and fPadC[ 1 ] are set equal to fC[ yFracL ]. and modified depending on padVal[ dir ] as follows for dir being equal to 0 and 1:

–	If padVal[ dir ] is equal to − 1, fPadC[ dir ] is modified as follows:

fPadC[ dir ][ 1 ] = fPadC[ dir ][ 1 ] + fPadC[ dir ][ 0 ]		(8-841)

fPadC[ dir ][ 0 ] = 0		(8-842)

–	Otherwise, if padVal[ dir ] is equal to 1, fPadC[ dir ] is modified as follows:

fPadC[ dir ][ 2 ] = fPadC[ dir ][ 2 ] + fPadC[ dir ][ 3 ]		(8-843)

fPadC[ dir ][ 3 ] = 0		(8-844)

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).