| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0415 |
| --- | --- |

| Title: | CE5: Coding tree block based adaptive loop filter (CE5-4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Hilmi Enes Egilmez, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | +1 858-651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this proposal, the coding tree block (CTB) based ALF scheme proposed in JVET-K0382, JVET-L0391 and JVET-M0429 was tested in VTM-4.0. Test results reportedly show 0.15%, 0.52% and 0.62% luma gain with similar encoding and decoding time, in AI, RA and LDB configuration respectively over VTM-4.0 anchor.

# Introduction

In this contribution, the coding tree block (CTB) based ALF scheme proposed in JVET-K0382, JVET-L0391 and JVET-M0429 was tested and results are reported.

# Algorithm description

## Slice-level temporal filter

Adaptive parameter set (APS) was adopted in VTM4. Each APS contains one set of signalled ALF filters, up to 32 APSs are supported. In the proposal, slice-level temporal filter is tested. A tile group can re-use the ALF information from an APS to reduce the overhead. The APSs are updated as a first-in-first-out (FIFO) buffer.

## CTB based ALF

For luma component, when ALF is applied to a luma CTB, the choice among 16 fixed, 5 temporal or 1 signaled filter sets is indicated. Only the filter set index is signalled. For one slice, only one new set of 25 filters can be signaled. If a new set is signalled for a slice, all the luma CTBs in the same slice share that set. Fixed filter sets can be used to predict the new slice-level filter set and can be used as candidate filter sets for a luma CTB as well. The number of filters is 64 in total.

For chroma component, when ALF is applied to a chroma CTB, if a new filter is signalled for a slice, the CTB used the new filter, otherwise, the most recent temporal chroma filter satisfying the temporal scalability constrain is applied.

As the slice-level temporal filter, the APSs are updated as a first-in-first-out (FIFO) buffer.

# Experimental results

The proposed method was implemented in VTM-4.0 software [1] with common test configuration [2]. In addition, smaller QPs (12 and 17) are used for evaluation under high rate case. Results are summarized in below tables and can be found in detail in the accompanies Excel tables.

## CE5-4.1: Slice-level temporal filter

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | 0.02% | 0.00% | 101% | 99% |
| Class A2 | -0.12% | -0.06% | -0.30% | 99% | 100% |
| Class B | -0.15% | -0.08% | -0.11% | 100% | 100% |
| Class C | -0.23% | -0.38% | -0.28% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.14% | -0.17% | 100% | 100% |
| Class D | -0.46% | -0.54% | -0.49% | 101% | 100% |
| Class F | -0.13% | -0.10% | -0.14% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.22% | -0.08% | -0.06% | 100% | 100% |
| Class C | -0.41% | -0.61% | -0.39% | 101% | 100% |
| Class E | -0.60% | 0.18% | 0.36% | 100% | 99% |
| Overall | -0.38% | -0.19% | -0.07% | 101% | 100% |
| Class D | -0.61% | -0.75% | -1.53% | 99% | 101% |
| Class F | -0.42% | -0.23% | -1.26% | 100% | 101% |

## CE5-4.2: CTB based ALF (Fixed filters + temporal filters)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.03% | 0.02% | 99% | 101% |
| Class A2 | -0.08% | 0.02% | 0.02% | 100% | 101% |
| Class B | -0.12% | -0.02% | -0.02% | 100% | 100% |
| Class C | -0.18% | -0.11% | -0.11% | 100% | 99% |
| Class E | -0.29% | -0.12% | -0.13% | 100% | 99% |
| Overall | -0.15% | -0.04% | -0.04% | 100% | 100% |
| Class D | -0.23% | -0.20% | -0.20% | 101% | 100% |
| Class F | -0.13% | -0.07% | -0.07% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | 0.07% | 0.02% | 100% | 100% |
| Class A2 | -0.57% | 0.06% | -0.17% | 101% | 101% |
| Class B | -0.54% | -0.22% | -0.20% | 99% | 101% |
| Class C | -0.59% | -0.35% | -0.42% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.52% | -0.14% | -0.21% | 100% | 101% |
| Class D | -0.61% | -0.76% | -0.84% | 101% | 100% |
| Class F | -0.26% | -0.11% | -0.14% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.51% | -0.08% | -0.37% | 100% | 99% |
| Class C | -0.77% | -0.26% | -0.09% | 100% | 100% |
| Class E | -0.59% | -0.60% | -0.52% | 101% | 100% |
| Overall | -0.62% | -0.27% | -0.32% | 100% | 100% |
| Class D | -0.74% | -0.16% | -0.68% | 101% | 100% |
| Class F | -0.64% | -0.20% | -0.63% | 100% | 101% |

## CE5-4.3: CTB based ALF (New filters + fixed filters + temporal filters) with QP=12, 17, 22 and 27

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.02% | 0.00% | 0.00% | 97% | 114% |
| Class C | -0.02% | -0.01% | -0.02% | 91% | 98% |
| Class E | -0.02% | -0.01% | -0.01% | 92% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.04% | -0.03% | -0.03% | 93% | 101% |
| Class F | -0.02% | -0.01% | -0.01% | 92% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.27% | -0.16% | -0.29% | 102% | 107% |
| Class C | -0.28% | -0.23% | -0.33% | 92% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.30% | -0.60% | -0.57% | 92% | 99% |
| Class F | -0.14% | -0.13% | -0.12% | 93% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.27% | -0.29% | -0.32% | 91% | 96% |
| Class E | -0.38% | -0.41% | -0.64% | 91% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.30% | -0.33% | -0.52% | 93% | 99% |
| Class F | -0.28% | -0.11% | -0.16% | #NUM! | #NUM! |

## CE5-4.4: CTB based ALF (New filters + temporal filters)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.01% | 0.01% | 0.00% | 99% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | 0.03% | 0.07% | 100% | 101% |
| Class A2 | -0.36% | -0.05% | -0.23% | 101% | 101% |
| Class B | -0.35% | -0.23% | -0.17% | 100% | 100% |
| Class C | -0.38% | -0.44% | -0.36% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.20% | -0.19% | 100% | 100% |
| Class D | -0.48% | -0.91% | -0.91% | 101% | 100% |
| Class F | -0.18% | -0.16% | -0.18% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | -0.40% | -0.16% | 101% | 99% |
| Class C | -0.63% | -0.36% | -0.39% | 99% | 99% |
| Class E | -0.59% | -0.98% | -0.23% | 98% | 101% |
| Overall | -0.53% | -0.53% | -0.26% | 100% | 99% |
| Class D | -0.66% | -0.16% | -1.69% | 100% | 100% |
| Class F | -0.62% | -0.28% | -0.55% | 100% | 100% |

## CE5-4.5: CTB based ALF (New filters + fixed filters)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.03% | 0.02% | 100% | 100% |
| Class A2 | -0.08% | 0.02% | 0.02% | 98% | 99% |
| Class B | -0.12% | -0.02% | -0.02% | 99% | 101% |
| Class C | -0.18% | -0.11% | -0.11% | 101% | 100% |
| Class E | -0.29% | -0.12% | -0.13% | 100% | 99% |
| Overall | -0.15% | -0.04% | -0.04% | 100% | 100% |
| Class D | -0.23% | -0.20% | -0.20% | 100% | 101% |
| Class F | -0.13% | -0.07% | -0.07% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | 0.18% | 0.13% | 100% | 99% |
| Class A2 | -0.39% | 0.06% | 0.05% | 100% | 100% |
| Class B | -0.35% | 0.03% | 0.04% | 99% | 99% |
| Class C | -0.39% | -0.08% | -0.15% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | 0.04% | 0.01% | 100% | 100% |
| Class D | -0.18% | -0.12% | -0.26% | 99% | 100% |
| Class F | -0.16% | 0.01% | -0.04% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.30% | -0.18% | 0.11% | 101% | 101% |
| Class C | -0.48% | 0.14% | 0.26% | 99% | 101% |
| Class E | -0.57% | -0.53% | -0.66% | 101% | 100% |
| Overall | -0.43% | -0.16% | -0.03% | 100% | 100% |
| Class D | -0.37% | -0.31% | -0.44% | 101% | 98% |
| Class F | -0.40% | -0.30% | -0.57% | 100% | 100% |

## CE5-4.6: CTB based ALF (New filters + fixed filters) with QP=12, 17, 22 and 27

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.02% | 0.00% | 0.00% | 95% | 116% |
| Class C | -0.02% | -0.01% | -0.02% | 89% | 98% |
| Class E | -0.02% | -0.01% | -0.01% | 89% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.04% | -0.03% | -0.03% | 88% | 96% |
| Class F | -0.02% | -0.01% | -0.01% | 89% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.16% | 0.01% | -0.04% | 104% | 116% |
| Class C | -0.12% | 0.00% | -0.08% | 92% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.09% | -0.22% | -0.13% | 92% | 97% |
| Class F | -0.06% | 0.01% | 0.00% | 93% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.11% | -0.09% | -0.03% | 91% | 95% |
| Class E | -0.27% | -0.39% | -0.46% | 90% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.06% | 0.06% | 0.05% | 92% | 98% |
| Class F | -0.22% | -0.26% | -0.33% | 93% | 97% |

# References

[1] VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring “JVET-M1010: JVET common test conditions and software reference configurations for SDR video”

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).