| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0429 |
| --- | --- |

| Title: | CE8-4.2 : Residual Coding for Transform Skip, with various maximum context coded bins |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jungah Choi Jin Heo Jangwon Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seunghwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the results of test CE8-4.2, an efficient coding for sign flags in a transform skip block. There are two tests – one is sending a flag which indicates whether the sign flags in a CG (coefficient group) are the same from each other with the context coded bin limitation by 2, the other is coding the sign flags with two context models when the context coded bins per sample is limited by 3. The experimental results for this CE tests are reported in the tables below:

(1) Normal QP (22, 27, 32, 37) IBC Off

|  | BD rates (AI / RA / LD) |  |  |
| --- | --- | --- | --- |
|  | Class F | SCC | Overall |
| CE8-4.2a | -3.80% / -2.12% / -1.62% | -9.61% -4.28% -2.61% | -0.12% -0.02% 0.03% |
| CE8-4.2b |  |  |  |

(2) Normal QP (22, 27, 32, 37) IBC On

|  | BD rates (AI / RA / LD) |  |  |
| --- | --- | --- | --- |
|  | Class F | SCC | Overall |
| CE8-4.2a | -2.03% -1.10% -1.21% | -2.13% -1.05% -1.76% | -0.12% -0.02% 0.05% |
| CE8-4.2b |  |  |  |

(3) Low QP (2, 7, 12, 17) IBC Off

|  | BD rates (AI / RA / LD) |  |  |
| --- | --- | --- | --- |
|  | Class F | SCC | Overall |
| CE8-4.2a | 0.65% 1.23% 0.85% | 2.14% 5.34% 2.99% | -0.03%// 0.11% |
| CE8-4.2b |  |  |  |

(4) Low QP (2, 7, 12, 17) IBC Off

|  | BD rates (AI / RA / LD) |  |  |
| --- | --- | --- | --- |
|  | Class F | SCC | Overall |
| CE8-4.2a | 0.97% 0.91% 0.95% | 3.75% 2.65% 3.12% | -0.03%// 0.11% |
| CE8-4.2b |  |  |  |

Introduction

It is observed that the signs of coefficients in the transform skip block have high correlation from each other than those of transformed coefficients. Therefore, in JVET-M0279 [1], it is proposed to have a flag that indicates whether all of the signs in the current block are identical or not, before the signs of each coefficient are coded (test a). Moreover, using the property that the signs of coefficients in TS blocks are biased, context coded sign flags in residual coding is proposed (test b). The tested algorithms are combined with the residual coding method in JVET-M0464 [2], which has limitations for context coded bins per sample by two or three for the residual coding of the TS blocks.

Proposed method

Test a

In test a, an additional flag is proposed which shows whether all of the signs in the current block are identical before the signs of each coefficient are transmitted. If the flag is true, then no more sign information is coded for the block. The proposed flag can be coded when the number of nonzero coefficients are greater than a threshold. In this contribution, the threshold is defined as 4. In this test, the number of context coded bins per sample is set to two.

Test b

In test b, the syntax element coeff_sign_flag is context coded with two context models. The ctxInc for this syntax element is determined whether the previous sign flag is positive or negative. If the lastly coded sign flag, in a coefficient scanning order, is positive (0), then ctxInc is set to 0, otherwise ctxInc is set to 1.Initially the last sign is considered as zero. In addition, the last sign flag is updated at a TU level. In this test, the number of context coded bins per sample is set to three.

Experimental results

The proposed method has been implemented using VTM-4.0 [3] and CE8-4.3 as software base and experimentally evaluated according to the test conditions in the CE8 description document [4] and the common test conditions [5].

Test a

IBC off, normal QP

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0_IBC0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | 0.21% | 0.40% | 101% | 103% |
| Class A2 | -0.01% | 0.06% | -0.06% | 101% | 104% |
| Class B | -0.04% | -0.05% | -0.01% | 99% | 99% |
| Class C | -0.24% | -0.26% | -0.18% | 100% | 105% |
| Class E | -0.09% | -0.15% | -0.22% | 102% | 106% |
| Overall | -0.12% | -0.05% | -0.02% | 100% | 103% |
| Class D | -0.15% | -0.16% | -0.19% | 101% | 107% |
| Class F | -3.80% | -2.36% | -2.33% | 106% | 102% |
| Class SCC | -9.61% | -6.94% | -7.24% | 109% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0_IBC0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.02% | 0.20% | 105% | 104% |
| Class A2 | -0.02% | -0.02% | -0.17% | 102% | 101% |
| Class B | 0.02% | 0.00% | -0.27% | 103% | 103% |
| Class C | -0.05% | -0.12% | -0.15% | 102% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.03% | -0.13% | 103% | 103% |
| Class D | 0.13% | -0.37% | -0.21% | 100% | 105% |
| Class F | -2.12% | -1.37% | -1.20% | 103% | 103% |
| Class SCC | -4.28% | -2.36% | -2.36% | 102% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0_IBC0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.29% | -0.40% | 103% | 108% |
| Class C | 0.05% | 0.14% | -0.03% | 102% | 107% |
| Class E | 0.00% | 0.89% | 0.33% | 105% | 111% |
| Overall | 0.03% | 0.15% | -0.09% | 103% | 108% |
| Class D | 0.30% | 0.24% | -0.42% | 101% | 106% |
| Class F | -1.62% | -1.06% | -1.37% | 105% | 106% |
| Class SCC | -2.61% | -1.56% | -1.46% | 106% | 109% |

IBC on, normal QP

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0_IBC1_NormalQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | 0.21% | 0.41% | 102% | 105% |
| Class A2 | 0.01% | -0.02% | -0.03% | 101% | 108% |
| Class B | -0.03% | -0.07% | -0.03% | 101% | 103% |
| Class C | -0.26% | -0.20% | -0.21% | 101% | 105% |
| Class E | -0.03% | -0.06% | 0.00% | 101% | 104% |
| Overall | -0.12% | -0.04% | 0.01% | 101% | 105% |
| Class D | -0.15% | -0.18% | 0.05% | 100% | 107% |
| Class F | -2.03% | -0.82% | -0.79% | 104% | 104% |
| Class SCC | -2.13% | -0.04% | -0.16% | 106% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0_IBC1_NormalQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.01% | 0.25% | 102% | 103% |
| Class A2 | -0.01% | -0.14% | -0.02% | 102% | 104% |
| Class B | 0.01% | -0.08% | -0.12% | 101% | 100% |
| Class C | -0.05% | -0.03% | -0.04% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.06% | 0.00% | 101% | 102% |
| Class D | 0.17% | -0.03% | -0.14% | 100% | 104% |
| Class F | -1.10% | -0.32% | -0.27% | 103% | 104% |
| Class SCC | -1.05% | 0.67% | 0.72% | 102% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0_IBC1_NormalQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.14% | -0.20% | 102% | 105% |
| Class C | 0.08% | -0.36% | -0.16% | 102% | 103% |
| Class E | 0.08% | -0.34% | 0.84% | 103% | 104% |
| Overall | 0.05% | -0.15% | 0.07% | 102% | 104% |
| Class D | 0.28% | 1.19% | -0.43% | 100% | 104% |
| Class F | -1.21% | -1.08% | -0.66% | 105% | 107% |
| Class SCC | -1.76% | -0.74% | -0.57% | 103% | 104% |

IBC off, low QP

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.10% | -0.18% | 103% | 108% |
| Class A2 | -0.02% | 0.00% | 0.00% | 102% | 102% |
| Class B | -0.02% | 0.00% | -0.02% | 101% | 99% |
| Class C | 0.04% | 0.06% | 0.08% | 100% | 102% |
| Class E | -0.02% | -0.01% | -0.02% | 101% | 102% |
| Overall | -0.03% | 0.00% | -0.02% | 101% | 102% |
| Class D | 0.09% | 0.10% | 0.09% | 101% | 111% |
| Class F | 0.65% | 0.36% | 0.37% | 107% | 104% |
| Class SCC | 2.14% | 2.22% | 2.22% | 118% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.07% | 0.07% | 102% | 106% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.04% | 0.04% | 0.05% | 101% | 100% |
| Class C | 0.25% | 0.04% | 0.03% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.63% | 0.41% | 0.25% | 99% | 107% |
| Class F | 1.23% | 0.61% | -2.71% | 105% | 105% |
| Class SCC | 5.34% | 6.82% | 6.92% | 105% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.07% | 0.03% | 101% | 103% |
| Class C | 0.24% | 0.14% | 0.13% | 102% | 105% |
| Class E | 0.03% | -0.05% | -0.04% | 100% | 100% |
| Overall | 0.11% | 0.06% | 0.05% | 101% | 103% |
| Class D | 0.64% | 0.34% | 0.46% | 99% | 107% |
| Class F | 0.85% | 1.14% | 0.89% | 104% | 107% |
| Class SCC | 2.99% | 3.87% | 3.90% | 103% | 103% |

IBC on, low QP

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.12% | -0.18% | 101% | 107% |
| Class A2 | -0.01% | 0.00% | 0.00% | 102% | 104% |
| Class B | -0.01% | -0.01% | 0.01% | 102% | 101% |
| Class C | 0.05% | 0.05% | 0.05% | 101% | 105% |
| Class E | -0.02% | -0.03% | -0.04% | 102% | 102% |
| Overall | -0.03% | -0.02% | -0.02% | 102% | 103% |
| Class D | 0.10% | 0.08% | 0.19% | 101% | 107% |
| Class F | 0.97% | 0.75% | 0.72% | 103% | 103% |
| Class SCC | 3.75% | 4.10% | 4.10% | 109% | 95% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 1.46% | 1.83% | 2.03% | 102% | 102% |
| Class C | 1.60% | 1.99% | 1.75% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 2.62% | 2.02% | 2.21% | 99% | 106% |
| Class F | 0.91% | 1.41% | 1.33% | 102% | 101% |
| Class SCC | 2.65% | 5.73% | 5.76% | 102% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.05% | 0.05% | 101% | 104% |
| Class C | 0.25% | 0.08% | 0.05% | 101% | 103% |
| Class E | 0.01% | -0.08% | -0.02% | 101% | 102% |
| Overall | 0.11% | 0.03% | 0.03% | 101% | 103% |
| Class D | 0.66% | 0.38% | 0.51% | 98% | 108% |
| Class F | 0.95% | 1.21% | 1.10% | 101% | 102% |
| Class SCC | 3.12% | 4.51% | 4.50% | 105% | 106% |

Test b

IBC off, normal QP

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0_IBC0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | 0.23% | 0.36% | 101% | 45% |
| Class A2 | 0.00% | 0.05% | -0.05% | 100% | 25% |
| Class B | -0.04% | -0.06% | -0.03% | 99% | 97% |
| Class C | -0.26% | -0.27% | -0.27% | 100% | 104% |
| Class E | -0.10% | -0.14% | -0.17% | 103% | 107% |
| Overall | -0.13% | -0.05% | -0.05% | 101% | 70% |
| Class D | -0.22% | -0.21% | -0.26% | 102% | 111% |
| Class F | -5.52% | -3.87% | -3.90% | 104% | 100% |
| Class SCC | -14.31% | -10.94% | -11.16% | 106% | 94% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0_IBC0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.02% | 0.20% | 104% | 106% |
| Class A2 | -0.02% | -0.02% | -0.17% | 102% | 102% |
| Class B | 0.02% | 0.00% | -0.27% | 103% | 103% |
| Class C | -0.05% | -0.12% | -0.15% | 102% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.03% | -0.13% | 103% | 104% |
| Class D | 0.13% | -0.37% | -0.21% | 101% | 105% |
| Class F | -3.60% | -2.59% | -2.40% | 104% | 103% |
| Class SCC | -7.12% | -4.77% | -4.73% | 102% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0_IBC0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.29% | -0.40% | 103% | 106% |
| Class C | 0.05% | 0.14% | -0.03% | 103% | 108% |
| Class E | 0.00% | 0.89% | 0.33% | 106% | 110% |
| Overall | 0.03% | 0.15% | -0.09% | 104% | 108% |
| Class D | 0.30% | 0.24% | -0.42% | 101% | 107% |
| Class F | -2.44% | -1.76% | -1.64% | 106% | 106% |
| Class SCC | -3.64% | -2.63% | -2.57% | 107% | 109% |

IBC on, normal QP

IBC off, low QP

IBC on, low QP

Reference(s)

- S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim, “Non-CE7 : Sign coding for transform skip,” JVET-M0279, Marrakech, MA, 9–18 Jan. 2019.

- B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, “Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip,” JVET-M0464, Marrakech, MA, 9–18 Jan. 2019.

- VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-M1028, Marrakech, MA, 9–18 Jan. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).