### 8.4.2	Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-1 specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 81..83 | INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM |

NOTE – : The intra prediction modes INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM are only applicable to chroma components.

IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb + cbHeight − 1 ) and ( xCb + cbWidth − 1, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_PLANAR.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and ciip_flag[ xNbX ][ yNbX ] is not equal to 1.

- pcm_flag[ xNbX ][ yNbX ] is equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The variables swapMode, ispDefaultMode1 and ispDefaultMode2 are defined as follows:

- If IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT, ispDefaultMode1 is set equal to INTRA_ANGULAR18 and ispDefaultMode2 is set equal to INTRA_ANGULAR50.

- Otherwise, ispDefaultMode1 is set equal to INTRA_ANGULAR50 and ispDefaultMode2 is set equal to INTRA_ANGULAR18.

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT, and abs( candIntraPredModeB − ispDefaultMode1 ) is less than abs( candIntraPredModeA − ispDefaultMode1 ), swapMode is set equal to true.

- Otherwise, swapMode is set equal to false;

- The candModeList[ x ] with x = 0..5 and backupMode[ y ] with y = 0, 1 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA and candIntraPredModeA is greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-9)

candModeList[ 1 ] = INTRA_PLANAR	(8-10)

candModeList[ 2 ] = INTRA_DC		(8-11)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-12)

candModeList[ 4 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-13)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-14)

backupMode[ 0 ] = 2 + ( ( candIntraPredModeA ) % 64 )

backupMode[ 1 ] = 2 + ( ( candIntraPredModeA + 59 ) % 64 )

-

-

-

-

-

-

-

-

-

- Otherwise if candIntraPredModeB is not equal to candIntraPredModeA and candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC, the following applies:

  - The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )	(8-24)

maxAB = Max( candIntraPredModeA, candIntraPredModeB )	(8-25)

  - If candIntraPredModeA and candIntraPredModeB are both greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = swapMode ? candIntraPredModeB : candIntraPredModeA	(8-26)

candModeList[ 2 ] = swapMode ? candIntraPredModeA : candIntraPredModeB	(8-27)

candModeList[ 1 ] = INTRA_PLANAR	(8-28)

candModeList[ 3 ] = INTRA_DC		(8-29)

  - If maxAB − minAB is in the range of 2 to 62, inclusive, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-30)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-31)

  - Otherwise, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-32)

candModeList[ 5 ] = 2 + ( ( maxAB ) % 64 )	(8-33)

backupMode[ 0 ] = 2 + ( ( minAB + 61 ) % 64 )

backupMode[ 1 ] = 2 + ( ( minAB − 1 ) % 64 )

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

  - Otherwise (candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC), candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = PLANAR_IDX	(8-52)

candModeList[ 1 ] = maxAB	(8-53)

candModeList[ 2 ] = –DC_IDX		(8-54)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-55)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-56)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-57)

backupMode[ 0 ] = 2 + ( ( maxAB ) % 64 )

backupMode[ 1 ] = 2 + ( ( maxAB + 59 ) % 64 )

-

-

-

-

-

-

-

-

-

-

-

-

- Otherwise, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-70)

candModeList[ 1 ] = ( candModeList[0]  = =  INTRA_PLANAR )  ?  INTRA_DC  :  	(8-71) 
		INTRA_PLANAR

candModeList[ 2 ] = ispDefaultMode1	(8-72)

candModeList[ 3 ] =  ispDefaultMode2	(8-73)

candModeList[ 4 ] = ispDefaultMode1 − 4	(8-74)

candModeList[ 5 ] = ispDefaultMode1 + 4	(8-75)

backupMode[ 0 ] = ispDefaultMode2 – 4

backupMode[ 1 ] = ispDefaultMode2 + 4

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

- The candModeList is updated as follows:

for( i = 0, j = 0, k = 0; i < 6; i++ )
	if( ( candModeList[ i ] > DC_IDX && IntraLumaRefLineIdx[ xCb ][ yCb ] > 0)  ||  
	( !candModeList[ i ]  && IntraSubPartitionsSplitType  !=  ISP_NO_SPLIT) )
		candModeList[ j++ ] = candModeList[ i ]. 
while( j < 6 )
	candModeList[ j++ ] = backupMode[ k++ ]

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- When candModeList[ 	 i ] is greater than candModeList[ j ] for i = 0..4 and for each i, j = ( i + 1 )..5, both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ] )	(8-94)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_luma_mpm_remainder[ xCb ][ yCb ].

  - For i equal to 0 to 5, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].