| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0433-v1 |
| --- | --- |

| Title: | CE3-related: Unification of MPM derivation for luma intra modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Geert Van der Auwera Luong Pham Van Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | aramasub@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes a combined MPM list derivation for luma intra mode coding. A set of six MPM candidates and two replacement modes are derived with starting point as the MPM list in VTM-4.0 for blocks predicted from reference line with index equal to 0. Modes that are disallowed for certain coding blocks are removed and replacement modes are added to the end of the MPM list as needed. This is the case for a block predicted from a reference line with index greater than 0, where both DC and planar modes are disallowed;in this case two replacement modes are added at the end of the MPM list, so that the MPM list contains 6 modes. Similarly, for a block coded with ISP, the DC mode is disallowed and in this case one replacement mode is added to the end of the list. The signalling of the MPM index, contexts and encoder RD-checks are identical to that in VTM-4.0. It is asserted that the proposed derivation is simpler and provides gains on top of the VTM-4.0. The BD-rate performance of the proposed modifications with respect to VTM-4.0 are as follows: (−0.11%, −0.03%, −0.03%) for (Y, Cb, Cr) for the AI configuration. Under the modified encoder test condition described in Section 6.3 of JVET-M1023, the BD-rate performance of the proposed modifications with respect to VTM-4.0 are as follows: (−0.04%, −0.07%, 0.01%) for (Y, Cb, Cr) for the AI configuration.

# Introduction

The MPM list derivation in VVC WD4 for luma intra mode coding (not considering combined inter-intra coding) consists of three derivations:

- For blocks predicted using reference line with line index 0 (Regular blocks)

- For blocks predicted using reference line with line index greater than 0 (MRL blocks)

- For intra subpartition (ISP)-coded blocks (ISP blocks)

The derivation of the MPM for these three cases diverge due to certain differences in the intra modes that are allowed for the three cases. For MRL blocks, planar and DC modes are disallowed. For ISP blocks, the DC mode is disallowed.

In this proposal, a combined MPM list derivation method is proposed where six intra modes are specified as MPM candidates and two replacement modes are also defined. For the special cases (where planar and/or DC modes are disallowed), the disallowed modes are removed and replacement modes are added at the end of the list as needed. The number of MPMs always remains equal to six. The proposed method provides coding benefits compared to the current MPM derivation in VVC WD4.

# Proposed method

The proposed method is very similar to the MPM list currently defined for regular blocks; some changes are included to account for supporting ISP blocks (refer to Figure 1, where the changes with respect to the VTM-4.0 regular block MPM list derivation are highlighted in yellow and the differences for the ISP blocks are highlighted in cyan; the MRL blocks share the same initial list with regular blocks. Two replacement modes are defined for the disallowed modes (PL and DC modes for MRL blocks and DC mode for ISP blocks). The preference of planar mode included in several CE3-3 tests is also included here. For some cases in ISP blocks, the HOR intra mode (or modes closer to HOR intra mode) are preferred for horizontally-split ISP subpartitions. Preference for modes is realized by placing them earlier in the MPM list.

Figure 1: Proposed MPM list derivation method. Disallowed modes are removed, and replacement modes are added to the end of the list as needed. Note that all the addition and subtraction of modes is done using the same modulo operation defined in VTM-4.0. E.g., A-1 would be implemented as 2 + ( (A+61) % 64), A+1 would be implemented as 2 + ( (A-1)%64), and so on.

The signalling of the MPM index, contexts and encoder RD-checks are identical to that in VTM-4.0.

# Simulation results

The simulation results of the proposed method are presented in Table 1. The BD-rate performance of the proposed modifications are as follows: (−0.11%, −0.03%, −0.03%) for (Y, Cb, Cr) for the AI configuration.

Table 1: Simulation results of proposed method with respect to VTM-4.0 anchor.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.07% | -0.08% |  |  |
| Class A2 | -0.09% | 0.09% | -0.07% |  |  |
| Class B | -0.10% | -0.07% | 0.02% |  |  |
| Class C | -0.03% | -0.12% | -0.02% |  |  |
| Class E | -0.16% | 0.07% | -0.07% |  |  |
| Overall | -0.11% | -0.03% | -0.03% |  |  |
| Class D | -0.09% | -0.14% | -0.19% |  |  |
| Class F | 0.12% | 0.16% | 0.15% |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.02% | -0.14% | -0.22% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.01% | 0.04% | -0.17% |  |  |
| Class F |  |  |  |  |  |

The simulation results obtained for the modified encoder test condition as described in Section 6.3 of the CE3 test description JVET-M1023 are provided in Table 2. The BD-rate performance of the proposed modifications under the modified encoder conditions (for both test and anchor) are as follows: (−0.04%, −0.07%, 0.01%) for (Y, Cb, Cr) for the AI configuration.

Table 2: Simulations results of the proposed method with modified encoder as defined in Section 6.3 or JVET-M1023. The anchor for this case is VTM-4.0 with the same modified encoder algorithm.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0-All |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.23% | -0.05% |  |  |
| Class A2 | -0.03% | -0.08% | -0.05% |  |  |
| Class B | -0.04% | 0.01% | 0.03% |  |  |
| Class C | 0.01% | -0.06% | 0.09% |  |  |
| Class E | -0.06% | -0.04% | 0.00% |  |  |
| Overall | -0.04% | -0.07% | 0.01% |  |  |
| Class D | -0.05% | 0.04% | -0.13% |  |  |
| Class F | 0.15% | 0.12% | 0.11% |  |  |

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).