| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0462 |
| --- | --- |

| Title: | Non-CE4: Mismatch between text specification and reference software on inter prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Hyeongmun Jang Naeri Park Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {junghak.nam, hm.jang, naeri.park, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

There is a mismatch between text specification and reference software in the derivation of context increment for inter_pred_idc syntax. This contribution investigated two approaches: 1) text specification is aligned to reference software and 2) reference software aligned to text specification. It has been observed some coding loss when software is aligned to text specification.

Introduction

A mismatch between current VVC WD 4 [1] and VTM-4.0 is founded. The inter_pred_idc syntax, which specifies inter prediction mode e.g. PRED_L0, PRED_L1, and PRED_BI, has different context increment behavior between text and software. Especially, value of context increment for several coding units like 4x8, 8x4, 8x8, 4x16, 16x4 blocks is equal to 4 in text but 3 in software. The current text shows as follows;

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| inter_pred_idc[ x0 ][ y0 ] | ( cbWidth + cbHeight ) != 8 ? 7 − ( ( 1 + Log2( cbWidth ) + Log2( cbHeight ) ) >> 1 ) : 4 | 4 | na | na | na | na |

Proposed method

To solve the mismatch, there are two options.

SW is aligned to text

To be aligned with text, clip operation is removed from the software. The value of context increment for 4x8, 8x4, 8x8, 4x16, 16x4 blocks is equal to 4 both text and software.

Text is aligned to SW

To be aligned with SW, clip operation is introduced. The value of context increment for 4x8, 8x4, 8x8, 4x16, 16x4 blocks is equal to 3 both text and software. In this case, there is no change of the coding performance. The proposed specification is as follows;

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| inter_pred_idc[ x0 ][ y0 ] | ( cbWidth + cbHeight ) != 8 ? Clip3(0, 3, 7 − ( ( 1 + Log2( cbWidth ) + Log2( cbHeight ) ) >> 1 )) : 4 | 4 | na | na | na | na |

To consider JVET-N0266 proposal, the value of context increment for 8x8, 4x16, 16x4 blocks could be equal to 3 for text. The modified specification is as follows;

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| inter_pred_idc[ x0 ][ y0 ] | ( cbWidth + cbHeight ) != 12 ? Clip3(0, 3, 7 − ( ( 1 + Log2( cbWidth ) + Log2( cbHeight ) ) >> 1 )) : 4 | 4 | na | na | na | na |

Experimental results

The proposed fix (2.1) has been implemented on VTM-4.0 and tested under common test conditions in [2].

Table 1. Performance of SW fix in RA

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.12% | 0.06% | 100% | 100% |
| Class A2 | 0.01% | 0.17% | 0.06% | 100% | 100% |
| Class B | 0.02% | 0.09% | 0.06% | 100% | 100% |
| Class C | 0.04% | -0.02% | -0.11% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | 0.02% | 100% | 100% |
| Class D | 0.03% | 0.00% | 0.16% | 100% | 101% |
| Class F | 0.02% | 0.05% | 0.00% | 100% | 100% |

Table 2. Performance of SW fix in LD

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.01% | -0.12% | 100% | 99% |
| Class C | 0.05% | 0.15% | -0.01% | 101% | 103% |
| Class E | 0.06% | -0.18% | -0.55% | 101% | 101% |
| Overall | 0.03% | 0.01% | -0.19% | 101% | 101% |
| Class D | 0.07% | 0.48% | -0.55% | 101% | 103% |
| Class F | -0.08% | -0.04% | 0.21% | 101% | 101% |

Conclusion

To solve the mismatch between text and software, two methods are proposed. In the case of modifying software, minor coding loss is observed. It is recommended to align text with software in the next specification.

Reference

- B. Bross, et al., “Versatile Video Coding (Draft 4),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1001, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1010, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).