| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0465 |
| --- | --- |

| Title: | CE3-related : Restrict 2xN, Nx2 chroma processing for dual tree structure |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposed draft |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Naeri Park Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seunghwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

Current 2x2 Chroma intra-prediction is a serious concern from the throughput and processing overhead point of view. In order to remove worst case complexity, CE3-2.1.1 disables chroma partitioning to 2x4, 4x2 and 2x2. In this contribution, two approaches to restrict these small blocks are suggested.

- The restriction for the chroma block partitioning is extended on top of CE3-2.1.1 by disabling 2xN, Nx2 partitioning for chroma block.

- The IBC chroma motion vector derivation process is applied based on 4x4 sub-block instead of 2x2 sub-block.

The proposed method test2 shows 0.12%, 0.04% BD-rate change for AI and RA CTC configuration, respectively.

Problem state

In the CE3-2.1.1 restricts 2x2, 2x4 and 4x2 chroma block partitioning. From this restriction, the worst case chroma processing is when the number of samples in a block is equal to 16 for the intra prediction. However in the VVC draft 4, IBC mode is allowed for I-Slice and the dual tree structure. In this case the motion vector for IBC coded Chroma block is derived based on 2x2 sub-block in dual tree structure, I_SLICE. For this reason, the worst case chroma processing is still 2x2. IBC mode refer spatial neighboring reconstructed sample similar with INTRA prediction therefore complexity form throughput and latency is not reduced in the worst case. In this contribution suggest that disable 2xN, Nx2 block partitioning for the chroma component and it is also proposed that IBC chroma motion vector derivation is applied based on 4x4 sub-block.

Proposed method

Aspect #1. CE3-2.1.1

CE3-2.1.1 describes block partition restriction in order to disable 2x2, 4x2 and 2x4 Chroma block in Dual tree.

Aspect #2. Extend Chroma block partitioning restriction

The proposed restriction for chroma block partitioning is that 2xN and Nx2 split is disabled.

Aspect #3. Proposed IBC Chroma motion vector derivation based 4x4 sub-block.

The proposed IBC Chroma motion vector derivation process is applied base on 4x4 sub-block size instead of 2x2 sub-block and luma motion vector which is corresponded with top-left position of the 4x4 Chroma sub-block.

Experimental results

#1. Test 1 CE3-2.1.1(reported by CE3-2.1.1 proponent)

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.16% | 0.11% | 100% | 100% |
| Class A2 | 0.04% | 0.39% | 0.23% | 99% | 99% |
| Class B | 0.00% | 0.18% | 0.29% | 100% | 100% |
| Class C | 0.05% | 1.06% | 1.56% | 100% | 99% |
| Class E | -0.02% | -0.05% | 0.05% | 100% | 100% |
| Overall | 0.02% | 0.37% | 0.49% | 100% | 100% |
| Class D | 0.12% | 1.80% | 2.48% | 99% | 98% |
| Class F | 0.19% | 1.56% | 1.96% | 100% | 99% |

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.05% | 100% | 100% |
| Class A2 | -0.01% | 0.20% | 0.16% | 100% | 101% |
| Class B | 0.01% | 0.17% | 0.23% | 101% | 101% |
| Class C | 0.03% | 0.77% | 0.90% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.30% | 0.36% | 101% | 101% |
| Class D | 0.05% | 0.88% | 1.48% | 101% | 101% |
| Class F | 0.12% | 0.97% | 1.26% | 100% | 100% |

# Test 2. 2xN, Nx2 chorma block size restriction

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.50% | 0.27% | 99% | 99% |
| Class A2 | 0.21% | 1.48% | 1.11% | 97% | 101% |
| Class B | 0.05% | 1.09% | 1.64% | 99% | 100% |
| Class C | 0.25% | 2.81% | 3.90% | 97% | 97% |
| Class E | -0.01% | 0.29% | 0.56% | 100% | 101% |
| Overall | 0.12% | 1.31% | 1.65% | 99% | 100% |
| Class D | 0.33% | 4.08% | 5.39% | 96% | 98% |
| Class F | 0.60% | 4.00% | 4.90% | 99% | 96% |

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.25% | -0.03% | 103% | 102% |
| Class A2 | 0.03% | 0.97% | 0.71% | 103% | 103% |
| Class B | 0.03% | 1.20% | 1.47% | 102% | 100% |
| Class C | 0.08% | 1.70% | 2.20% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 1.10% | 1.21% | 102% | 101% |
| Class D | 0.08% | 2.49% | 3.50% | 100% | 99% |
| Class F | 0.30% | 2.55% | 3.09% | 101% | 99% |

#Test 3. Test3 result on top of Test2

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.50% | 0.27% | 99% | 100% |
| Class A2 | 0.21% | 1.48% | 1.11% | 97% | 100% |
| Class B | 0.05% | 1.09% | 1.64% | 100% | 101% |
| Class C | 0.25% | 2.81% | 3.90% | 97% | 98% |
| Class E | -0.01% | 0.29% | 0.56% | 100% | 100% |
| Overall | 0.12% | 1.31% | 1.65% | 99% | 100% |
| Class D | 0.33% | 4.08% | 5.39% | 96% | 97% |
| Class F (optional) | 0.63% | 4.02% | 4.97% | 99% | 96% |

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.25% | -0.03% | 103% | 102% |
| Class A2 | 0.03% | 0.97% | 0.71% | 103% | 102% |
| Class B | 0.03% | 1.20% | 1.47% | 102% | 99% |
| Class C | 0.08% | 1.70% | 2.20% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 1.10% | 1.21% | 102% | 100% |
| Class D | 0.08% | 2.49% | 3.50% | 100% | 99% |
| Class F | 0.37% | 2.68% | 3.18% | 101% | 99% |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).