| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0472_v2 |
| --- | --- |

| Title: | Non-CE4: Simplification of decoding process for SMVD reference indices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hahyun Lee, Jungwon Kang, Sung-Chang Lim, Jinho Lee, Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-6138 {hanilee, jungwon, sclim, jinosoul, hykim5}@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution proposes that the first L0 and L1 reference picture in each reference picture list are only considered as reference pictures of SMVD to simply the decoding process of SMVD reference indices.

In version2, for the encoder flexibility, it is also proposed to explicitly signal the reference indices in the tile group header to specify the reference indices of SMVD.

Introduction

At the 13th JVET meeting, symmetric MVD (SMVD) is adopted in the Versatile Video Coding (VVC) Working Draft (WD) 4 [1]. In VVC WD 4, SMVD is only allowed when reference picture list has the nearest forward reference picture in list 0 and nearest backward reference picture in list 1 or vice versa. When there are a pair of nearest reference pictures in reference picture list, the ‘sym_mvd_flag’ can be parsed. A variable ‘RefIdxSymLX’, which indicates whether a pair of nearest reference picture exists and is used as reference indices of SMVD, is derived through the searching a pair of reference picture nearest to a current picture as below.

Table 1.  8.3.4	Decoding process for symmetric motion vector difference reference indices (VVC WD4)

Output of this process are RefIdxSymL0 and RefIdxSymL0 specifying the list 0 and list 1 reference picture indices for symmetric motion vector differences, i.e., when sym_mvd_flag is equal to 1 for a coding unit. The variable RefIdxSymLX with X being 0 and 1 is derived as follows: - The variable currPic specifies the current picture. - RefIdxSymL0 is set equal to −1. - For each index i with i = 0..NumRefIdxActive[ 0 ], the following applies: - When all of the following conditions are true, RefIdxSymL0 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > 0, - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1. - RefIdxSymL1 is set equal to −1. - For each index i with i = 0..NumRefIdxActive[ 1 ], the following applies: - When all of the following conditions are true, RefIdxSymL1 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < 0, - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1. - When RefIdxSymL1 is equal to −1 or RefIdxSymL1 is equal to −1, the following applies: - For each index i with i = 0..NumRefIdxActive[ 0 ], the following applies: - When all of the following conditions are true, RefIdxSymL0 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < 0, - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1. - For each index i with i = 0..NumRefIdxActive[ 1 ], the following applies: - When all of the following conditions are true, RefIdxSymL1 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > 0, - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1.

Proposed method

This contribution proposes that the first L0 and L1 reference picture in each reference picture list are only considered as reference pictures of SMVD to simply the decoding process of SMVD reference indices.

When first L0 and L1 reference picture exist and satisfy the following condition, the SMVD can be allowed in tile group and ‘sym_mvd_flag’ can be present and parsed in CU-level.

- DiffPicOrderCnt(currPic, RefPicList[ 0 ][ 0 ] ) is equal to DiffPicOrderCnt(RefPicList[ 1 ][ 0 ], currPic)

If this condition is satisfied and CU is bi-predictive, the ‘sym_mvd_flag’ can be parsed and when this flag is equal to 1, the reference indices (refIdx0 and refIdx1) of SMVD mode is inferred to be ‘0’. Otherwise, SMVD is not used.

Table 2.  8.3.4 Proposed decoding process for symmetric motion vector difference reference indices

Output of this process are RefIdxSymL0 and RefIdxSymL0 specifying the list 0 and list 1 reference picture indices for symmetric motion vector differences, i.e., when sym_mvd_flag is equal to 1 for a coding unit. The variable RefIdxSymLX with X being 0 and 1 is derived as follows: - The variable currPic specifies the current picture. - If DiffPicOrderCnt(currPic, RefPicList[ 0 ][ 0 ] ) is equal to DiffPicOrderCnt(RefPicList[ 1 ][ 0 ], currPic) - RefIdxSymL0 is set to 0 and RefIdxSymL1 is set to 0 - Otherwise, - RefIdxSymL0 is set to -1 and RefIdxSymL1 is set to -1

For the encoder flexibility, it is also proposed to explicitly signal the reference indices in the tile group header to specify the reference indices of symmetric motion vector difference as follows.

7.3.2.1.	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| … |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_cclm_enabled_flag | u(1) |
| if( sps_cclm_enabled_flag && chroma_format_idc = = 1 ) |  |
| sps_cclm_colocated_chroma_flag | u(1) |
| sps_mts_enabled_flag | u(1) |
| sps_num_ladf_intervals_minus2 | u(2) |
| … |  |
| } |  |

sps_smvd_enabled_flag equal to 1 specifies that the symmetric motion vector difference is enabled. sps_smvd_enabled_flag equal to 0 specifies that the symmetric motion vector difference is disable.

Tile group header syntax

General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( tile_group_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| { |  |
| mvd_l1_zero_flag | u(1) |
| if ( mvd_l1_zero_flag == 0 ) { |  |
| if ( sps_smvd_enabled_flag ) { |  |
| tile_group_smvd_enabled_flag | u(1) |
| if ( tile_group_smvd_enabled_flag ) { |  |
| for ( i = 0; i < 2; i++ ) { |  |
| if ( NumRefIdxActive[ i ] > 1 ) |  |
| sym_mvd_ref_idx[i] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| … |  |
| byte_alignment( ) |  |
| } |  |

tile_group_smvd_enabled_flag specifies whether the symmetric motion vector difference is used for inter prediction. When tile_group_smvd_enabled_flag is equal to 1, syntax element sym_mvd_flag in coding unit may be present. Otherwise, symmetric motion vector difference is not used for inter prediction. When it is not present, it is infered to be equal to 0.

sym_mvd_ref_idx[i] specifies the list i-th reference picture index for symmetric motion vector difference used for inter prediction.

When sym_mvd_ref_idx[i] is not present, it is inferred as follows:

- tile_group_smvd_enabled_flag is equal to 1, it is inferred to be equal to 0.

- Otherwise ( tile_group_smvd_enabled_flag is equal to 0 ), it is inferred to be equal to -1.

The values RefIdxSymL0 and RefIdxSymL1 are derived as follows:

- RefIdxSymL0 = sym_mvd_ref_idx[0]

- RefIdxSymL1 = sym_mvd_ref_idx[1]

It is a requirement of bitstream conformance that the following constraints apply:

- DiffPicOrderCnt(currPic, RefPicList[0] [RefIdxSymL0] ) * DiffPicOrderCnt (currPic, RefPicList[0][ RefIdxSymL1]) < 0

- DiffPicOrderCnt(currPic, RefPicList[0] [RefIdxSymL0]] ) is the smallest value among DiffPicOderCnt(currPic, RefPicList[0][i]), for i = 0..NumRefIdxActive[ 0 ] − 1.

- DiffPicOrderCnt(currPic, RefPicList[1] [RefIdxSymL1]] ) is the smallest value among among DiffPicOderCnt(currPic, RefPicList[0][j]), for j = 0..NumRefIdxActive[ 1 ] − 1.

7.3.6.5	Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { |  |
| --- | --- |
| … |  |
| } else { |  |
| if( tile_group_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && !inter_affine_flag[ x0 ][ y0 ] && RefIdxSymL0 > −1 && RefIdxSymL1 > −1 tile_group_smvd_enabled_flag ) |  |
| sym_mvd_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |

Experimental results

The proposed methods were integrated on VTM-4.0 and the experiments were performed under JVET common test conditions [2]. The experiments were conducted on the platform of Intel Xeon E5-2690 v2 (3.0GHz) and 64GB RAM with GCC 7.3.1 compiler under CentOS Linux.

Table 3 shows that there is no BD rate and complexity difference comparing to VTM-4.0.

Table 3. Performance of proposed SMVD mode compared to VTM-4.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |

Table 4 shows that when reference indices of SMVD are explicitly signaled in the tile group header, average Luma BD-rate is 0.01% with encoding times of 100% and decoding times of 100% compared to VTM-4.0

Table 4. Performance of alternative method (explicitly signaled reference indices) compared to VTM-4.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Class C | 0.02% | 0.02% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.06% | 0.06% | 0.06% | 100% | 100% |
| Class F | 0.03% | 0.03% | 0.03% | 100% | 100% |

Conclusion

This contribution proposes that the first L0 and L1 reference picture in each reference picture list are only considered as reference pictures of SMVD to simply the decoding process of SMVD reference indices. Also, it is proposed to explicitly signal the reference indices in the tile group header to specify the reference indices of SMVD. It is recommended to consider these changes into the VVC WD5 and VTM-5.0.

References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1001, Jan. 2019.

[2] F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1010, Jan. 2019.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).